NPAC SMS/Individual Service Provider Certification and Regression Test Plan

For New Entrants Certification and Existing Service Providers/Vendors Regression Testing up to and including NPAC Release 3.2.2

Chapter 10

January 19, 2004

Release 3.2.2a

Table of Contents

10.	Individual Turn Up Test Scenarios Related to NPAC Release 3.0.	3

10.1	Network Data Test Cases	4

10.2	NPA-NXX-X Test Cases	12

10.2.1	Create NPA-NXX-X Information Test Cases:	12

10.2.2	Modify NPA-NXX-X Information Test Cases:	19

10.2.3	Delete NPA-NXX-X Information Test Cases:	22

10.2.4	Query NPA-NXX-X Information Test Cases:	28

10.3	Block Information	44

10.3.1	Create Block Information Test Cases:	44

10.3.2	Modify Block Information Test Cases:	59

10.3.3	Delete Block Information Test Cases:	68

10.4	Query Block Information Test Cases:	70

10.5	[Section Place Holder Only]	74

10.6	Subscription Version Management Test Cases:	74

10.6.1	Query Subscription Version Test Cases:	74

10.7	Subscription Version Create Test Cases:	76

10.8	Subscription Version Modify Test Cases:	93

10.9	Subscription Version Delete Test Cases:	95

10.10	Subscription Version Disconnect Test Cases:	97

10.11	NPA Splits with Number Pooling	100

10.12	Resynchronization	120

10.13	Audit Test Cases:	129

# Individual Turn Up Test Scenarios Related to NPAC Release 3.0.

Section 10 contains all test cases written for individual Service Provider Turn Up testing of Release 3.0.x of the NPAC software.

## Network Data Test Cases

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 2.1 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | O |
|  |  |  |  |  |  |  | EDR LSMS |  | O |
|  | Objective: |  | SOA - Service Provider Personnel attempt to delete an NPA-NXX that is part of NPA-NXX-X Information (Block Data does not exist). - Error |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RX3-3.1 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | B4.1.6 NPA-NXX Deletion by the SOA |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  | N/A |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that the NPA-NXX-X Information exists on the NPAC SMS respective to the NPA-NXX being deleted. - Verify that there are no Subscription Versions with LNP Type of LISP or LSPP and a status other than ‘old’ without a Failed SP List or ‘cancelled’ associated with the NPA-NXX to be deleted. - Verify that a Block respective to the NPA-NXX-X that will be used in this Test Case does not exist, nor does a Block Create Event exist. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | N/A |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | - Using their SOA, Service Provider Personnel submit a request to the NPAC SMS to delete an NPA-NXX that they own and for which there is an associated NPA-NXX-X. - The SOA issues an M-DELETE Request serviceProvNPA-NXX to the NPAC. |  | NPAC |  | The NPAC SMS receives the M-DELETE Request from the SOA. |  |  |  |
| 2. | NPAC | - The NPAC SMS verifies that the Service Provider requesting the NPA-NXX delete request is the same as the Service Provider that owns the NPA-NXX on the NPAC SMS. - The NPAC SMS checks the NPA-NXX-X information table to see if any NPA-NXX-X objects exist for this NPA-NXX. |  | NPAC |  | - The NPAC SMS determines that an NPA-NXX-X object exists for this NPA-NXX (this violates system requirements). - The NPAC SMS rejects the NPA-NXX delete request. - The NPAC SMS logs an error indicting that the NPA-NXX delete request failed due to the existence of NPA-NXX-X information. - The NPAC SMS issues an M-DELETE Error Response to the SOA indicating processingFailure. |  |  |  |
| 3. | SP | The SOA receives the M-DELETE Response from the NPAC SMS. |  | SP |  | The NPA-NXX is not deleted. |  |  |  |
| 4. | NPAC | NPAC Personnel perform a query for the NPA-NXX. |  | NPAC |  | Verify that the NPA-NXX was not deleted from the local database. |  |  |  |
| 5. | SP – Optional | Service Provider Personnel, using either the SOA or LSMS, perform a local query for the NPA-NXX. |  | SP |  | Verify that the NPA-NXX was not deleted from their local database. |  |  |  |
| 6. | SP – Conditional | Service Provider Personnel, using either the SOA/SOA LTI or LSMS, perform an NPAC query for the NPA-NXX. |  | SP |  | Verify that the NPA-NXX was not deleted from the NPAC database. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 2.3 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | O |
|  |  |  |  |  |  |  | non-EDR LSMS |  | C |
|  |  |  |  |  |  |  | EDR LSMS |  | C |
|  | Objective: |  | LSMS - Service Provider Personnel attempt to delete an NPA-NXX that is part of NPA-NXX-X Information (Block exists with status of ‘failed’ and a Failed SP List). - Error |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RX3-3.1 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | B4.1.5 NPA-NXX Deletion by the Local SMS |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  | N/A |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that the NPA-NXX-X Information exists on the NPAC SMS respective to the NPA-NXX being deleted. - Verify that there are no Subscription Versions with LNP Type of LISP or LSPP and a status other than ‘old’ without a Failed SP List or ‘cancelled’ associated with the NPA-NXX to be deleted. - Verify that a Block exists with a status of ‘failed’ and a Failed SP List for this NPA-NXX. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | N/A |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | - Using their LSMS, Service Provider Personnel submit a request to the NPAC SMS to delete an NPA-NXX that they own and for which there is a respective NPA-NXX-X associated. - The LSMS issues an M-DELETE Request serviceProvNPA-NXX to the NPAC. |  | NPAC |  | The NPAC SMS receives the M-DELETE Request from the LSMS. |  |  |  |
| 2. | NPAC | - The NPAC SMS verifies that the Service Provider requesting the NPA-NXX delete request is the same as the Service Provider that owns the NPA-NXX on the NPAC SMS. - The NPAC SMS checks the NPA-NXX-X information table to see if any NPA-NXX-X objects exist for this NPA-NXX. |  | NPAC |  | - The NPAC SMS determines that an NPA-NXX-X object or Block with a status other than ‘old’ and an empty Failed SP List, or Subscription Versions with a status other than ‘old’ and an empty Failed SP List exist for this NPA-NXX (this violates system requirements). - The NPAC SMS rejects the NPA-NXX delete request. - The NPAC SMS logs an error indicting that the NPA-NXX delete request failed due to the existence of NPA-NXX-X information. - The NPAC SMS issues an M-DELETE Error Response to the LSMS. |  |  |  |
| 3. | SP | The LSMS receives the M-DELETE Response from the NPAC SMS. |  | SP |  | The NPA-NXX is not deleted. |  |  |  |
| 4. | NPAC | NPAC Personnel perform a query for the NPA-NXX. |  | NPAC |  | Verify that the NPA-NXX was not deleted from the local database. |  |  |  |
| 5. | SP – Optional | Service Provider Personnel, using either the SOA or LSMS, perform a local query for the NPA-NXX. |  | SP |  | Verify that the NPA-NXX was not deleted from their local database. |  |  |  |
| 6. | SP – Conditional | Service Provider Personnel, using either the SOA/SOA LTI or LSMS, perform an NPAC query for the NPA-NXX. |  | SP |  | Verify that the NPA-NXX was not deleted from the NPAC database. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 2.4 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | O |
|  |  |  |  |  |  |  | EDR LSMS |  | O |
|  | Objective: |  | SOA - Service Provider Personnel attempt to delete a LRN that is associated with a Block with a status of ‘old’ and a Failed SP List. – Error |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RX3-3.2 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | B.4.2.3 LRN Deletion by the SOA |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  | N/A |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that NPA-NXX-X and Block Information exist on the NPAC SMS that uses the LRN being deleted. - Verify that there are no Subscription Versions with LNP Type of LISP or LSPP and a status other than ‘old’ without a Failed SP List or ‘cancelled’ associated with the LRN to be deleted. - Verify that a Block with the LRN that will be used in this Test Case exists with a status of ‘old’ with a Failed SP List. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | N/A |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | - Using their SOA, Service Provider Personnel submit a request to delete an LRN that they own and for which there is an associated ‘Old’ with a FailedSP-List Block and NPA-NXX-X. - The SOA issues an M-DELETE Request serviceProvLRN to the NPAC. |  | NPAC |  | The NPAC SMS receives the M-DELETE Request from the SOA. |  |  |  |
| 2. | NPAC | - The NPAC SMS verifies that the Service Provider that submitted the LRN delete request is the same as the Service Provider that owns the LRN on the NPAC SMS. - The NPAC SMS checks the Block Information table to see if any Block objects that exist on the NPAC SMS are using this LRN. |  | NPAC |  | - The NPAC SMS determines that a Block object using this LRN exists on the NPAC SMS (this violates system requirements). - The NPAC SMS rejects the LRN delete request. - The NPAC SMS logs an error indicating that the LRN delete request failed due to the existence of an ‘active-like’ Block. - The NPAC SMS issues an M-DELETE Error Response indicating processingFailure. |  |  |  |
| 3. | SP | The SOA receives the M-DELETE Response from the NPAC SMS. |  | SP |  | The LRN is not deleted. |  |  |  |
| 4. | NPAC | NPAC Personnel perform a query for the LRN. |  | NPAC |  | Verify that the LRN was not deleted from the local database. |  |  |  |
| 5. | SP – Optional | Service Provider Personnel, using either the SOA or LSMS, perform a local query for the LRN. |  | SP |  | Verify that the LRN was not deleted from their local database. |  |  |  |
| 6. | SP – Conditional | Service Provider Personnel, using either the SOA/SOA LTI or LSMS, perform an NPAC query for the LRN. |  | SP |  | Verify that the LRN was not deleted from the NPAC database. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 2.6 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | O |
|  |  |  |  |  |  |  | EDR LSMS |  | O |
|  | Objective: |  | LSMS - Service Provider Personnel attempt to delete a LRN that is associated with a Block that has a status of ‘partial-fail’ and a Failed SP List. – Error |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RX3-3.2 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | B.4.2.7 LRN Deletion by the Local SMS |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  | N/A |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that NPA-NXX-X and Block Information exist on the NPAC SMS that uses the LRN being deleted. - Verify that there are no Subscription Versions with LNP Type of LISP or LSPP and a status other than ‘old’ without a Failed SP List or ‘cancelled’ associated with the LRN to be deleted. - Verify that a Block with the LRN that will be used in this Test Case exists with a status of ‘partial fail’ and a Failed SP List. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | N/A |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | - Using their LSMS, Service Provider Personnel submit a request to delete an LRN that they own and for which there is an associated ‘Partial-Failure’ Block (and NPA-NXX-X). - The LSMS issues an M-DELETE Request serviceProvLRN to the NPAC. |  | NPAC |  | The NPAC SMS receives the M-DELETE Request from the LSMS. |  |  |  |
| 2. | NPAC | - The NPAC SMS verifies that the Service Provider that submitted the LRN delete request is the same as the Service Provider that owns the LRN on the NPAC SMS. - The NPAC SMS checks the Block Information table to see if any Block objects that exist on the NPAC SMS are using this LRN. |  | NPAC |  | - The NPAC SMS determines that a Block object using this LRN exists on the NPAC SMS (this violates system requirements). - The NPAC SMS rejects the LRN delete request. - The NPAC SMS logs an error indicating that the LRN delete request failed due to the existence of an ‘active-like’ Block. - The NPAC SMS issues an M-DELETE error response to the LSMS. |  |  |  |
| 3. | SP | The LSMS receives the M-DELETE Response from the NPAC SMS. |  | SP |  | The LRN is not deleted. |  |  |  |
| 4. | NPAC | NPAC Personnel perform a query for the LRN. |  | NPAC |  | Verify that the LRN was not deleted from the local database. |  |  |  |
| 5. | SP – Optional | Service Provider Personnel, using either the SOA or LSMS, perform a local query for the LRN. |  | SP |  | Verify that the LRN was not deleted from their local database. |  |  |  |
| 6. | SP – Conditional | Service Provider Personnel, using either the SOA/SOA LTI or LSMS, perform an NPAC query for the LRN. |  | SP |  | Verify that the LRN was not deleted from the NPAC database. |  |  |  |

## NPA-NXX-X Test Cases

### Create NPA-NXX-X Information Test Cases:

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 3.1.1 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | R |
|  |  |  |  |  |  |  | EDR LSMS |  | R |
|  | Objective: |  | NPAC OP GUI - NPAC Personnel create NPA-NXX-X Information, where the Block Holder SPID is the same as the Code Holder SPID and must be neither a primary or secondary SPID and the NPAC SMS schedules the Number Pool Block create, and the NPAC SMS activates upon scheduled date and time. The following Service Provider configurations are in place: - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to receive the download. - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to NOT receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to NOT receive the download). - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR3-61, RR3-63, RR3-64, RR3-65, RR3-66, RR3-67.1, RR67.2, RR3-68, RR3-69, RR3-70, RR3-71, RR3-72, RR3-73, RR3-75.1, RR3-75.3, RR3-76.1, RR3-76.2, RR3-78, RR3-79.1, RR3-79.2, RR3-84, RR3-85, RR3-92, RR3-93, RR3-94, RR3-119, RR3-120, RR3-121, RR3-122, RR3-123, RR3-128, RR3-129, RR3-130, RR3-149, RR3-151, RR5-85, RR5-86, RR5-87 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 1.1 Service Provider NPA-NXX-X Create by NPAC SMS 2.2 Number Pool Block Create by NPAC SMS 2.3 Number Pool Block Create Broadcast: Successful 2.3.1 Number Pool block Create Broadcast Successful to Local SMS 2.3.2 Number Pool Block Create: Successful Broadcast |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify the NPA-NXX exists on the NPAC SMS for the NPA-NXX-X Information to be created. - Verify that there are not any ‘pending-like, no-active’ Subscription Versions (Subscription Versions with a status of ‘pending’, ‘conflict’, ‘cancel-pending’, or ‘failure’) existing for TNs within the 1K Block. - Verify that there are no ‘active’ Subscription Versions for the TNs in the NPA-NXX-X. (The Number Pool Block and 1K Block of Subscription Versions to be created should not be contaminated.) - Verify the following Service Provider configurations exist: - Service Provider (‘A’) has a filter set to receive the NPA-NXX, an EDR LSMS with the LSMS NPA-NXX-X Indicator of TRUE and a SOA NPA-NXX-X Indicator of FALSE. - Service Provider (‘B’) has a filter set to receive the NPA-NXX, an EDR LSMS with the LSMS NPA-NXX-X Indicator of FALSE and a SOA NPA-NXX-X Indicator of TRUE. - Service Provider (‘C’) has a filter set to not receive the NPA-NXX, a non-EDR LSMS with the LSMS NPA-NXX-X Indicator of TRUE and a SOA NPA-NXX-X Indicator of FALSE. - Service Provider (‘D’) has a filter set to not receive the NPA-NXX and a non-EDR LSMS with the LSMS NPA-NXX-X Indicator of FALSE and a SOA NPA-NXX-X Indicator of TRUE. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | - Using the NPAC OP GUI, NPAC Personnel submit a request to create NPA-NXX-X Information, specifying the following: - Service Provider ‘A’ as the Code Holder SPID and the Block Holder SPID - an Effective Date that is valid based on the Effective Date Window Tunable - the SOA Origination Indicator is set to FALSE - the default value as the scheduled date/time - The following attributes are required for the Number Pool Block Create Event to be scheduled: - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC - if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN - if supported by the Service Provider SOA |  | NPAC |  | - The NPAC SMS provides the serviceProvNPA-NXX-X Value, serviceProvNPA-NXX-X-EffectiveTimeStamp, and Block Holder SPID. - The NPAC SMS performs the following validations for the NPA-NXX-X Information: - Verifies that the serviceProvNPA-NXX-X value is an existing NPA-NXX on the NPAC SMS. - Verifies that the NPA-NXX-X Effective Date is greater than or equal to the Effective Date of the NPA-NXX - Verifies that the NPA-NXX-X Effective Date is greater than or equal to the current date plus the Effective Date tunable number of days. - Verifies that there is not a serviceProvNPA-NXX-X object that already exists with this NPA-NXX-X value. - Verifies that the NPA-NXX-X Service Provider ID is an existing Service Provider on the NPAC SMS. - Verifies there are not any Subscription Versions within the 1K Block with a status of ‘pending’, ‘conflict’, ‘cancel-pending’, or ‘failed’ without a respective ‘active’ Subscription Version. - The NPAC SMS performs the following validations for the Number Pool Block Create Information: - Verifies the NPA-NXX-X exists for the respective Number Pool Block. - Verifies all attributes specified are valid (performs field level validations, as well as verifies the scheduled date/time is a valid date and time and is greater than or equal to the Effective Date, and that the LRN specified is a valid LRN for the Block Holder SPID defined on the NPAC SMS). - Verifies a numberPoolBlock object does not already exist for the NPA-NXX-X specified. - Verifies there are not any Subscription Versions within the 1K Block with a status of ‘pending’, ‘conflict’, ‘cancel-pending’ or ‘failed’, without a respective ‘active’ Subscription Version. |  |  |  |
| 2. | NPAC | - The NPAC SMS issues an M-CREATE Request serviceProvNPA-NXX-X to itself. - The NPAC SMS sets the following attributes: - serviceProvNPA-NXX-X-ID - serviceProvNPA-NXX-X-Value - serviceProvNPA-NXX-X-CreationTimeStamp - serviceProvNPA-NXX-X-EffectiveTimeStamp - serviceProvNPA-NXX-X-ModifiedTimeStamp - serviceProvNPA-NXX-X-DownloadReason |  | NPAC |  | - The NPAC SMS issues an M-CREATE Response serviceProvNPA-NXX-X to itself. - The NPAC SMS ‘schedules’ the Number Pool Block Create Event based on the GUI entry for NPA-NXX-X Effective Date. |  |  |  |
| 3. | NPAC | - The NPAC SMS sends an M-CREATE request to all SOAs for the serviceProvNPA-NXX-X who support the object according to the ‘NPAC Customer SOA NPA-NXX-X’ Indicator in their Service Provider Profile, and are accepting downloads for this NPA-NXX. The following attributes are included: - serviceProvNPA-NXX-X-ID - serviceProvNPA-NXX-X-Value - serviceProvNPA-NXX-X-CreationTimeStamp - serviceProvNPA-NXX-X-EffectiveTimeStamp - serviceProvNPA-NXX-X-ModifiedTimeStamp - serviceProvNPA-NXX-X-DownloadReason - The NPAC SMS sends an M-CREATE Request to all LSMSs for the serviceProvNPA-NXX-X who support the object according to the ‘NPAC Customer LSMS NPA-NXX-X Indicator’ in their Service Provider Profile, and are accepting downloads for this NPA-NXX. The following attributes are included: - serviceProvNPA-NXX-X-ID - serviceProvNPA-NXX-X-Value - serviceProvNPA-NXX-X-CreationTimeStamp - serviceProvNPA-NXX-X-EffectiveTimeStamp - serviceProvNPA-NXX-X-ModifiedTimeStamp - serviceProvNPA-NXX-X-DownloadReason |  | SP |  | - SOAs, accepting downloads for this NPA-NXX and have their ‘NPAC Customer SOA NPA-NXX-X’ Indicator set to TRUE, receive the M-CREATE Request for the serviceProvNPA-NXX-X object. - LSMSs accepting downloads for this NPA-NXX and have their ‘NPAC Customer LSMS NPA-NXX-X’ Indicator set to TRUE, receive the M-CREATE Request for the serviceProvNPA-NXX-X object. |  |  |  |
| 4. | SP | - SOAs, who are accepting downloads for the NPA-NXX, send M-CREATE Response to the NPAC SMS indicating the serviceProvNPA-NXX-X object was successfully created. - LSMSs, who are accepting downloads for the NPA-NXX, send M-CREATE Response to the NPAC SMS indicating the serviceProvNPA-NXX-X object was successfully created. |  | NPAC |  | - The NPAC SMS receives the serviceProvNPA-NXX-X Responses from the SOAs in the region. - The NPAC SMS receives the serviceProvNPA-NXX-X Responses from the LSMSs in the region. |  |  |  |
| 5. | NPAC | NPAC Personnel perform an NPA-NXX-X Query on the NPAC SMS. |  | NPAC |  | Verify that the NPA-NXX-X exists on the NPAC SMS. |  |  |  |
| 6. | SP – Optional | Service Provider Personnel perform an NPA-NXX-X Query on their local system. |  | SP |  | - Service Provider 'A’ verifies that it has the NPA-NXX-X on its LSMS, but not its SOA (based on its NPA-NXX-X Indicators in its Service Provider Profile). - Service Provider ‘B’ verifies that it has the NPA-NXX-X on its SOA, but not its LSMS (Based on its NPA-NXX-X Indicators in its Service Provider Profile). - Service Providers ‘C’ and ‘D’ verify that they do not have the NPA-NXX-X on either system (this is based on the fact that they had a filter set to NOT receive downloads for this NPA-NXX regardless of their NPA-NXX-X Indicators in their Service Provider Profile). |  |  |  |
| 7. | SP –Conditional | Service Provider Personnel, using their local system perform an NPAC query for the NPA-NXX-X. |  | SP |  | Verify that the NPA-NXX-X exists on the NPAC SMS. |  |  |  |
| 8. | NPAC | NPAC Personnel query for the Number Pool Block Create Event. |  | NPAC |  | Verify that the Number Pool Block Create Event is scheduled according to the default, scheduled date/time. |  |  |  |
| 9. | NPAC | NPAC Personnel view the web bulletin board on the NPAC website for the respective region in which this NPA-NXX-X was created. |  | NPAC |  | Verify that the following attributes were added to the web bulletin board: - NPAC Customer ID - NPAC Customer Name - NPA-NXX-X Value - NPA-NXX-X Effective Date |  |  |  |
| 10. | NPAC | The NPA-NXX-X Effective Date is reached. |  | NPAC |  | - On the Effective Date (the scheduled date/time) the NPAC SMS issues an M-ACTION Request numberPoolBlock Create to itself. - The NPAC SMS verifies the following information: - The serviceProvNPA-NXX-X object exists for the NPA-NXX-X (respective NPA-NXX-X information). - All attributes specified are valid. - A numberPoolBlockNPAC object does not already exist for the NPA-NXX-X (a duplicate Number Pool Block does not already exist), or if one exists it has a status of ‘old’ with an empty failed SP list. - The current date is greater than or equal to the NPA-NXX-X Effective Timestamp. - No Subscription Version objects exist within the Number Pool Block with a status of ‘pending’, ‘conflict’, ‘cancel-pending’ or ‘failed’, and no active Subscription Versions exist for those TNs. |  |  |  |
| - 11. | - NPAC | - The NPAC SMS issues an M-CREATE Request numberPoolBlockNPAC to itself and sets the following attributes: - The numberPoolBlockSOA-Origination Indicator is set to FALSE. - The numberPoolBlockCreationTimeStamp, numberPoolBlockActivationTimeStamp, numberPoolBlockBroadcastTimeStamp and numberPoolBlockModifiedTimeStamp are set to the current date and time. - The numberPoolBlockStatus is set to ‘sending’. |  | NPAC |  | - The NPAC SMS issues an M-CREATE Response numberPoolBlockNPAC to itself. |  |  |  |
| - 12. | - NPAC | - The NPAC SMS issues an M-CREATE request to create the corresponding subscriptionVersionNPAC object(s). - The Subscription Versions that are created have an LNP Type set to ‘POOL’ and the status is set to ‘sending’. The subscriptionModifiedTimeStamp, subscriptionActivationTimeStamp, subscriptionBroadcastTimeStamp and subscriptionCreationTimeStamp are set to the current date and time. |  | - NPAC |  | - The NPAC SMS issues an M-CREATE Response subscriptionVersionNPAC to itself. |  |  |  |
| - 13. | - NPAC | - The NPAC SMS issues an M-ACTION Response numberPoolBlock-Create to itself. |  |  |  |  |  |  |  |
| - 14. | - NPAC | - The NPAC SMS issues an M-ACTION Request subscriptionVersionLocalSMS-Create to the non-EDR LSMSs in the region that are accepting downloads for this NPA-NXX. - At the same time as step 14.1, the NPAC SMS issues an M-CREATE Request numberPoolBlock to the EDR LSMSs in the region that are accepting downloads for this NPA-NXX. |  | SP |  | - The non-EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-ACTION Request, verify the action is valid and return an M-ACTION Response subscriptionVersionLocalSMS-Create. - The EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-CREATE Request, and return an M-CREATE Response numberPoolBlock. - Upon the first successful response from an LSMS, the NPAC SMS sets the following timestamps to the current date and time: - numberPoolBlockActivationCompleteTimeStamp - subscriptionActivationCompleteTimeStamp - numberPoolBlockModifiedTimeStamp - subscriptionModifiedTimeStamp - The non-EDR LSMSs proceed to execute all the creates specified by the M-ACTION Request and issue a subscriptionVersionLocalSMS-CreateResults M-EVENT-REPORT specifying the Subscription Versions were successfully created. |  |  |  |
| - 15. | - NPAC | - The NPAC SMS receives the M-EVENT-REPORT subscriptionVersionLocalSMS-CreateResults from each non-EDR LSMS in the region that initiated this message. |  | - NPAC |  | - The NPAC SMS responds to each of the M-EVENT-REPORT subscriptionVersionLocalSMS-CreateResults as it receives these notifications with M-EVENT-REPORT Confirmations. - The NPAC SMS waits for all subscriptionVersionLocalSMS-CreateResults notification for a tunable amount of time. - The NPAC SMS receives all the M-EVENT-REPORT confirmations from the LSMSs. |  |  |  |
| - 16. | - NPAC | - The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself. - The NPAC SMS updates all the subscriptionVersionNPAC objects (Subscription Versions) within the 1K Block that were broadcast by setting the subscriptionVersionStatus to 'active', and setting the subscriptionModifiedTimeStamp to the current date and time. |  | NPAC |  | - The NPAC SMS issues an M-SET Response to itself. |  |  |  |
| - 17. | - NPAC | - The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself. - The NPAC SMS updates the numberPoolBlock by setting the numberPoolBlockStatus to 'active' and setting the numberPoolBlockModifiedTimeStamp to the current date and time. |  | NPAC |  | - The NPAC SMS issues an M-SET Response to itself. |  |  |  |
| - 18. | - NPAC | - NPAC Personnel perform a query for the Number Pool Block and the 1K Block of Subscription Versions of LNP Type ‘POOL’. |  | - NPAC |  | - Verify the Number Pool Block exists with a status of ‘active’ and an empty Failed SP List. - Verify the 1K Block of Subscription Versions exist with an LNP Type set to ‘POOL’, a status of ‘active’ and an empty Failed SP List. - Verify data integrity (LRN and GTT data) has been maintained between the 1K Block and the Subscription Versions of LNP Type set to ‘POOL’. |  |  |  |
| - 19. | - SP – Optional | - Service Provider Personnel, perform a local query for the Number Pool Block and the 1K Block of Subscription Versions. |  | - SP |  | - Service Provider ‘A’ verifies that the Number Pool Block exists on its LSMS with a status of ‘active’ . - Service Provider ‘B’ verifies that the Number Pool Block exists on its SOA with a status of ‘active’ and an empty Failed SP List. - Service Provider ‘C’ verifies that the 1K Block of Subscription Versions exist on its LSMS with an LNP Type set to ‘POOL’. - Service Provider ‘D’ verifies that it does not have the 1K Block of Subscription Versions on its LSMS or the Number Pool Block on its SOA since it had a filter set to not receive downloads for the respective NPA-NXX. - Verify data integrity (LRN and GTT data) has been maintained between the 1K Block and the Subscription Versions of LNP Type set to ‘POOL’ on the SOA and/or LSMS. |  |  |  |
| - 20. | - SP – Conditional | - Service Provider Personnel, using their local system, perform an NPAC query for the Number Pool Block and the 1K Block of Subscription Versions. |  | - SP |  | - Verify the Number Pool Block exists with a status of ‘active’ and an empty Failed SP List exists on the NPAC SMS. - Verify the 1K Block of Subscription Versions exist with an LNP Type set to ‘POOL’, a status of ‘active’ and an empty Failed SP List exists on the NPAC SMS. - Verify data integrity (LRN and GTT data) has been maintained between the 1K Block and the Subscription Versions of LNP Type set to ‘POOL’ on the NPAC SMS. |  |  |  |

Note: When setting the ‘SOA Origination’ Indicator to FALSE in the NPA-NXX-X create, NPAC Personnel have to enter the Number Pool Block Default routing information.  This information is not sent with the NPA-NXX-X create it will be sent to LSMSs upon Number Pool Block creation/activation on the NPAC SMS.

### Modify NPA-NXX-X Information Test Cases:

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 3.2.1 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | C |
|  |  |  |  |  |  |  | EDR LSMS |  | C |
|  | Objective: |  | NPAC OP GUI - NPAC Personnel modify the Effective Date of the NPA-NXX-X Information - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR3-61, RR3-95, RR3-96, RR3-97, RR3-99, RR3-100, RR3-101 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 1.2 Service Provider NPA-NXX-X Modification by NPAC SMS |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  | 3.1.1 NPAC OP GUI - NPAC Personnel create NPA-NXX-X Information, where the Block Holder SPID is the same as the Code Holder SPID and must be neither a primary or secondary SPID and the NPAC SMS schedules the Number Pool Block create. The following Service Provider configurations are in place: - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to receive the download. - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to NOT receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to NOT receive the download). - Success |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify the NPA-NXX-X to be modified exists on the NPAC SMS, with a respective Number Pool Block Create Event scheduled to run. - Verify the current date is less than the current NPA-NXX-X Effective Date. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel submit a request to modify the Effective Date of an existing NPA-NXX-X on the NPAC SMS with a respective Number Pool Block Create Event scheduled to run. Service Provider Personnel modify the Effective Date to a date greater than the current date, as well as greater than the NPA-NXX-X Creation Date, plus the NPA-NXX-X Holder Effective Date Window tunable parameter. |  | NPAC |  | The NPAC SMS performs the following validations: - Verifies that the modified Effective Date is equal to or greater than the current date. - Verifies that the modified Effective Date for the NPA-NXX-X is equal to or greater than the NPA-NXX-X Creation Date plus the value of the NPA-NXX-X Holder Effective Date Window tunable parameter. - Determines that there is a respective Number Pool Block Create Event associated with this NPA-NXX-X, and modifies the scheduled date/time to the new NPA-NXX-X Effective Date. |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-SET Request serviceProvNPA-NXX-X to itself, to update the serviceProvNPA-NXX-X-EffectiveTimeStamp and set the serviceProvNPA-NXX-X-ModifiedTimeStamp. |  |  |  | The NPAC SMS issues an M-SET Response serviceProvNPA-NXX-X to itself. |  |  |  |
| 3. | NPAC | - The NPAC SMS sends an M-SET Request to update the serviceProvNPA-NXX-X object to all SOAs that support the NPA-NXX-X object according to the ‘SOA NPA-NXX-X Indicator’ in their Service Provider profile on the NPAC SMS and are accepting downloads for the respective NPA-NXX. - The NPAC SMS sends an M-SET Request to update the serviceProvNPA-NXX-X object to all LSMSs that support the NPA-NXX-X object according to the ‘NPAC Customer LSMS NPA-NXX-X Indicator’ in their Service Provider profile on the NPAC SMS and are accepting downloads for the respective NPA-NXX. |  | SP |  | - SOAs accepting downloads for this NPA-NXX receive the M-SET Request for the serviceProvNPA-NXX-X object. - LSMSs accepting downloads for this NPA-NXX receive the M-SET Request for the serviceProvNPA-NXX-X object. |  |  |  |
| 4. | SP | - SOAs, who are accepting downloads for the NPA-NXX and support the NPA-NXX-X object according to the ‘NPAC Customer SOA NPA-NXX-X Indicator’, send an M-SET Response to the NPAC SMS indicating the modification was successful. - LSMSs, who are accepting downloads for the NPA-NXX and support the NPA-NXX-X object according to the ‘NPAC Customer SOA NPA-NXX-X Indicator’, send an M-SET Response to the NPAC SMS indicating the modification was successful. |  | NPAC |  | - The NPAC SMS receives the serviceProvNPA-NXX-X Responses from the SOAs. - The NPAC SMS receives the serviceProvNPA-NXX-X Responses from the LSMSs. |  |  |  |
| 5. | NPAC | NPAC Personnel perform an NPA-NXX-X Query on the NPAC SMS. |  | NPAC |  | Verify that the NPA-NXX-X exists, and that the NPA-NXX-X Effective Date reflects the new, modified date. |  |  |  |
| 6. | SP – Optional | Service Provider Personnel perform an NPA-NXX-X Query on their SOA and/or LSMS. |  | SP |  | Verify the NPA-NXX-X exists on their local system and that it reflects the new, modified NPA-NXX-X Effective Date. |  |  |  |
| 7. | SP - Conditional | Service Provider Personnel, perform an NPAC SMS query for the NPA-NXX-X which was used in this Test Case. |  | SP |  | Verify the NPA-NXX-X exists on the NPAC SMS and that it reflects the new, modified NPA-NXX-X Effective Date. |  |  |  |
| 8. | NPAC | NPAC Personnel perform a Number Pool Block Create Event Query. |  | NPAC |  | Verify that the respective Number Pool Block Create Event, to this NPA-NXX-X is scheduled to run on the new, modified NPA-NXX-X Effective Date. |  |  |  |

### Delete NPA-NXX-X Information Test Cases:

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 3.3.1 | SUT Priority: |  |  | SOA LTI | N/A |  |  |
|  |  |  |  |  |  |  | SOA | C |  |  |
|  |  |  |  |  |  |  | non-EDR LSMS | R |  |  |
|  |  |  |  |  |  |  | EDR LSMS | R |  |  |
|  | Objective: |  | NPAC OP GUI - NPAC Personnel delete NPA-NXX-X Information when subordinate information (Number Pool Block and Subscription Versions) exist, post Effective Date, to 4 LSMSs with the following configurations: - 1 with LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to receive the download. - 1 with LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to NOT receive the download - 1 with LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to receive the download - 1 with LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to NOT receive the download - Success |  |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  | Change Order Number(s): |  |  | NANC 109 |  |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 | Relevant Requirement(s): |  |  | RR3-61, RR3-102, RR3-103, RR3-110, RR3-111, RR3-120, RR3-121, RR3-122, RR3-137.4 (row1), RR3-138.2 (row1), RR3-173, RR3-174, RR3-175, RR3-176, RR3-177, RR3-178, RR3-179, RR5-85, RR5-86, RR5-87, RR5-111 |  |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 | Relevant Flow(s): |  |  | 2.19 Number Pool Block De-Pool by NPAC SMS 2.20.1 Number Pool Block De-Pool Broadcast of Subscription Version and Number Pool Block Deletes 2.20.2 Number Pool Block De-Pool Broadcast Successful NPA-NXX-X Updates |  |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  | 3.1.1 NPAC OP GUI - NPAC Personnel create NPA-NXX-X Information, where the Block Holder SPID is the same as the Code Holder SPID and must be neither a primary or secondary SPID and the NPAC SMS schedules the Number Pool Block create. The following Service Provider configurations are in place: - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to receive the download. - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to NOT receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to NOT receive the download). - Success |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify the NPA-NXX-X and subordinate Number Pool Block to be deleted (in an ‘active’ status with an empty Failed-SP-List) exists on the NPAC SMS. - Verify there are not any ‘Pending-Like, with Active Pool’ Subscription Versions (Subscription Versions with ‘pending’, ‘conflict’, ‘cancel-pending’, or ‘failure’) where the Old Service Provider is the Block Holder SPID and the current active Subscription Version is of LNP Type set to ‘POOL’. - Verify there are not any ‘Pending-Like, Port-to-Original’ Subscription Versions (Subscription Versions with ‘pending’, ‘conflict’, ‘cancel-pending’, or ‘failure’) where the Port-to-Original Indicator is TRUE. - Verify that at least two Service Providers have a filter set to receive data downloads for this NPA-NXX. One Service Provider (‘A’) should have an LSMS NPA-NXX-X Indicator of TRUE and a SOA NPA-NXX-X Indicator of FALSE, and one Service Provider (‘B’) should have an LSMS NPA-NXX-X Indicator of FALSE and a SOA NPA-NXX-X Indicator of TRUE in their customer profiles on the NPAC SMS. - Verify that at least two Service Providers (known as Service Provider ‘C’ and ‘D’) have a filter set to NOT receive data downloads for this NPA-NXX. One Service Provider should have an LSMS NPA-NXX-X Indicator of TRUE and a SOA NPA-NXX-X Indicator of FALSE, and one should have an LSMS NPA-NXX-X Indicator of FALSE and a SOA NPA-NXX-X Indicator of TRUE. - Verify that the SOA Origination Indicator is set to TRUE, for the Number Pool Block that is being deleted. |  |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  |  | NPAC or SP | Expected Result |  |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel submit a request to delete an NPA-NXX-X when the NPA-NXX-X, subordinate Number Pool Block (with an ‘active’ status and empty Failed-SP-List) and subordinate, pooled Subscription Version information exist on the NPAC SMS. |  |  | NPAC | The NPAC SMS verifies that for the subordinate, pooled Subscription Versions that exist for this NPA-NXX-X, there are not any: - Subscription Versions with a status of ‘pending’, ‘conflict’, ‘cancel-pending’ or ‘failed’ where the Old Service Provider is Block Holder SPID and the current active Subscription Version is LNP Type of ‘POOL’. - Subscription Versions with a status of ‘pending’, ‘conflict’, ‘cancel-pending’ or ‘failed’ where the Port-to-Original Indicator is TRUE. |  |  |  |  |
| 2. | NPAC | - The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself, and sets the status of the Number Pool Block information to sending as well as set the numberPoolBlockBroadcastTimeStamp to the current date and time. - The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself, and sets the status of the Subscription Versions within the 1K Block to sending as well as set the subscriptionVersionModifiedTimeStamp to the current date and time. |  |  | NPAC | - The NPAC SMS issues an M-SET Response numberPoolBlock to itself. - The NPAC SMS issues an M-SET Response subscriptionVersionNPAC to itself. |  |  |  |  |
| 3. | NPAC | - The NPAC SMS sends the M-DELETE for the Subscription Version object(s) to the non-EDR LSMSs who are accepting downloads for the NPA-NXX. The Subscription Version TNs are within the 1K block and have the LNP Type set to 'POOL'. - The NPAC SMS sends the M-DELETE for the Number Pool Block object to the EDR LSMSs who are accepting downloads for the NPA-NXX. |  |  | SP | - Non-EDR LSMSs in the region, accepting downloads for this NPA-NXX receive the M-DELETE Request for subscriptionVersion objects, and issue an M-DELETE Response subscriptionVersion back to the NPAC SMS. - EDR LSMSs in the region, accepting downloads for this NPA-NXX receive the M-DELETE Request for numberPoolBlock object and issue an M-DELETE Response numberPoolBlock back to the NPAC SMS. - Upon the first successful response from an LSMS, the NPAC SMS sets the following time stamps to the current date and time: - subscriptionModifiedTimeStamp - subscriptionDisconnectCompleteTimeStamp - numberPoolBlockModifiedTimeStamp - numberPoolBlockDisconnectCompleteTimeStamp - The NPAC SMS waits for all the successful responses and retries as necessary. |  |  |  |  |
| 4. | NPAC | Once all LSMSs have responded successfully: - The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself, to update the subscriptionVersionStatus to ‘old’, and set the subscriptionModifiedTimeStamp to the current date and time. - The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself, to update the numberPoolBlockStatus to ‘old’ and set the numberPoolBlockModifiedTimeStamp to the current date and time. |  |  | NPAC | - The NPAC SMS issues an M-SET Response subscriptionVersionNPAC to itself. - The NPAC SMS issues an M-SET Response numberPoolBlockNPAC to itself. |  |  |  |  |
| 5. | NPAC | The NPAC SMS issues an M-EVENT-REPORT numberPoolBlockStatusAttributeValueChange updating the numberPoolBlockStatus to ‘old’ and setting the Failed-SP-List to empty (no SPIDs) to the Block Holder SOA. |  |  | SP | The Block Holder SOA receives the M-EVENT-REPORT from the NPAC SMS and issues an M-EVENT-REPORT Confirmation back to the NPAC SMS. |  |  |  |  |
| 6. | NPAC | The NPAC SMS issues an M-DELETE serviceProvNPA-NXX-X to itself in order to delete the NPA-NXX-X object from its database. |  |  | NPAC | The NPAC SMS issues an M-DELETE Response to itself. |  |  |  |  |
| 7. | NPAC | The NPAC SMS sends an M-DELETE Request serviceProvNPA-NXX-X to all SOAs that support this object according to the ‘NPAC Customer SOA NPA-NXX-X Indicator’ in their Service Provider profile on the NPAC SMS and are accepting downloads for this NPA-NXX. |  |  | SP | SOAs that are accepting downloads for this NPA-NXX and who support the NPA-NXX-X object receive the M-DELETE Request for the serviceProvNPA-NXX-X object. |  |  |  |  |
| 8. | NPAC | The NPAC SMS sends an M-DELETE Request serviceProvNPA-NXX-X to all LSMSs (EDR and non-EDR) that support this object according to the ‘NPAC Customer LSMS NPA-NXX-X Indicator’ in their Service Provider profile on the NPAC SMS and are accepting downloads for this NPA-NXX. |  |  | SP | Non-EDR and EDR LSMSs in the region, accepting downloads for this NPA-NXX and who support the NPA-NXX-X object receive the M-DELETE Request for serviceProvNPA-NXX-X object, and issue an M-DELETE Response serviceProvNPA-NXX-X back to the NPAC SMS. |  |  |  |  |
| 9. | SP | - SOAs send M-DELETE Response to the NPAC SMS indicating the serviceProvNPA-NXX-X object was successfully deleted. - LSMSs send M-DELETE Response to the NPAC SMS indicating the serviceProvNPA-NXX-X object was successfully deleted. |  |  | NPAC | - The NPAC SMS receives the serviceProvNPA-NXX-X Responses from the SOAs. - The NPAC SMS receives the serviceProvNPA-NXX-X Responses from the LSMSs. |  |  |  |  |
| 10. | NPAC | NPAC Personnel perform an NPA-NXX-X Query on the NPAC SMS. |  |  | NPAC | Verify that the NPA-NXX-X does not exist on the NPAC SMS. |  |  |  |  |
| 11. | SP – Optional | Service Provider Personnel perform an NPA-NXX-X Query to their local systems. |  |  | SP | - Service Provider 'A’ verifies that it does not have the NPA-NXX-X on its LSMS, nor its SOA (based on its NPA-NXX-X Indicators in its Service Provider Profile). - Service Provider ‘B’ verifies that it does not have the NPA-NXX-X on its SOA, nor its LSMS (Based on its NPA-NXX-X Indicators in its Service Provider Profile). - Service Providers ‘C’ and ‘D’ verify that they do not have the NPA-NXX-X on either system (this is based on the fact that they had a filter set to NOT receive downloads for this NPA-NXX – regardless of their NPA-NXX-X Indicators in their Service Provider Profile). |  |  |  |  |
| 12. | SP - Conditional | Service Provider Personnel, perform an NPAC SMS query for the NPA-NXX-X which was used in this Test Case. |  |  | SP | Verify that the NPA-NXX-X does not exist on the NPAC SMS. |  |  |  |  |
| 13. | NPAC | NPAC Personnel query for the Block. |  |  | NPAC | Verify that the Number Pool Block has a status of ‘old’ with an empty Failed-SP-List. |  |  |  |  |
| 14. | SP – Optional | Service Provider Personnel query for the Number Pool Block on their local system. |  |  | SP | Verify that the Number Pool Block was deleted from their SOA and/or (EDR) LSMS. |  |  |  |  |
| 15. | SP - Conditional | Service Provider Personnel, perform an NPAC SMS query for the Block which was used in this Test Case. |  |  | SP | Verify that the Number Pool Block does not exist on the NPAC SMS. |  |  |  |  |
| 16. | NPAC | NPAC Personnel query for pooled Subscription Versions within the 1K Block that was deleted in this Test Case. |  |  | NPAC | Verify that the pooled Subscription Versions have a status of ‘old’ with an empty Failed-SP-List. |  |  |  |  |
| 17. | SP - Conditional | Service Provider Personnel, perform an NPAC SMS query for pooled Subscription Versions within the 1K Block that were deleted in this Test Case. |  |  | SP | Verify that the pooled Subscription Versions do not exist on the NPAC SMS. |  |  |  |  |
| 18. | SP – Optional | Service Provider Personnel query for pooled Subscription Versions within the 1K Block that was deleted in this Test Case on their ‘non-EDR’ LSMS. |  |  | SP | Verify that the pooled Subscription Versions do not exist. |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 3.3.8 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | C |
|  |  |  |  |  |  |  | EDR LSMS |  | C |
|  | Objective: |  | NPAC OP GUI – NPAC Personnel delete an NPA-NXX-X value that has a respective Number Pool Block Create Event scheduled – Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR3-112 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 1.3 Service Provider NPA-NXX-X Deletion by NPAC SMS Prior to Number Pool Block Existence |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Verify that the NPA-NXX-X value to be deleted, exist on the NPAC SMS, with respective Number Pool Block Create Event scheduled to run. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, prior to the NPA-NXX-X Effective Date, submit a request to delete an NPA-NXX-X value that has a respective Number Pool Block Create Event scheduled to run. |  | NPAC |  | The NPAC SMS determines that there is a scheduled Number Pool Block Create Event respective to this NPA-NXX-X value – and deletes the event. |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-DELETE Request serviceProvNPA-NXX-X to itself. |  | NPAC |  | The NPAC SMS issues an M-DELETE Response numberPoolBlockNPAC to itself. |  |  |  |
| 3. | NPAC | - The NPAC SMS issues an M-DELETE serviceProvNPA-NXX-X to each SOA in the region that support the NPA-NXX-X object according to their ‘NPAC Customer SOA NPA-NXX-X Indicator’ in their Service Provider Profile on the NPAC SMS and are accepting downloads for this respective NPA-NXX. - The NPAC SMS issues an M-DELETE serviceProvNPA-NXX-X to each LSMS in the region that support the NPA-NXX-X object according to their ‘NPAC Customer LSMS NPA-NXX-X Indicator’ in their Service Provider Profile on the NPAC SMS and are accepting downloads for this respective NPA-NXX. |  | SP |  | - Each SOA in the region that is accepting downloads for this NPA-NXX, and support the NPA-NXX-X object according to their Service Provider Profile, receive the M-DELETE Request from the NPAC SMS and issue an M-DELETE Response back to the NPAC SMS indicating the object was successfully deleted. - Each LSMS in the region that is accepting downloads for this NPA-NXX, and support the NPA-NXX-X object according to their Service Provider Profile, receive the M-DELETE Request from the NPAC SMS and issue an M-DELETE Response back to the NPAC SMS indicating the object was successfully deleted. |  |  |  |
| 4. | NPAC | NPAC Personnel perform an NPA-NXX-X Query on the NPAC SMS. for the NPA-NXX-X that was deleted during this Test Case. |  | NPAC |  | Verify that the NPA-NXX-X and the Block Create Event was deleted from the NPAC SMS. |  |  |  |
| 5. | SP – Optional | Service Provider Personnel query their local system for the NPA-NXX-X value that was deleted in this Test Case. |  | SP |  | Verify that the NPA-NXX-X that was deleted in this Test Case was deleted from their respective system that supports the NPA-NXX-X object. |  |  |  |
| 6. | SP – Conditional | Service Provider Personnel, , perform an NPAC SMS query for the NPA-NXX-X value that was deleted in this Test Case. |  | SP |  | Verify that the NPA-NXX-X that was deleted in this Test Case was deleted from the NPAC SMS. |  |  |  |

### Query NPA-NXX-X Information Test Cases:

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 3.4.1 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | N/A |
|  |  |  |  |  |  |  | EDR LSMS |  | N/A |
|  | Objective: |  | SOA - Service Provider Personnel send a Query NPA-NXX-X Information request over the Interface by specifying an NPA-NXX-X-ID - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR3-113, RR3-114 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 1.4 Service Provider NPA-NXX-X Query by SOA or LSMS |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  | 3.1.1 NPAC OP GUI - NPAC Personnel create NPA-NXX-X Information, where the Block Holder SPID is the same as the Code Holder SPID and must be neither a primary or secondary SPID and the NPAC SMS schedules the Number Pool Block create. The following Service Provider configurations are in place: - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to receive the download. - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to NOT receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to NOT receive the download). - Success |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Verify that an NPA-NXX-X exists for the NPA-NXX-X ID that will be specified in this Test Case. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | - Service Provider Personnel using their SOA system, submit an NPA-NXX-X Query to the NPAC specifying an NPA-NXX-X-ID for which they are not the Block Holder. - SOA issues an M-GET Request serviceProvNPA-NXX-X for a single serviceProvNPA-NXX-X object by serviceProvNPA-NXX-X-ID to the NPAC. |  | NPAC |  | The NPAC SMS receives the M-GET Request from the SOA. |  |  |  |
| 2. | NPAC | The NPAC SMS finds the specified serviceProvNPA-NXX-X object that matches the input criteria, and issues an M-GET Response serviceProvNPA-NXX-X for the single serviceProvNPA-NXX-X object. |  | SP |  | SOA system receives the M-GET Response serviceProvNPA-NXX-X for the NPA-NXX-X query it initiated. |  |  |  |
| 3. | SP | Service Provider Personnel view the NPA-NXX-X that the NPAC SMS returned and verify the following NPA-NXX-X data attributes are provided: - NPA-NXX-X-ID - NPAC Customer ID (NPA-NXX-X Holder SPID) - NPA-NXX-X - NPA-NXX-X Effective Date - Creation Time Stamp - Last Modified Time Stamp - Download Reason |  | SP |  | All attributes are returned to the SOA. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 3.4.3 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | N/A |
|  |  |  |  |  |  |  | non-EDR LSMS |  | C |
|  |  |  |  |  |  |  | EDR LSMS |  | C |
|  | Objective: |  | LSMS - Service Provider Personnel send a Query NPA-NXX-X Information request over the Interface by specifying an NPA-NXX-X-ID - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR3-113, RR3-114 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 1.4 Service Provider NPA-NXX-X Query by SOA or LSMS |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  | 3.1.1 NPAC OP GUI - NPAC Personnel create NPA-NXX-X Information, where the Block Holder SPID is the same as the Code Holder SPID and must be neither a primary or secondary SPID and the NPAC SMS schedules the Number Pool Block create. The following Service Provider configurations are in place: - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to receive the download. - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to NOT receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to NOT receive the download). - Success |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  |  |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | - Service Provider Personnel, using their LSMS system, submit an NPA-NXX-X Query to the NPAC specifying an NPA-NXX-X-ID for which they are not the Block Holder. - LSMS issues an M-GET Request serviceProvNPA-NXX-X for a single serviceProvNPA-NXX-X object by serviceProvNPA-NXX-X-ID for the specified object. |  | SP |  | The NPAC SMS receives the M-GET Request from the LSMS. |  |  |  |
| 2. | NPAC | The NPAC SMS finds the specified serviceProvNPA-NXX-X object that matches the input criteria, and issues an M-GET Response serviceProvNPA-NXX-X for the single serviceProvNPA-NXX-X object. |  | SP |  | LSMS system receives the M-GET Response serviceProvNPA-NXX-X for the NPA-NXX-X query it initiated. |  |  |  |
| 3. | SP | Service Provider Personnel view the NPA-NXX-X that the NPAC SMS returned and verify the following NPA-NXX-X data attributes are provided: - NPA-NXX-X-ID - NPAC Customer ID (NPA-NXX-X Holder SPID) - NPA-NXX-X - NPA-NXX-X Effective Date - Creation Time Stamp - Last Modified Time Stamp - Download Reason |  | SP |  | All attributes are returned to the LSMS. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 3.4.4 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | N/A |
|  |  |  |  |  |  |  | EDR LSMS |  | N/A |
|  | Objective: |  | SOA - Service Provider Personnel send a Query NPA-NXX-X Information request over the Interface, specifying an attribute that will return many objects – Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR3-113, RR3-114 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 1.4 Service Provider NPA-NXX-X Query by SOA or LSMS |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  | 3.1.1 NPAC OP GUI - NPAC Personnel create NPA-NXX-X Information, where the Block Holder SPID is the same as the Code Holder SPID and must be neither a primary or secondary SPID and the NPAC SMS schedules the Number Pool Block create. The following Service Provider configurations are in place: - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to receive the download. - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to NOT receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to NOT receive the download). - Success |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  |  |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | - Service Provider Personnel, using their SOA system, submit an NPA-NXX-X Query to the NPAC by specifying an attribute that will return multiple NPA-NXX-Xs (e.g. SPID, a range of NPA-NXX-Xs). - SOA issues a scoped and filtered M-GET Request serviceProvNPA-NXX-X for more than one serviceProvNPA-NXX-X objects. |  | SP |  | The NPAC SMS receives the scoped and filtered M-GET Request from the SOA. |  |  |  |
| 2. | NPAC | The NPAC SMS finds all the specified serviceProvNPA-NXX-X objects that match the input criteria, and issues an M-GET Response serviceProvNPA-NXX-X linked reply, for all the serviceProvNPA-NXX-X objects. |  | SP |  | SOA system receives the M-GET Response serviceProvNPA-NXX-X for the NPA-NXX-X query it initiated. |  |  |  |
| 3. | SP | Service Provider Personnel view the NPA-NXX-X that the NPAC SMS returned and verify the following NPA-NXX-X data attributes are provided for each NPA-NXX-X: - NPA-NXX-X-ID - NPAC Customer ID (NPA-NXX-X Holder SPID) - NPA-NXX-X - NPA-NXX-X Effective Date - Creation Time Stamp - Last Modified Time Stamp - Download Reason |  | SP |  | All attributes are returned to the SOA. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 3.4.6 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | N/A |
|  |  |  |  |  |  |  | non-EDR LSMS |  | C |
|  |  |  |  |  |  |  | EDR LSMS |  | C |
|  | Objective: |  | LSMS - Service Provider Personnel send a Query NPA-NXX-X Information request over the Interface, specifying an attribute that will return many objects – Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR3-113, RR3-114 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 1.4 Service Provider NPA-NXX-X Query by SOA or LSMS |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  | 3.1.1 NPAC OP GUI - NPAC Personnel create NPA-NXX-X Information, where the Block Holder SPID is the same as the Code Holder SPID and must be neither a primary or secondary SPID and the NPAC SMS schedules the Number Pool Block create. The following Service Provider configurations are in place: - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to receive the download. - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to NOT receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to NOT receive the download). - Success |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  |  |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | - Service Provider Personnel, using their LSMS system, submit an NPA-NXX-X Query to the NPAC by specifying an attribute that will return multiple NPA-NXX-Xs (e.g., SPID, a range of NPA-NXX-Xs). - LSMS issues a scoped and filtered M-GET Request serviceProvNPA-NXX-X for more than one serviceProvNPA-NXX-X objects. |  | NPAC |  | The NPAC SMS receives the scoped and filtered M-GET Request from the SOA. |  |  |  |
| 2. | NPAC | The NPAC SMS finds all the specified serviceProvNPA-NXX-X objects that match the input criteria, and issues an M-GET Response serviceProvNPA-NXX-X linked reply, for all the serviceProvNPA-NXX-X objects. |  | SP |  | LSMS system receives the M-GET Response serviceProvNPA-NXX-X for the NPA-NXX-X query it initiated. |  |  |  |
| 3. | SP | Service Provider Personnel view the NPA-NXX-X that the NPAC SMS returned and verify the following NPA-NXX-X data attributes are provided for each NPA-NXX-X: - NPA-NXX-X-ID - NPAC Customer ID (NPA-NXX-X Holder SPID) - NPA-NXX-X - NPA-NXX-X Effective Date - Creation Time Stamp - Last Modified Time Stamp - Download Reason |  | SP |  | All attributes are returned to the LSMS. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 3.4.7 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | N/A |
|  |  |  |  |  |  |  | EDR LSMS |  | N/A |
|  | Objective: |  | SOA - Service Provider Personnel send a Query NPA-NXX-X Information request over the Interface when the SOA NPA-NXX-X Indicator is set to ‘Off’ - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR3-113, RR3-114 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 1.4 Service Provider NPA-NXX-X Query by SOA or LSMS |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  | 3.1.1 NPAC OP GUI - NPAC Personnel create NPA-NXX-X Information, where the Block Holder SPID is the same as the Code Holder SPID and must be neither a primary or secondary SPID and the NPAC SMS schedules the Number Pool Block create. The following Service Provider configurations are in place: - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to receive the download. - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to NOT receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to NOT receive the download). - Success |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Verify that for the SOA sending the NPA-NXX-X Query, their SOA NPA-NXX-X Indicator is set to FALSE in their Service Provider Profile. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | - Service Provider Personnel, using the SOA system, submit an NPA-NXX-X Query to the NPAC by specifying a single NPA-NXX-X Value. - SOA issues an M-GET Request serviceProvNPA-NXX-X for a single serviceProvNPA-NXX-X object by serviceProvNPA-NXX-X value to the NPAC. |  | NPAC |  | The NPAC SMS receives the M-GET Request from the SOA. |  |  |  |
| 2. | NPAC | The NPAC SMS finds the specified serviceProvNPA-NXX-X object that matches the input criteria, and issues an M-GET Response serviceProvNPA-NXX-X for the serviceProvNPA-NXX-X object. |  | SP |  | SOA system receives the M-GET Response serviceProvNPA-NXX-X for the NPA-NXX-X query it initiated. |  |  |  |
| 3. | SP | Service Provider Personnel view the NPA-NXX-Xs that the NPAC SMS returned and verify the following NPA-NXX-X data attributes are provided: - NPA-NXX-X-ID - NPAC Customer ID (NPA-NXX-X Holder SPID) - NPA-NXX-X - NPA-NXX-X Effective Date - Creation Time Stamp - Last Modified Time Stamp - Download Reason |  | SP |  | All attributes are returned to the SOA. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 3.4.8 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | N/A |
|  |  |  |  |  |  |  | non-EDR LSMS |  | C |
|  |  |  |  |  |  |  | EDR LSMS |  | C |
|  | Objective: |  | LSMS - Service Provider Personnel send a Query NPA-NXX-X Information request over the Interface when the LSMS NPA-NXX-X Indicator is set to ‘Off’ - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR3-113, RR3-114 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 1.4 Service Provider NPA-NXX-X Query by SOA or LSMS |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  | 3.1.1 NPAC OP GUI - NPAC Personnel create NPA-NXX-X Information, where the Block Holder SPID is the same as the Code Holder SPID and must be neither a primary or secondary SPID and the NPAC SMS schedules the Number Pool Block create. The following Service Provider configurations are in place: - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to receive the download. - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to NOT receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to NOT receive the download). - Success |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Verify that for the LSMS sending the NPA-NXX-X Query, their LSMS NPA-NXX-X Indicator is set to FALSE in their Service Provider Profile. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | - Service Provider Personnel, using the LSMS system, submit an NPA-NXX-X Query to the NPAC by specifying a single NPA-NXX-X Value. - LSMS issues an M-GET Request serviceProvNPA-NXX-X for a single serviceProvNPA-NXX-X object. |  | NPAC |  | The NPAC SMS receives the M-GET Request from the LSMS. |  |  |  |
| 2. | NPAC | The NPAC SMS finds the specified serviceProvNPA-NXX-X object that matches the input criteria, and issues an M-GET Response serviceProvNPA-NXX-X for the serviceProvNPA-NXX-X object. |  | SP |  | LSMS system receives the M-GET Response serviceProvNPA-NXX-X for the NPA-NXX-X query it initiated. |  |  |  |
| 3. | SP | Service Provider Personnel view the NPA-NXX-Xs that the NPAC SMS returned and verify the following NPA-NXX-X data attributes are provided: - NPA-NXX-X-ID - NPAC Customer ID (NPA-NXX-X Holder SPID) - NPA-NXX-X - NPA-NXX-X Effective Date - Creation Time Stamp - Last Modified Time Stamp - Download Reason |  | SP |  | All attributes are returned to the LSMS. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 3.4.9 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | N/A |
|  |  |  |  |  |  |  | EDR LSMS |  | N/A |
|  | Objective: |  | SOA - Service Provider Personnel send a Query NPA-NXX-X Information request over the Interface when a filter for the respective NPA-NXX is set for this Service Provider at the NPAC - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR3-113, RR3-114 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 1.4 Service Provider NPA-NXX-X Query by SOA or LSMS |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  | 3.1.1 NPAC OP GUI - NPAC Personnel create NPA-NXX-X Information, where the Block Holder SPID is the same as the Code Holder SPID and must be neither a primary or secondary SPID and the NPAC SMS schedules the Number Pool Block create. The following Service Provider configurations are in place: - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to receive the download. - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to NOT receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to NOT receive the download). - Success |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Verify that for the Service Provider sending the NPA-NXX-X Query, an NPA-NXX filter exists at the NPAC for the respective NPA-NXX-X value they are going to query for, such that Service Provider would not receive downloads for this value. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | - Service Provider Personnel, using the SOA system, submit an NPA-NXX-X Query to the NPAC by specifying a single NPA-NXX-X Value, when a respective NPA-NXX filter for this Service Provider exists at the NPAC. - SOA issues an M-GET Request serviceProvNPA-NXX-X for a single serviceProvNPA-NXX-X object. |  | NPAC |  | The NPAC SMS receives the M-GET Request from the SOA. |  |  |  |
| 2. | NPAC | The NPAC SMS finds the specified serviceProvNPA-NXX-X object that matches the input criteria, and issues an M-GET Response serviceProvNPA-NXX-X for the serviceProvNPA-NXX-X object. |  | SP |  | SOA system receives the M-GET Response serviceProvNPA-NXX-X for the NPA-NXX-X query it initiated. |  |  |  |
| 3. | SP | Service Provider Personnel view the NPA-NXX-Xs that the NPAC SMS returned and verify the following NPA-NXX-X data attributes are provided: - NPA-NXX-X-ID - NPAC Customer ID (NPA-NXX-X Holder SPID) - NPA-NXX-X - NPA-NXX-X Effective Date - Creation Time Stamp - Last Modified Time Stamp - Download Reason |  | SP |  | All attributes are returned to the SOA. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 3.4.10 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | N/A |
|  |  |  |  |  |  |  | non-EDR LSMS |  | C |
|  |  |  |  |  |  |  | EDR LSMS |  | C |
|  | Objective: |  | LSMS - Service Provider Personnel send a Query NPA-NXX-X Information request over the Interface when a filter for the respective NPA-NXX is set for this Service Provider at the NPAC - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR3-113, RR3-114 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 1.4 Service Provider NPA-NXX-X Query by SOA or LSMS |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  | 3.1.1 NPAC OP GUI - NPAC Personnel create NPA-NXX-X Information, where the Block Holder SPID is the same as the Code Holder SPID and must be neither a primary or secondary SPID and the NPAC SMS schedules the Number Pool Block create. The following Service Provider configurations are in place: - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to receive the download. - 1 with EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to TRUE and SOA NPA-NXX-X Indicator set to FALSE with a filter set to NOT receive the download. - 1 with non-EDR LSMS and the LSMS NPA-NXX-X Indicator set to FALSE and SOA NPA-NXX-X Indicator set to TRUE with a filter set to NOT receive the download). - Success |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Verify that for the Service Provider sending the NPA-NXX-X Query, an NPA-NXX filter exists at the NPAC for the respective NPA-NXX-X value they are going to query for, such that Service Provider would not receive downloads for this value. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | - Service Provider Personnel using the LSMS system submit an NPA-NXX-X Query to the NPAC by specifying a single NPA-NXX-X Value, when a respective NPA-NXX filter for this Service Provider exists at the NPAC. - LSMS issues an M-GET Request serviceProvNPA-NXX-X for a single serviceProvNPA-NXX-X object. |  | NPAC |  | The NPAC SMS receives the M-GET Request from the LSMS. |  |  |  |
| 2. | NPAC | The NPAC SMS finds the specified serviceProvNPA-NXX-X object that matches the input criteria, and issues an M-GET Response serviceProvNPA-NXX-X for the serviceProvNPA-NXX-X object. |  | SP |  | LSMS system receives the M-GET Response serviceProvNPA-NXX-X for the NPA-NXX-X query it initiated. |  |  |  |
| 3. | SP | Service Provider Personnel view the NPA-NXX-Xs that the NPAC SMS returned and verify the following NPA-NXX-X data attributes are provided: - NPA-NXX-X-ID - NPAC Customer ID (NPA-NXX-X Holder SPID) - NPA-NXX-X - NPA-NXX-X Effective Date - Creation Time Stamp - Last Modified Time Stamp - Download Reason |  | SP |  | All attributes are returned to the LSMS. |  |  |  |

## Block Information

### Create Block Information Test Cases:

| - A. | - TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: |  | - 4.1.1 |  | - SUT Priority: |  | - SOA LTI | - N/A |  |
|  |  |  |  |  |  |  | - SOA | - C |  |
|  |  |  |  |  |  |  | - non-EDR LSMS | - R |  |
|  |  |  |  |  |  |  | - EDR LSMS | - R |  |
|  | - Objective: |  | - SOA - Service Provider Personnel create a non-contaminated Number Pool Block – Success |  |  |  |  |  |  |
| - B. | - REFERENCES |  |  |  |  |  |  |  |  |
|  | - NANC Change Order Revision Number: |  |  |  | - Change Order Number(s): |  | - NANC 109 |  |  |
|  | - NANC FRS Version Number: |  | - 3.0.0 |  | - Relevant Requirement(s): |  | - RR3-124, RR3-125, RR3-126, RR3-130, RR3-132, RR3-144, RR3-146, RR3-150, RR3-151, RR3-152, RR3-143, RR3-180, RR5-85, RR5-86, RR5-87, RR5-89 |  |  |
|  | - NANC IIS Version Number: |  | - 3.0.0 |  | - Relevant Flow(s): |  | - 2.1 Number Pool Block Create by SOA - 2.3.1 Number Pool Block Create Broadcast to Local SMS - 2.3.2 Number Pool Block Create: Successful Broadcast |  |  |
| - C. | - PREREQUISITE |  |  |  |  |  |  |  |  |
|  | - Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | - Prerequisite NPAC Setup: |  | Verify that there are no contaminated TNs or ‘pending-like’ Subscription Versions for the range of TNs in the NPA-NXX-X. |  |  |  |  |  |  |
|  | - Prerequisite SP Setup: |  | - Verify that the NPA-NXX-X exists for the Number Pool Block that Service Provider Personnel will create during this Test Case. - Verify that the current date is equal to or greater than the NPA-NXX-X Effective Date. |  |  |  |  |  |  |
| - D. | - TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| - Row # | - NPAC or SP | - Test Step |  | - NPAC or SP |  | - Expected Result |  |  |  |
| - 1. | - SP | - Using the SOA, Service Provider Personnel, submit a M-ACTION numberPoolBlock-Create request to the NPAC SMS to create a Number Pool Block. - The request must include the following attributes: - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | NPAC |  | - The NPAC SMS receives the M-ACTION numberPoolBlock-Create Request. - The NPAC SMS verifies the following information: - The requesting SOA is the NPA-NXX-X Holder SOA. - The serviceProvNPA-NXX-X object exists for the NPA-NXX-X (respective NPA-NXX-X information). - All attributes specified are valid. - A numberPoolBlockNPAC object does not already exist for the NPA-NXX-X (a duplicate Number Pool Block does not already exist). - The current date is greater than or equal to the NPA-NXX-X-EffectiveTimeStamp. - There are not any ‘pending-like, no-active’ Subscription Version objects within the given TN range. |  |  |  |
| - 2. | - NPAC | - The NPAC SMS issues an M-CREATE Request numberPoolBlockNPAC to itself. - The NPAC SMS sets the numberPoolBlockSOA-Origination Indicator to TRUE. - The NPAC SMS sets the numberPoolBlockStatus to 'sending'. - The NPAC SMS sets the following timestamps to the current date and time: - numberPoolBlockCreationTimeStamp - numberPoolBlockActivationTimeStamp - numberPoolBlockBroadcastTimeStamp - numberPoolBlockModifiedTimeStamp |  | NPAC |  | - The NPAC SMS issues an M-CREATE Response numberPoolBlockNPAC to itself. |  |  |  |
| - 3. | - NPAC | - The NPAC SMS issues an M-CREATE Request subscriptionVersionNPAC to itself. - The NPAC SMS sets the LNP Type to ‘POOL’ for the Subscription Versions it creates within the 1K Block. - The NPAC SMS sets the Subscription Versions to ‘sending’. - The NPAC SMS sets the following timestamps to the current date and time for the Subscription Versions: - subscriptionModifiedTimeStamp - subscriptionActivationTimeStamp - subscriptionBroadcastTimeStamp - subscriptionCreationTimeStamp |  | NPAC |  | - The NPAC SMS issues an M-CREATE Response subscriptionVersionNPAC to itself. |  |  |  |
| - 4. | - NPAC | - The NPAC SMS issues an M-ACTION Response numberPoolBlock-Create to the respective NPA-NXX-X Holder SOA that initiated the Number Pool Block Create request. |  | - SP |  | - The NPA-NXX-X Holder SOA receives the M-ACTION Response from the NPAC SMS. |  |  |  |
| - 5. | - NPAC | - The NPAC SMS issues an M-EVENT-REPORT objectCreation for the numberPoolBlockNPAC to the NPA-NXX-X Holder SOA. - The following attributes are sent in the objectCreation notification: - numberPoolBlockId - numberPoolBlockSOA-Origination - numberPoolBlockCreationTimeStamp - numberPoolBlockStatus - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | SP |  | - The NPA-NXX-X Holder SOA issues an M-EVENT-REPORT Confirmation to the NPAC SMS. |  |  |  |
| - 6. | - NPAC | - The NPAC SMS issues an M-ACTION Request subscriptionVersionLocalSMS-Create to the non-EDR LSMSs in the region that are accepting downloads for this NPA-NXX. - At the same time as step 6.1, the NPAC SMS issues an M-CREATE Request numberPoolBlock to the EDR LSMSs in the region that are accepting downloads for this NPA-NXX. |  | SP |  | - The non-EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-ACTION Request, verify that the action is valid and return an M-ACTION Response subscriptionVersionLocalSMS-Create. - The EDR LSMSs that are accepting downloads for this NPA-NXX receives the M-CREATE Request, and return an M-CREATE Response numberPoolBlock. - The non-EDR LSMSs proceed to execute all the creates specified by the M-ACTION Request and issues an M-EVENT-REPORT subscriptionVersionLocalSMS-CreateResults specifying the Subscription Versions were successfully created. - The NPAC SMS waits for all subscriptionVersionLocalSMS-CreateResults notifications for a tunable amount of time. |  |  |  |
| - 7. | - NPAC | Upon the first successful response from an LSMS, the NPAC SMS sets the following timestamps to the current date and time: - numberPoolBlockActivationCompleteTimeStamp - subscriptionActivationCompleteTimeStamp - numberPoolBlockModifiedTimeStamp - subscriptionModifiedTimeStamp |  | NPAC |  | The NPAC SMS responds to each of the M-EVENT-REPORT subscriptionVersionLocalSMS-CreateResults as it receives these notifications with M-EVENT-REPORT Confirmations. |  |  |  |
| - 8. | - NPAC | - The NPAC SMS issues M-SET Request subscriptionVersionNPAC to itself. - The NPAC SMS updates the following attributes for each Subscription Version within the 1K Block with LNP Type set to ‘POOL’: - sets the subscriptionVersionStatus to 'active'. - sets the Subscription Version Failed SP List to empty. - sets the subscriptionModifiedTimeStamp to the current date and time. - The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself to update the following attributes: - sets the numberPoolBlockStatus to 'active'. - sets the Number Pool Block Failed SP List to empty. - sets the numberPoolBlockModifiedTimeStamp to the current date and time. |  | NPAC |  | - The NPAC SMS issues an M-SET subscriptionVersionNPAC Response to itself. - The NPAC SMS issues an M-SET numberPoolBlockNPAC Response to itself. |  |  |  |
| - 9. | - NPAC | - The NPAC SMS determines the SOA Origination Indicator is set to TRUE and issues an M-EVENT-REPORT numberPoolBlockStatusAttributeValueChange to the NPA-NXX-X Holder SOA to set the Number Pool Block status to 'active' and the Failed SP List to empty. |  | - SP |  | - The NPA-NXX-X Holder SOA receives the M-EVENT-REPORT from the NPAC SMS and issues an M-EVENT-REPORT Confirmation back to the NPAC SMS. |  |  |  |
| - 10. | - NPAC | - NPAC Personnel perform a query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that Service Provider Personnel created during this Test Case. |  | - NPAC |  | - Verify the Number Pool Block exists with status of ‘active’ and an empty Failed SP List. - Verify the 1K Block of Subscription Versions exist with LNP Type set to ‘POOL’, a status of ‘active’ and an empty Failed SP List. |  |  |  |
| - 11. | - SP – Optional | - Service Provider Personnel perform a local query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that Service Provider Personnel created during this Test Case. |  | - SP |  | - Verify the Number Pool Block exists with status of ‘active’ and an empty Failed SP List on the SOA. - Verify the Number Pool Block exists on the EDR LSMS. - Verify the 1K Block of Subscription Versions exist with LNP Type set to ‘POOL’, a status of ‘active’ on the non-EDR LSMS. |  |  |  |
| - 12. | - SP – Conditional | - Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that Service Provider Personnel created during this Test Case. |  | - SP |  | - Using an EDR LSMS, verify the Number Pool Block exists on the NPAC SMS with status of ‘active’ and an empty Failed SP List. - Using a non-EDR LSMS, verify the 1K Block of Subscription Versions exist on the NPAC SMS with LNP Type set to ‘POOL’, a status of ‘active’ and an empty Failed SP List. |  |  |  |
| - 13. | - NPAC | - NPAC Personnel perform a full audit for the Number Pool Block and respective POOLed Subscription Versions that were created during this test case. |  | - NPAC |  | Using the Audit Results Log verify that there were no updates issued as a result of performing the audit. If updates were made, the LSMS fails this test case. |  |  |  |

| - A. | - TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: |  | - 4.1.2 |  | - SUT Priority: |  | - SOA LTI | - N/A |  |
|  |  |  |  |  |  |  | - SOA | - O |  |
|  |  |  |  |  |  |  | - non-EDR LSMS | - R |  |
|  |  |  |  |  |  |  | - EDR LSMS | - R |  |
|  | - Objective: |  | - NPAC OP GUI - NPAC Personnel schedule a Number Pool Block Create for a contaminated Block to be run at a future date, and the NPAC SMS activates upon scheduled date and time – Success |  |  |  |  |  |  |
| - B. | - REFERENCES |  |  |  |  |  |  |  |  |
|  | - NANC Change Order Revision Number: |  |  |  | - Change Order Number(s): |  | - NANC 109 |  |  |
|  | - NANC FRS Version Number: |  | - 3.0.0 |  | - Relevant Requirement(s): |  | - RR3-75.2, RR5-92 |  |  |
|  | - NANC IIS Version Number: |  | - 3.0.0 |  | - Relevant Flow(s): |  | - 2.2 Number Pool Block Create by NPAC SMS - 2.3 Number Pool Block Create Broadcast: Successful - 2.3.1 Number Pool Block Create: Broadcast Successful to Local SMS - 2.3.2 Number Pool Block Create: Successful Broadcast |  |  |
| - C. | - PREREQUISITE |  |  |  |  |  |  |  |  |
|  | - Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | - Prerequisite NPAC Setup: |  | - Verify that the NPA-NXX-X for the Number Pool Block Create Event to be scheduled exists and the Effective Date has passed. - Verify that a respective Number Pool Block Create Event does not yet exist on the NPAC SMS. (In the original NPA-NXX-X create the SOA Origination Flag was set to TRUE but the Service Provider did not submit the Number Pool Block Create and has requested the NPAC to do it on his behalf.) - Verify that all possible cases of ‘active-like’ Subscription Versions exist for the Number Pool Block to be scheduled. - Verify that there are not any ‘pending-like, no-active’ Subscription Versions for the Number Pool Block to be scheduled. |  |  |  |  |  |  |
|  | - Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| - D. | - TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| - Row # | - NPAC or SP | - Test Step |  | - NPAC or SP |  | - Expected Result |  |  |  |
| - 1. | - NPAC | Using the NPAC OP GUI, NPAC Personnel submit a request to schedule the Number Pool Block Create for a future date. |  | NPAC |  | The NPAC SMS schedules the Number Pool Block Create Event. |  |  |  |
| - 2. | - NPAC | - NPAC Personnel perform a query for the Number Pool Block Create Event that was scheduled during this Test Case. |  | - NPAC |  | - Verify the Number Pool Block Create Event has been scheduled to run on the date and time entered in Row 1 above. |  |  |  |
| - 3. | - NPAC | - The Scheduled Date/Time of the Number Pool Block Create Event is reached. |  | - NPAC |  | - On the scheduled date specified in the Number Pool Block Create Event, the NPAC SMS issues an M-ACTION Request numberPoolBlock-Create to itself. - The NPAC SMS verifies the following information: - The serviceProvNPA-NXX-X object exists for the NPA-NXX-X (respective NPA-NXX-X information). - All attributes specified are valid. - A numberPoolBlockNPAC object does not already exist for the NPA-NXX-X (a duplicate Number Pool Block does not already exist). - The current date is greater than or equal to the NPA-NXX-X-EffectiveTimeStamp. - There are not any ‘pending-like, no-active’ Subscription Version objects within the given TN range. |  |  |  |
| - 4. | - NPAC | - The NPAC SMS issues an M-CREATE Request numberPoolBlockNPAC to itself. - The NPAC SMS sets the numberPoolBlockSOA-Origination Indicator to FALSE. - The NPAC SMS sets the numberPoolBlockStatus to ‘sending’. - The NPAC SMS sets the following timestamps to the current date and time: - numberPoolBlockCreationTimeStamp - numberPoolBlockActivationTimeStamp - numberPoolBlockBroadcastTimeStamp - numberPoolBlockModifiedTimeStamp are set to the current date and time. |  | NPAC |  | - The NPAC SMS issues an M-CREATE Response numberPoolBlockNPAC to itself. |  |  |  |
| - 5. | - NPAC | - For each non-ported TN within the 1K Block, the NPAC SMS issues an M-CREATE Request subscriptionVersionNPAC to itself. - The NPAC SMS sets the LNP Type to ‘POOL’ for the Subscription Versions it creates within the 1K Block. - The NPAC SMS sets the Subscription Version to ‘sending’. - The NPAC SMS sets the following timestamps to the current date and time for the Subscription Versions: - subscriptionModifiedTimeStamp - subscriptionActivationTimeStamp - subscriptionBroadcastTimeStamp - subscriptionCreationTimeStamp |  | NPAC |  | - The NPAC SMS issues an M-CREATE Response subscriptionVersionNPAC to itself. |  |  |  |
| - 6. | - NPAC | The NPAC SMS issues an M-ACTION Response numberPoolBlock-Create to itself. |  |  |  |  |  |  |  |
| - 7. | - NPAC | - The NPAC SMS issues an M-ACTION Request subscriptionVersionLocalSMS-Create to the non-EDR LSMSs in the region that are accepting downloads for this NPA-NXX. - At the same time as step 7.1, the NPAC SMS issues an M-CREATE Request numberPoolBlock to the EDR LSMSs in the region that are accepting downloads for this NPA-NXX. |  | SP |  | - The non-EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-ACTION Request, verify that the action is valid and return an M-ACTION Response subscriptionVersionLocalSMS-Create. - The EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-CREATE Request, and return an M-CREATE Response numberPoolBlock. - The non-EDR LSMSs proceed to execute all the creates specified by the M-ACTION Request and issues an M-EVENT-REPORT subscriptionVersionLocalSMS-CreateResults specifying the Subscription Versions were successfully created. - The NPAC SMS waits for all subscriptionVersionLocalSMS-CreateResults notification for a tunable amount of time. |  |  |  |
| - 8. | - NPAC | Upon the first successful response from an LSMS, the NPAC SMS sets the following timestamps to the current date and time: - numberPoolBlockActivationCompleteTimeStamp - subscriptionActivationCompleteTimeStamp - numberPoolBlockModifiedTimeStamp - subscriptionModifiedTimeStamp |  | NPAC |  | The NPAC SMS responds to each of the M-EVENT-REPORT subscriptionVersionLocalSMS-CreateResults as it receives these notifications with M-EVENT-REPORT Confirmations. |  |  |  |
| - 9. | - NPAC | - The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself and updates the following attributes for each Pooled Subscription Version within the 1K Block: - sets the subscriptionVersionStatus to ‘active’. - sets the Subscription Version Failed SP List to empty. - sets the subscriptionModifiedTimeStamp to the current date and time. - The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself and updates the following attributes: - sets the numberPoolBlockStatus to 'active' - sets the Number Pool Block Failed SP List to empty. - sets the numberPoolBlockModifiedTimeStamp to the current date and time. |  | NPAC |  | - The NPAC SMS issues an M-SET subscriptionVersionNPAC Response to itself. - The NPAC SMS issues an M-SET numberPoolBlockNPAC Response to itself. |  |  |  |
| - 10. | - NPAC | - The NPAC SMS determines the SOA Origination Indicator is set to FALSE and terminates processing here. |  |  |  |  |  |  |  |
| - 11. | - NPAC | - NPAC Personnel perform a query for the Number Pool Block, the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that were created during this Test Case, and the ‘active-like’ Subscription Versions that do not have LNP Type set to ‘POOL’ but are within the 1K Block. |  | - NPAC |  | - Verify the Number Pool Block exists with a status of ‘active’ and an empty Failed SP List. - Verify the 1K Block of Subscription Versions exists with LNP Type set to ‘POOL’, an ‘active’ status and an empty Failed SP List. - Verify that the ‘active-like’ Subscription Versions do not have LNP Type set to ‘POOL’ and were not modified when the Number Pool Block was created during this Test Case. |  |  |  |
| - 12. | - SP – Optional | - Service Provider Personnel perform a local query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that were created during this Test Case. |  | - SP |  | - Verify the Number Pool Block exists on the EDR LSMS. - Verify the 1K Block of Subscription Versions exists on the non-EDR LSMS. |  |  |  |
| - 13. | - SP – Conditional | - Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that were created during this Test Case. |  | - SP |  | - Verify the Number Pool Block exists with a status of ‘active’ and an empty Failed SP List on the NPAC SMS. - Verify the 1K Block of Subscription Versions exists with LNP Type set to ‘POOL’, an ‘active’ status and an empty Failed SP List on the NPAC SMS. |  |  |  |
| - 14. | - NPAC | - NPAC Personnel perform a full audit for the Number Pool Block and respective POOLed Subscription Versions that were created during this test case. Include the ‘contaminated’ Subscription Versions respective to the Number Pool Block. |  | - NPAC |  | Using the Audit Results Log verify that there were no updates issued as a result of performing the audit. If updates were made, the LSMS fails this test case. |  |  |  |

| - A. | - TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: |  | - 4.1.3 |  | - SUT Priority: |  | - SOA LTI |  | - N/A |
|  |  |  |  |  |  |  | - SOA |  | - C |
|  |  |  |  |  |  |  | - non-EDR LSMS |  | - O |
|  |  |  |  |  |  |  | - EDR LSMS |  | - O |
|  | - Objective: |  | - SOA - Service Provider Personnel create a Number Pool Block that already exists. - Error |  |  |  |  |  |  |
| - B. | - REFERENCES |  |  |  |  |  |  |  |  |
|  | - NANC Change Order Revision Number: |  |  |  | - Change Order Number(s): |  | - NANC 109 |  |  |
|  | - NANC FRS Version Number: |  | - 3.0.0 |  | - Relevant Requirement(s): |  | - RR3-129, RR3-131 |  |  |
|  | - NANC IIS Version Number: |  | - 3.0.0 |  | - Relevant Flow(s): |  | - 2.1 Number Pool Block Create by SOA |  |  |
| - C. | - PREREQUISITE |  |  |  |  |  |  |  |  |
|  | - Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | - Prerequisite NPAC Setup: |  |  |  |  |  |  |  |  |
|  | - Prerequisite SP Setup: |  | - Verify that the NPA-NXX-X exists for the Number Pool Block that Service Provider Personnel will create during this Test Case. - Verify that the current date is equal to or greater than the respective NPA-NXX-X Effective Date. - Verify that a Number Pool Block with a status other than ‘old’ with an empty Failed SP List already exists for the NPA-NXX-X that Service Provider Personnel will specify in their Number Pool Block Create Request and make a note of the Block ID. |  |  |  |  |  |  |
| - D. | - TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| - Row # | - NPAC or SP | - Test Step |  | - NPAC or SP |  | - Expected Result |  |  |  |
| - 1. | - SP | - Using the SOA, Service Provider Personnel, submit a M-ACTION numberPoolBlock-Create request to the NPAC SMS to create a Number Pool Block. - The request must include the following attributes: - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | NPAC |  | - The NPAC SMS receives the M-ACTION numberPoolBlock-Create request. - The NPAC SMS verifies the following information: - The requesting SOA is the NPA-NXX-X Holder SOA. - The serviceProvNPA-NXX-X object exists for the NPA-NXX-X (respective NPA-NXX-X information). - All attributes specified are valid. - A numberPoolBlockNPAC object already exists for the NPA-NXX-X (a duplicate Number Pool Block with a status of other than ‘old’ with an empty Failed SP List already exist). (This violates system requirements.) |  |  |  |
| - 3. | - NPAC | - The NPAC SMS rejects the request and issues an M-ACTION Error Response to the NPA-NXX-X Holder SOA indicating the error and further processing is terminated. |  | - SP |  | - The NPA-NXX-X Holder SOA receives the M-ACTION Error Response. |  |  |  |
| - 4. | - NPAC | - NPAC Personnel perform a query for the Number Pool Block and 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that Service Provider Personnel attempted to create during this Test Case. |  | - NPAC |  | - Verify the original Number Pool Block with the original Block ID is the only one that exists on the NPAC SMS and that it has not been modified. - Verify the original Subscription Versions with LNP Type set to ‘POOL’ are the only ones that exist on the NPAC SMS. |  |  |  |
| - 5. | - SP – Optional | - Service Provider Personnel perform a local query for the Number Pool Block and 1K Block of Subscription Versions of Subscription Versions with LNP Type set to ‘POOL’ that Service Provider Personnel attempted to create during this Test Case. |  | - SP |  | - Verify the original Number Pool Block with the original Block ID is the only one that exists on the SOA and/or EDR LSMS and that it has not been modified. - Verify the original Subscription Versions with LNP Type set to ‘POOL’ are the only ones that exist on the non-EDR LSMS. |  |  |  |
| - 6. | - SP – Conditional | - Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that Service Provider Personnel attempted to create during this Test Case. |  | - SP |  | - Verify the original Number Pool Block with the original Block ID is the only one that exists on the NPAC SMS and that it has not been modified. - Verify the original Subscription Versions with LNP Type set to ‘POOL’ are the only ones that exist on the NPAC SMS |  |  |  |

| - A. | - TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: |  | - 4.1.4 |  | - SUT Priority: |  | - SOA LTI |  | - N/A |
|  |  |  |  |  |  |  | - SOA |  | - C |
|  |  |  |  |  |  |  | - non-EDR LSMS |  | - O |
|  |  |  |  |  |  |  | - EDR LSMS |  | - O |
|  | - Objective: |  | - SOA – Service Provider Personnel create a Number Pool Block prior to the NPA-NXX-X Effective Date – Error |  |  |  |  |  |  |
| - B. | - REFERENCES |  |  |  |  |  |  |  |  |
|  | - NANC Change Order Revision Number: |  |  |  | - Change Order Number(s): |  | - NANC 109 |  |  |
|  | - NANC FRS Version Number: |  | - 3.0.0 |  | - Relevant Requirement(s): |  | - RR3-127 |  |  |
|  | - NANC IIS Version Number: |  | - 3.0.0 |  | - Relevant Flow(s): |  | - 2.2 Number Pool Block Create by NPAC SMS |  |  |
| - C. | - PREREQUISITE |  |  |  |  |  |  |  |  |
|  | - Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | - Prerequisite NPAC Setup: |  | - Verify the NPA-NXX-X exists with the SOA Origination Indicator set to TRUE for the Number Pool Block that is to be created during this Test Case. - Verify the current date is less than the NPA-NXX-X Effective Date. |  |  |  |  |  |  |
|  | - Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| - D. | - TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| - Row # | - NPAC or SP | - Test Step |  | - NPAC or SP |  | - Expected Result |  |  |  |
| - 1. | - SP | - Prior to the NPA-NXX-X Effective Date, using the SOA, Service Provider Personnel, submit a M-ACTION numberPoolBlock-Create request to the NPAC SMS to create a Number Pool Block. - The request must include the following attributes: - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | NPAC |  | - The NPAC SMS receives the M-ACTION numberPoolBlock-Create request. - The NPAC SMS verifies the following information: - The requesting SOA is the NPA-NXX-X Holder SOA. - The serviceProvNPA-NXX-X object exists for the NPA-NXX-X (respective NPA-NXX-X information). - All attributes specified are valid. - A numberPoolBlockNPAC object does not already exists for the NPA-NXX-X (a duplicate Number Pool Block does not already exist). - The scheduled date is prior to the NPA-NXX-X Effective Timestamp. (This violates system requirements.) |  |  |  |
| - 2. | - NPAC | - The NPAC SMS rejects the request and issues an M-ACTION Error Response indicating the error. Further processing is terminated. |  | - SP |  | - The NPA-NXX-X Holder SOA receives the M-ACTION Error Response. |  |  |  |
| - 3. | - NPAC | - NPAC Personnel perform a query for the Number Pool Block that Service Provider Personnel attempted to create during this Test Case. |  | - NPAC |  | - Verify the Number Pool Block was not created on the NPAC SMS. |  |  |  |
| - 4. | - SP – Optional | - Service Provider Personnel perform a local query for the Number Pool Block that Service Provider Personnel attempted to create during this Test Case. |  | - SP |  | - Verify the Number Pool Block does not exist on the SOA and/or EDR LSMS. - Verify that the 1K Block of Subscription Versions do not exist on the non-EDR LSMS. |  |  |  |
| - 5. | - SP - Conditional | - Service Provider Personnel perform an NPAC SMS query for the Number Pool Block that Service Provider Personnel attempted to create during this Test Case. |  | - SP |  | - Verify the Number Pool Block was not created on the NPAC SMS. - Verify that the 1K Block of Subscription Versions do not exist on the NPAC SMS. |  |  |  |

| - A. | - TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: |  | - 4.1.5 |  | - SUT Priority: |  | - SOA LTI |  | - N/A |
|  |  |  |  |  |  |  | - SOA |  | - C |
|  |  |  |  |  |  |  | - non-EDR LSMS |  | - O |
|  |  |  |  |  |  |  | - EDR LSMS |  | - O |
|  | - Objective: |  | - SOA - Service Provider Personnel attempt to create a Number Pool Block when ‘pending-like, no-active’ Subscription Versions exist – Error |  |  |  |  |  |  |
| - B. | - REFERENCES |  |  |  |  |  |  |  |  |
|  | - NANC Change Order Revision Number: |  |  |  | - Change Order Number(s): |  | - NANC 109 |  |  |
|  | - NANC FRS Version Number: |  | - 3.0.0 |  | - Relevant Requirement(s): |  | - RR3-148 |  |  |
|  | - NANC IIS Version Number: |  | - 3.0.0 |  | - Relevant Flow(s): |  | - 2.1 Number Pool Block Create by SOA |  |  |
| - C. | - PREREQUISITE |  |  |  |  |  |  |  |  |
|  | - Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | - Prerequisite NPAC Setup: |  | - Verify that the NPA-NXX-X for the Number Pool Block that Service Provider Personnel will attempt to create during this Test Case exists and the Effective Date has passed. - Verify that a respective Number Pool Block does not exist on the NPAC SMS. - Verify that all-possible cases of ‘pending-like, no-active’ Subscription Versions exist for the Number Pool Block to be created. |  |  |  |  |  |  |
|  | - Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| - D. | - TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| - Row # | - NPAC or SP | - Test Step |  | - NPAC or SP |  | - Expected Result |  |  |  |
| - 1. | - SP | - Using the SOA, Service Provider Personnel, submit a M-ACTION numberPoolBlock-Create request to the NPAC SMS to create a Number Pool Block. - The request must include the following attributes: - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | NPAC |  | - The NPAC SMS receives the M-ACTION numberPoolBlock-Create request. - The NPAC SMS verifies the following information: - The requesting SOA is the NPA-NXX-X Holder SOA. - The serviceProvNPA-NXX-X object exists for the NPA-NXX-X (respective NPA-NXX-X information). - All attributes specified are valid. - A numberPoolBlockNPAC object does not already exist for the NPA-NXX-X (a duplicate Number Pool Block does not already exist). - The current date is greater than or equal to the NPA-NXX-X-EffectiveTimeStamp. - Determines there are ‘pending-like, no-active’ Subscription Version objects within the given TN range. (This violates system requirements.) |  |  |  |
| - 2. | - NPAC | - The NPAC SMS issues an M-ACTION Error Response to the NPA-NXX-X Holder SOA indicating the error. Further processing is terminated. - (The Number Pool Block is not created on the NPAC SMS.) |  | - SP |  | - The NPA-NXX-X Holder SOA receives the M-ACTION Error Response. |  |  |  |
| - 3. | - NPAC | - NPAC Personnel perform a query for the Number Pool Block that Service Provider Personnel attempted to create during this Test Case. |  | - NPAC |  | - Verify the Number Pool Block was not created on the NPAC SMS |  |  |  |
| - 4. | - SP – Optional | - Service Provider Personnel perform a local query for the Number Pool Block that Service Provider Personnel attempted to create during this Test Case. |  | - SP |  | - Verify the Number Pool Block does not exist on the SOA and/or EDR LSMS. - Verify that the 1K Block of Subscription Versions do not exist on the non-EDR LSMS. |  |  |  |
| - 5. | - SP – Conditional | - Service Provider Personnel perform an NPAC SMS query for the Number Pool Block that Service Provider Personnel attempted to create during this Test Case. |  | - SP |  | - Verify the Number Pool Block was not created on the NPAC SMS. - Verify that the 1K Block of Subscription Versions do not exist on the NPAC SMS. |  |  |  |

### Modify Block Information Test Cases:

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.2.1 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | R |
|  |  |  |  |  |  |  | EDR LSMS |  | R |
|  | Objective: |  | SOA- Service Provider Personnel modify an active Number Pool Block with the SOA Origination Indicator set to FALSE (and contains Subscription Versions with LNP Types of ‘POOL’, ‘LISP’ and ‘LSPP’) for at least 4 LSMSs (2 non-EDR and 2 EDR). - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR3-119, RR3-120, RR3-121, RR3-122, RR3-128, RR3-133, RR3-157, RR3-159, RR3-160, RR3-162, RR3-163, RR3-164, RR3-165, RR3-167, RR3-168, RR5-85, RR5-86, RR5-87, RR5-103, RR5-104, RR5-105 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 2.11 Number Pool Block Modify by Block Holder SOA 2.12.1 Number Pool Block modify Successful Broadcast to Local SMS 2.12.2 Number Pool Block Modify Successful Broadcast NPAC SMS Updates |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify the Number Pool Block to be modified exists on the NPAC SMS with a status of ‘active’ and an empty Failed SP List. - Verify that at least two Service Providers are connected to the NPAC SMS and configured to receive data downloads for this NPA-NXX (one EDR and one non-EDR LSMS). - Verify that the Number Pool Block SOA-Origination Indicator is set to FALSE. - Verify that LISP and LSPP Subscription Versions exist for some TNs in the 1K Block. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | All Service Providers verify either the Number Pool Block or 1K Block of Subscription Versions with LNP Type set to ‘POOL’ to be modified exists locally. |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the SOA, Service Provider Personnel submit an M-SET Request numberPoolBlock to modify the LRN for a Number Pool Block. The following attributes may be modified: - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | NPAC |  | - The NPAC SMS receives the M-SET Request numberPoolBlock. - The NPAC SMS performs the following actions: - Updates the modified attributes in the Number Pool Block object. - Sets the numberPoolBlockStatus to 'sending'. - Updates the numberPoolBlockBroadcastTimeStamp and numberPoolBlockModifiedTimeStamp to the current date and time. |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-SET Response numberPoolBlock to the Service Provider SOA. |  | SP |  | The Service Provider SOA receives the M-SET Response numberPoolBlock. |  |  |  |
| 3. | NPAC | The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself to modify the attribute data on the corresponding subscriptionVersionNPAC object(s). |  | SP |  | - The NPAC SMS issues an M-SET Response subscriptionVersionNPAC to itself. - The NPAC SMS performs the following actions: - Updates the modified attributes in the Subscription Versions within the 1K Block with LNP Type set to ‘POOL’. - Sets the subscriptionVersionStatus to ‘sending’. - Updates the subscriptionVersionBroadcastTimeStamp and the subscriptionVersionModifiedTimeStamp to the current date and time. |  |  |  |
| 4. | NPAC | - The NPAC SMS issues an M-SET Request subscriptionVersion to update the attributes on all Subscription Versions in the 1K Block with LNP Type set to ‘POOL’ to the non-EDR LSMSs that are accepting downloads for this NPA-NXX. - At the same time as step 4.1, the NPAC SMS issues an M-SET Request numberPoolBlock to update the attributes on the Number Pool Block object to the EDR LSMSs that are accepting downloads for this NPA-NXX. |  | SP |  | - All non-EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-SET Request, verify that the action is valid and return an M-SET Response subscriptionVersion back to the NPAC SMS. - All EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-SET Request, verify that the action is valid and return an M-SET Response numberPoolBlock back to the NPAC SMS. |  |  |  |
| 5. | NPAC | Once the first successful response is received from any LSMS, the following occurs: - The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself to set the Subscription Version Status to 'active', update the Failed SP List to empty, and update the subscriptionModifiedTimeStamp to the current date and time. - The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself to set the Number Pool Block status to 'active', update the Failed SP List to empty and update the numberPoolBlockModifiedTimeStamp to the current date and time. |  | NPAC |  | - The NPAC SMS issues an M-SET Response subscriptionVersionNPAC. - The NPAC SMS issues an M-SET Response numberPoolBlockNPAC. |  |  |  |
| 6. | NPAC | The NPAC SMS determines the numberPoolBlockSOA-Origination indicator is set to FALSE, and further processing is terminated here. |  |  |  |  |  |  |  |
| 7. | NPAC | NPAC Personnel perform a query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ as well as ‘LISP’ and ‘LSPP’. |  | NPAC |  | - Verify the Number Pool Block was successfully modified and the status is set to ‘active’ with an empty Failed SP List. - Verify the Subscription Versions with LNP Type set to ‘POOL’ in the 1K Block were successfully modified and their status is set to ‘active’ with an empty Failed SP List. - Verify the Subscription Versions within the 1K Block with LNP Type set to ‘LISP’ and ‘LSPP’ have not been modified on any LSMS. - Verify the NPAC SMS generated a Number Pool Block with a unique ID, all attributes prior to modification, and the status is set to ‘old’ with an empty Failed SP List. |  |  |  |
| 8. | NPAC | NPAC Personnel verify that the ‘old’ Number Pool Block that was created as a result of the modification, did not get broadcast. |  | NPAC |  | Verify the NPAC SMS did not broadcast the ‘old’ Number Pool Block. |  |  |  |
| 9. | SP – Optional | Service Provider Personnel perform a local query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ as well as ‘LISP’ and ‘LSPP’. |  | SP |  | - Verify the Service Providers that were receiving downloads for the NPA-NXX and have an EDR LSMS received the modification for Number Pool Block and that it was modified appropriately. - Verify the Service Providers that were receiving downloads for the NPA-NXX and have a non-EDR LSMS received the modifications for the Subscription Versions with LNP Type set to ‘POOL’ in the 1K Block, and that the Subscription Versions were modified appropriately. - Verify the Service Providers that were not receiving downloads for the NPA-NXX (whether EDR or non-EDR LSMSs) did not receive the modifications and the Number Pool Block as well as the 1K Block of Subscription Version with LNP Type set to ‘POOL’, remain unchanged. - Verify the Subscription Versions within the 1K Block with LNP Type set to ‘LISP’ and ‘LSPP’ have not been modified on any LSMS. |  |  |  |
| 10. | SP - Conditional | Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and 1K Block of Subscription Versions with LNP Type set to ‘POOL’ as well as ‘LISP’ and ‘LSPP’. |  | SP |  | - Verify the Number Pool Block was successfully modified and the status is set to ‘active’ with an empty Failed SP List on the NPAC SMS. - Verify the Subscription Versions of LNP Type set to ‘POOL’ in the 1K Block were successfully modified and their status is set to ‘active’ with an empty Failed SP List on the NPAC SMS. - Verify the Subscription Versions within the 1K Block with LNP Type set to ‘LISP’ and ‘LSPP’ have not been modified on the NPAC SMS - Verify the Number Pool Block exists on the NPAC SMS with a unique ID, all attributes prior to modification, and the status is set to ‘old’ with an empty Failed SP List. |  |  |  |
| 11. | SP – Conditional | Service Provider Personnel verify that the ‘old’ Number Pool Block that was created as a result of the modification, did not get broadcast. |  | SP |  | Verify the ‘old’ Number Pool Block did not get broadcast. |  |  |  |
| 12. | NPAC | - NPAC Personnel perform a full audit for the Number Pool Block and respective POOLed Subscription Versions that were modified during this test case. - NPAC Personnel perform a full audit for the non-POOLed Subscription Versions respective to the Number Pool Block used during this test case. |  | NPAC |  | - Using the Audit Results Log verify that there were no updates issued to the Number Pool Block or respective POOLed Subscription Versions as a result of performing the audit. If updates were made, the LSMS fails this test case. - Using the Audit Results Log verify that there were no updates issues as a result of performing the audit of the non-POOLed Subscription Versions. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.2.5 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | O |
|  |  |  |  |  |  |  | EDR LSMS |  | O |
|  | Objective: |  | SOA – Service Provider Personnel modify an active Number Pool Block with the SOA Origination Indicator set to TRUE, using an LRN that does not exist on the NPAC SMS for that Service Provider. – Error |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR3-131 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 2.11 Number Pool Block Modify by Block Holder SOA |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify the Number Pool Block to be modified exists on the NPAC SMS with a status of ‘active’ and an empty Failed SP List. - Verify the LRN to be used does not exist on the NPAC SMS. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the SOA, Service Provider Personnel submit an M-SET Request numberPoolBlock to modify a Number Pool Block, specifying an LRN that does not exist on the NPAC SMS. |  | NPAC |  | - The NPAC SMS receives the M-SET Request numberPoolBlock - The NPAC SMS determines that the LRN value does not exist on the NPAC SMS. (This violates system requirements.) - The NPAC SMS does not modify the attribute on the numberPoolBlockNPAC object. |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-SET Error Response numberPoolBlockNPAC to the NPA-NXX-X Holder SOA indicating the error. |  | SP |  | The NPA-NXX-X Holder SOA receives the M-SET Error Response from the NPAC SMS. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a local query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | NPAC |  | - Verify the Number Pool Block has not been modified. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ has NOT been modified. |  |  |  |
| 4. | SP –Optional | Service Provider Personnel perform a local query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | SP |  | - Verify the Number Pool Block has not been modified. - Verify the 1K Block of Subscription Versions has NOT been modified. |  |  |  |
| 5. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | SP |  | - Verify the Number Pool Block has not been modified on the NPAC SMS. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ has NOT been modified on the NPAC SMS. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.2.6 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | R |
|  |  |  |  |  |  |  | EDR LSMS |  | R |
|  | Objective: |  | SOA – Service Provider Personnel attempt to modify a Number Pool Block for a Number Pool Block that has a status of ‘active’ with a Failed SP List. – Error |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR3-161 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 2.11 Number Pool Block Modify by Block Holder SOA |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Verify that the Number Pool Block to be modified exists on the NPAC SMS with a status of ‘active’ and a Failed SP List. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the SOA, Service Provider Personnel submit an M-SET Request numberPoolBlock to modify an ‘active’ Number Pool Block with a Failed SP List. |  | NPAC |  | - The NPAC SMS receives the M-SET Request numberPoolBlock. - The NPAC SMS determines that the Number Pool Block specified in the modify request exists on the NPAC SMS with a status of ‘active’ and a Failed SP List. (This violates system requirements.) - The NPAC SMS does not modify the attribute on the numberPoolBlockNPAC object. - |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-SET Error Response numberPoolBlockNPAC to the NPA-NXX-X Holder SOA indicating there was an error. |  | SP |  | The NPA-NXX-X Holder SOA receives the M-SET Error Response from the NPAC SMS. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a local query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | NPAC |  | - Verify the Number Pool Block has not been modified. The status is ‘active’ with the same Failed SP List. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ has not been modified. The status is ‘active’ with the same Failed SP List. |  |  |  |
| 4. | SP – Optional | Service Provider Personnel perform a local query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | SP |  | - Verify the Number Pool Block has not been modified. - Verify the 1K of Subscription Versions with LNP Type set to ‘POOL’ has not been modified. |  |  |  |
| 5. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | SP |  | - Verify the Number Pool Block has not been modified on the NPAC SMS. The status is ‘active’ with the same Failed SP List. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ has not been modified on the NPAC SMS. The status is ‘active’ with the same Failed SP List. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.2.7 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | N/A |
|  |  |  |  |  |  |  | EDR LSMS |  | N/A |
|  | Objective: |  | NPAC OP GUI – NPAC Personnel modify the SOA Origination Indicator for a Number Pool Block - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR3-154, RR3-155 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 2.18 Number Pool Block Modification of SOA-Origination Indicator |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Verify the Number Pool Block to be modified exists on the NPAC SMS with a status of ‘active’, an empty Failed SP List and the SOA Origination Indicator is set to FALSE. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | - Using the NPAC OP GUI, NPAC Personnel submit a request to modify the SOA Origination Indicator for a Number Pool Block that exists on the NPAC SMS. - The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself to change the value of the numberPoolBlockSOA-Origination to TRUE. |  | NPAC |  | The NPAC SMS receives the M-SET Request and issues an M-SET Response to itself. |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-EVENT-REPORT numberPoolBlockAttributeValueChange to the NPA-NXX-X Holder SOA for the Number Pool Block that contains the numberPoolBlockSOA-Origination Indicator set to TRUE. |  | SP |  | The NPA-NXX-X Holder SOA receives the M-EVENT-REPORT from the NPAC SMS and issues an M-EVENT-REPORT Confirmation back to the NPAC SMS. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a query for the Number Pool Block. |  | NPAC |  | Verify the Number Pool Block has the SOA Origination Indicator set to TRUE. |  |  |  |

### Delete Block Information Test Cases:

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.3.2 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | N/A |
|  |  |  |  |  |  |  | non-EDR LSMS |  | N/A |
|  |  |  |  |  |  |  | EDR LSMS |  | N/A |
|  | Objective: |  | SOA - Service Provider Personnel attempt to delete a Number Pool Block over the SOA to NPAC SMS interface - Error |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR3-170 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  |  |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Verify that an active Number Pool Block with an empty Failed SP List exists on the NPAC SMS. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | Using the SOA, Service Provider Personnel attempt to submit an M-DELETE Request numberPoolBlock for a Number Pool Block to the NPAC SMS. |  | NPAC |  | - The NPAC SMS receives the M-DELETE Request numberPoolBlock from the Service Provider SOA. - The NPAC SMS determines the request to delete the Number Pool Block is invalid. (This violates system requirements). |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-DELETE Error Response to the Service Provider SOA. |  | SP |  | The Service Provider SOA receives the M-DELETE Error Response. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a query for the Number Pool Block and 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | NPAC |  | - Verify the Number Pool Block exists on the NPAC SMS with a status of ‘active’ and an empty Failed SP List. - Verify the 1K Block of Subscription Version with LNP Type set to ‘POOL’ exist on the NPAC SMS with a status of ‘active’ and an empty Failed SP List. |  |  |  |
| 4. | SP - Optional | Service Provider Personnel perform a local query for the Number Pool Block and 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | SP |  | Verify the Number Pool Block exists with a status of ‘active’ and an empty Failed SP List. (Assuming that the Block existed on your SOA prior to attempting to delete it in this Test Case. If the Block did not exist on your SOA, then you do not need to perform Row 4.) |  |  |  |
| 5. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | SP |  | - Verify the Number Pool Block exists on the NPAC SMS with a status of ‘active’ and an empty Failed SP List. - Verify the 1K Block of Subscription Version with LNP Type set to ‘POOL’ exist on the NPAC SMS with a status of ‘active’ and an empty Failed SP List. |  |  |  |

* There is not a flow for this functionality – so this test case is based on the assumption that the Service Provider SOA would issue an M-DELETE numberPoolBlock in an attempt to delete a Number Pool Block.  Functional Requirements prohibit a Number Pool Block Delete Request (of any type) over the SOA to NPAC SMS Interface.

## Query Block Information Test Cases:

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.4.1 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | N/A |
|  |  |  |  |  |  |  | EDR LSMS |  | N/A |
|  | Objective: |  | SOA - Service Provider Personnel submit a Query Number Pool Block Request to the NPAC SMS using an NPA-NXX-X value as filter criteria. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR3-181, RR3-182 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 2.27 Number Pool Number Pool Block Query by SOA/LSMS |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Verify that more than one active Number Pool Block with an empty Failed SP List exist for a given Service Provider on the NPAC SMS. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | - Using the SOA, Service Provider Personnel submit a numberPoolBlock object query to the NPAC SMS for a Number Pool Block. Filter criteria used for the query is the NPA-NXX-X value. - The SOA issues an M-GET Request numberPoolBlock requesting a single numberPoolBlock object by NPA-NXX-X value to the NPAC SMS. |  | NPAC |  | The NPAC SMS receives the M-GET Request over the SOA to NPAC SMS interface. |  |  |  |
| 2. | NPAC | - The NPAC SMS locates the numberPoolBlock object that matched the query criteria submitted by the SOA. - The NPAC SMS issues an M-GET Response numberPoolBlock with a single M-GET reply with all the attributes associated with the numberPoolBlock. |  | SP |  | The SOA receives the M-GET response for the numberPoolBlock query results: |  |  |  |
| 3. | SP | Service Provider personnel view the Number Pool Blocks that the NPAC SMS returned and verify the following Number Pool Block attributes are provided for each Number Pool Block: - Block Id - Block Holder SPID - NPA-NXX-X - LRN - CLASS DPC - CLASS SSN - LIDB DPC - LIDB SSN - CNAM DPC - CNAM SSN - ISVM DPC - ISVM SSN - WSMSC DPC - WSMSC SSN - Creation Date - Activation Start TimeStamp - Activation Broadcast TimeStamp - Last Modified TimeStamp - Disconnect Request TimeStamp - Disconnect Broadcast TimeStamp - Disconnect Broadcast Complete TimeStamp - Old Time Stamp - Modify Request TimeStamp - Modify Broadcast TimeStamp - Modify Broadcast Complete TimeStamp - SOA Origination Indicator - Status - Download Reason |  | SP |  | All attributes are returned to the SOA. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.4.2 |  | SUT Priority: |  | SOA LTI |  | N/A |  |
|  |  |  |  |  |  |  | SOA |  | N/A |  |
|  |  |  |  |  |  |  | non-EDR LSMS |  | C |  |
|  |  |  |  |  |  |  | EDR LSMS |  | C |  |
|  | Objective: |  | LSMS - Service Provider Personnel submit a Number Pool Block query request over the LSMS to NPAC SMS Interface using the Number Pool Block ID as filter criteria. – Success |  |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR3-181, RR3-182 |  |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 2.27 Number Pool Block Query by SOA/LSMS |  |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Verify that more than one active Number Pool Block with an empty Failed SP List exist for the query criteria on the NPAC SMS. |  |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |  |
| 1. | SP | - Using the LSMS, Service Provider Personnel submit a numberPoolBlock object query to the NPAC SMS for a Number Pool Block. Filter criteria used for the query is the Number Pool Block ID. - The LSMS issues an M-GET Request numberPoolBlock requesting a single numberPoolBlock object by numberPoolBlockId to the NPAC SMS. |  | NPAC |  | - The NPAC SMS receives the M-GET Request over the LSMS to NPAC SMS interface. |  |  |  |  |
| 2. | NPAC | - The NPAC SMS locates the numberPoolBlock object that matched the query criteria submitted by the LSMS. - The NPAC SMS issues an M-GET Response numberPoolBlock with a single M-GET reply with all the attributes associated with the numberPoolBlock. |  | SP |  | The Service Provider Personnel, receives the M-GET response for the numberPoolBlock query results. |  |  |  |  |
| 3. | SP | Service Provider personnel view the Number Pool Blocks that the NPAC SMS returned and verify the following Number Pool Block attributes for each Number Pool Block: - Block Id - Block Holder SPID - NPA-NXX-X - LRN - CLASS DPC - CLASS SSN - LIDB DPC - LIDB SSN - CNAM DPC - CNAM SSN - ISVM DPC - ISVM SSN - WSMSC DPC - WSMSC SSN - Creation Date - Activation Start TimeStamp - Activation Broadcast TimeStamp - Last Modified TimeStamp - Disconnect Request TimeStamp - Disconnect Broadcast TimeStamp - Disconnect Broadcast Complete TimeStamp - Old Time Stamp - Modify Request TimeStamp - Modify Broadcast TimeStamp - Modify Broadcast Complete TimeStamp - SOA Origination Indicator - Status - Download Reason |  | SP |  | All attributes are returned to the LSMS. |  |  |  |  |

## [Section Place Holder Only]

## Subscription Version Management Test Cases:

### Query Subscription Version Test Cases:

| A. | TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Test Case Number: | 6.1.1 | SUT Priority: | SOA LTI | N/A |
|  |  |  |  | SOA | C |
|  |  |  |  | non-EDR LSMS | N/A |
|  |  |  |  | EDR LSMS | N/A |
|  | Objective: | SOA – Service Provider Personnel query the NPAC for multiple Subscription Versions with LNP Type set to ‘POOL’ – Success |  |  |  |
| B. | REFERENCES |  |  |  |  |
|  | NANC Change Order Revision Number: |  | Change Order Number(s): | NANC 109 |  |
|  | NANC FRS Version Number: | 3.0.0 | Relevant Requirement(s): | RR5-83 |  |
|  | NANC IIS Version Number: | 3.0.0 | Relevant Flow(s): | B.5.6 Subscription Version Query |  |

Test case procedures incorporated into test case 8.1.2.7.1.1 for Release 1.0.

| A. | TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Test Case Number: | 6.1.2 | SUT Priority: | SOA LTI | N/A |
|  |  |  |  | SOA | N/A |
|  |  |  |  | non-EDR LSMS | C |
|  |  |  |  | EDR LSMS | C |
|  | Objective: | LSMS – Service Provider Personnel query the NPAC for a single Subscription Version with LNP Type set to ‘POOL’ – Success |  |  |  |
| B. | REFERENCES |  |  |  |  |
|  | NANC Change Order Revision Number: |  | Change Order Number(s): | NANC 109 |  |
|  | NANC FRS Version Number: | 3.0.0 | Relevant Requirement(s): | RR5-83 |  |
|  | NANC IIS Version Number: | 3.0.0 | Relevant Flow(s): | B.5.6 Subscription Version Query |  |

Test Case procedures incorporated into test case 8.1.2.7.2.1 for Release 1.0.

## Subscription Version Create Test Cases:

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 6.2.2 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | N/A |
|  |  |  |  |  |  |  | EDR LSMS |  | N/A |
|  | Objective: |  | NPAC OP GUI - NPAC Personnel create an Intra-Service Provider Subscription Version where a previously ‘active’ Subscription Version does not exist, after the NPA-NXX-X Creation and prior to the NPA-NXX-X Effective Date - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR5-58 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | B.5.1.2 Subscription Version Create by the Initial SOA (New Service Provider) B.5.1.11 Subscription Version Create for Intra-Service Provider Port |  |  |
| D. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that the NPA-NXX-X exists for the TN to be used to create a ‘pending’ Intra-Service Provider Subscription Version. - Verify that the Effective Date for the NPA-NXX-X is a future date. - Verify that there is not a currently ‘active’ Subscription Version that exists for the TN to be used in this test case. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| E. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel submit an Intra-Service Provider Create on behalf of the Code Holder Service Provider for a TN that is within a 1K Block after the NPA-NXX-X Creation, but prior to NPA-NXX-X Effective Date. NPAC Personnel must specify the following attributes: - subscriptionTN or a valid subscriptionVersionTN-Range - subscriptionNewCurrentSP - subscriptionOldSP - subscriptionNewSP-DueDate (seconds set to zero) - subscriptionLNPType - subscriptionLRN - subscriptionCLASS-DPC - subscriptionCLASS-SSN - subscriptionLIDB-DPC - subscriptionLIDB-SSN - subscriptionCNAM-DPC - subscriptionCNAM-SSN - subscriptionISVM-DPC - subscriptionISVM-SSN - subscriptionWSMSC-DPC - if supported by the Service provider SOA - subscriptionWSMSC-SSN - if supported by the Service Provider SOA The following attributes are optional: - subscriptionEndUser LocationValue - subscriptionEndUser LocationType - subscriptionBillingID |  | NPAC |  | NPAC SMS receives the Subscription Version Create Request and performs the following validations: - Verify that each attribute specified is valid according to system requirements. - Verify that the Old/New Service Provider ID is the same as the Code Holder SPID. - Verify that the current date is prior to the NPA-NXX-X Effective Date. |  |  |  |
| 2. | NPAC | NPAC SMS issues an M-CREATE Request to itself to create the subscriptionVersionNPAC object (Subscription Version). - The Subscription Version status is set to 'pending'. - The subscriptionCreationTimeStamp, subscriptionNewSP-AuthorizationTimeStamp, subscriptionOldSP-AuthorizationTimeStamp, and subscriptionModifiedTimeStamp are set. |  | NPAC |  | NPAC SMS issues an M-CREATE Response to itself. |  |  |  |
| 3. | NPAC | NPAC SMS issues an M-EVENT-REPORT objectCreation to the Intra-Service Provider SOA including the following information: - subscriptionTN - subscriptionNewCurrentSP - subscriptionOldSP - subscriptionNewSP-DueDate (seconds set to zeros) - subscriptionVersionStatus indicating this Subscription Version has been created on the NPAC SMS. |  | SP |  | The Service Provider SOA receives the M-EVENT-REPORT objectCreation from the NPAC SMS. |  |  |  |
| 4. | SP | Service Provider SOA sends an M-EVENT-REPORT Confirmation to the NPAC SMS. |  | NPAC |  | NPAC SMS receives the M-EVENT-REPORT Confirmation from the Service Provider SOA. |  |  |  |
| 5. | NPAC | NPAC Personnel perform a query for the Subscription Version. |  | NPAC |  | NPAC Personnel verify that the Subscription Version with LNP Type set to ‘LISP’ exists on the NPAC SMS. |  |  |  |
| 6. | SP – Optional | Service Provider Personnel perform a local query for the Subscription Version. |  | SP |  | On the SOA, verify that the Subscription Version with LNP Type set to ‘LISP’ exists. |  |  |  |
| 7. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Subscription Version. |  | SP |  | Verify that the Subscription Version with LNP Type set to ‘LISP’ exists on the NPAC SMS. |  |  |  |
| 8. | SP – Optional | Service Provider Personnel using the SOA LTI perform an NPAC SMS query for the Subscription Version notification. |  | SP |  | Verify that the objectCreation notification for the create of the Subscription Version with LNP Type set to ‘LISP’ exists on the NPAC SMS. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 6.2.3 | SUT Priority: |  | SOA LTI | N/A |  |  |
|  |  |  |  |  |  | SOA | C |  |  |
|  |  |  |  |  |  | non-EDR LSMS | N/A |  |  |
|  |  |  |  |  |  | EDR LSMS | N/A |  |  |
|  | Objective: |  | SOA - Service Provider Personnel submit an Intra-Service Provider Subscription Version create request where a previously ‘active’ Subscription Version does not exist, after the NPA-NXX-X Creation and prior to the NPA-NXX-X Effective Date - Error |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  | Change Order Number(s): |  | NANC 109 |  |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 | Relevant Requirement(s): |  | RR5-59 |  |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 | Relevant Flow(s): |  | B.5.1.2 Subscription Version Create by the Initial SOA (New Service Provider) B.5.1.11 Subscription Version Create for Intra-Service Provider Port |  |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that the NPA-NXX-X exists for the TN to be used to create a ‘pending’ Intra-Service Provider Subscription Version. - Verify that the Effective Date for the NPA-NXX-X is a future date. - Verify that there is not a currently ‘active’ Subscription Version that exists for the TN to be used in this test case. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP | Expected Result |  |  |  |  |
| 1. | SP | - Using the SOA, the Code Holder Service Provider submit an Intra-Service Provider, Subscription Version create request for a TN within a1K Block after NPA-NXX-X Creation, but prior to the NPA-NXX-X Effective Date. - The SOA system sends an M-ACTION Request subscriptionVersionNewSP-Create to the NPAC SMS to create the subscriptionVersionNPAC (Subscription Version) on the NPAC SMS. - The following attributes must be provided: - subscriptionTN - subscriptionNewCurrentSP - subscriptionOldSP - subscriptionNewSP-DueDate - subscriptionLNPType - subscriptionPortingToOriginal-SP Switch - subscriptionLRN - subscriptionCLASS-DPC - subscriptionCLASS-SSN - subscriptionLIDB-DPC - subscriptionLIDB-SSC - subscriptionCNAM-DPC - subscriptionCNAM-SSN - subscriptionISVM-DPC - subscriptionISVM-SSN - subscriptionWSMSC-DPC– if supported by the Service Provider SOA - subscriptionWSMSC-SSN– if supported by the Service Provider SOA - subscriptionEndUserLocationValue - subscriptionEndUserLocationType - subscriptionBillingID |  | NPAC | The NPAC SMS receives the M-ACTION Request from the Code Holder SOA and determines the following: - This TN is part of a 1K Block. - The NPA-NXX-X object has been created - however, it is prior to the Effective Date. - There is not a currently ‘active’ Subscription Version for this TN. (This violates system requirements.) |  |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-ACTION Response failure indicating an error with the request to the SOA. |  | SP | The Service Provider SOA receives the M-ACTION Response. |  |  |  |  |
| 3. | NPAC | NPAC Personnel perform a query for the Subscription Version. |  | NPAC | NPAC Personnel verify that the Subscription Version does not exist on the NPAC SMS. |  |  |  |  |
| 4. | SP – Optional | Service Provider Personnel, perform a local query for the Subscription Version. |  | SP | On the SOA, verify that the Subscription Version does not exist. |  |  |  |  |
| 5. | SP – Conditional | Service Provider Personnel, perform an NPAC SMS query for the Subscription Version. |  | SP | Verify that the Subscription Version does not exist on the NPAC SMS. |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 6.2.4 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | N/A |
|  |  |  |  |  |  |  | EDR LSMS |  | N/A |
|  | Objective: |  | SOA - Service Provider Personnel submit an Inter-Service Provider, Port-to-Original Create request for the Code Holder after the NPA-NXX-X Creation and prior to NPA-NXX-X Effective Date – Error |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR5-56 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 3.7 Subscription Version Port-To-Original of a Pool TN-Creation Prior to NPA-NXX-X Effective Date |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that the NPA-NXX-X exists respective to the TN that Service Provider Personnel are going to attempt to create a ‘pending’, PTO Subscription Version. - Verify that there is a currently ‘active’ Subscription Version that exists for the TN to be used in this test case. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | - Using the SOA, Service Provider Personnel submit an Inter-Service Provider, Port-to-Original Subscription Version Create Request to the NPAC SMS, (for a TN that is part of a 1K Block) after NPA-NXX-X Creation, and prior to the NPA-NXX-X Effective Date. Service Provider Personnel must specify the following attributes: · subscriptionTN · subscriptionNewCurrentSP · subscriptionOldSP · subscriptionOldSP-DueDate · subscriptionOldSP- Authorization · subscriptionPort-To-Original indicator · subscriptionLNPType - The SOA issues an M-ACTION subscriptionVersionNewSP-Create to the NPAC SMS, specifying all required attributes. |  | NPAC |  | The NPAC SMS receives the M-ACTION from the Service Provider SOA with the Port-to-Original flag set to ‘TRUE’. The NPAC SMS determines that the TN specified is part of a 1K Block that has not yet been activated (the NPA-NXX-X exists, but the ‘active’ Block does not yet exist). – (This violates system requirements.) |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-ACTION Response back to the Service Provider specifying, ‘soa not authorized’. |  | SP |  | The Service Provider SOA receives the M-ACTION Response. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a query for the Subscription Version. |  | NPAC |  | NPAC Personnel verify that the Subscription Version does not exist on the NPAC SMS. |  |  |  |
| 4. | SP – Optional | Service Provider Personnel perform a local query for the Subscription Version. |  | SP |  | On the SOA, verify that the Subscription Version does not exist. |  |  |  |
| 5. | SP – Conditional | Service Provider Personnel, perform an NPAC SMS query for the Subscription Version. |  | SP |  | Verify that the Subscription Version does not exist on the NPAC SMS. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 6.2.5 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | N/A |
|  |  |  |  |  |  |  | EDR LSMS |  | N/A |
|  | Objective: |  | NPAC OP GUI - NPAC Personnel create a range of Intra-Service Provider Subscription Versions both within and outside of the 1K Block, where previously ‘active’ SVs do not exist for the Code Holder after the NPA-NXX-X Creation and prior to the NPA-NXX-X Effective Date - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR5-58 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | B.5.1.2 Subscription Version Create by the Initial SOA (New Service Provider) B.5.1.11 Subscription Version Create for Intra-Service Provider Port |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that the NPA-NXX-X exists for some of the TNs to be used to create a ‘pending’ Intra-Service Provider Subscription Version. - Verify that the Effective Date for the NPA-NXX-X is a future date. - Verify that there are not currently ‘active’ Subscription Versions that exists for all of the TNs to be used in this test case. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel submit an Intra-Service Provider Create on behalf of the Code Holder Service Provider for a range of TNs that are both within a 1K Block and outside of the 1K Block, after the NPA-NXX-X Creation, but prior to NPA-NXX-X Effective Date. NPAC Personnel must specify the following attributes: - valid subscriptionVersionTN-Range - subscriptionNewCurrentSP - subscriptionOldSP - subscriptionNewSP-DueDate (seconds set to zeros) - subscriptionLNPType - subscriptionLRN - subscriptionCLASS-DPC - subscriptionCLASS-SSN - subscriptionLIDB-DPC - subscriptionLIDB-SSN - subscriptionCNAM-DPC - subscriptionCNAM-SSN - subscriptionISVM-DPC - subscriptionISVM-SSN - subscriptionLNPType - subscriptionWSMSC-DPC - if supported by the Service Provider SOA - subscriptionWSMSC-SSN - if supported by the Service Provider SOA The following attributes are optional: - subscriptionEndUserLocationValue - subscriptionEndUserLocationType - subscriptionBillingID |  | NPAC |  | NPAC SMS receives the Subscription Version Create Request and performs the following validations: - Verify that each attribute specified is valid according to system requirements. - Verify that the Old/New Service Provider ID is the same as the Code Holder SPID. - Verify that the current date is prior to the NPA-NXX-X Effective Date. |  |  |  |
| 2. | NPAC | NPAC SMS issues an M-CREATE Request to itself to create the subscriptionVersionNPAC object (Subscription Version). - The Subscription Version status is set to 'pending'. - The subscriptionCreationTimeStamp, subscriptionNewSP-AuthorizationTimeStamp, subscriptionOldSP-AuthorizationTimeStamp, and subscriptionModifiedTimeStamp are set. |  | NPAC |  | NPAC SMS issues an M-CREATE Response to itself. |  |  |  |
| 3. | NPAC | NPAC SMS issues an M-EVENT-REPORT objectCreation to the Intra-Service Provider SOA including the following information: - valid subscriptionVersionTN-Range - subscriptionNewCurrentSP - subscriptionOldSP - subscriptionNewSP-DueDate (seconds set to zeros) - subscriptionVersionStatus indicating this Subscription Version has been created on the NPAC SMS. |  | SP |  | The Service Provider SOA receives the M-EVENT-REPORT objectCreation from the NPAC SMS. |  |  |  |
| 4. | SP | Service Provider SOA sends an M-EVENT-REPORT Confirmation to the NPAC SMS. |  | NPAC |  | NPAC SMS receives the M-EVENT-REPORT Confirmation from the Service Provider SOA. |  |  |  |
| 5. | NPAC | NPAC Personnel perform a query for the Subscription Versions. |  | NPAC |  | - NPAC Personnel verify that the Subscription Versions with LNP Type set to ‘LISP’ exist on the NPAC SMS. - The Subscription Versions created are both within and outside the TN range of the NPA-NXX-X. |  |  |  |
| 6. | SP – Optional | Service Provider Personnel perform a local query for the Subscription Versions. |  | SP |  | On the SOA, verify that the Subscription Versions with LNP Type set to ‘LISP’ both within and outside the TN range of the NPA-NXX-X exist. |  |  |  |
| 7. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Subscription Versions. |  | SP |  | Verify that the Subscription Versions with LNP Type set to ‘LISP’ both within and outside the TN range of the NPA-NXX-X exist on the NPAC SMS. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 6.2.7 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | N/A |
|  |  |  |  |  |  |  | EDR LSMS |  | N/A |
|  | Objective: |  | SOA - Service Provider Personnel submit an Inter-Service Provider, Port-to-Original Create request for the Code Holder after the NPA-NXX-X Effective Date and prior to the Block existence – Error |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR5-56 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 3.7 Subscription Version Port-To-Original of a Pool TN-Creation Prior to NPA-NXX-X Effective Date |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that the NPA-NXX-X exists with SOA Origination Flag set to TRUE respective to the TN that Service Provider Personnel are going to attempt to create a ‘pending’, PTO Subscription Version. - Verify that there is a currently ‘active’ Subscription Version that exists for the TN to be used in this test case. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | - Service Provider Personnel, using the SOA system as the Code Holder, submit an Inter-Service Provider, Port-to-Original Subscription Version Create Request to the NPAC SMS, (for a TN that is part of a 1K Block) after NPA-NXX-X Effective Date, and prior to the Block existence. Service Provider Personnel must specify the following attributes: · subscriptionTN · subscriptionNewCurrentSP · subscriptionOldSP · subscriptionOldSP-DueDate · subscriptionOldSP- Authorization · subscriptionLNPType - The SOA issues an M-ACTION subscriptionVersionNewSP-Create to the NPAC SMS, specifying all required attributes. |  | NPAC |  | The NPAC SMS receives the M-ACTION from the Service Provider SOA with the Port-to-Original flag set to ‘TRUE’. The NPAC SMS determines that the TN specified is part of a 1K Block that has not yet been activated (the NPA-NXX-X exists, but the ‘active’ Block does not yet exist). – (This violates system requirements). |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-ACTION Response back to the Service Provider specifying, ‘soa not authorized’. |  | SP |  | The Service Provider SOA receives the M-ACTION Response. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a query for the Subscription Version. |  | NPAC |  | NPAC Personnel verify that the Subscription Version does not exist on the NPAC SMS. |  |  |  |
| 4. | SP – Optional | Service Provider Personnel perform a local query for the Subscription Version. |  | SP |  | On the SOA, verify that the Subscription Version does not exist. |  |  |  |
| 5. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Subscription Version. |  | SP |  | Verify that the Subscription Version does not exist on the NPAC SMS. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 6.2.8 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | N/A |
|  |  |  |  |  |  |  | EDR LSMS |  | N/A |
|  | Objective: |  | SOA - Service Provider Personnel submit an Intra-Service Provider Create request after NPA-NXX-X Effective Date and Block Activation - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR5-55 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | B.5.1.2 Subscription Version Create by the Initial SOA (New Service Provider). |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  |  |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | - The Service Provider is the Block Holder. - Verify that the TN has a currently ‘active’ Subscription Version associated with it where the LNP Type is set to ‘POOL’. |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | - Using the SOA, Block Holder Service Provider Personnel submit a request to Create a ‘pending’, Intra-Service Provider, Subscription Version specifying a TN that is part of an ‘active’ 1K Block. - The New Service Provider SOA sends an M-ACTION subscriptionVersionNewSP-Create to the NPAC SMS lnpSubscription object to create a new subscriptionVersionNPAC. The New Service Provider must specify the following attributes: · subscriptionTN or a valid subscriptionVersionTN-Range · subscriptionNewCurrentSP · subscriptionOldSP · subscriptionNewSP-DueDate (seconds set to zero) · subscriptionLNPType · subscriptionLRN · subscriptionCLASS-DPC · subscriptionCLASS-SSN · subscriptionLIDB-DPC · subscriptionLIDB-SSN · subscriptionCNAM-DPC · subscriptionCNAM-SSN · subscriptionISVM-DPC · subscriptionISVM-SSN · subscriptionWSMSC-DPC - if supported by the Service provider SOA · subscriptionWSMSC-SSN - if supported by the Service Provider SOA The following attributes are optional: · subscriptionEndUser LocationValue · subscriptionEndUser LocationType · subscriptionBillingID |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the Service Provider SOA and determines the request is valid. |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-CREATE subscriptionVersionNPAC to itself to create the Subscription Version and set the status to ‘pending’, as well as the subscriptionModifiedTimeStamp and subscriptionCreationTimeStamp to the current date and time. |  | NPAC |  | The NPAC SMS issues an M-CREATE Response to itself. |  |  |  |
| 3. | NPAC | The NPAC SMS issues a successful M-ACTION Response to the originating SOA. |  | SP |  | The Originating SOA receives the M-ACTION Response from the NPAC SMS. |  |  |  |
| 4. | NPAC | NPAC SMS issues an M-EVENT-REPORT objectCreation to the Intra-Service Provider SOA including the following information: - subscriptionTN - subscriptionNewCurrentSP - subscriptionOldSP - subscriptionNewSP-DueDate (seconds set to zeros) - subscriptionVersionStatus indicating this Subscription Version has been created on the NPAC SMS. |  | SP |  | The Service Provider SOA receives the M-EVENT-REPORT objectCreation from the NPAC SMS. |  |  |  |
| 5. | NPAC | NPAC Personnel perform a query for the Subscription Version. |  | NPAC |  | NPAC Personnel verify that the Subscription Version with LNP Type set to ‘LISP’ exists on the NPAC SMS. |  |  |  |
| 6. | SP – Optional | Service Provider Personnel perform a local query for the Subscription Version. |  | SP |  | On the SOA, verify that the Subscription Version with LNP Type set to ‘LISP’ exists. |  |  |  |
| 7. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Subscription Version. |  | SP |  | Verify that the Subscription Version with LNP Type set to ‘LISP’ exists on the NPAC SMS. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 6.2.9 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | N/A |
|  |  |  |  |  |  |  | EDR LSMS |  | N/A |
|  | Objective: |  | SOA - Service Provider Personnel submit an Inter-Service Provider, Port-to-Original Create request for the Code Holder after the Block existence - Error |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR5-57 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | B.5.1.2 Subscription Version Create by the Initial SOA (New Service Provider) |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that the NPA-NXX-X and the 1K Block exist respective to the TN that Service Provider Personnel are going to attempt to create a ‘pending’, PTO Subscription Version. - Verify that there is a currently ‘active’ Subscription Version with LNP Type is set to ‘LSPP’, which exists for the TN to be used in this test case. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | - Using the SOA, the Code Holder Service Provider Personnel submit an Inter-Service Provider, Port-to-Original Subscription Version Create Request to the NPAC SMS, (for a TN that is part of a 1K Block) after the Block existence. - Service Provider Personnel must specify the following attributes: - subscriptionTN - subscriptionNewCurrentSP - subscriptionOldSP - subscriptionOldSP-DueDate - subscriptionOldSP-Authorization - subscriptionLNPType - The SOA issues an M-ACTION subscriptionVersionNewSP-Create to the NPAC SMS, specifying all required attributes. |  | NPAC |  | The NPAC SMS receives the M-ACTION from the Service Provider SOA with the Port-to-Original flag set to ‘TRUE’. The NPAC SMS determines that the TN specified is part of a 1K Block that is no longer owned by the Code Holder. – (This violates system requirements). |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-ACTION Response back to the Service Provider specifying, ‘soa not authorized’. |  | SP |  | The Service Provider SOA receives the M-ACTION Response. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a query for the Subscription Version. |  | NPAC |  | NPAC Personnel verify that the Subscription Version does not exist on the NPAC SMS. |  |  |  |
| 4. | SP – Optional | Service Provider Personnel perform a local query for the Subscription Version. |  | SP |  | On the SOA, verify that the Subscription Version does not exist. |  |  |  |
| 5. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Subscription Version. |  | SP |  | Verify that the Subscription Version does not exist on the NPAC SMS. |  |  |  |

## Subscription Version Modify Test Cases:

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 6.3.1 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | O |
|  |  |  |  |  |  |  | EDR LSMS |  | O |
|  | Objective: |  | SOA - Service Provider Personnel submit a request to modify a Subscription Version with LNP Type set to ‘POOL’ – Error |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR5-84 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | B.5.2.1 Subscription Version Modify Active Version Using M-ACTION by a Service Provider SOA |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Verify that an ‘active’ Subscription Version exist with LNP Type set to ‘POOL’. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | - Using the SOA, the Block Holder Service Provider Personnel submit a request to the NPAC SMS to modify an ‘active’ Subscription Version of LNP Type set to ‘POOL’. The request must specify the TN and the version status or the version ID of the Subscription Version to be modified and the data to be modified. - The following attributes must be specified: · subscriptionLRN · subscriptionCLASS-DPC · subscriptionCLASS-SSN · subscriptionLIDB-DPC · subscriptionLIDB-SSC · subscriptionCNAM-DPC · subscriptionCNAM-SSN · subscriptionISVM-DPC · subscriptionISVM-SSN · subscriptionWSMSC-DPC – if supported by the Service Provider SOA · subscriptionWSMSC-SSN – if supported by the Service Provider SOA - The Service Provider SOA submits an M-ACTION Request subscriptionVersionModify to the NPAC SMS lnpSubscription object to update the ‘active’ version. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request and determines that the specified Subscription Version for modification is of LNP Type set to ‘POOL’. (This violates system requirements.) |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-ACTION Failure Response back to the Current Service Provider SOA indicating a request error. |  | SP |  | The Current Service Provider SOA receives the M-ACTION Failure Response from the NPAC SMS. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a query for the Subscription Version. |  | NPAC |  | NPAC Personnel verify that the Subscription Version was not modified on the NPAC SMS. |  |  |  |
| 4. | SP – Optional | Service Provider Personnel perform a local query for the Subscription Version. |  | SP |  | - From the SOA, verify that the Subscription Version was not modified on the NPAC SMS. - From either the non-EDR or EDR LSMS, verify that the Subscription Version was not modified on the NPAC SMS. |  |  |  |
| 5. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Subscription Version. |  | SP |  | - From the SOA, verify that the Subscription Version was not modified on the NPAC SMS. - From either the non-EDR or EDR LSMS, verify that the Subscription Version was not modified on the NPAC SMS. |  |  |  |

## Subscription Version Delete Test Cases:

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 6.4.1 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | O |
|  |  |  |  |  |  |  | EDR LSMS |  | O |
|  | Objective: |  | SOA - Service Provider Personnel attempt to delete (submit a disconnect request) a Subscription Version with LNP Type set to ‘POOL’ - Error |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR5-84 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | B.5.4.1 Subscription Version Immediate Disconnect |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Verify that an ‘active’ Subscription Version of LNP Type set to ‘POOL’ exists, Service Provider Personnel should attempt to delete this Subscription Version. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | - Using the SOA, Block Holder Service Provider Personnel submit an Immediate Disconnect Request to the NPAC SMS for a Subscription Versions of LNP Type set to ‘POOL’. The request must specify the Subscription Version ID, or Subscription Version TN and also has future dated the subscriptionEffectiveReleaseDate and the subscriptionCustomerDisconnectDate. - The Current Service Provider SOA system issues an M-ACTION Request subscriptionVersionDisconnect to the NPAC SMS. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the Current Service Provider SOA and determines this request is for a Subscription Version of LNP Type set to ‘POOL’. (This violates system requirements.) |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-ACTION Failure Response to the Current Service Provider SOA indicating a request error. |  | SP |  | The Block Holder Service Provider SOA receives the M-ACTION Failure Response from the NPAC SMS. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a query for the Subscription Version. |  | NPAC |  | NPAC Personnel verify that the Subscription Version was not deleted on the NPAC SMS. |  |  |  |
| 4. | SP – Optional | Service Provider Personnel perform a local query for the Subscription Version. |  | SP |  | - On the Block Holder SOA, verify that the Subscription Version was not deleted. - On the non-EDR LSMS, verify that the Subscription Version was not deleted. - On the EDR LSMS, verify that the Subscription Version is part exists as part of the 1K Block. |  |  |  |
| 5. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Subscription Version. |  | SP |  | - From the Block Holder SOA, verify that the Subscription Version with LNP Type set to ‘POOL’ exists on the NPAC SMS. - From the non-EDR LSMS, verify that the Subscription Version with LNP Type set to ‘POOL’ exists on the NPAC SMS. - From the EDR LSMS, verify that the Subscription Version is part exists as part of the 1K Block, with LNP Type set to ‘POOL’ on the NPAC SMS. |  |  |  |

## Subscription Version Disconnect Test Cases:

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 6.5.4 |  |  | SUT Priority: | SOA LTI |  | N/A |  |
|  |  |  |  |  |  |  | SOA |  | O |  |
|  |  |  |  |  |  |  | non-EDR LSMS |  | R |  |
|  |  |  |  |  |  |  | EDR LSMS |  | R |  |
|  | Objective: |  | NPAC OP GUI - NPAC Personnel resend a ‘failed’ disconnect request – Success |  |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  |  | Change Order Number(s): | NANC 109 |  |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  |  | Relevant Requirement(s): | RR5-80, RR5-81.1, RR5-81.2 |  |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  |  | Relevant Flow(s): | 4.5 Subscription Version Disconnect of a Ported Pooled TN: Resend Successful to Local SMS 4.5.1 Subscription Version Disconnect of a Ported Pooled TN NPAC SMS Broadcast Successful Resend 4.5.2 Subscription Version Disconnect of a Ported Pooled TN Resend Successful NPAC SMS Updates |  |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that a failed Disconnect request for a ported pooled TN exists. - Verify that the systems that caused the failure before, are configured/connected to the NPAC SMS in order to successfully process the resend request. |  |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP | Expected Result |  |  |  |  |  |
| 1. | NPAC | - Using the NPAC OP GUI, NPAC Personnel resend a failed disconnect for a ported, pooled Subscription Version. - The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself to set the Subscription Version status for SV1 to ‘sending’ and update the subscriptionModifiedTimeStamp to the current date and time. |  | NPAC | The NPAC SMS issues an M-SET Response back to itself. |  |  |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself to set the Subscription Version status for SV2 to ‘sending’ and update the subscriptionModifiedTimeStamp to the current date and time. |  | NPAC | The NPAC SMS issues an M-SET Response back to itself. |  |  |  |  |  |
| 3. | NPAC | - The NPAC SMS issues an M-DELETE Request for SV1 to all EDR LSMSs that were in the FailedSP-List (previously failed the disconnect request). - The NPAC SMS issues an M-CREATE Request for SV2 to all non-EDR LSMSs that were in the FailedSP-List (previously failed the disconnect request). |  | SP/ NPAC | - All EDR LSMSs in the region that are accepting downloads for this NPA-NXX receive the Subscription Version Delete Request (M-DELETE Request) for SV1. - All non-EDR LSMSs in the region that are accepting downloads for this NPA-NXX receive a Subscription Version Create Request (M-CREATE Request) for SV2. - The NPAC SMS waits for response from all LSMSs accepting downloads for this NPA-NXX. - The NPAC SMS retries any LSMS (M-DELETE SV1 to EDR LSMSs, or an M-CREATE SV2 to non-EDR LSMSs) if they have not responded within a tunable amount of time. - All of the LSMSs in the region respond with a successful message (all LSMSs have failed the requests). |  |  |  |  |  |
| 4. | NPAC | The NPAC SMS issues an M-SET Request to itself to update the status of SV2 to ‘active’ and set the subscriptionModifiedTimeStamp to the current date and time. |  | NPAC | The NPAC SMS issues an M-SET Response back to itself. |  |  |  |  |  |
| 5. | NPAC | The NPAC SMS issues an M-SET Request to itself to update the status of SV1 to ‘old’ and set the failedSP-List to be empty, as well as set the subscriptionModifiedTimeStamp to the current date and time. |  | NPAC | The NPAC SMS issues an M-SET Response back to itself. |  |  |  |  |  |
| 6. | NPAC | The NPAC SMS issues an M-EVENT-REPORT subscriptionVersionStatusAttributeValueChange to the Current Service Provider SOA to set the status of SV1 to ‘old’ with an empty FailedSP-List. |  | SP | The Current Service Provider SOA receives the M-EVENT-REPORT from the NPAC SMS and issues an M-EVENT-REPORT Confirmation back to the NPAC SMS. |  |  |  |  |  |
| 7. | NPAC | NPAC Personnel perform a query for the Subscription Version. |  | NPAC | NPAC Personnel verify that an ‘active’ Subscription Version with LNP Type set to ‘POOL’ exists on the NPAC SMS. |  |  |  |  |  |
| 8. | SP – Optional | Service Provider Personnel perform a local query for the Subscription Version. |  | SP | - On the Block Holder SOA, verify that a Subscription Version with LNP Type ‘POOL’ exists with an empty Failed SP List. - On the non-EDR LSMS, verify that a Subscription Version with LNP Type ‘POOL’ exists. - On the EDR LSMS, verify that the Subscription Version exists as part of the 1K Block. |  |  |  |  |  |
| 9. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Subscription Version. |  | SP | - From the Block Holder SOA, verify that a Subscription Version with LNP Type ‘POOL’ exists with an empty Failed SP List on the NPAC SMS. - From the non-EDR LSMS, verify that a Subscription Version with LNP Type ‘POOL’ exists on the NPAC SMS - From the EDR LSMS, verify that the Subscription Version exists as part of the 1K Block on the NPAC SMS. |  |  |  |  |  |
| 10. | NPAC | NPAC Personnel perform a full audit for the Subscription Version resent during this test case. |  | NPAC | Using the Audit Results Log verify that there were no updates issues as a result of performing this audit. If updates were made, the LSMS fails this test case. |  |  |  |  |  |

## NPA Splits with Number Pooling

| A. | TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Test Case Number: | 7.1 | SUT Priority: | SOA LTI | - N/A |
|  |  |  |  | SOA | - C |
|  |  |  |  | non-EDR LSMS | - C |
|  |  |  |  | EDR LSMS | - C |
|  | Objective: | NPAC OP GUI - NPAC Personnel schedule a future-dated NPA Split specifying the Old NPA-NXX as one that is part of an ‘active’ Number Pool Block - Success |  |  |  |
| B. | REFERENCES |  |  |  |  |
|  | NANC Change Order Revision Number: |  | Change Order Number(s): | NANC 109, NANC 244 |  |
|  | NANC FRS Version Number: | 3.0.0 | Relevant Requirement(s): | RR3-31, RR3-33, RR3-34, RR3-39, RR3-40, RR3-41, RR3-51.1, RR3-51.2, RR3-219 |  |
|  | NANC IIS Version Number: | 3.0.0 | Relevant Flow(s): | 7 – NPA Split |  |

Test case procedures incorporated into test case 192-1 from Release 3.2.

| - A. | - TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: | - 7.3 | - SUT Priority: | - SOA LTI | - N/A |
|  |  |  |  | - SOA | - C |
|  |  |  |  | - non-EDR LSMS | - C |
|  |  |  |  | - EDR LSMS | - C |
|  | - Objective: | - NPAC OP GUI – NPAC Personnel remove an NPA-NXX from an NPA Split prior to the Permissive Dial Period (PDP) Start Date – Success |  |  |  |
| - B. | - REFERENCES |  |  |  |  |
|  | - NANC Change Order Revision Number: |  | Change Order Number(s): | - NANC 109 |  |
|  | - NANC FRS Version Number: | - 3.0.0 | - Relevant Requirement(s): | - RR3-35, RR3-39 |  |
|  | - NANC IIS Version Number: | - 3.0.0 | - Relevant Flow(s): | - 7 – NPA-NXX Split |  |

Test case procedures incorporated into test case 192-1 from Release 3.2.

| - A. | - TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: | - 7.4 | - SUT Priority: | - SOA LTI | - N/A |
|  |  |  |  | - SOA | - C |
|  |  |  |  | - non-EDR LSMS | - C |
|  |  |  |  | - EDR LSMS | - C |
|  | - Objective: | - NPAC OP GUI - NPAC Personnel remove an NPA-NXX from an NPA Split during the Permissive Dial Period (PDP), which has a respective ‘active’ Number Pool Block - Success |  |  |  |
| - B. | - REFERENCES |  |  |  |  |
|  | - NANC Change Order Revision Number: |  | Change Order Number(s): | - NANC 109 |  |
|  | - NANC FRS Version Number: | - 3.0.0 | - Relevant Requirement(s): | - RR3-35, RR3-39, RR3-42 |  |
|  | - NANC IIS Version Number: | - 3.0.0 | - Relevant Flow(s): |  |  |

Test case procedures incorporated into test case 192-1 from Release 3.2.

| - A. | - TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: | - 7.5 | SUT Priority: | - SOA LTI | - N/A |
|  |  |  |  | - SOA | - C |
|  |  |  |  | - non-EDR LSMS | - C |
|  |  |  |  | - EDR LSMS | - C |
|  | - Objective: | NPAC OP GUI - NPAC Personnel create an NPA-NXX-X specifying the Old NPA-NXX that is scheduled for an NPA Split, prior to the Permissive Dial Period (PDP) Start Date resulting in an auto-generated NPA-NXX-X with the Effective Date set to PDP Start Date- Success |  |  |  |
| - B. | - REFERENCES |  |  |  |  |
|  | - NANC Change Order Revision Number: |  | Change Order Number(s): | - NANC 109 |  |
|  | - NANC FRS Version Number: | - 3.0.0 | - Relevant Requirement(s): | - RR3-87, RR3-36.1 |  |
|  | - NANC IIS Version Number: | - 3.0.0 | - Relevant Flow(s): | - 1.1 Service Provider NPA-NXX-X Create by NPAC SMS |  |

Test case procedures incorporated into test case 192-1 from Release 3.2.

| - A. | - TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: | - 7.6 | SUT Priority: | - SOA LTI | - N/A |
|  |  |  |  | - SOA | - C |
|  |  |  |  | - non-EDR LSMS | - C |
|  |  |  |  | - EDR LSMS | - C |
|  | - Objective: | NPAC OP GUI - NPAC Personnel create an NPA-NXX-X specifying the Old NPA-NXX that is scheduled for an NPA Split, prior to the Permissive Dial Period (PDP) Start Date resulting in an auto-generated NPA-NXX-X with the Effective Date set to the Old NPA-NXX-X Effective Date- Success |  |  |  |
| - B. | - REFERENCES |  |  |  |  |
|  | - NANC Change Order Revision Number: |  | Change Order Number(s): | - NANC 109 |  |
|  | - NANC FRS Version Number: | - 3.0.0 | - Relevant Requirement(s): | - RR3-87, RR3-36.1 |  |
|  | - NANC IIS Version Number: | - 3.0.0 | - Relevant Flow(s): | - 1.1 Service Provider NPA-NXX-X Create by NPAC SMS |  |

Test case procedures incorporated into test case 192-1 from Release 3.2.

| - A. | - TEST IDENTITY |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: |  | - 7.8 | SUT Priority: | - SOA LTI | - N/A |  |
|  |  |  |  |  | - SOA | - C |  |
|  |  |  |  |  | - non-EDR LSMS | - C |  |
|  |  |  |  |  | - EDR LSMS | - C |  |
|  | - Objective: |  | - NPAC OP GUI – NPAC Personnel create an NPA-NXX-X specifying the Old NPA-NXX that is involved in an NPA Split, during Permissive Dial Period (PDP) - Success |  |  |  |  |
| - B. | - REFERENCES |  |  |  |  |  |  |
|  | - NANC Change Order Revision Number: |  |  | Change Order Number(s): | - NANC 109 |  |  |
|  | - NANC FRS Version Number: |  | - 3.0.0 | - Relevant Requirement(s): | - RR3-87, RR3-36.3 |  |  |
|  | - NANC IIS Version Number: |  | - 3.0.0 | - Relevant Flow(s): | - 1.1 Service Provider NPA-NXX-X Create by NPAC SMS |  |  |

Test case procedures incorporated into test case 192-1 from Release 3.2.

| - A. | - TEST IDENTITY |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: | - 7.9 | SUT Priority: | - SOA LTI | - N/A |  |
|  |  |  |  | - SOA | - C |  |
|  |  |  |  | - non-EDR LSMS | - C |  |
|  |  |  |  | - EDR LSMS | - C |  |
|  | - Objective: | - NPAC OP GUI - NPAC Personnel create an NPA-NXX-X specifying the New NPA-NXX, that is involved in an NPA Split, during Permissive Dial Period (PDP) – Success |  |  |  |  |
| - B. | - REFERENCES |  |  |  |  |  |
|  | - NANC Change Order Revision Number: |  | Change Order Number(s): | - NANC 109 |  |  |
|  | - NANC FRS Version Number: | - 3.0.0 | - Relevant Requirement(s): | - RR3-87, RR3-36.3 |  |  |
|  | - NANC IIS Version Number: | - 3.0.0 | - Relevant Flow(s): | - 1.1 Service Provider NPA-NXX-X Create by NPAC SMS |  |  |

Test case procedures incorporated into test case 192-1 from Release 3.2.

| - A. | - TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: | - 7.10 | - SUT Priority: | - SOA LTI | - N/A |
|  |  |  |  | - SOA | - C |
|  |  |  |  | - non-EDR LSMS | - C |
|  |  |  |  | - EDR LSMS | - C |
|  | - Objective: | - NPAC OP GUI – NPAC Personnel modify an NPA-NXX-X specifying the Old NPA-NXX, that is scheduled for an NPA Split, prior to Permissive Dial Period (PDP) Start Date – Success |  |  |  |
| - B. | - REFERENCES |  |  |  |  |
|  | - NANC Change Order Revision Number: |  | Change Order Number(s): | - NANC 109 |  |
|  | - NANC FRS Version Number: | - 3.0.0 | - Relevant Requirement(s): | - RR3-37.1 |  |
|  | - NANC IIS Version Number: | - 3.0.0 | - Relevant Flow(s): | - 1.2 Service Provider NPA-NXX-X Modification by NPAC SMS |  |

Test case procedures incorporated into test case 192-1 from Release 3.2.

| - A. | - TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: | - 7.12 | SUT Priority: | - SOA LTI | - N/A |
|  |  |  |  | - SOA | - C |
|  |  |  |  | - non-EDR LSMS | - C |
|  |  |  |  | - EDR LSMS | - C |
|  | - Objective: | - NPAC OP GUI – NPAC Personnel modify an NPA-NXX-X specifying the Old NPA-NXX, that is involved in an NPA Split, during Permissive Dial Period (PDP) – Success |  |  |  |
| - B. | - REFERENCES |  |  |  |  |
|  | - NANC Change Order Revision Number: |  | Change Order Number(s): | - NANC 109 |  |
|  | - NANC FRS Version Number: | - 3.0.0 | - Relevant Requirement(s): | - RR3-37.3 |  |
|  | - NANC IIS Version Number: | - 3.0.0 | - Relevant Flow(s): | - 1.2 Service Provider NPA-NXX-X Modification by NPAC SMS |  |

Test case procedures incorporated into test case 192-1 from Release 3.2.

| - A. | - TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: | - 7.13 | SUT Priority: | - SOA LTI | - N/A |
|  |  |  |  | - SOA | - C |
|  |  |  |  | - non-EDR LSMS | - C |
|  |  |  |  | - EDR LSMS | - C |
|  | - Objective: | - NPAC OP GUI – NPAC Personnel modify an NPA-NXX-X specifying the New NPA-NXX, that is involved in an NPA Split, during Permissive Dial Period (PDP) – Success |  |  |  |
| - B. | - REFERENCES |  |  |  |  |
|  | - NANC Change Order Revision Number: |  | Change Order Number(s): | - NANC 109 |  |
|  | - NANC FRS Version Number: | - 3.0.0 | - Relevant Requirement(s): | - RR3-37.3 |  |
|  | - NANC IIS Version Number: | - 3.0.0 | - Relevant Flow(s): |  |  |

Test case procedures incorporated into test case 192-1 from Release 3.2.

| - A. | - TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: | - 7.14 | SUT Priority: | - SOA LTI | - N/A |
|  |  |  |  | - SOA | - O |
|  |  |  |  | - non-EDR LSMS | - R |
|  |  |  |  | - EDR LSMS | - R |
|  | - Objective: | - NPAC OP GUI - NPAC Personnel create a Number Pool Block using the Old NPA-NXX-X that is part of an NPA Split, during Permissive Dial Period (PDP) - Success |  |  |  |
| - B. | - REFERENCES |  |  |  |  |
|  | - NANC Change Order Revision Number: |  | Change Order Number(s): | - NANC 109 |  |
|  | - NANC FRS Version Number: | - 3.0.0 | - Relevant Requirement(s): | - RR3-87, RR3-43, RR3-44, RR3-45, RR3-218 |  |
|  | - NANC IIS Version Number: | - 3.0.0 | - Relevant Flow(s): | - 2.2 Number Pool Block Create by NPAC SMS - 2.3 Number Pool Block Create Broadcast: Successful - 2.3.1 Number Pool Block Create Broadcast Successful to Local SMS - 2.3.2 Number Pool Block Create: Successful Broadcast |  |

Test case procedures incorporated into test case 192-1 from Release 3.2.

| A. | TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Test Case Number: | 7.15 | SUT Priority: | SOA LTI | - N/A |
|  |  |  |  | SOA | - C |
|  |  |  |  | non-EDR LSMS | - R |
|  |  |  |  | EDR LSMS | - R |
|  | Objective: | SOA – Service Provider Personnel create a Number Pool Block using the Old NPA-NXX-X that is part of an NPA Split, during Permissive Dial Period (PDP) - Success |  |  |  |
| B. | REFERENCES |  |  |  |  |
|  | NANC Change Order Revision Number: |  | Change Order Number(s): | NANC 109 |  |
|  | NANC FRS Version Number: | 3.0.0 | Relevant Requirement(s): | RR3-87, RR3-43, RR3-44, RR3-45, RR3-218 |  |
|  | NANC IIS Version Number: | 3.0.0 | Relevant Flow(s): | - 2.1 Number Pool Block Create by SOA - 2.3 Number Pool Block Create Broadcast: Successful - 2.3.1 Number Pool Block Create Broadcast to Local SMS 2.3.2 Number Pool Block Create: Successful Broadcast |  |

Test case procedures incorporated into test case 192-1 from Release 3.2.

| - A. | - TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: | - 7.17 | SUT Priority: | - SOA LTI | - N/A |
|  |  |  |  | - SOA | - O |
|  |  |  |  | - non-EDR LSMS | - R |
|  |  |  |  | - EDR LSMS | - R |
|  | - Objective: | - NPAC OP GUI -NPAC Personnel create a Number Pool Block using the New NPA-NXX-X involved in an NPA Split, during Permissive Dial Period (PDP) – Success |  |  |  |
| - B. | - REFERENCES |  |  |  |  |
|  | - NANC Change Order Revision Number: |  | Change Order Number(s): | - NANC 109 |  |
|  | - NANC FRS Version Number: | - 3.0.0 | - Relevant Requirement(s): | - RR3-44, RR3-45 |  |
|  | - NANC IIS Version Number: | - 3.0.0 | - Relevant Flow(s): | - 2.2 Number Pool Block Create by NPAC SMS - 2.3 Number Pool Block Create Broadcast: Successful - 2.3.2 Number Pool Block Create: Successful Broadcast |  |

Test case procedures incorporated into test case 192-1 from Release 3.2.

| - A. | - TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: | - 7.18 | SUT Priority: | - SOA LTI | - N/A |
|  |  |  |  | - SOA | - C |
|  |  |  |  | - non-EDR LSMS | - R |
|  |  |  |  | - EDR LSMS | - R |
|  | - Objective: | - SOA – Service Provider Personnel create a Number Pool Block using the New NPA-NXX-X involved in an NPA Split, during Permissive Dial Period (PDP) - Success |  |  |  |
| - B. | - REFERENCES |  |  |  |  |
|  | - NANC Change Order Revision Number: |  | Change Order Number(s): | - NANC 109 |  |
|  | - NANC FRS Version Number: | - 3.0.0 | - Relevant Requirement(s): | - RR3-87, RR3-44, RR3-45 |  |
|  | - NANC IIS Version Number: | - 3.0.0 | - Relevant Flow(s): | - 2.1 Number Pool Block Create by SOA - 2.3 Number Pool Block Create Broadcast: Successful - 2.3.1 Number Pool Block Create Broadcast to Local SMS - 2.3.2 Number Pool Block Create: Successful Broadcast |  |

Test case procedures incorporated into test case 192-1 from Release 3.2.

| - A. | - TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: | - 7.20 | SUT Priority: | - SOA LTI | - N/A |
|  |  |  |  | - SOA | - C |
|  |  |  |  | - non-EDR LSMS | - R |
|  |  |  |  | - EDR LSMS | - R |
|  | - Objective: | NPAC OP GUI - NPAC Personnel modify a Number Pool Block using the Old NPA-NXX-X that is part of an NPA Split, during Permissive Dial Period (PDP) – Success |  |  |  |
| - B. | - REFERENCES |  |  |  |  |
|  | - NANC Change Order Revision Number: |  | Change Order Number(s): | - NANC 109 |  |
|  | - NANC FRS Version Number: | - 3.0.0 | - Relevant Requirement(s): | - RR3-46, RR3-47, RR3-218 |  |
|  | - NANC IIS Version Number: | - 3.0.0 | - Relevant Flow(s): | - 2.10 Number Pool Block Modify by NPAC SMS - 2.12.1 Number Pool Block Modify Successful Broadcast to Local SMS - 2.12.2 Number Pool Block Modify Successful Broadcast NPAC SMS Updates |  |

Test case procedures incorporated into test case 192-1 from Release 3.2.

| - A. | - TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: | - 7.21 | SUT Priority: | - SOA LTI | - N/A |
|  |  |  |  | - SOA | - C |
|  |  |  |  | - non-EDR LSMS | - R |
|  |  |  |  | - EDR LSMS | - R |
|  | - Objective: | - SOA – Service Provider Personnel modify a Number Pool Block using the Old NPA-NXX-X that is part of an NPA Split, during Permissive Dial Period (PDP) - Success |  |  |  |
| - B. | - REFERENCES |  |  |  |  |
|  | - NANC Change Order Revision Number: |  | Change Order Number(s): | - NANC 109 |  |
|  | - NANC FRS Version Number: | - 3.0.0 | - Relevant Requirement(s): | - RR3-46, RR3-47, RR3-218 |  |
|  | - NANC IIS Version Number: | - 3.0.0 | - Relevant Flow(s): | - 2.11 Number Pool Block Modify by Block Holder SOA - 2.12 Number Pool Block Modify Broadcast to Local SMS Success - 2.12.1 Number Pool Block Modify Successful Broadcast to Local SMS - 2.12.2 Number Pool Block Modify Successful Broadcast NPAC SMS Updates |  |

Test case procedures incorporated into test case 192-1 from Release 3.2.

| - A. | - TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: | - 7.23 | SUT Priority: | - SOA LTI | - N/A |
|  |  |  |  | - SOA | - C |
|  |  |  |  | - non-EDR LSMS | - R |
|  |  |  |  | - EDR LSMS | - R |
|  | - Objective: | SOA – Service Provider Personnel modify a Number Pool Block using the New NPA-NXX-X that is part of an NPA Split, during Permissive Dial Period (PDP) - Success |  |  |  |
| - B. | - REFERENCES |  |  |  |  |
|  | - NANC Change Order Revision Number: |  | Change Order Number(s): | - NANC 109 |  |
|  | - NANC FRS Version Number: | - 3.0.0 | - Relevant Requirement(s): | - RR3-46, RR3-47 |  |
|  | - NANC IIS Version Number: | - 3.0.0 | - Relevant Flow(s): | - 2.11 Number Pool Block Modify by Block Holder SOA - 2.12.1 Number Pool Block Modify Successful Broadcast to Local SMS - 2.12.2 Number Pool Block Modify Successful Broadcast NPAC SMS Updates |  |

Test case procedures incorporated into test case 192-1 from Release 3.2.

| - A. | - TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: | - 7.25 | SUT Priority: | - SOA LTI | - N/A |
|  |  |  |  | - SOA | - O |
|  |  |  |  | - non-EDR LSMS | - R |
|  |  |  |  | - EDR LSMS | - R |
|  | - Objective: | - NPAC OP GUI – NPAC Personnel de-pool an NPA-NXX-X specifying the Old NPA-NXX-X that that has an ‘active’ Number Pool Block associated with it and is scheduled for an NPA Split, prior to Permissive Dial Period (PDP) Start Date – Success |  |  |  |
| - B. | - REFERENCES |  |  |  |  |
|  | - NANC Change Order Revision Number: |  | Change Order Number(s): | - NANC 109 |  |
|  | - NANC FRS Version Number: | - 3.0.0 | - Relevant Requirement(s): | - RR3-38.1 |  |
|  | - NANC IIS Version Number: | - 3.0.0 | - Relevant Flow(s): | - 2.19Number Pool Block De-Pool by NPAC SMS - 2.20.1 Number Pool Block De-Pool Successful Broadcast of Subscription Version and Number Pool Block Deletes - 2.20.2 Number Pool Block De-Pool Broadcast Successful NPA-NXX-X Updates |  |

Test case procedures incorporated into test case 192-1 from Release 3.2.

| - A. | - TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: | - 7.27 | SUT Priority: | - SOA LTI | - N/A |
|  |  |  |  | - SOA | - C |
|  |  |  |  | - non-EDR LSMS | - R |
|  |  |  |  | - EDR LSMS | - R |
|  | - Objective: | - NPAC OP GUI - NPAC Personnel de-pool an NPA-NXX-X specifying the Old NPA-NXX-X that has an ‘active’ Number Pool Block associated with it and is involved in an NPA Split, during Permissive Dial Period (PDP) – Success |  |  |  |
| - B. | - REFERENCES |  |  |  |  |
|  | - NANC Change Order Revision Number: |  | Change Order Number(s): | - NANC 109 |  |
|  | - NANC FRS Version Number: | - 3.0.0 | - Relevant Requirement(s): | - RR3-38.3, RR3-48, RR3-218 |  |
|  | - NANC IIS Version Number: | - 3.0.0 | - Relevant Flow(s): | - 2.19Number Pool Block De-Pool by NPAC SMS - 2.20.1 Number Pool Block De-Pool Successful Broadcast of Subscription Version and Number Pool Block Deletes - 2.20.2 Number Pool Block De-Pool Broadcast Successful NPA-NXX-X Updates |  |

Test case procedures incorporated into test case 192-1 from Release 3.2.

| - A. | - TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: | - 7.28 | SUT Priority: | - SOA LTI | - N/A |
|  |  |  |  | - SOA | - C |
|  |  |  |  | - non-EDR LSMS | - R |
|  |  |  |  | - EDR LSMS | - R |
|  | - Objective: | - NPAC OP GUI – NPAC Personnel de-pool an NPA-NXX-X specifying the New NPA-NXX-X that is involved in an NPA Split, during Permissive Dial Period (PDP) – Success |  |  |  |
| - B. | - REFERENCES |  |  |  |  |
|  | - NANC Change Order Revision Number: |  | Change Order Number(s): | - NANC 109 |  |
|  | - NANC FRS Version Number: | - 3.0.0 | - Relevant Requirement(s): | - RR3-38.3, RR3-48, RR3-218 |  |
|  | - NANC IIS Version Number: | - 3.0.0 | - Relevant Flow(s): | - 2.19Number Pool Block De-Pool by NPAC SMS - 2.20.1 Number Pool Block De-Pool Successful Broadcast of Subscription Version and Number Pool Block Deletes - 2.20.2 Number Pool Block De-Pool Broadcast Successful NPA-NXX-X Updates |  |

Test case procedures incorporated into test case 192-1 from Release 3.2.

## Resynchronization

| A. | TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Test Case Number: | 8.1 | SUT Priority: | SOA LTI | N/A |
|  |  |  |  | SOA | N/A |
|  |  |  |  | non-EDR LSMS | C |
|  |  |  |  | EDR LSMS | C |
|  | Objective: | LSMS - Service Provider Personnel for either an EDR or non-EDR LSMS submit a resynchronization request for Network Data, Block Data, SV Data and Notification Data by time range, over the LSMS to NPAC SMS Interface, with the Service Provider’s NPAC Customer LSMS NPA-NXX-X Indicator set to the value that they support. – Success |  |  |  |
| B. | REFERENCES |  |  |  |  |
|  | NANC Change Order Revision Number: |  | Change Order Number(s): | NANC 109 |  |
|  | NANC FRS Version Number: | 3.0.0 | Relevant Requirement(s): | RR6-29, RR6-30, RR6-31, RR6-32, RR6-34, RR6-78, RR6-77, RR6-75, RR6-74, RR6-73, RR6-45, RR6-46, RR6-47, RR6-48, RR6-49, RR3-120, RR6-64, RR6-65, RR6-68, RR6-69, RR6-71, RR6-72 |  |
|  | NANC IIS Version Number: | 3.0.0 | Relevant Flow(s): | 5.1 Sequencing of Events on Initialization/Resynchronization of EDR Local SMS 7.1.1 Sequencing of Events on Initialization/Resynchronization of Non-EDR Local SMS |  |

Test case procedures incorporated into test case 187-1 from Release 3.2.

| A. | TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Test Case Number: | 8.2 | SUT Priority: | SOA LTI | N/A |
|  |  |  |  | SOA | N/A |
|  |  |  |  | non-EDR LSMS | C |
|  |  |  |  | EDR LSMS | N/A |
|  | Objective: | LSMS - Service Provider Personnel for a non-EDR LSMS submit a resynchronization request for Network Data, Block Data, SV Data and Notification Data by time range, over the LSMS to NPAC SMS Interface, with the Service Provider’s NPAC Customer LSMS NPA-NXX-X Indicator set to TRUE. – Success |  |  |  |
| B. | REFERENCES |  |  |  |  |
|  | NANC Change Order Revision Number: |  | Change Order Number(s): | NANC 109 |  |
|  | NANC FRS Version Number: | 3.0.0 | Relevant Requirement(s): | RR6-78, RR6-77, RR6-76, RR6-74, RR6-45, RR6-46, RR6-47, RR6-48, RR6-49, RR3-121, RR6-68, RR6-69 |  |
|  | NANC IIS Version Number: | 3.0.0 | Relevant Flow(s): | 5.2 Sequencing of Events on Initialization/Resynchronization of Non-EDR Local SMS |  |

Test case procedures incorporated into test case 8.1 for release 3.0.  Test Case 8.1 has been superseded/incorporated into test case 187-1 from Release 3.2.

| A. | TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Test Case Number: | 8.3 | SUT Priority: | SOA LTI | N/A |
|  |  |  |  | SOA | C |
|  |  |  |  | non-EDR LSMS | N/A |
|  |  |  |  | EDR LSMS | N/A |
|  | Objective: | SOA - Service Provider Personnel submit a resynchronization request for Network Data and Notification Data by time range, over the SOA to NPAC SMS Interface, with the Service Provider’s NPAC Customer SOA NPA-NXX-X Indicator set to the value they support. - Success |  |  |  |
| B. | REFERENCES |  |  |  |  |
|  | NANC Change Order Revision Number: |  | Change Order Number(s): | NANC 109 |  |
|  | NANC FRS Version Number: | 3.0.0 | Relevant Requirement(s): | RR6-29, RR6-30, RR6-31, RR6-32, RR6-33, RR6-50, RR6-51, RR6-52, RR6-53, RR6-54 |  |
|  | NANC IIS Version Number: | 3.0.0 | Relevant Flow(s): | 5.3 Sequencing of Events on Initialization/Resynchronization of SOA |  |

Test case procedures incorporated into test case 187-4 from Release 3.2.

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 8.4 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | N/A |
|  |  |  |  |  |  |  | non-EDR LSMS |  | C |
|  |  |  |  |  |  |  | EDR LSMS |  | C |
|  | Objective: |  | LSMS - Service Provider Personnel submit a resynchronization request for network data, Number Pool Block Data, subscription version data, and notifications by time range (time range exceeds ‘Maximum Download Duration’ tunable), over the LSMS to NPAC SMS Interface. – Error |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR6-31, RR6-65, RR6-66, RR6-67 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 5.1 Sequencing of Events on Initialization/Resynchronization of non-EDR Local SMS 7.1.2 Sequencing of Events on Initialization/Resynchronization of EDR Local SMS |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Filter the data so that the LSMS under test and one other associated LSMS will accept messages from NPAC. - Verify the ‘Maximum Download Duration’ tunable is set to a value less than what the LSMS expects. - While the LSMS is disconnected from the NPAC SMS, NPAC Personnel should perform the following functions: - Create an NPA-NXX. - Add at least 1 Block for different Service Providers and let the retry timer expire before the Service Provider associates their LSMS. - Delete at least 1 NPA-NXX-X for different Service Providers and let the retry timer expire before the Service Provider associates their LSMS. - Modify at least 1 Block for different Service Providers and let the retry timer expire before the Service Provider associates their LSMS. - Issue the first create for an Inter-Service Provider Subscription Version using an NPA-NXX that has never been ported before. - Issue a Scheduled Downtime Notification. - Issue an immediate disconnect for a subscription version and let the retry timer expire before the Service Provider associates their LSMS. - Issue an activate request for an Inter-Service Provider Subscription Version and let the retry timer expire before the Service Provider associates their LSMS. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | The service provider LSMS should be ‘disassociated’ from the NPAC SMS while NPAC Personnel are performing the setup specified above. |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
|  | SP | The LSMS Service Provider establishes an association to the NPAC SMS with the resynchronization flag set to TRUE . |  | NPAC |  | The NPAC SMS receives the association bind request from the LSMS. Once the association is established, the NPAC SMS queues all current updates. |  |  |  |
|  | SP | The LSMS issues an M-ACTION Request for recovery to the NPAC SMS and specifies a time range. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the LSMS, verifies the duration exceeds the ‘Maximum Download Duration’ (this violates system requirements) and issues an M-ACTION Error Response indicating ‘time-range-invalid’. |  |  |  |
|  | NPAC | NPAC Personnel query the NPAC SMS for the following information which NPAC Personnel manipulated in the prerequisites for this test case: - The NPA-NXX that was created. - The Number Pool Block that was created. - The Number Pool Block that was modified. - The Number Pool Block that was de-pooled. - The NPA-NXX-X that was deleted. - The First Port Notification that was created. - The Scheduled Downtime Notification that was created. - The Subscription Version that was deleted. - The Subscription Version that was activated. |  | NPAC |  | NPAC Personnel verify the following information: - The NPA-NXX that was created exists. - The Number Pool Block that was created exists with a status of ‘partial failure’ and with a Failed SP List populated appropriately. - The Number Pool Block that was modified exists with a status of ‘active’, the appropriate attributes were modified, and the Failed SP List is populated appropriately. - The Number Pool Block that was de-pooled exists with a status of ‘old’ and the Failed SP List is populated appropriately. - The NPA-NXX-X still exists on the NPAC because a Failed SP List is not empty for the associated Number Pool Block. - The First Port Notification failed to the respective Service Provider in this test case. - The Scheduled Downtime Notification failed to the respective Service Provider in this test case. - The Subscription Version that was deleted exists with a status of ‘old’ and the Failed SP List is populated appropriately. - The Subscription Version that was activated exists with a status of ‘partial failure’ and the Failed SP List is populated appropriately. |  |  |  |
|  | SP - Optional | Service Provider Personnel, attempt to locate the First Port and NPAC Scheduled Downtime notifications on their LSMS. |  | SP - Optional |  | Service Provider Personnel verify that neither notification was received from the NPAC SMS. |  |  |  |
|  | SP - Optional | Service Provider Personnel, using the LSMS, perform a local query for the following data that NPAC Personnel manipulated in the prerequisites of this test case: - The NPA-NXX that was created. - The Number Pool Block that was created. - The Number Pool Block that was modified. - The Number Pool Block that was de-pooled. - The NPA-NXX-X that was deleted – if supported by the Service Provider LSMS. - The Subscription Version that was deleted. - The Subscription Version that was activated. |  | SP |  | Service Provider Personnel verify the following: - The NPA-NXX does not exist on their LSMS. - The Number Pool Block that was created does not exist on their LSMS. - The Number Pool Block that was modified exists on their LSMS, but the attributes which NPAC Personnel modified do not reflect their changes. - The Number Pool Block that was de-pooled still exists on their LSMS. - The NPA-NXX-X that was deleted still exists on their LSMS – if supported by the Service Provider LSMS. - The Subscription Version that was deleted still exists on their LSMS. - The Subscription Version that was activated does not exist on their LSMS. |  |  |  |
|  | SP - Conditional | Service Provider Personnel perform an NPAC SMS query for the following information manipulated by NPAC Personnel in the prerequisites of this test case: - The NPA-NXX that was created. - The Number Pool Block that was created. - The Number Pool Block that was modified. - The Number Pool Block that was de-pooled. - The NPA-NXX-X that was deleted. - The First Port Notification that was created. - The Scheduled Downtime Notification that was created. - The Subscription Version that was deleted. - The Subscription Version that was activated. |  | SP |  | Service Provider Personnel verify the following information on the NPAC SMS: - The NPA-NXX that was created exists. - The Number Pool Block that was created exists with a status of ‘partial failure’ and with a Failed SP List populated appropriately. - The Number Pool Block that was modified exists with a status of ‘active’, the appropriate attributes were modified, and the Failed SP List is populated appropriately. - The Number Pool Block that was de-pooled exists with a status of ‘old’ and the Failed SP List is populated appropriately. - The NPA-NXX-X still exists on the NPAC because a Failed SP List is not empty for the associated Number Pool Block. - The First Port Notification failed to the respective Service Provider in this test case. - The Scheduled Downtime Notification failed to the respective Service Provider in this test case. - The Subscription Version that was deleted exists with a status of ‘old’ and the Failed SP List is populated appropriately. - The Subscription Version that was activated exists with a status of ‘partial failure’ and the Failed SP List is populated appropriately. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Test Case Number: | 8.5 | SUT Priority: | SOA LTI | N/A |
|  |  |  |  | SOA | N/A |
|  |  |  |  | non-EDR LSMS | C |
|  |  |  |  | EDR LSMS | C |
|  | Objective: | LSMS - Service Provider Personnel submit a resynchronization request for a range of Number Pool Blocks (Number of Blocks exceeds the ‘Maximum Number of Download Records’ tunable), over the LSMS to NPAC SMS Interface. – Error |  |  |  |
| B. | REFERENCES |  |  |  |  |
|  | NANC Change Order Revision Number: |  | Change Order Number(s): | NANC 109 |  |
|  | NANC FRS Version Number: | 3.0.0 | Relevant Requirement(s): | RR6-65, RR6-66, RR6-67 |  |
|  | NANC IIS Version Number: | 3.0.0 | Relevant Flow(s): | 5.1 Sequencing of Events on Initialization/Resynchronization of EDR Local SMS |  |
| C. | PREREQUISITE |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |
|  | Prerequisite NPAC Setup: | - No filters are applied to the data being tested. - Verify the ‘Maximum Number of Download Records’ tunable is set to a value less than what the LSMS expects. - While the LSMS is disconnected from the NPAC SMS, NPAC Personnel should perform the following functions: - Add at least 2 Blocks for different Service Providers inside and outside of the requested Block range. - Delete at least 2 Blocks for different Service Providers inside and outside of the requested Block range. - Modify at least 2 Blocks for different Service Providers inside and outside of the requested Block range. |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |

Test case procedures incorporated into test case 187-3 from Release 3.2.

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 8.6 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | N/A |
|  |  |  |  |  |  |  | non-EDR LSMS |  | C |
|  |  |  |  |  |  |  | EDR LSMS |  | C |
|  | Objective: |  | LSMS - Service Provider Personnel submit a resynchronization request for a range of Number Pool Blocks over the LSMS to NPAC SMS Interface. (Blocks exist inside and outside of the requested Number Pool Block range.) – Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR3-120, RR6-64, RR6-65, RR6-70, RR6-71, RR6-72 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 5.1 Sequencing of Events on Initialization/Resynchronization of EDR Local SMS |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - An NPA-NXX filter applies to the data being tested. - While the LSMS is disconnected from the NPAC SMS, NPAC Personnel should perform the following functions: - Add at least 2 Blocks for different Service Providers inside and outside of the requested Block range. - Delete at least 2 Blocks for different Service Providers inside and outside of the requested Block range. - Modify at least 2 Blocks for different Service Providers inside and outside of the requested Block range. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | The LSMS Service Provider establishes an association to the NPAC SMS with the resynchronization flag set to TRUE. |  | NPAC |  | The NPAC SMS receives the association bind request from the LSMS. Once the association is established, the NPAC SMS queues all current updates. |  |  |  |
| 2. | SP | The LSMS issues an M-ACTION Request lnpDownload (Number Pool Block data) to the NPAC SMS and specifies a range of NPA-NXX-X values. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the LSMS and issues an M-ACTION Response lnpDownload with the no data to the LSMS (the applicable blocks are not sent because of the NPA-NXX filter). |  |  |  |
| 3. | NPAC | NPAC Personnel query the Number Pool Block data that was not sent to the LSMS. |  | NPAC |  | Verify that the Number Pool Block data was updated appropriately. |  |  |  |
| 4. | SP - Optional | Service Provider Personnel, using the LSMS, perform a local query for the Number Pool Block data updated in this test case. |  | SP |  | Verify that the following updates were not sent: - 1 Number Pool Block create - 1 Number Pool Block modify - 1 Number Pool Block delete |  |  |  |
| 5. | SP - Conditional | Service Provider Personnel, perform an NPAC SMS query for the updated Number Pool Block data. |  | SP |  | Verify that the following updates were made: - 1 Number Pool Block create - 1 Number Pool Block modify - 1 Number Pool Block delete |  |  |  |
| 6. | NPAC | NPAC Personnel perform a full audit for the Number Pool Blocks that were manipulated during this test case. |  | NPAC |  | Using the Audit Results log verify that no updates were made. . If any updates were made as a result of running this audit, this test case fails. |  |  |  |

## Audit Test Cases:

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 9.2 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | N/A |
|  |  |  |  |  |  |  | non-EDR LSMS |  | R |
|  |  |  |  |  |  |  | EDR LSMS |  | R |
|  | Objective: |  | NPAC OP GUI - NPAC Personnel initiate a full audit for a single TN, with LNP Type = POOL, for all Service Providers, discrepancies exist. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 109 |  |  |
|  | NANC FRS Version Number: |  | 3.0.0 |  | Relevant Requirement(s): |  | RR8-6, RR8-7, RR8-8, RR8-9, RR8-10, RR8-11, RR8-13, RR8-14, RR8-15, RR8-16 |  |  |
|  | NANC IIS Version Number: |  | 3.0.0 |  | Relevant Flow(s): |  | 6.1.2 NPAC Performs Audit Comparisons 6.1.3 NPAC SMS Reports Audit Results |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that there are both EDR and non-EDR systems accepting downloads for the NPA-NXX of the TN being audited. - Verify that the TN being audited is part of a Number Pool Block and is of LNP Type 'POOL'. - Create the following discrepancies for the TN being audited: - A discrepancy for some of the GTT data between the Subscription Version on the NPAC SMS and one of the non-EDR LSMSs. - A discrepancy where one of the EDR LSMSs does not have the respective Number Pool Block in its database. This Number Pool Block should have the SOA Origination set to 'TRUE' and should have a status of ‘partial failure’ with a Failed SP List entry. - A discrepancy where one of the non-EDR LSMSs does not have the Subscription Version in its database |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
|  | NPAC | Using the NPAC OP GUI, NPAC Personnel submit a full Audit request (specifying all Subscription Version attributes for audit) for a single TN of LNP Type 'POOL' to the NPAC SMS for all Service Providers in the region. |  | NPAC |  | The NPAC SMS receives the Audit Request from the NPAC Personnel, and determines the request is valid. |  |  |  |
|  | NPAC | - The NPAC SMS determines that this TN is within a 1K Block and begins the Audit to all Service Providers for the specified TN. - The NPAC SMS issues an M-GET Request (scoped and filtered) subscriptionVersion to the non-EDR LSMSs in the region to retrieve the subscription data for audit processing. - The NPAC SMS issues an M-GET Request numberPoolBlock to all EDR LSMSs in the region to retrieve respective Number Pool Block information for audit processing. - The NPAC SMS issues an M-GET Request (scoped and filtered) subscriptionVersion to all EDR LSMSs in the region to retrieve subscription data for audit processing. |  | SP |  | - All but one of the non-EDR LSMSs in the region receive the M-GET Request subscriptionVersion from the NPAC SMS and return the specified Subscription Version object in an M-GET Response subscriptionVersion to the NPAC SMS. - One of the non-EDR LSMSs in the region receive the M-GET Request subscriptionVersion from the NPAC SMS and DO NOT return the specified Subscription Version object in an M-GET Response subscriptionVersion to the NPAC SMS. - All but one of the EDR LSMSs in the region receive the M-GET Request numberPoolBlock from the NPAC SMS and return the specified Number Pool Block object in an M-GET Response numberPoolBlock to the NPAC SMS. - One of the EDR LSMSs in the region receive the M-GET Request numberPoolBlock from the NPAC SMS and DO NOT return the specified Block object in an M-GET Response numberPoolBlock to the NPAC SMS. - The EDR LSMSs in the region receive the M-GET Request subscriptionVersion from the NPAC SMS. The EDR LSMSs do not locate a respective Subscription Version with LNP Type of 'POOL' and issue an M-GET Response subscriptionVersion message back to the NPAC SMS specifying an empty set (no TNs). |  |  |  |
| - 3. | NPAC | The NPAC SMS performs object comparisons. |  |  |  | The NPAC SMS completes the comparisons and finds the following discrepancies: - The GTT data for the Subscription Version on the NPAC SMS and one of the non-EDR LSMSs is discrepant. - One of the EDR LSMSs does not have the respective Number Pool Block in its database. - One of the non-EDR LSMSs does not have the respective Subscription Version in its database. |  |  |  |
|  | NPAC | The NPAC SMS issues an M-SET Request subscriptionVersion to update the GTT attribute to equal the value on the NPAC SMS version of the Subscription Version to the discrepant, non-EDR LSMS system. |  | SP |  | The discrepant non-EDR LSMS receives the M-SET Request from the NPAC SMS, updates the Subscription Version appropriately and issues an M-SET Response back to the NPAC SMS. |  |  |  |
|  | NPAC | The NPAC SMS issues an M-CREATE Request numberPoolBlock to the discrepant EDR LSMS system. |  | SP |  | The discrepant EDR LSMS receives the M-CREATE Request from the NPAC SMS, and creates the respective Number Pool Block appropriately and issues an M-CREATE Response back to the NPAC SMS. |  |  |  |
|  | NPAC | The NPAC SMS issues an M-CREATE Request subscriptionVersion to the non-EDR LSMS that was missing the Subscription Version. |  | SP |  | The discrepant non-EDR LSMS receives the M-CREATE Request from the NPAC SMS, creates the Subscription Version and issues an M-CREATE Response back to the NPAC SMS. |  |  |  |
|  | NPAC | The NPAC SMS issues an M-EVENT-REPORT numberPoolBlockStatusAttributeValueChange for the Number Pool Block and set the Number Pool Block status to 'active' and update the subscriptionFailedSP-List to exclude the Service Provider LSMSs that were corrected to the Block Holder SOA. |  | SP |  | The Block Holder SOA receives the M-EVENT-REPORT for the Number Pool Block, from the NPAC SMS and issues an M-EVENT-REPORT Confirmation back to the NPAC SMS. |  |  |  |
|  | NPAC | The NPAC SMS issues an M-DELETE Request for the subscriptionAudit object to itself. |  | NPAC |  | The NPAC SMS deletes the audit object from its local database and issues an M-DELETE Response to itself indicating the audit object was successfully deleted. |  |  |  |
|  | NPAC | NPAC Personnel perform a query for the audit discrepancy report. |  | NPAC |  | Verify the audit discrepancy report exists. |  |  |  |

End of Chapter