NPAC SMS

Interoperable Interface Specification

NANC Version 3.2.1a

Prepared for:
The North American Numbering Council (NANC)

July 28, 2003

Release 3.2: © 1997 - 2003 NeuStar, Inc.

The Work is subject to the terms of the GNU General Public License (the “GPL”), a copy of which may be found at

ftp://prep.ai.mit.edu/pub/gnu/GPL.  Any use of this Work is subject to the terms of the GPL.  The “Work” covered by the GPL by

operation of this notice and license is this document and any and all modifications to or derivatives of this document.

Where the words “Program,” “software,” “source code,” “code,” or “files” are used in the GPL, users understand and agree that

the “Work” as defined here is substituted for purposes of this notice and license.

This page intentionally left blank.

Table Of Contents

1	Introduction	1

1.1	Document Overview	1

1.2	How To Use This Document	1

1.3	Document Numbering Strategy	2

1.4	Document Version History	2

1.4.1	Release 1.0	2

1.4.2	Release 2.0	3

1.4.3	Release 3.0	3

1.4.4	Release 3.1	3

1.4.5	Release 3.2	3

1.5	References	5

1.5.1	Standards	5

1.5.2	Related Publications	6

1.6	Abbreviations/Definitions	7

2	Interface Overview	9

2.1	Overview	9

2.2	OSI Protocol Support	9

2.3	SOA to NPAC SMS Interface	10

2.3.1	Subscription Administration	10

2.3.2	Audit Requests	11

2.3.3	Notifications	11

2.3.4	Service Provider Data Administration	12

2.3.5	Network Data Download	12

2.3.6	Number Pool Block Administration	12

2.4	NPAC SMS to Local SMS Interface	12

2.4.1	Subscription Version, Number Pool Block and Network Data Download	13

2.4.2	Service Provider Data Administration	13

2.4.3	Notifications	13

3	Hierarchy Diagrams	15

3.1	Overview	15

3.1.1	Managed Object Model Inheritance Hierarchy	15

3.1.2	Log Record Managed Object Hierarchy	17

3.1.3	NPAC SMS to Local SMS Naming Hierarchy for the NPAC SMS	18

3.1.4	NPAC SMS to Local SMS Naming Hierarchy for the Local SMS	19

3.1.5	SOA to NPAC SMS Naming Hierarchy for the NPAC SMS	20

3.1.6	NPAC SMS to SOA Naming Hierarchy for the SOA	21

4	Interface Functionality to CMIP Definition Mapping	23

4.1	Overview	23

4.1.1	Primary NPAC Mechanized Interface Operations	23

4.1.2	Managed Object Interface Functionality	27

4.1.3	Action Interface Functionality	31

4.1.4	Notification Interface Functionality	32

4.2	Scoping and Filtering Support	35

4.2.1	Scoping	35

4.2.2	Filtering	35

4.2.3	Action Scoping and Filtering Support	36

4.3	lnpLocal-SMS-Name and lnpNPAC-SMS-Name Values	36

4.4	OID Usage Information	37

4.4.1	OIDs Used for Bind Requests	37

4.4.2	Other OIDs of Interest	37

4.5	Naming Attributes	37

4.6	Subscription Version M_DELETE Messages	37

4.7	Number Pool Block M_DELETE Messages	38

5	Secure Association Establishment	39

5.1	Overview	39

5.2	Security	39

5.2.1	Authentication and Access Control Information	39

5.2.1.1	System Id	41

5.2.1.2	System Type	41

5.2.1.3	User Id	41

5.2.1.4	List Id	41

5.2.1.5	Key Id	42

5.2.1.6	CMIP Departure Time	43

5.2.1.7	Sequence Number	43

5.2.1.8	Association Functions	43

5.2.1.9	Recovery Mode	44

5.2.1.10	Signature	45

5.2.2	Association Establishment	45

5.2.3	Data Origination Authentication	47

5.2.4	Audit Trail	49

5.3	Association Management and Recovery	49

5.3.1	Establishing Associations	49

5.3.1.1	NpacAssociationUserInfo	49

5.3.1.2	Unbind Requests and Responses	50

5.3.1.3	Aborts	50

5.3.1.4	NPAC SMS Failover Behavior	50

5.3.1.5	Service Provider SOA and Local SMS Procedures	51

5.3.2	Releasing or Aborting Associations	52

5.3.3	Error Handling	52

5.3.3.1	NPAC SMS Error Handling	52

5.3.3.2	Processing Failure Error	53

5.3.4	Recovery	53

5.3.4.1	Local SMS Recovery	55

5.3.4.2	SOA Recovery	55

5.3.4.3	Linked Action Replies during Recovery	55

5.4	Congestion Handling	57

5.4.1	NPAC SMS Congestion	57

5.4.2	NPAC Handling of Local SMS and SOA Congestion	57

6	GDMO Definitions	59

6.1	Overview	59

6.2	Object Definitions	59

7	General ASN.1 Definitions	189

7.1	Overview	189

7.2	LNP ASN.1 Object Identifier Definitions	189

7.3	LNP General ASN.1 Definitions	190

8	Subscription Version Status	209

9	Number Pool Block Status	213

Appendix A:  Errors	A-1

Appendix B:  Message Flow Diagrams	B-1

# Introduction

1

## Document Overview

The NPAC SMS Interoperable Interface Specification contains the information model for the Number Portability Administration Center and Service Management System (NPAC SMS) mechanized interfaces. Both Service Order Activation (SOA) and Local Service Management System (LSMS or Local SMS) interfaces to the NPAC SMS are described in this document.

## How To Use This Document

The NPAC SMS Interoperable Interface Specification contains the following sections:

Section 1 Introduction -- This section describes the conventions and organization of this document. It also lists related documentation.

Section 2 Interface Overview -- This section contains an overview of protocol requirements and a brief description of the functionality provided in each interface.

Section 3 Hierarchy Diagrams -- This section contains the class hierarchy diagrams for all managed objects defined in the interoperable interface.

Section 4 Interface Functionality to CMOP Definition Mapping -- This section contains the mapping of the interface functionality to the managed objects, attributes, actions, and notifications.

Section 5 Secure Association Establishment-- This section contains information on secure association establishment

Section 6 GDMO Definitions -- This section contains the GDMO interface definitions supporting the SOA to NPAC SMS interface and the NPAC SMS to Local SMS interface

Section 7 General ASN.1 Definitions -- This section contains the ASN.1 definitions that support the GDMO definitions in Section 7.

Section 8 Subscription Version Status -- This section contains a Subscription Version Status diagram, which illustrates the transition from one subscription version state to another.

Appendix A Errors -- This appendix contains the valid errors associated with CMISE confirmed primitives used in the interoperable interface definitions.

Appendix B Message Flow Diagrams -- This appendix contains the message flow diagrams.

Appendix C Midwest Region Number Pooling Message Flow Diagrams -- This appendix is deleted in Release 3.0.0.

## Document Numbering Strategy

Starting with Release 2.0 the documentation number of the IIS document will be Version X.Y.Z as follows:

X – will only be incremented when a new major release of the NPAC SMS system is authorized.  It will contain only the Change Orders that have been authorized for inclusion in this new major release.

Y – will only be incremented when a new sub-release of an existing release X is authorized.  It will contain only the Change Orders that have been authorized for inclusion in this new sub-release.

Z – will be incremented when documentation only clarifications and/or backward compatibility issues or other deficiency corrections are made in the IIS and/or FRS.  This number will be reset to 0 when Y is incremented.

For example, the first release of the Release 2 IIS will be numbered 2.0.0.  If documentation only clarifications are introduced in the next release of the IIS document it will be numbered 2.0.1.  If requirements are added to Release 2.0 that require NPAC SMS software changes then the next release of the IIS document will be numbered 2.1.0.

Starting with Release 3.2, the documentation number of the FRS document will include a "lowercase letter" following the Z designation.  This "lowercase letter" will essentially serve as a version indicator for the release of the documentation, such that the X.Y.Za will be a unique identifier.  It will be used for both drafts and final versions.  For example, the first release using this new convention will be 3.2.0a, followed by 3.2.0b, and so on.  .  The “lower case letter” shall be reset to ‘a’ when Z is incremented.

This number scheme is intended to make the mapping between NPAC SMS and the FRS and IIS documentation consistent.

## Document Version History

### Release 1.0

NANC Version 1.0, released on 04/07/97, contains changes from the ICC Subcommittee IIS Version 1.1.5.

NANC Version 1.1, released on 05/08/97, contains changes from the NANC IIS Version 1.0.

NANC Version 1.2, released on 05/25/97, contains changes from the NANC IIS Version 1.1.

NANC Version 1.3, released on 07/09/97, contains changes from the NANC IIS Version 1.2.

NANC Version 1.4, released on 08/08/97, contains changes from the NANC IIS Version 1.3.

NANC Version 1.5, released on 09/09/97, contains changes from the NANC IIS Version 1.4.

NANC Version 1.6, released on 11/12/97, contains changes from the NANC IIS Version 1.5.

NANC Version 1.7, released on 12/12/97, contains changes from the NANC IIS Version 1.6.

NANC Version 1.8, released on 2/11/98, contains changes from the NANC IIS Version 1.7.

NANC Version 1.9, released on 5/13/98, contains changes from the NANC IIS Version 1.8.

- NANC Version 1.10, released on 7/8/98, contains changes from the NANC IIS Version 1.9.

### Release 2.0

NANC Version 2.0.0, released on 12/14/98, contains changes from the NANC IIS Version 1.10.

NANC Version 2.0.1, released on 2/25/99, contains changes from the NANC IIS Version 2.0.0.

NANC Version 2.0.2, released on 9/1/99, contains changes from the NANC IIS Version 2.0.1.

### Release 3.0

NANC Version 3.0.0, released on 1/28/00 and 2/14/00 (revised version), contains changes from the NANC IIS Version 2.0.2.

NANC Version 3.0.1, released on 6/6/00, contains changes from the NANC IIS Version 3.0.0.

NANC Version 3.0.2, released on 9/11/00, contains changes from the NANC IIS Version 3.0.0.

### Release 3.1

- NANC Version 3.1.0, released on 8/24/01, contains changes from the NANC IIS Version 3.0.2.

### Release 3.2

- NANC Version 3.2.0, released on 8/27/02, contains changes from the NANC IIS Version 3.1.0

- NANC Version 3.2.1a, released on 7/28/03, contains the following changes from the NANC IIS Version 3.2.0a:

- Change Order NANC 359 – Doc Only Change Order for SPID and Billing ID:  Change definition for SPID and Billing ID

- Change Order NANC 360 – Doc Only Change Order for Recovery:  Maximum TN Recovery Tunable

- Change Order NANC 361 – Doc Only Change Order for GDMO:  Range Version of Object Creation Notification

- Change Order NANC 364 – Doc Only Change Order for ASN.1:  Create Action comment

- Change Order NANC 365 – Doc Only Change Order for IIS/GDMO:  PTO and SV Query discrepancies between the tewo documents

- Change Order NANC 366 – Doc Only Change Order for FRS/IIS:  Remove reference that specify GUI is in Central Time

- Change Order NANC 369 – Doc Only Change Order for IIS:  Flow Updates

- Change Order NANC 371 – Doc Only Change Order for Audits:  Update Behavior

- Change Order NANC 373 – Doc Only Change Order:  Conflict AVC

- Change Order NANC 374 – Doc Only Change Order:  PTO SP

- Change Order NANC 376 – Doc Only Change Order:  Modify Active with Failed List

- Change Order NANC 377 – Doc Only Change Order:  Missing IIS Flow for 2nd Create by Old SP with Auth=FALSE

- Change Order NANC 378 – Doc Only Change Order:  Missing IIS Flow for cancellation of a disconnect-pending SV

- Change Order NANC 379 – Doc Only Change Order:  Update IIS Flow, 5.1.18

- Change Order NANC 380 – Doc Only Change Order:  IIS Updates

## References

### Standards

ANSI T1.224-1992, Operations, Administration, Maintenance, and Provisioning (OAM&P) - Protocols for Interfaces between Operations Systems in Different Jurisdictions.

ANSI T1.243-1995, Telecommunications, Operations, Administration, Maintenance and Provisioning (OAM&P) - Baseline Security Requirements for the Telecommunications Management Network (TMN).

ANSI T1.246, Operations, Administration, Maintenance and Provisioning (OAM&P) - Information Model and Services for Interfaces between Operations Systems across Jurisdictional Boundaries to Support Configuration Management - Customer Account Record Exchange (CARE).

Bellcore TA- 1253, Generic Requirements for Operations Interfaces Using OSI Tools: Network Element Security Administration.

Committee T1 Technical Report No, 40, Security Requirements for Electronic Bonding Between Two TMNs.

ISO/IEC 11183-1:1992, Information Technology - International Standardized Profiles AOM ln OSI Management - Management Communications - Part 1  Specification of ACSE, Presentation and Session Protocols for the use by ROSE and CMISE.

ISO/IEC 11183-2:1992, Information Technology - International Standardized Profiles AOM ln OSI Management - Management Communications - Part 2:  CMISE/ROSE for AOM12 - Enhanced Management Communications.

ISO/IEC 11183-3:1992, Information Technology - International Standardized Profiles AOM ln OSI Management - Management Communications - Part 3: CMISE/ROSE for AOM12 - Basic Management Communications.

ITU X.509, Information Technology - Open Systems Interconnection - The Directory Authentication Framework.

ITU X.690/ISO IS 8825-1 Annex D, ASNI/BER Encoding of Digital Signatures and Encrypted Cyphertext.

ITU X.741, OSI Systems Management, Objects and Attributes for Access Control

ITU X.803, Upper Layers Security Model.

NMF Forum 016, Issue 1.0, 1992, OMNIPoint 1 Specifications and Technical Reports, Application Services Security of Management.

OIW Stable Implementation Agreement, Part 12, 1995.

Rec. M.3100:1992 & 1995 draft, Generic Network Information Model.

Rec. X.701 | ISO/IEC 10040:1992, Information Technology - Open System Interconnection - Common Management Overview.

Rec. X.710 | ISO/IEC 9595:1990, Information Technology - Open System Interconnection - Common Management Information Service Definitions.

Rec. X.711 | ISO/IEC 9596-1:1991, Information Technology - Open System Interconnection - Common Management Information Protocol - Part 1: Specification.

Rec. X.720 | ISO/IEC 10165-1:1991, Information Technology - Open System Interconnection - Structure of Management Information - Part 1 Management Information Model.

Rec. X.721 | ISO/IEC 10165-2:1992, Information Technology - Open System Interconnection - Structure of Management Information:  Guidelines for the Definition of Managed Objects.

Rec. X.722 | ISO/IEC 10165-4:1992, Information Technology - Open System Interconnection - Structure of Management Information:  Guidelines for the Definition of Managed Objects.

Rec. X.730 | ISO/10164-1:1992, Information Technology - Open System Interconnection - System Management - Part 1:  Object Management Function.

Rec. X.734 | ISO/10164-5:1992, Information Technology - Open System Interconnection - System Management - Part 5:  Event Report Management Function.

Rec. X.735 | ISO/10164-6:1992, Information Technology - Open System Interconnection - System Management - Part 6:  Log Control Function.

Rec. X.209:  1988, Specification for Basic Encoding Rules for Abstract Syntax Notation One (ANS.1).

Rec. X.690:  1994, ASN.1 Encoding Rules: Specification of Basic Encoding Rules (BER), Canonical Encoding Rules (CER), and Distinguished Encoding Rules (DER).

Rec. X.208:  1988, Specification of Abstract Syntax Notation One (ASN.1).

Rec. X.680 | ISO/IEC 8824-1:  1994, Information Technology - Abstract Syntax Notation One (ASN.1) - Specification of Basic Notation.

Rec. X.680 Amd.1 | ISO/IEC 8824-1 Amd.1, Information Technology - Abstract Syntax Notation One (ASN.1) - Specification of Basic Notation 1 Amendment 1:  Rules of Extensibility.

ITU-T Recommendations are available from the US Department of Commerce, National Technical Information Service, 5285 Port Royal Road, Springfield, VA 22161.  ISO standard are available from the American National Standards Institute, 11 West 42nd Street, New York, NY 10036.

### Related Publications

Illinois Commerce Commission Number Portability Administration Center and Service Management System Request for Proposal (ICC NPAC/SMS RFP), February 6, 1996.

Lockheed Martin Team Response to the Illinois Commerce Commission Number Portability Administration Center and Management System Request for Proposal, March 18, 1996.

Scoggins, Sophia and Tang, Adrian 1992. Open networking with OSI. Englewood Cliffs, NJ, Prentice-Hall.

Stallings, William 1993. SNMP, SNMPv2, and CMIP, The Practical Guide to Network-Management Standards, Reading Massachusetts, Addison-Wesley.

North American Number Council (NANC) Functional Requirements Specification, Number Portability Administration Center (NPAC), Service Management System (SMS), Version 3.2.1a July 28, 2003.

CTIA Report on Wireless Portability Version 2, July 7, 1998

## Abbreviations/Definitions

| A-PDU | Application Protocol Data Unit |
| --- | --- |
| ASN.1 | Abstract Syntax Notation 1 |
| BER | Basic Encoding Rules |
| CARE | Customer Account Record Exchange |
| Central Time (standard/daylight) | This is the time in the central time zone, which includes daylight savings time. It changes twice a year based on standard time and daylight savings time. The NPAC SMS runs on hardware that uses this time. |
| CER | Canonical Encoding Rules |
| CLASS CME | Custom Local Area Signaling Services Conformance Management Entity |
| CMIP | Common Management Information Protocol |
| CMISE | Common Management Information Service Element |
| CNAM | Caller Id with Name |
| GDMO | Generalized Definitions of Managed Objects |
| DER | Distinguished Encoding Rules |
| DES | Data Encryption Standard |
| FR | Frame Relay |
| IEC | International Electrotechnical Commission |
| ISO | International Organization of Standardization |
| ISVM | Inter-Switch Voice Mail |
| Local Time | The time zone of the local user. Most time representations in the NPAC OP GUI are represented in the user’s local time zone based on the PC’s clock setting. The time zone label is included in time display in the GUI. EST for Eastern Time Zone CST for Central Time Zone MST for Mountain Time Zone PST for Pacific Time Zone |
| LIDB | Line Information Database |
| LNP | Local Number Portability |
| LRN | Location Routing Number |
| LSMS | Local Service Management System |
| LSPP | Local Service Provider Portability |
| MAC | Media Access Control |
| MD5 | Message Digest (Version 5) |
| MIB | Management Information Base |
| NE | Network Element |
| NMF | Network Management Forum |
| NPAC SMS | Number Portability Administration Center and Service Management System |
| NPA | Numbering Plan Area |
| NXX | Exchange |
| OCN OSI | Operating Company Number Open Systems Interconnect |
| PPP | Point-To-Point Protocol |
| RFP | Request for Proposal |
| RSA | Encryption Scheme |
| SOA | Service Order Activation |
| SMS | Service Management System |
| TMN | Telecommunications Management Network |
| TN | Telephone Number |
| WSMSC | Wireless Short Message Service Center |

# Interface Overview

Overview

2

This specification defines the interfaces between the NPAC SMS and the service providers’ Service Order Entry System and Local SMS.  The interfaces, defined using the CMIP protocol, are referred to as the SOA to NPAC SMS interface and the NPAC SMS to Local SMS interface respectively.  CMISE M-CREATE, M-DELETE, M-SET, M-GET, M-EVENT-REPORT, and M-ACTION primitives are fully supported in a confirmed mode. Thus, the sequencing of operations is implied by the receipt of the confirmation or operation response, and NOT by the sequence that the operation request is received. The relationship from the SOA to the NPAC SMS and  from the Local SMS to NPAC SMS is a manager to agent or an agent to manager relationship depending on the function being performed. The SOA and Local SMS interfaces are defined by Association Functions. These functions allow each association to define the services it supports. Association establishment from the SOAs and Local SMSs to the NPAC SMS, Association Function and security for each of these interfaces is discussed in Section 5, Secure Association Establishment.

Note:  The M-CANCEL-GET primitive may not be supported in some NPAC SMS implementations due to the fact that this functionality was not determined necessary for the interface defined.

The sections that follow provide an overview of protocol requirements and a brief description of the functionality provided in each interface.  Complete functional descriptions for the interfaces are provided in the process flow diagrams in Appendix B, Message Flow Diagrams, as well as the behavior for the managed objects.

The interface between the SOA and the NPAC SMS is called the “SOA to NPAC SMS interface”. The interface between the Local SMS and the NPAC SMS is called the “NPAC SMS to Local SMS interface”.  No direction for operations is implied by the names of these interfaces.

All timestamps (GeneralizedTime fields) that are sent over the SOA to NPAC SMS interface and NPAC SMS to Local SMS interface, shall use Greenwich Mean Time (GMT).  The universal time format (YYYYMMDDHHMMSS.0Z) is used. The default value is a non-specific format of 00000000000000.0Z.

## OSI Protocol Support

The SOA to NPAC SMS and NPAC SMS to Local SMS interfaces must be implemented over the protocol stack shown in Exhibit 1.

Exhibit 1. NPAC/SMS Primary Network Protocol Stacks

| Layer | Mechanized Interface | Function |
| --- | --- | --- |
|  | CMIP Agent Server | User |
| 7 | CMISE, ACSE, ROSE | Application |
| 6 | ANSI T1.224 | Presentation |
| 5 | ANSI T1.224 | Session |
| 4 | TCP, RFC1006, TPO | Transport |
| 3 | IP | Network |
| 2 | PPP, MAC, FRAME Relay, ATM (IEEE 802.3) | Link |
| 1 | DS-1, DS-0 x n, ISDN, V.34 | Physical |

Multiple associations per service provider to the NPAC SMS can be supported.  The secure association establishment is described in Section 5.

## SOA to NPAC SMS Interface

The SOA to NPAC SMS interface, which allows communication between a service provider’s Service Provisioning Operating Systems and/or Gateway systems and the NPAC SMS, supports the retrieval and update of subscription, service provider, and network information.  The following transactions occur to support local number portability functionality:

- SOA requests for subscription administration to the NPAC SMS and responses from the NPAC SMS to the SOA.

- Audit requests from the SOA to the NPAC SMS and responses from the NPAC SMS to the SOA.

- Notifications from the NPAC SMS to the SOA of subscription version data and number pool block data changes, needed for concurrence or authorization for number porting, conflict-resolution, cancellation, outage information, customer disconnect dates, or the first use of an NPA-NXX.

- Network data from the NPAC SMS to SOA.

- Service provider data administration from the SOA to the NPAC SMS.

- SOA requests for number pool block administration (creation and modification) to the NPAC SMS and responses from the NPAC SMS to the SOA.

Mapping of this functionality into the CMIP Definitions is provided in Section 4 (see Exhibit 8.) The NPAC SMS currently uses a 32-bit signed integer for the Naming ID Value.  The maximum value is ([2**31] – 1) or 2.14B.  It is anticipated that all Service Providers will be able to successfully handle Naming ID Values up to this maximum.  Additionally, it should be noted that there is NOT any rollover strategy (NANC 147), once this number is attained within the NPAC SMS.

### Subscription Administration

Service provider subscription administration functionality includes the capability to:

- Create a subscription version or range of versions

- Cancel a subscription version

- Acknowledge cancellation of a subscription version

- Modify a subscription version or range of versions

- Retrieve a specific subscription version or range of versions

- Activate a version or range of versions

- Disconnect a subscription version or range of versions

- Place a subscription into conflict

- Remove a subscription version from conflict

### Audit Requests

Audit functionality allows the SOAs to request audits for a subscription version or group of subscription versions based on a Telephone Number (TN) for a specified service provider or all service provider networks. The requesting SOA receives discrepancy reports as they are found in the network. Upon audit completion it receives a notification of the success or failure of the audit and the total number of discrepancies found.

### Notifications

SOAs are sent notifications to ensure that they are fully informed of relevant events for their subscriptions.  Notification of creation, deletion, or data value changes for subscription versions will be sent to the SOA as they occur. Notification will be sent to the SOA if the service provider has not authorized transfer of service for a TN in the amount of time specified in the “Service Provider Concurrence Interval” defined on the NPAC. This notification will indicate to the service provider that authorization is needed for the pending subscription version. If the service provider has not acknowledged version cancellation within a timeframe specified by the NPAC SMS, notifications will be sent requesting cancellation acknowledgment. The donor service provider SOA is notified of the customer’s disconnect date. SOA systems are also sent notifications to ensure they are aware of planned down time in the NPAC SMS. Notification of data value changes and object creations are sent for number pool block objects.

First usage notifications are also sent to the SOA when the first use of an NPA-NXX occurs from a subscription version or number pool block creation.

Each SOA notification is assigned a priority of  high, medium, low or none. The category of none indicates that a Service Provider does not want to receive a particular notification. Notifications are then sent in order of priority from high to low.

SOA Service Providers can receive single or range versions of some notifications. If the service provider’s TN Range Notification Indicator is turned OFF in their service provider profile on the NPAC SMS, the following notifications will be sent:

Attribute Value Change for subscriptionVersionNPAC objects

Object Creation for subscriptionVersionNPAC objects

subscriptionVersionCancellationAcknowledgeRequest

subscriptionVersionDonorSP-CustomerDisconnectDate

subscriptionVersionNewSP-CreateRequest

subscriptionVersionNewSP-FinalCreateWindowExpiration

subscriptionVersionOldSP-ConcurrenceRequest

subscriptionVersionOldSPFinalConcurrenceWindowExpiration

subscriptionVersionStatusAttributeValueChange

If the service provider’s TN Range Notification Indicator is turned ON, the following notifications will be sent:

subscriptionVersionRangeAttributeValueChange for subscriptionVersionNPAC objects

subscriptionVersionRangeCancellationAcknowledgeRequest

subscriptionVersionRangeDonorSP-CustomerDisconnectDate

subscriptionVersionRangeNewSP-FinalCreateWindowExpiration

subscriptionVersionRangeNewSP-CreateRequest

subscriptionVersionRangeObjectCreation for subscriptionVersionNPAC objects

subscriptionVersionRangeOldSP-ConcurrenceRequest

subscriptionVersionRangeOldSPFinalConcurrenceWindowExpiration

subscriptionVersionRangeStatusAttributeValueChange

Notifications can be recovered by the SOA from the NPAC SMS.  Notifications to be recovered are requested by time range and are recovered in the order the NPAC SMS attempted to sent them.

### Service Provider Data Administration

Service providers can use, read, and update their service provider information on the NPAC SMS using the SOA.  Service providers can update information in the service provider profile as well as add and delete their own network data.  Changes to network data that result in mass updates are prevented from the SOA to the NPAC.  Mass changes must be initiated by the service provider contacting the NPAC personnel directly.

### Network Data Download

When network data (NPA-NXX, NPA-NXX-X , Service Provider, or LRN data for service providers) is created, modified, or deleted on the NPAC SMS, the data is automatically downloaded from the NPAC SMS to the SOA.  The SOA may request that data be recovered using a recovery request that is sent from the SOA to the NPAC SMS.  The SOA then receives the data to be recovered in the request response.  Network data to be recovered can be requested based on a time range, service provider or all service providers, an NPA-NXX range or all NPA-NXX data, an NPA-NXX-X range or all NPA-NXX-X data, an LRN range or all LRN data, or all network data can be requested.  If all network data is specified and the “NPAC Customer SOA NPA-NXX-X Indicator” has been set to TRUE in the service provider’s profile on the NPAC SMS, then NPA-NXX-X object data will be included in the recovery response.

Service providers can also directly read data they wish to download from the NPAC SMS MIB.

### Number Pool Block Administration

Number pool blocks are a set of 1000 TNs represented by a 7 digit NPA-NXX-X (i.e. 555-333-1 represents 555-333-1000 through 1999).  Service providers can create and modify the number pool blocks for which they are the block holder.  Service providers can query all number pool block objects.  Only the NPAC Personnel can initiate the removal of a number pool block object.

## NPAC SMS to Local SMS Interface

The NPAC SMS to Local SMS interface is used for communications between a service provider’s Local SMS and the NPAC SMS for support of LNP network element provisioning.  The following transactions occur to support Local Number Portability:

- Subscription version, number pool block and network data from the NPAC SMS to the Local SMS.

- Service provider data administration from the Local SMS to the NPAC SMS.

- Notifications from the NPAC SMS to the Local SMS of planned NPAC SMS outages and the first use of a new NPA-NXX.

Mapping of this functionality into the CMIP Definitions is provided in Section 4 (see Exhibit 8.) The NPAC SMS currently uses a 32-bit signed integer for the Naming ID Value.  The maximum value is ([2**31] – 1) or 2.14B.  It is anticipated that all Service Providers will be able to successfully handle Naming ID Values up to this maximum.  Additionally, it should be noted that there is NOT any rollover strategy (NANC 147), once this number is attained within the NPAC SMS.

### Subscription Version, Number Pool Block and Network Data Download

When network data (NPA-NXX, NPA-NXX-X or LRN data for service providers) or subscription data or number pool block data is created, modified, or deleted on the NPAC SMS, the data is automatically downloaded from the NPAC SMS to the Local SMS.  The Local SMS may request that data be recovered using a recovery request that is sent from the Local SMS to the NPAC SMS.  The Local SMS then receives the data to be recovered in the request response.  Subscription data to be recovered can be requested based on time range, a TN, or a TN range.  If the “NPAC Customer LSMS EDR Indicator” is set to TRUE in the service provider’s profile on the NPAC SMS, only number pool block data will be sent and no subscription versions with LNP type set to ‘pool’ will be sent.  Number pool block data to be recovered can be requested by time-range, NPA-NXX-X or NPA-NXX-X range.  Network data to be recovered can be requested based on a time range, service provider or all service providers, an NPA-NXX range or all NPA-NXX data, an NPA-NXX-X range or all NPA-NXX-X data, an LRN range or all LRN data, or all network data can be requested.  If all network data is specified and the “NPAC Customer LSMS NPA-NXX-X Indicator” has been set to TRUE in the service provider’s profile on the NPAC SMS, then NPA-NXX-X object data will be included in the recovery response.

Service providers can also directly read data they wish to download from the NPAC SMS MIB.

### Service Provider Data Administration

Service providers can use, read, and update their service provider information on the NPAC SMS using the Local SMS to NPAC SMS interface.  Service providers can update information in the service provider profile as well as add and delete their own network data.  Changes to network data that result in mass updates are prevented by the NPAC SMS to Local SMS interface.  Mass changes must be initiated by the service provider contacting the NPAC personnel directly.

### Notifications

Local SMSs are sent notifications to ensure they are aware of planned down time in the NPAC SMS. Local SMSs are also sent notifications when a new NPA-NXX is to be used for the first time in a subscription version or number pool block by a serviceProvNPA-NXX-X creation.

Notifications can be recovered by the Local SMS  from the NPAC SMS.  Notifications to be recovered are requested by time range.

# Hierarchy Diagrams

Overview

3

The following five exhibits show the class hierarchy diagram for all managed objects (Exhibit 2), Log Record Objects (Exhibit 3), the Local SMS (Exhibit 4), the NPAC SMS naming hierarchies for the Local SMS (Exhibit 5), the SOA (Exhibit 6.), and the NPAC SMS naming hierarchies for the SOA. (Exhibit 7).  These exhibits will help the user gain a better understanding of the structure of the interface definitions provided.

### Managed Object Model Inheritance Hierarchy

The Managed Object Model Inheritance Hierarchy shows the inheritance hierarchy used for object definitions in the NPAC SMS to Local SMS and the SOA to NPAC SMS interfaces.

Exhibit 2. The Managed Object Model Inheritance Hierarchy

lnp

Audits

lnpSOA

number

PoolBlock

number

PoolBlockNPAC

service

ProvNPA-NXX-X

### Log Record Managed Object Hierarchy

The Log Record Managed Object Hierarchy shows the inheritance hierarchy of the log records used in the NPAC SMS to Local SMS and SOA to NPAC SMS interfaces.

Exhibit 3 . Log Record Managed Object Hierarchy

### NPAC SMS to Local SMS Naming Hierarchy for the NPAC SMS

The NPAC SMS to Local SMS Naming Hierarchy for the NPAC SMS shows the naming hierarchy used in the NPAC SMS to instantiate objects defined in the NPAC SMS to Local SMS interface.

Shaded objects are instantiated at NPAC SMS start-up and are not created via M-CREATE or M-DELETE requests. All other objects are created at start-up from a persistent object store on the NPAC SMS or from actions taken while the NPAC SMS is running.

Each object class belongs to one or more Association Functions.
Refer to Section 5.2.1.8, Association Functions.

Exhibit 4. The NPAC SMS to Local SMS Naming Hierarchy for the NPAC SMS.

root

NPAC SMS TO LOCAL SMS

NAMING HIERARCHY FOR THE NPAC SMS

InpNPAC-SMS

InpServiceProvs

serviceProv

InpNetwork

serviceProvNetwork

serviceProvNPA-NXX

serviceProvLRN

lsmsFilterNPA-NXX

InpSubscriptions

subscriptionVersionNPAC

numberPoolBlockNPAC

serviceProvNPA-NXX-X

### NPAC SMS to Local SMS Naming Hierarchy for the Local SMS

The NPAC SMS to Local SMS Naming Hierarchy for Local SMS shows the naming hierarchy used in the Local SMS to instantiate objects defined in the NPAC SMS to Local SMS interface.

Shaded objects are instantiated at Local SMS start-up and are not created via M-CREATE or M-DELETE requests. All other objects are created at start-up from a persistent object store on the Local SMS or from actions taken while the Local SMS is running.

Each object class belongs to one or more Association Functions.
Refer to Section 5.2.1.8, Association Functions.

Exhibit 5. The NPAC SMS to Local SMS Naming Hierarchy for the Local SMS.

NPAC SMS TO LOCAL SMS

NAMING HIERARCHY FOR THE LOCAL SMS

root

InpLocalSMS

InpSubscriptions

InpNetwork

serviceProvNetwork

serviceProvNPA-NXX

serviceProvLRN

serviceProvNPA-NXX-X

numberPoolBlock

subscriptionVersion

### SOA to NPAC SMS Naming Hierarchy for the NPAC SMS

The SOA to NPAC SMS Naming Hierarchy for the NPAC SMS shows the naming hierarchy used in the NPAC SMS to instantiate objects defined in the SOA to NPAC SMS interface.

Shaded objects are instantiated at NPAC SMS start-up and are not created via M-CREATE or M-DELETE requests. All other objects are created at start-up from a persistent object store on the NPAC SMS or from actions taken while the NPAC SMS is running.

Each object class belongs to one or more Association Functions.
Refer to Section 5.2.1.8, Association Functions.

Exhibit 6. The SOA to NPAC SMS Naming Hierarchy for the NPAC SMS.

root

SOA TO NPAC SMS

NAMING HIERARCHY FOR THE NPAC SMS

InpNPAC-SMS

InpAudits

subscriptionAudit

InpSubscriptions

subscriptionVersionNPAC

serviceProv

lnpServiceProvs

serviceProvNPA-NXX

serviceProvLRN

lnpNetwork

lsmsFilterNPA-NXX

serviceProvNetwork

numberPoolBlockNPAC

serviceProvNPA-NXX-X

### NPAC SMS to SOA Naming Hierarchy for the SOA

The NPAC SMS to SOA Naming Hierarchy for SOA shows the naming hierarchy used in the SOA to instantiate objects defined in the SOA to NPAC SMS interface.

Shaded objects are instantiated at SOA start-up and are not created via M-CREATE or M-DELETE requests. All other objects are created at start-up from a persistent object store on the SOA or from actions taken while the SOA is running.

Each object class belongs to one or more Association Functions.
Refer to Section 5.2.1.8, Association Functions.

Exhibit 7.  NPA SMS to SOA Naming Hierarchy for the SOA.

NPAC SMS TO SOA

NAMING HIERARCHY FOR THE SOA

root

lnpSOA

InpNetwork

serviceProvNetwork

serviceProvNPA-NXX

serviceProvLRN

serviceProvNPA-NXX-X

# Interface Functionality to CMIP Definition Mapping

4

## Overview

The following tables, Exhibits 8-12, contain the mapping of the interface functionality to managed objects, attributes, actions, and notifications.

### Primary NPAC Mechanized Interface Operations

The primary interface functions in support of the NPAC requirements are described in the table below, as well as their corresponding Common Management Information Exchange (CMISE) operation and referenced object type for that operation.  This table does not include miscellaneous operations, such as service provider network data querying or downloading, etc.  These functions are described in the object behaviors in the GDMO source below.

Exhibit 8. Primary NPAC Mechanized Interface Operations Table

| Function | Direction (To/From) | CMIP Operation | Referenced Object Type |
| --- | --- | --- | --- |
| Abort/Cancel Audit Request | from SOA | M-DELETE | subscriptionAudit |
| Audit Complete | to SOA | M-EVENT-REPORT: subscriptionAuditResults | subscriptionAudit |
| Audit Discrepancy | to SOA | M-EVENT-REPORT: subscriptionAuditDiscrepancyRpt | subscriptionAudit |
| Audit Query | from SOA | M-GET | subscriptionAudit |
| Audit Request SOA | from SOA | M-CREATE | subscriptionAudit |
| Cancellation Acknowledge-ment | from SOA (new service provider) | M-ACTION: subscriptionVersionNewSP- CancellationAcknowledge | lnpSubscriptions |
| Cancellation Acknowledg-ment | from SOA (old service provider) | M-ACTION: subscriptionVersionOldSP-CancellationAcknowledge | lnpSubscriptions |
| Conflict Removal | from SOA (new service provider) | M-ACTION: subscriptionVersionRemoveFromConflict | lnpSubscriptions |
| Customer Disconnect Date | to SOA | M-EVENT-REPORT: subscriptionVersionDonorSP-CustomerDisconnectDate or subscriptionVersionRangeDonorSP-CustomerDisconnectDate | subscriptionVersionNPAC or lnpSubscriptions |
| Final Request for Version Create | to SOA (old service provider) | M-EVENT-REPORT: subscriptionVersionOldSP-FinalConcurrenceWindowExpiration or subscriptionVersionRangeOldSPFinalConcurrenceWindowExpiration | subscriptionVersionNPAC or lnpSubscriptions |
| LSMS Filter NPA-NXX Create | from LOCAL SMS or from SOA | M-CREATE | lsmsFilterNPA-NXX |
| LSMS Filter NPA-NXX Delete | from LOCAL SMS or from SOA | M-DELETE | lsmsFilterNPA-NXX |
| LSMS Filter NPA-NXX Query | from LOCAL SMS or from SOA | M-GET | lsmsFilterNPA-NXX |
| Network Data Download | from LOCAL SMS or from SOA | M-ACTION: lnpDownload or M-GET: scoped and filtered for intended serviceProvLRN, serviceProvNPA-NXX serviceProvNPA-NXX-X, service provider attributes | lnpNetwork |
| Network Data Update | from LOCAL SMS or from SOA | M-CREATE | serviceProvLRN, serviceProvNPA-NXX |
| NPA-NXX-X Create | to LOCAL SMS or to SOA | M-CREATE; | serviceProvNPA-NXX-X |
| NPA-NXX-X Delete | to LOCAL SMS or to SOA | M-DELETE | serviceProvNPA-NXX-X |
| NPA-NXX-X Modify | to LOCAL SMS or to SOA | M-SET | serviceProvNPA-NXX-X |
| New NPA-NXX | to LOCAL SMS or to SOA | M-EVENT-REPORT: subscriptionVersionNewNPA-NXX | SubscriptionVersionNPAC lnpNPAC-SMS |
| Number Pool Block Change Notification | to SOA | M-EVENT-REPORT attributeValueChange Notification or numberPoolBlockStatusAttributeValueChange Notification | numberPoolBlockNPAC |
| Number Pool Block Create | from SOA | M-ACTION: numberPoolBlock-Create | lnpSubscriptions |
| Number Pool Block Create | to LOCAL SMS | M-CREATE: for a single numberPoolBlock | numberPoolBlock |
| Number Pool Block Modify | from SOA | M-SET: to a single numberPoolBlock | numberPoolBlockNPAC or lnpSubscriptions |
| Number Pool Block Modify | to LOCAL SMS | MSET: to a single numberPoolBlock or scoped and filtered by NPA-NXX-X range for mass update | numberPoolBlock or lnpSubscriptions |
| Number Pool Block Delete | to LOCAL SMS | M-DELETE: for a single numberPoolBlock | numberPoolBlock |
| Number Pool Block Query | from LOCAL SMS or SOA | M-GET: To a single numberPoolBlockNPAC or scoped and filtered for intended numberPoolBlocks | lnpSubscriptions numberPoolBlockNPAC |
| Number Pool Block Query | to LOCAL SMS | M-GET: scoped and filtered for intended numberPoolBlock | lnpSubscriptions |
| Notification Recovery | from LOCAL SMS or from SOA | M-ACTION: lnpNotificationRecovery | lnpNPAC-SMS |
| Recovery Complete | from LOCAL SMS or from SOA | M-ACTION: lnpRecoveryComplete | lnpNPAC-SMS |
| Request for Cancellation Acknowledg-ment | to SOA | M-EVENT-REPORT: subscription VersionCancellationAcknowledgment Request or subscriptionVersionRangeCancellationAcknowledgeRequest | subscriptionVersionNPAC or lnpSubscriptions |
| Request for Version Create | to SOA (new service provider) | M-EVENT-REPORT: subscriptionVersionNewSP-Create Request or subscriptionVersionRangeNewSP-CreateRequest | subscriptionVersionNPAC |
| Request for Version Create | to SOA (old service provider) | M-EVENT-REPORT: subscriptionVersionOldSP-Concurrence Request or subscriptionVersionRangeOldSP-ConcurrenceRequest | subscriptionVersionNPAC or lnpSubscriptions |
| Service Provider Network Creation | to LOCAL SMS or to SOA | M-CREATE | serviceProvNetwork |
| Service Provider Network Deletion | to LOCAL SMS or to SOA | M-DELETE | serviceProvNetwork |
| Service Provider Network Service Provider Name Change | to LOCAL SMS or to SOA | M-SET: serviceProvName | serviceProvNetwork |
| Subscription Version Activate | from SOA | M-ACTION: subscriptionVersionActivate | lnpSubscriptions |
| Subscription Version Cancel | from SOA | M-ACTION subscriptionVersionCancel | lnpSubscriptions |
| Subscription Version Change Notification | to SOA | M-EVENT-REPORT: attributeValueChangeNotification and subscriptionVersionStatusAttributeValue Change or subscriptionVersionRangeAttribute ValueChange subscriptionVersionRangeStatusAttribute ValueChange | subscriptionVersionNPAC or lnpSubscriptions |
| Subscription Version Conflict | from SOA (old service provider) | M-ACTION: subscriptionVersionOldSP-Create setting subscriptionOldSP-Authorization = FALSE | subscriptionVersion |
| Subscription Version Create | to LOCAL SMS | M-ACTION: subscriptionVersionLocalSMS-Create for multiple creates (i.e., range operations) where the data in the subscription versions is the same M-CREATE: for an individual subscriptionVersion | lnpSubscriptions subscriptionVersion |
| Subscription Version Create | from SOA | M-ACTION: subscriptionVersionOldSP-Create or subscriptionVersionNewSP-Create | lnpSubscriptions |
| Subscription Version Delete | to LOCAL SMS | M-DELETE: scoped and filtered for intended subscriptionVersion criteria | subscriptionVersion |
| Subscription Version Disconnect | from SOA | M-ACTION: subscriptionVersionDisconnect | lnpSubscriptions |
| Subscription Version Download | to LOCAL SMS | M-ACTION: subscriptionVersionLocalSMS-Create or M-CREATE: for an individual subscriptionVersion | lnpSubscriptions |
| Subscription Version Download Request | from LOCAL SMS | M-ACTION: lnpDownload or M-GET: scoped and filtered for intended subscriptionVersionNPAC criteria | lnpSubscriptions |
| Subscription Version Modify | from SOA | M-ACTION: subscriptionVersion Modify or M-SET: on relevant subscriptionVersionNPAC attributes for pending and conflict versions | lnpSubscriptions |
| Subscription Version Modify | to LOCAL SMS | M-SET: scoped and filtered for intended subscriptionVersion criteria setting relevant attributes | lnpSubscriptions |
| Subscription Version Query | from SOA from LOCAL SMS | M-GET: scoped and filtered for intended subscriptionVersionNPAC criteria setting relevant attributes | lnpSubscriptions |
| Subscription Version Query | to LOCAL SMS | M-GET: scoped and filtered for intended subscriptionVersion criteria | lnpSubscriptions |

### Managed Object Interface Functionality

The table below contains the mapping of the SOA to NPAC SMS and the Local SMS to NPAC SMS managed objects to the interface functionality.

Exhibit 9. Managed Object Interface Functionality Table

| Managed Object Name | Interface Functionality Mapping |
| --- | --- |
| lnpAudits | Container object used to contain all subscription audit objects on the NPAC SMS and the Local SMS. It is used in the SOA to NPAC SMS interface to support audit functionality. |
| lnpLocal SMS | Container object used to contain all objects on a Local SMS. It is used in the NPAC SMS to Local SMS interface to support NPAC SMS communication to the service provider Local SMS system. |
| lnpLogAudit-DiscrepancyRptRecord | Object used to log information from a subscriptionAudit-DiscrepancyRpt notification. |
| lnpLogAuditResultsRecord | Object used to log information from a subscriptionAuditResults notification. |
| lnpLogCancellation AcknowledgeRequest Record | Object used to log information from a subscriptionVersionCancellationAcknowledgeRequest notification. |
| lnpLogDonorSP-CustomerDisconnectDate Record | Object used to log information from a subscriptionVersionDonorSP-CustomerDisconnectDate notification. |
| lnpLogLocalSMS-ActionResultsRecord | Object used to log information from a subscriptionVersionLocalSMS-ActionResults notification. |
| lnpLogNewNPA-NXXRecord | Object used to log information from a subscriptionVersionNewNPA-NXX notification. |
| lnpLogNewSP-CreateRequestRecord | Object used to log information from a subscriptionVersionNewSP-CreateRequest notification. |
| LnpLogNumberPoolBlockStatusAttributeValueChangeRecord | Object used to log information from a numberPoolBlockStatusAttributeValueChange notification. |
| lnpLogOldSP-ConcurrenceRequestRecord | Object used to log information from a subscriptionVersionOldSP-ConcurrenceRequest notification. |
| lnpLogOldSP- FinalConcurrenceWindow- Expiration | Object used to log information from a subscriptionVersionOldSP-FinalConcurrenceWindowExpiration notification |
| lnpLogOperational-InformationRecord | Object used to log information from a lnpNPAC-SMS-Operational-Information notification. |
| lnpLogRangeAttributeValueChangeRecord | Object used to log information from a lnpLogRangeAttributeValueChange notification. |
| lnpLogRangeObjectCreationRecord | Object used to log information from a lnpLogRangeObjectCreation notification. |
| lnpLogRangeStatusAttributeValueChangeRecord | Object used to log information from a lnpLogRangeStatusAttributeValueChange notification. |
| lnpLogRangeDonorSP-CustomerDisconnectDateRecord | Object used to log information from a lnpLogRangeDonorSP-CustomerDisconnectDate notification. |
| lnpLogRangeCancellationAcknowledgeRecord | Object used to log information from a lnpLogRangeCancellationAcknowledge notification. |
| lnpLogRangeNewSP-CreateRequestRecord | Object used to log information from a lnpLogRangeNewSP-CreateRequest notification. |
| lnpLogRangeNewSP-FinalCreateWindowExpirationRecord | Object used to log information from a lnpLogRangeNewSP-FinalCreateWindowExpiration notification. |
| lnpLogNewSP-FinalCreateWindowExpirationRecord | Object used to log information from a lnpLogNewSP-FinalCreateWindowExpiration notification. |
| lnpLogRangeOldSP-ConcurrenceRequestRecord | Object used to log information from a lnpLogRangeOldSP-ConcurrenceRequest notification. |
| lnpLogRangeOldSPFinalConcurrenceWindowExpirationRecord | Object used to log information from a lnpLogRangeOldSPFinalConcurrenceWindowExpiration notification. |
| lnpLogStatusAttributeValueChangeRecord | Object used to log information from a subscriptionVersionStatusAttributeValueChange notification. |
| lnpNetwork | Container object used to contain all service provider network data on the NPAC SMS, SOA, and Local SMS. It is used in the NPAC SMS to Local SMS and SOA to NPAC SMS interfaces to support downloading of network data to the Local SMS and/or SOA and the functionality that allows service providers to create/delete their network data on the NPAC SMS. |
| lnpNPAC-SMS | Container object used to contain all objects on a NPAC SMS. It is used in the NPAC SMS to Local SMS and SOA to NPAC SMS interfaces to support NPAC SMS communication from the service provider Local SMS and the SOA systems. |
| lnpServiceProvs | Container object used to contain all service provider data on the NPAC SMS. It is used in the NPAC SMS to Local SMS interface and SOA to NPAC SMS interface to support retrieving of service provider data by the Local SMS and/or SOA and the functionality that allows service providers to update their service provider data on the NPAC SMS. Service providers can only retrieve their service provider data. |
| lnpSOA | Container object used to contain all objects on a SOA It is used in the SOA to NPAC SMS interface to support NPAC SMS communication to the service provider SOA system. |
| lnpSubscriptions | Container object used to contain all subscription versions and number pool blocks on the NPAC SMS and the Local SMS. It is used in the NPAC SMS to Local SMS and SOA to NPAC SMS interfaces to support query of subscription and number pool block data on the NPAC SMS and downloading of subscription and number pool block data to the Local SMS. |
| lsmsFilterNPA-NXX | Object used to represent the NPA-NXX values for which a service provider does not want to be informed of subscription version broadcasts. |
| numberPoolBlock | Object used to represent a number pool block on the Local SMS. These objects are used to support number pool block download from the NPAC SMS to the EDR Local SMS using the NPAC SMS to Local SMS interface. Local SMS may support this object by setting the “NPAC Customer LSMS EDR Indicator” to TRUE in their service provider profile on the NPAC SMS. |
| numberPoolBlockNPAC | Object used to represent a number pool block on the NPAC SMS. These objects are used to support number pool block administration from the SOA using the SOA to NPAC SMS interface. Capability is provided to the SOA for creation and modification. The NPAC SMS can create, modify and delete. |
| serviceProv | Object used to represent a service provider and its associated data on the NPAC SMS. These objects are used in the NPAC SMS to Local SMS and SOA to NPAC SMS interfaces to support retrieving of service provider data and the functionality that allows service providers to update their service provider data on the NPAC SMS except serviceProvId and serviceProvType. Service providers can only retrieve their service provider data. |
| serviceProvLRN | Object used to represent an LRN associated with a service provider on the NPAC SMS, SOA, or Local SMS. These objects are used to support downloading of network LRN data to the Local SMS and/or SOA and the functionality that allows service providers to create/delete their own network LRN data. The service provider will have to add a new object and delete the old one to modify the data. |
| serviceProvNetwork | Container object used to contain network data for a service provider on the NPAC SMS, SOA or Local SMS. It is used in the NPAC SMS to Local SMS and SOA to NPAC SMS interfaces to support downloading of network data to the Local SMS and the functionality that allows service providers to update their network data on the NPAC SMS. |
| serviceProvNPA-NXX | Object used to represent an NPA-NXX associated with a service provider on the NPAC SMS, SOA or Local SMS. These objects are used to support downloading of network NPA-NXX data to the Local SMS and/or SOA and the functionality that allows service providers to create/delete their own network NPA-NXX data. NPA splits are supported only through direct contact with NPAC personnel. |
| serviceProvNPA-NXX-X | Object used to represent an NPA-NXX-X associated with a service provider on the NPAC SMS, SOA or Local SMS. These objects are used in number pooling to support downloading of network NPA-NXX-X data to the Local SMS or SOA. Only the NPAC SMS is allowed to create, delete and modify a service provider’s NPA-NXX-X data. Local SMS may support this object by setting the “NPAC Customer LSMS NPA-NXX-X Indicator” to TRUE in their service provider profile on the NPAC SMS. SOA may support this object by setting the “NPAC Customer SOA NPA-NXX-X Indicator” to TRUE in their service provider profile on the NPAC SMS. |
| subscriptionAudit | Object used to represent a subscription audit request on the NPAC SMS. These objects are used to support subscription audit requests from the SOA to the NPAC SMS using the SOA to NPAC SMS interface. The object supports notifications for audit discrepancies found and audit completion results. If the subscription version LNP type is equal to ‘pool’, the appropriate number pool block will also be audited. |
| subscriptionVersion | Object used to represent a subscription version on the Local SMS. These objects are used to support subscription version download from the NPAC SMS to the Local SMS using the NPAC SMS to Local SMS interface |
| subscriptionVersionNPAC | Object used to represent a subscription version on the NPAC SMS. These objects are used to support subscription administration from the SOA using the SOA to NPAC SMS interface. Capability is provided for version creation, activation, modification, cancellation, disconnect, and query. |

### Action Interface Functionality

The table below contains the mapping of the SOA to NPAC SMS and the Local SMS to NPAC SMS actions to the interface functionality.

Exhibit 10. The Action Interface Functionality Table

| Action Name | Interface Requirements Mapping |
| --- | --- |
| lnpDownload | This action is used to support the downloading of subscription, number pool block and network data to the Local SMS from the NPAC SMS. It also supports the downloading of network data to the SOA from the NPAC SMS. |
| lnpRecoveryComplete | This action is used to specify the system has recovered from down time and the transactions performed since the association establishment can now be sent to the Local SMS from the NPAC SMS using the Local SMS to NPAC SMS interface or the SOA from the NPAC SMS using the SOA to NPAC SMS interface. |
| numberPoolBlock-Create | This action is used to support creation of the number pool block object by the block holder service provider from the SOA to the NPAC SMS using the SOA to NPAC SMS interface. |
| subscriptionVersionActivate | This action is used to support subscription version activation by the new service provider from the SOA to the NPAC SMS using the SOA to NPAC SMS interface. |
| subscriptionVersionCancel | This action is used to support subscription version cancellation by a service provider from the SOA to the NPAC SMS using the SOA to NPAC SMS interface. |
| subscriptionVersionDisconnect | This action is used to support subscription version disconnection by the current service provider from the SOA to the NPAC SMS using the SOA to NPAC SMS interface. |
| subscriptionVersionLocalSMS-Create | This action can be used by the NPAC SMS to create multiple subscription versions via the Local SMS to NPAC SMS interface. |
| subscriptionVersionModify | This action is used to support subscription version modification by a service provider from the SOA to the NPAC SMS using the SOA to NPAC SMS interface. |
| subscriptionVersionNewSP-CancellationAcknowledge | This action is used to support the acknowledgment of subscription versions with a status of cancel-pending by the old service provider from the SOA to the NPAC SMS using the SOA to NPAC SMS interface. |
| subscriptionVersionNewSP-Create | This action is used to support subscription version creation by the new service provider from the SOA to the NPAC SMS using the SOA to NPAC SMS interface. |
| subscriptionVersionOldSP-CancellationAcknowledge | This action is used to support the acknowledgment of subscription versions with a status of cancel-pending by the old service provider from the SOA to the NPAC SMS using the SOA to NPAC SMS interface. |
| subscriptionVersionOldSP-Create | This action is used to support subscription version creation by the old service provider from the SOA to the NPAC SMS using the SOA to NPAC SMS interface. |
| subscriptionVersion RemoveFromConflict | This action is used on the NPAC SMS via the SOA to NPAC SMS interface to set the subscription version status from conflict to pending. |
| lnpNotificationRecovery | This action is used on the NPAC SMS via the SOA to NPAC SMS or Local SMS to NPAC SMS interface to recover notifications. |

### Notification Interface Functionality

The table below contains the mapping of the SOA to NPAC SMS and the Local SMS to NPAC SMS notifications to the interface functionality.

Exhibit 11. The Notification Interface Functionality Table

| Notification Name | Interface Requirements Mapping |
| --- | --- |
| LnpNPAC-SMS-Operational-Information | This notification is used to support the reporting of NPAC SMS scheduled down time. This notification can be issued from the lnpNPAC-SMS object on the NPAC SMS to a SOA via the SOA to NPAC SMS interface or from the NPAC SMS to the Local SMS via the NPAC SMS to Local SMS interface. |
| numberPoolBlockStatusAttributeValueChange | This notification is issued when the number pool block status is modified and can contain the number pool block status and failed service provider list. This notification is issued over the NPAC SMS to SOA interface from the numberPoolBlockNPAC object. |
| subscriptionAudit-DiscrepancyRpt | This notification is used to support the reporting of audit discrepancies found during audit processing. This notification can be issued from an audit object on the NPAC SMS to a SOA via the SOA to NPAC SMS interface. |
| subscriptionAudit-Results | This notification is used to support the reporting of audit processing results. This notification can be issued from an audit object on the NPAC SMS to a SOA via the SOA to NPAC SMS interface. |
| subscriptionVersionCancellationAcknowledgeRequest or subscriptionVersionRangeNewSP-CancellationAcknowledge | This notification is issued to new and old service providers to request that a cancellation acknowledgment be sent for a subscription version in a cancel-pending state. This notification is issued via the SOA to NPAC SMS interface if the service provider fails to acknowledge the cancellation after a tunable amount of time specified in the NPAC SMS. The NPAC SMS sends the appropriate notification depending upon the Service Provider's TN Range Notification Indicator. |
| subscriptionVersionDonorSP-CustomerDisconnectDate or subscriptionVersionRangeDonorSP-CustomerDisconnectDate | This notification informs the donor service provider SOA that a subscription version is being disconnected. This notification is issued from the NPAC SMS to a SOA via the SOA to NPAC SMS interface. The NPAC SMS sends the appropriate notification depending upon the Service Provider's TN Range Notification Indicator. |
| subscriptionVersionLocalSMS-ActionResults | This notification contains the results of a subscriptionVersionLocalSMS-Create action once all the create requests have been attempted. It is issued from the Local SMS to the NPAC SMS via the NPAC SMS to Local SMS interface. |
| subscriptionVersionNew-NPA-NXX | This notification informs the Local SMS or SOA of a pending subscription version or new number pool block involving the first use of an NPA-NXX. |
| subscriptionVersionNewSP-CreateRequest or subscriptionVersionRangeNewSP-CreateRequest | This notification is issued to the new service provider to request that a create request be sent for the subscription version created by the old service provider to provide authorization and/or porting information. This notification is issued via the SOA to NPAC SMS interface if the new service provider failed to authorize porting of a number after a tunable amount of time specified in the NPAC SMS. The NPAC SMS sends the appropriate notification depending upon the Service Provider's TN Range Notification Indicator. |
| subscriptionVersionNewSPFinalCreateWindow Expiration or subscriptionVersionRangeNewSPFinalCreateWindowExpiration | This notification is issued to the new and old service provider, if they support the Final Create Window Expiration Notification in their Service Provider profile, to inform them of the expiration of the Final Concurrence Window on the NPAC SMS. This notification is issued from the NPAC SMS to the SOA via the SOA to NPAC SMS interface. The NPAC SMS sends the appropriate notification depending upon the Service Provider's TN Range Notification Indicator. |
| subscriptionVersionOldSP-ConcurrenceRequest or subscriptionVersionRangeOldSP-ConcurrenceRequest | This notification is issued to the old service provider to request that a create request be sent for the subscription version created by the new service provider to provide concurrence for porting. This notification is issued via the SOA to NPAC SMS interface if the old service provider failed to authorize porting of a number after a tunable amount of time specified in the NPAC SMS. The NPAC SMS sends the appropriate notification depending upon the Service Provider's TN Range Notification Indicator. |
| subscriptionVersionStatusAttributeValueChange or subscriptionVersionRangeStatusAttributeValue Change | This notification is issued when the subscription version status is modified. This notification is issued from the NPAC SMS to the SOA via the SOA to NPAC SMS interface. The NPAC SMS sends the appropriate notification depending upon the Service Provider's TN Range Notification Indicator. |
| subscriptionVersionOldSPFinalConcurrenceWindow Expiration or subscriptionVersionRangeOldSPFinalConcurrence WindowExpiration | This notification is issued to the old service provider to request for a final time that a create request be sent for the subscription version created by the new service provider to provide concurrence for porting. This notification is issued via the SOA to NPAC SMS interface if the old service provider failed to authorize porting of a number after a tunable amount of time. The NPAC SMS sends the appropriate notification depending upon the Service Provider's TN Range Notification Indicator. |
| subscriptionVersionRangeAttributeValueChange | This notification or the Attribute Value Change notification is sent when specified attributes have been updated. This notification is issued via the SOA to NPAC SMS interface. The NPAC SMS sends the appropriate notification depending upon the Service Provider's TN Range Notification Indicator. |
| subscriptionVersionRangeObjectCreation | This notification or the object creation notification is sent when a subscriptionVersionNPAC object has been created. This notification is issued via the SOA to NPAC SMS interface. The NPAC SMS sends the appropriate notification depending upon the Service Provider's TN Range Notification Indicator. |

## Scoping and Filtering Support

The following section defines the scoping and filtering support for both the SOA to NPAC SMS interface and LSMS to NPAC SMS interface.

### Scoping

The NPAC SMS to Local SMS or SOA to NPAC SMS interfaces do not support scoping of CMIP operations of any type by the LSMS or SOA for the following objects:

- root

- lnpLocal-SMS

- lnpNetwork

- any object with an “empty” filter

- NPAC SMS is not required to support Scope other than baseObject Scope for CMIP operations that specify baseManagedObjectClass of one of the following:

- lnpNPAC-SMS

- lnpServiceProvs

Scoped operations for subscriptionVersions or numberPoolBlocks to the LSMS must be supported on the baseObject (level 0) or from the lnpSubscriptions object with a non-empty filter.

The limit in scoping and functionality prevents the NPAC, SOA, and the LSMS systems from having to implement functionality or respond to large requests that are not necessary to support LNP over the mechanized interfaces.

### Filtering

Filtering on the NPAC SMS is supported as defined in the GDMO.  The NPAC SMS requires the Local SMS to support at a minimum the filter criteria specified below.

Limitations:

- OR and NOT filter support is not required for the Local SMS or SOA.

- NOT filter support is not required for the NPAC SMS.

- Filtering requests with a scope will not be issued to the Local SMS or SOA by the NPAC SMS for any object other than the subscriptionVersion and numberPoolBlock objects. No query will be used that requests both subscription versions and number pool blocks at the same time..

- All authorization rules apply to scoped and filtered operations.  For example, a query for data that a service provider is not authorized to view will be failed with a reason of access denied.

- CMISSync is not supported for any scoped/filtered CMIP operation.

The following table shows the CMISE primitive filtering support required of the Local SMS by the NPAC SMS for the subscriptionVersion object.

Exhibit 12 - CMISE Primitive Filtering Support for the Subscription Version Object

| CMISE Primitives | Filter Supported | Notes |
| --- | --- | --- |
| M-ACTION | N | No filtering is applied to the actions for the subscriptionVersion object. |
| M-GET | Y | TN Range with greaterOrEqual, lessOrEqual, equality must be supported for auditing. |
| M-SET | Y | TN Range with greaterOrEqual, lessOrEqual, equality must be supported for Mass Update or TN range modify requests. |
| M-DELETE | Y | TN Range with greaterOrEqual, lessOrEqual, equality will be supported for range disconnect or port to original requests. |

Exhibit 13 CMISE Primitive Filtering Support for the Number Pool Block Object

| CMISE Primitives | Filter Supported | Notes |
| --- | --- | --- |
| M-ACTION | N | No filtering is applied to the actions for the number pool block object. |
| M-GET | Y | NPA-NXX-X Range with greaterOrEqual, lessOrEqual, equality must be supported for auditing. |
| M-SET | Y | NPA-NXX-X Range with greaterOrEqual, lessOrEqual, equality must be supported for Mass Update modify requests. |
| M-DELETE | N | Single request deletes are sent to the number pool block. |

### Action Scoping and Filtering Support

For messages sent to any object, the scope and filter will be checked to ensure it is appropriate for that object class.

- All M-ACTIONs that relate to subscriptions and number pool blocks are targeted to lnpSubscriptions.

- The ONLY filters allowed by the GDMO for lnpSubscriptions are "equality" and "present" for the single attribute lnpSubscriptionsName.

- If any one of the above M-ACTIONs is sent to a subscriptionVerisonNPAC or numberPoolBlockNPAC object you will get a "no such action" error response from that object.

- If you send a scoped/filtered M-ACTION whose scope includes objects of class subscriptionVersionNPAC or numberPoolBlockNPAC, you will receive an error "no such action" from each object specified by the filter.

## lnpLocal-SMS-Name and lnpNPAC-SMS-Name Values

The following table (Exhibit 14) shows the values to be used for all currently identified NPAC regions for lnpNPAC-SMS-Name in the lnpNPAC-SMS object.  The lnpLocal-SMS-Name for the lnpLocal-SMS object will be the service provider ID followed by a dash and the lnpNPA-SMS Name (e.g., 9999-Midwest Regional NPAC SMS).

Exhibit 14 - Defined lnpLocal-SMS-Name and lnpNPAC-SMS-Name Values

| NPAC Customer Ids | NPAC SMS Region | lnpNPAC-SMS-Name |
| --- | --- | --- |
| 0000 | Midwest | Midwest Regional NPAC SMS |
| 0001 | Mid-Atlantic | Mid-Atlantic Regional NPAC SMS |
| 0002 | Northeast | Northeast Regional NPAC SMS |
| 0003 | Southeast | Southeast Regional NPAC SMS |
| 0004 | Southwest | Southwest Regional NPAC SMS |
| 0005 | Western | West Regional NPAC SMS |
| 0006 | West Coast | West Coast Regional NPAC SMS |
| 0007 | Canada | Region8 NPAC Canada |

## OID Usage Information

### OIDs Used for Bind Requests

| Value | OID |
| --- | --- |
| CMIPUserInfo | 2:1:1 (per standards and pp.49 IIS1.5) |
| CMIPAbortInfo | 2:1:1 (per standards and pp.51 IIS1.5) |
| LnpAccessControl | {lnp-attribute 1} = 1:3:6:1:4:1:103:7:0:0:2:1 |
| UserInfo (NpacAssociationInfo) | 1:3:6:1:4:1:103:7:0:0:2:105 |
| Application context | 2:9:0:0:2 (per standards) |

### Other OIDs of Interest

| Value | OID |
| --- | --- |
| AccessControl OID as part of a SMI notification | 1:3:6:1:4:1:103:7:0:0:8:1 |
| AccessControl as part of LNP notifications | {lnp-attribute 1} = 1:3:6:1:4:1:103:7:0:0:2:1 |

## Naming Attributes

Non-zero values are not supported in the auto-instance naming attributes for Local Number Portability objects defined in the IIS.

## Subscription Version M_DELETE Messages

M_DELETE commands are not sent for subscription versions set to old as a result of subsequent porting activity. M_DELETEs for subscription versions are only sent as a result of disconnect or port to original processing.  Local SMS systems are responsible for deletion of the subscription versions in their Local SMS database due to the fact that some LSMS implementations may choose to retain old subscription versions in their database.

## Number Pool Block M_DELETE Messages

M_DELETE commands are not sent for number pool blocks set to old as a result of subsequent porting activity. M_DELETEs for number pool blocks are only sent as a result of de-pool.  Local SMS systems are responsible for deletion of the number pool blocks in their Local SMS database due to the fact that some LSMS implementations may choose to retain old number pool blocks in their database.

# Secure Association Establishment

5

## Overview

This section describes the security, the association management and recovery procedures for the service provider SOAs and Local SMSs to follow, and how error information will be passed between interfaces.

The first section describes the security and authentication procedures used in the NPAC SMS interface. The second section describes the NPAC SMS's behavior and error handling and suggests how a service provider SOA or Local SMS should proceed when establishing an association.

## Security

This section describes the security processes and procedures necessary for service provider SOA systems and Local SMSs to establish a secure association and maintain secure communication with the NPAC SMS.  Security threats to the NPAC SMS include:

- Spoofing - An intruder may masquerade as either the SOA, Local SMS, or NPAC SMS to falsely report information.

- Message Tampering - An intruder may modify, delete, or create messages passed.

- Denial or Disruption of Service - An intruder may cause denial or disruption of service by generating or modifying messages.

- Diversion of Resources - An intruder may generate or modify messages that cause resources to be diverted to unnecessary tasks.

- Slamming - An intruder may generate or modify messages that cause customer’s service to be moved between service providers.

- Security threats are prevented in the NPAC SMS by use of the following methods:

- strong two way authentication at association.

- insuring data integrity by detection of replay, deletion, or modification to a message.

- insuring non-repudiation of data by guaranteeing integrity and supporting data origination authentication for each incoming message.

- implementation of access control and application level security that allows only authorized parties to cause changes to the NPAC SMS database.

### Authentication and Access Control Information

The following access control information definition will be used in the AccessControl field of the association and CMIP PDUs to ensure a secure communication for both the SOA to NPAC SMS interface and the NPAC SMS to Local SMS interface:

LnpAccessControl ::= SEQUENCE { systemId [0] SystemID, systemType [1] SystemType, userId [2] GraphicString60 OPTIONAL, listId [3] INTEGER, keyId [4] INTEGER, cmipDepartureTime [5] GeneralizedTime, sequenceNumber [6] INTEGER (0...4294967295), function [7] AssociationFunction, recoveryMode [8] BOOLEAN signature signature [9] BIT STRING } ServiceProvId ::= GraphicString4 SystemID ::= CHOICE { serviceProvID [0] ServiceProvId, npac-sms [1] GraphicString60 } SystemType ::= ENUMERATED { soa(0), local-sms(1), soa-and-local-sms(2), -- value not supported npac-sms(3) --value is only valid for AccessControl definition } AssociationFunction ::= SEQUENCE { soaUnits [0] SoaUnits, lsmsUnits [1] LSMSUnits } SoaUnits ::= SEQUENCE { soaMgmt [0] NULL OPTIONAL, networkDataMgmt [1] NULL OPTIONAL, dataDownload [2] NULL OPTIONAL }

LSMSUnits ::= SEQUENCE { dataDownload [0] NULL OPTIONAL LSMSUnits ::= SEQUENCE { dataDownload [0] NULL OPTIONAL, networkDataMgmt [1] NULL OPTIONAL, query [2] NULL OPTIONAL }

Exhibit 15. Access Control

#### System Id

The system Id is the unique Id for the system using an interoperable interface and must be specified in the systemId field. For a service provider using the SOA and/or Local SMS interfaces, this is the Service Provider ID. For the NPAC SMS, it is the unique identifier for the regional SMS.

In cases where a service provider is providing SOA services for an associated service provider,  the primary service provider must establish the association with their System Id set to their primary Service Provider ID.  PDUs that are subsequently sent to the NPAC SMS may contain the primary or associated Service Provider Ids of the requesting service provider.  Associated Service Provider Ids are sent in the System Id when actions are being taken on behalf of an associated service provider by the service provider providing SOA services (the primary service provider). The Service Provider ID specified in the access control for PDUs sent after association establishment, whether it's the primary or secondary Service Provider ID, is considered the requesting service provider and all validations will use this Service Provider ID.

#### System Type

The system type that indicates the type of system using the interoperable interface must be specified in the systemType field. The valid types are SOA and/or Local SMS and NPAC SMS.

#### User Id

The user Id of the user of the interface can optionally be specified in the userId field for the SOA interface. This is the 60 character graphics string user identifier for a user on a SOA system. It is not validated on the NPAC SMS, however, it is used for logging purposes.

#### List Id

The list Id must be specified as an integer in the listId field to identify a key list.  This key list is one of the key lists exchanged outside of the interface process that is known to both the NPAC SMS and the Local SMS or SOA system it is communicating with.

NPAC key lists and service provider key lists are to be managed based upon service provider id and presentations layer address (P-selector) of the service provider’s SOA system and/or Local SMS system.  Also, a given service provider id and P-selector value can exist for one or more Network Service Access Points (NSAP).

The NPAC SMS must generate and maintain NPAC key lists based upon the service provider’s  service provider id and P-selector value of the system(s) that support its SOA and LSMS interfaces.  In addition, service providers(SOA systems and Local SMS systems) must also manage the NPAC’s key lists.  Each side of the interface must support multiple NPAC key lists per service provider id and P-selector value.

Service providers (SOA system and Local SMS system) must generate and maintain key lists based upon the service provider’s service provider id and P-selector value of the system(s) that support its SOA and LSMS interfaces.  Furthermore, the NPAC SMS must also manage the service provider’s key lists.  Each side of the interface must support multiple service provider(SOA system and Local SMS) key lists per service provider id and P-selector value.

In cases where a service provider is providing SOA services for an associated service provider, key lists are only exchanged with  the primary service provider using the primary service provider id.

#### Key Id

The key Id of a key in the key list must be specified as an integer in the keyId field.  This uniquely identifies the key in the key list used to create the digital signature. The size of the modulus for the key is variable between 600 and 2048 bits.

Since key lists are to be managed based upon service provider id and the P-selector value of a service provider’s SOA system and/or Local SMS system, keys are to be treated independently at the presentation layer for an association.  By using presentation layer support of a key list, SOA and Local SMS systems can have one key or unique keys to support the SOA and LSMS interfaces.  The following situations are supported:

- If a service provider has one process supporting the SOA and LSMS interface, then the process has one P-selector value supporting both interfaces.  The SOA/Local SMS system would use the same key list and the same key for all associations created for the both the SOA and LSMS interface.  The NPAC SMS would in turn have one NPAC key list and key to support both interfaces.

- If a service provider has two processes supporting the SOA and LSMS interface, then each process would have different P-selector values.  The SOA and Local SMS systems would use separate key lists and keys per interface.  In detail, the SOA system would use a key list and key for all associations involving the SOA interface and the Local SMS system would use a different key list and key for all associations involving the LSMS interface.  The NPAC SMS would also manage separate key lists and keys per the SOA and LSMS interface.  Furthermore, the NPAC SMS would use the same key list and key for all associations within a given interface.

- If a service provider has an SOA system or a Local SMS system that consists of multiple processes, then each processes would have different P-selector values.  Therefore, each process would manage separate key lists and separate keys per process.  The NPAC SMS would also manage separate key lists/keys per process.  For example, if a Local SMS system consists of 2 processes (one process supporting subscription data and the other supporting network/query data), the processes would have separate P-selector values and use separate key lists/keys per association.  The NPAC SMS would also manage separate key lists and keys per process within the LSMS interface.

Note: In cases where a service provider is providing SOA services for an associated service provider, keys are used from primary service provider key lists

If the service provider determines their key is compromised they should change their own private key and list.  If the NPAC determines that their key is compromised then they should change their own private key and list.  The NPAC should not invalidate a service provider’s key and vice versa.  However, should either side of the industry interfaces (SOA and Local SMS interface) change keys, the remote side is expected to mark the previously used key as used (key expiration).  Previously used keys (ListId/KeyId combinations) are considered expired and result in a security violation across the industry interface when re-used.

#### CMIP Departure Time

The CMIP departure time must be specified in GeneralizedTime in the cmipDepartureTime field as the time the PDU departed the sending system.  The universal time format (YYYYMMDDHHMMSS.0Z) is used.  In order to ensure data integrity and no-repudiation the NPAC SMS system must be synchronized to within five minutes of the Local SMS and SOA systems that it communicates.

#### Sequence Number

The sequence number is a 32 bit integer that must be specified in the sequenceNumber field.  It should be specified as zero at association time and incremented by one for every message sent over the association. Once the sequence number reaches 4294967295 the counter will be reset to one for the association. Please note that each sender independently keeps its own counter for the sequence number of messages sent and received. For example, after association is established, a Local SMS could send three messages to the NPAC SMS with sequence numbers 1, 2, and 3 respectively.  The NPAC SMS when sending its first message to the Local SMS would use sequence number 1 not sequence number 4.

#### Association Functions

The Association Function(s) must be specified on the initial association request (AARQ PDU). The following table lists the possible Association Functions that can be specified for each of the Association Request Initiators and the associated bit mask value:

Exhibit 16 Association Functions

| Association Request Initiator Association Function | SOA | Local SMS |
| --- | --- | --- |
| SOA Management (Audit and Subscription Version) Classes: lnpSubscriptions numberPoolBlock numberPoolBlockNPAC subscriptionAudit subscriptionVersion subscriptionVersionNPAC | 0x01 |  |
| Service Provider and Network Data Management Classes: lnpNetwork lnpNPAC-SMS lnpServiceProvs lsmsFilterNPA-NXX serviceProv serviceProvLRN serviceProvNetwork serviceProv-NPA-NXX serviceProvNPA-NXX-X | 0x02 | 0x04 |
| LSMS Network and Subscription Data Download Classes: lnpNetwork lnpSubscriptions |  | 0x08 |
| SOA Network Data Download Classes: LnpNetwork | 0x20 |  |
| Query Outbound from the NPAC SMS Classes: All |  | 0x10 |

The association functions specified upon association are stored.  Then all subsequent operations performed by that associations are then validated against that data to verify that they are 'legal'. All outbound messages from the NPAC are also validated against the association functions and if a service provider does not have the correct masking set, they will not receive the transmission.  Note that the multiple Association Functions can be specified for an association. For example, a Local SMS can establish an association for both the process audit and network and subscription data download association functions.

#### Recovery Mode

The recovery mode flag is set to TRUE when a Local SMS or SOA is establishing a connection after a downtime. This flag indicates to the NPAC SMS to hold all current transactions until the Local SMS or SOA sends the Recovery Complete action. Once an association is established in recovery mode by a Local SMS, the Local SMS should request subscription and network downloads and notifications that occurred during downtime.  Once an association is established in recovery mode by a SOA, the SOA should request network downloads and notifications that occurred during downtime. After these steps are complete, the Local SMS or SOA should submit the Recovery Complete action. The NPAC SMS will respond to the recovery complete action, send all updates that occurred since association establishment and then normal processing will resume. See Appendix B, Section 7.1.

Service Provider Local SMS and SOA systems recover data independently. SOA systems can recover their information before, after, or concurrently with an LSMS using the same Service Provider Id.

A service provider providing SOA services for associated service providers can recover notifications for the primary and each associated service provider id prior to issuing the Recovery Complete action.

#### Signature

The signature field contains the MD5 hashed and encrypted systemId, the system type, the userId, the cmipDepartureTime, and sequenceNumber without separators between those fields or other additional characters.  Before hashing and encryptions, character fields are ASCII format and integer fields are 32 bit big endian.  Encryption is done using RSA encryption using the key from the key list specified. Validation of this field ensures data integrity and non-repudiation of data.  The following is additional information about how the information should be represented for digital signature encoding:

| Field | Format | Contents |
| --- | --- | --- |
| SystemID | ASCII |  |
| SystemType | Integer | e.g. local-sms = 1 |
| UserId | ASCII |  |
| cmipDepartureTime | ASCII | "YYYYMMDDHHMMSS.OZ" format |
| sequenceNumber | Integer |  |

### Association Establishment

Strong two way authentication at association is done for both the SOA to NPAC SMS interface and the NPAC SMS to Local SMS interface.  This secure association establishment is done at the application level using the access control field described above.  The access control information used during association set-up is sent in the association control messages. Association establishment can be done by the SOA to NPAC SMS or Local SMS to NPAC SMS. The NPAC SMS cannot initiate an association. The initiator of the association specifies its information in the AARQ PDU message and the responder in the AARE PDU.

When the SOA or LSMS initiate an association with the NPAC the NSAP and P-selector values will be validated to ensure that they are valid for the service provider initiating the association.  The following is an example of the information exchanged in the AARQ and AARE PDUs and the processing involved.  Assume for the example:

- A Local SMS is making an association with the NPAC SMS.

- The Local SMS systemId is “9999.”

- The NPAC SMS systemId is “NPAC SMS User Id.”

- the listId for the key list is 1.

- the keyId is 32.

- The key in listId 1 with a keyId of 32 is “ABC123.”

- The sequence number is 0 (as required).

- The Local SMS initiates the association request by creating and sending an AARQ PDU to the NPAC SMS.  This AARQ PDU contains the following access control information in the syntax described above:

- the systemId of “9999”.

- the listId of 1.

- the keyId of 32.

- the current Local SMS GMT time in the cmipDepartureTime.

- a sequence number of 0.

- the signature contains MD5 hashed and encrypted systemId, systemType, userId, cmipDepartureTime, and the sequenceNumber using the encryption key “ABC123” as found in key list 1 with key id 32.

- and all BOOLEAN items are set to FALSE in the functional groups field, except for the LSMSUnit of Query item which is set to TRUE.

- Once the AARQ PDU is sent, the sender (in this case the Local SMS), starts a tunable timer (with a default value of 2 minutes).  If the timer expires before the AARE PDU is received then the Local SMS will terminate the association attempt.

- When the NPAC SMS receives the association request it validates the data received.  The data is validated as follows:

- Ensure the systemId is present and valid for the association.

- Ensure the sequence number is 0.

- Ensure the cmipDepartureTime is within 5 minutes of the current NPAC SMS GMT time.

- find the key specified and decrypt the signature insuring that the systemId, systemType, userId, cmipDepartureTime, and sequenceNumber are the same as those specified in the PDU.

- The functional groups requested are valid for the system type that requested the association. In this example, the system type must be “local-sms(1)” {“soa-andlocal-sms(2)” value is to be removed from a future version of the IIS}.

- If validation of the AARQ PDU fails then an A-ABORT will be issued by the NPAC SMS with an error of access denied.  If the validation of the AARQ PDU is successful then an AARE PDU would be sent back to the Local SMS.  This AARE PDU contains the following access control information in the syntax described above:

- the systemId of “NPAC SMS User Id.”

- the listId of 1.

- the keyId of 32.

- the current NPAC SMS GMT time in the cmipDepartureTime.

- a sequence number of 0.

- and the signature contains MD5 hashed and encrypted systemId, systemType, userId, cmipDepartureTime, and the sequenceNumber using the encryption key “ABC123” as found in key list 1 with key id 32.

- The NPAC SMS may choose to optionally specify a new listId and keyId if for any reason it wants to make a key change.  Should either side of the interface change its listId/keyId values, both sides of the interface must mark the previously used keyId as used.

- When the Local SMS receives the association response it validates the data received.  The data is validated as follows:

- Ensure the systemId is present and valid for the association. (Note: the userId field is not required for Local SMS and NPAC SMS associations).

- Ensure the sequence number is 0.

- Ensure the cmipDepartureTime is within 5 minutes of the current Local SMS GMT time.

- find the key specified and decrypt the signature insuring that the systemId, systemType, userId, cmipDepartureTime, and sequenceNumber are the same as those specified in the PDU.

If validation of the AARE PDU fails then an A-ABORT will be issued by the Local SMS.  If validation is successful then a secure association has been established.

### Data Origination Authentication

For M-GET, M-SET, M-CREATE, M-DELETE, and M-ACTION, the access control field described above is used for data origination authentication.  Please note that any of the messages sent between manager and agent must be sent in confirmed mode.  The following is an example of the information exchanged in the CMIP PDUs and the processing involved.  Assume for the example:

- A SOA is making an association with the NPAC SMS.

- The SOA system provides SOA functionality for another Service Provider.

- The SOA systemId is “9999” for the primary Service Provider Id and is “8888” for an associated Service Provider Id.

- The NPAC SMS systemId is “NPAC SMS User Id.”

- the listId for the key list is 1.

- the keyId is 32.

- The key in listId 1 with a keyId of 32 is “ABC123.”

- The sequence number is 1.

- The SOA sends an M-GET to the NPAC SMS.  The M-GET PDU contains the following access control information in the syntax described above:

- the systemId of “8888.”

- the listId of 1.

- the keyId of 32.

- the current Local SMS GMT time in the cmipDepartureTime.

- a sequence number of 1.

- and the signature contains MD5 hashed and encrypted systemId, systemType, userId, cmipDepartureTime, and the sequenceNumber using the encryption key “ABC123” as found in key list 1 with key Id 32.

- Once the M-GET is sent, the sender (in this case the SOA), starts a tunable timer (with a default value of 2 minutes).  If the timer expires before the M-GET CMISE service response is received then the SOA will regenerate the sequenceNumber, cmipDepartureTime and signature and resend the request.  The SOA should resend a default of 3 times and abort the association if no response is received.  If a response is received after the timeout period, it should be discarded.  If an error message is received on a retry request, it should be evaluated to see if the request was processed or the error was received for other reasons.  For example, an error of “duplicateObjectInstance” for an M-CREATE request most likely indicates a successful create.

- When the NPAC SMS receives the M-GET request it validates the data received.  The data is validated as follows:

- Ensure the systemId is present and valid for the association. For the SOA the systemId can be the primary or associated Service Provider Id depending on the requestor.

- Ensure the sequence number is the next sequence number expected. (In this case 1).

- Ensure the cmipDepartureTime is within 5 minutes of the current NPAC SMS time.

- find the key specified and decrypt the signature, insuring that the systemId, systemType, userId, cmipDepartureTime, and sequenceNumber are the same as those specified in the PDU.

If validation of the M-GET PDU fails then an A-ABORT will be issued by the NPAC SMS without any additional information to prevent tampering and unauthorized use of network resources by intruders. If the validation of the M-GET PDU is successful then the NPAC SMS would get the data requested and send back an M-GET Response to the SOA.

Since CMIP notifications (M-EVENT-REPORT) do not have access control fields, all notifications defined contain the access control information in the notification definition.  ObjectCreation, ObjectDeletion, and AttributeValueChange should use the “information” attribute, which is an ANY DEFINED BY to contain the access control field.  The values and authentication for the notification access control fields are the same as above. For range ObjectCreation and AttributeValueChange notifications the access control would not be placed in the information attribute but rather in the access control attribute defined.  This would allow for the access control information to only be present once in the range notifications.

When the NPAC sends a notification, the destination service provider is uniquely identified in the distinguishedName of the M-EVENT-REPORT.  The lnpLocalSMS-Name attribute value(2.17) is appended to the service provider's id and is used to populate the value of the first element of the EventReportArgument's managedObjectInstance distinguishedName. This allows primary service providers to distinguish notifications destined for themselves and for each secondary service provider.

### Audit Trail

Audit trails will be maintained in logs on the NPAC SMS for the following association information:

- Association set-up messages.

- Association termination messages.

- Invalid messages:

- invalid digital signature.

- sequence number out of order.

- generalized time out of range.

- Invalid origination address.

- All incoming messages regardless of whether or not they cause changes to data stored in the NPAC SMS.

This information will be made available for report generation on the NPAC SMS system.  It will not be made available through the NPAC SMS Interoperable Interface.

## Association Management and Recovery

### Establishing Associations

#### NpacAssociationUserInfo

The following structure will be used to report the status of a login attempt or the current state of the NPAC SMS:

NpacAssociationUserInfo ::= SEQUENCE {

error-code [0] IMPLICIT ErrorCode,

error-text [1] IMPLICIT GraphicString(SIZE(1..80))

}

ErrorCode ::= ENUMERATED

{

success (0),

access-denied (1)

retry-same-host (2)

try-other-host (3)

}

Bind Requests and Responses

For AARQ (M-Bind requests) the NPAC SMS will be ignoring the CMIPUserInfo userInfo field.  The SMASEUserInfo will be ignored by the NPAC SMS.

In order to validate a successful login, the AARE (M-Bind response) from the NPAC SMS will contain the NpacAssociationUserInfo as the “userInfo” field of the CMIPUserInfo that is contained on the AARE.  The ErrorCode will be set to “success”.

The following structure will be used for CMIPUserInfo:

CMIPUserInfo ::= 2:9:1:1:4

--{joint-iso-ccitt(2) ms(9) cmip(1) cmip-pci(1)

abstractSyntax(4)}

CMIPUserInfo ::= SEQUENCE {

protocolVersion [0] IMPLICIT ProtocolVersion

DEFAULT {version1-cmip-assoc},

functionalUnits [1] IMPLICIT FunctionalUnits DEFAULT {},

accessControl   [2] EXTERNAL OPTIONAL

userInfo        [3] EXTERNAL OPTIONAL

}

#### Unbind Requests and Responses

The NPAC SMS will never be issuing the RLRQ (M-Unbind request), but will respond to them from the SOA or Local SMS.

#### Aborts

For unsuccessful logon attempts or situations where the NPAC SMS application must abort all associations, the ABRT CMIPAbortInfo structure’s “userInfo” will contain the NpacAssociationUserInfo structure.  The ErrorCode will be set to one of the enumeration values.

The following structure will be used for CMIPAbortInfo:

CMIPAbortInfo ::= 2:9:1:1:4

--{joint-iso-ccitt(2) ms(9) cmip(1) cmip-pci(1)

abstractSyntax(4)}

CMIPAbortInfo ::= SEQUENCE {

abortSource [0] IMPLICIT CMIPAbortSource,

userInfo    [1] EXTERNAL OPTIONAL

}

#### NPAC SMS Failover Behavior

Under normal conditions, the primary NPAC SMS will be responding by accepting association requests while the secondary NPAC SMS will be responding by denying association requests with an ABRT and error code of  TRY_OTHER_HOST.

When the primary NPAC SMS needs to go down for a short period of time (secondary will not take over),  the primary NPAC SMS will either not be responding (if down) or be denying association requests with an error code of  RETRY _SAME_HOST (if partially up). The secondary NPAC SMS will be responding by denying association requests with an ABRT and error code of  TRY_OTHER_HOST.

When the primary NPAC SMS goes down (scheduled or unscheduled) and the secondary NPAC SMS is re-synchronizing to become active, the primary NPAC SMS will be denying association requests with an ABRT and error code of TRY_OTHER_HOST.  The secondary NPAC SMS will be responding by denying association requests with an ABRT and error code of RETRY_SAME_HOST. Once the secondary NPAC SMS is done re-synchronizing, it will then start accepting association requests.

#### Service Provider SOA and Local SMS Procedures

The following is an algorithm that can be used by a service provider SOA or Local SMS when trying to establish an association with the NPAC SMS:

try to establish an association on the primary NPAC SMS if a response was obtained

{

if the response was an ABRT and the ABRT is from the NPAC
  Application

{

switch (error code)

{

case ACCESS_DENIED

find out what is causing the error and fix it

retry the association on the primary NPAC SMS

case RETRY_SAME_HOST

wait X seconds

retry the association on the primary NPAC SMS

case TRY_OTHER_HOST

wait X seconds

execute this algorithm again substituting

"secondary" for "primary"

}

}

else

{

if the response was an ABRT and from the PROVIDER
    (not application)

find out what is causing the error and fix it

retry the association on either the primary or

secondary NPAC SMS

}

else

{

# timeout - some type of network error has occurred

# a number of different things can be done:

#

#   wait X seconds

#   retry primary

#

#       or

#

#   find out what is causing the error and fix it

#   retry the association on the primary NPAC SMS

#

#       or

#

#   wait X seconds

#   execute this algorithm again substituting

#   "secondary" for "primary"

}

### Releasing or Aborting Associations

Any of the systems, NPAC SMS, service provider SOA or Local SMS can abort an association at any time.  Only the SOA and Local SMS can perform an RLRQ request.  Once a scheduled outage has arrived, the NPAC SMS will abort associations (error code of “Try Other Host” or “Retry Same Host” depending on the type of outage).

### Error Handling

#### NPAC SMS Error Handling

The NPAC SMS will issue errors to the Local SMS and SOA interfaces based upon the definitions and mappings in Appendix A.  The NPAC SMS expects the SOA and Local SMS to support the same error definitions when both issuing and receiving error responses for the operations each interface supports.

The NPAC SMS will attempt to interpret an error returned from a SOA or Local SMS.  The NPAC SMS will log the error.  If the request is not resent and the error response was returned from a Local SMS and related to a subscription version broadcast (M-CREATE or Create Action, M-DELETE, M-SET), a broadcast failure will be noted for the service provider on the subscription version.  If a service provider does not have an active Local SMS association at the time of a broadcast, the broadcast will be automatically failed for the service provider.

The Local SMS and SOA are expected to recover themselves with the NPAC SMS when their association is reestablished.  Thus it is the responsibility of the Local SMS and SOA to request the necessary data to rectify the failed transmission of M-EVENT-REPORTs, network data updates and non-broadcast oriented subscription version updates.

If the NPAC SMS sends a request to a Local SMS or SOA and receives no response from the CMISE service within the tunable period, the NPAC SMS will resend the message according to the tunable retry periods for the specific message type.  If a response is received after the timeout period, it will be discarded.  If the NPAC SMS receives no response, the NPAC SMS will assume the association is down and abort the connection.  The Local SMS and SOA systems should assume the same behavior with the NPAC SMS.

#### Processing Failure Error

In addition to the standard CMIP error reporting mechanisms, the following attribute will be passed in the SpecificErrorInfo structure on CMIP errors that return a PROCESSING FAILURE error. This structure will be used to detail errors not covered by the standard CMIP error codes.

GDMO Definition

lnpSpecificInfo ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.LnpSpecificInfo;

MATCHES FOR EQUALITY;

BEHAVIOUR lnpSpecificInfoBehavior;

REGISTERED AS {lnp-attribute 8};

lnpSpecificInfoBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to return more detailed error text information upon a CMIP Processing Failure error.

!;

ASN.1 Definition

LnpSpecificInfo ::= GraphicString(SIZE(1..256))

### Recovery

The SOA and Local SMS associations are viewed to be permanent connections by the NPAC SMS. Thus when the association is broken for any reason, the system connecting to the NPAC SMS must assume responsibility to recover and resynchronize themselves with the NPAC SMS.  One association should be established for recovery and no other associations should be established in normal mode until recovery is complete.

During the recovery processing, other messages may be generated at the NPAC SMS that are intended for the recovering SOA or LSMS.  These messages are queued on the NPAC SMS until the SOA or LSMS finishes the recovery process and sends an lnpRecoveryComplete action to the NPAC SMS.  Additionally, during the recovery process, the “x by y” retry functionality (where “x” is the number of attempts, and “y” is the interval in number of minutes in between attempts) continues on the NPAC SMS, but message sending is suspended to the SOA or LSMS, and the retry attempts counter is not decremented, as long as the SOA or LSMS is still in recovery mode.  Therefore, a Subscription Version could stay in a “sending” status for a period of time longer than expected, since the retry logic will not transition the status to “partial failure” or “failed” as long as a Service Provider is in recovery mode.

While recovering subscription data, the NPAC SMS excludes Subscription Versions with a status of failed.  The value in the Broadcast Timestamp field in each Subscription Version is used to determine whether or not a Subscription Version is included in the recovering LSMS’s requested criteria.

The SOA or LSMS is capable of recovering data based on the association functions.  The SOA recovers network data and notification data using the network data management association function (networkDataMgmt).  The LSMS recovers network data and subscription data using the data download association function (dataDownload), and recovers notification data using the network data management association function (networkDataMgmt).

NPAC data may be recovered in two ways, ‘time-based’, or ‘record-based’ criteria.  Based on the type of data being recovered, additional criteria may also be specified.  The table below show the type of data that can be recovered, and the criteria that may be used for each type.

| Data Type | Criteria | Additional Criteria |
| --- | --- | --- |
| Network Data | Time Based | Time Range (consisting of Start time, End time) |
|  | Record-Based | service provider ID |
|  |  | all service providers |
|  |  | NPA-NXX range |
|  |  | all NPA-NXX data |
|  |  | NPA-NXX-X range |
|  |  | all NPA-NXX-X data |
|  |  | LRN range |
|  |  | all LRN data |
|  |  | all network data |
| Subscription Data | Time-Based | Time Range (consisting of Start time, End time) |
|  | Record-Based | TN |
|  |  | TN range |
| Number Pool Block Data | Time-Based | Time Range (consisting of Start time, End time) |
|  | Record-Based | NPA-NXX-X |
|  |  | NPA-NXX-X range |
| Notification Data | Time-Based | Time Range (consisting of Start time, End time) |
|  | Record-Based | Not Available |

‘Time-Based’ Recovery Requests

All 'time-based' recovery requests specifying time range criteria are limited to the NPAC SMS tunable, “Maximum Download Duration”.  When the SOA or LSMS issues a recovery request (whether Network, Subscription, Number Pool Block, or Notification Data) with time-based criteria, the NPAC SMS will compare the time range indicated in the request to the “Maximum Download Duration” tunable.

For service providers that do not support linked replies, Subscription data 'time-based' recovery requests specifying time range criteria are also limited to the number of TNs specified in the Service Provider specific tunable, “Maximum TN Download in Recovery Request”.  Therefore, a valid request will fall within both the duration and quantity tunable values.

For service providers that do not support linked replies, Notification data 'time-based' recovery requests specifying time range criteria are also limited to the number of notifications specified in the NPAC SMS tunable, “Maximum Number of Download Notifications”.  Therefore, a valid request will fall within both the duration and quantity tunable values.

For service providers that do not support linked replies, for all types of 'time-based' recovery requests, where the tunable value is exceeded, an appropriate error message is issued over the interface from the NPAC SMS to the originating system.  This applies to both duration overages (“Maximum Download Duration”), and number of record overages (“Maximum TN Download in Recovery Request” for subscription data, and “Maximum Number of Download Notifications” for notification data).

‘Record-Based’ Recovery Requests

For service providers that do not support linked replies, all ‘record-based’ recovery requests specifying other criteria (for example, TN/TN range, NPA-NXX/NPA-NXX range) are limited by the number of records specified in the NPAC SMS tunable, “Maximum Number of Download Records”.  When the SOA or LSMS issues a network data recovery request or the LSMS issues a subscription version data recovery request, using 'record-based' criteria, the NPAC SMS will compare the records indicated in the request to the “Maximum Number of Download Records” tunable.  If the number of records exceeds this tunable value, an appropriate error message is issued over the interface from the NPAC SMS to the originating system.

#### Local SMS Recovery

To recover, the Local SMS starts by setting the recoveryMode flag of the access control parameter. This flag signals the NPAC SMS to hold all data updates to this Local SMS. The Local SMS should then request the network, subscription and number pool block data downloads and the notifications that occurred during downtime.  Once this is complete, the Local SMS should issue the lnpRecoveryComplete action to turn off the recoveryMode flag. After the NPAC SMS responds to the lnpRecovery Complete action it will send to the LSMS any other messages that have occurred since the association was established.

#### SOA Recovery

To recover, the SOA starts by setting the recoveryMode flag of the access control parameter. This flag signals the NPAC SMS to hold all data updates to this SOA. The SOA should then request the network data downloads and notifications that occurred during downtime.  Once this is complete, the SOA should issue the lnpRecoveryComplete action to turn off the recoveryMode flag. After the NPAC SMS responds to the lnpRecovery Complete action it will send to the SOA any other messages that have occurred since the association was established.

#### Linked Action Replies during Recovery

Linked Reply functionality applies to Service Providers that have their SOA Linked Replies Indicator set to TRUE, or their Local SMS Linked Replies Indicator set to TRUE.

For service provider that support linked replies the Maximum TN Download in Recovery Request, the Maximum Number of Download Notifications and Maximum Number of Download Records tunables do not apply to recovery processing.

Linked replies will be returned as the response to an lnpDownload action request for network data if the number of messages returned exceeds the "Network Data Linked Reply Blocking Factor" tunable but is less than the "Network Data Maximum Linked Recovered Objects" tunable.  If the number of network data objects to be returned exceeds the "Network Data Maximum Linked Recovered Objects" tunable, a "criteria-too-large" error will be returned to the requesting SOA/LSMS.

Linked replies will be returned as the response to an lnpDownload action request for subscription data if the number of objects returned exceeds the "Subscription Data Linked Reply Blocking Factor" tunable but is less than the "Subscription Data Maximum Linked Recovered Objects" tunable.  If the number of subscription data objects be returned exceeds the "Subscription Data Maximum Linked Recovered Objects" tunable, a "criteria-too-large" error will be returned to the requesting LSMS.

Linked replies will be returned as the response to an lnpDownload action request for Number Pool Block data if the number of objects returned exceeds the "Number Pool Block Data Linked Reply Blocking Factor" tunable but is less than the " Number Pool Block Data Maximum Linked Recovered Objects" tunable.  If the number of Number Pool Block data objects be returned exceeds the " Number Pool Block Data Maximum Linked Recovered Objects" tunable, a "criteria-too-large" error will be returned to the requesting LSMS.

Linked replies will be returned as the response to an lnpNotificationRecovery action request for notification data if the number of notifications returned exceeds the "Notification Data Linked Reply Blocking Factor" tunable but is less than the "Notification Data Maximum Linked Recovered Notifications" tunable.  If the number of notifications to be returned exceeds the "Notification Data Maximum Linked Recovered Notifications" tunable, a "criteria-too-large" error will be returned to the requesting SOA/LSMS.

As an example, a Service Provider’s SOA was down, and is now performing notification recovery.  During the downtime, 90 notifications were issued.  Assuming the Notification Blocking Factor is set to 50 notifications, the recovering SOA would receive data from the NPAC SMS in the form of three linked replies.  The first reply would contain 50 notifications, the second reply would contain 40 notifications, and the third reply would be empty (this is an indication that the NPAC SMS is finished sending data for this recovery request).  In the case where the amount of data to be returned is less than or equal to the associated Blocking Factor, the M-ACTION response will be a normal response (i.e., non-linked response) and will not be a linked reply.

Below are the tunables that specify the download size:

| Download Criteria | Tunable Name |
| --- | --- |
| Network data download request maximum linked reply size | Network Data Linked Replies Blocking Factor |
| Subscription download request maximum linked reply size | Subscription Data Linked Replies Blocking Factor |
| Number Pool Block download request maximum linked reply size | Number Pool Block Data Linked Replies Blocking Factor |
| Notification download request maximum linked reply size | Notification Data Linked Replies Blocking Factor |
| Total number of network data objects returned for a single download request | Network Data Maximum Linked Recovered Objects |
| Total number of subscription data objects returned for a single download request | Subscription Data Maximum Linked Recovered Objects |
| Total number of Number Pool Block data objects returned for a single download request | Number Pool Block Data Maximum Linked Recovered Objects |
| Total number of notification data notifications returned for a single download request | Notification Data Maximum Linked Recovered Objects |

Linked replies will be returned as the response to an action request from the recovering SOA/LSMS.  The entire operation is considered complete once all linked replies and the final empty reply are returned as the response to the original request.  Timeout processing is expected to start when the initial request is sent by the recovering SOA/LSMS, and terminate upon receipt of the final empty reply by the recovering SOA/LSMS.

## Congestion Handling

The following sections define NPAC SMS behavior when in congestion and the NPAC handling of Local SMS and SOA congestion.  The recommendation for Congestion Control follows the “Flow Control” mechanism and is described in OSI Communication Reference Model (ISO/IEC 7498).  The two types of flow control defined are:

- Peer Flow Control

- Inter-Layer Flow Control

Peer Flow Control can be used when two peer layers of the OSI Stack talk to each other.  The most common form of Peer Flow Control is the sliding window protocol.  This protocol is implemented by TCP.  This is the flow control approach used by the NPAC SMS.

### NPAC SMS Congestion

Once the number of incoming messages to be queued to the NPAC SMS is exceeded at the transport layer, TCP/IP, an indication will be sent to the sender from the transport layer, TCP/IP, that congestion is occurring.  Upon clearing of the congestion situation, the transport layer, TCP/IP will indicate to the sender that congestion has been cleared.  As the receiver, the NPAC SMS application will not be aware that it is congested.  The NPAC SMS application will be continually processing the information being sent as quickly as possible.  Only the sender will be aware that  the NPAC SMS is congested due to the fact that it can not send any more information to the NPAC SMS via the transport layer, TCP/IP.  Implementation of functionality to handle NPAC congestion situations is at the discretion of SOA and LSMS vendors.

### NPAC Handling of Local SMS and SOA Congestion

The NPAC SMS application must be able to handle congestion when attempting to send out a message to a SOA or LSMS system.  When receiving indications of congestion via the transport layer from a SOA or LSMS the NPAC SMS application stops dispatching messages for the SPID (primary or associated) and SOA or LSMS interface that returned congestion.  Note: If a SOA system returns congestion it will not affect the LSMS for the same service provider and vise versa.  When the NPAC SMS stops dispatching messages to a congested SOA or LSMS, the retry attempts and retry timer values and the behavior associated with them apply to the messages not dispatched.  The NPAC will abort the SOA or LSMS association once the retry attempts are exhausted.  Any unacknowledged messages at the NPAC SMS application layer will be handled as failures as they are when an association is aborted today, for example for security reasons.

Once the NPAC SMS gets an indication via the transport layer that a SOA or LSMS system that was previously congested is ready to receive information, the NPAC SMS resumes sending of messages to that system.  Note that the NPAC SMS will use the sequence number for the message it sends first that was the sequence number on the message that was sent when congestion indication was received.  This is done since the SOA or LSMS system did not receive this message.  If the sequence number were incremented this would cause the SOA or LSMS to abort the association due to the sequence number value being larger than expected.  SOA and LSMSs should use the same sequence number as well when communicating with the NPAC to prevent the NPAC from aborting the association due to the sequence number value being larger than expected.

# GDMO Definitions

6

## Overview

The GDMO interface definitions (gdmo_v3_1_0_081001) provided below support the SOA to NPAC SMS interface and the NPAC SMS to Local SMS interface.  Included in this section of the interface specification are object name bindings, attribute, package, action, and notification definitions.

## Object Definitions

---

-- Object Definitions

--

-- 1.0 LNP Audits Managed Object

lnpAudits MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

lnpAuditsPkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 1};

lnpAuditsPkg PACKAGE

BEHAVIOUR

lnpAuditsDefinition,

lnpAuditsBehavior;

ATTRIBUTES

lnpAuditsName GET;

;

lnpAuditsDefinition BEHAVIOUR

DEFINED AS !

The lnpAudits class is the managed object that is used as

the container object for the subscriptionAudit objects on the

NPAC SMS.  This object has been created for scoping efficiency.

!;

lnpAuditsBehavior BEHAVIOUR

DEFINED AS !

NPAC SMS Managed Object for the SOA to NPAC SMS interface.

The service provider SOA can M-GET any lnpAudits object on the

NPAC SMS. (SOA Management Association Function).

The Local SMS can not M-GET any lnpAudits object on the NPAC SMS.

The lnpAuditsName attribute is read only and can not

be changed via the Local SMS or SOA Interface once the object has

been created.  The value of lnpAuditsName will always be "lnpAudits".

Only one of these objects will exist per agent and it will only be

created at startup of the CMIP agent software on the NPAC SMS.

!;

-- 2.0 LNP Local SMS Managed Object Class

lnpLocalSMS MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

lnpLocalSMS-Pkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 2};

lnpLocalSMS-Pkg PACKAGE

BEHAVIOUR

lnpLocalSMS-Definition,

lnpLocalSMS-Behavior;

ATTRIBUTES

lnpLocal-SMS-Name GET;

;

lnpLocalSMS-Definition BEHAVIOUR

DEFINED AS !

The lnpLocalSMS class is the managed object that is used as the

container object for all Local SMS data in the NPAC SMS to

Local SMS Interface.

!;

lnpLocalSMS-Behavior BEHAVIOUR

DEFINED AS !

Local SMS Managed Object.

The NPAC SMS can M-GET any lnpLocalSMS object (Data Download

Association Function).

The lnp-LocalSMS-Name attribute is read only and can not

be changed via the Local SMS Interface once the object has

been created.  The value of lnpLocal-SMS-Name will always be

a unique identifier for the Local SMS for the NPAC SMS to

Local SMS Interface.

Only one of these objects will exist and it will only be

created at startup of the CMIP agent software on the Local

SMS.

!;

-- 3.0 LNP Log Record for the Subscription Audit Local SMS Discrepancy Report

lnpLogAudit-DiscrepancyRptRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":eventLogRecord;

CHARACTERIZED BY

lnpLogAudit-DiscrepancyRptPkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 3};

lnpLogAudit-DiscrepancyRptPkg PACKAGE

BEHAVIOUR

lnpLogAudit-DiscrepancyRptDefinition,

lnpLogAudit-DiscrepancyRptBehavior;

ATTRIBUTES

auditDiscrepancyTn GET,

auditDiscrepancyVersionId GET,

auditDiscrepancyLSMS-SP-Id GET,

auditDiscrepancyFailureReason GET,

accessControl GET;

;

lnpLogAudit-DiscrepancyRptDefinition BEHAVIOUR

DEFINED AS !

The lnpLogAudit-DiscrepancyRptRecord class is the managed

object that is used to create log records for the

subscriptionAudit-DiscrepancyRpt Notification.

!;

lnpLogAudit-DiscrepancyRptBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

subscriptionAudit-DiscrepancyRpt Notification.

!;

-- 4.0 LNP Log Record for the Subscription Audit Results

lnpLogAuditResultsRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":eventLogRecord;

CHARACTERIZED BY

lnpLogAuditResultsPkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 4};

lnpLogAuditResultsPkg PACKAGE

BEHAVIOUR

lnpLogAuditResultsDefinition,

lnpLogAuditResultsBehavior;

ATTRIBUTES

auditResultStatus GET,

auditResultFailed-SP-List GET,

auditResultNumberDiscrepancies GET,

auditResultCompletionTime GET,

accessControl GET;

;

lnpLogAuditResultsDefinition BEHAVIOUR

DEFINED AS !

The lnpLogAuditResultsRecord class is the managed object

that is used to create log records for the

subscriptionAuditResults Notification.

!;

lnpLogAuditResultsBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

subscriptionAuditResults Notification.

!;

-- 5.0 LNP Log Record for the Subscription Version Cancellation

-- Acknowledge Request Notification

lnpLogCancellationAcknowledgeRequestRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":eventLogRecord;

CHARACTERIZED BY

lnpLogCancellationAcknowledgeRequestPkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 5};

lnpLogCancellationAcknowledgeRequestPkg PACKAGE

BEHAVIOUR

lnpLogCancellationAcknowledgeRequestDefinition,

lnpLogCancellationAcknowledgeRequestBehavior;

ATTRIBUTES

subscriptionTN GET,

subscriptionVersionId GET,

accessControl GET;

;

lnpLogCancellationAcknowledgeRequestDefinition BEHAVIOUR

DEFINED AS !

The lnpLogCancellationAcknowledgeRequestRecord class is

the managed object that is used to create log records for the

subscriptionVersionCancellationAcknowledgeRequest

Notification.

!;

lnpLogCancellationAcknowledgeRequestBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

subscriptionVersionCancellationAcknowledgeRequest

Notification.

!;

-- 7.0 LNP Log Record for the Subscription Version New SP Create Request

--     Notification

lnpLogNewSP-CreateRequestRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":eventLogRecord;

CHARACTERIZED BY

lnpLogNewSP-CreateRequestPkg;

CONDITIONAL PACKAGES

subscriptionTimerTypePkg PRESENT IF

!present if the New SP SOA supports timer type!,

subscriptionBusinessTypePkg PRESENT IF

!present if the New SP SOA supports business type!;

REGISTERED AS {LNP-OIDS.lnp-objectClass 7};

lnpLogNewSP-CreateRequestPkg PACKAGE

BEHAVIOUR

lnpLogNewSP-CreateRequestDefinition,

lnpLogNewSP-CreateRequestBehavior;

ATTRIBUTES

subscriptionTN GET,

subscriptionVersionId GET,

subscriptionOldSP GET,

subscriptionOldSP-DueDate GET,

subscriptionOldSP-Authorization GET,

subscriptionOldSP-AuthorizationTimeStamp GET,

subscriptionStatusChangeCauseCode GET,

accessControl GET;

;

lnpLogNewSP-CreateRequestDefinition BEHAVIOUR

DEFINED AS !

The lnpLogNewSP-CreateRequestRecord class is the managed

object that is used to create log records for the

subscriptionVersionNewSP-CreateRequest Notification.

!;

lnpLogNewSP-CreateRequestBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

subscriptionVersionNewSP-CreateRequest Notification.

!;

-- 8.0 LNP Log Record for the Subscription Version Old SP Concurrence Request

--     Notification

lnpLogOldSP-ConcurrenceRequestRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":eventLogRecord;

CHARACTERIZED BY

lnpLogOldSP-ConcurrenceRequestPkg;

CONDITIONAL PACKAGES

subscriptionTimerTypePkg PRESENT IF

!present if the Old SP SOA supports timer type!,

subscriptionBusinessTypePkg PRESENT IF

!present if the Old SP SOA supports business type!;

REGISTERED AS {LNP-OIDS.lnp-objectClass 8};

lnpLogOldSP-ConcurrenceRequestPkg PACKAGE

BEHAVIOUR

lnpLogOldSP-ConcurrenceRequestDefinition,

lnpLogOldSP-ConcurrenceRequestBehavior;

ATTRIBUTES

subscriptionTN GET,

subscriptionVersionId GET,

subscriptionNewCurrentSP GET,

subscriptionNewSP-DueDate GET,

subscriptionNewSP-CreationTimeStamp GET,

accessControl GET;

;

lnpLogOldSP-ConcurrenceRequestDefinition BEHAVIOUR

DEFINED AS !

The lnpLogOldSP-ConcurrenceRequestRecord class is the managed

object that is used to create log records for the

subscriptionVersionOldSP-ConcurrenceRequest Notification.

!;

lnpLogOldSP-ConcurrenceRequestBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

subscriptionVersionOldSP-ConcurrenceRequest Notification.

!;

-- 9.0 LNP Log Record for the NPAC SMS Operational Information Notification

lnpLogOperational-InformationRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":eventLogRecord;

CHARACTERIZED BY

lnpLogOperational-InformationPkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 9};

lnpLogOperational-InformationPkg PACKAGE

BEHAVIOUR

lnpLogOperational-InformationDefinition,

lnpLogOperational-InformationBehavior;

ATTRIBUTES

downTime GET,

npacContactNumber GET,

additionalDownTimeInformation GET,

accessControl GET;

;

lnpLogOperational-InformationDefinition BEHAVIOUR

DEFINED AS !

The lnpLogOperational-InformationRecord class is the managed object

that is used to create log records for the

lnpNPAC-SMS-Operational-Information Notification.

!;

lnpLogOperational-InformationBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

lnpNPAC-SMS-Operational-Information Notification.

!;

-- 10.0 LNP Log Record for the Subscription Version Status Attribute Value

--     Change Notification

lnpLogStatusAttributeValueChangeRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":eventLogRecord;

CHARACTERIZED BY

lnpLogStatusAttributeValueChangePkg;

CONDITIONAL PACKAGES

subscriptionVersionAttributeValueChangeFailed-SP-ListPkg PRESENT IF

!the version broadcast failed!,

subscriptionStatusChangeCauseCodePkg PRESENT IF

!the version status is set to conflict by the old service

provider!;

REGISTERED AS {LNP-OIDS.lnp-objectClass 10};

lnpLogStatusAttributeValueChangePkg PACKAGE

BEHAVIOUR

lnpLogStatusAttributeValueChangeDefinition,

lnpLogStatusAttributeValueChangeBehavior;

ATTRIBUTES

subscriptionVersionAttributeValueChangeInfo GET,

accessControl GET;

;

lnpLogStatusAttributeValueChangeDefinition BEHAVIOUR

DEFINED AS !

The lnpLogStatusAttributeValueChangeRecord class is the managed

object that is used to create log records for the

subscriptionVersionStatusAttributeValueChange Notification.

!;

lnpLogStatusAttributeValueChangeBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

subscriptionVersionStatusAttributeValueChange Notification.

!;

-- 11.0 LNP Network Managed Object Class

lnpNetwork MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

lnpNetworkPkg;

CONDITIONAL PACKAGES

lnpDownloadPkg PRESENT IF

!the object is instantiated on the NPAC SMS!;

REGISTERED AS {LNP-OIDS.lnp-objectClass 11};

lnpNetworkPkg PACKAGE

BEHAVIOUR

lnpNetworkDefinition,

lnpNetworkBehavior;

ATTRIBUTES

lnpNetworkName GET;

;

lnpNetworkDefinition BEHAVIOUR

DEFINED AS !

The lnpNetwork class is the managed object that is used as the

container object for the serviceProvNetwork objects.

This object has been created primarily for scoping efficiency.

The lnpDownloadPkg will only be used for lnpNetwork object

instantiated on the NPAC SMS (Data Download Association Function).

This package is used for initiating from the Local SMS or SOA

downloading of serviceProvNetwork, serviceProvNPA-NXX,

serviceProvNPA-NXX-X and serviceProvLRN object creation,

modification, or deletion to the Local SMS or SOA from

the NPAC SMS.

!;

lnpNetworkBehavior BEHAVIOUR

DEFINED AS !

Local SMS, SOA, and NPAC SMS Managed Object used for the Local SMS to

NPAC SMS and the SOA to NPAC SMS interfaces.

The Local SMS, SOA, and the NPAC SMS can M-GET any lnpNetwork

object (Data Download Association Function).  The lnpNetworkName

attribute is read only and can not be changed via the NPAC SMS

to Local SMS or SOA to NPAC SMS Interfaces once the object

has been created.  The  value of lnpNetworkName will always

be "lnpNetwork".

Only one of these objects will exist and it will only be

created at startup of the CMIP agent software on the NPAC SMS

the Local SMS or SOA.

!;

-- 12.0 LNP NPAC SMS Managed Object Class

lnpNPAC-SMS MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

lnpNPAC-SMS-Pkg,

lnpRecoveryCompletePkg,

lnpNotificationRecoveryPkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 12};

lnpNPAC-SMS-Pkg PACKAGE

BEHAVIOUR

lnpNPAC-SMS-Definition,

lnpNPAC-SMS-Behavior;

ATTRIBUTES

lnpNPAC-SMS-Name GET;

NOTIFICATIONS

lnpNPAC-SMS-Operational-Information,

subscriptionVersionNewNPA-NXX;

;

lnpNPAC-SMS-Definition BEHAVIOUR

DEFINED AS !

The lnpNPAC-SMS class is the managed object that is used as

the container object for all NPAC SMS objects in the NPAC SMS to

Local SMS Interface and the SOA to NPAC SMS interface.

!;

lnpNPAC-SMS-Behavior BEHAVIOUR

DEFINED AS !

NPAC SMS Managed Object for the SOA to NPAC SMS and the Local SMS

to NPAC SMS interface.

A Local SMS and SOA can M-GET any lnpNPAC-SMS object.

The lnpNPAC-SMS-Name attribute is read only and can not be

changed via either Interface once the object has been created.

The lnpRecoveryComplete-Pkg is used to indicate the

recovery mode for the Local SMS or SOA is complete and to send all

updates made since the recovery mode began.  (Data Download Functional

Group).

The lnpNotificationRecoveryPkg is used to recover notifications

in recovery mode by the Local SMS or SOA. (Data Download

Functional Group).

Only one of these objects will exist and it will only be

created at startup of the CMIP agent software on the NPAC SMS.

The lnpNPAC-SMS-Operational-Information will be used to notify

service provider SOA and Local SMS systems of planned outages.

The subscriptionVersionNewNPA-NXX is used to support

number pooling.

!;

-- 13.0 LNP Service Providers Managed Object Class

lnpServiceProvs MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

lnpServiceProvsPkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 13};

lnpServiceProvsPkg PACKAGE

BEHAVIOUR

lnpServiceProvsDefinition,

lnpServiceProvsBehavior;

ATTRIBUTES

lnpServiceProvsName GET;

;

lnpServiceProvsDefinition BEHAVIOUR

DEFINED AS !

The lnpServiceProvs class is the managed object that is

used as the container object for the serviceProv

objects on the NPAC SMS.  This object has been created

for scoping efficiency.

!;

lnpServiceProvsBehavior BEHAVIOUR

DEFINED AS !

NPAC SMS Managed Object used for the Local SMS to NPAC

SMS and SOA to NPAC SMS interfaces.

A Local SMS and service provider SOA can M-GET any

lnpServiceProvs object (Network Data Association Function).

The lnpServiceProvsName attribute is read only and can not

be changed via the Local SMS Interface once the object has

been created.  The value of lnpServiceProvsName will

always be "lnpServiceProvs".

Only one of these objects will exist and it will only be created

at startup of the CMIP agent software on the NPAC SMS.

!;

-- 14.0 LNP Subscriptions Managed Object Class

lnpSubscriptions MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

lnpSubscriptionsPkg,

subscriptionVersionLocalSMS-CreatePkg;

CONDITIONAL PACKAGES

lnpDownloadPkg PRESENT IF

!the object is instantiated on the NPAC SMS!,

subscriptionVersionOldSP-CreatePkg PRESENT IF

!the object is instantiated on the NPAC SMS!,

subscriptionVersionNewSP-CreatePkg PRESENT IF

!the object is instantiated on the NPAC SMS!,

subscriptionVersionDisconnectPkg PRESENT IF

!the object is instantiated on the NPAC SMS!,

subscriptionVersionModifyPkg PRESENT IF

!the object is instantiated on the NPAC SMS!,

subscriptionVersionActivatePkg PRESENT IF

!the object is instantiated on the NPAC SMS!,

subscriptionVersionCancelPkg PRESENT IF

!the object is instantiated on the NPAC SMS!,

subscriptionVersionOldSP-CancellationPkg PRESENT IF

!the object is instantiated on the NPAC SMS!,

subscriptionVersionNewSP-CancellationPkg PRESENT IF

!the object is instantiated on the NPAC SMS!,

subscriptionVersionRemoveFromConflictPkg PRESENT IF

!the object is instantiated on the NPAC SMS!,

numberPoolBlock-CreatePkg PRESENT IF

!the object is instantiated on the NPAC SMS!,

subscriptionVersionRangeStatusAttributeValueChangePkg PRESENT IF

!the object is instantiated on the NPAC SMS!,

subscriptionVersionRangeAttributeValueChangePkg PRESENT IF

!the object is instantiated on the NPAC SMS!,

subscriptionVersionRangeObjectCreationPkg PRESENT IF

!the object is instantiated on the NPAC SMS!,

subscriptionVersionRangeDonorSP-CustomerDisconnectDatePkg PRESENT IF

!the object is instantiated on the NPAC SMS!,

subscriptionVersionRangeCancellationAcknowledgePkg PRESENT IF

!the object is instantiated on the NPAC SMS!,

subscriptionVersionRangeNewSP-CreateRequestPkg PRESENT IF

!the object is instantiated on the NPAC SMS!,

subscriptionVersionRangeOldSP-ConcurrenceRequestPkg PRESENT IF

!the object is instantiated on the NPAC SMS!,

subscriptionVersionRangeOldSPFinalConcurrenceWindowExpirationPkg PRESENT IF

!the object is instantiated on the NPAC SMS!,

subscriptionVersionRangeNewSP-FinalCreateWindowExpirationPkg PRESENT IF

!the object is instantiated on the NPAC SMS!;

REGISTERED AS {LNP-OIDS.lnp-objectClass 14};

lnpSubscriptionsPkg PACKAGE

BEHAVIOUR

lnpSubscriptionsDefinition,

lnpSubscriptionsBehavior;

ATTRIBUTES

lnpSubscriptionsName GET;

NOTIFICATIONS

subscriptionVersionLocalSMS-ActionResults;

;

lnpSubscriptionsDefinition BEHAVIOUR

DEFINED AS !

Local SMS and NPAC SMS Managed Object for the SOA to NPAC SMS

and the Local SMS to NPAC SMS interface.

The lnpSubscriptions class is the managed object that is used

as the container object for the subscription version objects

and numberPoolBlock objects on the NPAC SMS and the Local SMS.

Local SMS interfaces must be able to support scope/filtered

M-SETs and M-DELETEs with a TN range as the primary filter.

!;

lnpSubscriptionsBehavior BEHAVIOUR

DEFINED AS !

Local SMS and NPAC SMS Managed Object

The Local SMS (Data Download Association Function) and the service

provider SOA (SOA Management Association Function) can M-GET any

lnpSubscriptions object.  The lnpSubscriptionsName attribute

is read only and can not be changed via the Local SMS Interface

once the object has been created.  The value of

lnpSubscriptionsName will always be "lnpSubscriptions".

Only one of these objects will exist and it will only be

created at startup of the CMIP agent software on the NPAC SMS

or the Local SMS.

The lnpDownloadPkg will only be used for a lnpSubscriptions

object instantiated on the NPAC SMS.  This package is used

for initiating downloading of subscriptionVersions and

numberPoolBlock object creation, deletion, or modifications

to the Local SMS (Data Download Association Function).

The subscriptionVersionOldSP-CreatePkg will only be used for a

lnpSubscriptions object instantiated on the NPAC SMS.  This

package is used for creation of subscription versions for

porting TNs by the old service provider.

The subscriptionVersionNewSP-CreatePkg will only be used for a

lnpSubscriptions object instantiated on the NPAC SMS.  This

package is used for creation of subscription versions for

porting TNs by the new service provider.

The subscriptionVersionDisconnectPkg will only be used for a

lnpSubscriptions object instantiated on the NPAC SMS.  This

package is used for disconnection of a ported TN by the current

service provider.

The subscriptionVersionModifyPkg will only be used for a

lnpSubscriptions object instantiated on the NPAC SMS.  This

package is used for modification of a ported TN by a service

provider.

The subscriptionVersionActivatePkg will only be used for a

lnpSubscriptions object instantiated on the NPAC SMS.  This

package is used for activation of a ported TN by a new service

provider.

The subscriptionVersionCancelPkg will only be used for a

lnpSubscriptions object instantiated on the NPAC SMS.  This

package is used for cancellation of a ported TN by a service

provider.

The subscriptionVersionOldSP-CancellationPkg will only be

used for a lnpSubscriptions object instantiated on the NPAC SMS.

This package is used for acknowledgment of subscription versions

with status values of cancel-pending. This action is used by

the old service provider SOA.

The subscriptionVersionNewSP-CancellationPkg will only be

used for a lnpSubscriptions object instantiated on the NPAC SMS.

This package is used for acknowledgment of subscription versions

with status values of cancel-pending. This action is used by

the new service provider SOA.

The subscriptionVersionRemoveFromConflictPkg will only be

used for a lnpSubscriptions object instantiated on the NPAC SMS.

This package is used for setting the status of subscription versions

with status values of conflict to pending.  This action is used by

either the new or old service provider SOA.

The numberPoolBlock-CreatePkg will only be used for a

lnpSubscriptions object instantiated on the NPAC SMS.  This

package is used for creation of number pool blocks.

The SOA receives subscriptionVersionRangeStatusAttributeValueChange

notifications if their Service Provider TN Range Notification

Indicator is set to TRUE on the NPAC SMS.  The

subscriptionVersionRangeStatusAttributeValueChangePkg is used to

send the subscriptionVersionRangeStatusAttributeValueChange

notification.  When this package is sent, it will include one set

of information for the TN range. If the SVIDs are sequential for

the TNs then an SVID range will be included. If the SVIDs are not

sequential then a paired list of SVIDs and TNs will be sent.

If the feature data does not apply to all TNs in the original

range, notifications will be broken up into smaller TN Range

Notifications such that the feature data applies to all TNs in

the smaller TN Range, and will be sent in separate messages.

The SOA receives subscriptionVersionRangeAttributeValueChange

notifications if their Service Provider TN Range Notification

Indicator is set to TRUE on the NPAC SMS. The

subscriptionVersionRangeAttributeValueChangePkg is used to send the

subscriptionVersionRangeAttributeValueChange notification.  When this

package is sent, it will include one set of information for the

TN range.  If the SVIDs are sequential for the TNs then an SVID range

will be included.  If the SVIDs are not sequential then a paired list

of SVIDs and TNs will be sent. If the feature data does not apply to

all TNs in the original range, notifications will be broken up into

smaller TN Range notifications such that the feature data applies to

all TNs in the smaller TN Range, and will be sent in separate messages.

The SOA receives subscriptionVersionRangeObjectCreation notifications

if their Service Provider TN Range Notification Indicator is set to

TRUE on the NPAC SMS. The subscriptionVersionRangeObjectCreationPkg

is used to send the subscriptionVersionRangeObjectCreation

notification.  When this package is sent, it will include one set

of information for the TN range, plus a paired list of TN/Subscription

Version ID combinations or a range of TNs and Subscription Version Ids

if the Subscription Version Ids are sequential. If the feature data

does not apply to all TNs in the range, notifications will be broken

up into smaller TN Range Notifications such that the feature data

applies to all TNs in the smaller TN Range, and will be sent in

separate messages.

The ObjectInfo field will contain the same data as the current object

creation notifications sent to the old and new service provider.  The

TN and SVID fields that are sent in the single object creation

notification will contain the TN and subscription version id for the

first TN in the range or list.

The SOA receives subscriptionVersionRangeDonorSP-CustomerDisconnectDate

notifications if their Service Provider TN Range Notification

Indicator is set to TRUE on the NPAC SMS. The

subscriptionVersionRangeDonorSP-CustomerDisconnectDatePkg is used to

send the subscriptionVersionRangeDonorSP-CustomerDisconnectDate

notification. When this package is sent, it will include one set

of information for the TN range, plus a paired list of TN/Subscription

Version ID combinations or a range of TNs and Subscription Version Ids

if the Subscription Version Ids are sequential. If the feature data

does not apply to all TNs in the range, notifications will be broken

up into smaller TN Range Notifications such that the feature data

applies to all TNs in the smaller TN Range, and will be sent in

separate messages.

The SOA receives subscriptionVersionRangeCancellationAcknowledge

notifications if their Service Provider TN Range Notification

Indicator is set to TRUE on the NPAC SMS. The

subscriptionVersionRangeCancellationAcknowledgePkg is used

to send the subscriptionVersionRangeCancellationAcknowledge

notification. When this package is sent, it will include one set

of information for the TN range, plus a paired list of TN/Subscription

Version ID combinations or a range of TNs and Subscription Version Ids

if the Subscription Version Ids are sequential. If the feature data

does not apply to all TNs in the range, notifications will be broken

up into smaller TN Range Notifications such that the feature data

applies to all TNs in the smaller TN Range, and will be sent in

separate messages.

The SOA receives subscriptionVersionRangeNewSP-CreateRequest

notifications if their Service Provider TN Range Notification

Indicator is set to TRUE on the NPAC SMS. The

subscriptionVersionRangeNewSP-CreateRequestPkg is used to send the

subscriptionVersionRangeNewSP-CreateRequest notification.

When this package is sent, it will include one set of information

for the TN range, plus a paired list of TN/Subscription Version ID

combinations or a range of TNs and Subscription Version Ids if

the Subscription Version Ids are sequential. If the feature data

does not apply to all TNs in the range, notifications will be broken

up into smaller TN Range Notifications such that the feature data

applies to all TNs in the smaller TN Range, and will be sent in

separate messages.

The SOA receives subscriptionVersionRangeOldSP-ConcurrenceRequest

notifications if their Service Provider TN Range Notification

Indicator is set to TRUE on the NPAC SMS. The

subscriptionVersionRangeOldSP-ConcurrenceRequestPkg is used to

send the subscriptionVersionRangeOldSP-ConcurrenceRequest

notification. When this package is sent, it will include one set

of information for the TN range, plus a paired list of TN/Subscription

Version ID combinations or a range of TNs and Subscription Version Ids

if the Subscription Version Ids are sequential. If the feature data

does not apply to all TNs in the range, notifications will be broken

up into smaller TN Range Notifications such that the feature data

applies to all TNs in the smaller TN Range, and will be sent in

separate messages.

The SOA receives

subscriptionVersionRangeOldSPFinalConcurrenceWindowExpiration

notifications if their Service Provider TN Range Notification

Indicator is set to TRUE on the NPAC SMS. The

SubscriptionVersionRangeOldSPFinalConcurrenceWindow-ExpirationPkg

is used to send the

subscriptionVersionRangeOldSPFinalConcurrenceWindowExpiration

notification. When this package is sent, it will include one set

of information for the TN range, plus a paired list of TN/Subscription

Version ID combinations or a range of TNs and Subscription Version Ids

if the Subscription Version Ids are sequential. If the feature data

does not apply to all TNs in the range, notifications will be broken

up into smaller TN Range Notifications such that the feature data

applies to all TNs in the smaller TN Range, and will be sent in

separate messages.

The SOA receives

subscriptionVersionRangeNewSP-FinalCreateWindowExpiration

notifications if their Service Provider TN Range Notification

Indicator is set to TRUE on the NPAC SMS. The

SubscriptionVersionRangeNewSP-FinalCreateWindow-ExpirationPkg is

used to send the

subscriptionVersionRangeNewSP-FinalCreateWindowExpiration

notification. When this package is sent, it will include one set

of information for the TN range, plus a paired list of TN/Subscription

Version ID combinations or a range of TNs and Subscription Version Ids

if the Subscription Version Ids are sequential. If the feature data

does not apply to all TNs in the range, notifications will be broken

up into smaller TN Range Notifications such that the feature data

applies to all TNs in the smaller TN Range, and will be sent in

separate messages.

Range notifications are formatted according to the Service Provider

Profile.  If a Service Provider is an associated Service Provider

to a primary Service Provider then the primary Service Provider SOA

must be able to accept the notifications in the format indicated in

the associated Service Provider Profile.

!;

-- 15.0 LNP Service Provider Managed Object Class

serviceProv MANAGED OBJECT CLASS

DERIVED FROM serviceProvNetwork;

CHARACTERIZED BY

serviceProvPkg;

CONDITIONAL PACKAGES

serviceProvBillingAddressPkg PRESENT IF

!the service provider has billing address and contact

information!,

serviceProvSOA-AddressPkg PRESENT IF

!the service provider has SOA address and contact information!,

serviceProvLSMS-AddressPkg PRESENT IF

!the service provider has LSMS address and contact information!,

serviceProvWebAddressPkg PRESENT IF

!the service provider has Web address and contact information!,

serviceProvNetAddressPkg PRESENT IF

!the service provider has network and communication facilities

address and contact information!,

serviceProvConflictAddressPkg PRESENT IF

!the service provider has conflict resolution interface

address and contact information!,

serviceProvOperationsAddressPkg PRESENT IF

!the service provider has operations address and contact

information!,

serviceProvRepairCenterInfoPkg PRESENT IF

!the service provider has repair contact information!,

serviceProvSecurityAddressPkg PRESENT IF

!the service provider has security contact information!,

serviceProvUserAdminAddressPkg PRESENT IF

!the service provider has user administration interface address

and contact information!;

REGISTERED AS {LNP-OIDS.lnp-objectClass 15};

serviceProvPkg PACKAGE

BEHAVIOUR

serviceProvDefinition,

serviceProvBehavior;

ATTRIBUTES

npacCustomerAllowableFunctions GET,

serviceProvAddress GET-REPLACE,

serviceProvSysLinkInfo GET-REPLACE;

;

serviceProvDefinition BEHAVIOUR

DEFINED AS !

The serviceProv class is the managed object

used on the NPAC SMS to contain the data related to each

LNP service provider.

!;

serviceProvBehavior BEHAVIOUR

DEFINED AS !

NPAC SMS Managed Object used for the Local SMS to NPAC

SMS interface.

A Local SMS and service provider SOA can M-GET their

serviceProv object (Network Data Association Function).

Attempts to read any unauthorized service provider information

will be rejected.

All attributes in this object, except serviceProvID

and npacCustomerAllowableFunctions can be M-SET by the Local SMS

and SOA Interfaces once the object has been created on the NPAC SMS.

If any of the contact data is modified, ALL contact information must

be sent. Any missing contact data is presumed to be a deletion if

the data currently exists on the NPAC SMS. Note: this behaviour is

not following current CMIP standards.

!;

-- 16.0 LNP Service Provider LRN Managed Object Class

serviceProvLRN MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

serviceProvLRN-Pkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 16};

serviceProvLRN-Pkg PACKAGE

BEHAVIOUR

serviceProvLRN-Definition,

serviceProvLRN-Behavior;

ATTRIBUTES

serviceProvLRN-ID GET,

serviceProvLRN-Value GET,

serviceProvDownloadReason GET,

serviceProvLRN-CreationTimeStamp GET;

;

serviceProvLRN-Definition BEHAVIOUR

DEFINED AS !

The serviceProvLRN class is the managed object

used to identify Service Provider LRN values open for

porting.

!;

serviceProvLRN-Behavior BEHAVIOUR

DEFINED AS !

Local SMS and NPAC SMS Managed Object used for the Local SMS to

NPAC SMS and SOA to NPAC SMS interfaces.

All attributes are read only. Once created, the serviceProvLRN

object can only be deleted via the Local SMS or SOA interface.

The serviceProvLRN-ID is specified by the NPAC SMS. The

serviceProvLRN-CreationTimeStamp will reflect the current system

date and time when the object is created.

NPAC SMS can M-GET, M-DELETE and M-CREATE any

serviceProvLRN object on the Local SMS or SOA (Network Data

Functional Unit).  The Local SMS or SOA only creates local

copies of serviceProvLRN objects after receiving the

objects from an NPAC SMS create request, reading them from

the NPAC SMS for initial instantiation, or from a download request.

A Local SMS or SOA can M-GET any serviceProvLRN object (Network Data

Functional Unit).

The Local SMS or SOA can M-DELETE and M-CREATE any

serviceProvLRN object on the NPAC SMS for the

service provider id specified in the access control

(Network Data Functional Unit).  Attempts to take

actions on unauthorized service provider objects will be

rejected.

The creation or deletion of a serviceProvLRN object will be

distributed to all Local SMSs and SOAs.

The serviceProvLRN-Value attributes on the NPAC SMS can

not be modified by the Local SMS or SOA.  The service

provider will have to add a new object and delete the

old one to modify the data.

!;

-- 17.0 LNP Service Provider Network Managed Object Class

serviceProvNetwork MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

serviceProvNetworkPkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 17};

serviceProvNetworkPkg PACKAGE

BEHAVIOUR

serviceProvNetworkDefinition,

serviceProvNetworkBehavior;

ATTRIBUTES

serviceProvID GET,

serviceProvName GET-REPLACE;

;

serviceProvNetworkDefinition BEHAVIOUR

DEFINED AS !

The serviceProvNetwork class is the managed object

used to contain the network data for a service provider.

!;

serviceProvNetworkBehavior BEHAVIOUR

DEFINED AS !

Local SMS, SOA, and NPAC SMS Managed Object used for the

Local SMS to NPAC SMS interface and the SOA to NPAC SMS interface.

Service providers and the NPAC SMS can M-GET, M-CREATE, and M-SET

any serviceProvNetwork object (Network Data Association Function).

The serviceProvId attribute is read only and can not be

changed via the NPAC SMS to Local SMS Interface or SOA

to NPAC SMS interface once the object has been created on

the Local SMS, SOA or NPAC SMS.  The serviceProvName can be

M-SET via the NPAC SMS to Local SMS Interface or the SOA to

NPAC SMS interface by the NPAC SMS.  The Local SMS and SOA

only create or modify local copies of serviceProvNetwork objects

after receiving the objects from an NPAC SMS M-CREATE or M-SET

request or reading them from the NPAC SMS for initial instantiation.

!;

-- 18.0 LNP Service Provider NPA-NXX Managed Object Class

serviceProvNPA-NXX MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

serviceProvNPA-NXX-Pkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 18};

serviceProvNPA-NXX-Pkg PACKAGE

BEHAVIOUR

serviceProvNPA-NXX-Definition,

serviceProvNPA-NXX-Behavior;

ATTRIBUTES

serviceProvNPA-NXX-ID GET,

serviceProvNPA-NXX-Value GET,

serviceProvNPA-NXX-EffectiveTimeStamp GET,

serviceProvDownloadReason GET,

serviceProvNPA-NXX-CreationTimeStamp GET;

;

serviceProvNPA-NXX-Definition BEHAVIOUR

DEFINED AS !

The serviceProvNPA-NXX class is the managed object

used to identify Service Provider NPA-NXX values open for

porting.

!;

serviceProvNPA-NXX-Behavior BEHAVIOUR

DEFINED AS !

Local SMS, SOA, and NPAC SMS Managed Object used for the Local

SMS to NPAC SMS interface and the SOA to NPAC SMS interface.

All attributes are read only. Once created, the serviceProvNPA-NXX

object can only be deleted via the Local SMS or SOA interface.  The

serviceProvNPA-NXX-ID is specified by the NPAC SMS. The

serviceProvNPA-NXX-CreationTimeStamp will be set to the current

system date and time when the object is created.

NPAC SMS can M-GET, M-DELETE and M-CREATE any serviceProvNPA-NXX

object on the Local SMS or SOA (Network Data Association Function).

The Local SMS or SOA only creates local copies of

serviceProvNPA-NXX objects after receiving the objects from

an NPAC SMS create, after reading them from the NPAC SMS for

initial instantiation, or from a download.

Service providers can M-GET any serviceProvNPA-NXX object.

A Local SMS or SOA can M-DELETE and M-CREATE any

serviceProvNPA-NXX object on the NPAC SMS for their service

provider id specified in the access control

(Network Data Association Function).  Attempts to

take actions on unauthorized service provider objects will

be rejected.

A Local SMS or SOA can not modify any of the attributes.

To cause an NPA-NXX split to occur the service provider must

contact the NPAC SMS operations personnel.

!;

-- 19.0 LNP Subscription Audit Managed Object

subscriptionAudit MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

subscriptionAuditPkg;

CONDITIONAL PACKAGES

subscriptionAuditTN-ActivationRangePkg PRESENT IF

!the audit is being done on an activation data range!;

REGISTERED AS {LNP-OIDS.lnp-objectClass 19};

subscriptionAuditPkg PACKAGE

BEHAVIOUR

subscriptionAuditDefinition,

subscriptionAuditBehavior;

ATTRIBUTES

subscriptionAuditId GET,

subscriptionAuditName GET,

subscriptionAuditStatus GET,

subscriptionAuditAttributeList GET,

subscriptionAuditTN-Range GET,

subscriptionAuditServiceProvIdRange GET,

subscriptionAuditNumberOfTNs GET,

subscriptionAuditNumberOfTNsComplete GET,

subscriptionAuditRequestingSP GET;

NOTIFICATIONS

subscriptionAuditResults,

subscriptionAudit-DiscrepancyRpt,

"CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":attributeValueChange

accessControlParameter,

"CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":objectCreation

accessControlParameter,

"CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":objectDeletion

accessControlParameter;

;

subscriptionAuditDefinition BEHAVIOUR

DEFINED AS !

The subscriptionAudit class is the managed object that

represents a subscription audit request.  This object is

only instantiated on the NPAC SMS.

!;

subscriptionAuditBehavior BEHAVIOUR

DEFINED AS !

When the subscriptionAuditStatus changes an attribute value

change will be emitted to the audit requester.

All attributes must be specified upon create with the exception

of the subscriptionAuditTN-ActivationRange, if an audit is

not being performed on an activation date range.  If the

subscriptionAuditAttributeList is not specified then a full

audit is assumed. If the subscriptionAuditTN-ActivationRange is

specified then an audit of all TNs in the range specified in

subscriptionAuditTN-Range will be audited.  The serviceAuditId

is determined by the NPAC SMS.

The subscriptionAuditRequestingSP is the id of the service provider

who requested the audit.

The subscriptionAuditTN-Range is the range of TNs to be audited.

If only a single TN is to be audited, the ending TN station should be

set to the value of the starting TN station.

The NPAC SMS will be required to set the number of TNs that

will be audited in the subscriptionAuditNumberOfTNs attribute

based on the NPAC SMS audit request criteria.

The SOA or NPAC SMS can M-CREATE, M-GET subscriptionAudit

managed objects on the NPAC SMS (SOA Management Function).

When a subscriptionAudit object is created on the NPAC SMS the

NPAC SMS will begin the audit for the service provider specified

or all service providers.  The SOA can only M-GET subscriptionAudit

that they created.

The SOA will be required to set the requesting SP with

their service provider id so that the origination of the audit

request can be tracked and notifications can be sent to the

requesting SOA.

When this object is created and deleted, object creation and

deletion notifications will be sent to the requester.  Object

deletion indicates completion of an audit.  The audit results

notification will be sent before the object is deleted by the

entity performing the audit indicating how many discrepancies the

audit found and reported during execution.

If discrepancies are found during the audit, audit discrepancy

notifications will be sent to the requester at the time they

are found.  When audit discrepancy notifications are sent by

the NPAC SMS to the requesting SOA, create, modify or delete

requests will be sent to the Local SMS by the NPAC SMS to correct the

discrepancies found.

For audits involving TNs that are within a pooled block or

subscription versions with an LNPType of 'pool', the NPAC SMS

will query subscription version objects from the non-EDR Local

SMS and it will query the number pool block object and subscription

version objects from the EDR Local SMS. The EDR Local SMS should

not return any subscription versions with an LNPType of 'pool'.

The NPAC SMS will report to the SOA the discrepancies with

subscription version identifiers. Thus, if a numberPoolBlock object

is in error, the discrepancy will be reported as all TNs within the

audit range. Subscription version discrepancies will be reported

as usual.

If a subscription version or a number pool block is in a state

of 'sending' at the time of the audit, the NPAC SMS will not perform

comparisons and corrections to the requested objects.

Deletion of an audit object cancels an audit request.

The purge of audits are based on the tunable "Audit Log

Retention Period" which defaults to 90 days.

!;

-- 20.0 LNP subscription Version Managed Object Class

subscriptionVersion MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

subscriptionVersionPkg;

CONDITIONAL PACKAGES

subscriptionWSMSC-DataPkg PRESENT IF

!the service provider is supporting WSMSC information!;

REGISTERED AS {LNP-OIDS.lnp-objectClass 20};

subscriptionVersionPkg PACKAGE

BEHAVIOUR

subscriptionVersionDefinition,

subscriptionVersionBehavior;

ATTRIBUTES

subscriptionVersionId GET,

subscriptionTN GET,

subscriptionLRN GET-REPLACE,

subscriptionNewCurrentSP GET-REPLACE,

subscriptionActivationTimeStamp GET-REPLACE,

subscriptionCLASS-DPC GET-REPLACE,

subscriptionCLASS-SSN GET-REPLACE,

subscriptionLIDB-DPC GET-REPLACE,

subscriptionLIDB-SSN GET-REPLACE,

subscriptionCNAM-DPC GET-REPLACE,

subscriptionCNAM-SSN GET-REPLACE,

subscriptionISVM-DPC GET-REPLACE,

subscriptionISVM-SSN GET-REPLACE,

subscriptionEndUserLocationValue GET-REPLACE,

subscriptionEndUserLocationType GET-REPLACE,

subscriptionBillingId GET-REPLACE,

subscriptionLNPType GET-REPLACE,

subscriptionDownloadReason GET-REPLACE;

;

subscriptionVersionDefinition BEHAVIOUR

DEFINED AS !

The subscriptionVersion class is the managed object that

represents a subscription version on the Local SMS.

!;

subscriptionVersionBehavior BEHAVIOUR

DEFINED AS !

Local SMS Managed Object used for the NPAC SMS to Local SMS

Interface.

NPAC SMS can M-GET (Query Association Function), M-SET, M-DELETE

and M-CREATE (Data Download Association Function) any

subscriptionVersion object on the Local SMS (Data Download

Association Function).  The Local SMS only creates local copies

of subscriptionVersion objects after receiving the objects from an

NPAC SMS create request or reading them from the NPAC SMS for

initial instantiation.

The subscriptionVersionId is assigned upon creation by the

NPAC SMS and is read only.

The subscriptionTN, subscriptionLRN and associated routing

information, are specified by the new service provider SOA upon

creation of a new subscription version.

The subscriptionActivationTimeStamp is set by the NPAC SMS as the

current date and time when the subscriptionVersion activation

request is received from the new service provider.

The Local SMS will receive the subscriptionWSMSC-DataPkg

attributes if the 'NPAC New Functionality Support' indicator

is set for the 'LSMS WSMSC DPC SSN Data' in their service

provider profile on the NPAC SMS.

When the subscription version is downloaded to the locals, the

subscriptionDownloadReason is set to one of new, delete, modified,

or audit-discrepancy. This field is not validated in audits.

When the subscription version disconnect is broadcast,

the subscriptionVersionDonorSP-CustomerDisconnectDate is

sent to the donor SOA informing the service provider of the actual

customer disconnect date.

The Local SMS can not modify any of the subscription version

data locally unless changes were downloaded via a download

request.

The Local SMS must be able to support a scoped request (M-GET,

M-SET, M-DELETE) with a filter for equality and ordering on the

subscriptionTN from the NPAC SMS.

!;

-- 21.0 LNP NPAC Subscription Version Managed Object Class

subscriptionVersionNPAC MANAGED OBJECT CLASS

DERIVED FROM subscriptionVersion;

CHARACTERIZED BY

subscriptionVersionNPAC-Pkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 21};

subscriptionVersionNPAC-Pkg PACKAGE

BEHAVIOUR

subscriptionVersionNPAC-Definition,

subscriptionVersionNPAC-Behavior;

ATTRIBUTES

subscriptionVersionStatus GET-REPLACE,

subscriptionOldSP GET-REPLACE,

subscriptionNewSP-DueDate GET-REPLACE,

subscriptionNewSP-CreationTimeStamp GET-REPLACE,

subscriptionOldSP-DueDate GET-REPLACE,

subscriptionOldSP-Authorization GET-REPLACE,

subscriptionStatusChangeCauseCode GET-REPLACE,

subscriptionOldSP-AuthorizationTimeStamp GET-REPLACE,

subscriptionBroadcastTimeStamp GET-REPLACE,

subscriptionConflictTimeStamp GET-REPLACE,

subscriptionCustomerDisconnectDate GET-REPLACE,

subscriptionEffectiveReleaseDate GET-REPLACE,

subscriptionDisconnectCompleteTimeStamp GET-REPLACE,

subscriptionCancellationTimeStamp GET-REPLACE,

subscriptionCreationTimeStamp GET-REPLACE,

subscriptionFailed-SP-List GET-REPLACE,

subscriptionModifiedTimeStamp GET-REPLACE,

subscriptionOldTimeStamp GET-REPLACE,

subscriptionOldSP-CancellationTimeStamp GET-REPLACE,

subscriptionNewSP-CancellationTimeStamp GET-REPLACE,

subscriptionOldSP-ConflictResolutionTimeStamp GET-REPLACE,

subscriptionNewSP-ConflictResolutionTimeStamp GET-REPLACE,

subscriptionPortingToOriginal-SPSwitch GET-REPLACE,

subscriptionPreCancellationStatus GET-REPLACE,

subscriptionTimerType GET,

subscriptionBusinessType GET;

NOTIFICATIONS

subscriptionVersionOldSP-ConcurrenceRequest,

subscriptionVersionNewSP-CreateRequest,

subscriptionVersionOldSPFinalConcurrenceWindowExpiration,

subscriptionVersionNewNPA-NXX,

subscriptionVersionCancellationAcknowledgeRequest,

subscriptionVersionDonorSP-CustomerDisconnectDate,

subscriptionVersionStatusAttributeValueChange,

subscriptionVersionNewSP-FinalCreateWindowExpiration,

"CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":attributeValueChange

accessControlParameter,

"CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":objectCreation

accessControlParameter;

;

subscriptionVersionNPAC-Definition BEHAVIOUR

DEFINED AS !

The subscriptionVersionNPAC class is the managed object

that represents a subscription version on the NPAC SMS.

!;

subscriptionVersionNPAC-Behavior BEHAVIOUR

DEFINED AS !

NPAC SMS Managed Object for the SOA to NPAC SMS and the Local SMS

to NPAC SMS interface.

A Local SMS can M-GET any subscriptionVersionNPAC objects

from the NPAC SMS via the Local SMS Interface (Data Download

Association Function).

A Service Provider SOA can M-GET any subscriptionVersionNPAC

objects from the NPAC SMS via the SOA Interface (SOA Management

Association Function).

If a Service Provider SOA or Local SMS does a scoped filtered

M-GET for subscription versions, this request will only be

successful if the number of records to be returned is less

than or equal to the NPAC SMS tunable parameter,

"Max Subscriber Query", in the Service Data table.

When the status of an object is changed to "cancel-pending",

subscriptionPreCancellationStatus is first set to the current status.

The subscriptionCreationTimeStamp is set to the current system

time when the object is created.

When the subscription version is modified for any reason, the

subscriptionModifiedTimeStamp is updated with the current system time.

When the subscription version is broadcast to Local SMSs via

the NPAC to Local SMS interface, the subscriptionBroadcastTimeStamp

is updated with the current system time.

When the subscription version has its version status set to old,

the subscriptionOldTimeStamp is updated with the current system

time.

When the subscription version has its version status set to cancel,

the subscriptionCancellationTimeStamp is updated with the

current system time.

When the subscription version has its version status set to

conflict, the subscriptionConflictTimeStamp is updated with the

current system time.

When the subscription version is being disconnected and the first

successful response is received from the Local SMS to the

subscription version disconnect request, the

subscriptionDisconnectCompleteTimeStamp is updated with the current

date and time.

When the subscription version status is set to disconnect pending

the subscriptionEffectiveReleaseDate is set to the date the

disconnect should be broadcast.

When the subscription version in a cancel-pending

state is acknowledged by an old service provider SOA, the

subscriptionOldSP-CancellationTimeStamp is updated with the

current system time.

When the subscription version in a cancel-pending

state is acknowledged by a new service provider SOA, the

subscriptionNewSP-CancellationTimeStamp is updated with the

current system time.

When the subscription version in a conflict state is removed

from conflict by the old service provider SOA, the

subscriptionOldSP-ConflictResolutionTimeStamp is updated with the

current system time.

When the subscription version in a conflict state is removed

from conflict by the new service provider SOA, the

subscriptionNewSP-ConflictResolutionTimeStamp is updated with

the current system time.

When the subscription version broadcast is not successful to all

service providers, the subscriptionFailed-SP-List is populated with a

list of the failed service providers.

When the subscription version is created, the subscriptionTimerType

is set according to the type of timer the old and new service

providers support.

When the subscription version is created, the

subscriptionBusinessType is set according to the business days and

hours selection of the old and new service providers.

When the subscription version is created, modified, or mass updated

and contains a DPC value, a valid, corresponding SSN value must be

present and vice versa. An SSN value of 000 (zero) must be present

when its corresponding DPC value is populated, if the corresponding

SSN Edit Flag Indicator is set to TRUE. When modifying either DPC

or SSN, both values must be sent to the NPAC SMS in the request,

even if only one is being modified.

The Service Provider SOA can M-GET and M-SET subscriptionVersionNPAC

objects via the SOA to NPAC SMS interface (SOA Management Association

Function).  Rules for M-SET are described below.

For M-GET requests, the filter will support all attributes for

a specified ported TN.

Any service provider SOA can view any subscription version for any

active ported TN (SOA Management Association Function).

Subscription versions are created on the NPAC SMS via actions over

the SOA to NPAC SMS interface to the lnpSubscriptions object (SOA

Management Association Function).  New service provider SOAs must use

the subscriptionVersionNewSP-Create action and old service

provider SOAs must use the subscriptionVersionOldSP-Create action.

Creates can be performed provided there is only one

currently active subscription version for the TN.

subscriptionPortingToOriginal-SPSwitch can only be specified as

TRUE for a TN that is currently ported and is being ported back

to the original service provider, along with the home switch of

the NPA-NXX.   If the value of subscriptionPortingToOriginal-SPSwitch

is TRUE, the LRN and GTT data should not be specified.  This data

is not specified because when the activate occurs for the subscription

version, the Local SMS will receive requests to delete the old

subscription version routing data in their networks and they will not

receive any new network routing data for the subscription. Concurrence

from the old service provider is required. If the value of

subscriptionPortingToOriginal-SPSwitch is TRUE, the LNP

Type may be either "LSPP" or "LISP".

If the port of the subscription version is an intra-service

provider port, the new service provider SOA can use the

subscriptionVersionNewSP-Create action specifying the old service

provider equal to the new service provider.  In this case, the

old service provider create action is not required and processing

proceeds after a valid pending version is created in the same

manner as it does for inter-service provider porting.

The new service provider SOA must provide the

subscriptionWSMSC-DataPkg attributes if the 'NPAC New Functionality

Support' indicator is set for the 'SOA WSMSC DPC SSN Data'

in their service provider profile on the NPAC SMS.

Once a version has been created that passes validation, the

subscriptionVersionNPAC object subscriptionVersionStatus will

be set to pending and an object creation notification will be sent

to both old and new service provider SOAs.  If a version

previously existed, attribute value change notifications will be

sent to both old and new service provider SOAs.

If there is a pending version that does not have concurrence during

the "Service Provider Concurrence Window" specified in the Service

Data table, a subscriptionVersionConcurrenceRequest notification will be

sent to the service provider SOA that has not responded.  The

subscriptionVersionStatus will be set to cancel if the new

service provider SOA has not responded or to conflict if the old

service provider SOA has not responded after the "Service Provider

Final Concurrence Window".  A status attribute value change

will be sent to both service providers.

The Service Provider SOA can M-SET or use an M-ACTION to modify

attributes associated with pending or conflict

subscription versions (SOA Management Association Function).

Attempts to modify an active, sending, failed, partial failure,

canceled, cancel-pending, disconnect-pending or old version using

an M-SET will result in an access denied error.

Modification of an active subscription can be done only by the

current/new service provider SOA using the subscriptionVersionModify

action.

Old service provider SOAs can only modify the following attributes:

subscriptionOldSP-DueDate

subscriptionOldSP-Authorization

subscriptionStatusChangeCauseCode

New service provider SOAs can only modify the following attributes:

subscriptionLRN

subscriptionNewSP-DueDate

subscriptionCLASS-DPC

subscriptionCLASS-SSN

subscriptionLIDB-DPC

subscriptionLIDB-SSN

subscriptionCNAM-DPC

subscriptionCNAM-SSN

subscriptionISVM-DPC

subscriptionISVM-SSN

subscriptionWSMSC-DPC

subscriptionWSMSC-SSN

subscriptionEndUserLocationValue

subscriptionEndUserLocationType

subscriptionBillingId

Upon subscription version creation, the subscriptionOldSP-DueDate

and subscriptionNewSP-DueDate must match. If the due date for the

port is a previous date, the NPAC SMS accepts a value of a previous

date from a service provider, in order to match the due date of the

port that was previously received from the other Service Provider

(new or old).

Validation will be done for both old and new service

provider data that is specified on an M-SET.  If validation fails,

no changes will be made and a processing failure will be returned.

If the version passes validation, the version status will be

set to pending.  An error message will be returned to the service

provider if the status is not pending when they attempt to change

the version status to cancel-pending.

Once a pending version has been created, the new service provider

can activate the subscription version if the new

service provider due date has been reached and the NPA-NXX effective

date has been reached.

Once the version is activated, the version status is set to

sending, the broadcast time stamp is updated, and creates

are sent to the Local SMSs.

If the create requests are successful for all Local SMSs, the

version status will be marked as active and the previously active

subscription version will have its version status set to old.

If create requests fail for a subscription version after the

retry periods have expired, the version status will be set

to failed or partial failure based on whether the download failed

in all or some of the Local SMSs respectively.

Prior to the subscription version becoming active, a status version

attribute value change will be sent to both old

and new service providers when the subscriptionVersionStatus is

modified.  If the version status is failed or partial failure then

a list of failed service providers is provided in the

subscriptionVersionStatus notification.

A subscription version can be put into conflict by either the NPAC

SMS or by the old service provider explicitly setting the

subscriptionOldSP-Authorization off.

If the old service provider explicitly sets the

subscriptionOldSP-Authorization off, the

subscriptionStatusChangeCauseCode must be given.

The old service provider can only put a subscription version into

conflict once and only by explicitly setting the

subscriptionOldSP-Authorization off.

A service provider should acknowledge the cancel pending

state within a tunable time frame specified on the NPAC SMS

with a cancel acknowledgement action.

If a new service provider SOA fails to acknowledge the cancel pending

state, a subscriptionVersionCancellationAcknowledgeRequest is

sent to the service provider SOA.  If they do not respond to this

acknowledgement in a tunable time frame specified on the NPAC SMS,

the version status will be set to conflict. If the old service

fails to acknowledge the cancel pending state, the subscription

version status will be set to cancel.

Attribute value change notifications will be sent to both service

provider SOAs when the following attribute values change for

a pending, cancel-pending, or conflict subscription

versions (AVCs are not sent for modify active subscription

versions):

subscriptionNewSP-DueDate

subscriptionNewSP-CreationTimeStamp

subscriptionOldSP-DueDate

subscriptionOldSP-Authorization

subscriptionOldSP-AuthorizationTimeStamp

subscriptionStatusChangeCauseCode

subscriptionVersionStatus

Object creation notifications will be sent to both old and

new service provider SOAs when a subscriptionVersionNPAC associated

with their Service Provider id is created.  Object deletion

notifications will not be used. Objects will only be deleted by the

NPAC SMS as a result of housekeeping processing.

When a subscription version disconnect is broadcast,

the subscriptionVersionDonorSP-CustomerDisconnectDate

is sent to the donor SOA informing the service provider of the

actual customer disconnect date.

The subscriptionTimerType is only returned on SOA queries to service

providers that support the timer information and is only sent on

object creation notifications to service providers that support it.

The subscriptionBusinessType is only returned on SOA queries to

service providers that support business days/hours and is only

sent on object creation notifications to service providers that

support business days/hours.

For number pooling, the number pool block has corresponding

subscription versions with LNP type equal to 'pool'. The number

pool block status represents the status of the number pool block

and all subscription versions. The status of each subscription

version contains the results of the broadcast of that subscription

version to the non-EDR Local SMS and the result of the number pool

block broadcast to the EDR Local SMSs. If a subscription version

with LNP type equal to 'pool' fails a broadcast, the

failed service provider list on both that subscription version and

the number pool block will be set to the service provider(s)

who failed. If an EDR Local SMS fails the number pool block

broadcast, each subscription version with LNP type equal to 'pool'

within the number pool block TN range will contain that service

provider in the subscriptionFailed-SP-List.

Creation, modification or deletion of a subscription version

with subscriptionLNPType equal to 'pool' by the SOA is not

allowed. All updates must be performed on the number pool block

object. The NPAC SMS will update and broadcast the

subscription versions with LNP type equal to 'pool' that

are within the TN range specified by the number pool block object.

!;

-- 22.0 LNP Log Record for the Subscription Version Donor Service Provider

--      Customer Disconnect Date Notification

lnpLogDonorSP-CustomerDisconnectDateRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":eventLogRecord;

CHARACTERIZED BY

lnpLogDonorSP-CustomerDisconnectDatePkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 22};

lnpLogDonorSP-CustomerDisconnectDatePkg PACKAGE

BEHAVIOUR

lnpLogDonorSP-CustomerDisconnectDateDefinition,

lnpLogDonorSP-CustomerDisconnectDateBehavior;

ATTRIBUTES

subscriptionTN GET,

subscriptionVersionId GET,

subscriptionCustomerDisconnectDate GET,

subscriptionEffectiveReleaseDate GET,

accessControl GET;

;

lnpLogDonorSP-CustomerDisconnectDateDefinition BEHAVIOUR

DEFINED AS !

The lnpLogDonorSP-CustomerDisconnectDateRecord class is the managed

object that is used to create log records for the

subscriptionVersionDonorSP-CustomerDisconnectDate Notification.

!;

lnpLogDonorSP-CustomerDisconnectDateBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

subscriptionVersionDonorSP-CustomerDisconnectDate Notification.

!;

-- 23.0 LNP Log Record for the Subscription Version Local SMS Action Results

--      Notification

lnpLogLocalSMS-ActionResultsRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":eventLogRecord;

CHARACTERIZED BY

lnpLogLocalSMS-ActionResultsPkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 23};

lnpLogLocalSMS-ActionResultsPkg PACKAGE

BEHAVIOUR

lnpLogLocalSMS-ActionResultsDefinition,

lnpLogLocalSMS-ActionResultsBehavior;

ATTRIBUTES

actionId GET,

actionResultsStatus GET,

failedTN-List GET,

resultsCompletionTime GET,

accessControl GET;

;

lnpLogLocalSMS-ActionResultsDefinition BEHAVIOUR

DEFINED AS !

The lnpLogLocalSMS-ActionResultsRecord class is the managed

object that is used to create log records for the

subscriptionVersionLocalSMS-ActionResults Notification.

!;

lnpLogLocalSMS-ActionResultsBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

subscriptionVersionLocalSMS-ActionResults Notification.

!;

-- 24.0 LNP Log Record for the Subscription Version New NPA-NXX Notification

lnpLogNewNPA-NXXRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":eventLogRecord;

CHARACTERIZED BY

lnpLogNewNPA-NXXPkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 24};

lnpLogNewNPA-NXXPkg PACKAGE

BEHAVIOUR

lnpLogNewNPA-NXXDefinition,

lnpLogNewNPA-NXXBehavior;

ATTRIBUTES

serviceProvNPA-NXX-ID GET,

serviceProvNPA-NXX-Value GET,

serviceProvNPA-NXX-EffectiveTimeStamp GET,

serviceProvID GET,

accessControl GET;

;

lnpLogNewNPA-NXXDefinition BEHAVIOUR

DEFINED AS !

The lnpLogNewNPA-NXX class is the managed

object that is used to create log records for the

subscriptionVersionNewNPA-NXX Notification.

!;

lnpLogNewNPA-NXXBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

subscriptionVersionNewNPA-NXX Notification.

!;

-- 25.0 LNP Service Provider Filter NPA-NXX Managed Object Class

lsmsFilterNPA-NXX MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

lsmsFilterNPA-NXX-Pkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 25};

lsmsFilterNPA-NXX-Pkg PACKAGE

BEHAVIOUR

lsmsFilterNPA-NXX-Definition,

lsmsFilterNPA-NXX-Behavior;

ATTRIBUTES

lsmsFilterNPA-NXX-ID GET,

lsmsFilterNPA-NXX-Value GET;

;

lsmsFilterNPA-NXX-Definition BEHAVIOUR

DEFINED AS !

The lsmsFilterNPA-NXX class is the managed object

used to identify the NPA-NXX values for which a service provider

does not want to be informed of subscription version broadcasts,

network downloads, or SOA notifications.

!;

lsmsFilterNPA-NXX-Behavior BEHAVIOUR

DEFINED AS !

NPAC SMS Managed Object used for the Local SMS to NPAC SMS interface

and the NPAC SMS to SOA interface.

All attributes are read only. Once created, the lsmsFilterNPA-NXX

object can be deleted via the Local SMS or SOA interface.  The

lsmsFilterNPA-NXX-ID is specified by the NPAC SMS.

The Local SMS or SOA can M-DELETE, M-CREATE and M-GET the

lsmsFilterNPA-NXX objects on the NPAC SMS.  (LSMS Network Data

Association Function).

!;

-- 26.0 LNP Log Record for the Subscription Version Final Concurrence

-- Timer Expiration

lnpLogOldSPFinalConcurrenceWindowExpirationRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":eventLogRecord;

CHARACTERIZED BY

lnpLogOldSPFinalConcurrenceWindowExpirationPkg;

CONDITIONAL PACKAGES

subscriptionTimerTypePkg PRESENT IF

!present if the Old SP SOA supports timer type!,

subscriptionBusinessTypePkg PRESENT IF

!present if the Old SP SOA supports business type!;

REGISTERED AS {LNP-OIDS.lnp-objectClass 26};

lnpLogOldSPFinalConcurrenceWindowExpirationPkg PACKAGE

BEHAVIOUR

lnpLogOldSPFinalConcurrenceWindowExpirationDefinition,

lnpLogOldSPFinalConcurrenceWindowExpirationBehavior;

ATTRIBUTES

subscriptionTN GET,

subscriptionVersionId GET,

accessControl GET;

;

lnpLogOldSPFinalConcurrenceWindowExpirationDefinition BEHAVIOUR

DEFINED AS !

The lnpLogOldSPFinalConcurrenceWindowExpirationRecord class is

the managed object that is used to create log records for the

subscriptionVersionOldSPFinalConcurrenceWindowExpiration

Notification.

!;

lnpLogOldSPFinalConcurrenceWindowExpirationBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

subscriptionVersionOldSPFinalConcurrenceWindowExpiration

Notification.

!;

-- 27.0 LNP SOA Managed Object Class

lnpSOA MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

lnpSOA-Pkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 27};

lnpSOA-Pkg PACKAGE

BEHAVIOUR

lnpSOA-Definition,

lnpSOA-Behavior;

ATTRIBUTES

lnpSOA-Name GET;

;

lnpSOA-Definition BEHAVIOUR

DEFINED AS !

The lnpSOA class is the managed object that is used as the

container object for all SOA data in the SOA to NPAC SMS

Interface.

!;

lnpSOA-Behavior BEHAVIOUR

DEFINED AS !

SOA Managed Object.

The NPAC SMS can M-GET any lnpSOA object (Data Download

Association Function).

The lnp-SOA-Name attribute is read only and can not

be changed via the SOA Interface once the object has

been created.  The value of lnpSOA-Name will always be

a unique identifier for the SOA for the SOA to NPAC

Interface.

Only one of these objects will exist and it will only be

created at startup of the CMIP agent software on the SOA.

!;

-- 28.0 LNP Log Record for the Number Pool Block Status Attribute Value

--     Change Notification

lnpLogNumberPoolBlockStatusAttributeValueChangeRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":eventLogRecord;

CHARACTERIZED BY

lnpLogNumberPoolBlockStatusAttributeValueChangePkg;

CONDITIONAL PACKAGES

numberPoolBlockAttributeValueChangeFailed-SP-ListPkg PRESENT IF

!the number pool block broadcast failed!;

REGISTERED AS {LNP-OIDS.lnp-objectClass 28};

lnpLogNumberPoolBlockStatusAttributeValueChangePkg PACKAGE

BEHAVIOUR

lnpLogNumberPoolBlockStatusAttributeValueChangeDefinition,

lnpLogNumberPoolBlockStatusAttributeValueChangeBehavior;

ATTRIBUTES

subscriptionVersionAttributeValueChangeInfo GET,

accessControl GET;

;

lnpLogNumberPoolBlockStatusAttributeValueChangeDefinition BEHAVIOUR

DEFINED AS !

The lnpLogNumberPoolBlockStatusAttributeValueChangeRecord class is

the managed object that is used to create log records for the

numberPoolBlockStatusAttributeValueChange Notification.

!;

lnpLogNumberPoolBlockStatusAttributeValueChangeBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

numberPoolBlockStatusAttributeValueChange Notification.

!;

-- 29.0 Number Pool Block Data Managed Object Class

--

numberPoolBlock MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

numberPoolBlock-Pkg;

CONDITIONAL PACKAGES

numberPoolBlockWSMSC-DataPkg PRESENT IF

!the service provider is supporting WSMSC information!;

REGISTERED AS {LNP-OIDS.lnp-objectClass 29};

numberPoolBlock-Pkg PACKAGE

BEHAVIOUR

numberPoolBlock-Definition,

numberPoolBlock-Behavior;

ATTRIBUTES

numberPoolBlockId GET,

numberPoolBlockNPA-NXX-X GET,

numberPoolBlockHolderSPID GET,

numberPoolBlockActivationTimeStamp GET,

numberPoolBlockLRN GET-REPLACE,

numberPoolBlockCLASS-DPC GET-REPLACE,

numberPoolBlockCLASS-SSN GET-REPLACE,

numberPoolBlockLIDB-DPC GET-REPLACE,

numberPoolBlockLIDB-SSN GET-REPLACE,

numberPoolBlockCNAM-DPC GET-REPLACE,

numberPoolBlockCNAM-SSN GET-REPLACE,

numberPoolBlockISVM-DPC GET-REPLACE,

numberPoolBlockISVM-SSN GET-REPLACE,

numberPoolBlockDownloadReason GET-REPLACE;

;

numberPoolBlock-Definition BEHAVIOUR

DEFINED AS !

The numberPoolBlock class is the managed object

used to identify number pool block information as stored on the LSMS.

!;

numberPoolBlock-Behavior BEHAVIOUR

DEFINED AS !

Local SMS and NPAC SMS Managed Object used for the Local SMS to

NPAC SMS interface.

The NPAC Customer LSMS EDR Indicator in the service provider profile

on the NPAC SMS indicates which type of object will be broadcasted to

each LSMS. If the LSMS supports the number pool block object,

no subscription version objects should exist on the LSMS with the

LNP type equal to 'pool'.

The number pool block object represents the default routing

information for 1000 TNs. If a subscription version exists on the

LSMS for a TN within the number pool  block range, the subscription

routing information should be used. Otherwise, if no subscription

version exists for a TN within a number pool block range, the number

pool block routing information should be used.

The object can be created on the LSMS via the NPAC SMS to LSMS

interface.

The NPAC SMS can M-GET (Query Association Function), M-CREATE, M-DELETE,

and M-SET (Data Download Association Function) any numberPoolBlock

object on the Local SMS. The Local SMS only creates local copies of

the numberPoolBlock objects after receiving the objects from a NPAC

SMS create request or reading them from the NPAC SMS for initial

instantiation.

The numberPoolBlockId is assigned upon creation by the NPAC SMS and

is read only.

The numberPoolBlockHolderSPID must be the same as the service provider

ID of the service provider who sent in the request and the service

provider ID of the serviceProvNPA-NXX-X object for the specified

NPA-NXX-X.

The numberPoolBlockActivationTimeStamp is set by the NPAC SMS as the

current date and time when the create request is received.

The Local SMS will receive the numberPoolBlockWSMSC-DataPkg

attributes if the 'NPAC New Functionality Support' indicator

is set for the 'LSMS WSMSC DPC SSN Data' in their service

provider profile on the NPAC SMS.

When the number pool block is downloaded to the LSMS, the

numberPoolBlockDownloadReason is set to one of new, delete, modified

or audit discrepancy. This field is not validated in audits.

The Local SMS can not modify any of the number pool block

data locally unless changes were downloaded via a download

request.

The Local SMS must support equality and ordering on the

numberPoolBlockNPA-NXX-X attribute in a scoped and filtered request

for mass updates and audits.

!;

-- 30.0 Number Pool Block NPAC Data Managed Object Class

--

numberPoolBlockNPAC MANAGED OBJECT CLASS

DERIVED FROM numberPoolBlock;

CHARACTERIZED BY

numberPoolBlockNPAC-Pkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 30};

numberPoolBlockNPAC-Pkg PACKAGE

BEHAVIOUR

numberPoolBlockNPAC-Definition,

numberPoolBlockNPAC-Behavior;

ATTRIBUTES

numberPoolBlockBroadcastTimeStamp GET,

numberPoolBlockCreationTimeStamp GET,

numberPoolBlockDisconnectCompleteTimeStamp GET,

numberPoolBlockModifiedTimeStamp GET,

numberPoolBlockSOA-Origination GET-REPLACE,

numberPoolBlockStatus GET,

numberPoolBlockFailed-SP-List GET;

NOTIFICATIONS

numberPoolBlockStatusAttributeValueChange,

"CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":attributeValueChange

accessControlParameter,

"CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":objectCreation

accessControlParameter;

;

numberPoolBlockNPAC-Definition BEHAVIOUR

DEFINED AS !

The numberPoolBlock class is the managed object

used to identify number pool block NPAC information.

!;

numberPoolBlockNPAC-Behavior BEHAVIOUR

DEFINED AS !

SOA SMS and NPAC SMS Managed Object used for the SOA SMS to

NPAC SMS interface.

The numberPoolBlock object is used to represent a block of 1000 TN

values. A service provider LSMS either supports the number pool block

representation of the TNs or individual subscription versions with

LNPType value of 'pool'. The NPAC Customer LSMS EDR Indicator in the

service provider profile on the NPAC SMS indicates which type of

objects will be broadcasted to each LSMS.

An object creation attempt will be rejected by the NPAC SMS if

any subscription versions exist with a status of pending,

conflict, cancel-pending or failed ("pending-like") for a TN implied

by the NPA-NXX-X value and an active subscription version object does

not exist for that TN.

Once created, the numberPoolBlock object can only be deleted

via the NPAC SMS.

The numberPoolBlockCreationTimeStamp will be set to the

current date and time when the object is created by the NPAC

SMS.

If the SOA sends the numberPoolBlock-Create M-ACTION to create a

number pool block object, the numberPoolBlockSOA-Origination

will be set to true when the object is created by the NPAC SMS.

Otherwise, the value will be set to false.

The numberPoolBlockStatus is set at creation time by the NPAC SMS

to a value of 'sending'. Other values include: active, failed,

partial failure and old.

The numberPoolBlockBroadcastTimeStamp is set to the date and time

of the start of the last broadcast to the LSMSs for the number

pool block.

The numberPoolBlockDisconnectCompleteTimeStamp is set to the date and

time once the first successful response is received to a

delete broadcast to the LSMSs for the number pool block or

corresponding subscription versions with LNP type equal to 'pool'.

The numberPoolBlockModifiedTimeStamp is set to the date and time of the

last modification to the number pool block object.

The numberPoolBlockFailed-SP-List is set when a failure occurs

in sending to the LSMSs. It contains the list of EDR and non-EDR

service provider IDs who failed to successfully respond to the

broadcasted data.

When the number pool block is created, modified, or mass updated

and contains a DPC value, a valid, corresponding SSN value must

be present and vice versa.  An SSN value of 000 (zero) must be

present when its corresponding DPC value is populated, if the

corresponding SSN Edit Flag Indicator is set to TRUE.

When modifying either DPC or SSN, both values must be sent to

the NPAC SMS in the request, even if only one is being modified.

Service providers can M-GET any numberPoolBlockNPAC object

from the LSMS and SOA.

If the SOA supports the numberPoolBlock object, it can M-ACTION and

M-SET any numberPoolBlockNPAC object on the NPAC SMS for their own

service provider id (SOA Management Association Function). The NPAC

SMS will reject any attempts to take actions on other service

provider objects (soa-not-authorized) or attempts to M-DELETE

the objects (accessDenied).

When the SOA or NPAC SMS creates the numberPoolBlockNPAC, the status is

set to "sending". The NPAC SMS creates the corresponding subscription

versions that represent the block with a status of "sending" and LNP

type equal to 'pool'. The NPAC SMS broadcasts to the LSMSs either the

numberPoolBlock object or subscriptionVersion objects according to

their service provider NPAC Customer LSMS EDR Indicator on the NPAC SMS.

The object creation notification will be sent to the SOA once the

number pool block object has been created on the NPAC SMS,

if the SOA-origination flag is true, and contain the following

attributes:

numberPoolBlockId

numberPoolBlockNPA-NXX-X

numberPoolBlockHolderSPID

numberPoolBlockSOA-Origination

numberPoolBlockCreationTimeStamp

numberPoolBlockStatus

numberPoolBlockLRN

numberPoolBlockCLASS-DPC

numberPoolBlockCLASS-SSN

numberPoolBlockLIDB-DPC

numberPoolBlockLIDB-SSN

numberPoolBlockCNAM-DPC

numberPoolBlockCNAM-SSN

numberPoolBlockISVM-DPC

numberPoolBlockISVM-SSN

numberPoolBlockWSMSC-DPC (OPTIONAL)

numberPoolBlockWSMSC-SSN (OPTIONAL)

The SOA can only M-SET a single number pool block instance. The

M-SET can specify the numberPoolBlockNPA-NXX-X in a scoped and

filtered request or direct the request at the specific

numberPoolBlock object.

If the NPAC SMS or SOA sends a valid M-SET for the number pool block

object, the NPAC SMS will also set the data on the corresponding

subscription versions with LNPType of 'pool' and broadcast the

number pool block modifications to the EDR LSMSs and the subscription

versions to the Non-EDR LSMSs. The non-EDR Local SMS will receive a

scope level-1 request with the filter specifying the TN-Range and LNP

type equal to 'pool'.

The attribute value change notification will be sent out to the SOA,

if the SOA-origination flag is true, when any of the following

attributes change:

numberPoolBlockSOA-Origination

numberPoolBlockLRN

numberPoolBlockCLASS-DPC

numberPoolBlockCLASS-SSN

numberPoolBlockLIDB-DPC

numberPoolBlockLIDB-SSN

numberPoolBlockCNAM-DPC

numberPoolBlockCNAM-SSN

numberPoolBlockISVM-DPC

numberPoolBlockISVM-SSN

numberPoolBlockWSMSC-DPC (OPTIONAL)

numberPoolBlockWSMSC-SSN (OPTIONAL)

The attribute value change will not be sent when the SOA-Origination

flag is set to false.

The attribute value change notification attribute list contains

only the modified attribute(s).

The number pool block status attribute value change notification

will be sent out to the SOA, if the SOA-origination flag is true,

when the status changes and can contain the following attributes:

numberPoolBlockStatus

numberPoolBlockFailed-SP-List

If the NPAC SMS M-DELETEs the number pool block object, the

corresponding subscription versions with LNPType of 'pool' will

also be disconnected. The NPAC SMS will send the M-DELETE for the

number pool block object to the EDR LSMSs and send the M-DELETE(s) for

the subscription versions to the Non-EDR LSMSs. The non-EDR Local SMS

will receive a scope level-1 request with the filter specifying the

TN-Range and LNP type equal to 'pool'. The NPAC SMS can only remove

one numberPoolBlockNPAC object at a time.

!;

-- 31.0 Service Provider NPA-NXX-X Data Managed Object Class

--

serviceProvNPA-NXX-X MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

serviceProvNPA-NXX-X-Pkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 31};

serviceProvNPA-NXX-X-Pkg PACKAGE

BEHAVIOUR

serviceProvNPA-NXX-X-Definition,

serviceProvNPA-NXX-X-Behavior;

ATTRIBUTES

serviceProvNPA-NXX-X-ID GET,

serviceProvNPA-NXX-X-Value GET,

serviceProvNPA-NXX-X-CreationTimeStamp GET,

serviceProvNPA-NXX-X-ModifiedTimeStamp GET-REPLACE,

serviceProvNPA-NXX-X-DownloadReason GET-REPLACE,

serviceProvNPA-NXX-X-EffectiveTimeStamp GET-REPLACE;

;

serviceProvNPA-NXX-X-Definition BEHAVIOUR

DEFINED AS !

The serviceProvNPA-NXX-X class is the managed object

used to identify Service Provider NPA-NXX-X data.

!;

serviceProvNPA-NXX-X-Behavior BEHAVIOUR

DEFINED AS !

This is the Local SMS, SOA and NPAC SMS managed object used for

the Local SMS to NPAC SMS interface and SOA to NPAC SMS interface.

A Local SMS supports the serviceProvNPA-NXX-X via the NPAC

Customer LSMS NPA-NXX-X Indicator in their service provider

profile. A SOA supports the object via the NPAC Customer SOA

NPA-NXX-X Indicator in their service provider profile.

Only the NPAC SMS can create, modify or delete the

object. All updates are initiated by the NPAC SMS and downloaded to

the Local SMSs and SOAs.

An object creation attempt will be rejected by the NPAC SMS if

any subscription versions exist with a status of pending,

conflict, cancel-pending or failed ("pending-like") for a TN implied

by the NPA-NXX-X value and an active subscription version object does

not exist for that TN or the subscription version is a

port-to-original request.

NPAC SMS personnel can modify the date of the

serviceProvNPA-NXX-X-EffectiveTimeStamp only prior to the number

pool block activation and only if the current date is less than the

effective date and the new serviceProvNPA-NXX-X-EffectiveTimeStamp

is greater than or equal to the serviceProvNPA-NXX-X CreationTimeStamp

plus the NPAC SMS's NPA-NXX-X Holder Effective Date Window Tunable.

The serviceProvNPA-NXX-X-ID is specified by the NPAC SMS at creation

time.

The serviceProvNPA-NXX-X-CreationTimeStamp is set to the current

date and time of when the NPAC SMS creates the object.

The serviceProvNPA-NXX-X-ModifiedTimeStamp is set to the current

date and time of when the NPAC SMS last modified the object.

NPAC SMS can M-DELETE, M-SET and M-CREATE a single

serviceProvNPA-NXX-X object on the Local SMS or SOA (Network Data

Functional Unit).  The Local SMS or SOA only creates local

copies of serviceProvNPA-NXX-X objects after receiving the

objects from an NPAC SMS create request, reading them from

the NPAC SMS download file, or from an lnpDownload request.

The following attribute can be modified by the NPAC SMS:

serviceProvNPA-NXX-X-EffectiveTimeStamp

The serviceProvNPA-NXX-X-DownloadReason and

serviceProvNPA-NXX-X-ModifiedTimeStamp can be updated as a

result.

The NPAC SMS will send to the Local SMS or SOA the M-SET with

the new serviceProvNPA-NXX-X-EffectiveTimeStamp and

serviceProvNPA-NXX-X-ModifiedTimeStamp.

The SOA and Local SMS can M-GET the serviceProvNPA-NXX-X object.

The subscriptionVersionNewNPA-NXX will be sent to notify the service

providers SOA and Local SMS systems of the creation of this object

if the creation is the first usage of the NPA-NXX. However, this

notification is not emitted from the serviceProvNPA-NXX-X object,

but rather from the lnpNPAC-SMS object (top-level) instead.

!;

-- 32.0 LNP Log Record for the Subscription Version Range Attribute Value

--      Change Notification

lnpLogRangeAttributeValueChangeRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":eventLogRecord;

CHARACTERIZED BY

lnpLogRangeAttributeValueChangePkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 32};

lnpLogRangeAttributeValueChangePkg PACKAGE

BEHAVIOUR

lnpLogRangeAttributeValueChangeDefinition,

lnpLogRangeAttributeValueChangeBehavior;

ATTRIBUTES

subscriptionVersionRangeAttributeValueChangeInfo GET,

accessControl GET;

;

lnpLogRangeAttributeValueChangeDefinition BEHAVIOUR

DEFINED AS !

The lnpLogAttributeValueChangeRecord class is the managed object

that is used to create log records for the

subscriptionVersionRangeAttributeValueChange Notification.

!;

lnpLogRangeAttributeValueChangeBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

subscriptionVersionRangeAttributeValueChange Notification.

!;

-- 33.0 LNP Log Record for the Subscription Version Range Object Creation

--      Notification

lnpLogRangeObjectCreationRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":eventLogRecord;

CHARACTERIZED BY

lnpLogRangeObjectCreationPkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 33};

lnpLogRangeObjectCreationPkg PACKAGE

BEHAVIOUR

lnpLogRangeObjectCreationDefinition,

lnpLogRangeObjectCreationBehavior;

ATTRIBUTES

subscriptionVersionRangeObjectCreationInfo GET,

accessControl GET;

;

lnpLogRangeObjectCreationDefinition BEHAVIOUR

DEFINED AS !

The lnpLogRangeObjectCreationRecord class is the managed object

that is used to create log records for the

subscriptionVersionRangeObjectCreation Notification.

!;

lnpLogRangeObjectCreationBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

subscriptionVersionObjectCreation Notification.

!;

-- 34.0 LNP Log Record for the Subscription Version Range Status Attribute

--      Value Change Notification

lnpLogRangeStatusAttributeValueChangeRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":eventLogRecord;

CHARACTERIZED BY

lnpLogRangeStatusAttributeValueChangePkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 34};

lnpLogRangeStatusAttributeValueChangePkg PACKAGE

BEHAVIOUR

lnpLogRangeStatusAttributeValueChangeDefinition,

lnpLogRangeStatusAttributeValueChangeBehavior;

ATTRIBUTES

subscriptionVersionRangeStatusAttributeValueChangeInfo GET,

accessControl GET;

;

lnpLogRangeStatusAttributeValueChangeDefinition BEHAVIOUR

DEFINED AS !

The lnpLogStatusAttributeValueChangeRecord class is the managed

object that is used to create log records for the

subscriptionVersionRangeStatusAttributeValueChange Notification.

!;

lnpLogRangeStatusAttributeValueChangeBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

subscriptionVersionRangeStatusAttributeValueChange Notification.

!;

-- 35.0 LNP Log Record for the Subscription Version Range Donor Disconnect

--      Notification

lnpLogRangeDonorSP-CustomerDisconnectDateRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":eventLogRecord;

CHARACTERIZED BY

lnpLogRangeDonorSP-CustomerDisconnectDatePkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 35};

lnpLogRangeDonorSP-CustomerDisconnectDatePkg PACKAGE

BEHAVIOUR

lnpLogRangeDonorSP-CustomerDisconnectDateDefinition,

lnpLogRangeDonorSP-CustomerDisconnectDateBehavior;

ATTRIBUTES

subscriptionVersionRangeDonorSP-CustomerDisconnectDateInfo GET,

accessControl GET;

;

lnpLogRangeDonorSP-CustomerDisconnectDateDefinition BEHAVIOUR

DEFINED AS !

The lnpLogRangeDonorSP-CustomerDisconnectDateRecord class is

the managed object that is used to create log records for the

subscriptionVersionRangeDonorSP-CustomerDisconnectDate Notification.

!;

lnpLogRangeDonorSP-CustomerDisconnectDateBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

subscriptionVersionRangeDonorSP-CustomerDisconnectDate Notification.

!;

-- 36.0 LNP Log Record for the Subscription Version Range Cancellation

--      Acknowledge Request Notification

lnpLogRangeCancellationAcknowledgeRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":eventLogRecord;

CHARACTERIZED BY

lnpLogRangeCancellationAcknowledgePkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 36};

lnpLogRangeCancellationAcknowledgePkg PACKAGE

BEHAVIOUR

lnpLogRangeCancellationAcknowledgeDefinition,

lnpLogRangeCancellationAcknowledgeBehavior;

ATTRIBUTES

subscriptionVersionRangeCancellationAcknowledgeInfo GET,

accessControl GET;

;

lnpLogRangeCancellationAcknowledgeDefinition BEHAVIOUR

DEFINED AS !

The lnpLogRangeCancellationAcknowledgeRecord class is the

managed object that is used to create log records for the

subscriptionVersionRangeCancellationAcknowledge Notification.

!;

lnpLogRangeCancellationAcknowledgeBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

subscriptionVersionRangeCancellationAcknowledge Notification.

!;

-- 37.0 LNP Log Record for the Subscription Version Range New SP Create

--      Request Notification

lnpLogRangeNewSP-CreateRequestRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":eventLogRecord;

CHARACTERIZED BY

lnpLogRangeNewSP-CreateRequestPkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 37};

lnpLogRangeNewSP-CreateRequestPkg PACKAGE

BEHAVIOUR

lnpLogRangeNewSP-CreateRequestDefinition,

lnpLogRangeNewSP-CreateRequestBehavior;

ATTRIBUTES

subscriptionVersionRangeNewSP-CreateRequestInfo GET,

accessControl GET;

;

lnpLogRangeNewSP-CreateRequestDefinition BEHAVIOUR

DEFINED AS !

The lnpLogRangeNewSP-CreateRequestRecord class is the managed object

that is used to create log records for the

subscriptionVersionRangeNewSP-CreateRequest Notification.

!;

lnpLogRangeNewSP-CreateRequestBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

subscriptionVersionRangeNewSP-CreateRequest Notification.

!;

-- 38.0 LNP Log Record for the Subscription Version Range Old SP

--      Concurrence Request Notification

lnpLogRangeOldSP-ConcurrenceRequestRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":eventLogRecord;

CHARACTERIZED BY

lnpLogRangeOldSP-ConcurrenceRequestPkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 38};

lnpLogRangeOldSP-ConcurrenceRequestPkg PACKAGE

BEHAVIOUR

lnpLogRangeOldSP-ConcurrenceRequestDefinition,

lnpLogRangeOldSP-ConcurrenceRequestBehavior;

ATTRIBUTES

subscriptionVersionRangeOldSP-ConcurrenceRequestInfo GET,

accessControl GET;

;

lnpLogRangeOldSP-ConcurrenceRequestDefinition BEHAVIOUR

DEFINED AS !

The lnpLogRangeOldSP-ConcurrenceRequestRecord class is the

managed object that is used to create log records for the

subscriptionVersionRangeOldSP-ConcurrenceRequest Notification.

!;

lnpLogRangeOldSP-ConcurrenceRequestBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

subscriptionVersionRangeOldSP-ConcurrenceRequest Notification.

!;

-- 39.0 LNP Log Record for the Subscription Version Range Old SP Final

--      Concurrence Window Expiration Notification

lnpLogRangeOldSPFinalConcurrenceWindowExpirationRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":eventLogRecord;

CHARACTERIZED BY

lnpLogRangeOldSPFinalConcurrenceWindowExpirationPkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 39};

lnpLogRangeOldSPFinalConcurrenceWindowExpirationPkg PACKAGE

BEHAVIOUR

lnpLogRangeOldSPFinalConcurrenceWindowExpirationDefinition,

lnpLogRangeOldSPFinalConcurrenceWindowExpirationBehavior;

ATTRIBUTES

subscriptionVersionRangeOldSPFinalConcurrenceWindowExpirationInfo GET,

accessControl GET;

;

lnpLogRangeOldSPFinalConcurrenceWindowExpirationDefinition BEHAVIOUR

DEFINED AS !

The lnpLogRangeOldSPFinalConcurrenceWindowExpirationRecord class

is the managed object that is used to create log records for the

subscriptionVersionRangeOldSPFinalConcurrenceWindowExpiration

Notification.

!;

lnpLogRangeOldSPFinalConcurrenceWindowExpirationBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

subscriptionVersionRangeOldSPFinalConcurrenceWindowExpiration

Notification.

!;

-- 40.0 LNP Log Record for the Subscription Version Range New SP Final

--      Create Window Expiration Notification

lnpLogRangeNewSP-FinalCreateWindowExpirationRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":eventLogRecord;

CHARACTERIZED BY

lnpLogRangeNewSP-FinalCreateWindowExpirationPkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 40};

lnpLogRangeNewSP-FinalCreateWindowExpirationPkg PACKAGE

BEHAVIOUR

lnpLogRangeNewSP-FinalCreateWindowExpirationDefinition,

lnpLogRangeNewSP-FinalCreateWindowExpirationBehavior;

ATTRIBUTES

subscriptionVersionRangeNewSP-FinalCreateWindowExpirationInfo GET,

accessControl GET;

;

lnpLogRangeNewSP-FinalCreateWindowExpirationDefinition BEHAVIOUR

DEFINED AS !

The lnpLogRangeNewSP-FinalCreateWindowExpirationRecord class

is the managed object that is used to create log records for the

subscriptionVersionRangeNewSP-FinalCreateWindowExpiration Notification.

!;

lnpLogRangeNewSP-FinalCreateWindowExpirationBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

subscriptionVersionRangeNewSP-FinalCreateWindowExpiration Notification.

!;

-- 41.0 LNP Log Record for the LNP Subscription version New Service

--   Provider Final Create Window Expiration Notification

lnpLogNewSP-FinalCreateWindowExpirationRecord MANAGED OBJECT CLASS

DERIVED FROM "CCITT Rec. X.721 (1992) | ISO/IEC 10165-2 :

1992":eventLogRecord;

CHARACTERIZED BY

lnpLogNewSP-FinalCreateWindowExpirationPkg;

REGISTERED AS {LNP-OIDS.lnp-objectClass 41};

lnpLogNewSP-FinalCreateWindowExpirationPkg PACKAGE

BEHAVIOUR

lnpLogNewSP-FinalCreateWindowExpirationDefinition,

lnpLogNewSP-FinalCreateWindowExpirationBehavior;

ATTRIBUTES

subscriptionTN GET,

subscriptionVersionId GET,

subscriptionNewCurrentSP GET,

subscriptionOldSP GET,

subscriptionOldSP-DueDate GET,

subscriptionOldSP-Authorization GET,

subscriptionOldSP-AuthorizationTimeStamp GET,

subscriptionStatusChangeCauseCode GET,

subscriptionTimerType GET,

subscriptionBusinessType GET,

accessControl GET;

;

lnpLogNewSP-FinalCreateWindowExpirationDefinition BEHAVIOUR

DEFINED AS !

The lnpLogNewSP-FinalCreateWindowExpirationRecord class is the

managed object that is used to create log records for the

subscriptionVersionNewSP-FinalCreateWindowExpiration Notification.

!;

lnpLogNewSP-FinalCreateWindowExpirationBehavior BEHAVIOUR

DEFINED AS !

This log record can be used by any CME wanting to log the

subscriptionVersionNewSP-FinalCreateWindowExpiration Notification.

!;

--

-- Name Binding Definitions

--

-- 1.0 LNP Audits Managed Object Name Bindings

lnpAudits-lnpNPAC-SMS NAME BINDING

SUBORDINATE OBJECT CLASS lnpAudits AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS lnpNPAC-SMS AND SUBCLASSES;

WITH ATTRIBUTE lnpAuditsName;

-- Note: Create through interface is not supported.

-- Note: Delete through interface is not supported.

REGISTERED AS {LNP-OIDS.lnp-nameBinding 1};

-- 2.0 LNP Local SMS Managed Object Name Bindings

lnpLocalSMS-root NAME BINDING

SUBORDINATE OBJECT CLASS lnpLocalSMS AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "CCITT Rec. X.660 (1992) | ISO/IEC 9834-1 :

1992":root;

WITH ATTRIBUTE lnpLocal-SMS-Name;

-- Note: Create through interface is not supported.

-- Note: Delete through interface is not supported.

REGISTERED AS {LNP-OIDS.lnp-nameBinding 3};

-- 3.0 LNP Network Managed Object Name Bindings

lnpNetwork-lnpNPAC-SMS NAME BINDING

SUBORDINATE OBJECT CLASS lnpNetwork AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS lnpNPAC-SMS AND SUBCLASSES;

WITH ATTRIBUTE lnpNetworkName;

-- Note: Create through interface is not supported.

-- Note: Delete through interface is not supported.

REGISTERED AS {LNP-OIDS.lnp-nameBinding 4};

lnpNetwork-lnpLocalSMS NAME BINDING

SUBORDINATE OBJECT CLASS lnpNetwork AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS lnpLocalSMS AND SUBCLASSES;

WITH ATTRIBUTE lnpNetworkName;

-- Note: Create through interface is not supported.

-- Note: Delete through interface is not supported.

REGISTERED AS {LNP-OIDS.lnp-nameBinding 5};

lnpNetwork-lnpSOA NAME BINDING

SUBORDINATE OBJECT CLASS lnpNetwork AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS lnpSOA AND SUBCLASSES;

WITH ATTRIBUTE lnpNetworkName;

-- Note: Create through interface is not supported.

-- Note: Delete through interface is not supported.

REGISTERED AS {LNP-OIDS.lnp-nameBinding 17};

-- 4.0 LNP NPAC SMS Managed Object Name Bindings

lnpNPAC-SMS-root NAME BINDING

SUBORDINATE OBJECT CLASS lnpNPAC-SMS AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "CCITT Rec. X.660 (1992) | ISO/IEC 9834-1 :

1992":root;

WITH ATTRIBUTE lnpNPAC-SMS-Name;

-- Note: Create through interface is not supported.

-- Note: Delete through interface is not supported.

REGISTERED AS {LNP-OIDS.lnp-nameBinding 6};

-- 5.0 LNP Service Providers Managed Object Name Bindings

lnpServiceProvs-lnpNPAC-SMS NAME BINDING

SUBORDINATE OBJECT CLASS lnpServiceProvs AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS lnpNPAC-SMS AND SUBCLASSES;

WITH ATTRIBUTE lnpServiceProvsName;

-- Note: Create through interface is not supported.

-- Note: Delete through interface is not supported.

REGISTERED AS {LNP-OIDS.lnp-nameBinding 7};

-- 6.0 LNP Subscriptions Managed Object Class Name Bindings

lnpSubscriptions-lnpNPAC-SMS NAME BINDING

SUBORDINATE OBJECT CLASS lnpSubscriptions AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS lnpNPAC-SMS AND SUBCLASSES;

WITH ATTRIBUTE lnpSubscriptionsName;

-- Note: Create through interface is not supported.

-- Note: Delete through interface is not supported.

REGISTERED AS {LNP-OIDS.lnp-nameBinding 8};

lnpSubscriptions-lnpLocalSMS NAME BINDING

SUBORDINATE OBJECT CLASS lnpSubscriptions AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS lnpLocalSMS AND SUBCLASSES;

WITH ATTRIBUTE lnpSubscriptionsName;

-- Note: Create through interface is not supported.

-- Note: Delete through interface is not supported.

REGISTERED AS {LNP-OIDS.lnp-nameBinding 9};

-- 7.0 LNP Service Provider Managed Object Class Name Bindings

serviceProv-lnpServiceProvs NAME BINDING

SUBORDINATE OBJECT CLASS serviceProv AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS lnpServiceProvs AND SUBCLASSES;

WITH ATTRIBUTE serviceProvID;

CREATE;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {LNP-OIDS.lnp-nameBinding 10};

-- 8.0 LNP Service Provider LRN Managed Object Class Name Bindings

serviceProvLRN-serviceProvNetwork NAME BINDING

SUBORDINATE OBJECT CLASS serviceProvLRN AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS serviceProvNetwork AND SUBCLASSES;

WITH ATTRIBUTE serviceProvLRN-ID;

CREATE WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {LNP-OIDS.lnp-nameBinding 11};

-- 9.0 LNP Service Provider Network Managed Object Class Name Bindings

serviceProvNetwork-lnpNetwork NAME BINDING

SUBORDINATE OBJECT CLASS serviceProvNetwork AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS lnpNetwork AND SUBCLASSES;

WITH ATTRIBUTE serviceProvID;

CREATE;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {LNP-OIDS.lnp-nameBinding 12};

-- 10.0 LNP Service Provider NPA-NXX Managed Object Class Name Bindings

serviceProvNPA-NXX-serviceProvNetwork NAME BINDING

SUBORDINATE OBJECT CLASS serviceProvNPA-NXX AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS serviceProvNetwork AND SUBCLASSES;

WITH ATTRIBUTE serviceProvNPA-NXX-ID;

CREATE WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {LNP-OIDS.lnp-nameBinding 13};

-- 11.0 LNP Subscription Audit for the NPAC SMS Managed Object

subscriptionAudit-lnpAudits NAME BINDING

SUBORDINATE OBJECT CLASS subscriptionAudit AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS lnpAudits AND SUBCLASSES;

WITH ATTRIBUTE subscriptionAuditId;

CREATE WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {LNP-OIDS.lnp-nameBinding 14};

-- 12.0 LNP Subscription Version Managed Object Class

subscriptionVersion-lnpSubscriptions NAME BINDING

SUBORDINATE OBJECT CLASS subscriptionVersion AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS lnpSubscriptions AND SUBCLASSES;

WITH ATTRIBUTE subscriptionVersionId;

CREATE WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {LNP-OIDS.lnp-nameBinding 15};

-- 13.0 LNP Service Provider LSMS Filter NPA-NXX Managed Object Class

--      Name Bindings

lsmsFilterNPA-NXX-serviceProv NAME BINDING

SUBORDINATE OBJECT CLASS lsmsFilterNPA-NXX AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS serviceProv AND SUBCLASSES;

WITH ATTRIBUTE lsmsFilterNPA-NXX-ID;

CREATE WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {LNP-OIDS.lnp-nameBinding 16};

-- 14.0 LNP SOA Managed Object Name Bindings

lnpSOA-root NAME BINDING

SUBORDINATE OBJECT CLASS lnpSOA AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "CCITT Rec. X.660 (1992) | ISO/IEC 9834-1 :

1992":root;

WITH ATTRIBUTE lnpSOA-Name;

-- Note: Create through interface is not supported.

-- Note: Delete through interface is not supported.

REGISTERED AS {LNP-OIDS.lnp-nameBinding 18};

-- 15.0 LNP Number Pool Block Managed Object Class

--     Name Bindings

numberPoolBlock-lnpSubscriptions NAME BINDING

SUBORDINATE OBJECT CLASS numberPoolBlock AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS lnpSubscriptions AND SUBCLASSES;

WITH ATTRIBUTE numberPoolBlockId;

CREATE WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {LNP-OIDS.lnp-nameBinding 19};

-- 16.0 LNP Service Provider NPA-NXX-X Managed Object Class

--     Name Bindings

serviceProvNPA-NXX-X-serviceProvNetwork NAME BINDING

SUBORDINATE OBJECT CLASS serviceProvNPA-NXX-X AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS serviceProvNetwork AND SUBCLASSES;

WITH ATTRIBUTE serviceProvNPA-NXX-X-ID;

CREATE WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS {LNP-OIDS.lnp-nameBinding 20};

--

-- Attribute Definitions

--

-- 1.0  LNP Access Control Attribute

accessControl ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.LnpAccessControl;

MATCHES FOR EQUALITY;

BEHAVIOUR accessControlBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 1};

accessControlBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store/define access control

information for security.

!;

-- 2.0  LNP Action Id Attribute

actionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.Integer;

MATCHES FOR EQUALITY;

BEHAVIOUR actionIdBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 2};

actionIdBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the action id associated

with an action that sends back an asynchronous notification.

!;

-- 3.0  LNP Action Results Status Attribute

actionResultsStatus ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.ActionResultsStatus;

MATCHES FOR EQUALITY;

BEHAVIOUR actionResultsStatusBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 3};

actionResultsStatusBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the status of an action that

sends back an asynchronous notification with the results.

!;

-- 4.0  LNP Additional Down Time Information

additionalDownTimeInformation ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GraphicString255;

MATCHES FOR EQUALITY;

BEHAVIOUR additionalDownTimeInformationBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 4};

additionalDownTimeInformationBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to provide additional information

about planned NPAC SMS down time in an NPAC operations notification

in a log record.

!;

-- 5.0  LNP Audit Discrepancy Failure Reason

auditDiscrepancyFailureReason ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AuditFailureData;

MATCHES FOR EQUALITY;

BEHAVIOUR auditDiscrepancyFailureReasonBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 5};

auditDiscrepancyFailureReasonBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the audit discrepancy failure reason

in an audit discrepancy notification in a log record.

!;

-- 6.0  LNP Audit Discrepancy Local SMS Service Provider Id

auditDiscrepancyLSMS-SP-Id ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.ServiceProvId;

MATCHES FOR EQUALITY;

BEHAVIOUR auditDiscrepancyLSMS-SP-Id-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 6};

auditDiscrepancyLSMS-SP-Id-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the service provider id

associated with the Local SMS in an audit discrepancy notification

in a log record.

!;

-- 7.0  LNP Audit Discrepancy TN

auditDiscrepancyTn ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.PhoneNumber;

MATCHES FOR EQUALITY;

BEHAVIOUR auditDiscrepancyTnBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 7};

auditDiscrepancyTnBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the TN for which the discrepancy

was found in an audit discrepancy notification in a log record.

!;

-- 8.0  LNP Audit Discrepancy Version Id

auditDiscrepancyVersionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.SubscriptionVersionId;

MATCHES FOR EQUALITY;

BEHAVIOUR auditDiscrepancyVersionId-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 8};

auditDiscrepancyVersionId-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the version id for the TN for

which the discrepancy was found in an audit discrepancy

notification in a log record.

The NPAC SMS currently uses a 32-bit signed integer for the

Naming ID Value.  The maximum value is ([2**31] - 1) or 2.14B.

It is anticipated that all Service Providers will be able to

successfully handle Naming ID Values up to this maximum.

!;

-- 10.0  LNP Audit Results Audit Completion Time

auditResultCompletionTime ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR auditResultCompletionTimeBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 10};

auditResultCompletionTimeBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the completion time of the audit

in an audit results notification in a log record.

!;

-- 11.0  LNP Audit Result Failed Service Provider List

auditResultFailed-SP-List ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.Failed-SP-List;

MATCHES FOR EQUALITY;

BEHAVIOUR auditResultFailed-SP-ListBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 11};

auditResultFailed-SP-ListBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store, in an audit results

notification in a log record, the list of failed service

providers for an audit that failed due to failures on Local

SMSs.

!;

-- 12.0 LNP Audit Results Number of Discrepancies

auditResultNumberDiscrepancies ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.Integer;

MATCHES FOR EQUALITY;

BEHAVIOUR auditResultNumberDiscrepanciesBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 12};

auditResultNumberDiscrepanciesBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the number of discrepancies found

in an audit results notification in a log record.

!;

-- 13.0 LNP Audit Result Status

auditResultStatus ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AuditResultStatus;

MATCHES FOR EQUALITY;

BEHAVIOUR auditResultStatusBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 13};

auditResultStatusBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the final status of the audit

in an audit results notification in a log record.

!;

-- 14.0 LNP Operational Notification Down Time

downTime ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.TimeRange;

MATCHES FOR EQUALITY;

BEHAVIOUR downTimeBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 14};

downTimeBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to indicate the down time in an

NPAC operations notification in a log record.

!;

-- 15.0 LNP Failed TN List

failedTN-List ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.FailedTN-List;

MATCHES FOR EQUALITY;

BEHAVIOUR failedTN-ListBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 15};

failedTN-ListBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to indicate the tn(s) and errors for

a failed action in the return asynchronous notification.

!;

-- 16.0 LNP Audits Name

lnpAuditsName ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.LnpAuditsName;

MATCHES FOR EQUALITY;

BEHAVIOUR lnpAuditsNameBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 16};

lnpAuditsNameBehavior BEHAVIOUR

DEFINED AS !

This attribute provides an identifier for the lnpAudits managed

object.  The value for this attribute is "lnpAudits".

!;

-- 17.0 LNP Local SMS Name

lnpLocal-SMS-Name ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.LnpSMS-Name;

MATCHES FOR EQUALITY;

BEHAVIOUR lnpLocal-SMS-NameBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 17};

lnpLocal-SMS-NameBehavior BEHAVIOUR

DEFINED AS !

This attribute provides an identifier for the lnpLocalSMS

object. The valid value is the service provider id of the

Local SMS followed by a dash and then the region name of the

NPAC-SMS specified in the lnpNPAC-SMS-Name for the NPAC SMS to

Local SMS Interface. For example, if the region name is

"Midwest Regional NPAC SMS" and the service provider id is "1234",

the lnpLocal-SMS-Name would be "1234-Midwest Regional NPAC SMS".

!;

-- 18.0 LNP Network Name

lnpNetworkName ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.LnpNetworkName;

MATCHES FOR EQUALITY;

BEHAVIOUR lnpNetworkNameBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 18};

lnpNetworkNameBehavior BEHAVIOUR

DEFINED AS !

This attribute provides an identifier for the lnpNetwork

object.  Valid values are "lnpNetwork" for the NPAC SMS to Local

SMS Interface.

!;

-- 19.0 LNP NPAC SMS Name

lnpNPAC-SMS-Name ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.LnpSMS-Name;

MATCHES FOR EQUALITY;

BEHAVIOUR lnpNPAC-SMS-NameBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 19};

lnpNPAC-SMS-NameBehavior BEHAVIOUR

DEFINED AS !

This attribute provides an identifier for the lnpNPAC-SMS

object.  Valid values for NPAC SMS regional systems can be obtained

from NPAC personnel.

The lnpNPAC-SMS-Name values are listed in the IIS document.

!;

-- 20.0 LNP Service Providers Name

lnpServiceProvsName ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.LnpServiceProvsName;

MATCHES FOR EQUALITY;

BEHAVIOUR lnpServiceProvsNameBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 20};

lnpServiceProvsNameBehavior BEHAVIOUR

DEFINED AS !

This attribute provides an identifier for the

lnpServiceProvs object.   The value for this attribute

will be "lnpServiceProvs" in the NPAC SMS to Local SMS

Interface.

!;

-- 21.0 LNP Specific Info

lnpSpecificInfo ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.LnpSpecificInfo;

MATCHES FOR EQUALITY;

BEHAVIOUR lnpSpecificInfoBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 21};

lnpSpecificInfoBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to pass specific error information in the

case of a cmip processing failure error.

!;

-- 22.0 LNP Subscriptions Name

lnpSubscriptionsName ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.LnpSubscriptionsName;

MATCHES FOR EQUALITY;

BEHAVIOUR lnpSubscriptionsNameBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 22};

lnpSubscriptionsNameBehavior BEHAVIOUR

DEFINED AS !

This attribute provides an identifier for the

lnpSubscriptions object.   The value for this attribute

will be "lnpSubscriptions" in the NPAC SMS to Local SMS

Interface.

!;

-- 23.0 LNP NPAC Contact Number

npacContactNumber ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.PhoneNumber;

MATCHES FOR EQUALITY;

BEHAVIOUR npacContactNumberBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 23};

npacContactNumberBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to indicate the NPAC contact number

to be called concerning an NPAC SMS outage in an NPAC operations

notification in a log record.

!;

-- 24.0 LNP NPAC Customer Allowable Functions

npacCustomerAllowableFunctions ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AssociationFunction;

MATCHES FOR EQUALITY;

BEHAVIOUR npacCustomerAllowableFunctionsBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 24};

npacCustomerAllowableFunctionsBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify what functions a service provider

can perform on the SOA to NPAC SMS and NPAC SMS to Local SMS

interfaces.

!;

-- 25.0 LNP Results Completion Time

resultsCompletionTime ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR resultsCompletionTimeBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 25};

resultsCompletionTimeBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the completion time of the

action in the action results notification.

!;

-- 26.0 LNP Service Provider Address

serviceProvAddress ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AddressInformation;

MATCHES FOR EQUALITY;

BEHAVIOUR serviceProvAddressBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 26};

serviceProvAddressBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the address information

for a service provider.

!;

-- 27.0 LNP Service Provider Billing Address

serviceProvBillingAddress ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AddressInformation;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvBillingAddressBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 27};

serviceProvBillingAddressBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the billing address information

for a service provider.

!;

-- 28.0 LNP Service Provider Conflict Resolution Contact Address

serviceProvConflictAddress ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AddressInformation;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvConflictAddressBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 28};

serviceProvConflictAddressBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the service provider conflict

resolution contact address and contact information.

!;

-- 29.0 LNP Service Provider Data Download Reason

serviceProvDownloadReason ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.DownloadReason;

MATCHES FOR EQUALITY;

BEHAVIOUR serviceProvDownloadReasonBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 29};

serviceProvDownloadReasonBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the reason the data was

downloaded to the Local SMS from NPAC SMS.  This attribute only

has meaning in objects instantiated on the Local SMS.

!;

-- 30.0 LNP Service Provider ID

serviceProvID ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.ServiceProvId;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvID-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 30};

serviceProvID-Behavior BEHAVIOUR

DEFINED AS !

This attribute provides an identifier for the

serviceProvNetwork and serviceProv objects as

well as an identifier for the service provider who has requested

an audit on the NPAC SMS.  Valid values are the Facilities Id

(or OCN) of the service provider.

!;

-- 31.0 LNP Service Provider LRN Creation Time Stamp

serviceProvLRN-CreationTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvLRN-CreationTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 31};

serviceProvLRN-CreationTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute provides the timestamp of when the

serviceProvLRN object was created on the NPAC SMS.

!;

-- 32.0 LNP Service Provider LRN ID

serviceProvLRN-ID ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.LRN-ID;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvLRN-ID-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 32};

serviceProvLRN-ID-Behavior BEHAVIOUR

DEFINED AS !

This attribute provides an identifier for the serviceProvLRN

object.  The NPAC SMS determines the value for this attribute.

The NPAC SMS currently uses a 32-bit signed integer for the

Naming ID Value.  The maximum value is ([2**31] - 1) or 2.14B.

It is anticipated that all Service Providers will be able to

successfully handle Naming ID Values up to this maximum.

!;

-- 33.0 LNP Service Provider LRN Value

serviceProvLRN-Value ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.LRN;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvLRN-Value-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 33};

serviceProvLRN-Value-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the value for a service

provider LRN.

The data is stored as packed decimal. For example, if the octets

contained 01 23 45 67 89 then the LRN value would be displayed

as 0123456789).

!;

-- 34.0 LNP Service Provider LSMS Address

serviceProvLSMS-Address ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AddressInformation;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvLSMS-AddressBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 34};

serviceProvLSMS-AddressBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the service provider LSMS

address and contact information.

!;

-- 35.0 LNP Service Provider Name

serviceProvName ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.ServiceProvName;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvNameBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 35};

serviceProvNameBehavior BEHAVIOUR

DEFINED AS !

This attribute is the English name for the service provider.

!;

-- 36.0 LNP Service Provider Network and Communications Address

serviceProvNetAddress ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AddressInformation;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvNetAddressBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 36};

serviceProvNetAddressBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the service provider network

and communications facilities address and contact information.

!;

-- 37.0 LNP Service Provider NPA-NXX Creation Time Stamp

serviceProvNPA-NXX-CreationTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvNPA-NXX-CreationTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 37};

serviceProvNPA-NXX-CreationTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute provides the timestamp of the creation of the

serviceProvNPA-NXX object on the NPAC SMS.

!;

-- 38.0 LNP Service Provider NPA-NXX Effective Time Stamp

serviceProvNPA-NXX-EffectiveTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvNPA-NXX-EffectiveTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 38};

serviceProvNPA-NXX-EffectiveTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute provides a timestamp as to when the

NPA-NXX is available for LNP in the service provider networks.

!;

-- 39.0 LNP Service Provider NPA-NXX ID

serviceProvNPA-NXX-ID ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.NPA-NXX-ID;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvNPA-NXX-ID-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 39};

serviceProvNPA-NXX-ID-Behavior BEHAVIOUR

DEFINED AS !

This attribute provides an identifier for the

serviceProvNPA-NXX object.

The NPAC SMS determines the value for this attribute.

The NPAC SMS currently uses a 32-bit signed integer for the

Naming ID Value.  The maximum value is ([2**31] - 1) or 2.14B.

It is anticipated that all Service Providers will be able to

successfully handle Naming ID Values up to this maximum.

!;

-- 40.0 LNP Service Provider NPA-NXX Value

serviceProvNPA-NXX-Value ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.NPA-NXX;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvNPA-NXX-ValueBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 40};

serviceProvNPA-NXX-ValueBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify a portable NPA-NXX value.

!;

-- 41.0 LNP Service Provider Operations Address

serviceProvOperationsAddress ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AddressInformation;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvOperationsAddressBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 41};

serviceProvOperationsAddressBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the service provider

operations contact address and contact information.

!;

-- 42.0 LNP Service Provider Repair Center Information

serviceProvRepairCenterInfo ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AddressInformation;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvRepairCenterInfoBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 42};

serviceProvRepairCenterInfoBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the repair center information

for a service provider.

!;

-- 43.0 LNP Service Provider SOA Address

serviceProvSOA-Address ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AddressInformation;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvSOA-AddressBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 43};

serviceProvSOA-AddressBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the service provider SOA address

and contact information.

!;

-- 44.0 LNP Service Provider System Link Information

serviceProvSysLinkInfo ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.NetworkAddressInformation;

MATCHES FOR EQUALITY;

BEHAVIOUR serviceProvSysLinkInfoBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 44};

serviceProvSysLinkInfoBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the system link address

information for service provider for the SOA to NPAC SMS and

NPAC SMS to Local SMS interfaces.

!;

-- 46.0 LNP Service Provider User Administration Contact Address

serviceProvUserAdminAddress ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AddressInformation;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvUserAdminAddressBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 46};

serviceProvUserAdminAddressBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the service provider

user administration contact address and contact information.

!;

-- 47.0 LNP Service Provider Web Address

serviceProvWebAddress ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AddressInformation;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvWebAddressBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 47};

serviceProvWebAddressBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the service provider Web

interface address and contact information.

!;

-- 48.0 LNP Subscription Activation Time Stamp

subscriptionActivationTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionActivationTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 48};

subscriptionActivationTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute is set by the NPAC SMS as the current time and date

that the subscription version activation request was received

from the new service provider.

!;

-- 49.0 LNP Subscription Audit Attribute List

subscriptionAuditAttributeList ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AuditAttributes;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionAuditAttributeListBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 49};

subscriptionAuditAttributeListBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the list of attributes in

a subscription version that are to be audited.

!;

-- 50.0 LNP Subscription Audit ID

subscriptionAuditId ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AuditId;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionAuditIdBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 50};

subscriptionAuditIdBehavior BEHAVIOUR

DEFINED AS !

This attribute provides an identifier for the subscriptionAudit

managed objects.  The value for this attribute is specified by

the NPAC SMS.

The NPAC SMS currently uses a 32-bit signed integer for the

Naming ID Value.  The maximum value is ([2**31] - 1) or 2.14B.

It is anticipated that all Service Providers will be able to

successfully handle Naming ID Values up to this maximum.

!;

-- 51.0 LNP Subscription Audit Name

subscriptionAuditName ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AuditName;

MATCHES FOR EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR subscriptionAuditNameBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 51};

subscriptionAuditNameBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the English name associated

with an audit.

!;

-- 52.0 LNP Subscription Audit Number of TNs to be Audited

subscriptionAuditNumberOfTNs ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AuditNumberOfTNs;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionAuditNumberOfTNsBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 52};

subscriptionAuditNumberOfTNsBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the number of TNs that

will be audited based on the audit request criteria.

!;

-- 53.0 LNP Subscription Audit Number of TNs having Completed Audit

subscriptionAuditNumberOfTNsComplete ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AuditNumberOfTNsComplete;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionAuditNumberOfTNsCompleteBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 53};

subscriptionAuditNumberOfTNsCompleteBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the number of TNs that

have completed in an in progress or complete audit.

!;

-- 54.0 LNP Subscription Audit Requesting Service Provider

subscriptionAuditRequestingSP ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.ServiceProvId;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionAuditRequestingSP-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 54};

subscriptionAuditRequestingSP-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the service provider who

requested the audit.

!;

-- 55.0 LNP Subscription Audit Service Provider Id Range

subscriptionAuditServiceProvIdRange ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AuditServiceProvIdRange;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionAuditServiceProvIdRangeBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 55};

subscriptionAuditServiceProvIdRangeBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify a specific service provider

or if all service providers should be audited in the subscription

audit.

!;

-- 56.0 LNP Subscription Audit Status

subscriptionAuditStatus ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AuditStatus;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionAuditStatusBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 56};

subscriptionAuditStatusBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the status of an audit.  Valid

values are in-progress, suspended, canceled, and complete.

!;

-- 57.0 LNP Subscription Audit TN Activation Range

subscriptionAuditTN-ActivationRange ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AuditTN-ActivationRange;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionAuditTN-ActivationRangeBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 57};

subscriptionAuditTN-ActivationRangeBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the activation date and time

range for which TNs should be audited in the subscription audit.

!;

-- 59.0 LNP Subscription Audit TN Range

subscriptionAuditTN-Range ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.TN-Range;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionAuditTN-RangeBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 59};

subscriptionAuditTN-RangeBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the TN range to be used for

the subscription audit.  The stop TN in the range must be

greater than or equal to the start TN in the range.

!;

-- 60.0 LNP Subscription Billing Id

subscriptionBillingId ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.BillingId;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionBillingIdBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 60};

subscriptionBillingIdBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the Billing Id for the

subscription version.

!;

-- 61.0 LNP Subscription Broadcast Time Stamp

subscriptionBroadcastTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionBroadcastTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 61};

subscriptionBroadcastTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the time stamp of when

the subscription version was broadcast to the service provider

Local SMSs.

!;

-- 62.0 LNP Subscription Cancellation Time Stamp

subscriptionCancellationTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionCancellationTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 62};

subscriptionCancellationTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the cancellation time

stamp for the subscription version.  This field is only valid

if the subscription version status is cancel.

!;

-- 63.0 LNP Subscription Version Class Destination Point Code

subscriptionCLASS-DPC ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.DPC;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionCLASS-DPCBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 63};

subscriptionCLASS-DPCBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the subscription version

CLASS Destination Point Code.

The 9-digit point code (DPC) is broken down into 3 components:

3-digit Network ID: Valid range 001-255.

3-digit Cluster ID: Valid range 000-255.

3-digit Member number: Valid range 000-255.

If the subscriptionCLASS-DPC attribute is provided, the

subscriptionCLASS-SSN attribute must be provided.  An

SSN value of 000 (zero) must be present when its

corresponding DPC value is populated, if the

corresponding SSN Edit Flag Indicator is set to TRUE.

The data is stored in BCD (e.g. a value of FFFFFF would be displayed

as 255.255.255).

!;

-- 64.0 LNP Subscription Version Class SSN

subscriptionCLASS-SSN ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.SSN;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionCLASS-SSN-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 64};

subscriptionCLASS-SSN-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the subscription version

CLASS SSN. The valid values on the NPAC SMS are 000-255.

If the subscriptionCLASS-SSN attribute is provided, the

subscriptionCLASS-DPC attribute must be provided. An SSN

value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit

Flag Indicator is set to TRUE.

!;

-- 65.0 LNP Subscription CNAM Destination Point Code

subscriptionCNAM-DPC ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.DPC;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionCNAM-DPC-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 65};

subscriptionCNAM-DPC-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the CNAM Destination Point

value for the subscription version.

The 9-digit point code (DPC) is broken down into 3 components:

3-digit Network ID: Valid range 001-255.

3-digit Cluster ID: Valid range 000-255.

3-digit Member number: Valid range 000-255.

If the subscriptionCNAM-DPC attribute is provided, the

subscriptionCNAM-SSN attribute must be provided.  An SSN

value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit

Flag Indicator is set to TRUE.

The data is stored in BCD (e.g. a value of FFFFFF would be displayed

as 255.255.255).

!;

-- 66.0 LNP Subscription CNAM SSN

subscriptionCNAM-SSN ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.SSN;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionCNAM-SSN-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 66};

subscriptionCNAM-SSN-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the CNAM SSN

value for the subscription version.

The valid values on the NPAC SMS are 000-255.

If the subscriptionCNAM-SSN attribute is provided, the

subscriptionCNAM-DPC attribute must be provided. An SSN

value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit

Flag Indicator is set to TRUE.

!;

-- 67.0 LNP Subscription Conflict Time Stamp

subscriptionConflictTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionConflictTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 67};

subscriptionConflictTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the time stamp of when

the subscription version was put into conflict.

!;

-- 68.0 LNP Subscription Creation Time Stamp

subscriptionCreationTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionCreationTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 68};

subscriptionCreationTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the creation date

and time for a subscription version.

!;

-- 69.0 LNP Subscription Customer Disconnect Date

subscriptionCustomerDisconnectDate ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionCustomerDisconnectDateBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 69};

subscriptionCustomerDisconnectDateBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the time stamp of when the

Subscription version was disconnected by the service provider.

!;

-- 70.0 LNP Subscription Disconnect Complete Date

subscriptionDisconnectCompleteTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionDisconnectCompleteTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 70};

subscriptionDisconnectCompleteTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the time stamp of when the

subscription version disconnect broadcast was complete.

!;

-- 71.0 LNP Subscription Download Reason

subscriptionDownloadReason ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.DownloadReason;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionDownloadReasonBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 71};

subscriptionDownloadReasonBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the reason the data was

downloaded to the Local SMS from NPAC SMS.  This attribute

only has meaning in objects instantiated on the Local SMS and is

not audited in subscription versions.

!;

-- 72.0 LNP Subscription Effective Release Date

subscriptionEffectiveReleaseDate ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionEffectiveReleaseDateBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 72};

subscriptionEffectiveReleaseDateBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the time stamp of when the

subscription version is to be disconnected.  The status

of the version must be disconnect pending.

!;

-- 73.0 LNP Subscription End User Location Type

subscriptionEndUserLocationType ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.EndUserLocationType;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionEndUserLocationTypeBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 73};

subscriptionEndUserLocationTypeBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the End User Location Type

for the subscription version.  This field is included for

future use.

!;

-- 74.0 LNP Subscription End User Location Value

subscriptionEndUserLocationValue ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.EndUserLocationValue;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionEndUserLocationValueBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 74};

subscriptionEndUserLocationValueBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the End User Location Value

for the subscription version.  This field is included for

future use.

!;

-- 75.0 LNP Subscription Failed Service Provider List

subscriptionFailed-SP-List ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.Failed-SP-List;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionFailed-SP-ListBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 75};

subscriptionFailed-SP-ListBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the failed service providers after

a subscription version and/or number pool block broadcast results in a

failed or partial failure subscription version and/or number

pool block status.

A subscription version of LNP type equal to 'pool' will have a

subscriptionFailed-SP-List for any non-EDR service providers

who failed to receive the subscription version and any EDR service

providers who failed to receive the number pool block.

!;

-- 76.0 LNP Subscription ISVM Destination Point Code

subscriptionISVM-DPC ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.DPC;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionISVM-DPC-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 76};

subscriptionISVM-DPC-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the ISVM Destination Point

value for the subscription version.

The 9-digit point code (DPC) is broken down into 3 components:

3-digit Network ID: Valid range 001-255.

3-digit Cluster ID: Valid range 000-255.

3-digit Member number: Valid range 000-255.

If the subscriptionISVM-DPC attribute is provided, the

subscriptionISVM-SSN attribute must be provided. An SSN

value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit

Flag Indicator is set to TRUE.

The data is stored in BCD (e.g. a value of FFFFFF would be

displayed as 255.255.255).

!;

-- 77.0 LNP Subscription ISVM SSN

subscriptionISVM-SSN ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.SSN;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionISVM-SSN-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 77};

subscriptionISVM-SSN-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the ISVM SSN

value for the subscription version.

The valid values on the NPAC SMS are 000-255.

If the subscriptionISVM-SSN attribute is provided, the

subscriptionISVM-DPC attribute must be provided. An SSN

value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit

Flag Indicator is set to TRUE.

!;

-- 78.0 LNP Subscription LIDB Destination Point Code

subscriptionLIDB-DPC ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.DPC;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionLIDB-DPC-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 78};

subscriptionLIDB-DPC-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the LIDB Destination Point

value for the subscription version.

The 9-digit point code (DPC) is broken down into 3 components:

3-digit Network ID: Valid range 001-255.

3-digit Cluster ID: Valid range 000-255.

3-digit Member number: Valid range 000-255.

If the subscriptionLIDB-DPC attribute is provided, the

subscriptionLIDB-SSN attribute must be provided. An SSN

value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit

Flag Indicator is set to TRUE.

The data is stored in BCD (e.g. a value of FFFFFF would be

displayed as 255.255.255).

!;

-- 79.0 LNP Subscription LIDB SSN

subscriptionLIDB-SSN ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.SSN;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionLIDB-SSN-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 79};

subscriptionLIDB-SSN-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the LIDB SSN

value for the subscription version.

The valid values on the NPAC SMS are 000-255.

If the subscriptionLIDB-SSN attribute is provided, the

subscriptionLIDB-DPC attribute must be provided. An SSN

value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit

Flag Indicator is set to TRUE.

!;

-- 80.0 LNP Subscription Local Number Portability Type

subscriptionLNPType ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.LNPType;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionLNPTypeBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 80};

subscriptionLNPTypeBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the Local Number Portability

type for the subscription version.

!;

-- 81.0 LNP Subscription LRN

subscriptionLRN ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.LRN;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionLRNBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 81};

subscriptionLRNBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the subscription LRN

for a subscription version.

The data is stored as packed decimal. For example, if the octets

contained 01 23 45 67 89 then the LRN value would be displayed

as 0123456789).

!;

-- 82.0 LNP Subscription Modified Time Stamp

subscriptionModifiedTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionModifiedTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 82};

subscriptionModifiedTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the last modification date

for a subscription version.

!;

-- 83.0 LNP Subscription New or Current Service Provider

subscriptionNewCurrentSP ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.ServiceProvId;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionNewCurrentSPBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 83};

subscriptionNewCurrentSPBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the subscription New or Current

Service Provider for a subscription version.

This attribute is also used to store the new service provider

for an old SP concurrence request notification in a log record.

!;

-- 84.0 LNP Subscription New Service Provider Cancellation Time Stamp

subscriptionNewSP-CancellationTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionNewSP-CancellationTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 84};

subscriptionNewSP-CancellationTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the subscription cancellation

concurrence time stamp for the subscription in a

cancel-pending state.  This value is specified by the

concurrence of the new service provider.

!;

-- 85.0 LNP Subscription New Service Provider Conflict Resolution Time Stamp

subscriptionNewSP-ConflictResolutionTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionNewSP-ConflictResolutionTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 85};

subscriptionNewSP-ConflictResolutionTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify when the subscription

version was removed from conflict by the new service provider.

!;

-- 86.0 LNP Subscription New Service Provider Creation Time Stamp

subscriptionNewSP-CreationTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionNewSP-CreationTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 86};

subscriptionNewSP-CreationTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the time stamp of when

the new service provider creates the cutover for the

subscription from the old service provider.  This timestamp is set

by the NPAC SMS when the new service provider sends its create

request.

This attribute is also used to store the new service provider

creation time stamp for an old SP concurrence request notification

in a log record.

!;

-- 87.0 LNP Subscription New Service Provider Activation Due Date

subscriptionNewSP-DueDate ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionNewSP-DueDateBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 87};

subscriptionNewSP-DueDateBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the subscription due

date and time for the subscription when they are being ported to

a new service provider.  This value is specified by the new service

provider.

If not specified, the time defaults to 00:00.00.

The seconds field should always be populated with zeros for

wireless ports. The NPAC SMS will not edit for compliance.

!;

-- 88.0 LNP Subscription Old Service Provider

subscriptionOldSP ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.ServiceProvId;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionOldSPBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 88};

subscriptionOldSPBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the subscription Old

Service Provider for a subscription version.

This attribute is also used to store the old service provider id

for a new service provider create request notification in a

log record.

!;

-- 89.0 LNP Subscription Old Service Provider Authorization

subscriptionOldSP-Authorization ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.ServiceProvAuthorization;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionOldSP-AuthorizationBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 89};

subscriptionOldSP-AuthorizationBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to indicate the old service

provider authorization or denial of cutover for the subscription

to the new service provider.

This attribute is also used to store the old service provider

authorization for a new service provider create request

notification in a log record.

!;

-- 90.0 LNP Subscription Old Service Provider Authorization Time Stamp

subscriptionOldSP-AuthorizationTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionOldSP-AuthorizationTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 90};

subscriptionOldSP-AuthorizationTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the time stamp of when

the old service provider authorizes or denies the cutover for the

subscription to the new service provider.  This timestamp is set

by the NPAC SMS when the old service provider sends its create

request or modifies the authorization information for

activation.

This attribute is also used to store the old service provider

authorization timestamp for an old service provider concurrence

request notification in a log record.

!;

-- 91.0 LNP Subscription Old Service Provider Cancellation Time Stamp

subscriptionOldSP-CancellationTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionOldSP-CancellationTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 91};

subscriptionOldSP-CancellationTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the subscription cancellation

time stamp for the subscription version.  This value is set by

the NPAC SMS.

!;

-- 92.0 LNP Subscription Old Service Provider Conflict Resolution Time Stamp

subscriptionOldSP-ConflictResolutionTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionOldSP-ConflictResolutionTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 92};

subscriptionOldSP-ConflictResolutionTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify when the subscription

version was removed from conflict by the old service provider.

!;

-- 93.0 LNP Subscription Old Service Provider Cutover Due Date

subscriptionOldSP-DueDate ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionOldSP-DueDateBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 93};

subscriptionOldSP-DueDateBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the subscription due

date and time for the subscription when they are being ported to a new

service provider from an old service provider.  This value

is specified by the old service provider.

The time if not specified with the date is defaulted to 00:00.00.

The seconds field should always be populated with zeros for

wireless ports. The NPAC SMS will not edit for compliance.

!;

-- 94.0 LNP Subscription Old Time Stamp

subscriptionOldTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionOldTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 94};

subscriptionOldTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the old time

stamp for the subscription version.  This field is only valid

if the subscription version status is old.

!;

-- 95.0 LNP Subscription Porting To Original SP Switch

subscriptionPortingToOriginal-SPSwitch ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.SubscriptionPortingToOriginal-SPSwitch;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionPortingToOriginal-SPSwitchBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 95};

subscriptionPortingToOriginal-SPSwitchBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify that the subscription version

created is to be to ported back to the original service

provider switch, along with the home switch of

the NPA-NXX.

!;

-- 96.0 LNP Subscription Pre-Cancellation Status

subscriptionPreCancellationStatus ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.SubscriptionPreCancellationStatus;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionPreCancellationStatusBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 96};

subscriptionPreCancellationStatusBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the previous status of a

canceled subscription version.

!;

-- 97.0 LNP Subscription Version TN

subscriptionTN ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.PhoneNumber;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionTN-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 97};

subscriptionTN-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the subscription version TN .

This attribute is also used to store the subscription version TN

for a new SP create request and a old service provider concurrence

request notification in a log record.

!;

-- 98.0 LNP Subscription Version Attribute Value Change Information

subscriptionVersionAttributeValueChangeInfo ATTRIBUTE

WITH ATTRIBUTE SYNTAX Notification-ASN1Module.AttributeValueChangeInfo;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionVersionAttributeValueChangeInfoBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 98};

subscriptionVersionAttributeValueChangeInfoBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the attribute value change

information for a subscription version or number pool block

status attribute value change notification in a log record.

!;

-- 99.0 LNP Subscription Version Id

subscriptionVersionId ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.SubscriptionVersionId;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionVersionIdBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 99};

subscriptionVersionIdBehavior BEHAVIOUR

DEFINED AS !

This attribute provides an identifier for the

lnpSubscriptions and subscriptionVersion objects.   The

NPAC SMS determines the value for this attribute.

The NPAC SMS currently uses a 32-bit signed integer for the

Naming ID Value.  The maximum value is ([2**31] - 1) or 2.14B.

It is anticipated that all Service Providers will be able to

successfully handle Naming ID Values up to this maximum.

This attribute is also used to store the subscription version Id

in notification log records.

!;

-- 100.0 LNP Subscription Version Status

subscriptionVersionStatus ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.VersionStatus;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionVersionStatusBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 100};

subscriptionVersionStatusBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the status of the

subscription version.  Valid values are pending,

conflict, sending, active, failed, partial failure, old,

canceled, disconnect-pending, and cancel-pending.

!;

-- 101.0 LNP LSMS Filter NPA-NXX ID

lsmsFilterNPA-NXX-ID ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.NPA-NXX-ID;

MATCHES FOR EQUALITY;

BEHAVIOUR lsmsFilterNPA-NXX-ID-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 101};

lsmsFilterNPA-NXX-ID-Behavior BEHAVIOUR

DEFINED AS !

This attribute provides an identifier for the

lsmsFilterNPA-NXX object.  The NPAC SMS determines the value

for this attribute.

The NPAC SMS currently uses a 32-bit signed integer for the

Naming ID Value.  The maximum value is ([2**31] - 1) or 2.14B.

It is anticipated that all Service Providers will be able to

successfully handle Naming ID Values up to this maximum.

!;

-- 102.0 LNP LSMS Filter NPA-NXX Value

lsmsFilterNPA-NXX-Value ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.NPA-NXX;

MATCHES FOR EQUALITY;

BEHAVIOUR lsmsFilterNPA-NXX-ValueBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 102};

lsmsFilterNPA-NXX-ValueBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify a portable NPA-NXX value.

!;

-- 103.0 LNP Subscription Status Change Cause Code

subscriptionStatusChangeCauseCode ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.SubscriptionStatusChangeCauseCode;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionStatusChangeCauseCodeBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 103};

subscriptionStatusChangeCauseCodeBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to indicate the reason for putting a

subscription version into conflict.

!;

-- 104.0 LNP Service Provider Security Address

serviceProvSecurityAddress ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.AddressInformation;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvSecurityAddressBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 104};

serviceProvSecurityAddressBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the security contact information

for a service provider.

!;

-- 105.0 LNP NPAC Association User Info

npacAssociationUserInfo ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.NpacAssociationUserInfo;

MATCHES FOR EQUALITY;

BEHAVIOUR npacAssociationUserInfoBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 105};

npacAssociationUserInfoBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to report the status of a login attempt or

the current state of the NPAC SMS.

!;

-- 106.0 LNP SOA Name

lnpSOA-Name ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.LnpSMS-Name;

MATCHES FOR EQUALITY;

BEHAVIOUR lnpSOA-NameBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 106};

lnpSOA-NameBehavior BEHAVIOUR

DEFINED AS !

This attribute provides an identifier for the lnpSOA

object. The valid value is the service provider id of the

SOA followed by a dash and then the region name of the

NPAC-SMS specified in the lnpNPAC-SMS-Name for the SOA to

NPAC SMS Interface. For example, if the region name is

"Midwest Regional NPAC SMS" and the service provider id is "1234",

the lnpSOA-Name would be "1234-Midwest Regional NPAC SMS".

!;

-- 107.0 Subscription Version Timer Type

subscriptionTimerType ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.Integer;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionTimerTypeBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 107};

subscriptionTimerTypeBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the subscription version

timer type being used to set tunable timers.

Current valid values are:

0 for long timers (used primarily for wireline to wireline)

1 for short timers (anticipated use for wireless to wireless)

Long timers (0) is set if any of the two service providers

supports only long timers.

!;

-- 108.0 Subscription Version Business Type

subscriptionBusinessType ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.Integer;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionTimerTypeBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 108};

subscriptionBusinessTypeBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the subscription version

business hours/days type being used to set tunable timers.

Current valid values are:

0 for short business hours/days

(used primarily for wireline to wireline)

1 for long business hours/days

(anticipated use for wireless to wireless)

Short business hours (0)is set if any of the two

service providers supports only short business hours.

!;

-- 109.0 Subscription Version WSMSC Destination Point Code

subscriptionWSMSC-DPC ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.DPC;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionWSMSC-DPCBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 109};

subscriptionWSMSC-DPCBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the subscription version

WSMSC Destination Point Code.

The 9-digit point code (DPC) is broken down into 3 components:

3-digit Network ID: Valid range 001-255.

3-digit Cluster ID: Valid range 000-255.

3-digit Member number: Valid range 000-255.

If the subscriptionWSMSC-DPC attribute is provided, the

subscriptionWSMSC-SSN attribute must be provided. An SSN

value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit

Flag Indicator is set to TRUE.

The data is stored in BCD (e.g. a value of FFFFFF would be

displayed as 255.255.255).

!;

-- 110.0 LNP Subscription Version WSMSC SSN

subscriptionWSMSC-SSN ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.SSN;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR subscriptionWSMSC-SSN-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 110};

subscriptionWSMSC-SSN-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the subscription version

WSMSC SSN. The valid values on the NPAC SMS are 000-255.

If the subscriptionWSMSC-SSN attribute is provided, the

subscriptionWSMSC-DPC attribute must be provided. An SSN

value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit

Flag Indicator is set to TRUE.

!;

-- 111.0 LNP Number Pool Block Activation TimeStamp

numberPoolBlockActivationTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR numberPoolBlockActivationTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 111};

numberPoolBlockActivationTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute defines the current date and time when the

create request is received for the number pool block.

.

!;

-- 112.0 LNP Number Pool Block Broadcast TimeStamp

numberPoolBlockBroadcastTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR numberPoolBlockBroadcastTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 112};

numberPoolBlockBroadcastTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute defines the date and time when the NPAC starts any

broadcast of the number pool block to the LSMSs.

!;

-- 113.0 LNP Number Pool Block Creation TimeStamp

numberPoolBlockCreationTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR numberPoolBlockCreationTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 113};

numberPoolBlockCreationTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute defines the date and time of the number pool block

creation.

!;

-- 114.0 LNP Number Pool Block Class Destination Point Code

numberPoolBlockCLASS-DPC ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.DPC;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR numberPoolBlockCLASS-DPCBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 114};

numberPoolBlockCLASS-DPCBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the numberPoolBlock

CLASS Destination Point Code.

The 9-digit point code (DPC) is broken down into 3 components:

3-digit Network ID: Valid range 001-255.

3-digit Cluster ID: Valid range 000-255.

3-digit Member number: Valid range 000-255.

If the numberPoolBlockCLASS-DPC attribute is provided, the

numberPoolBlockCLASS-SSN attribute must be provided. An SSN

value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit

Flag Indicator is set to TRUE.

The data is stored in BCD (e.g. a value of FFFFFF would be displayed

as 255.255.255).

!;

-- 115.0 LNP Number Pool Block Class SSN

numberPoolBlockCLASS-SSN ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.SSN;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR numberPoolBlockCLASS-SSN-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 115};

numberPoolBlockCLASS-SSN-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the numberPoolBlock

CLASS SSN. The valid values on the NPAC SMS are 000-255.

If the numberPoolBlockCLASS-SSN attribute is provided, the

numberPoolBlockCLASS-DPC attribute must be provided. An SSN

value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit

Flag Indicator is set to TRUE.

!;

-- 116.0 LNP Number Pool Block CNAM Destination Point Code

numberPoolBlockCNAM-DPC ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.DPC;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR numberPoolBlockCNAM-DPC-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 116};

numberPoolBlockCNAM-DPC-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the numberPoolBlock

CNAM Destination Point Code.

The 9-digit point code (DPC) is broken down into 3 components:

3-digit Network ID: Valid range 001-255.

3-digit Cluster ID: Valid range 000-255.

3-digit Member number: Valid range 000-255.

If the numberPoolBlockCNAM-DPC attribute is provided, the

numberPoolBlockCNAM-SSN attribute must be provided. An SSN

value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit

Flag Indicator is set to TRUE.

The data is stored in BCD (e.g. a value of FFFFFF would be displayed

as 255.255.255).

!;

-- 117.0 LNP Number Pool Block CNAM SSN

numberPoolBlockCNAM-SSN ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.SSN;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR numberPoolBlockCNAM-SSN-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 117};

numberPoolBlockCNAM-SSN-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the numberPoolBlock

CNAM SSN. The valid values on the NPAC SMS are 000-255.

If the numberPoolBlockCNAM-SSN attribute is provided, the

numberPoolBlockCNAM-DPC attribute must be provided. An SSN

value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit

Flag Indicator is set to TRUE.

!;

-- 118.0 LNP Number Pool Block Disconnect Complete TimeStamp

numberPoolBlockDisconnectCompleteTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR numberPoolBlockDisconnectCompleteTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 118};

numberPoolBlockDisconnectCompleteTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute defines the date and time the disconnect broadcast

of the number pool block is complete to the LSMSs. This is set

once the first successful response is received for the number pool

block or corresponding subscription version with LNP type equal

to 'pool'.

!;

-- 119.0 LNP Number Pool Block Data Download Reason

numberPoolBlockDownloadReason ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.DownloadReason;

MATCHES FOR EQUALITY;

BEHAVIOUR numberPoolBlockDownloadReasonBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 119};

numberPoolBlockDownloadReasonBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the reason the number pool block was

downloaded to the Local SMS from NPAC SMS.  This attribute only

has meaning in objects instantiated on the Local SMS.

!;

-- 120.0 LNP Number Pool Block Failed Service Provider List

numberPoolBlockFailed-SP-List ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.Failed-SP-List;

MATCHES FOR EQUALITY;

BEHAVIOUR numberPoolBlockFailed-SP-ListBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 120};

numberPoolBlockFailed-SP-ListBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the failed service providers after

a subscription version and/or number pool block broadcast results in a

failure to one or more Local SMSs for a subscription version with

LNP type equal to 'pool' and/or number pool block.

A numberPoolBlockFailed-SP-List reflects the EDR service

providers who failed to receive the number pool block and any non-EDR

service provider who failed to receive the corresponding subscription

versions of LNP type equal to 'pool'.

!;

-- 121.0 LNP Number Pool Block Service Provider ID

numberPoolBlockHolderSPID ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.ServiceProvId;

MATCHES FOR EQUALITY;

BEHAVIOUR numberPoolBlockHolderSPIDBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 121};

numberPoolBlockHolderSPIDBehavior BEHAVIOUR

DEFINED AS !

This attribute defines the service provider ID of the block holder.

!;

-- 122.0 LNP Number Pool Block Id

numberPoolBlockId ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.BlockId;

MATCHES FOR EQUALITY;

BEHAVIOUR numberPoolBlockIdBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 122};

numberPoolBlockIdBehavior BEHAVIOUR

DEFINED AS !

This attribute provides an identifier for the

numberPoolBlock object.  The NPAC SMS determines the value

for this attribute.

The NPAC SMS currently uses a 32-bit signed integer for the

Naming ID Value.  The maximum value is ([2**31] - 1) or 2.14B.

It is anticipated that all Service Providers will be able to

successfully handle Naming ID Values up to this maximum.

!;

-- 123.0 LNP Number Pool Block ISVM Destination Point Code

numberPoolBlockISVM-DPC ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.DPC;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR numberPoolBlockISVM-DPC-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 123};

numberPoolBlockISVM-DPC-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the ISVM Destination Point

Code for the numberPoolBlock.

The 9-digit point code (DPC) is broken down into 3 components:

3-digit Network ID: Valid range 001-255.

3-digit Cluster ID: Valid range 000-255.

3-digit Member number: Valid range 000-255.

If the numberPoolBlockISVM-DPC attribute is provided, the

numberPoolBlockISVM-SSN attribute must be provided. An SSN

value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit

Flag Indicator is set to TRUE.

The data is stored in BCD (e.g. a value of FFFFFF would be

displayed as 255.255.255).

!;

-- 124.0 LNP Number Pool Block ISVM SSN

numberPoolBlockISVM-SSN ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.SSN;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR numberPoolBlockISVM-SSN-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 124};

numberPoolBlockISVM-SSN-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the numberPoolBlock

ISVM SSN. The valid values on the NPAC SMS are 000-255.

If the numberPoolBlockISVM-SSN attribute is provided, the

numberPoolBlockISVM-DPC attribute must be provided. An SSN

value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit

Flag Indicator is set to TRUE.

!;

-- 125.0 LNP Number Pool Block LIDB Destination Point Code

numberPoolBlockLIDB-DPC ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.DPC;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR numberPoolBlockLIDB-DPC-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 125};

numberPoolBlockLIDB-DPC-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the numberPoolBlock

LIDB Destination Point Code.

The 9-digit point code (DPC) is broken down into 3 components:

3-digit Network ID: Valid range 001-255.

3-digit Cluster ID: Valid range 000-255.

3-digit Member number: Valid range 000-255.

If the numberPoolBlockLIDB-DPC attribute is provided, the

numberPoolBlockLIDB-SSN attribute must be provided. An SSN

value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit

Flag Indicator is set to TRUE.

The data is stored in BCD (e.g. a value of FFFFFF would be

displayed as 255.255.255).

!;

-- 126.0 LNP Number Pool Block LIDB SSN

numberPoolBlockLIDB-SSN ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.SSN;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR numberPoolBlockLIDB-SSN-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 126};

numberPoolBlockLIDB-SSN-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the numberPoolBlock

LIDB SSN. The valid values on the NPAC SMS are 000-255.

If the numberPoolBlockLIDB-SSN attribute is provided, the

numberPoolBlockLIDB-DPC attribute must be provided. An SSN

value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit

Flag Indicator is set to TRUE.

!;

-- 127.0 LNP Number Pool Block LRN

numberPoolBlockLRN ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.LRN;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR numberPoolBlockLRNBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 127};

numberPoolBlockLRNBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the numberPoolBlock LRN

for a numberPoolBlock.

The data is stored as packed decimal. For example, if the octets

contained 01 23 45 67 89 then the LRN value would be displayed

as 0123456789).

!;

-- 128.0 LNP Number Pool Block Last Modified TimeStamp

numberPoolBlockModifiedTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR numberPoolBlockModifiedTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 128};

numberPoolBlockModifiedTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute defines the date and time of the last

modification to the numberPoolBlock object.

!;

-- 129.0 LNP Number Pool Block SOA Origination

numberPoolBlockSOA-Origination ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.Boolean;

MATCHES FOR EQUALITY;

BEHAVIOUR numberPoolBlockSOA-OriginationBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 129};

numberPoolBlockSOA-OriginationBehavior BEHAVIOUR

DEFINED AS !

This boolean value indicates whether or not the service provider's

SOA initiated the block over the SOA to NPAC SMS interface and whether

the SOA will receive notifications for this object.

This attribute will be set by the NPAC SMS at the time of block

creation and can be updated only by NPAC SMS personnel.

!;

-- 130.0 LNP Number Pool Block NPA-NXX-X

numberPoolBlockNPA-NXX-X ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.NPA-NXX-X;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR numberPoolBlockNPA-NXX-X-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 130};

numberPoolBlockNPA-NXX-X-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the number pool block NPA-NXX-X value.

!;

-- 131.0 LNP Number Pool Block Status

numberPoolBlockStatus ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.BlockStatus;

MATCHES FOR EQUALITY;

BEHAVIOUR numberPoolBlockStatusBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 131};

numberPoolBlockStatusBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the status of the

number pool block object.  Valid values are

active(A), sending(S), failed(F), partial failure(PF) and old(O).

!;

-- 132.0 LNP Number Pool Block WSMSC Destination Point Code

numberPoolBlockWSMSC-DPC ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.DPC;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR numberPoolBlockWSMSC-DPC-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 132};

numberPoolBlockWSMSC-DPC-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the numberPoolBlock

WSMSC Destination Point Code.

The 9-digit point code (DPC) is broken down into 3 components:

3-digit Network ID: Valid range 001-255.

3-digit Cluster ID: Valid range 000-255.

3-digit Member number: Valid range 000-255.

If the numberPoolBlockWSMSC-DPC attribute is provided, the

numberPoolBlockWSMSC-SSN attribute must be provided. An SSN

value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit

Flag Indicator is set to TRUE.

The data is stored in BCD (e.g. a value of FFFFFF would be

displayed as 255.255.255).

!;

-- 133.0 LNP Number Pool Block WSMSC SSN

numberPoolBlockWSMSC-SSN ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.SSN;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR numberPoolBlockWSMSC-SSN-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 133};

numberPoolBlockWSMSC-SSN-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the numberPoolBlock

WSMSC SSN. The valid values on the NPAC SMS are 000-255.

If the numberPoolBlockWSMSC-SSN attribute is provided, the

numberPoolBlockWSMSC-DPC attribute must be provided. An SSN

value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit

Flag Indicator is set to TRUE.

!;

-- 134.0 LNP Service Provider NPA-NXX-X Creation Timestamp

serviceProvNPA-NXX-X-CreationTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvNPA-NXX-X-CreationTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 134};

serviceProvNPA-NXX-X-CreationTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute provides the date and time the

serviceProvNPA-NXX-X object was created on the NPAC SMS.

!;

-- 135.0 LNP Service Provider NPA-NXX-X Data Download Reason

serviceProvNPA-NXX-X-DownloadReason ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.DownloadReason;

MATCHES FOR EQUALITY;

BEHAVIOUR serviceProvNPA-NXX-X-DownloadReasonBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 135};

serviceProvNPA-NXX-X-DownloadReasonBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the reason the serviceProvNPA-NXX-X

was downloaded to the Local SMS from NPAC SMS. This attribute only

has meaning in objects instantiated on the Local SMS.

!;

-- 136.0 LNP Service Provider NPA-NXX-X Effective Timestamp

serviceProvNPA-NXX-X-EffectiveTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvNPA-NXX-X-EffectiveTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 136};

serviceProvNPA-NXX-X-EffectiveTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute provides the date and time the block holder service

provider assumes responsibility for the TN-Range of the NPA-NXX-X.

The time portion of the timestamp is always set to 00:00:00.

!;

-- 137.0 LNP Service Provider NPA-NXX-X ID

serviceProvNPA-NXX-X-ID ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.NPA-NXX-X-ID;

MATCHES FOR EQUALITY;

BEHAVIOUR serviceProvNPA-NXX-X-ID-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 137};

serviceProvNPA-NXX-X-ID-Behavior BEHAVIOUR

DEFINED AS !

This attribute provides an identifier for the

serviceProvNPA-NXX-X object.

The NPAC SMS currently uses a 32-bit signed integer for the

Naming ID Value.  The maximum value is ([2**31] - 1) or 2.14B.

It is anticipated that all Service Providers will be able to

successfully handle Naming ID Values up to this maximum.

!;

-- 138.0 LNP Service Provider NPA-NXX-X Modified Timestamp

serviceProvNPA-NXX-X-ModifiedTimeStamp ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.GeneralTime;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvNPA-NXX-X-ModifiedTimeStampBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 138};

serviceProvNPA-NXX-X-ModifiedTimeStampBehavior BEHAVIOUR

DEFINED AS !

This attribute provides the date and time the

serviceProvNPA-NXX-X object was last modified on the NPAC SMS.

!;

-- 139.0 LNP Service Provider NPA-NXX-X Value

serviceProvNPA-NXX-X-Value ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.NPA-NXX-X;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR serviceProvNPA-NXX-X-Value-Behavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 139};

serviceProvNPA-NXX-X-Value-Behavior BEHAVIOUR

DEFINED AS !

This attribute is used to specify the service provider NPA-NXX-X

value.

!;

-- 140.0 LNP Subscription Version Range Status Attribute Value Change

--      Information

subscriptionVersionRangeStatusAttributeValueChangeInfo ATTRIBUTE

WITH ATTRIBUTE SYNTAX

LNP-ASN1.RangeStatusAttributeValueChangeInfo;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionVersionRangeStatusAttributeValueChangeInfoBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 140};

subscriptionVersionRangeStatusAttributeValueChangeInfoBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the range status attribute value

change information for subscription version range status

attribute value change notifications in a log record.

!;

-- 141.0 LNP Subscription Version Range Attribute Value Change Information

subscriptionVersionRangeAttributeValueChangeInfo ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.RangeAttributeValueChangeInfo;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionVersionRangeAttributeValueChangeInfoBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 141};

subscriptionVersionRangeAttributeValueChangeInfoBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the range attribute value

change information for subscription version range attribute

value change notifications in a log record.

!;

-- 142.0 LNP Subscription Version Object Creation Information

subscriptionVersionRangeObjectCreationInfo ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.RangeObjectCreationInfo;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionVersionRangeObjectCreationInfoBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 142};

subscriptionVersionRangeObjectCreationInfoBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the range object creation

information for subscription version object creation information

in a log record.

!;

-- 143.0 LNP Subscription Version Donor Disconnect Information

subscriptionVersionRangeDonorSP-CustomerDisconnectDateInfo ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.RangeCustomerDisconnectDateInfo;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionVersionRangeDonorSP-CustomerDisconnectDateInfoBehavior

;

REGISTERED AS {LNP-OIDS.lnp-attribute 143};

subscriptionVersionRangeDonorSP-CustomerDisconnectDateInfoBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the range donor disconnect

information for subscription version donor disconnect information

in a log record.

!;

-- 144.0 LNP Subscription Version Cancellation Acknowledge Information

subscriptionVersionRangeCancellationAcknowledgeInfo ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.RangeCancellationAcknowledgeInfo;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionVersionRangeCancellationAcknowledgeInfoBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 144};

subscriptionVersionRangeCancellationAcknowledgeInfoBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the range cancellation acknowledge

information for subscription version cancellation acknowledge

information in a log record.

!;

-- 145.0 LNP Subscription Version New SP Create Information

subscriptionVersionRangeNewSP-CreateRequestInfo ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.RangeNewSP-CreateRequestInfo;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionVersionRangeNewSP-CreateRequestInfoBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 145};

subscriptionVersionRangeNewSP-CreateRequestInfoBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the range new SP create information

for subscription version new SP create information in a log record.

!;

-- 146.0 LNP Subscription Version Old SP Concurrence Information

subscriptionVersionRangeOldSP-ConcurrenceRequestInfo ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.RangeOldSP-ConcurrenceRequestInfo;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionVersionRangeOldSP-ConcurrenceRequestInfoBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 146};

subscriptionVersionRangeOldSP-ConcurrenceRequestInfoBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the range old SP concurrence

information for subscription version old SP concurrence information

in a log record.

!;

-- 147.0 LNP Subscription Version Old SP Final Concurrence Window Expiration

--      Information

subscriptionVersionRangeOldSPFinalConcurrenceWindowExpirationInfo ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.RangeOldSPFinalConcurrenceWindowExpirationInfo;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionVersionRangeOldSPFinalConcurrenceWindowExpirationInfoBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 147};

subscriptionVersionRangeOldSPFinalConcurrenceWindowExpirationInfoBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the range old SP final concurrence

window expiration information for subscription version old SP

final concurrence window expiration information in a log record.

!;

-- 148.0 LNP Subscription Version New SP Final Create Window Expiration

--      Information

subscriptionVersionRangeNewSP-FinalCreateWindowExpirationInfo ATTRIBUTE

WITH ATTRIBUTE SYNTAX LNP-ASN1.RangeNewSP-FinalCreateWindowExpirationInfo;

MATCHES FOR EQUALITY;

BEHAVIOUR subscriptionVersionRangeNewSP-FinalCreateWindowExpirationInfoBehavior;

REGISTERED AS {LNP-OIDS.lnp-attribute 148};

subscriptionVersionRangeNewSP-FinalCreateWindowExpirationInfoBehavior BEHAVIOUR

DEFINED AS !

This attribute is used to store the range new SP final create

window expiration information for subscription version new SP

final create window expiration information in a log record.

!;

--

-- Package Definitions

--

-- 1.0 LNP Download Package

lnpDownloadPkg PACKAGE

BEHAVIOUR lnpDownloadPkgBehavior;

ACTIONS

lnpDownload;

REGISTERED AS {LNP-OIDS.lnp-package 1};

lnpDownloadPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

lnpDownload action.

!;

-- 2.0 LNP Recovery Complete Package

lnpRecoveryCompletePkg PACKAGE

BEHAVIOUR lnpRecoveryCompletePkgBehavior;

ACTIONS

lnpRecoveryComplete;

REGISTERED AS {LNP-OIDS.lnp-package 2};

lnpRecoveryCompletePkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

lnpRecoveryCompletePkg action.

!;

-- 3.0 LNP Service Provider Billing Address Package

serviceProvBillingAddressPkg PACKAGE

BEHAVIOUR serviceProvBillingAddressPkgBehavior;

ATTRIBUTES

serviceProvBillingAddress GET-REPLACE;

REGISTERED AS {LNP-OIDS.lnp-package 3};

serviceProvBillingAddressPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

serviceProvBillingAddress attribute.

!;

-- 4.0 LNP Service Provider Conflict Address Package

serviceProvConflictAddressPkg PACKAGE

BEHAVIOUR serviceProvConflictAddressPkgBehavior;

ATTRIBUTES

serviceProvConflictAddress GET-REPLACE;

REGISTERED AS {LNP-OIDS.lnp-package 4};

serviceProvConflictAddressPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

serviceProvConflictAddress attribute.

!;

-- 5.0 LNP Service Provider LSMS Address Package

serviceProvLSMS-AddressPkg PACKAGE

BEHAVIOUR serviceProvLSMS-AddressPkgBehavior;

ATTRIBUTES

serviceProvLSMS-Address GET-REPLACE;

REGISTERED AS {LNP-OIDS.lnp-package 5};

serviceProvLSMS-AddressPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

serviceProvLSMS-Address attribute.

!;

-- 6.0 LNP Service Provider Net Address Package

serviceProvNetAddressPkg PACKAGE

BEHAVIOUR serviceProvNetAddressPkgBehavior;

ATTRIBUTES

serviceProvNetAddress GET-REPLACE;

REGISTERED AS {LNP-OIDS.lnp-package 6};

serviceProvNetAddressPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

serviceProvNetAddress attribute.

!;

-- 7.0 LNP Service Provider Operations Address Package

serviceProvOperationsAddressPkg PACKAGE

BEHAVIOUR serviceProvOperationsAddressPkgBehavior;

ATTRIBUTES

serviceProvOperationsAddress GET-REPLACE;

REGISTERED AS {LNP-OIDS.lnp-package 7};

serviceProvOperationsAddressPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

serviceProvOperationsAddress attribute.

!;

-- 8.0 LNP Service Provider Repair Center Info Package

serviceProvRepairCenterInfoPkg PACKAGE

BEHAVIOUR serviceProvRepairCenterInfoPkgBehavior;

ATTRIBUTES

serviceProvRepairCenterInfo GET-REPLACE;

REGISTERED AS {LNP-OIDS.lnp-package 8};

serviceProvRepairCenterInfoPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

serviceProvRepairCenterInfo attribute.

!;

-- 9.0 LNP Service Provider SOA Address Package

serviceProvSOA-AddressPkg PACKAGE

BEHAVIOUR serviceProvSOA-AddressPkgBehavior;

ATTRIBUTES

serviceProvSOA-Address GET-REPLACE;

REGISTERED AS {LNP-OIDS.lnp-package 9};

serviceProvSOA-AddressPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

serviceProvSOA-Address attribute.

!;

-- 10.0 LNP Service Provider User Administration Address Package

serviceProvUserAdminAddressPkg PACKAGE

BEHAVIOUR serviceProvUserAdminAddressPkgBehavior;

ATTRIBUTES

serviceProvUserAdminAddress GET-REPLACE;

REGISTERED AS {LNP-OIDS.lnp-package 10};

serviceProvUserAdminAddressPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

serviceProvUserAdminAddress attribute.

!;

-- 11.0 LNP Service Provider Web Address Package

serviceProvWebAddressPkg PACKAGE

BEHAVIOUR serviceProvWebAddressPkgBehavior;

ATTRIBUTES

serviceProvWebAddress GET-REPLACE;

REGISTERED AS {LNP-OIDS.lnp-package 11};

serviceProvWebAddressPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

serviceProvWebAddress attribute.

!;

-- 12.0 LNP Subscription Version Activate Package

subscriptionVersionActivatePkg PACKAGE

BEHAVIOUR subscriptionVersionActivatePkgBehavior;

ACTIONS

subscriptionVersionActivate;

REGISTERED AS {LNP-OIDS.lnp-package 12};

subscriptionVersionActivatePkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionVersionActivate action.

!;

-- 13.0 LNP Subscription Version Attribute Value Change Failed Service

--      Providers List

subscriptionVersionAttributeValueChangeFailed-SP-ListPkg PACKAGE

BEHAVIOUR

subscriptionVersionAttributeValueChangeFailed-SP-ListPkgBehavior;

ATTRIBUTES

subscriptionFailed-SP-List GET;

REGISTERED AS {LNP-OIDS.lnp-package 13};

subscriptionVersionAttributeValueChangeFailed-SP-ListPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionVersionAttributeValueChangeFailed-SP-List

attribute.

!;

-- 14.0 LNP Subscription Version Cancel Package

subscriptionVersionCancelPkg PACKAGE

BEHAVIOUR subscriptionVersionCancelPkgBehavior;

ACTIONS

subscriptionVersionCancel;

REGISTERED AS {LNP-OIDS.lnp-package 14};

subscriptionVersionCancelPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionVersionCancel action.

!;

-- 15.0 LNP Subscription Version Disconnect Package

subscriptionVersionDisconnectPkg PACKAGE

BEHAVIOUR subscriptionVersionDisconnectPkgBehavior;

ACTIONS

subscriptionVersionDisconnect;

REGISTERED AS {LNP-OIDS.lnp-package 15};

subscriptionVersionDisconnectPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionVersionDisconnect action.

!;

-- 16.0 LNP Subscription Version Local SMS Create Package

subscriptionVersionLocalSMS-CreatePkg PACKAGE

BEHAVIOUR subscriptionVersionLocalSMS-CreatePkgBehavior;

ACTIONS

subscriptionVersionLocalSMS-Create;

REGISTERED AS {LNP-OIDS.lnp-package 16};

subscriptionVersionLocalSMS-CreatePkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for including the

subscriptionVersionLocalSMS-Create action.

!;

-- 17.0 LNP Subscription Version Modify Package

subscriptionVersionModifyPkg PACKAGE

BEHAVIOUR subscriptionVersionModifyPkgBehavior;

ACTIONS

subscriptionVersionModify;

REGISTERED AS {LNP-OIDS.lnp-package 17};

subscriptionVersionModifyPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionVersionModify action.

!;

-- 18.0 LNP New Service Provider Subscription Version Cancellation

-- Acknowledge Package

subscriptionVersionNewSP-CancellationPkg PACKAGE

BEHAVIOUR subscriptionVersionNewSP-CancellationPkgBehavior;

ACTIONS

subscriptionVersionNewSP-CancellationAcknowledge;

REGISTERED AS {LNP-OIDS.lnp-package 18};

subscriptionVersionNewSP-CancellationPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionVersionNewSP-CancellationAcknowledge action.

!;

-- 20.0 LNP Subscription Version Remove From Conflict

-- Pending Package

subscriptionVersionRemoveFromConflictPkg PACKAGE

BEHAVIOUR subscriptionVersionRemoveFromConflictPkgBehavior;

ACTIONS

subscriptionVersionRemoveFromConflict;

REGISTERED AS {LNP-OIDS.lnp-package 20};

subscriptionVersionRemoveFromConflictPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionVersionRemoveFromConflict action.

!;

-- 21.0 LNP New Service Provider Subscription Version Create Package

subscriptionVersionNewSP-CreatePkg PACKAGE

BEHAVIOUR subscriptionVersionNewSP-CreatePkgBehavior;

ACTIONS

subscriptionVersionNewSP-Create;

REGISTERED AS {LNP-OIDS.lnp-package 21};

subscriptionVersionNewSP-CreatePkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionVersionNewSP-Create action.

!;

-- 22.0 LNP Old Service Provider Subscription Version Cancellation

-- Acknowledge Package

subscriptionVersionOldSP-CancellationPkg PACKAGE

BEHAVIOUR subscriptionVersionOldSP-CancellationPkgBehavior;

ACTIONS

subscriptionVersionOldSP-CancellationAcknowledge;

REGISTERED AS {LNP-OIDS.lnp-package 22};

subscriptionVersionOldSP-CancellationPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionVersionOldSP-CancellationAcknowledge action.

!;

-- 24.0 LNP Old Service Provider Subscription Version Create Package

subscriptionVersionOldSP-CreatePkg PACKAGE

BEHAVIOUR subscriptionVersionOldSP-CreatePkgBehavior;

ACTIONS

subscriptionVersionOldSP-Create;

REGISTERED AS {LNP-OIDS.lnp-package 24};

subscriptionVersionOldSP-CreatePkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionVersionOldSP-Create action.

!;

-- 25.0 LNP Subscription Status Change Cause Code Package

subscriptionStatusChangeCauseCodePkg PACKAGE

BEHAVIOUR subscriptionStatusChangeCauseCodePkgBehavior;

ATTRIBUTES

subscriptionStatusChangeCauseCode GET;

REGISTERED AS {LNP-OIDS.lnp-package 25};

subscriptionStatusChangeCauseCodePkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionStatusChangeCauseCode attribute.

!;

-- 26.0 LNP Service Provider Security Address Package

serviceProvSecurityAddressPkg PACKAGE

BEHAVIOUR serviceProvSecurityAddressPkgBehavior;

ATTRIBUTES

serviceProvSecurityAddress GET-REPLACE;

REGISTERED AS {LNP-OIDS.lnp-package 26};

serviceProvSecurityAddressPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

serviceProvSecurityAddress attribute.

!;

-- 27.0 LNP Notification Recovery Package

lnpNotificationRecoveryPkg PACKAGE

BEHAVIOUR lnpNotificationRecoveryPkgBehavior;

ACTIONS

lnpNotificationRecovery;

REGISTERED AS {LNP-OIDS.lnp-package 27};

lnpNotificationRecoveryPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

lnpNotificationRecovery action.

!;

-- 28.0 LNP Subscription Audit TN Activation Range Package

subscriptionAuditTN-ActivationRangePkg PACKAGE

BEHAVIOUR subscriptionAuditTN-ActivationRangePkgBehavior;

ATTRIBUTES

subscriptionAuditTN-ActivationRange GET;

REGISTERED AS {LNP-OIDS.lnp-package 28};

subscriptionAuditTN-ActivationRangePkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionAuditTN-ActivationRange attribute.

!;

-- 29.0 LNP Subscription Timer Type Package

subscriptionTimerTypePkg PACKAGE

BEHAVIOUR subscriptionTimerTypePkgBehavior;

ATTRIBUTES

subscriptionTimerType GET;

REGISTERED AS {LNP-OIDS.lnp-package 29};

subscriptionTimerTypePkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionTimerType attribute.

!;

-- 30.0 LNP Subscription Business Type Package

subscriptionBusinessTypePkg PACKAGE

BEHAVIOUR subscriptionBusinessTypePkgBehavior;

ATTRIBUTES

subscriptionBusinessType GET;

REGISTERED AS {LNP-OIDS.lnp-package 30};

subscriptionBusinessTypePkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionBusinessType attribute.

!;

-- 31.0 LNP Subscription WSMSC Data Package

subscriptionWSMSC-DataPkg PACKAGE

BEHAVIOUR subscriptionWSMSC-DataPkgBehavior;

ATTRIBUTES

subscriptionWSMSC-DPC GET-REPLACE,

subscriptionWSMSC-SSN GET-REPLACE;

REGISTERED AS {LNP-OIDS.lnp-package 31};

subscriptionWSMSC-DataPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

WSMSC DPC and SSN attributes.

!;

-- 32.0 LNP Number Pool Block Create Package

numberPoolBlock-CreatePkg PACKAGE

BEHAVIOUR numberPoolBlock-CreatePkgBehavior;

ACTIONS

numberPoolBlock-Create;

REGISTERED AS {LNP-OIDS.lnp-package 32};

numberPoolBlock-CreatePkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

numberPoolBlock-Create action.

!;

-- 33.0 LNP Number Pool Block Attribute Value Change Failed Service

--      Providers List

numberPoolBlockAttributeValueChangeFailed-SP-ListPkg PACKAGE

BEHAVIOUR

numberPoolBlockAttributeValueChangeFailed-SP-ListPkgBehavior;

ATTRIBUTES

numberPoolBlockFailed-SP-List GET;

REGISTERED AS {LNP-OIDS.lnp-package 33};

numberPoolBlockAttributeValueChangeFailed-SP-ListPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

numberPoolBlockAttributeValueChangeFailed-SP-List

attribute on the

lnpLogNumberPoolBlockStatusAttributeValueChangeRecord object.

!;

-- 34.0 LNP Number Pool Block WSMSC Data Package

numberPoolBlockWSMSC-DataPkg PACKAGE

BEHAVIOUR numberPoolBlockWSMSC-DataPkgBehavior;

ATTRIBUTES

numberPoolBlockWSMSC-DPC GET-REPLACE,

numberPoolBlockWSMSC-SSN GET-REPLACE;

REGISTERED AS {LNP-OIDS.lnp-package 34};

numberPoolBlockWSMSC-DataPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

WSMSC DPC and SSN attributes.

!;

-- 35.0 LNP Subscription Version Range Attribute Value Change Package

subscriptionVersionRangeAttributeValueChangePkg PACKAGE

BEHAVIOUR subscriptionVersionRangeAttributeValueChangePkgBehavior;

NOTIFICATIONS

subscriptionVersionRangeAttributeValueChange;

REGISTERED AS {LNP-OIDS.lnp-package 35};

subscriptionVersionRangeAttributeValueChangePkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionVersionRangeAttributeValueChange notification.

!;

-- 36.0 LNP Subscription Version Range Object Creation Package

subscriptionVersionRangeObjectCreationPkg PACKAGE

BEHAVIOUR subscriptionVersionRangeObjectCreationPkgBehavior;

NOTIFICATIONS

subscriptionVersionRangeObjectCreation;

REGISTERED AS {LNP-OIDS.lnp-package 36};

subscriptionVersionRangeObjectCreationPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionVersionRangeObjectCreation notification.

!;

-- 37.0 LNP Subscription Version Range Status Attribute Value Change Package

subscriptionVersionRangeStatusAttributeValueChangePkg PACKAGE

BEHAVIOUR subscriptionVersionRangeStatusAttributeValueChangePkgBehavior;

NOTIFICATIONS

subscriptionVersionRangeStatusAttributeValueChange;

REGISTERED AS {LNP-OIDS.lnp-package 37};

subscriptionVersionRangeStatusAttributeValueChangePkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionVersionRangeStatusAttributeValueChange notification.

!;

-- 38.0 LNP Subscription Version Range Donor Disconnect Package

subscriptionVersionRangeDonorSP-CustomerDisconnectDatePkg PACKAGE

BEHAVIOUR subscriptionVersionRangeDonorSP-CustomerDisconnectDatePkgBehavior;

NOTIFICATIONS

subscriptionVersionRangeDonorSP-CustomerDisconnectDate;

REGISTERED AS {LNP-OIDS.lnp-package 38};

subscriptionVersionRangeDonorSP-CustomerDisconnectDatePkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionVersionRangeDonorSP-CustomerDisconnectDate notification.

!;

-- 39.0 LNP Subscription Version Range Cancellation Acknowledge Package

subscriptionVersionRangeCancellationAcknowledgePkg PACKAGE

BEHAVIOUR subscriptionVersionRangeCancellationAcknowledgePkgBehavior;

NOTIFICATIONS

subscriptionVersionRangeCancellationAcknowledgeRequest;

REGISTERED AS {LNP-OIDS.lnp-package 39};

subscriptionVersionRangeCancellationAcknowledgePkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionVersionRangeCancellationAcknowledge notification.

!;

-- 40.0 LNP Subscription Version Range New SP Create Package

subscriptionVersionRangeNewSP-CreateRequestPkg PACKAGE

BEHAVIOUR subscriptionVersionRangeNewSP-CreateRequestPkgBehavior;

NOTIFICATIONS

subscriptionVersionRangeNewSP-CreateRequest;

REGISTERED AS {LNP-OIDS.lnp-package 40};

subscriptionVersionRangeNewSP-CreateRequestPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionVersionRangeNewSP-CreateRequest notification.

!;

-- 41.0 LNP Subscription Version Range Old SP Concurrence Package

subscriptionVersionRangeOldSP-ConcurrenceRequestPkg PACKAGE

BEHAVIOUR subscriptionVersionRangeOldSP-ConcurrenceRequestPkgBehavior;

NOTIFICATIONS

subscriptionVersionRangeOldSP-ConcurrenceRequest;

REGISTERED AS {LNP-OIDS.lnp-package 41};

subscriptionVersionRangeOldSP-ConcurrenceRequestPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionVersionRangeOldSP-ConcurrenceRequest notification.

!;

-- 42.0 LNP Subscription Version Range Old SP Final Concurrence Window

--      Expiration Package

subscriptionVersionRangeOldSPFinalConcurrenceWindowExpirationPkg PACKAGE

BEHAVIOUR subscriptionVersionRangeOldSPFinalConcurrenceWindowExpirationPkgBehavior;

NOTIFICATIONS

subscriptionVersionRangeOldSPFinalConcurrenceWindowExpiration;

REGISTERED AS {LNP-OIDS.lnp-package 42};

subscriptionVersionRangeOldSPFinalConcurrenceWindowExpirationPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionVersionRangeOldSPFinalConcurrenceWindowExpiration notificati

on.

!;

-- 43.0 LNP Subscription Version Range New SP Final Create Window

--      Expiration Package

subscriptionVersionRangeNewSP-FinalCreateWindowExpirationPkg PACKAGE

BEHAVIOUR subscriptionVersionRangeNewSP-FinalCreateWindowExpirationPkgBehavior;

NOTIFICATIONS

subscriptionVersionRangeNewSP-FinalCreateWindowExpiration;

REGISTERED AS {LNP-OIDS.lnp-package 43};

subscriptionVersionRangeNewSP-FinalCreateWindowExpirationPkgBehavior BEHAVIOUR

DEFINED AS !

This package provides for conditionally including the

subscriptionVersionRangeNewSP-FinalCreateWindowExpiration notification.

!;

--

-- Parameter Definitions

--

-- 1.0 Access Control Parameter

accessControlParameter PARAMETER

CONTEXT EVENT-INFO;

WITH SYNTAX LNP-ASN1.LnpAccessControl;

REGISTERED AS {LNP-OIDS.lnp-parameter 1};

-- 2.0 LNP Specific Info Parameter

lnpSpecificInfoParameter PARAMETER

CONTEXT SPECIFIC-ERROR;

--    ATTRIBUTE lnpSpecificInfo;

WITH SYNTAX LNP-ASN1.LnpSpecificInfo;

REGISTERED AS {LNP-OIDS.lnp-parameter 2};

-- Action Definitions

-- 1.0 LNP Download Action

lnpDownload ACTION

BEHAVIOUR

lnpDownloadDefinition,

lnpDownloadBehavior;

MODE CONFIRMED;

WITH INFORMATION SYNTAX LNP-ASN1.DownloadAction;

WITH REPLY SYNTAX LNP-ASN1.DownloadReply;

REGISTERED AS {LNP-OIDS.lnp-action 1};

lnpDownloadDefinition BEHAVIOUR

DEFINED AS !

The lnpDownload action is the action that is used by the Local SMS

and SOA to specify the objects to be downloaded from the NPAC SMS.

!;

lnpDownloadBehavior BEHAVIOUR

DEFINED AS !

Preconditions: This action is issued from an lnpSubscriptions

or an lnpNetwork object and all objects to be downloaded

are specified in the action request.

Postconditions: After this action has been executed by the Local

SMS or SOA specifying which objects to download, the NPAC SMS will

determine which objects satisfy the download request and return

them in the download action reply. Creation, deletion, and

modification information will be included in the reply.  All data

for objects that have been modified is downloaded not just the

information that was modified.

Data to be downloaded can be specified by a time range of last

modification/creation or by other criteria.  Time range requests

will be limited to a tunable range specified in the NPAC SMS, and

must be specified in Coordinated Universal Time (UTC).

All data modified/created in the download time period, regardless

of the amount of data, will be downloaded.  For download requests

not specifying a time range, the amount of data downloaded will be

limited to a tunable amount as specified in the NPAC SMS.

Criteria for a subscription download is a time range or a TN or

TN range.  For TN ranges the stop TN in the range must be greater

than the start TN in the range. If the NPAC Customer LSMS EDR

Indicator is set in the service provider's profile

on the NPAC SMS, no subscription versions with LNP type equal to

'pool' will be returned. NPA-NXX filters are also applied.

Criteria for a number pool block download is time range, NPA-NXX-X

range or NPA-NXX-X.

Criteria for a network data download is a time range, service

provider id or all service providers, an npa-nxx range or all

npa-nxx data, an npa-nxx-x range or all npa-nxx-x data, an LRN

range or all LRN data, or all network data. Specifying

"all-network-data" includes the serviceProvNPA-NXX-X object if the

Local SMS supports the object according to their service provider

profile NPAC Customer LSMS NPA-NXX-X Indicator. The SOA supports

the object according to their service provider profile NPAC Customer

SOA NPA-NXX-X Indicator.

For all download requests, the Local SMS or SOA should behave as

follows in response to the possible download M-ACTION response

from the NPAC SMS:

Success - process the data received from the NPAC SMS, continue

processing.

No-data-selected -- no data was found, continue processing.

Criteria-too-large (using the Maximum Number of Download Records

tunable) - break up the request into a smaller time range and

re-issue the request to the NPAC SMS (only applies to the

subscription version requests).

OR

Criteria-too-large (using the Maximum Number of Download

Notifications tunable) - break up the request into a smaller time

ranges and re-issue the request to the NPAC SMS.

Time-range-invalid (using the Maximum Download Duration tunable) -

break up the request into shorter time ranges and

re-issue the request to the NPAC SMS.

Failed - go into retry mode. Re-issue the request a configurable

number of additional retry attempts with an "x" amount of delay

between requests ("x" is a configurable amount of time after

receiving the failure for each request). If a failed response

is received for the final retry request, abort the association

and re-start the recovery process. Note: It is recommended that

the Local SMS or SOA use the same value that the NPAC SMS uses

for the retry interval. It is also recommended that the Local SMS

use a value of at least two (2) for configurable number of

additional retry attempts.

For activities that specify "continue processing", the Local SMS or

SOA should send the NPAC SMS, either the next lnpDownload action for

a different type of data, or an lnpRecoveryComplete request, depending

on where the response appears in the flow.

SOAs can use the lnpDownload action to recover network data. The

lnpNotificationRecovery action is used to recover notification data.

Local SMSs can use the lnpDownload action to recover network data,

subscription version data, and number pool block data.  The

lnpNotificationRecovery action is used to recover notification data.

Subscription versions and number pool blocks with a status of active

and partial-failure will be recovered.  Also Subscription

versions and number pool blocks with a status of sending will be

recovered.

The SOA or LSMS is capable of recovering data based on the association

functions. The SOA recovers network data using the data download

association function (dataDownload). The SOA recovers notification data

using the network data management association function (networkDataMgmt).

The LSMS recovers network data, subscription data, and number pool block

data using the data download association function (dataDownload) and

recovers notification data using the network data management association

function (networkDataMgmt).

Linked replies.

The SOA receives linked replies if their Service Provider SOA Linked

Replies Indicator is set to TRUE on the NPAC SMS, and the amount of

data is greater than the associated Blocking Factor.

The LSMS receives linked replies if their Service Provider LSMS Linked

Replies Indicator is set to TRUE on the NPAC SMS, and the amount of

data is greater than the associated Blocking Factor.

When the SOA Linked Replies Indicator is set to TRUE for SOA requests,

or the Local SMS Linked Replies Indicator is set to TRUE for Local

SMS requests, linked replies will be returned as the response to a

lnpDownload action request for network data if the number of messages

returned is less than the "Network Data Maximum Linked Recovered

Objects" tunable, and the amount of data is greater than the "Network

Data Linked Replies Blocking Factor" tunable.

If the number of network data objects to be returned exceeds the

"Network Data Maximum Linked Recovered Objects" tunable, a

"criteria-too-large" error will be returned to the requesting SOA/LSMS.

Each linked reply sent in response to a successfully processed

lnpDownload action request for network data will contain a status of

success (0).  The returned data is sent in replies based on the

"Network Data Linked Replies Blocking Factor" tunable, such that each

linked reply is no larger than the "Network Data Linked Replies

Blocking Factor" tunable value.  The returned data is followed by an

empty reply, to indicate the end of the data.  If the number of

network data objects to be returned is less than or equal to the

"Network Data Linked Replies Blocking Factor" tunable, a single

non-linked reply will be sent back to the requesting SOA/LSMS.

When the Local SMS Linked Replies Indicator is set to TRUE, linked

replies will be returned as the response to a lnpDownload Action

request for subscription data if the number of messages returned is

less than the "Subscription Data Maximum Linked Recovered Objects"

tunable, and the amount of data is greater than the "Subscription

Data Linked Replies Blocking Factor" tunable.

If the number of subscription data messages to be returned exceeds

the "Subscription Data Maximum Linked Recovered Objects" tunable, a

"criteria-too-large" error will be returned to the requesting LSMS.

Each linked reply sent in response to a successfully processed

lnpDownload action request for subscription data will contain a

status of success (0).  The returned data is sent in replies based on

the "Subscription Data Linked Replies Blocking  Factor" tunable, such

that each linked reply is no larger than the "Subscription Data Linked

Replies Blocking Factor" tunable value.  The returned data is followed

by an empty reply, to indicate the end of the data.  If the number of

subscription data objects to be returned is less than or equal to the

"Subscription Data Linked Replies Blocking Factor" tunable, a single

non-linked reply will be sent back to the requesting LSMS.

When the Local SMS Linked Replies Indicator is set to TRUE, linked

replies will be returned as the response to a lnpDownload Action

request for number pool block data if the number of messages returned

is less than the "Number Pool Block Data Maximum Linked Recovered

Objects" tunable, and the amount of data is greater than the "Number

Pool Block Data Linked Replies Blocking Factor" tunable.

If the number of number pool block data messages to be returned exceeds

the "Number Pool Block Data Maximum Linked Recovered Objects" tunable,

a "criteria-too-large" error will be returned to the requesting LSMS.

Each linked reply sent in response to a successfully processed

lnpDownload action request for number pool block data will contain a

status of success (0).  The returned data is sent in replies based on

the "Number Pool Block Data Linked Replies Blocking Factor" tunable,

such that each linked reply is no larger than the "Number Pool Block

Data Linked Replies Blocking Factor" tunable value.  The returned data

is followed by an empty reply, to indicate the end of the data.  If the

number of number pool block data objects to be returned is less than or

equal to the "Number Pool Block Data Linked Replies Blocking Factor"

tunable, a single non-linked reply will be sent back to the requesting

LSMS.

!;

-- 2.0 LNP Recovery Complete Action

lnpRecoveryComplete ACTION

BEHAVIOUR

lnpRecoveryCompleteDefinition,

lnpRecoveryCompleteBehavior;

MODE CONFIRMED;

WITH INFORMATION SYNTAX LNP-ASN1.RecoveryCompleteAction;

WITH REPLY SYNTAX LNP-ASN1.RecoveryCompleteReply;

REGISTERED AS {LNP-OIDS.lnp-action 2};

lnpRecoveryCompleteDefinition BEHAVIOUR

DEFINED AS !

The lnpRecoveryComplete action is used by the Local SMS or SOA

to specify the system has recovered from downtime and the

transactions performed since the association establishment can now be

sent from the NPAC SMS.

!;

lnpRecoveryCompleteBehavior BEHAVIOUR

DEFINED AS !

Preconditions: This action is issued from an LSMS or SOA that

specified the recovery mode flag in the access control as true at

association establishment.

Postconditions: After this action has been executed by the Local

SMS or SOA specifying recovery is complete, the NPAC SMS will

forward those updates requested which took place for the network

subscription and number pool block data as well as any notifications

since the association was established. The

NPAC SMS will queue up all new events while the Local SMS is in

recovery mode and send them to the Local SMS at the next

scheduled retry interval after responding with the lnpRecoveryComplete

action reply.

If a recovery complete request fails in the NPAC SMS the failure reason

will be returned in the reply.

The NPAC SMS will queue up all new events while the Local SMS is in

recovery mode, and send them to the Local SMS after responding with the

lnpRecoveryComplete action reply.

!;

-- 3.0 LNP Subscription Version Activate Action

subscriptionVersionActivate ACTION

BEHAVIOUR

subscriptionVersionActivateDefinition,

subscriptionVersionActivateBehavior;

MODE CONFIRMED;

WITH INFORMATION SYNTAX LNP-ASN1.ActivateAction;

WITH REPLY SYNTAX LNP-ASN1.ActivateReply;

REGISTERED AS {LNP-OIDS.lnp-action 3};

subscriptionVersionActivateDefinition BEHAVIOUR

DEFINED AS !

The subscriptionVersionActivate action is the action that can be

used by the SOA of the new service provider to activate a

subscription version id, tn or a range of tns via the SOA to

NPAC SMS interface.

!;

subscriptionVersionActivateBehavior BEHAVIOUR

DEFINED AS !

Preconditions: This action is issued from an lnpSubscriptions

object specifying the object or range of objects to be activated by

either subscriptionVersionId, the subscriptionTN or range of TNs

(where the stop TN in the range is greater than the start TN).

Subscription versions with subscriptionLNPType equal to 'pool'

cannot be specified in the action.

Postconditions: The service provider has activated the subscription

version.  An error will be returned if the subscription version

can not be activated for any reason.

Only pending subscription versions can be activated.  Attempts to

port subscription versions that have not been authorized by both

service providers will fail unless the due date has been reached.

!;

-- 4.0 LNP Subscription Version Cancel Action

subscriptionVersionCancel ACTION

BEHAVIOUR

subscriptionVersionCancelDefinition,

subscriptionVersionCancelBehavior;

MODE CONFIRMED;

WITH INFORMATION SYNTAX LNP-ASN1.CancelAction;

WITH REPLY SYNTAX LNP-ASN1.CancelReply;

REGISTERED AS {LNP-OIDS.lnp-action 4};

subscriptionVersionCancelDefinition BEHAVIOUR

DEFINED AS !

The subscriptionVersionCancel action is the action that can be

used by the SOA to cancel a subscription version via the SOA to

NPAC SMS interface.

!;

subscriptionVersionCancelBehavior BEHAVIOUR

DEFINED AS !

Preconditions: This action is issued from an lnpSubscriptions

object specifying the object or objects to be canceled by either

the subscriptionVersionId, the subscriptionTN or a range of TNs

(where the stop TN in the range is greater than the start TN).

Postconditions: If the status was pending or conflict,

the service provider has set the version status

to cancel-pending if the other service provider has concurred, or

to cancel if the other service provider has not concurred. If the

status was disconnect-pending, the service provider has

set the version status back to active. An

error will be returned if there is no version that can be canceled

or the service provider is not authorized.

!;

-- 5.0 LNP Subscription Version Disconnect Action

subscriptionVersionDisconnect ACTION

BEHAVIOUR

subscriptionVersionDisconnectDefinition,

subscriptionVersionDisconnectBehavior;

MODE CONFIRMED;

WITH INFORMATION SYNTAX LNP-ASN1.DisconnectAction;

WITH REPLY SYNTAX LNP-ASN1.DisconnectReply;

REGISTERED AS {LNP-OIDS.lnp-action 5};

subscriptionVersionDisconnectDefinition BEHAVIOUR

DEFINED AS !

The subscriptionVersionDisconnect action is the action that is

used by the SOA to disconnect a subscription version via the SOA to

NPAC SMS interface.

!;

subscriptionVersionDisconnectBehavior BEHAVIOUR

DEFINED AS !

Preconditions: This action is issued from an lnpSubscriptions

object and specifies the object or objects to be disconnected by

either stating the subscriptionVersionId, the subscriptionTN or a

range of TNs (where the stop TN in the range is greater than the

start TN).  In addition, the customer's disconnect date is

specified. An optional effective release date can be specified for

a time deferred disconnect.

Subscription versions with subscriptionLNPType equal to 'pool'

cannot be specified in the action.

Postconditions: The current service provider can disconnect an active

subscription version.  An error will be returned to the service

provider if there is no active version. If there is a pending

version and an active version, the disconnect of the active

subscription version will fail.

If the version is active, no outstanding versions exist,

and the time stamp for disconnect has not been reached, the

subscription version will be modified with a version status of

disconnect-pending and the subscriptionEffectiveReleaseDate

set to the effective release date specified in the action.

If the version is active, there are no outstanding versions, and

the time stamp for effective release has not been specified, the

subscription version will be updated with a version status of

sending.

When the new subscription version status is set to sending either

immediately or at the date and time specified in the

subscriptionEffectiveReleaseDate, the broadcast time stamp is

set to the current time when the disconnect version sending starts

to the Local SMSs via the NPAC SMS to Local SMS interface.

Before the broadcast of deletes begins, the

subscriptionVersionDonorSP-CustomerDisconnectDate notification

is sent to the donor SOA informing the service provider of the

actual customer disconnect date.

If the delete requests are successful for all Local SMSs, the

current active version will have its version status marked as old

and the subscriptionDisconnectCompleteTimeStamp is set to the

current system date and time.

If a delete request fails for the disconnect subscription

version after the retry periods have expired, the version

status will be set to active if all Local SMSs fail, or set to

old if one or more, but not all, Local SMSs fail.

!;

-- 6.0 LNP Subscription Version Local SMS Create Action

subscriptionVersionLocalSMS-Create ACTION

BEHAVIOUR

subscriptionVersionLocalSMS-CreateDefinition,

subscriptionVersionLocalSMS-CreateBehavior;

MODE CONFIRMED;

WITH INFORMATION SYNTAX LNP-ASN1.LocalSMS-CreateAction;

WITH REPLY SYNTAX LNP-ASN1.LocalSMS-CreateReply;

REGISTERED AS {LNP-OIDS.lnp-action 6};

subscriptionVersionLocalSMS-CreateDefinition BEHAVIOUR

DEFINED AS !

The subscriptionVersionLocalSMS-Create action is the action that is

used by the NPAC SMS to create multiple subscription versions via the

Local SMS to NPAC SMS interface.

!;

subscriptionVersionLocalSMS-CreateBehavior BEHAVIOUR

DEFINED AS !

Preconditions: This action is issued from an lnpSubscriptions

object specifying the objects in a range to be created by

the subscriptionVersionId and the subscriptionTN.  All attribute

values required for creation will be supplied.

Postconditions: A successful reply indicates the Local SMS can

decipher the subscription version create action. An error will be

returned to the NPAC SMS if the Local SMS cannot recognize the

action data.

The Local SMS will attempt to create all the specified subscription

versions. It will return the subscriptionVersionActionResults

notification to the NPAC SMS informing it of the success or

failure of the creation attempts.

For Release 1.4 Number Pooling Support:

There will be no need on the part of the LSMS to validate

the TN-range.  The LSMS will use the subscriptionVersionObjects

to create the subscription versions for the TN range in the LSMS.

This is done to insure that the subscription version ids used

in the NPAC SMS and the Local SMS are the same.

!;

-- 7.0 LNP Subscription Version Modify Action

subscriptionVersionModify ACTION

BEHAVIOUR

subscriptionVersionModifyDefinition,

subscriptionVersionModifyBehavior;

MODE CONFIRMED;

WITH INFORMATION SYNTAX LNP-ASN1.ModifyAction;

WITH REPLY SYNTAX LNP-ASN1.ModifyReply;

REGISTERED AS {LNP-OIDS.lnp-action 7};

subscriptionVersionModifyDefinition BEHAVIOUR

DEFINED AS !

The subscriptionVersionModify action is the action that can be

used by the SOA to modify a subscription version via the SOA to

NPAC SMS interface.

!;

subscriptionVersionModifyBehavior BEHAVIOUR

DEFINED AS !

Preconditions: This action is issued from an lnpSubscriptions

object specifying the object to be modified by specifying

the subscriptionVersionId or by specifying the subscriptionTN or

a range of TNs (where the stop TN in the range is greater than

the start TN) and the status of the subscription version.  All

attribute values to be modified shall also be specified.

Postconditions: The NPAC SMS has modified the subscription

version.  An error will be returned to the service provider if

there is no version that is modifiable or if the modification fails

due to authorization of the service provider or data validation.

Subscription versions with subscriptionLNPType equal to 'pool'

cannot be specified in the action.

Service Providers can modify attributes associated with active,

pending, disconnect-pending or conflict subscription versions.

Old service providers can only modify the following attributes

for pending subscription versions:

subscriptionOldSP-DueDate

subscriptionOldSP-Authorization

subscriptionStatusChangeCauseCode

If the subscription version has a status of conflict, only the

subscriptionOldSP-DueDate can be modified because a subscription

version can only be put into conflict one time.

The subscriptionStatusChangeCauseCode is an optional field and is

only specified if the subscriptionOldSP-Authorization is false.

New service providers can only modify the following attributes

for pending or conflict subscription versions:

subscriptionLRN

subscriptionNewSP-DueDate

subscriptionCLASS-DPC

subscriptionCLASS-SSN

subscriptionLIDB-DPC

subscriptionLIDB-SSN

subscriptionCNAM-DPC

subscriptionCNAM-SSN

subscriptionISVM-DPC

subscriptionISVM-SSN

subscriptionEndUserLocationValue

subscriptionEndUserLocationType

subscriptionBillingId

New service providers may specify modified valid values for the

following attributes, when the service provider's "SOA WSMSC DPC SSN

Data" indicator is TRUE, and may NOT specify these values when the

indicator is set to FALSE:

subscriptionWSMSC-DPC

subscriptionWSMSC-SSN

A valid SSN value must be present for the corresponding DPC value and

vice versa.  An SSN value of 000 (zero) must be present when its

corresponding DPC value is populated, if the corresponding SSN Edit

Flag Indicator is set to TRUE.

Validation will be done for both old and new service provider

data that is specified for pending or conflict

subscription versions.

If validation fails no changes will be made and an error

will be returned. If validation passes, the version will be

modified and remain in a pending, disconnect-pending or active state.

New service providers can only modify the following attributes

for active subscription versions:

subscriptionLRN

subscriptionCLASS-DPC

subscriptionCLASS-SSN

subscriptionLIDB-DPC

subscriptionLIDB-SSN

subscriptionCNAM-DPC

subscriptionCNAM-SSN

subscriptionISVM-DPC

subscriptionISVM-SSN

subscriptionEndUserLocationValue

subscriptionEndUserLocationType

subscriptionBillingId

New service providers may specify modified valid values for the

following attributes, when the service provider's "SOA WSMSC DPC SSN

Data" indicator is TRUE, and may NOT specify these values when the

indicator is set to FALSE:

subscriptionWSMSC-DPC

subscriptionWSMSC-SSN

A valid SSN value must be present for the corresponding DPC value and

vice versa. An SSN value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit Flag Indicator

is set to TRUE.

For the modification of an active subscription version where

the data specified passes validation, the modified version

is immediately broadcast.  The modified subscription version will have

a status of sending and broadcasts will begin.  If validation fails,

no changes will be made and an error will be returned in the action

reply.

The new/current service provider can only modify the following

attributes for a disconnect-pending subscription version:

subscriptionCustomerDisconnectDate (required)

subscriptionEffectiveReleaseDate (optional)

For the modification of a disconnect-pending subscription version

where the Effective Release Date is specified to a current or

previous date/time, and passes validation, then the modified version

is immediately broadcast.  The modified subscription version will have

a status of sending and broadcasts will begin.  If validation fails,

no changes will be made and an error will be returned in the action

reply.  No attributes other than Customer Disconnect Date or Effective

Release Date can be modified on a disconnect-pending subscription version.

!;

-- 8.0 LNP New Service Provider Cancellation Acknowledge Request

subscriptionVersionNewSP-CancellationAcknowledge ACTION

BEHAVIOUR

subscriptionVersionNewSP-CancellationAcknowledgeDefinition,

subscriptionVersionNewSP-CancellationAcknowledgeBehavior;

MODE CONFIRMED;

WITH INFORMATION SYNTAX LNP-ASN1.CancellationAcknowledgeAction;

WITH REPLY SYNTAX LNP-ASN1.CancellationAcknowledgeReply;

REGISTERED AS {LNP-OIDS.lnp-action 8};

subscriptionVersionNewSP-CancellationAcknowledgeDefinition BEHAVIOUR

DEFINED AS !

The subscriptionVersionNewSP-CancellationAcknowledge action

is the action that is used via the SOA to NPAC

SMS interface by the new service provider to acknowledge

cancellation of a subscriptionVersionNPAC with a status of

cancel-pending.

!;

subscriptionVersionNewSP-CancellationAcknowledgeBehavior BEHAVIOUR

DEFINED AS !

Preconditions: This action was issued from an lnpSubscriptions

object specifying the object or objects to be acknowledged by either

the subscriptionVersionId, the subscriptionTN or a range of

subscriptionTNs (where the stop TN in the range is greater than

the start TN).

Postconditions: The service provider has acknowledged the

subscription version.  An error will be returned to the service

provider if no version exists that can have the cancellation

acknowledged or if the acknowledgement fails due to

the service provider not being authorized to perform the action.

The subscriptionNewSP-CancellationTimeStamp will be

updated to the current time if the action is successful and the

version status is changed to cancel.

!;

-- 10.0 LNP Subscription Version Remove From Conflict

subscriptionVersionRemoveFromConflict ACTION

BEHAVIOUR

subscriptionVersionRemoveFromConflictDefinition,

subscriptionVersionRemoveFromConflictBehavior;

MODE CONFIRMED;

WITH INFORMATION SYNTAX LNP-ASN1.RemoveFromConflictAction;

WITH REPLY SYNTAX LNP-ASN1.RemoveFromConflictReply;

REGISTERED AS {LNP-OIDS.lnp-action 10};

subscriptionVersionRemoveFromConflictDefinition BEHAVIOUR

DEFINED AS !

The subscriptionVersionRemoveFromConflict action

is the action that is used via the SOA to NPAC

SMS interface by either the old or new service provider to set the

subscription version status from conflict to pending.

!;

subscriptionVersionRemoveFromConflictBehavior BEHAVIOUR

DEFINED AS !

Preconditions: This action was issued from an lnpSubscriptions

object specifying the object or objects to be updated by either

the subscriptionVersionId, the subscriptionTN or a range of

subscriptionTNs (where the stop TN in the range is greater than

the start TN).

Postconditions: The NPAC SMS has acknowledged the

subscription version.  An error will be returned to the service

provider if there is no version that can have the conflict

status removed or if the service provider is not authorized to

perform the action.

If the action is successful, either the

subscriptionNewSPConflictResolutionTimeStamp or

subscriptionOldSP-ConflictResolutionTimeStamp will be updated to

the current time, the version status will be changed from conflict

to pending, and the subscriptionOldSP-Authorization attribute

will be modified to true.

If the old service provider issues the action, the

subscriptionOldSP-AuthorizationTimeStamp is also updated to the

current date and time.

!;

-- 11.0 LNP New Service Provider Subscription Version Create

subscriptionVersionNewSP-Create ACTION

BEHAVIOUR

subscriptionVersionNewSP-CreateDefinition,

subscriptionVersionNewSP-CreateBehavior;

MODE CONFIRMED;

WITH INFORMATION SYNTAX LNP-ASN1.NewSP-CreateAction;

WITH REPLY SYNTAX LNP-ASN1.NewSP-CreateReply;

REGISTERED AS {LNP-OIDS.lnp-action 11};

subscriptionVersionNewSP-CreateDefinition BEHAVIOUR

DEFINED AS !

The subscriptionVersionNewSP-Create action is the action that is

used via the SOA to NPAC SMS interface by the

new service provider to create a new subscriptionVersionNPAC.

!;

subscriptionVersionNewSP-CreateBehavior BEHAVIOUR

DEFINED AS !

Preconditions: This action is issued from an lnpSubscriptions

object.  Creates can be performed provided there is only one

currently active subscription or no subscription version in the

NPAC; otherwise an action failure will be returned.

The new service provider must specify valid values for the

following attributes:

subscriptionTN or a valid subscriptionVersionTN-Range

subscriptionLRN

subscriptionNewCurrentSP

subscriptionOldSP

subscriptionNewSP-DueDate

subscriptionCLASS-DPC

subscriptionCLASS-SSN

subscriptionLIDB-DPC

subscriptionLIDB-SSN

subscriptionCNAM-DPC

subscriptionCNAM-SSN

subscriptionISVM-DPC

subscriptionISVM-SSN

subscriptionLNPType

subscriptionPortingToOriginal-SPSwitch

The new service provider must specify valid values for the following

attributes, when the service provider's "SOA WSMSC DPC SSN Data"

indicator is TRUE, and must NOT specify these values when the

indicator is set to FALSE:

subscriptionWSMSC-DPC

subscriptionWSMSC-SSN

A valid SSN value must be present for the corresponding DPC value and

vice versa. An SSN value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit Flag Indicator

is set to TRUE.

The new service provider may specify valid values for the

following attributes:

subscriptionEndUserLocationValue

subscriptionEndUserLocationType

subscriptionBillingId

subscriptionPortingToOriginal-SPSwitch can only be specified as

TRUE for a TN that is currently ported and is being ported back

to the original service provider, along with the home switch of

the NPA-NXX.  If the value of subscriptionPortingToOriginal-SPSwitch

is TRUE, the LRN and GTT data should be specified as NULL.  If

the value of subscriptionPortingToOriginal-SPSwitch is TRUE,

when the activate occurs for the subscription version, the Local

SMSs will receive a request to delete the old subscription version

routing data in their networks. They will not receive any

new network routing data for the subscription. Concurrence from the

old service provider is required.  If the value of

subscriptionPortingToOriginal-SPSwitch is TRUE, the LNP

Type may be either "LSPP" or "LISP".

If the port of the subscription version is an intra-service

provider port, the new service provider can use the

subscriptionVersionNewSP-Create action specifying the old service

provider equal to the new service provider.  In this case, the

old service provider create action is not required.

Postconditions: After this action has been executed, if

the data specified passes validation, a pending subscription

version or range of subscription versions will exist in the

NPAC SMS.  These validations are done as follows:

subscriptionTN or range of TNs are valid in a range open for

porting by the new service provider. TN ranges must be specified

where the stop TN in the range is greater than the start TN.

subscriptionLNPType is specified to be "LSPP" or "LISP".

subscriptionNewSP-DueDate is a future date.  If not specified,

the time defaults to 00:00.00.

Old and New SP are valid service providers in the NPAC SMS.

LRN data is associated with the New Service Provider.

If a pre-existing version exists, validation will be done to insure

that the new service provider previously specified is the same

as the executor of the action.

If the validations succeed and the subscription version does not

currently exist, a new subscription version will be created with

a status of pending.

If the validations succeed and a pending subscription version exists,

the new service provider create information will be applied to the

existing pending subscription version.

If the validations fail, a new subscription version will not

be created if one does not exist.  If one already existed, it

will be retained.

The action success or failure and reasons for failure will be

returned in the action reply.

!;

-- 12.0 LNP Old Service Provider Cancellation Acknowledge Request

subscriptionVersionOldSP-CancellationAcknowledge ACTION

BEHAVIOUR

subscriptionVersionOldSP-CancellationAcknowledgeDefinition,

subscriptionVersionOldSP-CancellationAcknowledgeBehavior;

MODE CONFIRMED;

WITH INFORMATION SYNTAX LNP-ASN1.CancellationAcknowledgeAction;

WITH REPLY SYNTAX LNP-ASN1.CancellationAcknowledgeReply;

REGISTERED AS {LNP-OIDS.lnp-action 12};

subscriptionVersionOldSP-CancellationAcknowledgeDefinition BEHAVIOUR

DEFINED AS !

The subscriptionVersionOldSP-CancellationAcknowledge action

is the action that is used via the SOA to NPAC

SMS interface by the old service provider to acknowledge

cancellation of a subscriptionVersionNPAC with a status of

cancel-pending.

!;

subscriptionVersionOldSP-CancellationAcknowledgeBehavior BEHAVIOUR

DEFINED AS !

Preconditions: This action was issued from an lnpSubscriptions

object specifying the object or objects to be acknowledged by either

the subscriptionVersionId, the subscriptionTN or a range of

subscriptionTNs (where the stop TN in the range is greater than the

start TN) and status.

Postconditions: The service provider has acknowledged the

subscription version.  An error will be returned to the service

provider if there is no version that can have cancellation

acknowledged or if the acknowledgement fails due to

the service provider not being authorized to perform the action.

The subscriptionOldSP-CancellationTimeStamp will be

updated to the current time if the action is successful and the

version status will be changed to cancel.

!;

-- 14.0 LNP Old Service Provider Subscription Version Create

subscriptionVersionOldSP-Create ACTION

BEHAVIOUR

subscriptionVersionOldSP-CreateDefinition,

subscriptionVersionOldSP-CreateBehavior;

MODE CONFIRMED;

WITH INFORMATION SYNTAX LNP-ASN1.OldSP-CreateAction;

WITH REPLY SYNTAX LNP-ASN1.OldSP-CreateReply;

REGISTERED AS {LNP-OIDS.lnp-action 14};

subscriptionVersionOldSP-CreateDefinition BEHAVIOUR

DEFINED AS !

The subscriptionVersionOldSP-Create action is the action that is

used via the SOA to NPAC SMS interface by the

old service provider to create a new subscriptionVersionNPAC.

!;

subscriptionVersionOldSP-CreateBehavior BEHAVIOUR

DEFINED AS !

Preconditions: This action was issued from an lnpSubscriptions

object.  Creates can be performed provided there is only one

currently active subscription or action failure will be returned.

The old service provider must specify valid values for the

following attributes:

subscriptionTN or a valid subscriptionVersionTN-Range

subscriptionNewCurrentSP

subscriptionOldSP

subscriptionOldSP-DueDate

subscriptionOldSP-Authorization

subscriptionLNPType

If the subscriptionOldSP-Authorization is false, the old service

provider must specify a subscriptionStatusChangeCauseCode.

Postconditions: After this action has been executed if

the data specified passes validation, a pending subscription

version will exist in the NPAC SMS.  These validations are

done as follows:

subscriptionTN or range of TNs are valid in a range open for

porting. TN ranges must be specified where the stop TN in the

range is greater than the start TN.

subscriptionLNPType is specified as "LSPP" or "LISP".

subscriptionOldSP-DueDate is a future date. If not specified,

the time defaults to 00:00.00.

Old and New SP are valid service providers in the NPAC SMS and

the new service provider is not equal to the old service provider.

If a pre-existing version exists, validation will be done to insure

that the old service provider previously specified is the same

as the executor of the action.

If the validations succeed and a pending subscription version does

not exist, a subscription version will be created with a status

of pending.

If the validations succeed and a pending subscription version

exists, the old service provider create information will be applied

to the existing pending subscription version.

If the validations fail, a new subscription version will not

be created if one does not exist.  If one already existed it

will be retained and an error returned.

The action success or failure and reasons for failure will be

returned in the action reply.

!;

-- 15.0 Notification Recovery Action

lnpNotificationRecovery ACTION

BEHAVIOUR

lnpNotificationRecoveryDefinition,

lnpNotificationRecoveryBehavior;

MODE CONFIRMED;

WITH INFORMATION SYNTAX LNP-ASN1.NetworkNotificationRecoveryAction;

WITH REPLY SYNTAX LNP-ASN1.NetworkNotificationRecoveryReply;

REGISTERED AS {LNP-OIDS.lnp-action 15};

lnpNotificationRecoveryDefinition BEHAVIOUR

DEFINED AS !

The lnpNotificationRecovery action is the action that can be

used by the SOA or LSMS to recover notification information that

cannot be recovered by other means.

!;

lnpNotificationRecoveryBehavior BEHAVIOUR

DEFINED AS !

Preconditions: This action is issued from an lnpNPAC-SMS object

from a SOA or LSMS that specified the recovery mode flag in the access

control as true at association establishment.

Postconditions: After this action has been executed by the SOA or LSMS

specifying recovery, the NPAC SMS will forward the notifications

that occurred in the time range specified for the requesting system

(SOA or LSMS) for the primary or associated SPID specified in the

access control. Notifications are forwarded in the action reply.

Notifications to be recovered are requested by time range. Time range

requests will be limited to a tunable range specified in the NPAC

SMS. All data in the download time period, regardless of the amount

of data, will be returned.

The recovery of the SOA and LSMS notifications are independent

requests.  Notifications can be recovered until they are

purged from the database.  The tunable used to determine when to

purge the notifications is "Notify Log Retention Period" which

defaults to 90 days.

For all download requests, the Local SMS or SOA should behave as

follows in response to the possible download M-ACTION response

from the NPAC SMS:

Success - process the data received from the NPAC SMS, continue

processing.

No-data-selected -- no data was found, continue processing.

Criteria-too-large (using the MaxNotificationRecovery tunable) -

break up the request into a smaller time range and

re-issue the request to the NPAC SMS.

Time-range-invalid (using the Maximum Download Duration tunable) -

break up the request into shorter time ranges and

re-issue the request to the NPAC SMS.

Failed - go into retry mode. Re-issue the request a configurable

number of additional retry attempts with an "x" amount of delay

between requests ("x" is a configurable amount of time after

receiving the failure for each request). If a failed response

is received for the final retry request, abort the association

and re-start the recovery process. Note: It is recommended that

the Local SMS or SOA use the same value that the NPAC SMS uses

for the retry interval. It is also recommended that the Local SMS

use a value of at least two (2) for configurable number of

additional retry attempts.

For activities that specify "continue processing", the Local SMS or

SOA should send the NPAC SMS, either the next lnpDownload action for

a different type of data, or an lnpRecoveryComplete request, depending

on where the response appears in the flow.

The SOA or LSMS is capable of recovering data based on the association

functions. The SOA recovers network data using the data download

association function (dataDownload). The SOA recovers notification data

using the network data management association function (networkDataMgmt).

The LSMS recovers network data and subscription data using the data

download association function (dataDownload) and recovers notification

data using the network data management association function

(networkDataMgmt).

The SOA recovers the range notifications when the Service Provider

TN Range Notification Indicator is set to TRUE on the NPAC SMS and

the single notifications when the Service Provider TN Range

Notification Indicator is set to FALSE on the NPAC SMS.

Linked Replies.

The SOA receives linked replies if their Service Provider

SOA Linked Replies Indicator is set to TRUE on the NPAC SMS, and

the amount of data is greater than the associated Blocking Factor.

The LSMS receives linked replies if their Service Provider

LSMS Linked Replies Indicator is set to TRUE on the NPAC SMS, and

the amount of data is greater than the associated Blocking Factor.

When the SOA or Local SMS Linked Replies Indicator is set to TRUE,

linked replies will be returned as the response to an

LnpNotificationRecovery Action request for notification data if

The number of notifications returned is less than the

"Notification Data Maximum Linked Recovered Notifications"

tunable, and the amount of data is greater than the "Notification

Data Linked Replies Blocking Factor" tunable.

If the number of Notifications to be returned exceeds

the "Notification Data Maximum Linked Recovered Notifications"

tunable, a "criteria-too-large" error will be returned to the

requesting SOA/LSMS.

Each linked reply sent in response to a successfully processed

lnpNotificationRecovery action request for notification data

will contain a status of success (0).  The returned data is

sent in replies based on the "Notification Data Linked Replies

Blocking Factor" tunable, such that each linked reply is no

larger than the "Notification Data Linked Replies Blocking Factor"

tunable value.  The returned data is followed by an empty reply,

to indicate the end of the data.  If the number of notification

data objects to be returned is less than or equal to the

"Notification Data Linked Replies Blocking Factor" tunable, a

single non-linked reply will be sent back to the requesting SOA/LSMS.

!;

-- 16.0 LNP Service Provider Number Pool Block Create

numberPoolBlock-Create ACTION

BEHAVIOUR

numberPoolBlock-CreateDefinition,

numberPoolBlock-CreateBehavior;

MODE CONFIRMED;

WITH INFORMATION SYNTAX LNP-ASN1.NumberPoolBlock-CreateAction;

WITH REPLY SYNTAX LNP-ASN1.NumberPoolBlock-CreateReply;

REGISTERED AS {LNP-OIDS.lnp-action 16};

numberPoolBlock-CreateDefinition BEHAVIOUR

DEFINED AS !

The numberPoolBlock-Create action is the action that is

used on the NPAC SMS via the SOA to NPAC SMS interface by the

block holder SOA to create a new numberPoolBlockNPAC.

!;

numberPoolBlock-CreateBehavior BEHAVIOUR

DEFINED AS !

Preconditions: This action is issued from an lnpSubscriptions

object.

The service provider block holder must specify valid values for the

following attributes:

numberPoolBlockNPA-NXX-X

numberPoolBlockHolderSPID

numberPoolBlockLRN

numberPoolBlockCLASS-DPC

numberPoolBlockCLASS-SSN

numberPoolBlockLIDB-DPC

numberPoolBlockLIDB-SSN

numberPoolBlockCNAM-DPC

numberPoolBlockCNAM-SSN

numberPoolBlockISVM-DPC

numberPoolBlockISVM-SSN

If the SOA WSMSC DPC SSN Data Indicator is set in the service

provider's profile, the following attributes must be provided:

numberPoolBlockWSMSC-DPC

numberPoolBlockWSMSC-SSN

A valid SSN value must be present for the corresponding DPC value and

vice versa. An SSN value of 000 (zero) must be present when its corresponding

DPC value is populated, if the corresponding SSN Edit Flag Indicator

is set to TRUE.

Postconditions: After this action has been executed, if

the data specified passes validation, a number pool block with a

status of 'sending' will exist on the NPAC SMS.

The validations performed are as follows:

The serviceProvNPA-NXX-X object exists and the current date and time

are greater than or equal to the effective date of the

serviceProvNPA-NXX-X object.

The requesting service provider is the block holder and service

provider ID on the serviceProvNPA-NXX-X object.

LRN data is associated with the Block Holder Service Provider.

The attributes are all valid and correctly formatted.

If the validations succeed, a new number pool block and corresponding

subscription versions will be created with a status of 'sending'.

The NPAC SMS sends the object creation notification for the

number pool block.

If the validations fail, no new number pool block or subscription

versions will be created. If a number pool block already existed, it

will be retained.

The action success or failure and reasons for failure will be

returned in the action reply.

If the requesting SOA is not the owner of the serviceProvNPA-NXX-X,

'soa-not-authorized' will be selected in the error reply.

If the corresponding serviceProvNPA-NXX-X is not found,

'no-npa-nxx-x-found' will be selected in the error reply.

If one of the attribute values is in error, 'invalid-data-values'

will be selected in the error reply and the block-invalid-values

will be present.

If the number pool block object already exists,

'number-pool-block-already-exists' will be selected in the error

reply.

If the request is sent prior to the effective date,

'prior-to-effective-date' will be selected in the error reply.

If the request is sent and any subscription version objects exist

within the TN range with a status of pending, conflict,

cancel-pending or failed ("pending-like") and no active subscription

version for that TN exists, 'invalid-subscription-versions' will be

selected in the error reply.

!;

--

-- Notification Definitions

--

-- 1.0 LNP NPAC SMS Operational Information Notification

lnpNPAC-SMS-Operational-Information NOTIFICATION

BEHAVIOUR  lnpNPAC-SMS-Operational-InformationBehavior;

WITH INFORMATION SYNTAX LNP-ASN1.NPAC-SMS-Operational-Information

AND ATTRIBUTE IDS

down-time downTime,

npac-contact-number npacContactNumber,

additional-down-time-information additionalDownTimeInformation,

access-control accessControl;

REGISTERED AS {LNP-OIDS.lnp-notification 1};

lnpNPAC-SMS-Operational-InformationBehavior BEHAVIOUR

DEFINED AS !

This notification contains information about the NPAC SMS's

scheduled down time.  This notification contains the start and

stop date and time for the planned down time. It is sent to both the

SOA and Local SMS systems.

This notification is prioritised and transmitted according to its

SOA Notification Priority tunable in the NPAC SMS when sent over

the NPAC SMS to SOA interface.

!;

-- 2.0 LNP Subscription Audit Local SMS Discrepancy Report

subscriptionAudit-DiscrepancyRpt NOTIFICATION

BEHAVIOUR  subscriptionAudit-DiscrepancyRptBehavior;

WITH INFORMATION SYNTAX LNP-ASN1.AuditDiscrepancyRpt

AND ATTRIBUTE IDS

tn auditDiscrepancyTn,

version-id auditDiscrepancyVersionId,

lsms-service-prov-id auditDiscrepancyLSMS-SP-Id,

failure-reason auditDiscrepancyFailureReason,

access-control accessControl;

REGISTERED AS {LNP-OIDS.lnp-notification 2};

subscriptionAudit-DiscrepancyRptBehavior BEHAVIOUR

DEFINED AS !

This notification contains a report on a discrepancy found during

an audit.  The discrepancy contains the subscription TN and Version

ID for which the discrepancy was found and the error.  Valid

errors are:

audited subscription version fields mismatched between NPAC SMS

and Local SMS; records missing in Local SMS; extra subscription

versions on the Local SMS.

If field mismatches are found, the attribute(s) for which the

mismatch, the Local SMS value(s), and the NPAC SMS value(s)

will be returned as well as the Service Provider Id associated

with the Local SMS.

When audit discrepancy notifications are sent by the NPAC SMS,

the Local SMS create, modification, or deletion requests to correct

the discrepancy will be done by the NPAC SMS.

This notification is prioritised and transmitted according to its

SOA Notification Priority tunable in the NPAC SMS.

!;

-- 3.0 LNP Subscription Audit Results

subscriptionAuditResults NOTIFICATION

BEHAVIOUR  subscriptionAuditResultsBehavior;

WITH INFORMATION SYNTAX LNP-ASN1.AuditResults

AND ATTRIBUTE IDS

status auditResultStatus,

failed-service-prov-list auditResultFailed-SP-List,

number-of-discrepancies auditResultNumberDiscrepancies,

time-of-completion auditResultCompletionTime,

access-control accessControl;

REGISTERED AS {LNP-OIDS.lnp-notification 3};

subscriptionAuditResultsBehavior BEHAVIOUR

DEFINED AS !

This notification contains the results of an audit.  It contains

the name of the audit, the number of discrepancies found during the

audit, the success or failure of the audit, and the time of audit

completion or failure.

The audit status will be returned with the following priority

on the return values:

Highest - failed due to discrepancies

High - failed on Local SMS

Low - no audit performed

Lowest - success

A higher priority status condition will override a lower.  For

example, any error will override a 'no audit performed', and

'failed due to discrepancies' will override all other status

conditions.

This notification is prioritised and transmitted according to its

SOA Notification Priority tunable in the NPAC SMS.

!;

-- 4.0 LNP Subscription Version Cancellation Resolution Request

-- Notification

subscriptionVersionCancellationAcknowledgeRequest NOTIFICATION

BEHAVIOUR  subscriptionVersionCancellationAcknowledgeBehavior;

WITH INFORMATION SYNTAX

LNP-ASN1.VersionCancellationAcknowledgeRequest

AND ATTRIBUTE IDS

tn subscriptionTN,

version-id subscriptionVersionId,

access-control accessControl;

REGISTERED AS {LNP-OIDS.lnp-notification 4};

subscriptionVersionCancellationAcknowledgeBehavior BEHAVIOUR

DEFINED AS !

This notification requests that a service provider send

a cancellation acknowledgement for a subscription

version.  The TN and the version id are sent.

The service provider supports this notification if the Service

Provider TN Range Notification Indicator is turned off on the NPAC SMS.

This notification is prioritised and transmitted according to its

SOA Notification Priority tunable in the NPAC SMS.

!;

-- 6.0 LNP Subscription Version Donor Service Provider Customer

--     Disconnect Date Notification

subscriptionVersionDonorSP-CustomerDisconnectDate NOTIFICATION

BEHAVIOUR  subscriptionVersionDonorSP-CustomerDisconnectDateBehavior;

WITH INFORMATION SYNTAX LNP-ASN1.VersionCustomerDisconnectDate

AND ATTRIBUTE IDS

tn subscriptionTN,

version-id subscriptionVersionId,

service-prov-customer-disconnect-date

subscriptionCustomerDisconnectDate,

service-prov-effective-release-date

subscriptionEffectiveReleaseDate,

access-control accessControl;

REGISTERED AS {LNP-OIDS.lnp-notification 6};

subscriptionVersionDonorSP-CustomerDisconnectDateBehavior BEHAVIOUR

DEFINED AS !

This notification informs the donor service provider SOA

that a subscription version is being disconnected.

The TN, the version id, customer disconnect date and

effective release date (optional) values are sent.

The service provider supports this notification if the Service

Provider TN Range Notification Indicator is turned off on the NPAC SMS.

This notification is prioritised and transmitted according to its

SOA Notification Priority tunable in the NPAC SMS.

!;

-- 7.0 LNP Subscription Version Local SMS Action Results

subscriptionVersionLocalSMS-ActionResults NOTIFICATION

BEHAVIOUR  subscriptionVersionLocalSMS-ActionResultsBehavior;

WITH INFORMATION SYNTAX LNP-ASN1.LocalSMS-ActionResults

AND ATTRIBUTE IDS

actionId actionId,

status actionResultsStatus,

failed-tn-list failedTN-List,

time-of-completion resultsCompletionTime,

accessControl accessControl;

REGISTERED AS {LNP-OIDS.lnp-notification 7};

subscriptionVersionLocalSMS-ActionResultsBehavior BEHAVIOUR

DEFINED AS !

This notification contains the results of a

subscriptionVersionLocalSMS-Create action from a Local SMS.

It contains the id of the create action, the success

or failure of the action, the completion time and an

optional list of failed subscription TNs and error codes.

!;

-- 8.0 LNP Subscription Version New NPA-NXX Notification

subscriptionVersionNewNPA-NXX NOTIFICATION

BEHAVIOUR  subscriptionVersionNewNPA-NXXBehavior;

WITH INFORMATION SYNTAX

LNP-ASN1.VersionNewNPA-NXX

AND ATTRIBUTE IDS

service-prov-npa-nxx-id serviceProvNPA-NXX-ID,

service-prov-npa-nxx-value serviceProvNPA-NXX-Value,

service-prov-npa-nxx-effective-time-stamp

serviceProvNPA-NXX-EffectiveTimeStamp,

service-prov-id serviceProvID,

access-control accessControl;

REGISTERED AS {LNP-OIDS.lnp-notification 8};

subscriptionVersionNewNPA-NXXBehavior BEHAVIOUR

DEFINED AS !

This notification informs the SOA and Local SMS of a pending

subscription version involving a new NPA-NXX or creation of

a serviceProvNPA-NXX-X object whose NPA-NXX-X value is the

first use of the NPA-NXX.  The

service-prov-npa-nxx-id, service-prov-npa-nxx-value,

service-prov-npa-nxx-effective-time-stamp and service-prov-id

are sent.

This notification is prioritised and transmitted according to its

SOA Notification Priority tunable in the NPAC SMS when sent over

the NPAC SMS to SOA interface.

!;

-- 9.0 LNP Subscription Version New SP Create Request Notification

subscriptionVersionNewSP-CreateRequest NOTIFICATION

BEHAVIOUR  subscriptionVersionNewSP-CreateRequestBehavior;

WITH INFORMATION SYNTAX LNP-ASN1.VersionNewSP-CreateRequest

AND ATTRIBUTE IDS

tn subscriptionTN,

version-id subscriptionVersionId,

service-prov-id subscriptionOldSP,

service-prov-due-date subscriptionOldSP-DueDate,

service-prov-old-authorization subscriptionOldSP-Authorization,

service-prov-authorization-creation-time-stamp

subscriptionOldSP-AuthorizationTimeStamp,

status-change-cause-code subscriptionStatusChangeCauseCode,

access-control accessControl,

subscription-timer-type subscriptionTimerType,

subscription-business-type subscriptionBusinessType;

REGISTERED AS {LNP-OIDS.lnp-notification 9};

subscriptionVersionNewSP-CreateRequestBehavior BEHAVIOUR

DEFINED AS !

This notification requests that a new service provider send

a create request for a subscription version for which

concurrence for porting the number has not been received.

The TN, the version id and the old service provider id,

authorization flag and authorization timestamp values are sent.

If the new service provider supports timer type, it will be sent.

If the new service provider supports business type, it will be sent.

The service provider supports this notification if the Service

Provider TN Range Notification Indicator is turned off on the NPAC SMS.

This notification is prioritised and transmitted according to its

SOA Notification Priority tunable in the NPAC SMS.

!;

-- 10.0 LNP Subscription Version Old SP Concurrence Request Notification

subscriptionVersionOldSP-ConcurrenceRequest NOTIFICATION

BEHAVIOUR  subscriptionVersionOldSP-ConcurrenceRequestBehavior;

WITH INFORMATION SYNTAX LNP-ASN1.VersionOldSP-ConcurrenceRequest

AND ATTRIBUTE IDS

tn subscriptionTN,

version-id subscriptionVersionId,

service-prov-id subscriptionNewCurrentSP,

service-prov-due-date subscriptionNewSP-DueDate,

service-prov-authorization-creation-time-stamp

subscriptionNewSP-CreationTimeStamp,

access-control accessControl,

subscription-timer-type subscriptionTimerType,

subscription-business-type subscriptionBusinessType;

REGISTERED AS {LNP-OIDS.lnp-notification 10};

subscriptionVersionOldSP-ConcurrenceRequestBehavior BEHAVIOUR

DEFINED AS !

This notification requests that a old service provider send

a create request for a subscription version for which

concurrence for porting the number has not been received.

The TN, the version id, and the new service provider id,

authorization flag and creation timestamp values are sent. If

the old service provider supports timer type, it will be sent. If

the old service provider supports business type, it will be sent.

The service provider supports this notification if the Service

Provider TN Range Notification Indicator is turned off on the NPAC SMS.

This notification is prioritised and transmitted according to its

SOA Notification Priority tunable in the NPAC SMS.

!;

-- 11.0 LNP Subscription Version Status Attribute Value Change Notification

subscriptionVersionStatusAttributeValueChange NOTIFICATION

BEHAVIOUR  subscriptionVersionStatusAttributeValueChangeBehavior;

WITH INFORMATION SYNTAX  LNP-ASN1.VersionStatusAttributeValueChange

AND ATTRIBUTE IDS

value-change-info subscriptionVersionAttributeValueChangeInfo,

failed-service-provs subscriptionFailed-SP-List,

status-change-cause-code subscriptionStatusChangeCauseCode,

access-control accessControl;

REGISTERED AS {LNP-OIDS.lnp-notification 11};

subscriptionVersionStatusAttributeValueChangeBehavior BEHAVIOUR

DEFINED  AS !

This notification type is used to report changes to the

subscriptionVersionStatus field.  It is identical to an

attribute value change notification as defined in M.3100

except for the addition of the list of failed service

providers in cases where the version status is active, failed or

partial failure and the subscriptionStatusChangeCauseCode if

it is set.

Failed lists will also be potentially sent for subscription versions

with statuses of active, failed, partial failure, and old.

The service provider supports this notification if the Service

Provider TN Range Notification Indicator is turned off on the NPAC SMS.

This notification is prioritised and transmitted according to its

SOA Notification Priority tunable in the NPAC SMS.

!;

-- 12.0 LNP Subscription Version Old SP Final Concurrence Timer Expiration

--      Notification

subscriptionVersionOldSPFinalConcurrenceWindowExpiration NOTIFICATION

BEHAVIOUR

subscriptionVersionOldSPFinalConcurrenceWindowExpirationBehavior;

WITH INFORMATION SYNTAX

LNP-ASN1.VersionOldSPFinalConcurrenceWindowExpiration

AND ATTRIBUTE IDS

tn subscriptionTN,

version-id subscriptionVersionId,

access-control accessControl,

subscription-timer-type subscriptionTimerType,

subscription-business-type subscriptionBusinessType;

REGISTERED AS {LNP-OIDS.lnp-notification 12};

subscriptionVersionOldSPFinalConcurrenceWindowExpirationBehavior BEHAVIOUR

DEFINED AS !

This notification will be sent by the NPAC SMS upon expiration of

the Final Concurrence Timer to the old service provider via the SOA

to NPAC SMS interface to inform them of the timer expiration. If

the old service provider supports timer type, it will be sent. If

the old service provider supports business type, it will be sent.

This notification is prioritised and transmitted according to its

SOA Notification Priority tunable in the NPAC SMS.

!;

-- 13.0 LNP Number Pool Block Status Attribute Value Change Notification

numberPoolBlockStatusAttributeValueChange NOTIFICATION

BEHAVIOUR  numberPoolBlockStatusAttributeValueChangeBehavior;

WITH INFORMATION SYNTAX  LNP-ASN1.NumberPoolBlockStatusAttributeValueChange

AND ATTRIBUTE IDS

value-change-info subscriptionVersionAttributeValueChangeInfo,

failed-service-provs numberPoolBlockFailed-SP-List,

access-control accessControl;

REGISTERED AS {LNP-OIDS.lnp-notification 13};

numberPoolBlockStatusAttributeValueChangeBehavior BEHAVIOUR

DEFINED  AS !

This notification is used to report changes to the

numberPoolBlockStatus field. It is identical

to an attribute value change notification as defined in M.3100

except for the addition of the list of failed service

providers.

The failed service provider list reflects the EDR service

providers who failed to receive the number pool block and any non-EDR

service provider who failed to receive the corresponding subscription

versions of LNP type equal to 'pool'.

Failed lists will be potentially sent for number pool blocks

with statuses of active, failed, partial failure and old. This

notification will be sent to the SOAs when the

numberPoolBlockSOA-Origination is true for the number pool block

object.

This notification is prioritised and transmitted according to its

SOA Notification Priority tunable in the NPAC SMS.

!;

-- 14.0 LNP Subscription Version Range Status Attribute Value Change

--      Notification

subscriptionVersionRangeStatusAttributeValueChange NOTIFICATION

BEHAVIOUR  subscriptionVersionRangeStatusAttributeValueChangeBehavior;

WITH INFORMATION SYNTAX  LNP-ASN1.VersionRangeStatusAttributeValueChange

AND ATTRIBUTE IDS

range-status-attribute-value-change-info

subscriptionVersionRangeStatusAttributeValueChangeInfo,

access-control accessControl;

REGISTERED AS {LNP-OIDS.lnp-notification 14};

subscriptionVersionRangeStatusAttributeValueChangeBehavior BEHAVIOUR

DEFINED  AS !

This notification type is used to report changes to the

subscriptionVersionStatus field for range operations. It uses the

status attribute value change notification which is based on the

attribute value change notification as defined in M.3100 with the

addition of the list of failed service providers in cases where

the version status is active, failed or partially failed and the

subscriptionStatusChangeCauseCode if it is set.

The service provider supports this notification if the Service

Provider TN Range Notification Indicator is set on the NPAC SMS and

the service provider will no longer receive the

subscriptionVersionStatuaAttributeValueChange notification.

This notification is prioritised and transmitted according to its

SOA Notification Priority tunable in the NPAC SMS.

!;

-- 15.0 LNP Subscription Version Range Attribute Value Change Notification

subscriptionVersionRangeAttributeValueChange NOTIFICATION

BEHAVIOUR  subscriptionVersionRangeAttributeValueChangeBehavior;

WITH INFORMATION SYNTAX  LNP-ASN1.VersionRangeAttributeValueChange

AND ATTRIBUTE IDS

range-attribute-value-change-info

subscriptionVersionRangeAttributeValueChangeInfo,

access-control accessControl;

REGISTERED AS {LNP-OIDS.lnp-notification 15};

subscriptionVersionRangeAttributeValueChangeBehavior BEHAVIOUR

DEFINED  AS !

This notification type is used to report changes to the attributes

in the subscription versions for range operations.  It uses the

attribute value change notification, which is based on the attribute

value change notification as defined in M.3100.

The service provider supports this notification if the Service

Provider TN Range Notification Indicator is set on the NPAC SMS and

the service provider will no longer receive an

attribute value change notification for a subscription version.

This notification is prioritised and transmitted according to its

SOA Notification Priority tunable in the NPAC SMS.

!;

-- 16.0 LNP Subscription Version Range Object Creation Notification

subscriptionVersionRangeObjectCreation NOTIFICATION

BEHAVIOUR  subscriptionVersionRangeObjectCreationBehavior;

WITH INFORMATION SYNTAX  LNP-ASN1.VersionRangeObjectCreation

AND ATTRIBUTE IDS

range-object-creation-info subscriptionVersionRangeObjectCreationInfo,

access-control accessControl;

REGISTERED AS {LNP-OIDS.lnp-notification 16};

subscriptionVersionRangeObjectCreationBehavior BEHAVIOUR

DEFINED  AS !

This notification type is used to report creation of subscription

versions for range operations.  It uses the object creation

notification as defined in M.3100.

The service provider supports this notification if the Service

Provider TN Range Notification Indicator is set on the NPAC SMS and

the service provider will no longer receive an

object creation notification for a subscription version.

This ObjectInfo field will contain the same data as the current

object creation notifications sent to the old and new service

provider. The TN and SVID fields that are sent in the single

object creation notification will contain the TN and subscription

version id for the first TN in the range or list.

This notification is prioritised and transmitted according to its

SOA Notification Priority tunable in the NPAC SMS.

!;

-- 17.0 LNP Subscription Version Range Donor Disconnect Notification

subscriptionVersionRangeDonorSP-CustomerDisconnectDate NOTIFICATION

BEHAVIOUR  subscriptionVersionRangeDonorSP-CustomerDisconnectDateBehavior;

WITH INFORMATION SYNTAX  LNP-ASN1.VersionRangeCustomerDisconnectDate

AND ATTRIBUTE IDS

range-donor-disconnect-info

subscriptionVersionRangeDonorSP-CustomerDisconnectDateInfo,

access-control accessControl;

REGISTERED AS {LNP-OIDS.lnp-notification 17};

subscriptionVersionRangeDonorSP-CustomerDisconnectDateBehavior BEHAVIOUR

DEFINED  AS !

This notification informs the donor service provider SOA that a

range of subscription versions is being disconnected.  The TN,

the version id, customer disconnect date and effective release

date (optional) values are sent.

The service provider supports this notification if the Service

Provider TN Range Notification Indicator is set on the NPAC SMS and

the service provider will no longer receive a

subscriptionVersionDonorSP-CustomerDisconnectDate notification.

This notification is prioritised and transmitted according to its

SOA Notification Priority tunable in the NPAC SMS.

!;

-- 18.0 LNP Subscription Version Range Cancellation Acknowledge Notification

subscriptionVersionRangeCancellationAcknowledgeRequest NOTIFICATION

BEHAVIOUR subscriptionVersionRangeCancellationAcknowledgeBehavior;

WITH INFORMATION SYNTAX  LNP-ASN1.VersionRangeCancellationAcknowledgeRequest

AND ATTRIBUTE IDS

range-cancellation-acknowledge-info

subscriptionVersionRangeCancellationAcknowledgeInfo,

access-control accessControl;

REGISTERED AS {LNP-OIDS.lnp-notification 18};

subscriptionVersionRangeCancellationAcknowledgeBehavior BEHAVIOUR

DEFINED  AS !

This notification requests that a service provider send a

cancellation acknowledgement for a range of subscription

versions.  The TNs and version ids are sent.

The service provider supports this notification if the Service

Provider TN Range Notification Indicator is set on the NPAC SMS and

the service provider will no longer receive a

subscriptionVersionCancellationAcknowledgeRequest notification.

This notification is prioritised and transmitted according to its

SOA Notification Priority tunable in the NPAC SMS.

!;

-- 19.0 LNP Subscription Version Range New SP Create Notification

subscriptionVersionRangeNewSP-CreateRequest NOTIFICATION

BEHAVIOUR  subscriptionVersionRangeNewSP-CreateRequestBehavior;

WITH INFORMATION SYNTAX  LNP-ASN1.VersionRangeNewSP-CreateRequest

AND ATTRIBUTE IDS

range-new-sp-create-info

subscriptionVersionRangeNewSP-CreateRequestInfo,

access-control accessControl;

REGISTERED AS {LNP-OIDS.lnp-notification 19};

subscriptionVersionRangeNewSP-CreateRequestBehavior BEHAVIOUR

DEFINED  AS !

This notification requests that a new service provider send a

create request for a range of subscription versions for which

concurrence for porting the number has not been received.  The

TNs, the version ids and the old service provider id, authorization

flag and authorization timestamp values are sent.  If the new

service provider supports timer type, it will be sent.  If the new

service provider supports business type, it will be sent.

The service provider supports this notification if the Service

Provider TN Range Notification Indicator is set on the NPAC SMS and

the service provider will no longer receive a

subscriptionVersionNewSP-CreatRequest notification.

This notification is prioritised and transmitted according to its

SOA Notification Priority tunable in the NPAC SMS.

!;

-- 20.0 LNP Subscription Version Range Old SP Concurrence Notification

subscriptionVersionRangeOldSP-ConcurrenceRequest NOTIFICATION

BEHAVIOUR subscriptionVersionRangeOldSP-ConcurrenceRequestBehavior;

WITH INFORMATION SYNTAX  LNP-ASN1.VersionRangeOldSP-ConcurrenceRequest

AND ATTRIBUTE IDS

range-old-sp-concurrence-info

subscriptionVersionRangeOldSP-ConcurrenceRequestInfo,

access-control accessControl;

REGISTERED AS {LNP-OIDS.lnp-notification 20};

subscriptionVersionRangeOldSP-ConcurrenceRequestBehavior BEHAVIOUR

DEFINED  AS !

This notification requests that an old service provider send a

create request for a range of subscription versions for which

concurrence for porting the number has not been received.  The

TNs, the version ids, and the new service provider id, authorization

flag and creation timestamp values are sent. If the old service

provider supports timer type, it will be sent. If the old service

provider supports business type, it will be sent.

The service provider supports this notification if the Service

Provider TN Range Notification Indicator is set on the NPAC SMS and

the service provider will no longer receive a

subscriptionVersionOldSP-ConcurrenceRequest notification.

This notification is prioritised and transmitted according to its

SOA Notification Priority tunable in the NPAC SMS.

!;

-- 21.0 LNP Subscription Version Range Old SP Final Concurrence Window

--      Expiration Notification

subscriptionVersionRangeOldSPFinalConcurrenceWindowExpiration NOTIFICATION

BEHAVIOUR subscriptionVersionRangeOldSPFinalConcurrenceWindowExpirationBehavior;

WITH INFORMATION SYNTAX  LNP-ASN1.VersionRangeOldSPFinalConcurrenceWindowExpiration

AND ATTRIBUTE IDS

range-old-sp-finalconcurrence-window-expiration-info

subscriptionVersionRangeOldSPFinalConcurrenceWindowExpirationInfo,

access-control accessControl;

REGISTERED AS {LNP-OIDS.lnp-notification 21};

subscriptionVersionRangeOldSPFinalConcurrenceWindowExpirationBehavior BEHAVIOUR

DEFINED  AS !

This notification will be sent by the NPAC SMS upon expiration of the

Final Concurrence Timer to the old service provider via the SOA to

NPAC SMS interface to inform them of the timer expiration for a

range of TNs. If the old service provider supports timer type, it

will be sent. If the old service provider supports business type,

it will be sent.

The service provider supports this notification if the Service

Provider TN Range Notification Indicator is set on the NPAC SMS and

the service provider will no longer receive a

subscriptionVersionOldSPFinalConcurrenceWindowExpiration notification.

This notification is prioritised and transmitted according to its

SOA Notification Priority tunable in the NPAC SMS.

!;

-- 22.0 LNP Subscription Version Range New SP Final Create Window

--      Expiration Notification

subscriptionVersionRangeNewSP-FinalCreateWindowExpiration NOTIFICATION

BEHAVIOUR subscriptionVersionRangeNewSP-FinalCreateWindowExpirationBehavior;

WITH INFORMATION SYNTAX  LNP-ASN1.VersionRangeNewSP-FinalCreateWindowExpiration

AND ATTRIBUTE IDS

range-new-sp-final-create-window-expiration-info

subscriptionVersionRangeNewSP-FinalCreateWindowExpirationInfo,

access-control accessControl;

REGISTERED AS {LNP-OIDS.lnp-notification 22};

subscriptionVersionRangeNewSP-FinalCreateWindowExpirationBehavior BEHAVIOUR

DEFINED  AS !

This notification indicates the final create window has expired

for concurrence of a subscription version.  This occurs when an

old service provider has sent a create request for a subscription

version and the new service provider has not concurred to the port

within the period of time of both the initial and final concurrence

timers.  Once the final create window has expired, this notification

is sent to both the old and new service providers that support this

notification.

On the NPAC SMS, the Final Create Window has expired, but the NPAC

SMS has not updated the status to cancel.

The TN, the version id and the new and old service provider id,

cause code, authorization flag and authorization timestamp values

are sent.  If the new service provider supports timer type, it will

be sent.  If the new service provider supports business type,

it will be sent.

The subscription version remains in a status of pending or conflict.

The service provider supports this notification if the Service

Provider TN Range Notification Indicator is set on the NPAC SMS and

the service provider will no longer receive a

subscriptionVersionNewSP-FinalCreateWindowExpiration notification.

This notification is prioritised and transmitted according to its

SOA Notification Priority tunable in the NPAC SMS.

!;

-- 23.0 LNP Subscription Version New SP Final Create Window Expiration

--      Notification

subscriptionVersionNewSP-FinalCreateWindowExpiration NOTIFICATION

BEHAVIOUR  subscriptionVersionNewSP-FinalCreateWindowExpirationPkgBehavior;

WITH INFORMATION SYNTAX LNP-ASN1.VersionNewSP-FinalCreateWindowExpiration

AND ATTRIBUTE IDS

tn subscriptionTN,

version-id subscriptionVersionId,

new-service-prov-id subscriptionNewCurrentSP,

old-service-prov-id subscriptionOldSP,

service-prov-due-date subscriptionOldSP-DueDate,

service-prov-old-authorization subscriptionOldSP-Authorization,

service-prov-authorization-creation-time-stamp

subscriptionOldSP-AuthorizationTimeStamp,

status-change-cause-code subscriptionStatusChangeCauseCode,

access-control accessControl,

subscription-timer-type subscriptionTimerType,

subscription-business-type subscriptionBusinessType;

REGISTERED AS {LNP-OIDS.lnp-notification 23};

subscriptionVersionNewSP-FinalCreateWindowExpirationPkgBehavior BEHAVIOUR

DEFINED AS !

This notification indicates the final create window has expired

for concurrence of a subscription version.  This occurs when an

old service provider has sent a create request for a subscription

version and the new service provider has not concurred to the port

within the period of time of both the initial and final concurrence

timers.  Once the final create window has expired, this notification

is sent to both the old and new service providers that support

this notification.

On the NPAC SMS, the Final Create Window has expired, but the NPAC

SMS has not updated the status to cancel.

The TN, the version id and the new and old service provider id,

cause code, authorization flag and authorization timestamp values

are sent.  If the new service provider supports timer type, it

will be sent.  If the new service provider supports business type,

it will be sent.

The subscription version remains in a status of pending or conflict.

The service provider supports this notification if the Service

Provider TN Range Notification Indicator is turned off on the NPAC SMS.

This notification is prioritised and transmitted according to its

SOA Notification Priority tunable in the NPAC SMS.

!;

# General ASN.1 Definitions

7

## Overview

The ASN.1 definitions (asn1_v3_1_0_082401) provided below support the GDMO definitions in Section 6.  Included below are the ASN.1 object identifier definitions and the syntax definitions for the interface attributes, notifications, and actions.

Note:  The exact lengths must be specified for ASN.1 data send across the interface.  This will prevent trailing null characters that may not be accepted by some CMIP vendor products.

## LNP ASN.1 Object Identifier Definitions

-- LNP ASN.1 Object Identifier Definitions

--#include   "smi.asn"

LNP-OIDS

{iso(1) org(3) dod(6) internet(1) private(4) enterprises(1)

lockheedMartin(103) cis(7) npac(0) iis(0) oids(0)}

DEFINITIONS ::=

BEGIN

-- EXPORTS all definitions

lnp-npac OBJECT IDENTIFIER ::=

{iso(1) org(3) dod(6) internet(1) private(4) enterprises(1)

lockheedMartin(103) cis(7) npac(0)}

lnp-npac-iis OBJECT IDENTIFIER ::=

{lnp-npac iis(0)}

-- If additional MIB specializations are needed (e.g. regional or vendor

-- specific implementations) object identifiers can be added at the npac level (e.g.

-- {lnp-npac special(1)}, etc.)

-- LNP NPAC SMS categories of Interoperable Interface Specification (IIS)

-- information objects

lnp-attribute OBJECT IDENTIFIER ::= {lnp-npac-iis attribute(2) }

lnp-objectClass OBJECT IDENTIFIER ::= {lnp-npac-iis objectClass(3) }

lnp-nameBinding OBJECT IDENTIFIER ::= {lnp-npac-iis nameBinding(4) }

lnp-notification OBJECT IDENTIFIER ::= {lnp-npac-iis notification(5) }

lnp-action OBJECT IDENTIFIER ::= {lnp-npac-iis action(6) }

lnp-package OBJECT IDENTIFIER ::= {lnp-npac-iis package(7) }

lnp-parameter OBJECT IDENTIFIER ::= {lnp-npac-iis parameter(8) }

END -- LNP-OIDS

## LNP General ASN.1 Definitions

LNP-ASN1

{iso(1) org(3) dod(6) internet(1) private(4) enterprises(1)

lockheed(103) cis(7) npac(0) iis(0) asn1(1)}

DEFINITIONS IMPLICIT TAGS ::= BEGIN

-- EXPORTS everything

IMPORTS

-- CMIP

ObjectClass, ObjectInstance

FROM CMIP-1 {joint-iso-ccitt ms(9) cmip(1) modules(0) protocol(3)}

-- DMI

AttributeValueChangeInfo, ObjectInfo

FROM Notification-ASN1Module {joint-iso-ccitt ms(9) smi(3) part2(2)

asn1Module(2) 2};

ActivateAction ::= SubscriptionVersionAction

ActionResultsStatus ::= ResultsStatus

ActivateReply ::= SubscriptionVersionActionReply

AddressInformation ::= SEQUENCE {

line1 GraphicString40,

line2  GraphicString40,

city   GraphicString20,

state  GraphicString(SIZE(2)),

zip  GraphicString(SIZE(9)),

province GraphicString(SIZE(2)),

country GraphicString20,

contactPhone  PhoneNumber,

contact  GraphicString40,

contactFax  PhoneNumber,

contactPager  PhoneNumber,

contactPagerPIN  DigitString, -- value should be no more than 10 digits

contactE-mail  GraphicString60

}

AssociationFunction ::= SEQUENCE {

soaUnits SoaUnits,

lsmsUnits LSMSUnits

}

AuditAttributes ::= CHOICE {

specific-audit [0] SEQUENCE {

lidb-data BOOLEAN,

class-data BOOLEAN,

cnam-data BOOLEAN,

isvm-data BOOLEAN,

lrn-data BOOLEAN,

wsmsc-data BOOLEAN OPTIONAL -- Optional for backward compatability

},

all-data [1] NULL

}

AuditDiscrepancyRpt ::= SEQUENCE {

tn PhoneNumber,

version-id SubscriptionVersionId,

lsms-service-prov-id ServiceProvId,

failure-reason AuditFailureData,

access-control LnpAccessControl

}

AuditDiscrepancyRptRecovery ::= SEQUENCE {

tn PhoneNumber,

version-id SubscriptionVersionId,

lsms-service-prov-id ServiceProvId,

failure-reason AuditFailureData

}

AuditFailureData ::= CHOICE {

tn-version-missing-NPAC [0] NULL,

tn-version-missing-LSMS [1] NULL,

mismatch-data [2] MismatchAttributes

}

AuditId ::= LnpKey

AuditName ::= GraphicString40

AuditNumberOfTNs ::= INTEGER

AuditNumberOfTNsComplete ::= INTEGER

AuditResults ::= SEQUENCE {

status [0] AuditResultStatus,

failed-service-prov-list [1] Failed-SP-List OPTIONAL,

number-of-discrepancies [2] INTEGER,

time-of-completion [3] GeneralizedTime,

access-control [4] LnpAccessControl

}

AuditResultsRecovery ::= SEQUENCE {

status [0] AuditResultStatus,

failed-service-prov-list [1] Failed-SP-List OPTIONAL,

number-of-discrepancies [2] INTEGER,

time-of-completion [3] GeneralizedTime

}

AuditResultStatus ::= ENUMERATED {

success (0),

failed-due-to-discrepancies (1),

failed-on-local-sms (2),

no-audit-performed (3)

}

AuditServiceProvIdRange ::= CHOICE {

allServiceProvs [0] NULL,

serviceProv [1] ServiceProvName -- ServiceProvId value allowed

}

AuditStatus ::= ENUMERATED {

in-progress (0),

suspended (1),

complete (2)

}

AuditTN-ActivationRange ::= TimeRange

BillingId ::= CHOICE {

value [0] GraphicString4,

no-value-needed [1] NULL

}

BlockDownloadCriteria ::= CHOICE {

time-range [0] TimeRange,

block-npa-nxx-x [1] NPA-NXX-X,

block-npa-nxx-x-range [2] NPA-NXX-X-Range

}

BlockDownloadData ::= SET OF SEQUENCE {

block-id [0] BlockId,

block-npa-nxx-x [1] NPA-NXX-X OPTIONAL,

block-holder-sp [2] ServiceProvId OPTIONAL,

block-activation-timestamp [3] GeneralizedTime OPTIONAL,

block-lrn [4] LRN OPTIONAL,

block-class-dpc [5] EXPLICIT DPC OPTIONAL,

block-class-ssn [6] EXPLICIT SSN OPTIONAL,

block-lidb-dpc [7] EXPLICIT DPC OPTIONAL,

block-lidb-ssn [8] EXPLICIT SSN OPTIONAL,

block-isvm-dpc [9] EXPLICIT DPC OPTIONAL,

block-isvm-ssn [10] EXPLICIT SSN OPTIONAL,

block-cnam-dpc [11] EXPLICIT DPC OPTIONAL,

block-cnam-ssn [12] EXPLICIT SSN OPTIONAL,

block-download-reason [13] DownloadReason,

block-wsmsc-dpc [14] EXPLICIT DPC OPTIONAL,

block-wsmsc-ssn [15] EXPLICIT SSN OPTIONAL

}

BlockId ::= LnpKey

BlockStatus ::= ENUMERATED {

active (1),

sending (3),

download-failed (4),

download-failed-partial (5),

old (7)

}

Boolean ::= BOOLEAN

CancellationAcknowledgeAction ::= SubscriptionVersionAction

CancellationAcknowledgeReply ::= SubscriptionVersionActionReply

CancelAction::= SubscriptionVersionAction

CancelReply ::= SubscriptionVersionActionReply

DPC ::= CHOICE {

dpc-value       [0] OCTET STRING (SIZE(3)),

no-value-needed [1] NULL

}

DigitString ::= GraphicString (FROM ("0" | "1" | "2" | "3" | "4" | "5" |

"6" | "7" | "8" | "9" | "*" | "#" ))

DisconnectAction::= SEQUENCE {

subscription-version-action [0] EXPLICIT SubscriptionVersionAction,

customer-disconnect-date [1] GeneralizedTime,

effective-release-date [2] GeneralizedTime OPTIONAL

}

DisconnectReply ::= SEQUENCE {

status SubscriptionVersionActionReply,

version-id SET OF SubscriptionVersionId OPTIONAL

}

DownloadAction ::= CHOICE {

subscriber-download [0] EXPLICIT SubscriptionDownloadCriteria,

network-download [1] NetworkDownloadCriteria,

block-download [2] BlockDownloadCriteria

}

DownloadReason ::= ENUMERATED {

new1 (0),

delete1(1),

modified (2),

audit-discrepancy (3)

}

DownloadReply ::= SEQUENCE {

status ENUMERATED {

success (0),

failed (1),

time-range-invalid (2),

criteria-to-large (3),

no-data-selected (4)

},

downloaddata CHOICE {

subscriber-data [0] SubscriptionDownloadData,

network-data [1] NetworkDownloadData,

block-data [2] BlockDownloadData

} OPTIONAL

}

EndUserLocationType ::= CHOICE {

value [0] NumberString(SIZE(2)),

no-value-needed [1] NULL

}

EndUserLocationValue ::= CHOICE {

value [0] NumberString(SIZE(1..12)),

no-value-needed [1] NULL

}

Failed-SP-List  ::= SET OF SEQUENCE {

service-prov-id ServiceProvId,

service-prov-name ServiceProvName

}

CMIPErrorCode ::= ENUMERATED {

noSuchObjectClassEr (0),

noSuchObjectInstanceEr (1),

accessDeniedEr (2),

syncNotSupportedEr (3),

invalidFilterEr (4),

noSuchAttributeEr (5),

invalidAttributeValueEr (6),

getListErrorEr (7),

setListErorrEr (8),

noSuchActionEr (9),

processingFailureEr (10),

duplicateManagedObjectInstanceEr (11),

noSuchReferenceObjectEr (12),

noSuchEventTypeEr (13),

noSuchArgumentEr (14),

invalidArgumentValueEr (15),

invalidScopeEr (16),

invalidObjectInstanceEr (17),

missingattributeValueEr (18),

classInstanceConflictEr (19),

complexityLimitationEr (20),

mistypedOperationEr (21),

noSuchInvokeIdEr (22),

operationCancelledEr (23)

}

FailedTN-List  ::= SET OF SEQUENCE {

subscriptionVersionId SubscriptionVersionId,

tn PhoneNumber,

errorId CMIPErrorCode

}

GeneralTime ::= GeneralizedTime

GraphicStringBase ::= GraphicString

GraphicString4 ::= GraphicStringBase(SIZE(1..4))

GraphicString16 ::= GraphicStringBase(SIZE(1..16))

GraphicString20 ::= GraphicStringBase(SIZE(1..20))

GraphicString25 ::= GraphicStringBase(SIZE(1..25))

GraphicString28 ::= GraphicStringBase(SIZE(1..28))

GraphicString40 ::= GraphicStringBase(SIZE(1..40))

GraphicString60 ::= GraphicStringBase(SIZE(1..60))

GraphicString255 ::= GraphicStringBase(SIZE(1..255))

ID-Range ::= SEQUENCE {

start-id LnpKey,

end-id LnpKey

}

Integer ::= INTEGER

LnpAccessControl ::= [0] SEQUENCE {

systemId [0] EXPLICIT SystemID,

systemType [1] SystemType,

userId [2] GraphicString60 OPTIONAL,

listId [3] INTEGER,

keyId [4] INTEGER,

cmipDepartureTime [5] GeneralizedTime,

sequenceNumber [6] INTEGER(0..4294967295),

function [7] AssociationFunction,

recoveryMode [8] BOOLEAN,

signature [9] BIT STRING

}

LnpAuditsName ::= GraphicString ("lnpAudits")

LnpKey ::= INTEGER

LnpNetworkName ::= GraphicString ("lnpNetwork")

LnpSMS-Name ::= GraphicString40

LnpServiceProvsName ::= GraphicString ("lnpServiceProvs")

LnpSubscriptionsName ::= GraphicString ("lnpSubscriptions")

LnpSpecificInfo ::= GraphicString255

LNPType ::= ENUMERATED {

lspp (0),

lisp (1),

pool (2)

}

LocalSMS-ActionResults ::= SEQUENCE {

actionId [0] INTEGER,

status [1] ActionResultsStatus,

failed-tn-list [2] FailedTN-List OPTIONAL,

time-of-completion [3] GeneralizedTime,

accessControl [4] LnpAccessControl

}

LocalSMS-CreateAction ::= SEQUENCE {

actionId INTEGER,

subscriptionVersionObjects SET OF SubscriptionVersionObject,

tn-range TN-Range OPTIONAL -- used only on pooled ports for release 1.4

}

LocalSMS-CreateReply ::= ResultsStatus

LRN ::= CHOICE {

value [0] OCTET STRING (SIZE(5)),

no-value-needed [1] NULL

}

LRN-ID ::= LnpKey

LRN-DownloadData ::= SET OF SEQUENCE {

service-prov-lrn-id LRN-ID,

service-prov-lrn-value LRN OPTIONAL,

service-prov-download-reason DownloadReason,

service-prov-lrn-creation-timestamp GeneralizedTime OPTIONAL

}

LRN-Range ::= SEQUENCE {

start-lrn LRN,

stop-lrn LRN

}

LSMSUnits ::= SEQUENCE {

dataDownload [0] NULL OPTIONAL,

networkDataMgmt [1] NULL OPTIONAL,

query [2] NULL OPTIONAL

}

MismatchAttributes ::= SEQUENCE {

seq0 [0] SEQUENCE {

lsms-subscriptionLRN LRN,

npac-subscriptionLRN LRN

} OPTIONAL,

seq1 [1] SEQUENCE {

lsms-subscriptionNewCurrentSP ServiceProvId,

npac-subscriptionNewCurrentSP ServiceProvId

} OPTIONAL,

seq2 [2] SEQUENCE {

lsms-subscriptionActivationTimeStamp GeneralizedTime,

npac-subscriptionActivationTimeStamp GeneralizedTime

} OPTIONAL,

seq3 [3] SEQUENCE {

lsms-subscriptionCLASS-DPC DPC,

npac-subscriptionCLASS-DPC DPC

} OPTIONAL,

seq4 [4] SEQUENCE {

lsms-subscriptionCLASS-SSN SSN,

npac-subscriptionCLASS-SSN SSN

} OPTIONAL,

seq5 [5] SEQUENCE {

lsms-subscriptionLIDB-DPC DPC,

npac-subscriptionLIDB-DPC DPC

} OPTIONAL,

seq6 [6] SEQUENCE {

lsms-subscriptionLIDB-SSN SSN,

npac-subscriptionLIDB-SSN SSN

} OPTIONAL,

seq7 [7] SEQUENCE {

lsms-subscriptionISVM-DPC DPC,

npac-subscriptionISVM-DPC DPC

} OPTIONAL,

seq8 [8] SEQUENCE {

lsms-subscriptionISVM-SSN SSN,

npac-subscriptionISVM-SSN SSN

} OPTIONAL,

seq9 [9] SEQUENCE {

lsms-subscriptionCNAM-DPC DPC,

npac-subscriptionCNAM-DPC DPC

} OPTIONAL,

seq10 [10] SEQUENCE {

lsms-subscriptionCNAM-SSN SSN,

npac-subscriptionCNAM-SSN SSN

} OPTIONAL,

seq11 [11] SEQUENCE {

lsms-subscriptionEndUserLocationValue EndUserLocationValue,

npac-subscriptionEndUserLocationValue EndUserLocationValue

} OPTIONAL,

seq12 [12] SEQUENCE {

lsms-subscriptionEndUserLocationType EndUserLocationType,

npac-subscriptionEndUserLocationType EndUserLocationType

} OPTIONAL,

seq13 [13] SEQUENCE {

lsms-subscriptionBillingId BillingId,

npac-subscriptionBillingId BillingId

} OPTIONAL,

seq14 [14] SEQUENCE {

lsms-subscriptionLNPType LNPType,

npac-subscriptionLNPType LNPType

} OPTIONAL,

seq15 [15] SEQUENCE {

lsms-subscriptionWSMSC-DPC DPC,

npac-subscriptionWSMSC-DPC DPC

} OPTIONAL,

seq16 [16] SEQUENCE {

lsms-subscriptionWSMSC-SSN SSN,

npac-subscriptionWSMSC-SSN SSN

} OPTIONAL

}

ModifyAction::= SEQUENCE {

subscription-version-action [0] EXPLICIT SubscriptionVersionAction,

version-status [1] VersionStatus OPTIONAL,

data-to-modify [2] SubscriptionModifyData

}

ModifyReply ::= SEQUENCE {

status SubscriptionVersionActionReply,

invalid-data SubscriptionModifyInvalidData OPTIONAL

}

NetworkAddressInformation ::= SET OF SEQUENCE {

interfaceAddress OSI-Address,

systemType SystemType

}

NetworkDownloadCriteria ::= SEQUENCE {

time-range [0] TimeRange OPTIONAL,

chc1 [1] EXPLICIT CHOICE {

service-prov [0] ServiceProvId,

all-service-provs [1] NULL

},

chc2 [2] EXPLICIT CHOICE {-- A decision was made by

-- NANC to leave this structure a CHOICE of

-- CHOICEs instead of using one CHOICE to

-- simplify tagging

npa-nxx-data [0] EXPLICIT CHOICE {

npa-nxx-range [0] NPA-NXX-Range,

all-npa-nxx [1] NULL

},

lrn-data [1] EXPLICIT CHOICE {

lrn-range [0] LRN-Range,

all-lrn [1] NULL

},

all-network-data [2] NULL,

npa-nxx-x-data [3] EXPLICIT CHOICE {

npa-nxx-x-range [0] NPA-NXX-X-Range,

all-npa-nxx-x [1] NULL

}

}

}

NetworkDownloadData ::= SET OF SEQUENCE {

service-prov-data [0] SEQUENCE {

service-prov-id ServiceProvId,

service-prov-name ServiceProvName OPTIONAL

},

service-prov-npa-nxx-data [1] NPA-NXX-DownloadData OPTIONAL,

service-prov-lrn-data [2] LRN-DownloadData OPTIONAL,

service-prov-npa-nxx-x-data [3] NPA-NXX-X-DownloadData OPTIONAL

}

NetworkNotificationRecoveryAction ::= TimeRange

NetworkNotificationRecoveryReply ::= SEQUENCE {

status ENUMERATED {

success (0),

failed (1),

time-range-invalid (2),

criteria-to-large (3),

no-data-selected (4)

},

system-choice CHOICE {

lsms [1] SET OF SEQUENCE {

managedObjectClass ObjectClass,

managedObjectInstance ObjectInstance,

notification CHOICE {

subscription-version-new-npa-nxx [1] VersionNewNPA-NXX-Recovery,

lnp-npac-sms-operational-information [2]

NPAC-SMS-Operational-InformationRecovery

}

},

soa [2] SET OF SEQUENCE {

managedObjectClass ObjectClass,

managedObjectInstance ObjectInstance,

notification CHOICE {

subscription-version-new-npa-nxx [1] VersionNewNPA-NXX-Recovery,

subscription-version-donor-sp-customer-disconnect-date [2]

VersionCustomerDisconnectDateRecovery,

subscription-version-audit-discrepancy-report [3]

AuditDiscrepancyRptRecovery,

subscription-audit-results [4] AuditResultsRecovery,

lnp-npac-sms-operational-information [5]

NPAC-SMS-Operational-InformationRecovery,

subscription-version-new-sp-create-request [6]

VersionNewSP-CreateRequestRecovery,

subscription-version-old-sp-concurrence-request [7]

VersionOldSP-ConcurrenceRequestRecovery,

subscription-version-old-sp-final-window-expiration [8]

VersionOldSPFinalConcurrenceWindowExpirationRecovery,

subscription-version-cancellation-acknowledge-request [9]

VersionCancellationAcknowledgeRequestRecovery,

subscriptionVersionStatusAttributeValueChange [10]

VersionStatusAttributeValueChangeRecovery,

attribute-value-change [11] AttributeValueChangeInfo,

object-creation [12] ObjectInfo,

object-deletion [13] ObjectInfo,

numberPoolBlockStatusAttributeValueChange [14]

NumberPoolBlockStatusAttributeValueChangeRecovery,

versionRangeAttributeValueChangeRecovery [15]

VersionRangeAttributeValueChangeRecovery,

versionRangeObjectCreationRecovery [16]

VersionRangeObjectCreationRecovery,

versionRangeStatusAttributeValueChangeRecovery [17]

VersionRangeStatusAttributeValueChangeRecovery,

versionRangeCustomerDisconnectDateRecovery [18]

VersionRangeCustomerDisconnectDateRecovery,

versionRangeCancellationAcknowledgeRecovery [19]

VersionRangeCancellationAcknowledgeRecovery,

versionRangeNewSP-CreateRequestRecovery [20]

VersionRangeNewSP-CreateRequestRecovery,

versionRangeOldSP-ConcurrenceRequestRecovery [21]

VersionRangeOldSP-ConcurrenceRequestRecovery,

versionRangeOldSPFinalConcurrenceWindowExpirationRecovery [22]

VersionRangeOldSPFinalConcurrenceWindowExpirationRecovery,

versionRangeNewSP-FinalCreateWindowExpirationRecovery [23]

VersionRangeNewSP-FinalCreateWindowExpirationRecovery,

subscription-version-new-sp-final-window-expiration [24]

VersionNewSP-FinalCreateWindowExpirationRecovery

}

}

} OPTIONAL

}

NewSP-CreateAction ::= NewSP-CreateData

NewSP-CreateData ::= SEQUENCE {

chc1 [0] EXPLICIT CHOICE {

subscription-version-tn [0] PhoneNumber,

subscription-version-tn-range [1] TN-Range

},

subscription-lrn [1] LRN OPTIONAL,

subscription-new-current-sp [2] ServiceProvId,

subscription-old-sp [3] ServiceProvId,

subscription-new-sp-due-date [4] GeneralizedTime,

subscription-class-dpc [6] EXPLICIT DPC OPTIONAL,

subscription-class-ssn [7] EXPLICIT SSN OPTIONAL,

subscription-lidb-dpc [8] EXPLICIT DPC OPTIONAL,

subscription-lidb-ssn [9] EXPLICIT SSN OPTIONAL,

subscription-isvm-dpc [10] EXPLICIT DPC OPTIONAL,

subscription-isvm-ssn [11] EXPLICIT SSN OPTIONAL,

subscription-cnam-dpc [12] EXPLICIT DPC OPTIONAL,

subscription-cnam-ssn [13] EXPLICIT SSN OPTIONAL,

subscription-end-user-location-value [14]

EndUserLocationValue OPTIONAL,

subscription-end-user-location-type [15] EndUserLocationType OPTIONAL,

subscription-billing-id [16] BillingId OPTIONAL,

subscription-lnp-type [17] LNPType,

subscription-porting-to-original-sp-switch [18]

SubscriptionPortingToOriginal-SPSwitch,

subscription-wsmsc-dpc [19] EXPLICIT DPC OPTIONAL,

subscription-wsmsc-ssn [20] EXPLICIT SSN OPTIONAL

}

NewSP-CreateReply ::= SEQUENCE {

status [0] SubscriptionVersionActionReply,

invalid-data [1] NewSP-CreateInvalidData OPTIONAL

}

NewSP-CreateInvalidData ::= CHOICE {

subscription-version-tn [0] EXPLICIT PhoneNumber,

subscription-version-tn-range [1] EXPLICIT TN-Range,

subscription-lrn [2] EXPLICIT LRN,

subscription-new-current-sp [3] EXPLICIT ServiceProvId,

subscription-old-sp [4] EXPLICIT ServiceProvId,

subscription-new-sp-due-date [5] EXPLICIT GeneralizedTime,

subscription-class-dpc [6] EXPLICIT DPC,

subscription-class-ssn [7] EXPLICIT SSN,

subscription-lidb-dpc [8] EXPLICIT DPC,

subscription-lidb-ssn [9] EXPLICIT SSN,

subscription-isvm-dpc [10] EXPLICIT DPC,

subscription-isvm-ssn [11] EXPLICIT SSN,

subscription-cnam-dpc [12] EXPLICIT DPC,

subscription-cnam-ssn [13] EXPLICIT SSN,

subscription-end-user-location-value [14] EXPLICIT EndUserLocationValue,

subscription-end-user-location-type [15] EXPLICIT EndUserLocationType,

subscription-billing-id [16] EXPLICIT BillingId,

subscription-lnp-type [17] EXPLICIT LNPType,

subscription-porting-to-original-sp-switch [18]

EXPLICIT SubscriptionPortingToOriginal-SPSwitch,

subscription-wsmsc-dpc [19] EXPLICIT DPC,

subscription-wsmsc-ssn [20] EXPLICIT SSN

}

NpacAssociationUserInfo ::= SEQUENCE {

error-code [0] IMPLICIT ErrorCode,

error-text [1] IMPLICIT GraphicString(SIZE(1..80))

}

ErrorCode ::= ENUMERATED {

success (0),

access-denied (1),

retry-same-host (2),

try-other-host (3)

}

NPA ::= NumberString(SIZE(3))

NPA-NXX ::= SEQUENCE {

npa-value NPA,

nxx-value NumberString(SIZE(3))

}

NPA-NXX-DownloadData ::= SET OF SEQUENCE {

service-prov-npa-nxx-id NPA-NXX-ID,

service-prov-npa-nxx-value NPA-NXX OPTIONAL,

service-prov-npa-nxx-effective-timestamp GeneralizedTime OPTIONAL,

service-prov-download-reason DownloadReason,

service-prov-npa-nxx-creation-timestamp GeneralizedTime OPTIONAL

}

NPA-NXX-ID ::= LnpKey

NPA-NXX-Range ::= SEQUENCE {

start-npa-nxx NPA-NXX,

stop-npa-nxx NPA-NXX

}

NPA-NXX-X ::= NumberString(SIZE(7))

NPA-NXX-X-DownloadData ::= SET OF SEQUENCE {

service-prov-npa-nxx-x-id [0] NPA-NXX-X-ID,

service-prov-npa-nxx-x [1] NPA-NXX-X OPTIONAL,

service-prov-npa-nxx-x-effective-timestamp [2] GeneralizedTime OPTIONAL,

service-prov-npa-nxx-x-creation-timestamp [3] GeneralizedTime OPTIONAL,

service-prov-npa-nxx-x-modified-timestamp [4] GeneralizedTime OPTIONAL,

service-prov-npa-nxx-x-download-reason [5] DownloadReason

}

NPA-NXX-X-ID ::= LnpKey

NPA-NXX-X-Range ::= SEQUENCE {

start-npa-nxx-x NPA-NXX-X,

stop-npa-nxx-x NPA-NXX-X

}

NPAC-SMS-Operational-Information ::= SEQUENCE {

down-time TimeRange,

npac-contact-number PhoneNumber,

additional-down-time-information GraphicString255,

access-control LnpAccessControl

}

NPAC-SMS-Operational-InformationRecovery ::= SEQUENCE {

down-time TimeRange,

npac-contact-number PhoneNumber,

additional-down-time-information GraphicString255

}

NumberPoolBlock-CreateAction ::= SEQUENCE {

block-npa-nxx-x NPA-NXX-X,

block-holder-sp ServiceProvId,

block-lrn LRN,

block-class-dpc DPC,

block-class-ssn SSN,

block-lidb-dpc DPC,

block-lidb-ssn SSN,

block-isvm-dpc DPC,

block-isvm-ssn SSN,

block-cnam-dpc DPC,

block-cnam-ssn SSN,

block-wsmsc-dpc [0] DPC OPTIONAL,

block-wsmsc-ssn [1] SSN OPTIONAL

}

NumberPoolBlock-CreateInvalidData ::= CHOICE {

block-npa-nxx-x [0] EXPLICIT NPA-NXX-X,

block-lrn [1] EXPLICIT LRN,

block-class-dpc [2] EXPLICIT DPC,

block-class-ssn [3] EXPLICIT SSN,

block-lidb-dpc [4] EXPLICIT DPC,

block-lidb-ssn [5] EXPLICIT SSN,

block-isvm-dpc [6] EXPLICIT DPC,

block-isvm-ssn [7] EXPLICIT SSN,

block-cnam-dpc [8] EXPLICIT DPC,

block-cnam-ssn [9] EXPLICIT SSN,

block-wsmsc-dpc [10] EXPLICIT DPC,

block-wsmsc-ssn [11] EXPLICIT SSN

}

NumberPoolBlock-CreateReply ::= SEQUENCE {

block-id [0] BlockId,

status [1] ENUMERATED {

success (0),

failed (1),

soa-not-authorized (2),

no-npa-nxx-x-found (3),

invalid-data-values (4),

number-pool-block-already-exists (5),

prior-to-effective-date (6),

invalid-subscription-versions (7)

},

block-invalid-values [2] NumberPoolBlock-CreateInvalidData OPTIONAL

}

NumberPoolBlockStatusAttributeValueChange ::= SEQUENCE {

value-change-info [0] AttributeValueChangeInfo,

failed-service-provs [1] Failed-SP-List OPTIONAL,

access-control [2] LnpAccessControl

}

NumberPoolBlockStatusAttributeValueChangeRecovery ::= SEQUENCE {

value-change-info [0] AttributeValueChangeInfo,

failed-service-provs [1] Failed-SP-List OPTIONAL

}

NumberString ::= GraphicString (FROM ("0" | "1" | "2" | "3" | "4" | "5" |

"6" | "7" | "8" | "9"))

OldSP-CreateAction ::= OldSP-CreateData

OldSP-CreateData ::= SEQUENCE {

chc1 [0] EXPLICIT CHOICE {

subscription-version-tn [0] PhoneNumber,

subscription-version-tn-range [1] TN-Range

},

subscription-new-current-sp [1] ServiceProvId,

subscription-old-sp [2] ServiceProvId,

subscription-old-sp-due-date [3] GeneralizedTime,

subscription-old-sp-authorization [4] ServiceProvAuthorization,

subscription-status-change-cause-code [5] SubscriptionStatusChangeCauseCode,

subscription-lnp-type [6] LNPType

}

OldSP-CreateReply ::= SEQUENCE {

status SubscriptionVersionActionReply,

invalid-data OldSP-CreateInvalidData OPTIONAL

}

OldSP-CreateInvalidData ::= CHOICE {

subscription-version-tn [0] EXPLICIT PhoneNumber,

subscription-version-tn-range [1] EXPLICIT TN-Range,

subscription-new-current-sp [2] EXPLICIT ServiceProvId,

subscription-old-sp [3] EXPLICIT ServiceProvId,

subscription-old-sp-due-date [4] EXPLICIT GeneralizedTime,

subscription-old-sp-authorization [5] EXPLICIT ServiceProvAuthorization,

subscription-status-change-cause-code [6]

EXPLICIT SubscriptionStatusChangeCauseCode,

subscription-lnp-type [7] EXPLICIT LNPType

}

OSI-Address ::= SEQUENCE {

nsap            OCTET STRING(SIZE(12)),

tsap            OCTET STRING(SIZE(1..4)),

ssap            OCTET STRING(SIZE(1..4)),

psap            OCTET STRING(SIZE(1..4))

}

PhoneNumber ::= NumberString(SIZE(10))

RangeNotifyTN-ID-Info ::= CHOICE {

list-data [0] SET OF TN-VersionId,

range-data [1] SEQUENCE {

tn-range TN-Range,

id-range ID-Range

}

}

RangeNotifyID-Info ::= CHOICE {

list-data [0] SET OF LnpKey,

range-data [1] ID-Range

}

RangeStatusAttributeValueChangeInfo ::= SEQUENCE {

version-id [0] RangeNotifyID-Info,

value-change-info [1] AttributeValueChangeInfo,

failed-service-provs [2] Failed-SP-List OPTIONAL,

subscription-status-change-cause-code [3] SubscriptionStatusChangeCauseCode OPTIONAL

}

RangeAttributeValueChangeInfo ::= SEQUENCE {

version-id RangeNotifyID-Info,

value-change-info AttributeValueChangeInfo

}

RangeObjectCreationInfo ::= SEQUENCE {

tn-version-id RangeNotifyTN-ID-Info,

object-info SET OF ObjectInfo

}

RangeCustomerDisconnectDateInfo ::= SEQUENCE {

tn-version-id [0] RangeNotifyTN-ID-Info,

customer-disconnect-date [1] GeneralizedTime,

effective-release-date [2] GeneralizedTime OPTIONAL

}

RangeCancellationAcknowledgeInfo ::= RangeNotifyTN-ID-Info

RangeNewSP-CreateRequestInfo ::= SEQUENCE {

tn-version-id RangeNotifyTN-ID-Info,

service-prov-id ServiceProvId,

service-prov-due-date GeneralizedTime,

service-prov-authorization-creation-time-stamp GeneralizedTime,

service-prov-old-authorization ServiceProvAuthorization,

subscription-status-change-cause-code SubscriptionStatusChangeCauseCode,

subscription-timer-type [0] Integer OPTIONAL,

subscription-business-type [1] Integer OPTIONAL

}

RangeOldSP-ConcurrenceRequestInfo ::= SEQUENCE {

tn-version-id RangeNotifyTN-ID-Info,

service-prov-id ServiceProvId,

service-prov-due-date GeneralizedTime,

service-prov-authorization-creation-time-stamp GeneralizedTime,

subscription-timer-type [0] Integer OPTIONAL,

subscription-business-type [1] Integer OPTIONAL

}

RangeOldSPFinalConcurrenceWindowExpirationInfo ::= SEQUENCE {

tn-version-id RangeNotifyTN-ID-Info,

subscription-timer-type [0]Integer OPTIONAL,

subscription-business-type [1] Integer OPTIONAL

}

RangeNewSP-FinalCreateWindowExpirationInfo ::= SEQUENCE {

tn-version-id [0] RangeNotifyTN-ID-Info,

service-prov-id [1] ServiceProvId,

service-prov-due-date [2] GeneralizedTime,

service-prov-authorization-creation-time-stamp [3] GeneralizedTime,

subscription-timer-type [4] Integer OPTIONAL,

subscription-business-type [5] Integer OPTIONAL,

new-service-prov-id [6] ServiceProvId,

service-prov-old-authorization [7] ServiceProvAuthorization,

subscription-status-change-cause-code [8] SubscriptionStatusChangeCauseCode OPTIONAL

}

RecoveryCompleteAction ::= NULL

RecoveryCompleteReply ::= SEQUENCE {

status ResultsStatus,

subscriber-data [1] SubscriptionDownloadData OPTIONAL,

network-data [2] NetworkDownloadData OPTIONAL,

block-data [3] BlockDownloadData OPTIONAL

}

RemoveFromConflictAction ::= SubscriptionVersionAction

RemoveFromConflictReply ::= SubscriptionVersionActionReply

ServiceProvAuthorization ::= BOOLEAN

ServiceProvId ::= GraphicString4

ServiceProvName ::= GraphicString40

SoaUnits ::= SEQUENCE {

soaMgmt [0] NULL OPTIONAL,

networkDataMgmt [1] NULL OPTIONAL,

dataDownload [2] NULL OPTIONAL

}

ResultsStatus ::= ENUMERATED {

success(0),

failure(1)

}

SSN ::= CHOICE {

ssn-value       [0] INTEGER(0..255),

no-value-needed [1] NULL

}

SubscriptionData ::= SEQUENCE {

subscription-lrn [1] LRN OPTIONAL,

subscription-new-current-sp [2] ServiceProvId OPTIONAL,

subscription-activation-timestamp [3] GeneralizedTime OPTIONAL,

subscription-class-dpc [4] EXPLICIT DPC,

subscription-class-ssn [5] EXPLICIT SSN,

subscription-lidb-dpc [6] EXPLICIT DPC,

subscription-lidb-ssn [7] EXPLICIT SSN,

subscription-isvm-dpc [8] EXPLICIT DPC,

subscription-isvm-ssn [9] EXPLICIT SSN,

subscription-cnam-dpc [10] EXPLICIT DPC,

subscription-cnam-ssn [11] EXPLICIT SSN,

subscription-end-user-location-value [12]

EndUserLocationValue OPTIONAL,

subscription-end-user-location-type [13] EndUserLocationType OPTIONAL,

subscription-billing-id [14] BillingId OPTIONAL,

subscription-lnp-type [15] LNPType,

subscription-download-reason [16] DownloadReason,

subscription-wsmsc-dpc [17] EXPLICIT DPC OPTIONAL,

subscription-wsmsc-ssn [18] EXPLICIT SSN OPTIONAL

}

SubscriptionDownloadCriteria ::= CHOICE {

time-range [0] TimeRange,

tn  [1] PhoneNumber,

tn-range [2] TN-Range

}

SubscriptionDownloadData ::= SET OF SEQUENCE {

subscription-version-id [0] SubscriptionVersionId,

subscription-version-tn [1] PhoneNumber OPTIONAL,

subscription-data SubscriptionData

}

SubscriptionModifyData ::= SEQUENCE {

subscription-lrn [0] LRN OPTIONAL,

subscription-new-sp-due-date [1] GeneralizedTime OPTIONAL,

subscription-old-sp-due-date [2] GeneralizedTime OPTIONAL,

subscription-old-sp-authorization [3] ServiceProvAuthorization OPTIONAL,

subscription-class-dpc [4] EXPLICIT DPC OPTIONAL,

subscription-class-ssn [5] EXPLICIT SSN OPTIONAL,

subscription-lidb-dpc [6] EXPLICIT DPC OPTIONAL,

subscription-lidb-ssn [7] EXPLICIT SSN OPTIONAL,

subscription-isvm-dpc [8] EXPLICIT DPC OPTIONAL,

subscription-isvm-ssn [9] EXPLICIT SSN OPTIONAL,

subscription-cnam-dpc [10] EXPLICIT DPC OPTIONAL,

subscription-cnam-ssn [11] EXPLICIT SSN OPTIONAL,

subscription-end-user-location-value [12] EndUserLocationValue OPTIONAL,

subscription-end-user-location-type [13] EndUserLocationType OPTIONAL,

subscription-billing-id [14] BillingId OPTIONAL,

subscription-status-change-cause-code [15]

SubscriptionStatusChangeCauseCode OPTIONAL,

subscription-wsmsc-dpc [16] EXPLICIT DPC OPTIONAL,

subscription-wsmsc-ssn [17] EXPLICIT SSN OPTIONAL,

subscription-customer-disconnect-date [18] GeneralizedTime OPTIONAL,

subscription-effective-release-date [19] GeneralizedTime OPTIONAL

}

SubscriptionModifyInvalidData ::= CHOICE {

subscription-lrn [0] EXPLICIT LRN,

subscription-new-sp-due-date [1] EXPLICIT GeneralizedTime,

subscription-old-sp-due-date [2] EXPLICIT GeneralizedTime,

subscription-old-sp-authorization [3] EXPLICIT ServiceProvAuthorization,

subscription-class-dpc [4] EXPLICIT DPC,

subscription-class-ssn [5] EXPLICIT SSN,

subscription-lidb-dpc [6] EXPLICIT DPC,

subscription-lidb-ssn [7] EXPLICIT SSN,

subscription-isvm-dpc [8] EXPLICIT DPC,

subscription-isvm-ssn [9] EXPLICIT SSN,

subscription-cnam-dpc [10] EXPLICIT DPC,

subscription-cnam-ssn [11] EXPLICIT SSN,

subscription-end-user-location-value [12] EXPLICIT EndUserLocationValue,

subscription-end-user-location-type [13] EXPLICIT EndUserLocationType,

subscription-billing-id [14] EXPLICIT BillingId,

subscription-status-change-cause-code [15]

EXPLICIT SubscriptionStatusChangeCauseCode,

subscription-wsmsc-dpc [16] EXPLICIT DPC,

subscription-wsmsc-ssn [17] EXPLICIT SSN,

subscription-customer-disconnect-date [18] EXPLICIT GeneralizedTime,

subscription-effective-release-date [19] EXPLICIT GeneralizedTime

}

SubscriptionPortingToOriginal-SPSwitch ::= BOOLEAN

SubscriptionPreCancellationStatus ::= ENUMERATED {

conflict (0),

pending (2),

disconnect-pending (6)

}

SubscriptionStatusChangeCauseCode ::= CHOICE {

value           [0] INTEGER,

no-value-needed [1] NULL

}

SubscriptionVersionAction ::= CHOICE {

subscription-version-action-key [0] EXPLICIT SubscriptionVersionActionKey,

subscription-version-tn-range [1] TN-Range

}

SubscriptionVersionActionKey ::= CHOICE {

version-id [0] SubscriptionVersionId,

tn [1] PhoneNumber

}

SubscriptionVersionActionReply ::= ENUMERATED {

success (0),

failed (1),

soa-not-authorized (2),

no-version-found (3),

invalid-data-values (4),

version-create-already-exists (5)

}

SubscriptionVersionId ::= LnpKey

SubscriptionVersionObject ::= SEQUENCE {

tn-version-id SET OF TN-VersionId,

subscription-data SubscriptionData

}

TimeRange ::= SEQUENCE {

startTime [0] GeneralizedTime,

stopTime [1] GeneralizedTime

}

SystemID ::= CHOICE {

serviceProvId [0] ServiceProvId,

npac-sms [1] GraphicString60

}

SystemType ::= ENUMERATED {

soa(0),

local-sms(1),

soa-and-local-sms(2), -- it is assumed this value will not be sent by any

-- local system

npac-sms(3)  -- value is only valid for AccessControl definition

}

TN-Range ::= SEQUENCE {

tn-start  NumberString(SIZE(10)),

tn-stop   NumberString(SIZE(4))

}

TN-VersionId ::= SEQUENCE {

tn PhoneNumber,

version-id SubscriptionVersionId

}

VersionCancellationAcknowledgeRequest ::= SEQUENCE {

tn PhoneNumber,

version-id LnpKey,

access-control LnpAccessControl

}

VersionCancellationAcknowledgeRequestRecovery ::= SEQUENCE {

tn PhoneNumber,

version-id LnpKey

}

VersionCreateConcurrenceRequest ::= SEQUENCE {

tn PhoneNumber,

version-id LnpKey,

service-prov-id ServiceProvId,

service-prov-due-date GeneralizedTime,

service-prov-authorization-creation-time-stamp GeneralizedTime,

access-control LnpAccessControl,

subscription-timer-type [0] Integer OPTIONAL,

subscription-business-type [1] Integer OPTIONAL

}

VersionCreateConcurrenceRequestRecovery ::= SEQUENCE {

tn [0] PhoneNumber,

version-id [1] LnpKey,

service-prov-id [2] ServiceProvId,

service-prov-due-date [3] GeneralizedTime,

service-prov-authorization-creation-time-stamp [4] GeneralizedTime,

subscription-timer-type [5] Integer OPTIONAL,

subscription-business-type [6] Integer OPTIONAL

}

VersionCustomerDisconnectDate ::= SEQUENCE {

tn PhoneNumber,

version-id LnpKey,

service-prov-customer-disconnect-date GeneralizedTime,

service-prov-effective-release-date GeneralizedTime OPTIONAL,

access-control LnpAccessControl

}

VersionCustomerDisconnectDateRecovery ::= SEQUENCE {

tn PhoneNumber,

version-id LnpKey,

service-prov-customer-disconnect-date GeneralizedTime,

service-prov-effective-release-date GeneralizedTime OPTIONAL

}

VersionNewNPA-NXX ::= SEQUENCE {

service-prov-npa-nxx-id NPA-NXX-ID,

service-prov-npa-nxx-value NPA-NXX,

service-prov-npa-nxx-effective-time-stamp GeneralizedTime,

service-prov-id ServiceProvId,

access-control LnpAccessControl

}

VersionNewNPA-NXX-Recovery ::= SEQUENCE {

service-prov-npa-nxx-id NPA-NXX-ID,

service-prov-npa-nxx-value NPA-NXX,

service-prov-npa-nxx-effective-time-stamp GeneralizedTime,

service-prov-id ServiceProvId

}

VersionNewSP-CreateRequest ::= SEQUENCE {

version-create-request [0] VersionCreateConcurrenceRequest,

service-prov-old-authorization [1] ServiceProvAuthorization,

subscription-status-change-cause-code [2] SubscriptionStatusChangeCauseCode

}

VersionNewSP-CreateRequestRecovery ::= SEQUENCE {

version-create-request [0] VersionCreateConcurrenceRequestRecovery,

service-prov-old-authorization [1] ServiceProvAuthorization,

subscription-status-change-cause-code [2] SubscriptionStatusChangeCauseCode

}

VersionNewSP-FinalCreateWindowExpiration ::= SEQUENCE {

version-create-request [0] VersionCreateConcurrenceRequest,

new-service-prov-id [1] ServiceProvId,

service-prov-old-authorization [2] ServiceProvAuthorization,

subscription-status-change-cause-code [3]

SubscriptionStatusChangeCauseCode OPTIONAL

}

VersionNewSP-FinalCreateWindowExpirationRecovery ::= SEQUENCE {

version-create-request [0] VersionCreateConcurrenceRequestRecovery,

new-service-prov-id [1] ServiceProvId,

service-prov-old-authorization [2] ServiceProvAuthorization,

subscription-status-change-cause-code [3]

SubscriptionStatusChangeCauseCode OPTIONAL

}

VersionOldSP-ConcurrenceRequest ::= VersionCreateConcurrenceRequest

VersionOldSP-ConcurrenceRequestRecovery ::=

VersionCreateConcurrenceRequestRecovery

VersionOldSPFinalConcurrenceWindowExpiration ::= SEQUENCE {

tn PhoneNumber,

version-id LnpKey,

access-control LnpAccessControl,

subscription-timer-type [0]Integer OPTIONAL,

subscription-business-type [1] Integer OPTIONAL

}

VersionOldSPFinalConcurrenceWindowExpirationRecovery ::= SEQUENCE {

tn [0] PhoneNumber,

version-id [1] LnpKey,

subscription-timer-type [2]Integer OPTIONAL,

subscription-business-type [3] Integer OPTIONAL

}

VersionRangeAttributeValueChange ::= SEQUENCE {

range-attribute-value-change RangeAttributeValueChangeInfo,

access-control LnpAccessControl

}

VersionRangeAttributeValueChangeRecovery ::= RangeAttributeValueChangeInfo

VersionRangeObjectCreation ::= SEQUENCE {

range-object-creation RangeObjectCreationInfo,

access-control LnpAccessControl

}

VersionRangeObjectCreationRecovery ::= RangeObjectCreationInfo

VersionRangeStatusAttributeValueChange ::= SEQUENCE {

range-status-attribute-value-changes RangeStatusAttributeValueChangeInfo,

access-control LnpAccessControl

}

VersionRangeStatusAttributeValueChangeRecovery ::= RangeStatusAttributeValueChangeInfo

VersionRangeCustomerDisconnectDate ::= SEQUENCE {

range-donor-disconnect RangeCustomerDisconnectDateInfo,

access-control LnpAccessControl

}

VersionRangeCustomerDisconnectDateRecovery ::= RangeCustomerDisconnectDateInfo

VersionRangeCancellationAcknowledgeRequest ::= SEQUENCE {

tn-version-id  RangeNotifyTN-ID-Info,

access-control LnpAccessControl

}

VersionRangeCancellationAcknowledgeRecovery ::= RangeNotifyTN-ID-Info

VersionRangeNewSP-CreateRequest ::= SEQUENCE {

range-new-sp-create RangeNewSP-CreateRequestInfo,

access-control LnpAccessControl

}

VersionRangeNewSP-CreateRequestRecovery ::= RangeNewSP-CreateRequestInfo

VersionRangeOldSP-ConcurrenceRequest::= SEQUENCE {

range-oldsp-concurrence RangeOldSP-ConcurrenceRequestInfo,

access-control LnpAccessControl

}

VersionRangeOldSP-ConcurrenceRequestRecovery ::= RangeOldSP-ConcurrenceRequestInfo

VersionRangeOldSPFinalConcurrenceWindowExpiration ::= SEQUENCE {

range-oldsp-final-concurrence RangeOldSPFinalConcurrenceWindowExpirationInfo,

access-control LnpAccessControl

}

VersionRangeOldSPFinalConcurrenceWindowExpirationRecovery ::= RangeOldSPFinalConcurrenceWindowExpirationInfo

VersionRangeNewSP-FinalCreateWindowExpiration ::= SEQUENCE {

range-newsp-final-create RangeNewSP-FinalCreateWindowExpirationInfo,

access-control LnpAccessControl

}

VersionRangeNewSP-FinalCreateWindowExpirationRecovery ::= RangeNewSP-FinalCreateWindowExpirationInfo

VersionStatus ::= ENUMERATED {

conflict (0),

active (1),

pending (2),

sending (3),

download-failed (4),

download-failed-partial (5),

disconnect-pending (6),

old (7),

canceled (8),

cancel-pending (9)

}

VersionStatusAttributeValueChange ::= SEQUENCE {

value-change-info [0] AttributeValueChangeInfo,

failed-service-provs [1] Failed-SP-List OPTIONAL,

subscription-status-change-cause-code [2] SubscriptionStatusChangeCauseCode

OPTIONAL,

access-control [3] LnpAccessControl

}

VersionStatusAttributeValueChangeRecovery ::= SEQUENCE {

value-change-info [0] AttributeValueChangeInfo,

failed-service-provs [1] Failed-SP-List OPTIONAL,

subscription-status-change-cause-code [2] SubscriptionStatusChangeCauseCode

OPTIONAL

}

END -- LNP-ASN1

--multipart-boundary--

# Subscription Version Status

8

| Subscription Version Status Interaction Descriptions |  |  |  |
| --- | --- | --- | --- |
| # | Interaction Name | Type | Description |
| 1 | Conflict to Canceled | NPAC SMS Internal | NPAC SMS automatically sets a Subscription Version in conflict directly to canceled after it has been in conflict for a tunable number of calendar days. |
|  |  | SOA to NPAC SMS Interface or NPAC Operations Interface - NPAC Personnel | The old Service Provider User (or NPAC personnel acting on behalf of the Service Provider) sends a cancellation request for a Subscription Version created by that Service Provider with a status of conflict that has not been concurred by the other new Service Provider. |
| 2 | Conflict to Cancel Pending | NPAC Operations Interface - NPAC Personnel | User cancels a Subscription Version in conflict or cancels a Subscription Version that was created by or concurred to by both Service Providers. |
|  |  | SOA to NPAC SMS Interface | User sends a cancellation request for a Subscription Version that was created by or concurred to by both Service Providers. |
| 3 | Cancel Pending to Conflict | NPAC Operations Interface - NPAC Personnel | User sets a Subscription Version with a status of cancel pending to conflict. |
|  |  | NPAC SMS Internal | NPAC SMS automatically sets a Subscription Version with a status of cancel pending to conflict if cancel pending acknowledgment has not been received from the new Service Provider within a tunable timeframe. |
| 4 | Conflict to Pending | NPAC Operations Interface - NPAC Personnel and SOA to NPAC SMS Interface - Old Service Provider | User removes a Subscription Version from conflict. |
|  |  | SOA to NPAC SMS Interface - New Service Provider | New Service Provider User removes a Subscription Version from conflict. This action can only occur if a tunable number of hours have elapsed since the Subscription Version was placed in conflict. |
| 5 | Pending to Conflict | NPAC Operations Interface - NPAC Personnel | - User sets a Subscription Version with a status of pending to conflict. - User creates a Subscription Version for an existing pending Subscription Version for the old Service Provider and does not provide authorization for the transfer of service. |
|  |  | SOA to NPAC SMS Interface - Old Service Provider | Old Service Provider sends a Subscription Version creation or modification request for a Subscription Version with a status of pending, which revokes the old Service Provider’s authorization for transfer of service. This action can only be taken once, and must be taken a tunable number of hours prior to the new Service Provider due date. |
| 6 | Pending to Canceled | NPAC Operations Interface - NPAC Personnel | User cancels a Subscription Version with a status of pending that has not been concurred by both service providers. |
|  |  | SOA to NPAC SMS Interface | Service Provider User sends a cancellation request for a Subscription Version created by that Service Provider with a status of pending that has not been concurred by the other Service Provider. |
|  |  | NPAC SMS Internal | - NPAC SMS automatically sets a pending Subscription Version to canceled after authorization for the transfer of service has not been received from the new Service Provider within a tunable timeframe. - NPAC SMS automatically sets a pending Subscription Version to canceled if an activation request is not received a tunable amount of time after new Service Provider due date. |
| 7 | Pending to Cancel Pending | NPAC Operations Interface - NPAC Personnel | User cancels a Subscription Version with a status of pending that has been created/concurred by both Service Providers. |
|  |  | SOA to NPAC SMS Interface | Service Provider User sends a cancellation request for a Subscription Version with a status of pending that has been concurred by the other Service Provider. |
| 8 | Cancel Pending to Canceled | NPAC SMS Internal | NPAC SMS automatically sets a cancel pending Subscription Version to canceled after receiving cancel pending acknowledgment from the concurring Service Provider, or the final cancellation concurrence window has expired without cancel concurrence from the old Service Provider. |
| 9 | Creation - Set to Conflict | NPAC Operations Interface - NPAC Personnel | User creates a Subscription Version for the old Service Provider and does not provide authorization for the transfer of service. |
|  |  | SOA to NPAC SMS Interface - Old Service Provider | User sends an old Service Provider Subscription Version creation request and does not provide authorization for the transfer of service. |
| 10 | Creation - Set to Pending | NPAC Operations Interface - NPAC Personnel | User creates a Subscription Version for either the new or old Service Provider. If the create is for the old Service Provider and authorization for the transfer of service is not provided, refer to # 9, Creation - Set to Conflict, NPAC Operations Interface. |
|  |  | SOA to NPAC SMS Interface | User sends a Subscription Version creation request for either the new or old Service Provider. If the create is for the old Service Provider, and authorization for the transfer of service is not provided, refer to # 9, Creation - Set to Conflict, SOA to NPAC SMS Interface. |
| 11 | Disconnect Pending to Sending | NPAC SMS Internal | NPAC SMS automatically sets a deferred disconnect pending Subscription Version to sending after the effective release date is reached. |
| 13 | Pending to Sending | NPAC Operations Interface - NPAC Personnel | User activates a pending Subscription Version for a Subscription Version with a new Service Provider due date less than or equal to today. |
|  |  | SOA to NPAC SMS Interface - New Service Provider | New Service Provider User sends an activation message for a pending Subscription Version for a Subscription Version with a new Service Provider due date less than or equal to today. |
| 14 | Sending to Failed | NPAC SMS Internal | NPAC SMS automatically sets a Subscription Version from sending to failed after all Local SMSs fail Subscription Version activation after the tunable retry period expires. |
| 15 | Failed to Sending | NPAC Operations Interface - NPAC Personnel | User re-sends a failed Subscription Version. |
| 16 | Partially Failed to Sending | NPAC Operations Interface - NPAC Personnel | User re-sends a partial failure Subscription Version. |
| 17 | Sending to Partially Failed | NPAC SMS Internal | NPAC SMS automatically sets a Subscription Version from sending to partial failure after one or more, but not all, of the Local SMSs fail the Subscription Version activation after the tunable retry period expires. |
| 18 | Sending to Old | NPAC SMS Internal | NPAC SMS automatically sets a sending Subscription Version to old after a disconnect or “porting to original” port to all Local SMSs successfully completes. Disconnects that fail on one or more, but not all, Local SMSs will also be set to old. |
| 19 | Sending to Active | NPAC SMS Internal | - NPAC SMS automatically sets a sending Subscription Version to active after the Subscription Version activation is successful in all of the Local SMSs. - NPAC SMS automatically sets a sending Subscription Version to active after the Subscription Version modification is successfully broadcast to any of the Local SMSs after all have responded. - NPAC SMS automatically sets a sending Subscription Version to active after a failure to all Local SMSs on a disconnect. |
| 20 | Active to Sending | NPAC Operations Interface - NPAC Personnel | User disconnects an active Subscription Version and does not supply an effective release date, or User modifies an active Subscription Version or resends a failed disconnect or modify. |
|  |  | SOA to NPAC SMS Interface - Current Service Provider | User sends a disconnect request for an active Subscription Version and does not supply an effective release date, or User modifies an active Subscription Version. |
| 21 | Active to Old | NPAC SMS Internal | NPAC SMS automatically sets the currently active Subscription Version to old once a currently active subscription version is superceded by a pending subscription version, due to the fact that the current version is set to old when an activate occurs. The new pending version is set to sending and then to active, partially failed, or old. On a disconnect the sending state occurs before the old. |
| 22 | Disconnect Pending to Active | NPAC Operations Interface - NPAC Personnel | User cancels a Subscription Version with a disconnect pending status. |
|  |  | SOA to NPAC SMS Interface - New Service Provider | User sends a cancellation request for a disconnect pending Subscription Version. |
| 23 | Active to Disconnect Pending | NPAC Operations Interface - NPAC Personnel | User disconnects an active Subscription Version and supplies a future effective release date. |
|  |  | SOA to NPAC SMS Interface - Current Service Provider | User sends a disconnect request for an active Subscription Version and supplies a future effective release date. |
| 24 | Old to Sending | NPA Operations Interface – NPAC Personnel | User re-sends a partial failure of a disconnect or partial failure or failure of a port-to-original Subscription Version. |
| 25 | Old to Old | NPAC SMS Internal | NPAC SMS automatically sets a Subscription Version from old to old after one or more previously failed Local SMSs successfully disconnect a Subscription Version, as a result of an audit or LSMS recovery. The Failed_SP_List is updated to reflect the updates to the previously failed SPs. |
| 26 | Partially Failed to Active | NPAC SMS Internal | NPAC SMS automatically sets a Subscription Version from partial failure to active after all previously failed Local SMSs successfully activate a Subscription Version, as a result of an audit or LSMS recovery. The Failed_SP_List is updated to reflect the updates to the previously failed SPs. |
| 27 | Partially Failed to Partially Failed | NPAC SMS Internal | NPAC SMS automatically sets a Subscription Version from partial failure to partial failure after one or more, but not all previously failed Local SMSs successfully activate a Subscription Version, as a result of an audit or LSMS recovery. The Failed_SP_List is updated to reflect the updates to the previously failed SPs. |

# Number Pool Block Status

In the following table, the reference to “Number Pool Block” data when broadcasting to an LSMS is based on that Service Provider’s EDR Indicator (i.e., Number Pool Block object or Subscription Versions with LNP Type of POOL).

9

| Number Pool Block Version Status Interaction Descriptions |  |  |  |
| --- | --- | --- | --- |
| # | Interaction Name | Type | Description |
| 1 | Creation - Set to Sending | NPAC SMS Internal | NPAC SMS creates a Number Pool Block for the Block Holder Service Provider. |
|  |  | NPAC Operations Interface - NPAC Personnel | User sends a Number Pool Block creation request for the Block Holder Service Provider. |
|  |  | SOA to NPAC SMS Interface - Block Holder Service Provider | The Service Provider User sends a Number Pool Block creation request for itself (the Block Holder Service Provider). |
| 2 | Sending to Partial Failure | NPAC SMS Internal | NPAC SMS automatically sets a Number Pool Block from sending to partial failure after one or more, but not all, of the Local SMSs fail the Number Pool Block activation after the tunable retry period expires. |
| 3 | Partial Failure to Sending | NPAC Operations Interface - NPAC Personnel | User re-sends a partial failure Number Pool Block. |
| 4 | Sending to Failed | NPAC SMS Internal | NPAC SMS automatically sets a Number Pool Block from sending to failed after all Local SMSs fail Number Pool Block activation after the tunable retry period expires. |
| 5 | Failed to Sending | NPAC Operations Interface - NPAC Personnel | User re-sends a failed Number Pool Block. |
| 6 | Sending to Active | NPAC SMS Internal | - NPAC SMS automatically sets a sending Number Pool Block to active after the Number Pool Block activation is successful in all of the Local SMSs. - NPAC SMS automatically sets a sending Number Pool Block to active after the Number Pool Block modification is broadcast to all of the Local SMSs and either all have responded or retries have been exhausted. - NPAC SMS automatically sets a sending Number Pool Block to active after a failure to all Local SMSs on a de-pool. |
| 7 | Active to Sending | NPAC Operations Interface - NPAC Personnel | - User de-pools an active Number Pool Block. - User modifies an active Number Pool Block. - User resends a failed de-pool or modify Number Pool Block. |
|  |  | SOA to NPAC SMS Interface - Block Holder Service Provider | User modifies an active Number Pool Block. |
| 8 | Sending to Old | NPAC SMS Internal | - NPAC SMS automatically sets a sending Number Pool Block to old after a de-pool to all Local SMSs successfully completes. - NPAC SMS automatically sets a sending Number Pool Block to old after a de-pool that fails on one or more, but not all Local SMSs. |
| 9 | Old to Sending | NPA Operations Interface – NPAC Personnel | User re-sends a partial failure of a de-pool. |
| 10 | Partial Failure to Partial Failure | NPAC SMS Internal | NPAC SMS automatically sets a Number Pool Block from partial failure to partial failure after one or more, but not all previously failed Local SMSs successfully activate a Number Pool Block, as a result of an audit or LSMS recovery. The Failed_SP_List is updated to reflect the updates to the previously failed SPs. |
| 11 | Partial Failure to Active | NPAC SMS Internal | NPAC SMS automatically sets a Number Pool Block from partial failure to active after all previously failed Local SMSs successfully activate a Number Pool Block, as a result of an audit or LSMS recovery. The Failed_SP_List is updated to reflect the updates to the previously failed SPs. |
| 12 | Old to Old | NPAC SMS Internal | NPAC SMS automatically sets a Number Pool Block from old to old after one or more previously failed Local SMSs successfully de-pools a Number Pool Block, as a result of an audit or LSMS recovery. The Failed_SP_List is updated to reflect the updates to the previously failed SPs. |