| ITEM NO. | ISSUE DESCRIPTION | DATE SUBMITTED | STATUS / RESOLUTION OF ISSUE | DATE RESOLVED |
| --- | --- | --- | --- | --- |
| PIMs |  |  |  |  |
| 1 (PIM 22) Inter-modal Issue | This issue, submitted by a wireline carrier, addressed customers being taken out of service inadvertently due to the New Service Provider continuing with a port that was placed into conflict by the Old Service Provider after the 6 hour timer expired, instead of investigating why the port was placed into conflict. At their 5/18/04 meeting, the NANC assigned an action item to the LNPA to “finish technical requirements for PIM 22 (NANC change order 375) and submit to NAPM LLC for next software release.” | August 2002 Subsequent action item assigned by NANC in May 2004. | This issue was resolved with the implementation of NANC Change Order 375 in NPAC Release 3.3, which prevents the activation of a port placed into conflict by the Old SP if an LSR/WPR was not issued or an FOC/WPRR was not received. | March 2006 |
| 2 (PIM 24) Inter-modal Issue | This issue, submitted by the Pool Administrator and a wireless carrier, addressed instances where service providers are not following guidelines for block donation. For example, in some instances, contaminated blocks are being donated as non-contaminated blocks, or blocks with greater than 10% contamination are being donated. This is causing customers to be taken out of service or blocks to be exchanged for a less contaminated or non-contaminated block. | May 2003 | This issue was resolved as a result of INC changes to the 1K block donation form at the request of the LNPA WG, and the completion of the one-time scrub of all rate center pools by the Pool Administrator. | November 2006 |
| 3 (PIM 28) Inter-modal Issue | This issue, submitted by a wireless carrier, addressed interface differences between the wireline FOC and the wireless WPRR. The Firm Order Confirmation (FOC) allows for a due date and time change on confirmations, however, the Wireless Port Request Response (WPRR) does not. When a wireline carrier sends an FOC with a change in due date or time, the wireless carrier cannot process the change and does not allow the port to complete. | January 2004 | A workaround was put into place in 2004 to temporarily resolve this issue until a permanent fix could be implemented in the WICIS standards to support due date and time changes on the WPRR. | February 2006 with the completion of WICIS Release 3.0 implementations |
| 4 (PIM 29) Inter-modal Issue | This issue, submitted by a wireless carrier, addressed scenarios where customers porting from a wireline carrier are disconnected in the donor switch before the wireless carrier activates the port. | January 2004 | Research determined that a significant cause of this issue was related to due date changes on the wireline FOC (see Item 3 above). The wireless carrier submitting this issue agreed to close the issue as a result of the workaround implemented for PIM 28. | June 2004 |
| 5 (PIM 30) Inter-modal Issue | This issue, submitted by a wireless carrier, sought clarification of the N-1 LNP architecture query responsibilities, and whether wireless carriers are obligated to perform default number portability queries when the N-1 carrier fails to dip the call. | January 2004 | This issue was resolved with the completion of the attached detailed consensus report developed by the LNPA WG, which was presented to the NANC in January 2005 and submitted to the FCC in July 2005. | December 2004 |
| 6 Inter-modal issue | At their 5/18/04 meeting, the NANC assigned an action item to the LNPA to “Review PIM 30 taking into account EAS implications and Enforcement Bureau Decision regarding N-1 responsibilities and recommend whether or not NANC should recommend a rules change to the FCC?” | May 2004 | This issue was resolved with the completion of the detailed consensus report, attached above (see Item 5), developed by the LNPA WG, which was presented to the NANC in January 2005 and submitted to the FCC in July 2005. | December 2004 |
| 7 (PIM 31) Inter-modal Issue | This issue, submitted by a wireless Clearinghouse Vendor, addressed fallout that occurs in cases where the wireline Old Service Provider involved in a port issues a jeopardy notification with a change in due date to the wireless New Service Provider. Wireless carriers currently cannot support jeopardy notices with changes to the due date and time. | February 2004 | Based on additional data gathering and the observed reduction in the number of jeopardies, the submitter of this issue proposed to close PIM 31. | November 2004 |
| 8 (PIM 32) Inter-modal Issue | This issue, submitted by a wireless Clearinghouse Vendor, addresses issues related to the process for obtaining a Customer Service Record (CSR), which contains information necessary to complete a Local Service Request (LSR) for porting in a reseller number. | February 2004 | At the direction of NANC to continue to work the issue, the LNPA WG formed a Subcommittee to continue to work toward consensus on a resolution. Consensus was reached at the March 2007 LNPA WG meeting that direction by resellers to Old Network Service Providers (ONSPs) to provide the specific customer information where possible would greatly reduce the unsuccessful ports. The LNPA WG also believes that resellers should not be allowed to withhold end user specific customer information necessary for the porting process. This was communicated to NANC. The LNPA WG’s consensus position was placed in the NP Best Practices document. | July 2007 |
| 9 (PIM 34) Inter-modal Issue | This issue, submitted by a wireless Clearinghouse Vendor, addressed issues related to the process for obtaining a Customer Service Record (CSR), which contains information necessary to complete a Local Service Request (LSR) for porting in a Type 1 cellular number. | March 2004 | All carriers cooperated in developing alternative solutions to port a Type 1 number. Wireline and wireless providers worked together to migrate Type 1 numbers to Type 2. | July 2005 |
| 10 (PIM 36) Inter-modal issue | This issue, submitted by a wireless Clearinghouse Vendor, addressed NPA-NXX codes being opened in the wrong NPAC regional database by service providers. | April 2004 | This issue was resolved as a result of the implementation of NANC Change Order 321 in NPAC Release 3.3. | March 2006 |
| 11 (PIM 38) Inter-modal Issue | This issue, submitted by a wireless carrier, addressed unnecessary delays in 1K block activation due to the current 5 day minimum interval between when a pooled block is created in NPAC, and the effective date of block activation, when the 1st port has already occurred in the NXX code containing the pooled block. | May 2004 | This issue was resolved as a result of the implementation of NANC Change Order 394 in NPAC Release 3.3. | March 2006 |
| 12 (PIM 39) Inter-modal Issue | This issue, submitted by several wireless carriers, addressed frequent changes in wireline business practices and rules related to porting requirements. | June 2004 | At the December 2004 LNPA WG meeting, consensus was reached that this issue is best addressed in the Change Management meetings with the ILECs or through their Account Management process. This continues to be addressed through the Account Management process. | December 2004 |
| 13 (PIM 40) Inter-modal Issue | This issue, submitted by a wireless carrier, addressed minimum industry standards for LNP readiness that must be adhered to by all companies in order to port. | May 2004 | The LNPA WG researched the relevant industry guidelines and requirements for LNP readiness and documented them in the PIM 40 document, placing it on the LNPA WG’s website for reference. | October 2004 |
| 14 (PIM 41) Inter-modal Issue | This issue, submitted by a wireless carrier, addressed fallout that can occur during SPID migrations when methods other that NANC 323 are used to accomplish the migration. | July 2004 | Text was added to the Industry Numbering Committee’s (INC’s) Central Office Code Administration Guidelines (COCAG) resolving this issue. This issue and its resolution was also placed in the LNPA WG’s NP Best Practices document. | September 2005 |
| 15 (PIM 42) Inter-modal Issue | This issue, submitted by a wireless Clearinghouse Vendor, seeks to review wireline requirements for certain fields on the LSR in order to facilitate mapping of the Wireless Port Request (WPR) to the Wireline LSR. | July 2004 | This issue is being worked through wireline companies’ Account Management process. It was also referred to the Ordering & Billing Forum (OBF) and was tracking the outcomes of Issues 2943 and 3029 in the OBF. The OBF determined that no agreement could be reached within the Intermodal Subcommittee, consisting of ATIS OBF’s Wireless Committee and Local Service Ordering and Provisioning Committee, to resolve this issue. | Issue remains open and tracking awaiting implementation of FCC Order 07-188. |
| 16 (PIM 43) Inter-modal Issue | This issue, submitted by a wireless carrier, addressed concerns related to large porting volumes and mass changes, such as rehomes. | July 2004 | The submitter agreed to withdraw this issue due to the implementation of NANC Change Order 393, which increased the interface throughput requirements, and the development of technical requirements for NANC Change Order 397. NANC 397 is also under discussion in the LNPA WG’s Architecture Planning Team (APT). | August 2004 |
| 17 (PIM 44) Inter-modal Issue | This issue, submitted by a number of wireless carriers, seeks to address varying rules among wireline carriers for developing a Local Service Request (LSR) in order to port a number. | July 2004 | This issue is being worked through wireline companies’ Account Management process. It was also referred to the Ordering & Billing Forum (OBF) and was tracking the outcomes of Issues 2943 and 3029 in the OBF. The OBF determined that no agreement could be reached within the Intermodal Subcommittee, consisting of ATIS OBF’s Wireless Committee and Local Service Ordering and Provisioning Committee, to resolve this issue. | Issue remains open and tracking awaiting implementation of FCC Order 07-188. |
| 18 (PIM 45) Inter-modal Issue | This issue, submitted by a number of wireless carriers, addressed instances when there are errors in Local Service Requests (LSRs) to port a number and some service providers respond identifying a single error only. Additional LSRs and responses are required until all errors are finally cleared. | July 2004 | The LNPA WG reached consensus to place this issue in its Number Portability Best Practices document with text advising that service providers should avoid a process of only reporting one error on each response to a port request resulting in a prolonged process of submitting multiple, iterative port requests for a single port. | October 2005 |
| 19 (PIM 47) Inter-modal Issue | This PIM, submitted by a wireless carrier, addressed minimum industry inter-modal standards for purging old/abandoned ports. | July 2004 | The resolution reached by consensus at the LNPA WG was placed in the LNPA WG’s Number Portability Best Practices document. | January 2005 |
| 20 (PIM 48) Inter-modal Issue | This PIM, submitted by a number of wireless carriers and a wireless Clearinghouse Vendor, proposed a new category in the NIIF National LNP Contact Directory for post-port carrier-to-carrier support. | September 2004 | This PIM was referred to the NIIF. The NIIF/NIOC responded with development of the NIIF Company Specific Contact Directory and LNP Contact Directory New Category Proposal and Processes for Updating Contacts in response the LNPA WG PIM. | February 2005 |
| 21 (PIM 49) Inter-modal Issue | This PIM, submitted by a number of wireless carriers, addressed the NANC LNP Provisioning Flows related to the porting of Type 1 numbers. It also addressed the inadvertent porting of paging numbers. | September 2004 | Multiple solutions were put in place for the various providers to prevent paging numbers from being ported and for obtaining and validating Type 1 information from the Type 1 provider. | July 2005 |
| 22 (PIM 50) Inter-modal Issue | This PIM, submitted by a wireless Clearinghouse Vendor, addressed instances where wireline to wireless ports fail the automated process because they are from large accounts where the Customer Service Record (CSR) is too large to return on a CSR query. | January 2005 | The LNPA WG reached consensus to add to the Number Portability Best Practices document text stating that if wireline carriers sent only the information requested in the customer inquiry per the LSOG CSI guidelines, this error would be greatly reduced if not eliminated. | March 2007 |
| 23 (PIM 51) Inter-modal Issue | This PIM, submitted by a wireless carrier, seeks the prevention of NXX codes being opened to portability in NPAC by the incorrect provider. | March 2005 | NANC Change Order 414 was developed to address a mechanized solution. The LNPA WG has recommended to the NAPM LLC a manual cleanup in the NPAC database. | Issue is now in a Tracking state for NANC 414. |
| 24 (PIM 52) Inter-modal Issue | This PIM, submitted by a wireless carrier, addresses issues related to carriers receiving 1K blocks from the pool in which the Intra-Service Provider ports have not been completed by the donor provider prior to block donation to the pool. | November 2005 | This issue was referred to the INC (INC Issue 506), which made changes to the Thousands Block PA Guidelines and form. This issue is now awaiting implementation of FCC-approved PA Change Order 51. | April 2007 |
| 25 (PIM 53) Inter-modal Issue | This PIM, submitted by a wireless carrier, addressed instances of providers who were taking back numbers that had ported out from them when they do not have evidence that they issued a Firm Order Confirmation (FOC). | February 2006 | The LNPA WG reached consensus on a resolution for this issue and placed it in its Number Portability Best Practices document. | July 2006 |
| 26 (PIM 54) Inter-modal Issue | This PIM, submitted by a wireline carrier, seeks to investigate the feasibility of shortening the wireline-wireline and inter-modal porting interval for certain ports. | April 2006 | Consensus was reached at the LNPA WG to investigate ways to shorten the pre-port interval. A Pre-Port Subcommittee was formed and continues to work the issue. | Issue continues to be worked. |
| 27 (PIM 55) Inter-modal Issue | This PIM, submitted by a wireless Clearinghouse Vendor, addresses issues related to wireline Provider Initiated Activity, e.g., Jeopardies. | May 2006 | This PIM is now in a tracking state awaiting inclusion in the next WICIS Release beyond 4.0, which will likely be deployed sometime in 2009 | Tracking |
| 28 (PIM 56) Inter-modal Issue | This PIM, submitted by a wireless carrier, addresses instances where LNP database updates are not always propagated by all providers down to their network element routing databases in a timely manner. | May 2006 | The LNPA WG reached consensus to place text addressing this issue in its Number Portability Best Practices document. The text is currently under development. | September 2007 |
| 29 (PIM 57) Inter-modal Issue | This PIM, submitted by a number of wireless carriers, addresses issues related to attempting to port a consumer when a Reseller abruptly discontinues business and/or declares bankruptcy. | August 2006 | The LNPA WG is currently reviewing contributions from the PIM submitters regarding methods to deal with this scenario. | Issue continues to be worked. |
| 30 (PIM 58) Inter-modal Issue | This PIM, submitted by a number of wireline carriers, addressed a process for the legitimate opening of NXX codes to portability in the NPAC when the current codeholder cannot be contacted or refuses to open the code themselves. | October 2006 | The LNPA WG developed a process which includes the proper regulatory authorization that was incorporated in its Number Portability Best Practices document. | July 2007 |
| 31 (PIM 59) Inter-modal Issue | This PIM, submitted by a wireless Clearinghouse Vendor, addresses issues related to the unlocking of the 911 database when numbers are ported to VoIP providers. | November 2006 | Text for the NP Best Practices document was accepted on the June 12th LNPA WG conference call. | June 2007 |
| 32 (PIM 60) Inter-modal Issue | This PIM, submitted by a wireline carrier, requests that the LNPA WG provide an opinion on whether or not a customer, who is physically relocating to a different Rate Center, should be allowed to port their number. | March 2007 | Text for the NP Best Practices document was accepted at the July 2007 LNPA WG meeting. | July 2007 |
| CHANGE ORDERS |  |  |  |  |
| 33 (ILL 130) Inter-modal Issue | This Change Order, submitted by a wireline carrier, enhanced the error message text over the SOA/LSMS interface. | January 1997 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 34 (NANC 138) Inter-modal Issue | This Change Order, submitted by the Change Management Administrator, clarified and resolved discrepancies in the Cause Value definitions. | August 1997 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 35 (NANC 151) Inter-modal Issue | This Change Order, submitted by a local system vendor, requested that the applicable telephone number or pooled block be added to SOA notifications. | September 1997 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 36 (NANC 227) Inter-modal Issue | - This Change Order, submitted by a wireline carrier, requested that a mechanism be developed to clean up partially failed ported number records that need to be subsequently modified or deleted, by the New SP. | August 1998 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 37 (NANC 285) Inter-modal Issue | This Change Order, submitted by the LNPA WG, requested enhancement to the SOA and LSMS query functionality when the maximum query size was exceeded. | May 1999 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 38 (NANC 299) Inter-modal Issue | This Change Order, submitted by the LNPA WG, requested implementation of NPAC monitoring of SOA and LSMS associations via a heartbeat message. | September 1999 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 39 (NANC 300) Inter-modal Issue | - This Change Order, submitted by the LNPA WG, requested that a mechanism be developed to clean up partially failed pooled block records that need to be subsequently modified or deleted, by the New SP. | December 1999 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 40 (NANC 343) Inter-modal Issue | This Change Order, submitted by the LNPA WG, corrected the filtering operations currently supported by the NPAC as detailed in the Interoperability Interface Specification (IIS). | November 2001 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 41 (NANC 346) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, enabled modification of the timestamp for pooled block activations when discrepancies are found during audits. | January 2002 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 42 (NANC 347 and 350) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, enhanced system abort behavior to reduce the frequency of association aborts, thus improving reliability and performance. | March 2002 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 43 (NANC 348) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, implemented bulk data download functionality for SOA notifications, thus reducing operational issues and the work effort required for a Service Provider to get back in sync with the NPAC. | March 2002 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 44 (NANC 351) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, improved Service Provider recovery functionality and their ability to recover previously missed data. | April 2002 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 45 (NANC 352) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, further improved Service Provider recovery functionality and their ability to recover previously missed SPID data. | April 2002 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 46 (NANC 357) Inter-modal Issue | This Change Order, submitted by a wireline carrier, requested unique identifiers in NPAC for wireline vs. wireless carriers. This assists in the proper handling of port requests, e.g., 911 records. | April 2002 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 47 (NANC 358) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, corrected the ASN.1 document to reflect that a provider SPID is a 4 character alphanumeric. | April 2002 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 48 (NANC 359) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, corrected the ASN.1 document to reflect that a provider Billing ID is a 4 character alphanumeric. | April 2002 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 49 (NANC 360) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, corrected the maximum TN recovery tunable definition in industry documentation. | April 2002 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 50 (NANC 361) Inter-modal Issue | This Change Order, submitted by a wireline carrier, requested correction of the definition for certain range notifications in industry documentation. | May 2002 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 51 (NANC 364) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, removed confusing text in industry documentation related to TN ranges. | July 2002 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 52 (NANC 365) Inter-modal Issue | This Change Order, submitted by a wireless Clearinghouse Vendor, requested correction of discrepancies identified in text in industry documentation related to Port-to-Originals and SV Queries. | August 2002 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 53 (NANC 368) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, implemented flow control functionality to prevent interface congestion. | October 2002 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 54 (NANC 371) Inter-modal Issue | This Change Order, submitted by a wireline carrier, requested that industry documentation be corrected to explicitly state that the NPAC requires audit names to be unique. | November 2002 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 55 (NANC 373) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, corrects industry documentation to reflect current NPAC behavior related to setting an SV in conflict. | November 2002 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 56 (NANC 374) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, corrects industry documentation to reflect current NPAC behavior related to Port-to-Originals. | November 2002 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 57 (NANC 376) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, corrects industry documentation to reflect current NPAC behavior related to modifying an active SV. | December 2002 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 58 (NANC 383) Inter-modal Issue | This Change Order, submitted by the LNPA WG, implemented functionality in support of a separate SOA channel for notifications. | May 2003 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 59 (NANC 385) Inter-modal Issue | This Change Order, submitted by the LNPA WG, modified the Timer Expiration Calculation such that a time period designated as an NPAC Maintenance Window that falls within normal business operating hours will NOT “use up” any hours, when calculating the expiration of a timer. | July 2003 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 60 (NANC 386) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, modified the NPAC to only allow a single CMIP association between a SOA/LSMS and the NPAC. | July 2003 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 61 (NANC 387) Inter-modal Issue | This Change Order, submitted by a wireless Clearinghouse Vendor, requested correction of industry documentation related to port CREATE requests. | September 2003 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 62 (NANC 388) Inter-modal Issue | This Change Order, submitted by a wireless carrier, requested functionality in support of undoing a Cancel port request. | September 2003 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 63 (NANC 391) Inter-modal Issue | This Change Order, submitted by the LNPA WG, updated the appropriate industry documentation to reference wireless operations and functions, e.g., Wireless Port Request (WPR), Wireless Port Request Response (WPRR). | January 2004 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 64 (NANC 392) Inter-modal Issue | This Change Order, submitted by the LNPA WG Architecture Planning Team, eliminated the creation of intermediate “cloned” copies of SVs, which were a source of confusion for providers initiating queries on a number. | March 2004 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 65 (NANC 393) Inter-modal Issue | This Change Order, submitted by the LNPA WG Architecture Planning Team, increased the interface throughput requirements to meet the projected increase in porting and pooling volumes. | May 2004 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 66 (NANC 399) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, added SV Type and Alternative SPID fields to ported and pooled block SVs. One identified use is the identification of Resellers. | January 2005 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 67 (NANC 404) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, corrected industry documentation to be consistent with NPAC behavior. | July 2005 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 68 (NANC 405) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, corrected industry documentation related to provider SOA and LSMS recovery functionality. | July 2005 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 69 (NANC 406) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, corrected industry documentation related to NPAC functionality such as canceling an SV, querying an SV. | July 2005 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 70 (NANC 407) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, fixed an issue related to TN range operations that was impacting NPAC response time. | September 2005 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 71 (NANC 409) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, corrected industry documentation to be consistent with NPAC behavior. | October 2005 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 72 (NANC 410) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, corrected industry documentation related to provider recovery, disconnects, and added additional error messages that have been implemented in the NPAC. | November 2005 | This Change Order was developed and implemented in NPAC Release 3.3. | March 2006 |
| 73 (NANC 388v2) Inter-modal Issue | This Change Order, submitted by the NPAC vendor at the request of a wireline carrier, fixed an issue with the undo cancel functionality that was enabling providers to circumvent the conflict status functionality implemented in NANC 375. | May 2006 | This Change Order was developed and implemented in a Point Release to NPAC Release 3.3. | November 2006 |
| 74 (NANC 411) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, corrected industry documentation related to provider recovery, disconnects, create flows, and error mapping. | April 2006 | This Change Order was developed and implemented in a Point Release to NPAC Release 3.3. | November 2006 |
| 75 (NANC 412) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, corrected industry documentation related to functionality including SV Type, Alternative SPID, SPID recovery, Bulk Data Download file for notifications, Cause Code definitions, and SPID migrations. | May 2006 | This Change Order was developed and implemented in a Point Release to NPAC Release 3.3. | November 2006 |
| 76 (NANC 372) Inter-modal Issue | This Change Order, submitted by a wireline carrier, requests investigation of alternatives to the CMIP protocol for the SOA and LSMS interfaces. | November 2002 | This Change Order is being revisited by the LNPA WG Architecture Planning Team (APT). | Change Order remains open. |
| 77 (NANC 396) Inter-modal Issue | This Change Order, submitted by the LNPA WG, proposes that providers be able to place filters in the NPAC on NPA-NXXs prior to the code being opened up in the NPAC, and the ability to place filters at the NPA level, for codes that a provider does not wish to receive downloads. | September 2004 | This Change Order remains in the OPEN category. To date, it has not been prioritized for inclusion in a future NPAC release. | Change Order remains open. |
| 78 (NANC 402) Inter-modal Issue | This Change Order, submitted by a wireless carrier, requests that the NPAC validate the code owner (SPID) prior to the code being opened up in the NPAC. | February 2005 | This Change Order remains in the OPEN category. To date, it has not been prioritized for inclusion in a future NPAC release. | Change Order remains open. |
| 79 (NANC 408) Inter-modal Issue | This Change Order, submitted by a wireless carrier, requests automation of certain SPID migration functions. | October 2005 | This Change Order remains in the OPEN category. To date, it has not been prioritized for inclusion in a future NPAC release. | Change Order remains open. |
| 80 (NANC 413) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, proposes corrections to industry documentation related to functionality including CREATE and MODIFY. | May 2006 | This Change Order remains in the OPEN category. To date, it has not been prioritized for inclusion in a future NPAC release. | Change Order remains open. |
| 81 (NANC 415) Inter-modal Issue | This Change Order, submitted by a Video Relay Service (VRS) provider, requests the addition of two URI fields in ported and pooled block SVs in support of VRS. | December 2006 | The LNPA WG completed development of the technical requirements for this Change Order and forwarded the document to the Industry Numbering Committee for consideration in their analysis of potential solutions. | January 2007 |
| 82 (NANC 416) Inter-modal Issue | This Change Order, submitted by the LNPA WG, proposes that two attributes be added to the Notification Bulk Data Download (BDD) file to make it consistent with the actual notification that is sent over the SOA interface. | September 2006 | This Change Order remains in the OPEN category. To date, it has not been prioritized for inclusion in a future NPAC release. | Change Order remains open. |
| 83 (NANC 417) Inter-modal Issue | This Change Order, submitted by a wireless Clearinghouse Vendor, requests that a count of records changed be included for Bulk Data Download (BDD) files and Delta BDD files. | December 2006 | This Change Order remains in the OPEN category. To date, it has not been prioritized for inclusion in a future NPAC release. | Change Order remains open. |
| 84 (NANC 418) Inter-modal Issue | This Change Order, submitted by a wireless Clearinghouse Vendor, requests record count information of the contents of the SPID migration update (SMURF) files that are distributed to perform updates to the LSMS platforms throughout the industry. | December 2006 | This Change Order remains in the OPEN category. To date, it has not been prioritized for inclusion in a future NPAC release. | Change Order remains open. |
| 85 (NANC 147) Inter-modal Issue | This Change Order, submitted by a wireline carrier, proposes that a strategy be developed for Version Identifications (IDs) that are approaching the maximum value and will roll over to zero. | August 1997 | This Change Order remains in the ACCEPTED category, but has not been prioritized for inclusion in a future NPAC release. The LNPA WG is now revisiting the issue due to approaching projected rollovers. | Change Order remains open. |
| 86 (NANC 355) Inter-modal Issue | This Change Order, submitted by a wireline carrier, requests the ability for NPAC personnel to modify the effective dates of NXX codes involved in an NPA split. | April 2002 | This Change Order remains in the ACCEPTED category, but has not been prioritized for inclusion in a future NPAC release. | Change Order remains open. |
| 87 (NANC 382) Inter-modal Issue | This Change Order, submitted by the NPAC vendor and a wireline carrier, proposes development of a “port protection” system in the NPAC to prevent inadvertent porting or intentional slamming of numbers. | April 2003 | This Change Order remains in the ACCEPTED category, but has not been prioritized for inclusion in a future NPAC release. | Change Order remains open. |
| 88 (NANC 390) Inter-modal Issue | This Change Order, submitted by a wireline carrier, proposes development and implementation of a positive acknowledgement from the NPAC that the NPAC has received their request message, so the systems (SOA/LSMS) do not unnecessarily resend the message and cause duplicate transactions for the same request. | October 2003 | This Change Order remains in the ACCEPTED category, but has not been prioritized for inclusion in a future NPAC release. | Change Order remains open. |
| 89 (NANC 397) Inter-modal Issue | This Change Order, submitted by a wireless carrier, requests that the performance impacts to the SOAs, NPAC, and LSMSs be determined for large volume ports, i.e., up to 25,000 TNs in an hour. | July 2004 | This Change Order is being revisited by the LNPA WG Architecture Planning Team (APT). | Change Order remains open. |
| 90 (NANC 400) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, proposes additional fields in ported and pooled block records in order to provision IP addresses (Uniform Resource Identifiers) in support of IP-enabled services and for routing to numbers that have been ported or pooled to VoIP providers. | January 2005 | The technical requirements for this Change Order have been completed by the LNPA WG. The FCC directed that this Change Order be placed on hold until further notice. | April 2005 |
| 91 (NANC 401) Inter-modal Issue | This Change Order, submitted by a Service Bureau, proposes development of a separate LSMS association for downloading optional data fields in ported number and pooled block records, e.g., the proposed URI fields in NANC 400. | January 2005 | This Change Order remains in the ACCEPTED category, but has not been prioritized for inclusion in a future NPAC release. | Change Order remains open. |
| 92 (NANC 403) Inter-modal Issue | This Change Order, submitted by the NPAC vendor, proposes that industry documentation be corrected to reflect that the NPAC will reject any Recovery messages from providers outside of Recovery (i.e., normal mode). | March 2005 | This Change Order remains in the ACCEPTED category, but has not been prioritized for inclusion in a future NPAC release. | Change Order remains open. |
| BEST PRACTICES |  |  |  |  |
| 93 Inter-modal Issue | Some VoIP Services Providers are porting in customers but are refusing to port out customers. | December 2004 | It is the LNPA WG’s position that VoIP service providers along with Wireless and Wireline service providers, have the obligation to port a telephone number to any other service provider when the consumer requests, and the port is within FCC mandates. Porting of telephone numbers used by VoIP service providers should follow the industry porting guidelines and the NANC Inter-Service Provider LNP Operations flows. | April 2005 |
| 94 Inter-modal Issue | Some SPs are requiring that the New SP present a physical copy of the Evidence of Authorization. | March 2005 | It is the LNPA WG’s position that Firm Order Confirmation (FOC) of a port request, or return of requested customer information, e.g., Customer Service Record (CSR), shall not be predicated on the Old Local Service Provider obtaining a physical copy of the evidence of authorization from the New Local Service Provider. In the event of an end user allegation of an unauthorized change, the New Local Service Provider shall, upon request and in accordance with all applicable laws and rules, provide the evidence of authorization to the Old Local Service Provider. At its May 2005 meeting, the North American Numbering Council (NANC) endorsed the LNPA-WG’s position as stated above, and sent a letter to the FCC with its endorsement of the LNPA-WG position. | May 2005 |
| 95 Inter-modal Issue | It has been brought to the LNPA WG’s attention that some service providers, when acting as the Old Local Service Provider in a port, are requiring the New Local Service Provider involved in the port to provide the Social Security Number (SSN) or Tax Identification Number of the consumer wishing to port their number for identification purposes. | February 2005 | It is the position of the LNPA WG that the consumer’s Social Security Number/Tax Identification Number shall not be required on an LSR/WPR to port that consumer’s telephone number if the consumer’s Account Number associated with the Old Local Service Provider is provided on the LSR/WPR for identification. At its May 2005 meeting, the North American Numbering Council (NANC) endorsed the LNPA-WG’s position as stated above, and sent a letter to the FCC with its endorsement of the LNPA-WG position. | May 2005 |
| 96 Inter-modal Issue | Compliance to INC LRN Assignment Practices | September 2005 | The LNPA WG agrees with the INC guidelines and recommends all Service Providers, to the extent possible based on current Business Integrated Routing and Rating Database Systems (BIRRDS) edits, follow these practices and insure all their LRNs are published in the LERG. | November 2005 |
| 97 Inter-modal Issue | Compliance to JIP Standards and Guidelines: The ISUP Jurisdiction Information Parameter (JIP) is a 6-digit parameter in the format of NPA-NXX that is signaled in the Initial Address Message (IAM) by the originating switch. The JIP is used by carriers downstream in the call path to identify the originating switch for billing settlement purposes. When carriers signal an incorrect JIP to another carrier, e.g., signaling an NPA-NXX in the JIP that is LERG-assigned to another carrier, this will result in improper identification of the originating switch. | October 2005 | The LNPA WG supports and reiterates the following signaling requirements and guidelines for JIP as documented in ATIS’ (www.atis.org) industry standard for Local Number Portability – Technical Requirement on Number Portability Switching Systems (T1.TRQ.2-2001) and in ATIS’ Network Interconnection Interoperability Forum’s (NIIF) (www.atis.org/niif/index.asp) Reference Document, Part III, Installation and Maintenance Responsibilities for SS7 Links and Trunks. | December 2005 |
| 98 Inter-modal Issue | It can be difficult and extremely time consuming when a reseller is involved in the port request. The New SP needs to be able to identify who the reseller is in order to obtain the validation information required to submit a valid port request and receive an FOC. | July 2006 | Alternative SPID field introduced in NANC 399: Reseller SPIDs, for use in the alternative SPID data element of an SV, are created in NPAC’s network data only upon an NPAC User’s request. Consistent with the historical use of an entity’s OCN as the entity’s NPAC SPID, the industry strongly encourages each reseller to obtain an OCN from NECA for use as an NPAC SPID. This in turn allows the identity of a reseller associated with a ported number to be displayed as that number’s “alternative SPID.” Notwithstanding this strong industry preference, an NPAC User can request that the NPAC assign a surrogate SPID to a reseller in NPAC’s network data; that surrogate SPID then could be used as the alternative SPID to identify the reseller associated with a ported number. (Surrogate NPAC SPIDs are values that NECA does not assign as OCNs. Currently these values are made up of the alphanumeric values X000 through X999.) | November 2006 |
| 99 Inter-modal Issue | SPs have discrepancies between the Pooling Administration System (PAS) and NPAC pooled blocks which cause double assignment of numbers, blocks that are over-contaminated, and Intra-SP ports that have not been performed. | November 2006 | Change Order 41 directed the Pooling Administrator (PA) to perform a one-time scrub of the entire PAS Database to reduce the likelihood that carriers will receive over-contaminated blocks or incorrectly identified contaminated blocks in lieu of pristine blocks. Following is a list of explanations from the carriers as to why they had discrepancies: - Lack of communication between the carriers departments; - The SPs did not realize they needed to do intra-SP ports prior to donating blocks; - The SPs did not have a process in place to notify the PA when the contamination status of a previously donated block goes from contaminated to non-contaminated; - Some SPs mistakenly believed that updating NRUF automatically updated the NPAC; and - Some SPs thought they could donate the block even though it was over 10% contaminated, if the numbers were ported to another carrier. | December 2006 |
| 100 Inter-modal Issue | It has been brought to the attention of the Local Number Portability Administration Working Group (LNPA WG) that a number of Service Providers participating in local number portability are failing to comply with the requirement that all simple wireline and intermodal port requests shall be confirmed by the Old Service Provider (OSP) within 24 hours, excluding weekends and holidays. | November 2006 | It is the LNPA WG’s position that the return of either the Firm Order Confirmation (FOC) in response to a valid Local Service Request (LSR), or an appropriate error message in response to an invalid LSR, by the Old Service Provider for a simple port request shall not exceed 24 hours, excluding weekends and holidays. | March 2007 |
| MISC. ISSUES |  |  |  |  |
| 101 (NANC 323) Inter-modal Issue | As a result of merger and acquisition activity, a method was needed to change the Service Provider ID (SPID) (code ownership) on NXX codes in the NPAC and service provider systems without temporarily deleting active records, which is a service-affecting process. | January 2001 | The LNPA WG developed a detailed and comprehensive process and functionality to perform mass SPID changes without sending traffic over the SOA and LSMS interfaces and without affecting customer service. | June 2004 |
| 102 Inter-modal Issue | The ATIS Network Interconnection Interoperability Forum (NIIF) and Network Inter-Operability Committee (NIOC), in their effort working with the TR45 standards group on Jurisdictional Information Parameter (JIP) population standards, requested that the LNPA WG review their proposed document. The JIP parameter is used for accurate billing and settlements between carriers. | May 2004 | The LNPA WG reached consensus on proposed modifications to the NIIF/NIOC document in order to remove draft text related to the assignment of JIP parameters per cell site since wireless companies do not have an NPA-NXX assigned at the cell site level. This was accepted by the NIIF/NIOC. | May 2004 |
| 103 Inter-modal Issue | An issue was raised at the June 2004 LNPA WG meeting regarding deleting NXX codes in the NPAC with active ported records. | June 2004 | The LNPA WG determined that NeuStar needs written authority from the appropriate State Regulatory Commission to delete the ported records. In response to a liaison from the LNPA WG, the INC added text to Section 3 of Appendix C of COCAG (INC Issue 434). | June 2004 |
| 104 Inter-modal Issue | The INC’s Central Office Code Administration Guidelines need to be revised to address change in code ownership in NPAC when a provider exits the market. | July 2004 | The LNPA WG submitted a contribution providing suggested text to INC for inclusion of LNP Change Order NANC 323 SPID Migration in the INC COCAG Appendix C. This contribution was accepted as part of INC Issue 445. | July 2004 |
| 105 Inter-modal Issue | A test plan and testing coordination is required to ensure the timely and successful implementation of WICIS Release 3.0. The WICIS standard is used for inter-carrier communication for wireless-wireless ports and for the wireless side of inter-modal ports. | August 2004 | The LNPA WG formed the Wireless Testing Subcommittee which developed the WICIS Release 3.0 test plan and coordinated vendor and service provider testing, which resulting in the successful rollout of the release. | February 2006 |
| 106 Inter-modal Issue | An issue was raised at the December 2004 LNPA meeting related to the portability of a Type 1 number, where the wireless carrier is required to port, but the underlying wireline carrier for the Type 1 number has a waiver. | December 2004 | It was the consensus of the group that due to technical issues, e.g., inability to perform default queries, code not opened in NPAC or marked portable in the LERG, without LNP capability, these numbers cannot be ported or migrated. The LNPA WG reported on this at the January 2005 NANC meeting. | January 2005 |
| 107 Inter-modal Issue | In a liaison to the LNPA WG, the INC asked for input and comments on the Department of Navy’s request for a dedicated NPA. | January 2005 | In response to the INC liaison, the LNPA WG developed a series of questions/comments/issues that were forwarded to the Department of Navy for their consideration. | January 2005 |
| 108 Inter-modal Issue | At the request of a wireline carrier, the LNPA WG was requested to address instances where contaminated TNs within a pooled 1K block have the same routing data as the pooled block itself, thus resulting in unnecessary ported records. | April 2005 | The LNPA WG reached consensus on suggested language for proposal to the INC to be added in their Thousands Block Pooling Administration Guidelines (TBPAG). INC Issue 484 was finalized in November 2005. | November 2005 |
| 109 Inter-modal Issue | An issue was raised at the June 2005 LNPA WG meeting regarding the voluntary transfer of NXX codes between service providers in order to be able to assign an LRN. | June 2005 | The LNPA WG reached consensus to send a liaison to the INC proposing technical considerations for the possible transfer of an NXX code from one provider to another for the purpose of assigning an LRN. The liaison was accepted by the INC (INC Issue 483). The LNPA WG and INC worked cooperatively to finalize the text for the INC’s Central Office Code Administration Guidelines (COCAG). | October 2005 |
| 110 Inter-modal Issue | In July 2005, the LNPA WG received a liaison from the INC requesting answers to specific questions related to NPAC block effective dates for the activation, modification, and disconnect of pooled 1K blocks. | July 2005 | The LNPA WG provided detailed responses to the INC’s questions in order to assist in the resolution of INC Issue 477. | November 2005 |
| 111 Inter-modal Issue | In order to affect a change in NXX code ownership in the NPAC, it may be more prudent and cost effective to perform a delete and add process if there are minimal ported number records in the NXX code. | July 2005 | The LNPA WG reached consensus on text to propose to the INC, for inclusion in Appendix C of their Central Office Code Administration Guidelines (COCAG), for determining when a coordinated industry effort for an NXX code ownership change (delete and add of ported TNs) in the NPAC may be preferable to the NANC 323 SPID migration process. This became INC Issue 482. | September 2005 |
| 112 Inter-modal Issue | At its September 2005 meeting, the NANC directed the LNPA WG to research and develop a detailed report on the impacts of porting outside the LATA in response to Hurricane Katrina. | September 2005 | The LNPA WG completed its interim report in November 2005 and submitted it to NANC. The final report was completed and submitted to NANC in April 2006. Both reports were forwarded to the FCC. | April 2006 |
| 113 Inter-modal Issue | The industry needs to ensure that all providers connected to the NPAC can quickly and seamlessly failover to the backup NPAC in the event of primary NPAC failure to maintain porting capability. | October 2005 | The LNPA WG developed a detailed and comprehensive annual failover exercise that tests and verifies all providers’ ability to failover and back their circuits connected to the primary and backup NPACs, and their applications and databases. | October 2005 |
| 114 Inter-modal Issue | At the November 2005 LNPA WG meeting, a provider presented an issue where a customer was being denied the ability to port to that provider because the customer had a Remote Call Forwarding (RCF) arrangement where there number was forwarded to a number in a different Rate Center. | November 2005 | The consensus of the LNPA WG was that the complaining provider should be allowed to provide the same service, RCF or FX, to this customer as the current provider does. Furthermore, the consensus was that the customer should be allowed to port their number to the provider in this scenario. The rating characteristics of the number were not changing in this case and it was the view of the group that this is not porting across Rate Center boundaries based on the stated plans of the provider to serve this customer. | November 2005 |
| 115 Inter-modal Issue | The block information for pooled blocks that were allocated back to the donating provider and switch was built into the NPAC, but the blocks were not being activated in NPAC because there was no need for LRN routing for these blocks. When the –X (block information) is created in NPAC and the block not activated, porting is prevented until it is manually removed. | March 2006 | The LNPA WG and INC worked cooperatively to revise the Part 1B pooled block form to support and clarify when the block information should not be created in NPAC. This was INC Issue 504. | September 2006 |
| 116 Inter-modal Issue | An issue was raised by a wireline carrier related to pooled blocks being allocated to providers and associated by that provider with an LRN whose NPA-NXX was not yet active in the network. This results in call routing failures. | June 2006 | INC Issue 527 addressed blocks with effective dates earlier than the effective date of the NPA-NXX of the associated LRN. At the suggestion of the LNPA WG, a footnote was placed on the Part 1A form for SPs to make sure the NPA-NXX of an LRN associated with a 1K block is effective in the network. | November 2006 |
| 117 Inter-modal Issue | INC Issue 496 addressed codeholders changing the rate center of an NXX code. The INC requested that the LNPA WG provide input on the process for affecting this change. | July 2006 | The LNPA WG communicated to the INC that the COCAG guidelines should reflect that the code holder will remove the code from NPAC if no SVs exist until the Rate Center change is effective, and then open it back up in NPAC after it is effective. This will prevent the Rate Center change from affecting other providers’ customers, which would be the case should porting take place prior to the effective date of the change. | July 2006 |
| 118 Inter-modal Issue | In a liaison to the LNPA WG, the INC asked for input and comments on a NIIF issue related to the need for call-through testing of allocated 1K blocks (INC Issue 528). | November 2006 | LNPA WG, INC, NIIF, and CIGRR leadership held a joint call in December 2006 to discuss this issue. As a result of the call, a contribution was submitted to INC to add consistent language in the COCAG and TBPAG around code testing. | December 2006 |
| 119 Inter-modal Issue | In a liaison to the LNPA WG, the NRRIC NIIF group requested review and comment on the LNP Section of their Network and Routing Resources Educational Document: Intercompany Responsibilities in the Telecommunications Industry. This document was being revised through the work of NIIF Issue #0253 in the NRRIC. | November 2006 | The LNPA WG sent a liaison to the NRRIC suggesting clarifying revisions to the LNP Section of the Network and Routing Resources Educational Document: Intercompany Responsibilities in the Telecommunications Industry, as agreed upon by the LNPA WG. | January 2007 |
| 120 Inter-modal Issue | A test plan and testing coordination is required to ensure the timely and successful implementation of WICIS Releases 3.1 and 4.0. The WICIS standard is used for inter-carrier communication for wireless-wireless ports and for the wireless side of inter-modal ports. | November 2006 | The LNPA WG restarted the Wireless Testing Subcommittee which is developing the WICIS Release 3.1 and 4.0 test plans and schedule for coordinated vendor and service provider testing. | Work is ongoing. |
| 121 Inter-modal Issue | At the request of a number of providers, a test to verify the industry’s ability to process up to 10,000 mass updates of TNs in an hour via the NPAC was proposed. | November 2006 | The LNPA WG developed a detailed end-to-end test from the NPAC down to the network element and successfully verified the industry’s ability to process 10,000 updates in an hour in each of the 7 U.S. NPAC regions. | February 2007 |
| 122 Inter-modal Issue | At the January 2007 LNPA WG meeting, a request was raised from a number of smaller wireless carriers to designate their pre-paid wireless ported records with an SV Type of 4, in order to assist Law Enforcement. | January 2007 | At their March 2007 meeting, the LNPA WG approved the request and instructed NeuStar to designate SV Type 4 as Pre-Paid Wireless in an upcoming maintenance window. | March 2007 |

STATISTICS:

ISSUES SUBMITTED:									122

ISSUES RESOLVED:					  				100

ISSUES REMAINING OPEN OR UNRESOLVED:	  				  22

ISSUES PENDING LONGER THAN 2 YEARS:		13

ISSUES PENDING BETWEEN 1 AND 2 YEARS:		  5

ISSUES PENDING LESS THAN 1 YEAR:			  4

PERCENTAGE OF ISSUES RESOLVED:			  				  82%

NOTE: Inter-modal porting refers to wireline-to-wireless porting and wireless-to-wireline porting.  The term “Inter-modal Issue” refers to an issue that, while not necessarily exclusive to inter-modal porting, could impact inter-modal porting through its resolution or lack thereof.  For example, an inter-modal issue could have an impact operationally on an inter-modal port or impact the data population of an inter-modal ported record.