Origination Date:  4/28/03

Originator:  NeuStar

### Change Order Number:  TBD

Description:  “Port-Protection” System

Pure Backwards Compatible:  TBD

IMPACT/CHANGE ASSESSMENT

| FRS | IIS | GDMO | ASN.1 | ##### NPAC | SOA | LSMS |
| --- | --- | --- | --- | --- | --- | --- |
| TBD | TBD | TBD | TBD | TBD | TBD | N/A |

Overview:

The “Port Protection” system is a competitively neutral approach to preventing inadvertent ports that gives end-users the ability to define their portable telephone numbers as “not-portable.”  The NPAC SMS enforces the “not-portable” status of a telephone number so long as it remains in effect.  No Local Service Provider (LSP) can invoke or revoke “port protection” on a working telephone number; end-users completely control the portability of their portable telephone numbers.

Business Need:

Inadvertent porting of working numbers is a concern to both Local Service Providers (LSPs) and their customers.  In today’s LNP environment, an LSP cannot absolutely assure its customers that their terminating service will not be interrupted, even if it can insure that physical plant is operated without failure.  This is because any LSP by mistake may port a telephone number away from that number’s current serving switch.

The inadvertent port can occur in a number of ways, but the most common occurrences appear to be caused by two errors: (1.) when the wrong telephone number submitted to NPAC for a conventional inter-SP port, and (2.) when intra-SP ports are not done before a pooled block is created.  There is a similar inadvertent port problem for non-working numbers, but erroneous moves of non-working numbers are not directly service-affecting and are not addressed here.

NeuStar suggests the following competitively neutral method to prevent inadvertent ports of working numbers.

Description of Change:

System Architecture

Changes to the NPAC SMS are required, to establish a table of “Port-Protected TNs” in which portable numbers that no longer can be ported are listed.  A step must be added to the NPAC SMS’s validation process in order to check this new table whenever an inter-SP port or pooled block create is attempted.  An interface change could be required as well if industry wishes to know when a request’s rejection is due to the involved number being on the “Port Protection” list.

Creation of an IVR system is required, to receive end-user requests for protection of their numbers from porting (or to remove this protection) and to relay the information to the NPAC SMS.  The system would automatically modify the NPAC’s “Port-Protection” tables based on the end-user requests it receives.  Access to the IVR would be through the end-user’s current LSP customer rep.  Any other LSP willing to assist the end-user could be involved.

System Operation

The end-user’s telephone number is entered in the NPAC’s “Port Protection” tables whenever “port-protection” is requested.  The end-user cannot reach the “Port-Protection” IVR system directly, but instead must be connected through a local Service Provider’s customer contact system, much like what is done in the PIC selection process, where the Service Provider’s customer rep advances the call to a third-party verification service, then leaves the call to allow the third-party verifier and end-user to converse.

The IVR system must recognize the LSP as authorized to participate in the “Port Protect” process.  (The LSP need not be a facility-based provider.)

Arrangements for security handshakes must be made in advance with each participating LSP.

A telephone number may be added to or removed from the “Port Protection” list whenever and as often as the end-user wishes.

To maintain the proposal’s competitive neutrality, the process assumes any LSP may assist the end-user.  However, the possibility of end-users invoking or revoking “Port Protection” on telephone numbers other than their own would be mitigated if only an LSP with which the end-user had a contractual relationship could particpate, i.e., only the current LSP or a new LSP in a pending port request situation.

When the NPAC attempts to create a pending SV or a pooled block, the NPAC will check the “Port Protection” list in its validation process for inter-SP port (including Port-to-Original) and “-X” create requests.

The “Port Protection” validation does not occur for intra-SP ports.  These may represent inadvertent ports, but validation necessary to determine whether override would be appropriate is not feasible.  The validation occurs for only those deletes that are “Port-to-Original” situations.

Process Flow

The end-user contacts an LSP (or an LSP contacts the end-user).  (It is not inherently necessary for there to be Service Provider involvement in this process, but NeuStar is not prepared to operate a system which does not involve LSP participation.)

End-user indicates desire to invoke (or revoke) “Port Protection.”

LSP customer rep places end-user on hold and calls the “Port-Protection” IVR.

LSP provides its pre-assigned ID information to IVR system.  (LSP arrange for security codes before attempting to assist end-users with the “Port-protection” process.)

LSP brings end-user on to the active line and leaves call; end-user interacts with IVR.

Using a standard script, the IVR confirms caller is authorized to make changes to the telephone number account, determines the caller’s name, and lists the telephone number(s) to be added to (or removed from) the “port-protection” table.  The customer may actually enter the TN desired.  The call is recorded.

The IVR system then enters this information into an automated ticket system.

Completion of the ticket automatically sends triggers an update of the NPAC’s “port-protection” table.

In the case of a number that has been entered in the port-protection table, but is no longer assigned to an end-user, the current Service Provider itself can ask that the number be removed from the “port-protection” table.  The provider would have to be recognized by the NPAC as the code/block owner and would have to state that the number is not assigned to an end-user.

It is appropriate to prevent the creation of a pooled block if any non-ported number in the block is “port-protected” since to allow the block’s creation would result in an inadvertent port of these numbers if the block eventually is assigned to another switch.  But the intra-SP porting activity required before creating a contaminated block must be allowed to occur without requiring end-users to temporarily lift the port restrictions on their numbers.  It therefore appears that an exception to the port protection validation is required, to allow a protected number to be intra-SP ported even if the number is “Port Protected.”  Without network data that is unavailable to NPAC today, the NPAC could not reliably determine whether an intra-SP port maintains the telephone number’s association with the same switch from which the number was served before the intra-SP port occurred.  A reasonable compromise appears to suppress the “Port-Protect” check when validating intra-SP ports rather than develop an elaborate validation process to address this scenario more completely.

A modify of an active SV’s or block’s LRN can result in the move of a telephone number to a different switch and thus could result in an inadvertent port.  NeuStar is not proposing the “Port Protect” validation be applied to Modify actions because of the complexity of such validation.