NANC CHANGE ORDER SUMMARY

FOR

NPAC SMS FUNCTIONALITY

Rev: 122
to be used for Jul 2007 (Monterey) meeting

07/01/07

Table of Contents

Open Change Orders	3

Accepted Change Orders	3

Next Documentation Release Change Orders	3

Next Release (TBD) Change Orders	3

Cancel – Pending Change Orders	3

Current Release Change Orders	3

Summary of Change Orders	3

# Open Change Orders

| Open Change Orders |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chg Order # | Orig. / Date | Description | Priority | Category | Proposed Resolution | Level of Effort |  |
|  |  |  |  |  |  | NPAC | SOA LSMS |
| NANC 372 | Bellsouth 11/15/02 | SOA/LSMS Interface Protocol Alternatives Business Need: Currently the only interface protocol supported by the NPAC to SOA and NPAC to LSMS interface is CMIP. The purpose of this change order is to request analysis be done to determine the feasibility of adding other protocol support such as CORBA or XML. The primary reasons for looking into a change would be 1) Performance, and 2) Implementation complexity. (continued) |  |  | Func Backwards Compatible: TBD Dec ’02 LNPAWG, discuss this change order in January ’03 in the new arch review meeting. | TBD | TBD / TBD |
| - NANC 372 (con’t) | Jan ’03 APT, discussion: - The team began with a discussion on the CMIP Alternative Business Need in order to determine if we need to improve CMIP or identify an alternative. - Dave Cochran, BellSouth and the originator of NANC Change Order 372, discussed potential drivers and cited: - Cost of maintaining internal CMIP interface expertise and resources - Ability to take advantage of in-house expertise for some of the newer architectures, e.g., CORBA, XML, JAVA, J2E - It was stated that CMISE was considered a reasonable protocol for managing network elements in the mid-1990s due to its flexibility. - LNP rules include encryption/decryption functionality. We need to discuss authentication and associated issues. - It was mentioned that if lowering the level of encryption is identified as a benefit for a new protocol, we should also consider that for CMIP. - CMIP is a very robust protocol for describing and managing network elements, but where that robustness begins to become burdensome is subjective. - We need to keep in mind that we need a real-time interface. Feb ’03 APT, discussion: Dave Cochran, BellSouth, will be providing more input (business drivers, data, operational feedback, etc.) to facilitate further discussion. Sub-tasks still need to be prioritized. Dec ’03 APT, discussion: No further discussion at this time. Leave off list of change orders discussed during the APT meeting. Jan ’07 APT, discussion: The APT was activated during the Nov ’06 LNPAWG meeting. No discussion on alternative interfaces took place during that meeting, but change orders (including 372) were reviewed during the Jan ’07 meeting. The brief discussion included: CMIP-to-XML/SOAP -- It was asked if there is a business need to transition from CMIP to XML/SOAP? It was suggested that since we are tunneling XML into CMIP, we should explore the future evolution of the interface. Service Providers are to discuss internally any drivers for moving from CMIP to XML/SOAP for the SOA and LSMS interfaces including the impact of increasing the size of messages. Mar ’07 APT, discussion: More discussion took place regarding an additional NPAC interface using XML/SOAP. For the May ’07 meeting, Service Providers and vendors are to bring any additional data or information to share with the group. (continued) |  |  |  |  |  |  |
| - NANC 372 (con’t) | May ’07 APT, discussion: 1. The IT industry is generally moving towards an XML/SOAP interface. However, there are performance issues and questions. Message size would be greatly increased. Need to investigate compression capabilities. 2. It will be worth pursuing for the long term. Not sure what is next step. Need to find a business driver for pursuing this. 3. The WICIS transfer is planning on implementing a flash-cut to XML (Sep ’08). Plan is to continue to support CORBA interface for testing purposes only. Keep this in mind when planning the NPAC implementation. 4. The group will discuss more during the Jul ’07 mtg, including pros/cons analysis, LOE, and any input on the business case. |  |  |  |  |  |  |
| NANC 396 | LNPA WG 9/9/04 | NPAC Filter Management – NPA-NXX Filters Business Need: The existing NPAC Filter Management process only allows a filter to be applied for a particular NPA-NXX if that particular NPA-NXX has previously been opened within NPAC. The NPAC also supports the ability for a SOA/LSMS to manage their own filters over the CMIP interface. Using this method, however, SOA/LSMS administrators must still wait upon receipt of a new code opening from the NPAC to create a new filter for those cases where they do not want to receive any Subscription Versions for that NPA-NXX. Because of how the NPAC Filter Management process works in conjunction with the SOA/LSMS implementation options, SOA/LSMS administrators are manually unable to efficiently filter out unnecessary Subscription Versions based on NPA-NXX for the purpose of SOA/LSMS capacity management. As a result, unnecessary Subscription Versions are sent to a SOA/LSMS or an unnecessary amount of resources are spent by the end user monitoring NPA-NXX activity at the NPAC in real-time to ensure Subscription Versions that are not needed are indeed not being sent to their SOA/LSMS. An unnecessary amount of resources are also spent by the NPAC maintaining these filters for carriers. Alternatively, a SOA/LSMS could implement an automated mechanism to manage filters over the CMIP interface, based on a local database table (or file). This table (or file) would contain codes that the SOA/LSMS wishes to filter out. So, when a new code is opened in NPAC and broadcast to the SOA/LSMS, the automated mechanism could issue a new filter request to the NPAC over the CMIP interface. The issue with this approach is that it requires every SOA/LSMS (that wishes to use this functionality) to implement this feature. | TBD | FRS, IIS | Func Backwards Compatible: YES This Change order proposes that filters may be implemented for an NPA-NXX before it is entered into the NPAC or a filter should be able to be implemented at the NPA level to account for any NXX in a particular NPA, even before an NXX may exist under that NPA within NPAC. | N/A | N/A / N/A |
| - NANC 396 (con’t) | Proposed Solution (continued): Major points/processing flow/high-level requirements: - The NPAC will continue to support filters at the NPA-NXX level. - The NPAC will keep the existing edit rule where an NPA-NXX must already exist in the NPAC in order to create a filter for that NPA-NXX. - The existing NPA-NXX filters will continue to be supported for NPAC personnel to maintain, via the NPAC GUI, for a requesting Service Provider. - The existing NPA-NXX filters will continue to be supported across the CMIP interface. - The NPAC will add support of filters at the NPA level. - The NPAC existing “NPA-NXX must exist” edit rule will NOT apply when creating NPA filters. - The new NPA filters will be supported for NPAC personnel to maintain, via the NPAC GUI, for a requesting Service Provider. - The new NPA filters will be supported across the CMIP interface (same as the NPA-NXX filter is currently). - Once an NPA filter is added, all subordinate NPA-NXX filters will be deleted. - Existing filter functionality related to broadcasts will remain in the NPAC (i.e., the NPAC will NOT broadcast data to an LSMS that has a filter for a given NPA or NPA-NXX). - No modifications required to local systems (SOA, LSMS). - No tunable changes. - No report changes. |  |  |  |  |  |  |
| NANC 402 | Nextel 2/9/05 | Validate Code Owner (SPID) Before Opening Code Business Need: Refer to separate document. | TBD | TBD | Func Backwards Compatible: Yes |  |  |
| NANC 408 | T-Mobile 10/20/05 | SPID Migration Automation Change Business Need: Refer to separate document. | TBD | TBD | Func Backwards Compatible: Yes |  |  |
| NANC 413 | NeuStar 05/31/06 | Doc Only Change Order: GDMO The current documentation needs to be updated: 1. subscriptionVersionNewSP-Create ACTION. Behavior clarification (new text in bold). New service providers must specify valid values for the following attributes, when the service provider's "SOA Sv Type Data" indicator is TRUE, and must NOT specify these values when the indicator is set to FALSE or when the subscriptionPortingToOriginal-SPSwitch is FALSE (ignored if value set to TRUE): subscriptionSvType When the subscriptionPortingToOriginal-SPSwitch is FALSE (ignored if value set to TRUE) the new service provider may specify valid values for the following attributes: subscriptionEndUserLocationValue subscriptionEndUserLocationType subscriptionBillingId 2. subscriptionVersionModify ACTION. Behavior clarification (new text in bold). New service providers can only modify the following attributes for pending or conflict subscription versions, and when the subscriptionPortingToOriginal-SPSwitch is FALSE (ignored if value set to TRUE): |  | GDMO | Func Backwards Compatible: YES Correct the current documentation. | N/A | N/A / N/A |
| NANC 413 (con’t) | Doc Only Change Order: GDMO (continued) 3. Behavior clarification (new text in bold) for the following attributes: auditDiscrepancyVersionId, serviceProvLRN-ID, serviceProvNPA-NXX-ID, subscriptionAuditId, subscriptionVersionId, lsmsFilterNPA-NXX-ID, numberPoolBlockId, serviceProvNPA-NXX-X-ID. For the attribute actionId, this entire paragraph will be added. The NPAC SMS currently uses a 32-bit signed integer for the Naming ID Value. The maximum value is ([2**31] - 1) or 2.14B 2147483647 and the minimum value is -(2**31) or -2147483648. Rollover will take place when an ID of maximum value is incremented. The next ID value after the maximum of 2147483647 will be -2147483648. It is anticipated that all Service Providers will be able to successfully handle Naming ID Values up to this maximum within this range as well as rollover after the maximum value is reached. 4. Behavior clarification (new text in blue) for both the SV Type attribute (#153, shown below) and the Number Pool Block SV Type attribute (#155, not shown below, but same change): -- -- 153.0 Subscription Version SV Type -- subscriptionSvTypeBehavior BEHAVIOUR DEFINED AS ! This attribute is used to specify the subscription version type. The possible values are: 0 : wireline 1 : wireless 2 : VoIP 3 : voWiFi 4 : sv-type-4 prepaid-wireless 5 : sv-type-5 6 : sv-type-6 !; |  |  |  |  |  |  |
| NANC 413 (con’t) | Doc Only Change Order: GDMO (continued) 5. Behavior clarification (new text in blue) for the incorrect usage of >: -- -- 21.0 LNP NPAC Subscription Version Managed Object Class -- subscriptionVersionNPAC-Behavior-2 BEHAVIOUR DEFINED AS ! been returned. The subscription version linked replies will be sorted by TN and then by subscription version ID so a filter can be created to return the next set of data where the TN value is greater than or equal to the last TN returned plus one, OR the TN is equal to the last TN returned AND the subscription version id is greater than or equal to the last subscription version id returned plus one. (e.g., (TN >= 123-456-78901 OR (TN = 123-456-7890 AND ID >= 12345)) !; |  |  |  |  |  |  |
| NANC 414 | LNPA WG (from PIM 51) 11/14/06 | Validation of Code Ownership in the NPAC Business Need: Refer to separate document. | TBD | TBD | Func Backwards Compatible: TBD |  |  |
| NANC 415 | NeuStar 12/1/06 | SIP and H.323 URIs in the NPAC Business Need: Refer to separate document. | TBD | TBD | Func Backwards Compatible: YES |  |  |
| NANC 416 | LNPA WG 09/13/06 | BDD File for Notifications – Adding New Attributes Business Need: As indicated in NANC 412, doc-only FRS updates, two attributes are not included in the Notification BDD file, even though they are part of the actual notification that is sent to the SOA. With this change order (action item 0906-02), those two attributes will be added to the BDD file, Business Type and Timer Type for Object Creation Notifications, so that the CMIP notification and the BDD file are consistent. This change order would require development effort for both SOA systems and the NPAC. | TBD | FRS | Func Backwards Compatible: TBD |  |  |
| NANC 417 | Syniverse 12/18/06 | Provide record count(s) for BDD Files and Delta BDD Files Business Need: Refer to separate document. | TBD | FRS | Func Backwards Compatible: TBD |  |  |
| NANC 418 | Syniverse 12/18/06 | Add post-migration SV Count to LRN SMURF Files Business Need: Refer to separate document. | TBD | M&P | Func Backwards Compatible: YES | Low | N/A |
| NANC 420 | AT&T 3/31/07 | Doc-Only Change Order: FRS Updates Business Need: 1. Remove unnecessary page break in Table 0-1 Notation Key between RR and RX abbreviation description. Remove RR table entry described as “This is a requirement that was identified in a NPAC SMS release subsequent to 1.X.” – this description was erroneously added in version 3.0.0. The original RR description (last table entry), “This is a requirement that was identified as a new requirement for the system, during post-award meetings with the Illinois LCC.” – should remain (with correction of LCC to LLC). 2. Prepaid Wireless SV Type -- With the implementation of NANC 399 and SV Type, several placeholder values were set aside for future use. During the Mar ’07 LNPAWG mtg, it was agreed to begin using one of these placeholder values. In both the intro section (1.2.16) and the data model section (SV data model – table 3-6, and Number Pool Block data model – table 3-8), the text for “SV Type 4” should be replaced with “Prepaid Wireless”. |  |  | Func Backwards Compatible: Yes Update the current documentation to be consistent and reflect the current behavior. |  |  |
| NANC 421 | NeuStar 03/31/07 | ASN.1 Updates for Prepaid Wireless SV Type The current ASN.1 needs to be updated: 1. Prepaid Wireless SV Type. With the implementation of NANC 399 and SV Type, several placeholder values were set aside for future use. During the Mar ’07 LNPAWG mtg, it was agreed to begin using one of these placeholder values. The ASN.1 change is shown below: SVType ::= ENUMERATED { wireline (0), wireless (1), voIP (2), voWiFi (3), sv-type-4 prepaid-wireless (4), sv-type-5 (5), sv-type-6 (6) } |  |  | Func Backwards Compatible: YES Update the current documentation. |  |  |
| NANC 422 | NeuStar 6/30/07 | Doc-Only Change Order: IIS Updates Business Need: 1. Correct section 4.8, Subscription Version Queries, for the enhanced SV Query functionality over the SOA/LSMS interfaces. The text gives an example using the > operator. CMIP does not support >, so the reference text should be changed from “> value”, to “>= value + 1”, as shown below: All subscription versions where ((TN >= 303-555-01501) OR (TN = 303-555-0150 AND subscription version ID >= 12345). |  |  | Func Backwards Compatible: Yes Update the current documentation to be consistent and reflect the current behavior. | N/A | N/A / N/A |

# Accepted Change Orders

| - Accepted Change Orders |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chg Order # | Orig. / Date | Description | Priority | Category | Proposed Resolution | Level of Effort |  |
|  |  |  |  |  |  | NPAC | SOA LSMS |
| - NANC 147 | - AT&T - 8/27/97 | #### Version ID Rollover Strategy - Currently there is no strategy defined for rollover if the maximum value for any of the id fields (sv id, lrn id, or npa-nxx id) is reached. One should be defined so that the vendor implementations are in sync. Currently the max value used by Lockheed is a 4 byte-signed integer and for Perot it is a 4 byte-unsigned integer. - Sep 99 LNPA-WG (Chicago), since the version ID for all data is driven by the NPAC SMS, the rollover strategy should be developed by Lockheed. SPs/vendors can provide input, but from a high level, the requirement is to continue incrementing the version ID until the maximum ([2**31] –1) is achieved, then start over at 1 (Jan/Mar/May ’07 LNPAWG mtgs – it was mentioned that the reference here to “1” is confusing since that is not the decimal equivalent when a 32-bit number is rolled over, so instead of “1” the correct reference should say “minus [2**31] – 1”.), and use all available numbers at that point in time when a new version ID needs to be assigned (e.g., new SV-ID for a TN). - Dec ’05 comments: NeuStar provided a list of five record types that could have numbers that roll over (since they come across the interface). Local vendors have action item to determine if they will have a prob with numbers that come “out of order”. | - High | - FRS | - Func Backwards Compatible: NO - A strategy on how we look for conflicts for new version id’s must be developed as well as a method to provide warnings when conflicts are found. - Oct 98 LNPAWG (Kansas City), it was requested that we begin discussing this in detail starting with the Jan 99 LNPAWG meeting. Beth will be providing some information on current data for the ratio of SV-ID to active TNs (so that we can get a feel for how much larger the SV-ID number is compared to the active TNs). - Sep 99 LNPA-WG (Chicago), Lockheed will begin developing a strategy for this. - Jun 00 LNPA-WG (Chicago), AT&T analysis and calculation (using current and projected porting volumes) indicate that a need for a version ID rollover strategy is more than five years away. Therefore, this change order is removed from R5, and will be discussed internally by NeuStar technical staff. - Jul 00 LNPAWG: NeuStar will track the problem. It will be a NeuStar internal design. Change order to stay on open list for possible later Document Only changes. - Jan 06 LNPAWG: Moved to accepted. | - High | - High? / High? |
| - NANC 147 (con’t) |  |  |  |  | - Mar 06 LNPAWG: Action IDs and Audit IDs are now expected to rollover in 7 months in the SE Region. NANC 147 will document the rollover strategy. There will be no initiative to go to 64 bit IDs. - Sep 06 LNPAWG: Action IDs and Audit IDs are now expected to rollover in less than two (2) months in the SE Region. Since these numbers are really transaction numbers and are purged on a regular basis, reuse is not an issue. The rollover strategy is to begin at 1. No vendor reported an issue with this approach. (Jan/Mar/May ’07 LNPAWG mtgs – it was mentioned that the reference here to “1” is confusing since that is not the decimal equivalent when a 32-bit number is rolled over, so instead of “1” the correct reference should say “minus [2**31] – 1”. As discovered during industry testing in early 2007, some vendors did have a problem with this; these vendors plan to address the problem with software patches to their customers). - NANC 147 is still needed to document the rollover strategy for long-term data (like SV-ID), where an inventory of available numbers needs to be established. At last check, this will be needed in ~850 months. NeuStar will continue to monitor the usage of SV-IDs. |  |  |
| NANC 355 | SBC 4/12/02 | Modification of NPA-NXX Effective Date (son of ILL 77) Business Need: When the NPAC inputs an NPA Split requested by the Service Provider and the effective date and/or time of the new NPA-NXX does not match the start of PDP, the NPAC cannot create the NPA Split in the NPAC SMS. To correct this problem the NPAC can contact the Service Provider and have them delete and re-enter the new NPA-NXX specified by the NPA Split at the correct time, or the NPAC can delete and re-enter the NPA-NXX for the Service Provider. However, the NPA-NXX may already be associated with the NPA Split at the Local SMS, and the subsequent deletion of the NPA-NXX will cause that specific record to be old time-stamped. When the NPA-NXX is re-created, that new record will have a different time stamp, and it requires a manual task for the Service Provider to search for new NPA-NXX records which might match the NPA Split. If identified and corrected, it will be added. If not identified, it will affect call routing after PDP. |  | FRS, IIS, GDMO | Func Backwards Compatible: NO This activity would only be allowed by NPAC personnel, via the GUI, to modify the NPA-NXX Effective Date. At the time of modification request, all existing pending subscription versions must have a due date greater than the new effective date in order for the change to occur. If one or more pending subscription versions have a due date less than the new effective date, a change would not be made and an error message would be returned to the NPAC user. It would be the responsibility of the owner of the NPA-NXX to resolve issues of pending versions with due dates prior to the new effective date before a change could be made. For valid requests, the NPAC will notify the SOA/LSMS of a modified effective date (M-SET). Jan ’03 LNPAWG, approved, move to accepted category. | Med-Low | TBD / TBD |
| NANC 382 | NeuStar 4/4/03 | “Port-Protection” System - (The following is the original request. Subsequent modifications were made during several LNPAWG meetings. Refer to the bottom of this change order for the current version.) Overview: The “Port Protection” system is a competitively neutral approach to preventing inadvertent ports that gives end-users the ability to define their portable telephone numbers as “not-portable.” The NPAC SMS enforces the “not-portable” status of a telephone number so long as it remains in effect. No Local Service Provider (LSP) can invoke or revoke “port protection” on a working telephone number; end-users completely control the portability of their portable telephone numbers. Business Need: Inadvertent porting of working numbers is a concern to both Local Service Providers (LSPs) and their customers. In today’s LNP environment, an LSP cannot absolutely assure its customers that their terminating service will not be interrupted, even if it can insure that physical plant is operated without failure. This is because any LSP by mistake may port a telephone number away from that number’s current serving switch. The inadvertent port can occur in a number of ways, but the most common occurrences appear to be caused by two errors: (1.) when the wrong telephone number submitted to NPAC for a conventional inter-SP port, and (2.) when intra-SP ports are not done before a pooled block is created. There is a similar inadvertent port problem for non-working numbers, but erroneous moves of non-working numbers are not directly service-affecting and are not addressed here. NeuStar suggests the following competitively neutral method to prevent inadvertent ports of working TNs. | TBD | FRS, IIS, GDMO, ASN.1 | Interface and Functional Backwards Compatible: NO Description of Change: - (The following is the original request. Subsequent modifications were made during several LNPAWG meetings. Refer to the bottom of this change order for the current version.) See next page. | TBD | TBD / TBD |
| NANC 382 (con’t) | Continuation of NANC 382, Port-Protection System, Proposed Resolution section: -- System Architecture -- Changes to the NPAC SMS are required, to establish a table of “Port-Protected TNs” in which portable numbers that no longer can be ported are listed. A step must be added to the NPAC SMS’s validation process in order to check this new table whenever an inter-SP port or pooled block create is attempted. An interface change could be required as well if industry wishes to know when a request’s rejection is due to the involved number being on the “Port Protection” list. Creation of an IVR system is required, to receive end-user requests for protection of their numbers from porting (or to remove this protection) and to relay the information to the NPAC SMS. The system would automatically modify the NPAC’s “Port-Protection” tables based on the end-user requests it receives. Access to the IVR would be through the end-user’s current LSP customer rep. Any other LSP willing to assist the end-user could be involved. The end-user’s telephone number is entered in the NPAC’s “Port Protection” tables whenever “port-protection” is requested. The end-user cannot reach the “Port-Protection” IVR system directly, but instead must be connected through a local Service Provider’s customer contact system, much like what is done in the PIC selection process, where the Service Provider’s customer rep advances the call to a third-party verification service, then leaves the call to allow the third-party verifier and end-user to converse. The IVR system must recognize the LSP as authorized to participate in the “Port Protect” process. (The LSP need not be a facility-based provider.) Arrangements for security handshakes must be made in advance with each participating LSP. A telephone number may be added to or removed from the “Port Protection” list whenever and as often as the end-user wishes. To maintain the proposal’s competitive neutrality, the process assumes any LSP may assist the end-user. However, the possibility of end-users invoking or revoking “Port Protection” on telephone numbers other than their own would be mitigated if only an LSP with which the end-user had a contractual relationship could participate, i.e., only the current LSP or a new LSP in a pending port request situation. (con’t) |  |  |  |  |  |  |
| NANC 382 (con’t) | Continuation of NANC 382, Port-Protection System, Proposed Resolution section: -- System Operation -- The end-user’s telephone number is entered in the NPAC’s “Port Protection” tables whenever “port-protection” is requested. The end-user cannot reach the “Port-Protection” IVR system directly, but instead must be connected through a local Service Provider’s customer contact system, much like what is done in the PIC selection process, where the Service Provider’s customer rep advances the call to a third-party verification service, then leaves the call to allow the third-party verifier and end-user to converse. The IVR system must recognize the LSP as authorized to participate in the “Port Protect” process. (The LSP need not be a facility-based provider.) Arrangements for security handshakes must be made in advance with each participating LSP. A telephone number may be added to or removed from the “Port Protection” list whenever and as often as the end-user wishes. To maintain the proposal’s competitive neutrality, the process assumes any LSP may assist the end-user. However, the possibility of end-users invoking or revoking “Port Protection” on telephone numbers other than their own would be mitigated if only an LSP with which the end-user had a contractual relationship could participate, i.e., only the current LSP or a new LSP in a pending port request situation. When the NPAC attempts to create a pending SV or a pooled block, the NPAC will check the “Port Protection” list in its validation process for inter-SP port (including Port-to-Original) and “-X” create requests. The “Port Protection” validation does not occur for intra-SP ports. These may represent inadvertent ports, but validation necessary to determine whether override would be appropriate is not feasible. The validation occurs for only those deletes that are “Port-to-Original” situations. (con’t) |  |  |  |  |  |  |
| NANC 382 (con’t) | Continuation of NANC 382, Port-Protection System, Proposed Resolution section: -- Process Flow -- The end-user contacts an LSP (or an LSP contacts the end-user). (It is not inherently necessary for there to be Service Provider involvement in this process, but NeuStar is not prepared to operate a system which does not involve LSP participation.) End-user indicates desire to invoke (or revoke) “Port Protection.” LSP customer rep places end-user on hold and calls the “Port-Protection” IVR. LSP provides its pre-assigned ID information to IVR system. (LSP arrange for security codes before attempting to assist end-users with the “Port-protection” process.) LSP brings end-user on to the active line and leaves call; end-user interacts with IVR. Using a standard script, the IVR confirms caller is authorized to make changes to the telephone number account, determines the caller’s name, and lists the telephone number(s) to be added to (or removed from) the “port-protection” table. The customer may actually enter the TN desired. The call is recorded. The IVR system then enters this information into an automated ticket system. Completion of the ticket automatically sends triggers an update of the NPAC’s “port-protection” table. In the case of a number that has been entered in the port-protection table, but is no longer assigned to an end-user, the current Service Provider itself can ask that the number be removed from the “port-protection” table. The provider would have to be recognized by the NPAC as the code/block owner and would have to state that the number is not assigned to an end-user. |  |  |  |  |  |  |
| Continuation of NANC 382, “Port-Protection” System - This change order was reviewed and revised during the May through Sep ’03 LNPAWG meetings. The final version of the open change order at the time of acceptance (for development of more detailed information) is shown below: Overview: The “Port Protection” system is a competitively neutral approach to preventing inadvertent ports. The system makes it possible for end-users to define their portable telephone numbers as “not-portable.” The NPAC SMS prevents the port of a “not-portable” telephone number (TN) through its automated validation processes. A Local Service Provider (LSP) can invoke or revoke “port protection” for a working TN, but only at the end-user’s request. Business Need: Inadvertent porting of working TNs is a concern to both Local Service Providers (LSPs) and their customers. In today’s LNP environment, an LSP cannot absolutely assure its customers that their terminating service will not be interrupted, even if it can insure that the physical plant is operated without failure. This is because another LSP by mistake may port a TN away from that number’s current serving switch. The inadvertent port can occur in a number of ways, but the most common occurrences appear to be caused by two errors: (1.) the wrong TN is submitted to the NPAC SMS for a conventional inter-SP port, and (2.) intra-SP ports are not done before a thousands-block is created. There are similar inadvertent port scenarios for non-working TNs, but erroneous moves of non-working TNs are not immediately service-affecting and are not addressed here. NeuStar suggests the following competitively neutral method to prevent inadvertent ports of working TNs. |  |  |  | Interface and Functional Backwards Compatible: NO - This change order was reviewed and revised during the May through Sep ’03 LNPAWG meetings. The final version of the open change order at the time of acceptance (for development of more detailed information) is shown below: Description of Change: -- System Architecture -- Changes to the NPAC SMS are required to establish a table of “Port Protected” TNs, in which portable numbers that no longer can be ported are listed, and to add a validation step that rejects attempts to port a TN that is on the list. The validation is performed on the new-SP’s Create message for an inter-SP port, when a thousands block is created, and, optionally, for an intra-SP port. (The optional intra-SP port validation is invoked on a SPID-specific basis.) The rejection notification sent when a request fails this NPAC SMS validation will indicate that the TN is on the Port Protection list. No interface change is required for this rejection message, since a new optional attribute will be added to accommodate the new error text. LSP requests to add TNs to the Port Protection table are made to the NPAC Help Desk via e-mail (the TNs involved are shown on an Excel attachment to the e-mail message). LSPs use the same approach to delete TNs from the table. (con’t) |  |  |  |
| NANC 382 (con’t) | Continuation of NANC 382, Port-Protection System, Proposed Resolution section: -- System Operation -- A TN is added to the NPAC’s Port Protection table when an LSP requests this action. The same process applies when an LSP requests the removal of a TN from the table. The NPAC Help Desk accepts requests to change Port Protection table entries only from pre-authorized representatives of an LSP. (The LSP need not be a facility-based provider.) A TN may be added to or removed from the “Port Protection” list as often as required. When the NPAC SMS receives the new SP’s Create request, it will check the Port Protection table during the Pending SV Create validation process for inter-SP ports (including Port-to-Original SV deletes). Optionally, the validation is performed for intra-SP ports. The NPAC SMS also will make this validation check in connection with “-X” create requests. The validation is not applied to Modify requests In the disconnect scenario, the NPAC SMS will check the Port Protection list and, if the TN is found, will remove the involved disconnected ported TN from the list. This automatic removal of a disconnected TN from the Port Protection list can occur only in the case of a disconnected TN that was ported. A non-ported TN that is disconnected must be removed from the list by the LSP having the disconnected non-ported TN in its inventory. (con’t) |  |  |  |  |  |  |
| NANC 382 (con’t) | Continuation of NANC 382, Port-Protection System, Proposed Resolution section: -- Process Flow -- NPAC Help Desk - The end-user contacts an LSP (or an LSP contacts the end-user). - End-user indicates to LSP his desire to invoke (or revoke) “Port Protection.” - LSP contacts NPAC Help Desk via e-mail to request change. - The NPAC Help Desk updates the Port Protection table. NPAC SMS - NPAC SMS applies the Port Protection validation (1.) to the new-SP Create request of an inter-SP port, (2.) to a Block Creation request, and (3.) optionally at the individual SPID level, to an intra-SP port request. If the TN is found on the Port Protection list, NPAC SMS rejects the request and indicates that a Port Protection validation failure is the reason for the request’s rejection. - Disconnect of a ported TN results in automatic removal of the TN from the Port Protection list; disconnect of a non-ported TN requires owning LSP to request the disconnected TN’s removal from the list. - An LSP’s regional NPAC SMS Profile indicates whether the Port Protection validation should be applied also to its intra-SP port requests. |  |  |  |  |  |  |
| 382 (cont) | - Nov ’03 LNPAWG, discussion: The group discussed the high-level steps. There were a couple of updates that were requested. These steps will be evaluated once the policy issues/questions are discussed: - For intra-ports, let the port go through and keep them on the list. - In steps 4.b, no need to look at the list, just allow the Old SP Create to happen. If they are on the list, then for now, leave it on the list. - For step 8, add that this does NOT apply to PTO. Policy issues/questions: (at the Jan ’04 LNPAWG, we would discuss if and how, we might Tee this up at NANC). - What types/classes of numbers can be placed on the list? What criteria? What kind of criteria. - Who can put it on the list and remove it from the list? This is an authorization question. - What is the PROCESS for getting them on and off the list? How mechanically, do you put/remove it on the list. - Who can access the list, need a process to access the list. What is shown when they access the list (police, other authority) Other points discussed: - Want more than just the IVR way to get numbers on/off the list. - Want some type of pre-validation process to “ping” the list and see if someone is on the PPL. - Want the ability to audit the list. |  |  |  |  |  |  |
| NANC 390 | Qwest 10/16/03 | New Interface Confirmation Messages SOA/LSMS – to - NPAC Business Need: Service Provider systems (SOA/LSMS) need to know (in the form of a positive acknowledgement from the NPAC) that the NPAC has received their request message, so the systems (SOA/LSMS) do not unnecessarily resend the message and cause duplicate transactions for the same request. Based on the current requirements for the NPAC, the NPAC acknowledgement message (generally referred to as "a response to a request" from the SOA/LSMS) is not returned until AFTER the NPAC has completed the activity required by that request. During heavy porting periods, transactions that require many records to be updated may take longer than normal for a response to be received from the NPAC. In the case of a delayed response, the SOA/LSMS may abort the association to the NPAC (e.g., after the 15 minute Abort timer expires). When the association is re-established, the SOA/LSMS may resend messages to the NPAC because they haven’t received a response to the first message and thus believe the NPAC did not receive the original message. This behavior can lead to a duplicate transaction for the same request thus: 1.) causing a heavy volume of transactions over the NPAC to SOA/LSMS interface, 2.) slowing Porting completion, 3.) causing an increase of Porting costs, 4.) causing duplicate message processing at the NPAC, and 5.) possibly causing manual intervention by NPAC and Service Provider personnel, etc. | TBD | FRS, IIS, GDMO, ASN.1 | Func Backwards Compatible: NO A new message will be explored during the Nov ’03 LNPAWG meeting. Additionally, a discussion item needs to occur regarding the possible inclusion of Service Provider profile settings to support this new feature. | N/A | N/A / N/A |
| NANC 390 (con’t) | - Nov ’03 LNPAWG, discussion: Explained the current functionality, and the fact that higher priority transactions will be worked before other requested work, which can cause delays in responses. In the case where previously submitted work was re-sent to the NPAC, the NPAC may have to re-do work it has already done. Providers may see a backup in their SOA traffic, thereby causing them to process extra data as well. A toggle would need to be added for backwards compatibility. Providers that support the new confirmation message would use the new method/flow, and other providers would continue to use the current method/flow. There is definitely a benefit to this, but to obtain the benefit would require changes to the SOA as well. It was agreed that this would be accepted as a change order, and would continue to be worked with the Architecture group in December. Feb ‘04 – Refer to the Architecture Planning Team’s working document for the latest information on this change order. |  |  |  |  |  |  |
| NANC 397 | Verizon Wireless and SNET Diversif’d Group 7/28/04 | Large Volume Port Transactions and SOA Throughput Overview: Service Providers have voiced concerns about the volume of port transactions that the NPAC can process per second when mass changes need to be made and broadcasted to the industry. Now that wireless service providers are porting throughout the United States, the volume of port transactions has increased and will continue to increase in general, and mass changes will need to be made more frequently as well. The consolidations of Carriers and Switches will also generate an increase in the number of Mass Modifications for the update of the Network Data Tables (LIDB, CNAM, CLASS, ISVM and SMSSC). Business Need: As wireless service providers are continually managing their networks and load-balancing the traffic and subscribers on them, the need for HLR and DPC database changes may become more frequent and of larger volumes in the future. For example, the wireless carrier may need to modify LRNs for 100,000 ported in subscribers to effectively change their switch designations. Ultimately, the NPAC must be able to handle those 100,000 transactions in a short amount of time. The desired process would be to modify all the records in one evening rather than having to split up the changes over a period of days or weeks. Similarly, Service Providers who have consolidated or have changed business plans need to update the Network Tables in order to ensure proper routing to Database Storage (LIDB, CNAM, etc.). (continued) | TBD | N/A | Func Backwards Compatible: YES The performance impacts to the SOAs, NPAC, and LSMSs need to be determined for large volume ports. As porting volumes increase, it will be very important for all systems to be capable of reliably receiving downloads while retaining their association under heavier loads. All systems should be able to maintain their current required availability level under heavy loads. Large volume porting should not require scheduled downtime. The current plan is for service providers to start compiling technology migration forecast estimates and provide this information to Steve Addicks by March ’05. At that time, the Architecture Team will begin a review of the data (without service provider names) and begin some analysis on next steps. | TBD | TBD / TBD |
| NANC 397 con’t | Intense coordination is required to effect the changes necessary to properly route the queries associated with these databases, including LERG, LARG and CNARG updates, GTT changes in STPs and end office routing changes. Additionally, modifications need to be made to the Network Tables in the NPAC and the transaction limitations force such modifications to be spread over weeks and/or months straining the resources of an industry already processing changes on a 24X7 basis. The two methods available for large volume NPAC changes are 1) modifications done through the SOA and 2) modifications done using the industry Mass Modification process. Processing through the SOA, at the current rate of 4 to 6 transactions per second, it could take more than 4 hours to make LRN changes to 100,000 subscribers. If something goes wrong and the Service Provider needs to back out of the changes, then another 4 hours would be required to make the corrections. This could start to creep into regular business hours in large volume ports. There is a concern about technology migrations and the current 25K/night operational limitation (originally submitted as PIM 43, and now turned into a change order). This is not an immediate need, but something that should be planned for the three-five years out timeframe. (May ’07 LNPAWG mtg – the following paragraph is retained for historical purposes, even though the quantity limitation on the industry Mass Modification notification process has been updated. The current value as of Mar ’07 is set to 10,000 changes per hour, per region, seven days a week). The industry Mass Modification process is limited to 25,000 changes per region per day Monday through Friday and 50,000 changes per region per day Saturday and Sunday. This limitation applies to all service providers requesting a change, so if more than one service provider wishes to make changes on a particular day, the limitation encompasses all service providers wishing to modify records. A wireless subscriber migration involves more than just that service provider; it also involves each of that service provider’s roaming partners updating their networks on the same night, resulting in a very large coordinated effort among many parties. There are also concerns about multiple wireless service providers doing these same types of migrations on the same nights and what coordination needs to take place to ensure that all service providers are able to manage their networks as needed and when needed. Using the Mass Modification method for large volume projects requires a high level of coordination and scheduling especially if other service providers in the region also need to do large modifications at the same time. Additional updates between the NPAC and the SOA may be needed using the Mass Modification process. This adds additional time and coordination to fully complete a large volume project. |  |  |  | Jan 06 – moved to Accepted per LNPAWG discussion. Jan, Mar 07 – continued discussion in Architecture Planning Team’s meeting. For the May meeting, the requirements will be included to reflect current values and new values that would be necessary for 25K/hr. (Continued next page) |  |  |
| NANC 397 con’t | Large Volume Port Transactions and SOA Throughput (Proposed Resolution section, continued) The current (Mar ‘07) industry Mass Modification notification process is set to 10,000 changes per hour, per region, seven days a week. Current requirements, NANC 393, FRS 3.3, downloads to the LSMS are 14,760/hr. Change bars indicate new numbers to support 25K/hr. R6-28.1 SOA to NPAC SMS interface transaction rates - sustained A transaction rate of 4.0 7.0 CMIP transactions (sustained) per second shall be supported by each SOA to NPAC SMS interface association. R6-28.2 SOA to NPAC SMS interface transaction rates - peak NPAC SMS shall support a rate of 10.0 CMIP operations per second (peak for a five minute period, within any 60 minute window) over a single SOA to NPAC SMS interface association. R6-29.2 NPAC SMS to Local SMS interface transaction rates - peak NPAC SMS shall, support a rate of 5.2 CMIP operations per second (peak for a five minute period, within any 60 minute window) over each NPAC SMS to Local SMS interface association. This requirement will be deleted. Therefore, the LSMS performance rate will be strictly a sustained rate. RR6-107 SOA to NPAC SMS interface transaction rates – total bandwidth NPAC SMS shall support a total bandwidth of 40.0 70.0 SOA CMIP transactions per second (sustained) for a single NPAC SMS region. (previously NANC 393, NewReq 1) RR6-108 NPAC SMS to Local SMS interface transaction rates – sustained NPAC SMS shall support a rate of 4.0 7.0 CMIP transactions per second (sustained) over each NPAC SMS to Local SMS interface association. (previously NANC 393, NewReq 2) |  |  |  |  |  |  |
| NANC 397 con’t | Large Volume Port Transactions and SOA Throughput (Proposed Resolution section, continued) Current requirements, NANC 393, FRS 3.3, downloads to the LSMS are 14,760/hr. Change bars indicate new numbers to support 25K/hr. RR6-109 NPAC SMS to Local SMS interface transaction rates – total bandwidth NPAC SMS shall support a total bandwidth of 156 210 Local SMS CMIP transactions per second (sustained) for a single NPAC SMS region. (previously NANC 393, NewReq 3) May 07 – continued discussion in Architecture Planning Team’s meeting. The updated requirements were reviewed. The performance increase would likely affect more than just software changes (i.e., hardware, network). When questioned again on the need to allow half the time for the backout, Verizon Wireless responded that a problem may not be known until the entire migration was completed, and therefore the back-out requirement would need a comparable time interval to perform the backout. NeuStar suggested an option that would use a new message to indicate “starting migration now”, and a subsequent message to indicate “migration complete” or “migration should be backed out”. This approach allows a potential to use much more of the maintenance window for the initial broadcast, since database backout or commits will be much faster than additional SV modification broadcasts. Discussion will continue during the Jul ’07 APT mtg. |  |  |  |  |  |  |
| NANC 400 | NeuStar 1/5/05 | URI Fields Business Need: Refer to separate document. | TBD | TBD | Func Backwards Compatible: Yes Dec 05 – moved to Accepted per LNPAWG discussion |  |  |
| NANC 401 | VeriSign 1/13/05 | Separate LSMS Association for OptionalData Fields Business Need: Refer to separate document. | TBD | TBD | Func Backwards Compatible: Yes Jan 06 – moved to Accepted per LNPAWG discussion |  |  |
| NANC 403 | NeuStar 3/30/05 | Only allow Recovery Messages to be sent during Recovery The current documentation does NOT specifically state that ALL recovery messages should only be sent to the NPAC during recovery (it is currently indicated for notifications and SWIM data). This change order will clarify the documentation to include ALL data. This will require some operational changes for Service Providers that utilize Network Data and/or Subscription Data recovery while in normal mode. | TBD | TBD | Func Backwards Compatible: Yes The proposed solution is to update the FRS, IIS and GDMO recovery description to indicate that network data and subscription data recovery requests sent during normal mode will be rejected. No sunset policy will be implemented with this change order. |  |  |
| NANC 403 (con’t) | Proposed Solution: FRS, new requirements: Req 1 All Data Recovery Only in Recovery Mode NPAC SMS shall allow a SOA or LSMS to recover data ONLY in recovery mode. Req 2 Recovery Restriction Tunable Parameter NPAC SMS shall provide a Regional Recovery Restriction in Recovery Mode Only tunable parameter which is defined as an indicator on whether or not the restriction of recovery requests only be allowed while in recovery mode is supported by the NPAC SMS for a particular NPAC Region. Req 3 Recovery Restriction Tunable Parameter Default NPAC SMS shall default the Regional Recovery Restriction in Recovery Mode Only tunable parameter to TRUE. Req 4 Recovery Restriction Tunable Parameter Modification NPAC SMS shall allow NPAC Personnel, via the NPAC Administrative Interface, to modify the Regional Recovery Restriction in Recovery Mode Only tunable parameter. IIS, section 5.2.1.9, add the following text: All recovery requests can only be sent to the NPAC when the SOA/LSMS is in recovery mode, otherwise an error message is returned (failed). IIS, section 5.3.4, change the following text: Service Provider and Notification All recovery requests can only be sent to the NPAC when the SOA/LSMS is in recovery mode, otherwise an error message is returned (failed). GDMO, lnpDownload notification, add the following text in the behavior section: All recovery requests can only be sent to the NPAC when the SOA/LSMS is in recovery mode, otherwise an error message is returned (failed). Dec 05 – moved to Accepted per LNPAWG discussion. |  |  |  |  |  |  |
| NANC 419 | AT&T 3/15/07 | User Prioritization of Recovery-Related Notifications Business Need: The existing NPAC Notification Priority process only allows a certain type of notification to have a different priority from another type. Using this method, however, SOAs cannot distinguish between the reason for a certain type of notification. For example, a Status Attribute Value Change notification could indicate that all LSMSs successfully responded and a pending SV is moving to active, or it could indicate that a discrepant LSMS has just completed recovery and a partial-failure SV is moving to active. As a result, an SP that is recovering SVs could cause the activating SOA to experience unintended delays in receiving notifications for different activities because the recovery process generates its own set of notifications. This unintended delay could happen hours after the initial activity, when the SOA is otherwise relatively lightly loaded, causing confusion to the SOA users. |  |  | Func Backwards Compatible: TBD Develop a mechanism that further defines certain notifications as initiated by regular activity versus recovery activity. With this change order the two instances would be differentiated, and an SP could indicate a different prioritization for one versus the other. May ’07 APT: The business need/scenario was explained during the APT meeting, with agreement from the group that the text captured the current business need. The group also agreed to recommend acceptance of this change order by the LNPAWG. The CMA will add additional text to this change order, then send out prior to the Jun ’07 LNPAWG con call, with a recommendation of approval from the APT. Example of current notification: Notification -- L-11.0 A1 SV SAVC Activates to new SP priority. Definition -- When an INTER or INTRA SV has been created in the Local SMSs (or ‘activated‘ by the SOA) and the SV status has been set to: Active or Partial-Failure. The notification is sent to both SOAs: Old and New. If the status has been set to Partial-Failure, this notification contains the list of Service Providers (SP) LSMSs that have failed to receive the broadcast. |  |  |
| NANC 419 (con’t) |  |  |  | Proposed change: Add a new scenario to the list of notification priorities (42 listed in the FRS, Appendix C). The new one will be specific to notifications generated as a result of recovery requests (not to be confused with notification recovery). This will allow notifications generated where the reason is recovery to have a lower priority than the same notification generated where the reason is a SOA GUI user working real-time with a customer request. In the example above, notification L-11.0 A1 would have a lower priority in a recovery-related SV activate scenario where one LSMS failed the initial SV activate download, but successfully recovered that SV activate download at a later time, whereas a different instance of notification L-11.0 A1 would have a higher priority in a regular SV activate scenario where all LSMSs successfully processed the SV activate download. Jun ’07 LNPAWG con call: The change order was accepted by the LNPAWG during the call. Detailed requirements will begin to be developed. |  |  |  |

# Next Documentation Release Change Orders

| Next Documentation Release Change Orders |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chg Order # | Orig. / Date | Description | Priority | Category | Proposed Resolution | Level of Effort |  |
|  |  |  |  |  |  | NPAC | SOA LSMS |

# Next Release (TBD) Change Orders

| - Next Release (TBD) Change Orders |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chg Order # | Orig. / Date | Description | Priority | Category | Proposed Resolution | Level of Effort |  |
|  |  |  |  |  |  | NPAC | SOA LSMS |

# Cancel – Pending Change Orders

| - Cancel - Pending Change Orders |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chg Order # | Orig. / Date | Description | Priority | Category | Proposed Resolution | Level of Effort |  |
|  |  |  |  |  |  | NPAC | SOA LSMS |

# Current Release Change Orders

| - Current Release Change Orders |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chg Order # | Orig. / Date | Description | Priority | Category | Proposed Resolution | Level of Effort |  |
|  |  |  |  |  |  | NPAC | SOA LSMS |
|  |  | See Implemented List for details on Release 3.3. |  |  |  |  |  |

# Summary of Change Orders

| Release # / Target Date | Change Orders | Backwards Compatible |
| --- | --- | --- |
| Open | NANC 372 – SOA/LSMS Interface Protocol Alternatives NANC 396 –NPAC Filter Management – NPA-NXX Filters NANC 402 – Validate Code Owner (SPID) Before Opening Code NANC 408 –SPID Migration Automation Changes NANC 413 – Doc Only Change Order: GDMO NANC 414 – Validation of Code Ownership in the NPAC NANC 415 – SIP and H.323 URIs in the NPAC NANC 416 – BDD File for Notifications – Adding New Attributes NANC 417 – Provide record count(s) for BDD Files and Delta BDD Files NANC 418 – Add post-migration SV Count to LRN SMURF Files NANC 420 – Doc Only Change Order: FRS NANC 421 – ASN.1 Updates for Prepaid Wireless SV Type |  |
| Accepted | NANC 147 – Version ID Rollover Strategy NANC 193 – TN Processing During NPAC SMS NPA Split Processing NANC 355 – Modification of NPA-NXX Effective Date (son of ILL 77) NANC 382 – “Port-Protection” System NANC 390 – New Interface Confirmation Messages SOA/LSMS – to - NPAC ion Version Creation and its Activation NANC 397 – Large Volume Port Transactions and SOA Throughput NANC 400 – URI Fields NANC 401 – Separate LSMS Association for OptionalData Fields NANC 403 –Only allow Recovery Messages to be sent during Recovery NANC 419 – User Prioritization of Recovery-Related Notifications |  |
| Next Documentation Release |  |  |
| Next Release |  |  |
| Cancel-Pending |  |  |
| Current Release | See Implemented List for details on R3.3 |  |

It is appropriate to prevent the creation of a pooled block if any non-ported number in the block is “port-protected” since to allow the block’s creation would result in an inadvertent port of these numbers if the block eventually is assigned to another switch.  But the intra-SP porting activity required before creating a contaminated block must be allowed to occur without requiring end-users to temporarily lift the port restrictions on their numbers.  It therefore appears that an exception to the port protection validation is required, to allow a protected number to be intra-SP ported even if the number is “Port Protected.”  Without network data that is unavailable to NPAC today, the NPAC could not reliably determine whether an intra-SP port maintains the telephone number’s association with the same switch from which the number was served before the intra-SP port occurred.  A reasonable compromise appears to suppress the “Port-Protect” check when validating intra-SP ports rather than develop an elaborate validation process to address this scenario more completely.

A modify of an active SV’s or block’s LRN can result in the move of a telephone number to a different switch and thus could result in an inadvertent port.  NeuStar is not proposing the “Port Protect” validation be applied to Modify actions because of the complexity of such validation.

The validation of intra-SP ports occurs only if the involved SP has indicated in its NPAC SMS profile that this validation is desired.

It is appropriate to prevent the creation of a pooled block if any non-ported number in the block is on the Port Protection list, since to allow the block’s creation would result in an inadvertent port of these numbers when (if) the block eventually is assigned to another switch.  But the intra-SP porting activity, necessary before creating a contaminated block, is allowed to occur without requiring that the port restrictions be lifted from TNs in the block.  This exception to the Port Protection validation is provided in order to allow a TN to be intra-SP ported even if the TN is on the Port Protection list.  The option to include intra-SP ports in the Port Protection validation process is provided at the individual LSP’s request.

A modify of the LRN in an active SV or block record also can result in the move of a telephone number to a different switch and thus could result in an inadvertent port.  However, NeuStar is not proposing the Port Protection validation be applied to Modify actions because of the complexity of such a validation.