NANC CHANGE ORDER SUMMARY

FOR

NPAC SMS FUNCTIONALITY

Rev: 105
to be used for November 2003 (Kansas City) meeting

11/01/03

Table of Contents

Open Change Orders	3

Accepted Change Orders	21

Next Documentation Release Change Orders	84

LTI Change Orders	90

Cancel – Pending Change Orders	91

Current Release Change Orders	92

MR Change Orders	93

Summary of Change Orders	94

# Open Change Orders

| Open Change Orders |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chg Order # | Orig. / Date | Description | Priority | Category | Proposed Resolution | Level of Effort |  |
|  |  |  |  |  |  | NPAC | SOA LSMS |
| - NANC 147 | - AT&T - 8/27/97 | #### Version ID Rollover Strategy - Currently there is no strategy defined for rollover if the maximum value for any of the id fields (sv id, lrn id, or npa-nxx id) is reached. One should be defined so that the vendor implementations are in sync. Currently the max value used by Lockheed is a 4 byte-signed integer and for Perot it is a 4 byte-unsigned integer. - Sep 99 LNPA-WG (Chicago), since the version ID for all data is driven by the NPAC SMS, the rollover strategy should be developed by Lockheed. SPs/vendors can provide input, but from a high level, the requirement is to continue incrementing the version ID until the maximum ([2**31] –1) is achieved, then start over at 1, and use all available numbers at that point in time when a new version ID needs to be assigned (e.g., new SV-ID for a TN). | - High | - FRS | - Func Backwards Compatible: NO - A strategy on how we look for conflicts for new version id’s must be developed as well as a method to provide warnings when conflicts are found. - Oct 98 LNPAWG (Kansas City), it was requested that we begin discussing this in detail starting with the Jan 99 LNPAWG meeting. Beth will be providing some information on current data for the ratio of SV-ID to active TNs (so that we can get a feel for how much larger the SV-ID number is compared to the active TNs). - Sep 99 LNPA-WG (Chicago), Lockheed will begin developing a strategy for this. - Jun 00 LNPA-WG (Chicago), AT&T analysis and calculation (using current and projected porting volumes) indicate that a need for a version ID rollover strategy is more than five years away. Therefore, this change order is removed from R5, and will be discussed internally by NeuStar technical staff. - Jul 00 LNPAWG: NeuStar will track the problem. It will be a NeuStar internal design. Change order to stay on open list for possible later Document Only changes. | - High | - High? / High? |
| NANC 340 | CMA 11/6/01 | Doc Only Change Order for IIS: Update Appendix A The information in Appendix A is out of date and needs to be updated. | Low | IIS | 11/14/01 – Reviewed at November 2001 LNPA WG. Waiting for feedback from NeuStar. 01/09/02 – This item has low priority. Change Order to remain in “open” status until updated information is provided by NPAC Systems Engineering. | N/A | N/A / N/A |
| NANC 349 | NeuStar 3/6/02 | Batch File Processing Business Need: Service Providers periodically generate large porting activity. The current definition includes ports with 500 or more TNs. The NPAC receives these large port requests via an online mechanism (CMIP interface or LTI), and processes them at that point in time. The current requirements do not allow for “off-line” processing of activity. As an alternative to generating all the messages associated with large porting activity, and sending them across a Service Provider’s CMIP interface, a batch mode can be implemented whereby a Service Provider can send a batch request to the NPAC, and request that it be processed after a certain date and time. With this change order, the NPAC and the Service Provider can offload processing that can be worked separately, but still meet the need to incorporate that work after a specified date and time. Since all large porting activity is known well in advance, both planning and processing can be addressed, thereby benefiting risk management. The functionality covered in this change order could be any activity that is not time critical and typically done over a 24 hour period (e.g., pooled blocks where not time sensitive, or an LSMS for DPC codes). | TBD | FRS | - Interface and Functional Backwards Compatible: YES The NPAC would incorporate an offline batch processing engine that handles batch requests from a requesting Service Provider. The Service Provider would place the request in their ftp site directory. The NPAC would periodically scan for requests, pick them up, and process them offline. After reaching the Service Provider’s requested date and time, the request would become “active” and the NPAC would process this request during off hours (e.g., during nightly housekeeping). Upon completion, the requested activity would be incorporated into the production database. Updates or notifications could be either placed in a response file at the Service Provider’s ftp site directory, or sent across the interface to the Service Provider. A new indicator would be added to the customer profile record. This would indicate whether the Service Provider supports batch processing. If yes, any batch requests would be responded back to the Service Provider in batch mode, via a “processing done, here are the details” response file (placed in the ftp site directory). If the Service Provider does not support batch processing, the NPAC would send the responses to the requested activity over the interface. | TBD | TBD / TBD |
| NANC 349 (con’t) | Jul ’03 APT: The intention is to off load the interface and have it done at off peak times. The benefit is to move large volume transactions off the CMIP interface. SPs need to categorize the real-world scenarios, and provide feedback on this change order. Aug ’03 APT: Real-world scenario - bulk port over 500K numbers. Business need to move numbers off the switch. This change order will be prioritized behind the other SOA requirements. So, move out of APT document and back into main change mgmt list. |  |  |  |  |  |  |
| NANC 353 | AT&T 4/12/02 | Round-Robin Broadcasts Across SOA Associations (sister of ILL 5) Business Need: (the following text is copied from the existing ILL 5 change order). The NPAC SMS would support additional SOA associations and manage the distribution of transactions in a round robin algorithm across the associations. For example, due to performance conditions a Service Provider may want to start another SOA association for network/notification downloads. The NPAC SMS would accept the association, manage security, and distribute network/subscription PDUs across the 2 or more associations using the round robin algorithm (One unique PDU will be sent over one association only.) (New text for NANC 353, which is a variant of ILL 5) This change order applies to SOA only. | Medium Low | FRS, IIS | Func Backwards Compatible: YES (the following text is copied from the existing ILL 5 change order). 01/15/02 – Refer to the Future Change Orders document for the latest information on this change order. | Med | TBD / TBD |
| NANC 362 | ESI 5/30/02 | Vendor Metrics Business Need: SOA/LSMS vendors request that NPAC volume metrics be captured that would allow SOA/LSMS vendors to create a model for LNP transactional performance based on actual porting data to the SOA and LSMS. Once a model is developed, the intent is to continue to capture various porting data (nominal, peak, duration at peak) to determine the validity of the model. Once the model has been validated and accepted, SOA/LSMS vendors will use this model to intelligently establish the current performance requirements, and by extrapolation, the future requirements. As porting volumes increase, the business need for this change order becomes more time sensitive to help with the situation where porting is delayed because of a slow horse situation. |  |  | Pure Backwards Compatible: YES Both SOA and LSMS data should be gathered. An extract is shown below from the Minutes from the Vendor Metrics Call, May 2, 2002, version 1.2. Refer to the Vendor Call Minutes for full details. Discussion of the LSMS metrics we should gather. The group proposed monthly reports showing message traffic mix. Items to be gathered are: - TN range size (including range of 1), - Message type (create, modify, delete, queries, etc), - Number of messages of this range size and type, - aggregated in 15-minute intervals, - whether transmission congestion occurred during the period, - if congestion occurred, start and end times of congestion, - whether an abort occurred i.e. downstream did not respond during the period. | TBD | N/A / N/A |
| Continuation of NANC 262, Vendor Metrics, Proposed Resolution section: It was agreed that at this time the following report would be a sufficient starting place. For each 15 minute interval, - For the category of prepared messages, report - Message type, - Range size, - and the number of messages with that range size and message type, - For the category of transmitted messages, for the best case report - Message type, - Range size, - The number of messages with that range size and message type, - Count of number of times entered into congestion, - List of congestion intervals, - Count of aborts, - and count of aborts due to timeout. Discussion of SOA metrics proposed by the Slow Horse subcommittee in August and September of 2000. We discussed SOA metrics and agreed that what kind of data that the Slow Horse had proposed was still valid. It was agreed that the sampling interval should be 15-minute intervals and that the LTI information was not relevant. Furthermore, the data should be reported for both the prepared messages and the transmitted messages as was specified above for the LSMS. Consequently, for the SOA the report needs to contain: - All NPAC notifications to SOA. - All SOA requests to NPAC. This information should be reported in 15-minute intervals and categorized as specified above for LSMS messages. For messages sent to the NPAC, they should be reported as: - TN range size (including range of 1), - Message type (create, modify, delete, queries, etc)., - Number of messages of this range size and type, - aggregated in 15-minute intervals. |  |  |  |  |  |  |  |
| Continuation of NANC 262, Vendor Metrics, Proposed Resolution section: June ’02 LNPAWG, additional discussion. The desire is to obtain the offered load, versus what the NPAC is actually producing. In other words, the request versus the result of the request. Colleen Collard would like lots of data on both the inbound and outbound traffic, but realize that the more data that is requested, the longer and more expensive to produce that data. So, initially the group can accept what the NPAC is sending down to the LSMS. Jim Rooks – porting business need is driving SOA, which drives NPAC, which drives LSMS. John Malyar – problem is porting that happens at any single point in time. Jim Rooks – we really need to smooth out data. We are currently looking at request data, the report is sent to NAPM. Steve Addicks – the past doesn’t necessarily reflect future needs/load with wireless (mostly single ports), and also pooling. Dave Garner – need to know what we have today, and also need to do a forecast/projection for the future. NeuStar action item: provide a list of metrics for a baseline of data elements as the NPAC’s side of the projected load, as to what is occurring today. Jim Rooks provided this information at the Aug ’02 LNPAWG meeting. |  |  |  |  |  |  |  |
| NANC 372 | Bellsouth 11/15/02 | SOA/LSMS Interface Protocol Alternatives Business Need: Currently the only interface protocol supported by the NPAC to SOA and NPAC to LSMS interface is CMIP. The purpose of this change order is to request analysis be done to determine the feasibility of adding other protocol support such as CORBA or XML. The primary reasons for looking into a change would be 1) Performance, and 2) Implementation complexity. |  |  | TBD Dec ’02 LNPAWG, discuss this change order in January ’03 in the new arch review meeting. | TBD | TBD / TBD |
| NANC 383 | LNPA WG Archcture Planning Team 5/6/03 | Separate SOA channel for notifications (subset of NANC 353) Business Need: (similar to the existing ILL 5 and NANC 353 change orders). This change order will separate out notifications with other messages, such that a separate channel will be established for SOA notifications versus all other SOA messages. This performance related change order allows additional throughput on both channels. | Medium Low | FRS, IIS | Func Backwards Compatible: YES (similar to the existing ILL 5 and NANC 353 change orders). In order to separate out SOA notifications from all other SOA messages, additional processing logic will need to be developed. | Med | TBD / TBD |
| NANC 384 | LNPA WG Archcture Planning Team 7/10/03. Originally from ESI 6/5/03 | NPAC Change Order Effectiveness Metrics Abstract: This contribution proposes specific metrics for evaluating the operating characteristics of the NPAC RSMS, based on characteristics that have a direct impact on individual carriers cost of operations. It is expected that proposed change orders to NPAC RSMS could be evaluated based on projected improvements to the measurement of one or more of these metrics. Projected improvements in these measurements would be used by individual carriers to justify the cost associated with specific change orders. | Medium Low | FRS, IIS | Func Backwards Compatible: YES In order to separate out SOA notifications from all other SOA messages, additional processing logic will need to be developed. | Med | TBD / TBD |
| NANC 384 (con’t) | NPAC Change Order Effectiveness Metrics (continued) Contribution: As local number portability matures in its processes and supporting systems, and as telecommunications carriers continue to implement significant financial controls on their expenses, carriers are increasingly looking for justification for particular investments. The table below represents a list of 6 characteristic metrics that can be measured at the NPAC RSMS and have a direct impact on an individual carriers’ cost of operation. It is proposed that this set of metrics be used for regular reporting of NPAC RSMS performance capabilities, and that proposed change orders be evaluated by the potential improvement that the change may have on one or more of these metrics. The second table represents an example of the measurements that should be captured to create a baseline measurement set and delta measurements for individual changes. These represent only estimates, and are included to illustrate the estimate or measurement data that could be provided going forward, for use in allowing businesses to make informed investment decisssions with respect to LNP capabilities. Metrics Metric; Units; Measurement Technique. Throughput Capacity Reflects the steady-state porting capacity of the NPAC without queuing (assuming infinitely fast LSMS and SOA systems); TNs/Second; Test Technique 1, item 3. Individual Create Processing Time Measurement in seconds of the time from receipt to SOA notification of create activity; Seconds; Test Technique 1, item 4. Individual Activate Processing Time Measurement in seconds of the time from receipt to SOA notification of activate activity (assuming no late LSMS notifications); Seconds; Test Technique 1, item 4. Individual Modify Processing Time Measurement in seconds of the time from receipt to SOA notification of modify activity; Seconds; Test Technique 1, item 4. Query Response Rate Measurement in Queries/Second that represent the steady-state capacity of the NPAC.; Query Requests/ Second; Test Technique 1, item 3. Individual Query Response Time Measurement in seconds of the time it takes the NPAC to respond to a representative query; Seconds; Test Technique 1, item 4. |
| NANC 384 (con’t) | Test Technique 1: - Establish a representative traffic load that includes a production-like proportion of Create, Concur, Activate, Modify, and Query operations. - Subject the NPAC to the representative proportions of traffic at increasingly high TN/seconds rates, and measure the output LSMS notification rate (the combined rate of SV Activate, SV Modify, and SV Disconnect requests, also in TNs/second). - At sufficiently low rates, the NPAC will reach a steady-state where the input rate and the output rate are approximately equal. As the input rate increases, there will come a point where the input rate exceeds the output rate, indicating that the NPAC is queuing activities internally. The maximum input rate without queuing represents an effective through-put of the system, measured in TNs/second. - When the NPAC loaded at its effective through-put rate, individual transactions each have a start and end time, the difference of which yields a duration calculation for the individual transaction. An average transaction processing time can be calculated for each transaction type from these individual records. The measurement of the start and end time are most accurately measured by a tool placed external to the NPAC. However, it may be acceptable to do initial measurements from transaction log records internal to the NPAC RSMS application software. This is measured in seconds. Change Order Effectiveness Estimates Metric; Units; Assumed Current Value; NPAC Prioritization of Notifications; NANC 179 - Ranged Notifications; NANC 347/350 - 15/60 minute abort timers; NANC 348 - BDD for notifications; NANC 351 - Send what I missed; NANC 352 - SPID recovery; NANC 368 - NPAC OBFC. Throughput Capacity; TNs/Second; 25; +3; +20; +5. Individual Create Processing Time; Seconds; 1; No change; No change; No change. Individual Activate Processing Time; Seconds; 2; No change; No change; No change. Individual Modify Processing Time; Seconds; 2; No change; No change; No change. Query Response Rate; Query Requests/ Second; 12; +1; +14; +2. Individual Query Response Time; Seconds; 2; No change; No change; No change. |
| NANC 384 (con’t) | NPAC Change Order Effectiveness Metrics (continued) - Aug ’03 LNPAWG, discuss this change order in the Sep’03 APT meeting. Requirements will be worked in that forum. |  |  |  |  |  |  |
| NANC 385 | LNPA WG 7/10/03 | Timer Calculation – Maintenance Window Behavior same as Holidays Business Need: NPAC Timers. As defined in the FRS, concurrence windows/timers are generated at the time an activity occurs in the NPAC that requires the use of a window/timer. Specifically, the future expiration time is calculated and stored, based on the NPAC settings, at the time of the activity. These windows/timers will then expire based on the pre-calculated date/time. Therefore, a timer is not a meter that “runs” only during the Business Day intervals, but rather is a calculation in GMT of the timer's expiration date/time. Currently, there are no FRS requirements that address timers and NPAC Maintenance Window time periods. An operational issue can arise when an NPAC Maintenance Window time period overlaps with normal business operating hours. This change order proposes an update to the NPAC so that NPAC Maintenance Window time periods will be factored in when calculating timer expiration date/time (i.e., excluding that period of time from the calculation). This will alleviate the problem where timers expire during the NPAC Maintenance Window time period. | TBD | FRS, GDMO | Func Backwards Compatible: YES The Timer Expiration Calculation will be modified such that a time period designated as an NPAC Maintenance Window that falls within normal business operating hours will NOT “use up” any hours, when calculating the expiration of a timer. Effectively, the NPAC Maintenance Window time period will be treated the same way as Holidays are currently treated in the NPAC (i.e., excluded from the timer expiration calculation). This will require entry of Maintenance Window information in the OpGUI by NPAC Personnel (same as Holidays are currently done). Additionally, a discussion item needs to occur regarding the possible inclusion of Service Provider profile settings to support this new feature. | Med | N/A / N/A |
| NANC 385 (con’t) | - Aug ’03 LNPAWG, discussion: Sprint PCS offered the following: 1.) following up on the Jul ’03 mtg comment about SPID profile toggles, after internal discussions it was deemed to be unnecessary to have SPID toggles. 2.) this functionality was no longer high priority, since it was agreed to shorten the extended Sunday Service Provider Maintenance Window to 8 hours, assuming NPAC stays within the 8 hours for maintenance. 3.) current concern is that NANC 323 migrations may push maintenance windows beyond the 8 hours. 4.) this functionality would have to be in place before agreeing to move the extended maintenance window back to 11 hours. |  |  |  |  |  |  |
| NANC 386 | NeuStar 7/24/03 | Single Association for SOA/LSMS Business Need: Currently, the FRS does NOT address the number of concurrent connections to the NPAC using the same CMIP association function and specific bit mask value. Therefore, there are no requirements to either support or deny this functionality. Because change order ILL-5 was proposed during the initial implementation of the NPAC, the NPAC partially supports multiple associations. This partial implementation can allow a situation where there are one or more non-functional CMIP associations between a SOA/LSMS and the NPAC. This situation causes an unnecessary consumption of NPAC resources (and possibly SOA/LSMS resources as well). This change order will remedy this situation (close the hole) by only allowing a single CMIP association between a SOA/LSMS and the NPAC, for any given association function and specific bit mask value. | TBD | FRS, IIS, GDMO | Func Backwards Compatible: YES The association management function within the NPAC will be modified to allow a single CMIP association between a SOA/LSMS and the NPAC. In the proposed update, if a valid association is active, and a new association request is sent from a SOA/LSMS to the NPAC, the NPAC will abort the first association, and process the request for the second association. | TBD | TBD / TBD |
| NANC 386 (con’t) | - Aug ’03 LNPAWG, discussion: This Change Order would only allow a single association for each SOA/LSMS. NPAC would abort the existing association if a new request came in to establish a second association. If implemented, and if we want ILL-5 down the road, we would have to back this functionality out. Tekelec supports this Change Order but would want it fully tested because it is a behavioral change. BellSouth stated they are concerned that this would preclude multiple associations as a means of addressing interface performance. There was agreement to work the requirements for this Change Order. If the next release package contains a need for multiple associations, then NANC 386 would not be implemented. If no need for multiple associations, we could possibly implement NANC 386 in the next package. |  |  |  |  |  |  |
| NANC 387 | TSE 9/3/03 | Doc-Only Change Order: IIS Updates Business Need: Need to correct some inconsistencies between the IIS flow pictures and/or the corresponding text. 1. B.5.1.6.5: 1a. The second paragraph of the text states "In this case, the new service provider SOA issued the create request". It should state "In this case, the old service provider SOA issued the create request." 1b. The picture and the text don't match. In the picture we have a M-EVENT-REPORT subscriptionVersionNewSP-CreateRequest (subscriptionVersionRangeNewSP-CreateRequest) but in the text we have subscriptionVersionNewSP-ConcurrenceRequest (subscriptionVersionRangeNewSP-ConcurrenceRequest). The text is incorrect. 2. B.4.4.13: Step 1 of the flow indicates the SOA is sending 'M-SET Request numberPoolBlock.' The SOA cannot set the object numberPoolBlock but they can set numberPoolBlockNPAC. 3. B.5.5.2: In the picture Item 1 indicates M-ACTION Request subscriptionVersionRemoveFromConflict and Item 4 indicates M-ACTION Response subscriptionVersionRemoveFromConflict. In the text the corresponding items indicate M-ACTION Request/Response subscriptionVersionNewSP-RemoveFromConflict. The text is in error and needs to be corrected. 4. B.6.4: The text indicates that the SOA is sending the message to the NPAC but the picture shows the NPAC sending the message to the SOA. The labels on the picture need to be reversed. | TBD | IIS | Func Backwards Compatible: YES Update the current documentation to be consistent and reflect the current behavior. | N/A | N/A / N/A |
| NANC 387 (con’t) |  | Doc-Only Change Order: IIS Updates (continued) 5. B.5.3.4: Typo in the Title (Verison, should be Version). |  |  |  |  |  |
| NANC 389 | AT&T Wireless 10/16/03 | Production Equivalent Test-Bed Business Need: Service Providers have expressed a desire to perform a performance volume test to mimic production behavior prior to “go-live”, and to “stress” and certify system readiness, but without having to use simulators to perform the NPAC role. Simulators have been used because the test platform provided under SOW 34 does not support testing at performance volume load levels. It is possible for a Service Provider to impact the overall stability of the SOW 34 test platform and negatively impact other NPAC users. Even with the coordination and scheduling of performance tests in the off-hours, a single Service Provider still can negatively impact the NPAC test-bed, causing downtime to clear the inbound and outbound queues. This change order defines system requirements for a separate NPAC test-bed suitable to meet the industry performance volume test needs. Service Providers could use this test-bed at any time without support. Testing support, including setup, would be provided as agreed. | TBD | Contractual | Func Backwards Compatible: YES This will be explored during the Nov ’03 LNPAWG meeting. | N/A | N/A / N/A |
| NANC 390 | Qwest 10/16/03 | New Interface Confirmation Messages SOA/LSMS – to - NPAC Business Need: Service Provider systems (SOA/LSMS) need to know (in the form of a positive acknowledgement from the NPAC) that the NPAC has received their request message, so the systems (SOA/LSMS) do not unnecessarily resend the message and cause duplicate transactions for the same request. Based on the current requirements for the NPAC, the NPAC acknowledgement message (generally referred to as "a response to a request" from the SOA/LSMS) is not returned until AFTER the NPAC has completed the activity required by that request. During heavy porting periods, transactions that require many records to be updated may take longer than normal for a response to be received from the NPAC. In the case of a delayed response, the SOA/LSMS may abort the association to the NPAC (e.g., after the 15 minute Abort timer expires). When the association is re-established, the SOA/LSMS may resend messages to the NPAC because they haven’t received a response to the first message and thus believe the NPAC did not receive the original message. This behavior can lead to a duplicate transaction for the same request thus: 1.) causing a heavy volume of transactions over the NPAC to SOA/LSMS interface, 2.) slowing Porting completion, 3.) causing an increase of Porting costs, 4.) causing duplicate message processing at the NPAC, and 5.) possibly causing manual intervention by NPAC and Service Provider personnel, etc. | TBD | FRS, IIS, GDMO, ASN.1 | Func Backwards Compatible: NO A new message will be explored during the Nov ’03 LNPAWG meeting. Additionally, a discussion item needs to occur regarding the possible inclusion of Service Provider profile settings to support this new feature. | N/A | N/A / N/A |

# Accepted Change Orders

| - Accepted Change Orders |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chg Order # | Orig. / Date | Description | Priority | Category | Proposed Resolution | Level of Effort |  |
|  |  |  |  |  |  | NPAC | SOA LSMS |
| ILL 5 | AT&T 10/15/96 | Round-Robin Broadcasts Across LSMS Associations The NPAC SMS would support additional LSMS associations and manage the distribution of transactions in a round robin algorithm across the associations. For example, due to performance conditions a Service Provider may want to start another LSMS association for network/subscription downloads. The NPAC SMS would accept the association, manage security, and distribute network/subscription PDUs across the 2 or more associations using the round robin algorithm (One unique PDU will be sent over one association only.) | Medium Low | FRS, IIS | - Func Backwards Compatible: NO This feature may already be implemented in the Lockheed Martin developed NPAC SMS. 01/15/02 – Refer to the Future Change Orders document for the latest information on this change order. | Low | N/A / High |
| ILL 130 | - AT&T 1/6/97 | Application Level Errors - Errors in the SOA and LSMS interfaces are being treated as CMIP errors and it may sometimes be difficult for a SOA to know the true reason for an error from the NPAC SMS and therefore indicate a meaningful error message to its users. It has been requested that application level errors be defined where appropriate and returned as text to the SOA. | High | FRS, IIS, GDMO, ASN.1 | - Func Backwards Compatible: NO - Application level errors would be defined in the IIS. - Refer to R4 Change Orders for current proposed resolution. - 01/02/02 – NPAC R4.0 as submitted to the LLC in 2000 is not going forward. This change order has been moved back into the “accepted” section of this document. - 01/15/02 – Refer to the Future Change Orders document for the latest information on this change order. | High | High / High |
| NANC 138 | - CMA - 8/11/97 | ### Definition of Cause Code Values – REVISITED - NANC 54 defined the cause code values and the FRS was to be updated. Due to an oversight this update was not made in the FRS. The change was going to be applied in FRS 1.4 and 2.2. However, a discrepancy as found. The defined values specified in NANC 54 where are as follows: - The values less than 50 were reserved for SMS NPAC internal use. - Other defined values are: - 0 – NULL (DO NOT MODIFY) - 1 - NPAC automatic cancellation - 50 - LSR Not Received - 51 - FOC Not Issued - 52 - Due Date Mismatch - 53 - Vacant Number Port 54 - General Conflict - In the table in the FRS the following cause code is defined: NPAC SMS Automatic Conflict from Cancellation - There is no corresponding code defined in Change Order NANC 54. Is there a numeric value or is this cause code valid? (continued) | Medium Low | FRS | - Func Backwards Compatible: NO - Update to be made to the FRS. - Pending review by the vendors. Lockheed does not set a cause code when the NPAC SMS automatically puts a cancelled order into conflict. Perot is reviewing their implementation. - There is not a requirement in the FRS for a cause code of NPAC SMS Automatic Conflict from Cancellation. - Operations flows are being reviewed. In figure 6, box 3. - Perot like Lockheed, does not use the cause code in question. - A SOA vendor has been asked to evaluate the impact of not receiving a cause code value with a status of conflict. - Flows in Appendix A also need to be updated. | Low | Low / Low |
| - NANC 138 (cont.) | - Requirements for the cause code addition would be as follows: - RR5-36 should be renumbered to RR5-36.2. - RR5-36.1 Cancel Subscription Version – Cause Code for New SP Timer Expiration - NANC SMS shall set the cause code to “NPAC SMS Automatic Conflict from Cancellation” after setting the Subscription Version status to conflict from cancel-pending when the new Service Provider has not acknowledged cancellation after the Cancellation-Final Concurrence Window. - 2 will be the value defined for the “NPAC SMS Automatic Conflict from Cancellation” cause code. |  |  | - Awaiting sizing from NPAC vendors, and validation of functionality (reference existing requirements) from cancellation to conflict. - SOA vendors heard from to date do not have a problem with the cause code not being present. - This is an "OLD" Release 2.0 change order, that has been moved into the "Accepted" category, awaiting prioritization Refer to R4 Change Orders for current proposed resolution. 01/02/02 – NPAC R4.0 as submitted to the LLC in 2000 is not going forward. This change order has been moved back into the “accepted” section of this document. 01/15/02 – Refer to the Future Change Orders document for the latest information on this change order. |  |  |  |
| - NANC 151 | - Bellcore 9/4/97 | TN and Number Pool Block Addition to Notifications It has been requested that the TN for the subscription version be added to all notifications that currently contain SV-ID but not TN from the NPAC SMS. It is possible for a SOA in a disconnect or modify-active situation, to not have the SV record in their database. Therefore, when the attribute/status change notification comes from the NPAC SMS, there is no way to correlate its version id with the TN on the disconnect or modify request in SOA. Jun 00 LNPA-WG meeting, additionally, the same type of change should be done for Number Pool Block (i.e., add the NPA-NXX-X to all notifications that currently contain Block-ID but not NPA-NXX-X). | - Low | - IIS | - Func Backwards Compatible: NO #### This would be a deviation from the standard since the TN would not have been an attribute that has changed. - This is an "OLD" Release 2.0 change order, that has been moved into the "Accepted" category, awaiting prioritization - 01/15/02 – Refer to the Future Change Orders document for the latest information on this change order. | - Low | - Low / N/A |
| - NANC 193 | - NANC T&O 1/23/1998 | - TN Processing During NPAC SMS NPA Split Processing - There was group consensus that NPAC behavior would not change until the start of permissive dialing. An example would be an audit that occurred during split processing one-minute before the start of permissive dialing. The NPAC should act as if permissive dialing has not yet started for the audit initiated during split processing. The Split processing should have no effect on operations of the system. - A clarification requirement should be added as follows: - NPAC SMS shall processes requests during split processing prior to the start of permissive dialing as if the split processing has not yet occurred. - Additional clarification requirement: - NPAC SMS shall in a download request made after permissive dialing start for subscription version data sent prior to permissive dialing start, return the new NPA-NXX for subscription versions involved in an NPA Split. - The above requirements do not reflect the current Lockheed NPAC SMS implementation. | - Medium High | - FRS | - Pure Backwards Compatible: YES - Lockheed in release 1.2 currently holds requests until the NPA Split processing completes (regardless of the NPA or NPA-NXX). Nortel/Perot rejects the requests during NPA split processing. It was not clear if errors were for all requests or just requests related to the NPA or NPA-NXX being split. - Desired behavior would be to have no errors occur. Requests put on hold or queued would only be those related to NPA-NXXs involved in the NPA split being processed. - Lockheed in Release 1.3 will perform NPA- NXX locking. - The following questions need to be answered by vendors: - What will the SOA do if it sends an old NPA-NXX prior to PDP and the NPAC returns the new SV with the new NPA-NXX? What would happen for a create/audit/query? - What will LSMS systems do if an audit is sent for new NPA prior to PDP? - Are there LSMS that will not be able to handle audits on new NPA-NXX right at the start of PDP? - (continued) | - High + | - N/A / N/A |
| - NANC 193 - (con't) | - Continued | - Proposed Solution (continued): - How long does it take for NPAC/SOA/LSMS to split an NPA-NXX? - What is the NPAC behavior for recovery spanning time before & after PDP? - If NPAC splits starting at midnight and SOA sends new NPA-NXX for an NPA-NXX not in split what would happen? - After reviewing the above questions. It was determined that the NPAC should act as if the split had not occurred during split processing prior to permissive dialing. - A matrix of answers received above has been created. - It was discussed that this requirement would have to be implemented by SOA, LSMS, and NPAC vendors. This requirement would shorten the window when errors could occur for the change of an NPA. It was requested that we review and document on behavior in the following situations: When the NPAC receives a request sent before the splits after the split start, how should it respond? Also when an SOA or LSMS receives a request sent before the split after the split start, how should it respond? - IIS flows for error scenarios will be created. If an active is received by the NPAC SMS before PDP it will be rejected. If the old SP is received after the end of PDP it will be treated as the old NPA-NXX if that NPA- NXX is still a valid portable NPA-NXX in the NPAC SMS otherwise it will be rejected. Download requests after the start of PDP for information occurring before PDP should reflect the new NPA- NXX for subscription versions involved in a Port. - The matrix was finalized on the 5/22 T&O call. 01/15/02 – Refer to the Future Change Orders document for the latest information on this change order. |  |  |  |  |  |
| - NANC 200 | - AGCS 2/28/1998 | - Notification of NPA Splits - It has been requested that to facilitate synchronization during NPA split, the NPAC via the mechanized interface should notify the SOA and LSMSs. The preferred method would be to have a new managed object that contains all split information. It would still be up to the respective system to perform the splits, but all systems would be in sync. A second alternative would be to have the NPAC issue a notification that states the NPAC is start/ending split processing. | - High | - FRS, IIS, GDMO, ASN.1 | - Func Backwards Compatible: NO - This change order is related to change order NANC 192 that proposes getting the split information from the LERG. Refer to R4 Change Orders for current proposed resolution. 01/02/02 – NPAC R4.0 as submitted to the LLC in 2000 is not going forward. This change order has been moved back into the “accepted” section of this document. 01/15/02 – Refer to the Future Change Orders document for the latest information on this change order. | - Med / Low | - Med / Med |
| - NANC 219 | - AT&T 6/5/1998 | - NPAC Monitoring of SOA/LSMS Associations - It has been requested that NPAC Monitoring of SOA and LSMS associations be put into the NPAC SMS at the application (CMIP) layer. The approach suggested by the requestor would be to alarm whenever aborts are received or sent by the NPAC. When these alarms occur, the NPAC Personnel would contact the affected Service Provider to work the problem and ensure the association is brought back up. - From this point forward, this change order will deal with the alarm abort option. The heartbeat abort option is NANC 299. | - High | - FRS | - Pure Backwards Compatible: YES - Sep LNPAWG (Seattle), discussed various options for working the problem of dropped associations (i.e., causes partial failures for the new SP trying to activate). - Options include, - 1.) sending a notification to all SPs that "an SP is currently not associated", then another notifications once it is back up, "all SPs associated". - 2.) stopping an activation request, because an association is down. - 3.) sending a notification to the New SP when an activate is received, that an association is down, "do you still want to activate?". - NEXT STEP: all SPs should consider issues and potential options for activates during a missing association that will cause a partial failure. - Oct LNPAWG (Kansas City), the conversation migrated away from the three options discussed in Seattle, and back to the NPAC proactively monitoring the association. This would require the NPAC to provide an attendant notification that a Service Provider is down, then notifying them of their missing association. - (continued) | - Low (alarm abort) - Med (heartbeat abort) - High (ops costs for all options) | - N/A / N/A |
| - NANC 219 - (con't) | - Continued | - Proposed Solution (continued): - So, anytime the NPAC receives an abort from a Service Provider, an NPAC alarm should be triggered, and an M&P should kick in where NPAC personnel notify the downed SP. - This has been moved into the "Accepted" category, awaiting prioritization. - Refer to R4 Change Orders for current proposed resolution. - 01/02/02 – NPAC R4.0 as submitted to the LLC in 2000 is not going forward. This change order has been moved back into the “accepted” section of this document. 01/15/02 – Refer to the Future Change Orders document for the latest information on this change order. |  |  |  |  |  |
| - NANC 227 | - MCI - 8/7/98 | - 10-digit TN Filters (previously know as "Ability to Modify/Delete of Partial Failure SV") - OLD TEXT: The NPAC SMS currently rejects a request to "modify active" or "delete" an SV that has a partial failure status. Nothing can be done to the SV until the discrepant LSMS(s) come back on line, and either recover the broadcast, or accept a re-send from the NPAC. - OLD TEXT: A business scenario arose whereby a partial failure was affecting a customer's main number, and the New SP couldn't do anything to the SV until the partial failure was resolved. - NEW TEXT: The NPAC should provide a mechanism that allows 10-digit filters, in order to clean up partial failure SVs that need to be subsequently modified or deleted, by the New SP. - Jun 99, during the Pooling Assumptions walk-thru, four SV requirements were modified, and the functionality was moved into this change order. Basically, the “partial failure/failed” text is moved to this change order. The affected requirements are listed below: - SV-230 Modification of Number Pooling Subscription Version Information – Subscription Data - SV-240 Modification of Number Pooling Subscription Version Information – Status Update to Sending - SV-270 Modification of Number Pooling Subscription Version Information – Status Update - SV-280 Modification of Number Pooling Subscription Version Information – Failed SP List - This change order is related to NANC 254. | - High | - FRS, GDMO | - Func Backwards Compatible: NO - Discussed during 8/12/98 face-to-face T&O meeting (Detroit). - OLD TEXT: It was determined that the business scenario was primarily human error, and the NPAC should NOT be modified to allow a partial failure to go to active, but still have out-of-sync LSMS(s). - OLD TEXT: A workaround (available with 1.3 [with the exception of PTO]) would be to temporarily set up a filter for the discrepant LSMS(s), do a re-send which would clear up the failed-SP-List and set the SV to active, then remove the filter. - OLD TEXT: NEXT STEP: all SPs and vendors should evaluate if this is an acceptable solution. - OLD TEXT: Sep LNPAWG (Seattle), this potential M&P work-around has been forwarded to NPAC Operations (Jan Trout-Avery) for further analysis, and will be discussed at the x-regional in New Orleans. - (continued) | - High | - Med-Low / N/A |
| - NANC 227 - (con't) |  | - OLD TEXT: This change order will be left open pending the discussion in New Orleans. - Oct LNPAWG (Kansas City), after discussions in New Orleans at the x-reg meeting, it was requested by Service Providers that Lockheed use the M&P for "partial failures where the customer is out of service" only. - Jan will be doing an M&P on this, and will accumulate data on the frequency of this situation. Everyone should be aware that the risk for the M&P is that any other SVs that are coming down in the NPA-NXX will NOT be sent to the LSMS. From an NPAC functional perspective, a potential problem is the complexity of having to keep "versions" of versions, when you have an activate that fails, then allow a modify on top of this. - Jim Rooks provided info on this, to state that he is uncomfortable with the modify of a partial failure. We further discussed the potential of a 10-digit filter that would override the existing 6-digit filter. This should be the same change order, but will replace the title from modify partial failure to 10-digit filter. - Nov LNPAWG (Dallas), re-capped discussion from KC. Desire of this functionality is to have NPAC Personnel perform this activity (of putting up 10-digit filters), and NOT allow SPs to send this over the interface. - This has been moved into the “Accepted” category, awaiting prioritization. The group will flush out the details once this gets placed into a specific release. - Jul LNPAWG (Ottawa), no comments on pooling additions. - Refer to R4 Change Orders for current proposed resolution. - 01/02/02 – NPAC R4.0 as submitted to the LLC in 2000 is not going forward. This change order has been moved back into the “accepted” section of this document. - 01/15/02 – Refer to the Future Change Orders document for the latest information on this change order. Also note that this change order was merged with NANC 254 sometime during or prior to the R4.0 discussions and is now referred to NANC 227/254. |  |  |  |  |  |
| - NANC 232 | - MetroNet - 8/14/98 | - Web Site for First Port Notifications - Currently all SOAs and LSMSs receive "first port" notifications. A request has been submitted to provide this information on the NPAC Web Site. - Sep LNPAWG (Seattle). This change order was introduced by MetroNet as a means for LTI users to obtain "first port" notifications. - The current process does NOT send this information to the LTI user (unlike SPs that have a CMIP-based SOA), but requires the LTI user to "query" the NPAC for notifications contained in the NPAC notification log (for that specific SP). Currently, this log contains the most recent 25 notifications for that SP. The user may also generate an NPAC report of all notifications for that SP. - The desire is to have these "first port" notifications on the web, similar to the NPA-NXX openings that are on the web today. | - High | - FRS | - Pure Backwards Compatible: YES - Sep LNPAWG (Seattle). This change order was discussed by those in attendance. It was agreed that this change order was acceptable, and should be moved to the "Future Release CLOSED" List, and await prioritization from the group. - NOTE: This change order is similar to the existing requirements, R3-10 and R3-11 (Web bulletin board updates of NPA-NXXs and LRNs). - Refer to R4 Change Orders for current proposed resolution. - 01/02/02 – NPAC R4.0 as submitted to the LLC in 2000 is not going forward. This change order has been moved back into the “accepted” section of this document. 01/15/02 – Refer to the Future Change Orders document for the latest information on this change order. | - Low | - N/A / N/A |
| - NANC 285 | LNPA WG - 5/12/99 | SOA/LSMS Requested Subscription Version Query Max Size A SOA/LSMS request for a Subscription Version query that exceeds the maximum size tunable (“Maximum Subscriber Query”), returns an error message to the SOA. Similar to the processing in NANC 273, it has been requested the NPAC return SVs up to the max tunable amount instead. The SOA/LSMS would accept this message, then use it’s contents to send another query to the NPAC, starting with the next TN, and so on until all SVs are returned to the SOA/LSMS. It will be up to the SOA/LSMS to manage the data returned from the NPAC and determine the next request to send to the NPAC in order to get the next set of SVs. The NPAC will continue to return SVs that meet the selection criteria. However, the NPAC will not return a “count” to the SOA/LSMS for number of records that match the selection criteria. This solution will resolve the problem described in NANC 279 (SOA Resynchronization for Large Ranges), where a problem exists for recovering the SOA for large ranges, because the SV time stamp that the NPAC users for recovery is the same for large ranges. The example used for NANC 279 was, if all the TNs in the range contain the same time stamp (e.g., 17 minutes and 20 seconds after 3p, 15:17:20), and the number of TNs in the range exceeds the tunable allowed for queries, the SOA cannot recover since the NPAC, for any time range, will respond with an error for maximum TN query reached. | - High | - FRS, IIS, GDMO | - Func Backwards Compatible: NO June LNPAWG (San Ramon), discussed in conjunction with NANC 279. Group decided to close out 279, and merge the requested functionality into this change order, since this is query functionality issue, and not just a recovery issue. Jim Rooks will provide additional information on a proposed solution given the inclusion of NANC 279 into this change order. Jim’s response is shown below: This change order requests the 'more' capability that will be supported by queries in the LTI. This implementation requires 2 changes. #1, the NPAC must be modified to always return the first n (tunable) records on the SV query. Currently, the NPAC determines that the query will return more than n records and returns an error. - (continued) | - Low | - Med-High / Med-High |
| - NANC 285 (con’t) | - continued | Proposed Solution (continued): #2, the service providers should modify their systems to support the following SV query operations to the NPAC: - When data is returned from an SV Query and there are exactly n (tunable) records returned, the SP must assume that they didn't get all the data from their query. - After processing the first n records, they should send a new query that picks up where the data from the prior query ended. - The SV data returned from the NPAC for SV queries will be sorted by TN and then by SVID so a filter can be created to pick up where the prior query ended. - For example, if a SOA query to the NPAC returns exactly 150 records and the last SV returned was TN '303-555-0150' with SVID of 1234. The filter used on the next query would be: All SVs where ((TN > 303-555-0150) OR (TN = 303-555-0150 AND SVID > 1234). The NPAC does support OR filters. - Once the results from the NPAC returns less than 150 records, the SP can assume they received all records in the requested query. Refer to R4 Change Orders for current proposed resolution. - 01/02/02 – NPAC R4.0 as submitted to the LLC in 2000 is not going forward. This change order has been moved back into the “accepted” section of this document. 01/15/02 – Refer to the Future Change Orders document for the latest information on this change order. |  |  |  |  |  |
| NANC 299 | LNPA-WG 9/15/99 | NPAC Monitoring of SOA and LSMS Associations via Heartbeat This is an extension of NANC 219 and NANC 301. Instead of utilizing a TCP Heartbeat and an abort message, the NPAC SMS would utilize an application level heartbeat message on every association. If a response was not returned for any given application level heartbeat message, an alarm would be initiated for NPAC Personnel. Oct LNPAWG (KC), this change order is designed to establish the application level heartbeat process (which requires an interface change to both the NPAC and the SOA/LSMS). This process will allow two-way communication and allow either side to initiate the application level heartbeat message. The application level heartbeat process should be set up so that the functionality can be optionally set up per association. The alarming process is the same as 219, such that an alarm would be initiated whenever application level heartbeat responses are not sent by the NPAC or SOA/LSMS. When these alarms occur, the NPAC Personnel would contact the affected Service Provider to work the problem and ensure the association is brought back up. | High | FRS, IIS, GDMO, ASN.1 | - Func Backwards Compatible: NO The current working assumption is that this heartbeat would be a new message, it would not have any access control, it would be at a low level in the protocol stack, this heartbeat would occur on the same port as the association, this message would only occur if no traffic was sent/received after a configurable period of time, and this heartbeat would be two-way to allow either side to initiate this message. All parties still need to examine if there might be an issue with filtering in their firewalls. The need for both a network level heartbeat and application level heartbeat still needs to be decided. Jan ‘00 LNPAWG meeting, the group has not been able to determine the feasibility of implementing an application level heartbeat. It was agreed to put this change order on hold, pending the outcome of NANC 301 (NPAC TCP Level Heartbeat [transport layer]). The functionality documented in this change order needs further review before this change order can be considered “accepted and ready for selection into a release”. (continued) | Med | Med -High / Med - High |
| NANC 299 (con’t) | continued | Proposed Solution (continued): May ‘00 LNPAWG (Atlanta), leave open until further analysis of NANC 219 and NANC 301 (i.e., after R4 implementation). June ‘00 LNPAWG meeting, group consensus (during R5 discussion) is to move to cancel-pending. July 2000 meeting – LNPA WG consensus is that they do not want to cancel this change order but move it back to an accepted change order for a future release. Metrics and reports that will be provided after R4.0 will give more information to determine whether or not this change order is needed. 01/15/02 – Refer to the Future Change Orders document for the latest information on this change order. |  |  |  |  |  |
| NANC 300 | LNPA-WG 12/6/99 | 7-digit Block Filters for Number Pooling This is an extension of NANC 227. During the Dec 99 LNPA-WG meeting, it was proposed to remove Number Pooling functionality from NANC 227, and create a new change order for this functionality. | ??? | FRS, GDMO | - Functional Backwards Compatible: NO 01/15/02 – Refer to the Future Change Orders document for the latest information on this change order. | Med | Med-Low |
| NANC 321 | WorldCom 12/13/00 | Regional NPAC NPA Edit of Service Provider Network Data - NPA-NXX Data Business Need: When a service provider submits a message to the NPAC in order to create a pending subscription version, the NPAC verifies that the old service provider identified in the message is the current service provider and that the number to be ported is from a portable NPA-NXX. If the telephone number already is a ported number, the NPAC will look at the active SV for that number to determine the identity of the current SP as shown in the active SV. If no active SV exists, then the number is not currently ported and the NPAC determines the current SP instead based on NPA-NXX ownership as shown in the NPAC's network data for each service provider. The NPAC also looks at the network data to confirm that the NPA-NXX has been identified as open to portability. If a service provider has entered an NPA-NXX in its network data but has done it for its network data associated with the wrong region, then the correct NPAC region, when receiving create messages involving numbers in that NPA-NXX, will be unable to see that the TNs involve a portable NPA-NXX; in this case the create message will be rejected by NPAC. Furthermore, another service provider could erroneously enter the NPA-NXX in its network data for the correct NPAC region. Then the NPAC's portable NPA-NXX validation would pass, but the current service provider validation would fail. In either case the telephone number could not be ported until the service provider network data error were corrected. | ??? | FRS | Functional Backwards Compatible: Yes January 2001 meeting: Accepted pending review of the final write-up in February. February 2001 meeting: Accepted 01/15/02 – Refer to the Future Change Orders document for the latest information on this change order. | ??? | N/A / N/A |
| NANC 321 (cont’d) |  | It is important therefore to assure that service provider NPA-NXX network data be populated only in the proper NPAC region and to allow only the LERG-assignee to populate the data. The introduction of an NPA edit function, to validate that an NPA-NXX input is to network data associated with the NPAC region encompassing the involved NPA will effectively serve both functions. Such an edit function would not allow a service provider to put its NPA-NXX data in the wrong NPAC region's database and it consequently would not allow the improper LERG-assignee entries to remain long undetected. Description of Change: Network Data is submitted by service providers over their SOA/LSMS interfaces or via the NPAC Administrative OpGUI or the SOA LTI. A provider is required to enter each portable NPA-NXX for which it is the LERG assignee. The NPAC uses this service provider network data to perform certain validation functions of subscription version data -- to confirm current SPID correct and that TN is from portable NXX -- and to determine TN ownership in snap-back situations. Detailed requirements are as follows: 1. The NPAC will reject an NPA-NXX network data entry attempt if the NPA involved is not encompassed by the NPAC region to which the data is being submitted. 2. A table of valid NPAs will be established for each regional NPAC. 3. Each table of valid NPAs open in the NPAC service area will be maintained by NPAC personnel for each regional NPAC. 4. The NPAC will obtain information on new NPAs from the LERG. 5. The change order would be implemented on a regional basis. |  |  |  |  |  |
| NANC 343 | LNPA WG 11/14/01 | Doc Only Change Order for IIS: Exhibit 12 of IIS section 4.2.2 does not reflect all filtering operations currently supported by the NPAC SMS. “From Section 4.2.2: The following table shows the CMISE primitive filtering support required of the Local SMS by the NPAC SMS for the subscriptionVersion object. (continued) | Medium | IIS | Incorporate into next release of IIS. 12/12/01 – Reviewed during December LNPA WG meeting. Needs more revisions. Will be reviewed again during January 2002 meeting. 01/09/02 – Reviewed revisions. More revisions required. The new revisions are highlighted in yellow. Will review again during the February 2002 meeting. Nov ‘02 LNPAWG – Reviewed at meeting, move to accepted. Additional text has been added to make consistent with the numberPoolBlockNPAC MANAGED OBJECT CLASS in the GDMO, related to LNP Type. | N/A | N/A / N/A |
| NANC 343 (cont’d) | Exhibit 1 - CMISE Primitive Filtering Support for the Subscription Version Object CMISE Primitives; Filter Supported; Notes. M-ACTION; N; No filtering is applied to the actions for the subscriptionVersion object.. M-GET; Y; TN Range with greaterOrEqual, lessOrEqual, equality must be supported for auditing.. M-SET; Y; TN Range with greaterOrEqual, lessOrEqual, equality must be supported for Mass Update or TN range modify requests.. M-DELETE; Y; TN Range with greaterOrEqual, lessOrEqual, equality will be supported for range disconnect or port to original requests.. “ Modify text and table as follows to clarify exact functionality for TNs and for Number Pooling functionality: From Section 4.2.2: The following table shows the CMISE primitive filtering support required of the Local SMS by the NPAC SMS for the subscriptionVersion object. (continued) |
| NANC 343 (cont’d) | Exhibit 1 - CMISE Primitive Filtering Support for Local System Objects CMISE Primitives; Filter Supported; Notes. M-ACTION; N; No filtering is applied to the actions.. M-GET; Y; TN Query Range with greaterOrEqual and lessOrEqual, and equality must be supported for auditing. The fields used with greaterOrEqual and lessOrEqual filters are subscriptionTN and subscriptionActivationTimeStamp. The field used with equality is subscriptionTN. Filters supported contain either a greaterOrEqual and lessOrEqual filter, or equality filter, for subscriptionTN only or a more complex filter. The more complex filter uses two criteria for filtering. The first criteria used is greaterOrEqual and lessOrEqual filters with subscriptionTN. The second criteria uses greaterOrEqual and lessOrEqual filters for subscriptionActivationTimeStamp. Both criteria must be matched for the data being queried (logical and). The scope for the filters is level 1 only with a base managed object class of lnpSubscriptions.. Number Pool Block Query with greaterOrEqual and lessOrEqual, and equality for EDR support. The fields used with greaterOrEqual and lessOrEqual filters are numberPoolBlockNPA-NXX-X and numberPoolBlockActivationTimeStamp. The field used with equality is numberPoolBlockNPA-NXX-X. Filters supported contain either a greaterOrEqual and lessOrEqual filter, or equality filter, for numberPoolBlockNPA-NXX-X only or a more complex filter. The more complex filter uses two criteria for filtering. The first criteria used is equality filter with numberPoolBlockNPA-NXX-X. The second criteria uses greaterOrEqual and lessOrEqual filters for numberPoolBlockActivationTimeStamp. Both criteria must be matched for the data being queried (logical and). The scope for the filters is level 1 only with a base managed object class of lnpSubscriptions.. (continued) |
| NANC 343 (cont’d) | M-SET; Y; TN Range Modify with greaterOrEqual and lessOrEqual, and equality must be supported for Mass Update or TN modify requests. The field used with greaterOrEqual and lessOrEqual filters is subscriptionTN. The fields used with equality are subscriptionTN and subscriptionNewCurrentSP. Filters supported contain either a greaterOrEqual and lessOrEqual filter, or equality filter, for subscriptionTN only, or a more complex filter. In the case of Modification of TNs for non-EDR number pool block the filter is more complex and uses two criteria for modification. The first criteria uses the subscriptionNewCurrentSP field with equality. The second criteria uses lessOrEqual and greaterOrEqual for subscriptionTN. Both criteria must be matched for the data being set (logical and). Additionally, a filter for LNP Type equal to ‘pool’ may be used. The scope for the filters is level 1 only with a base managed object class of lnpSubscriptions.. Number Pool Block Modify with greaterOrEqual and lessOrEqual, and equality for EDR support. The field used with greaterOrEqual and lessOrEqual is numberPoolBlockNPA-NXX-X. The field used with equality is numberPoolBlockNPA-NXX-X. The scope for the filters is level 1 only with a base managed object class of lnpSubscriptions.. NOTE: Exhibit 13 will be removed from the IIS. (continued) |
| NANC 343 (cont’d) | M-DELETE; Y; TN Range Delete with greaterOrEqual and lessOrEqual, and equality will be supported. for range disconnect or port to original requests. The field used with greaterOrEqual and lessOrEqual filters is subscriptionTN. The field used with equality is subscriptionTN. The scope for the filter is level 1 only with a base managed object class of lnpSubscriptions. In the case of Deletion of TNs for non-EDR number pool block the filter is more complex and uses two criteria for deletion. The first criteria uses the subscriptionNewCurrentSP field with equality. The second criteria uses lessOrEqual and greaterOrEqual for subscriptionTN. Both criteria must be matched for the data being set (logical and). Additionally, a filter for LNP Type equal to ‘pool’ may be used.. NOTE: Exhibit 13 will be removed from the IIS. (continued) |
| NANC 343 (cont’d | GDMO Documentation DOCUMENTATION changes should be made in the GDMO behavior for the following objects to accurately reflect scooping and filtering support required for the NPAC SMS to the LSMS: - lnpSubscriptions - subscriptionVersion - numberPoolBlock Further GDMO modifications will be necessary to reflect SOA and LSMS scoping and filtering support when sending requests to the NPAC SMS for the following objects: - subscriptionVersionNPAC - numberPoolBlockNPAC Additional GDMO text will be added to reflect SOA and LSMS scoping and filtering support when sending requests to the NPAC SMS for other objects. ### lnpSubscriptions: The lnpSubscriptionsDefinition BEHAVIOUR should be modified as follows: lnpSubscriptionsDefinition BEHAVIOUR DEFINED AS ! Local SMS and NPAC SMS Managed Object for the SOA to NPAC SMS and the Local SMS to NPAC SMS interface. The lnpSubscriptions class is the managed object that is used as the container object for the subscription version objects and numberPoolBlock objects on the NPAC SMS and the Local SMS. Local SMS interfaces must be able to support scoped/filtered and filtered requests with a level 1 scope and a base managed object class of lnpSubscription.M-SETs and M-DELETEs with a TN range as the primary filter. Specific filter criteria support is defined in the behavior for the subscriptionVersion and numberPoolBlock managed objects. !; (continued) |  |  |  |  |  |  |
| NANC 343 (cont’d) | ### subscriptionVersion: The subscriptionVersionBehaviour BEHAVIOUR should be modified as follows: subscriptionVersionBehavior BEHAVIOUR DEFINED AS ! . . . The Local SMS can not modify any of the subscription version data locally unless changes were downloaded via a download request. The Local SMS must be able to support scoped and filtered requests with a level 1 scope and a base managed object class of lnpSubscription for subscription version (M-GET, M-SET, and M-DELETE) requests. with a filter for equality and ordering on the subscriptionTN from the NPAC SMS. Filtering Support for M-GET: TN Query with greaterOrEqual and lessOrEqual, and equality must be supported for auditing. The fields used with greaterOrEqual and lessOrEqual filters are subscriptionTN and subscriptionActivationTimeStamp. The field used with equality is subscriptionTN. Filters supported contain either a greaterOrEqual and lessOrEqual filter, or equality filter, for subscriptionTN only or a more complex filter. The more complex filter uses two criteria for filtering. The first criteria used is greaterOrEqual and lessOrEqual filters with subscriptionTN. The second criteria uses greaterOrEqual and lessOrEqual filters for subscriptionActivationTimeStamp. Both criteria must be matched for the data being queried (logical and). Filtering Support for M-SET: TN Modify with greaterOrEqual and lessOrEqual, and equality must be supported for Mass Update or TN modify requests. (continued) |  |  |  |  |  |  |
| NANC 343 (cont’d) | The field used with greaterOrEqual and lessOrEqual filters is subscriptionTN. The fields used with equality are subscriptionTN and subscriptionNewCurrentSP. Filters supported contain either a greaterOrEqual and lessOrEqual filter, or equality filter, for subscriptionTN only, or a more complex filter. In the case of Modification of TNs for non-EDR number pool block the filter is more complex and uses two criteria for modification. The first criteria uses the subscriptionNewCurrentSP field with equality. The second criteria uses greaterOrEqual and lessOrEqual for subscriptionTN. Both criteria must be matched for the data being set (logical and). Additionally, a filter for LNP Type equal to ‘pool’ may be used. The scope for the filters is level 1 only with a base managed object class of lnpSubscriptions. Filtering Support for M-DELETE: TN Delete with greaterOrEqual and lessOrEqual, and equality will be supported. The field used with greaterOrEqual and lessOrEqual filters is subscriptionTN. The field used with equality is subscriptionTN. The scope for the filters is level 1 only with a base managed object class of lnpSubscriptions. In the case of Deletion of TNs for non-EDR number pool block the filter is more complex and uses two criteria for deletion. The first criteria uses the subscriptionNewCurrentSP field with equality. The second criteria uses lessOrEqual and greaterOrEqual for subscriptionTN. Both criteria must be matched for the data being set (logical and). Additionally, a filter for LNP Type equal to ‘pool’ may be used. !; (continued) |  |  |  |  |  |  |
| NANC 343 (cont’d) | ### numberPoolBlock: The numberPoolBlock-Behaviour BEHAVIOUR should be modified as follows: numberPoolBlock-Behavior BEHAVIOUR DEFINED AS ! . . . The Local SMS can not modify any of the number pool block data locally unless changes were downloaded via a download request. The Local SMS must support scoped and filtered requests with a level 1 scope and a base managed object class of lnpSubscriptions for numberPoolBlock M-GET and M-SET requests. equality and ordering on the numberPoolBlockNPA-NXX-X attribute in a scoped and filtered request for mass updates and audits. Filtering Support for M-GET: Number Pool Block Query with greaterOrEqual and lessOrEqual, and equality for EDR support. The fields used with greaterOrEqual and lessOrEqual filters are numberPoolBlockNPA-NXX-X and numberPoolBlockActivationTimeStamp. The field used with equality is numberPoolBlockNPA-NXX-X. Filters supported contain either a greaterOrEqual and lessOrEqual filter, or equality filter, for numberPoolBlockNPA-NXX-X only or a more complex filter. The more complex filter uses two criteria for filtering. The first criteria used is equality filter with numberPoolBlockNPA-NXX-X. The second criteria uses greaterOrEqual and lessOrEqual filters for numberPoolBlockActivationTimeStamp. Both criteria must be matched for the data being queried (logical and). The scope for the filters is level 1 only with a base managed object class of lnpSubscriptions. (con’t) |  |  |  |  |  |  |
| NANC 343 (cont’d) | Filtering Support for M-SET: Number Pool Block Modify with greaterOrEqual and lessOrEqual, and equality for EDR support. The field used with greaterOrEqual and lessOrEqual filters is numberPoolBlockNPA-NXX-X. The field used with equality is numberPoolBlockNPA-NXX-X. The scope for the filters is level 1 only with a base managed object class of lnpSubscriptions. !; |  |  |  |  |  |  |
| NANC 346 | NeuStar 1/21/02 | GDMO Change to Number Pool Block Data Managed Object Class (Section 29.0) and Documentation Change to Subscription Version Managed Object Class (Section 20.0) Change the numberPoolBlock-Pkg to support updates to the numberPoolBlockActivationTimeStamp attribute. Currently this attribute is not modifiable so when it is audited by the NPAC SMS and found to be discrepant there is no way to update it. The NPAC SMS attempts to correct the attribute on the LSMS and the M-SET is failed by the service provider’s system because the attribute is GET only. Currently the numberPoolBlock-Pkg reads: numberPoolBlock-Pkg PACKAGE BEHAVIOUR numberPoolBlock-Definition, numberPoolBlock-Behavior; ATTRIBUTES numberPoolBlockId GET, numberPoolBlockNPA-NXX-X GET, numberPoolBlockHolderSPID GET, numberPoolBlockActivationTimeStamp GET, numberPoolBlockLRN GET-REPLACE, numberPoolBlockCLASS-DPC GET-REPLACE, numberPoolBlockCLASS-SSN GET-REPLACE, numberPoolBlockLIDB-DPC GET-REPLACE, numberPoolBlockLIDB-SSN GET-REPLACE, numberPoolBlockCNAM-DPC GET-REPLACE, numberPoolBlockCNAM-SSN GET-REPLACE, numberPoolBlockISVM-DPC GET-REPLACE, numberPoolBlockISVM-SSN GET-REPLACE, numberPoolBlockDownloadReason GET-REPLACE; ; | High | GDMO | Modify the numberPoolBlock-Pkg to read: numberPoolBlock-Pkg PACKAGE BEHAVIOUR numberPoolBlock-Definition, numberPoolBlock-Behavior; ATTRIBUTES numberPoolBlockId GET, numberPoolBlockNPA-NXX-X GET, numberPoolBlockHolderSPID GET, numberPoolBlockActivationTimeStamp GET-REPLACE, numberPoolBlockLRN GET-REPLACE, numberPoolBlockCLASS-DPC GET-REPLACE, numberPoolBlockCLASS-SSN GET-REPLACE, numberPoolBlockLIDB-DPC GET-REPLACE, numberPoolBlockLIDB-SSN GET-REPLACE, numberPoolBlockCNAM-DPC GET-REPLACE, numberPoolBlockCNAM-SSN GET-REPLACE, numberPoolBlockISVM-DPC GET-REPLACE, numberPoolBlockISVM-SSN GET-REPLACE, numberPoolBlockDownloadReason GET-REPLACE; ; (continued) | N/A | Low / Low |
| NANC 346 (cont’d) |  | Proposed Solution (continued): Number Pool Block, object 29.0 -- Update the GDMO behavior text (add to the end). The Local SMS can only modify the numberPoolBlockActivationTimeStamp locally upon receiving a modify request from the NPAC SMS. Subscription Version, object 20.0 -- Update the GDMO behavior text (add to the end). The Local SMS can only modify the subscriptionVersionActivationTimeStamp locally upon receiving a modify request from the NPAC SMS. Nov ‘02 LNPAWG – Reviewed at meeting, move to accepted. |  |  |  |  |  |
| NANC 347/350 | NeuStar 3/6/02 | CMIP Interface Enhancements – abort behavior Business Need: Note: During the Nov ‘02 LNPAWG meeting, it was decided by the industry to consolidate NANC 347 and 350 into a single change order that would capture abort behavior. All parties will also consider how these changes relate to the elimination of aborts (all or just time-related) and outbound flow control. The expectation is that Service Providers would implement similar abort processes/procedures on their systems, such that “sender” and “receiver” can be used to indicate either NPAC or SOA/LSMS for abort behavior. 15 minute abort behavior. The NPAC SMS and Service Provider SOA/LSMS exchange messages and a response is required for each message. The current NPAC architecture requires a response to every message within a 15 minute window, or the requestor will abort the association. If a Service Provider fails to respond to an NPAC message, the NPAC aborts that specific association and the Service Provider must re-associate in recovery mode, request, receive and process all missed messages, then start processing in normal mode until they are totally caught up with any backlog of messages. During the recovery timeframe, the NPAC must “hold” all messages destined for that Service Provider, and only send them once the Service Provider has completed the recovery process. This only further delays the desired processing of messages by both the NPAC and the Service Provider. Additionally, any SV operations except range activate will remain in a sending status until the Service Provider has competed recovery. (continued) | TBD | FRS, IIS | - Interface and Functional Backwards Compatible: YES 15 minute abort behavior. Change the 15-minute abort timer (tunable by region, defaulted to 15 minutes) to “credit” the Service Provider for responding to some traffic, even if they don’t respond to a specific message within the 15 minute window. - This would allow Service Providers that have fallen behind to keep processing the backlog, instead of getting aborted and having to re-associate to the NPAC in recovery mode, which in turn increases workload for both the NPAC and the Service Provider. - If the Service Provider fails to respond to ANY of the outstanding message during that 15 minute window, the NPAC would abort the association as is currently done (i.e., at the end of the 15 minute window). - If the SP is responding to messages at a slower pace, the NPAC using new timers, would “roll-up” the downloaded data (e.g., SV activate to LSMS with a slow SP) at the end of 15 minutes, to obtain closure on this porting activity. In this example, the SV would be in partial-failure status, and a notification would be sent to both the activating SOA and old SOA. The new timer allows the NPAC to separate association abort/monitoring and event completion. (continued) | TBD | TBD / TBD |
| 347/350 (cont) | With the current NPAC implementation based on the requirements, especially during periods of high demand with large porting activity, a Service Provider that falls more than 15 minutes behind will get aborted by the NPAC, thus exacerbating the problem of timely processing of messages. This occurs even though that Service Provider is still processing messages from the NPAC, albeit more than 15 minutes later. With this change order, the audit behavior in the 15 minute window of the NPAC would not adversely impact a Service Provider that falls behind, but is still processing messages. The business need for efficient transmission of messages will only increase as porting volumes increase. 60 minute abort behavior. With the changes described above, the audit behavior in the 60 minute window of the NPAC would allow a Service Provider to fall behind, but put a cap on how far behind (i.e., 60 minutes). This enhancement could assist a Service Provider in the area of timeliness of updating network data due to a lessening of aborts, customer service, and fewer audits for troubleshooting purposes. |  |  | This change applies to a single SV broadcast. The flow for SV ranges is a response to the range event (M-EVENT-REPORT response) within 60 minutes (same as today). 60 minute abort behavior. Create a new “60” minute window (tunable by region, defaulted to 60 minutes). Use this new window the same way that the 15 minute window is used in Release 3.1 (i.e., abort the association for a lack of a response to an individual message from the NPAC). - This would allow Service Providers that have fallen behind to keep processing the backlog, instead of getting aborted and having to re-associate to the NPAC in recovery mode, but would put a limit on the amount of time allotted for slower Service Providers. - If the Service Provider fails to respond to a given outstanding message during that new 60 minute window, the NPAC would abort the association. So with this change the Service Provider gets an additional 45 minutes to respond beyond the current 15 minute window. The logic representation is shown below: IF the slow Service Provider responds to this message within 60 minutes: NPAC updates the appropriate data NPAC sends appropriate notification to the SOAs (in an example of a partial failure activate request, the SV would go from PF to active status and the Service Provider would be removed from the failed list) ELSE, NPAC aborts the association the Service Provider must re-associate to the NPAC the Service Provider goes through recovery processing. This change applies to both single and range SV broadcasts. The SP will have 60 minutes to respond to the LSMS download message from NPAC, and in the case of an ACTION, the response to the event (M-EVENT-REPORT response) as well, or rollup at the NPAC will occur. This new timer will separate the activities, but they will both be defaulted to 60 minutes. |  |  |  |
| 347/350 (cont) | Oct ’02 LNPAWG – Major points/processing flow/high-level requirements: redlined original text and copied in updated text from APT working document (see text for May ’03 below) - The NPAC exchanges messages with the SOA/LSMS. For every request from the NPAC, a response is required from the SOA/LSMS. - The following behavior (points 3 and 4) applies to non-range broadcasts. In the case of ranges, the current 60 minute response window is still used (M_EVENT_REPORT response), but the response to the download message (confirmed mode) from the NPAC uses the behavior defined below. - The NPAC utilizes a roll-up timer for every message. The roll-up timer uses the “x by y” window (currently set to 1 by 15). The response from the SOA/LSMS is one of a, b, c below: - SOA/LSMS responds before the end of the window. - The NPAC expires the roll-up timer for that SOA/LSMS. - With a successful response, the NPAC considers this SOA/LSMS as “successful” to the request (i.e., not on failed SP list). - SOA/LSMS does NOT respond before the end of the window (i.e., expiration of the roll-up timer). - The NPAC performs “roll-up” activities for all messages sent to SOAs/LSMSs on this event (status is set, notifications to SOAs). - SOA/LSMS has any activity within this window? - If yes, the NPAC continues processing other activity, and takes no further action on this SOA/LSMS at this time. - If no, the NPAC aborts the association. - SOA/LSMS responds to request AFTER the expiration of the window. - The NPAC updates status/failed SP list, and sends notifications to SOAs. - The NPAC allows a SOA/LSMS to fall behind in processing messages. Only in the case, where NO activity is registered during the timer window, will abort processing be invoked. - The following behavior (points 6 and 7) applies to both single and range broadcasts. - The NPAC utilizes a new abort timer for every message. This timer is tunable by region. The default value is 60 minutes. The valid range is TBD (e.g., 5-120 minutes). The response from the SOA/LSMS is either a or b below: - SOA/LSMS responds before the end of the window. - The NPAC expires the timer for that SOA/LSMS. - With a successful response, the NPAC considers this SOA/LSMS as “successful” to the request. - The NPAC updates appropriate data and sends notification to the SOAs (continued) |  |  |  |  |  |  |
| 347/350 (cont) | - SOA/LSMS does NOT respond before the end of the window. - The NPAC aborts the association to the SOA/LSMS. - SOA/LSMS must re-associate to the NPAC. - SOA/LSMS goes through recovery processing (recovery based on SOA/LSMS linked replies indicator). - The NPAC updates status/failed SP list, and sends notifications to SOAs. - The NPAC allows a SOA/LSMS to fall behind in processing messages. However, the limit is defined by this new abort timer. If this timer is exceeded for any given message, the NPAC will abort the association to the SOA/LSMS. Nov ’02 LNPAWG, upon approval of the merged version of 347/350, this will be move to the accepted category. Jan ’03 LNPAWG, approved, move to accepted category. |  |  |  |  |  |  |
| 347/350 (cont) | Continuation of NANC 347/350, Abort Behavior, Proposed Resolution section: May ’03 APT Working Document – Major points/processing flow/high-level requirements: - The NPAC exchanges messages with the SOA/LSMS. For every request from the NPAC, a response is required from the SOA/LSMS. - A SOA/LSMS that fails to respond to a message is subject to Abort Processing Behavior (APB). - A new Roll-Up Activity Timer (RAT) allows for the separation between the completion of events and association abort/monitoring. There will be separate timers for single SV broadcasts versus range broadcasts. - APB applies to normal mode, not recovery mode. - RAT applies to both normal mode and recovery mode. - APB is applicable for the following types of data: SP, network, NPB, SV, notification. - No reports are required for APB. - NPAC tunables for APB allow for the separation between the completion of events and association abort/monitoring. Separate timers apply to singles versus ranges. - RAT tunable for SV singles, unit = minutes, range = 5-60, default = 15, definition = Number of minutes before roll-up activity is initiated for an event involving a single SV. - RAT tunable for SV ranges, unit = minutes, range = 5-60, default = 60, definition = Number of minutes before roll-up activity is initiated for an event involving a range of SVs. - APB Upper Threshold Tunable, unit = minutes, range = 10-1440, default = 60, definition = Number of minutes before an NPAC abort will occur for a SOA/LSMS that has at least one outstanding message with a delta between the origination time and the current time that is equal to or greater than the tunable window, regardless of whether the SOA/LSMS has incurred any other activity (request or response). - No SP specific tunables are required for APB or RAT. - SV broadcast information from NPAC to LSMS. - For a single SV broadcast: - The existing retry functionality applies. This is designed to perform existing retry behavior, and to provide the initial check for invoking an association abort of the LSMS. At the completion of the “X by Y” window, a failure to either initiate a request, or respond to any outstanding messages, results in an abort. - The single SV RAT Tunable applies. This is designed to capture roll-up activity. - The Upper Threshold Tunable applies. This will provide the secondary check for invoking an association abort of the LSMS. - For a range SV broadcast: - The existing retry functionality applies. This is designed to perform existing retry behavior, as abort processing does NOT apply because the LSMS has 15 minutes to respond to the LSMS download message from the NPAC, and in the case of an ACTION, the response to the event (M-EVENT-REPORT response) within 60 minutes as well. Therefore, range activate broadcasts will only perform abort behavior checks based on the Upper Threshold window. The response to the download message (confirmed mode) from the NPAC will still be required. - The range SV RAT Tunable applies. This is designed to capture roll-up activity. - The Upper Threshold Tunable applies. This new timer will separate the activities, but it is defaulted to 60 minutes, same as the current response window for the event. The response to the download message (confirmed mode) from the NPAC, will still be required. |  |  |  |  |  |  |
| 347/350 (cont) | Continuation of NANC 347/350, Abort Behavior, Proposed Resolution section: - The NPAC sends messages to the associated SOA/LSMS. For every message sent, abort behavior is initiated, and a RAT (response timer or event timer) is started. The initial abort timer is based on the existing retry functionality. The RAT uses either the single SV RAT tunable value or range SV RAT tunable value based on 10a and 10b above. The secondary abort timer is a new timer and it uses the Upper Threshold tunable window. The NPAC allows a SOA/LSMS to fall behind in processing messages. However, the limit is defined by this new abort timer. The response from the SOA/LSMS is one or more of the options below, based on the tunable settings: - All SOAs/LSMSs responds before the end of the retry window and RAT window. - The NPAC expires the RAT for that event. - With a successful response, the NPAC considers the responding SOA/LSMS as “successful” to the request (i.e., not on failed SP list). - All SOAs/LSMSs do NOT respond before the end of the retry window (i.e., end of the “X by Y” window). - The retry timer has expired based on the applicable retry value. - If the event was for a single SV, NPAC determines if any messages/responses were received from this SOA/LSMS during the retry window. The NPAC allows a SOA/LSMS to fall behind in processing messages. Only in the case, where NO activity is registered during the retry window, will abort processing be invoked. - If at least one message/response received, processing continues. - If no message/response received, the SOA/LSMS association is aborted. - If the event was for a range of SVs, NPAC does NOT take any action. - All SOAs/LSMSs do NOT respond before the end of the RAT window. - The RAT has expired based on the applicable value (either single or range). - The NPAC performs “roll-up” activities for all messages sent to SOAs/LSMSs on this event (status is set, notifications to SOAs). - SOA/LSMS responds to request AFTER the expiration of the RAT window. - The NPAC updates status/failed SP list, and sends notifications to SOAs. - SOA/LSMS does NOT respond before the end of the secondary abort window. - The NPAC aborts the association to the SOA/LSMS. - SOA/LSMS must re-associate to the NPAC. - SOA/LSMS goes through recovery processing (recovery based on SOA/LSMS linked replies indicator). - The NPAC updates status/failed SP list, and sends notifications to SOAs. |  |  |  |  |  |  |
| 347/350 (cont) | Continuation of NANC 347/350, Abort Behavior, Proposed Resolution section: Requirements - Roll-Up Activity-Single Tunable – NPAC SMS shall provide a Roll-Up Activity Timer – Single tunable parameter which is defined as the number of minutes before roll-up activity is initiated for an event involving a single SV. - Roll-Up Activity-Single Tunable Default – NPAC SMS shall default the Roll-Up Activity Timer – Single tunable parameter to 15 minutes. - Roll-Up Activity-Single Tunable Modification – NPAC SMS shall allow NPAC Personnel, via the NPAC Administrative Interface, to modify the Roll-Up Activity Timer – Single tunable parameter. - Roll-Up Activity-Range Tunable – NPAC SMS shall provide a Roll-Up Activity Timer – Range tunable parameter which is defined as the number of minutes before roll-up activity is initiated for an event involving a range of SVs. - Roll-Up Activity- Range Tunable Default – NPAC SMS shall default the Roll-Up Activity Timer – Range tunable parameter to 60 minutes. - Roll-Up Activity- Range Tunable Modification – NPAC SMS shall allow NPAC Personnel, via the NPAC Administrative Interface, to modify the Roll-Up Activity Timer – Range tunable parameter. - Abort Processing Behavior Upper Threshold Tunable – NPAC SMS shall provide an Abort Processing Behavior Upper Threshold tunable parameter which is defined as the number of minutes before an NPAC abort will occur for a SOA/LSMS that has at least one outstanding message with a delta between the origination time and the current time that is equal to or greater than the tunable window, regardless of whether the SOA/LSMS has incurred any other activity (request or response). - Abort Processing Behavior Upper Threshold Tunable Default – NPAC SMS shall default the Abort Processing Behavior Upper Threshold tunable parameter to 60 minutes. - Abort Processing Behavior Upper Threshold Tunable Modification – NPAC SMS shall allow NPAC Personnel, via the NPAC Administrative Interface, to modify the Abort Processing Behavior Upper Threshold tunable parameter. Message Flows None. This change order does not impact interface messaging, just behavior. Other IIS Updates. The behavior description listed in this document Chapter VII, Section C, will be added to the IIS Part I, Chapter 5 – Secure Association Establishment, new section 5.5 – Abort Processing Behavior. |  |  |  |  |  |  |
| NANC 348 | NeuStar 3/6/02 | Bulk Data Download File for Notifications Business Need: Service Providers use Bulk Data Download (BDD) files to recover customer, network, block, and subscription data in file format. This occurs when automated recovery functionality is either not available or not practical (e.g., too large of time range) for the data that needs to be recovered. The current requirements do not address BDD files for notifications. In order to provide more complete functionality for a Service Provider to “replay” messages sent by the NPAC, the ability for the NPAC to generate a BDD file for a time range of notifications would potentially reduce operational issues and the work effort required for a Service Provider to get back in sync with the NPAC, by providing the Service Provider with all information that they would have received had they been associated with the NPAC. Additionally, this would be needed for LTI users transitioning to a SOA, or SOA users that need to recover notifications for more than the industry-recommended timeframe of 24 hours. With this change order, the NPAC would have the capability to generate a BDD file of notifications for a Service Provider within a certain date and time range. | TBD | FRS | - Interface and Functional Backwards Compatible: YES The NPAC would provide the functionality for NPAC Help Desk personnel to generate a BDD file of notifications for a requesting Service Provider. Selection criteria would be any single SPID, date and time range (notification attempt timestamp), and include all types of notifications. The sort criteria will be chronologically by date and time. The file name will contain an indication that this is a notification file, along with the requested date and time range. The output file would be placed in that Service Provider’s ftp site directory. | TBD | TBD / TBD |
| 348 (cont) | Oct ’02 LNPAWG – Major points/processing flow/high-level requirements: redlined original text and copied in updated text from APT working document (see text for May ’03 below) - The request for a BDD is originated by an SP, and follows M&P steps on contacting NPAC personnel, and providing required information. - The GUI allows: - NPAC personnel to generate a BDD for notifications for a requesting Service Provider. - Time-based delta BDD files to be generated. - Selection criteria includes requesting Service Provider, time range based on notification attempt timestamp (available data based on retention/aging interval), and TN range. - The BDD file: - Contains results based on the selection criteria. - Sorted in date/time/notification type order. - Uses SP Profile flags for ranges, and notification types. - Uses NPA-NXX filters. - File name indicates notification file and requested date and time. - The results file is put in the requesting Service Provider’s FTP sub-directory. Nov ‘02 LNPAWG – Reviewed at meeting, move to accepted. Start working on detailed requirements. |  |  |  |  |  |  |
| 348 (cont) | Continuation of NANC 348, BDD for Notifications, Proposed Resolution section: May ’03 APT Working Document – Major points/processing flow/high-level requirements: - The request for a BDD is originated by an SP, and follows M&P steps on contacting NPAC personnel, and providing required information. - The GUI allows: - NPAC personnel to generate a BDD for notifications for a requesting Service Provider. - Only time-based delta BDD files to be generated. - Selection criteria include requesting Service Provider, time range based on notification attempt timestamp (available data based on retention/aging interval). - The BDD file: - Contains results based on the selection criteria. - Sorted in date/time/notification type order. - Uses SP Profile flags for ranges, and notification types (at the time the notification was created). - Uses NPA-NXX filters (at the time the notification was created). - File name indicates notification file and requested date and time. - Uses variable length records to accommodate the various notifications that are of different lengths. - The results file is put in the requesting Service Provider’s FTP sub-directory. - The amount of historical data available for the results file will be based on housekeeping processes, and the notification purge tunable value. Mar ’03 APT: Other than the need to capture the variable length records, the rest of the text captures the desired functionality. Requirements - File Creation – NPAC SMS shall provide a mechanism that allows a Service Provider to recovery notification data in file format. - Selection Criteria Fields – NPAC SMS shall include the requesting Service Provider and a time range, as selection criteria fields for the Notification bulk data download file, via the NPAC Administrative Interface. - Required Selection Criteria – NPAC SMS shall require, as selection criteria for notification bulk data download file generation, a requesting Service Provider ID and a time range. - File Name – NPAC SMS shall provide a bulk data download file for notification data, use a file name that indicates the Notification data and requested time range. - Time Range – NPAC SMS shall use the Start Time Range entry field as an exclusive start range, and the End Time Range entry field as an inclusive end range, for Notification data that were broadcast during the specified time range, based on notification attempt timestamp. - Results – NPAC SMS shall provide a bulk data download file, based on selection criteria, that contains all Notification data in the NPAC SMS. - Sort Order – NPAC SMS shall sort the Notification bulk data download file, in ascending order based on the value for data/time/notification type. - Filters – NPAC SMS shall apply SP Profile Flags for ranges and notification type (based on the settings at the time the notification was created). - FTP Sub-Directory – NPAC SMS shall automatically put the Notification bulk data download file into the FTP sub-directory of the Service Provider, based on the SPID value of the requesting Service Provider. Message Flows None. |  |  |  |  |  |  |
| NANC 351 | NeuStar 4/12/02 | Recovery Enhancements – “Send What I Missed” recovery message Business Need: The NPAC SMS and Service Provider SOA/LSMS exchange messages and a response is required for each message. The current NPAC architecture requires a response to every message within a 15-minute window, or the requestor will abort the association. If a Service Provider fails to respond to an NPAC message, the NPAC aborts that specific association and the Service Provider must re-associate in recovery mode, request a “best guess” time range of missed messages from the NPAC, receive and process all missed messages, then start processing in normal mode until they are totally caught up with the backlog of messages. One problem of the current “best guess” approach is the trial-and-error recovery processing that a Service Provider must perform in certain circumstances (e.g., when there is too much data to send in a response to a single request). This can create unnecessary workload on both the NPAC and the Service Provider. A better method is to implement the “Send What I Missed” approach (SWIM). Service Providers can optionally use this new message to perform the recovery function. This improves the efficiency of recovery processing for the NPAC and Service Providers because guesswork is eliminated. | TBD | FRS, IIS, GDMO, ASN.1 | - Interface and Functional Backwards Compatible: YES Create a new process that incorporates the ability for a Service Provider to request that the NPAC send missed messages. In order to accomplish this, the NPAC will need to keep track of messages that were both “not sent” and “not responded to” from the NPAC to the SOA/LSMS. The behavior of the “Send What I Missed” message (SWIM) which will be initiated by a SOA/LSMS, is the same as the current recovery process (i.e., request from the SP, response from the NPAC includes the recoverable data). The implementation would use the existing recovery message, and incorporate a new attribute (SWIM, to go along with time range and TN range). When this is received, the NPAC would send back a SWIM Response which contains the missed messages. With the new SWIM attribute, the NPAC would use the same Blocking Factor tunables as used in 187-Linked Replies in order to send data to the SOA/LSMS in “chunks”. | TBD | TBD / TBD |
| 351 (cont) | Oct ’02 LNPAWG – Major points/processing flow/high-level requirements: redlined original text and copied in updated text from APT working document (see text for June ’03 below) - Adding a new send me what I missed message (new Action). Capable of recovering network, subscription, number pool block, and notification data. This is an optional message. - New SPIDables are added to define the maximum number of messages that can be recovered under this new message. - Network Data-based. - SV-based. - NPB-based - Notification-based. - Service Providers can use the existing recovery mechanism/messages (lnpDownload, lnpNotificationRecovery), or this new message to recover missed data between the SOA/LSMS and the NPAC. - This new message can be used by both 187-Service Providers (linked replies will be sent), and non-187-Service Providers (regular non-linked reply will be sent). - The NPAC will keep track of messages destined for a SOA/LSMS that were NOT successfully responded to by the SOA/LSMS. - SOA/LSMS associates to the NPAC and uses the new message. The NPAC: - Determines the messages missed by the requesting SOA/LSMS. - Validates maximum recovery size (if other the max size, an error message is returned). - Uses SP Profile flags for ranges, notification types, EDR, linked replies. - Applies appropriate NPA-NXX filters. - Sends the data. - Updates status/failed SP list, and sends notifications to SOAs. - Upon completion of recovery, SOA/LSMS sends existing recovery complete message (lnpRecoveryComplete), and processing between SOA/LSMS and NPAC continues in normal mode. - Additional: Need to discuss need for switch me to recovery mode message (new Action). Nov ‘02 LNPAWG – Reviewed at meeting, move to accepted. Start working on detailed requirements. Also, everyone needs to consider a new message from the NPAC (“you need to recover some missing data”). This will be discussed once detailed requirements are drafted. |  |  |  |  |  |  |
| 351 (cont) | Continuation of NANC 351, SWIM Recovery, Proposed Resolution section: June’03 APT Working Document – Major points/processing flow/high-level requirements: - This recovery enhancement will use the current recovery process and ASN.1 definitions. Any exceptions will be noted. - This recovery enhancement will implement a new attribute in the current recovery ACTION messages (lnpDownload, lnpNotificationRecovery). Both of these are optional functionality. - Add a new Send What I Missed criteria (SWIM). This new criteria is initiated by a recovering SOA/LSMS, and allows for the recovery of network, subscription, number pool block, and notification data. SWIM will be sent by the recovering SOA/LSMS as part of their association bind request. The NPAC will reply back to the originating SOA/LSMS with the missed data, by using linked replies. This message can only be sent when the SOA/LSMS is in recovery. - The recovering SP will be required to submit SWIM requests for the different types of data, e.g., SWIM for network data, then SWIM for SV data, then SWIM for notification data. - An action ID will be added. This will be generated by the NPAC and sent in the SWIM response linked replies. Upon completion of each type of data, the requesting SOA/LSMS will respond back with the action ID (for each type of data, using an ACTION with the action ID). Upon receipt, the NPAC will remove the SP from the failed list and the “missed” list. May ’03 – Action ID is optional. It will be sent in the last message with data (then followed by empty reply). A separate M-EVENT-REPORT will be sent back by the SP with the Action ID to indicate the replies were successfully processed. This is similar to the current behavior for range activates. - No reports are required for this recovery enhancement. - NPAC regional tunables. - For SWIM requests, the existing 187 Blocking Factor and Maximum tunables will be used by the requesting SOA/LSMS. - A new “SWIM Maximum” tunable will be added that will allow a larger number of missed messages than the current 187 Maximum. However, these will need to be recovered in separate requests. A new M&P will be added to inform an SP when they reach 80% (tunable value) of this SWIM Maximum. May ’03 – In the scenario where a SOA/LSMS reaches the maximum (“crit-too-large” msg), the NPAC would clear out the list, and set some indicator that they can’t recover using this mechanism anymore. Additionally, have functionality to be able to reset the collection mechanism, and start capturing missed messages again. Move this to regular working group. Next month start on reqs for fleshing out the mechanism to drill down into this. - A new “continuation” indicator will be added to the 187 functionality to inform the requesting SOA/LSMS that they exceeded the 187 maximums and need to perform an additional request(s). - A new SP profile flag is added to define whether or not an SP supports the SWIM message set. Once the flag is set to TRUE, history data will be stored that allows for the implementation of SWIM. - Service Providers can continue to use the existing recovery mechanism/messages (lnpDownload, lnpNotificationRecovery) to recover missed data between the SOA/LSMS and the NPAC, using the current Time Range or TN Range criteria. - The NPAC will keep track of messages destined for a SOA/LSMS that were NOT successfully responded to by the SOA/LSMS, once the SP Profile Flag is set to TRUE, and as long as it remains TRUE. If modified from TRUE to FALSE, the NPAC will no longer maintain a “missed messages” list for that SOA/LSMS. |  |  |  |  |  |  |
| 351 (cont) | Continuation of NANC 351, SWIM Recovery, Proposed Resolution section: - SOA/LSMS associates to the NPAC and uses SWIM criteria. The NPAC: - Determines the messages missed by the requesting SOA/LSMS - Uses SP Profile flags for ranges, notification types, EDR - Applies appropriate NPA-NXX filters - Packages up and sends the maximum data given the different variables and tunable settings (NPAC SWIM Response to SOA/LSMS Recovery Request message). The recovering SOA/LSMS processes each SWIM Response message (separate messages by type of data, and possibly multiple messages for any given type of data). This process continues until all missed data has been sent to the requesting SOA/LSMS. - Updates status/failed SP list, and sends notifications to SOAs - Upon completion of recovery, SOA/LSMS sends an lnpRecoveryComplete message (current functionality) indicating the end of the missed data. At this point in time, processing between SOA/LSMS and NPAC continues in normal mode. - If implemented in conjunction with or after NANC 352 (Recovery of SPID), then that functionality will also be included in this change order. Requirements TBD. Message Flows TBD. |  |  |  |  |  |  |
| NANC 352 | NeuStar 4/12/02 | Recovery Enhancements – recovery of SPID (customer data) Business Need: The NPAC SMS allows for the recovery of missed messages for network data, block data, and SV data. However, the NPAC functionality based on current requirements does not allow recovery of customer information (SPIDs). So, if customer information is downloaded, and the Service Provider misses it, it is not recoverable. This new functionality would improve the recovery process by adding customer (i.e., header data) to the list of recoverable messages, so that subordinate network/block/SV data does not cause rejects or errors. | TBD | FRS, IIS, GDMO, ASN.1 | Interface and Functional Backwards Compatible: YES Implement a new optional recovery request that allows the Service Provider to recover customer information (SPIDs). This new optional feature would send missed customer adds, modifies, or deletes to the Service Provider during the recovery process. A Service Provider could implement this optional feature at any time, and would send this request during the recovery process similar to the requests sent for network, block, and SV data today. The data representation would be something like, SPID, text, and download reason. Nov ‘02 LNPAWG – Reviewed at meeting, move to accepted. Start working on detailed requirements. | TBD | TBD / TBD |
| NANC 352 (cont) | Continuation of NANC 352, Recovery of SPID, Proposed Resolution section: May ’03 APT Working Document – Major points/processing flow/high-level requirements: - This recovery of SPID enhancement will implement a new recovery request type. This will be used with the lnpDownload message. This is optional functionality. - This recovery of SPID enhancement only applies to recovery mode, not normal mode. - No reports are required for this recovery enhancement. - The data representation would include, SPID, SP name, and download reason. - NPAC regional tunables will be added for 187-Linked Replies capable Service Providers (maximum recoverable data, Blocking Factor). - No NPAC SPIDables are required for this recovery enhancement. - This new request type can be used by both 187-Service Providers (linked replies will be sent), and non-187-Service Providers (regular non-linked reply will be sent). - SOA/LSMS associates to the NPAC and uses the new request type with the lnpDownload message. The NPAC: - Validates the message by the requesting SOA/LSMS - Validates maximum recovery size (if over the max size, an error message is returned) - Uses SP Profile flags for linked replies - Skips checks for SP Profile flags for ranges, notification types, EDR, and skips check for NPA-NXX filters - Packages up and sends the maximum data given the different variables and tunable settings. This process continues until all requested recoverable data has been sent to the requesting SOA/LSMS. - Upon completion of recovery, SOA/LSMS sends existing recovery complete message (lnpRecoveryComplete), and processing between SOA/LSMS and NPAC continues in normal mode. Requirements - Modification of existing requirements – The words of the requirements (approximately dozen) in the Network Data Recovery section, will be modified to include SPID. - Linked Replies – The words of the linked replies requirements will be modified to include SPID. Message Flows - Modification of existing IIS Flows – The flow pictures for recovery remain the same, i.e., M-ACTION Response (network data). The words of the flow descriptions will be changed to include SPID. |  |  |  |  |  |  |
| NANC 355 | SBC 4/12/02 | Modification of NPA-NXX Effective Date (son of ILL 77) Business Need: When the NPAC inputs an NPA Split requested by the Service Provider and the effective date and/or time of the new NPA-NXX does not match the start of PDP, the NPAC cannot create the NPA Split in the NPAC SMS. To correct this problem the NPAC can contact the Service Provider and have them delete and re-enter the new NPA-NXX specified by the NPA Split at the correct time, or the NPAC can delete and re-enter the NPA-NXX for the Service Provider. However, the NPA-NXX may already be associated with the NPA Split at the Local SMS, and the subsequent deletion of the NPA-NXX will cause that specific record to be old time-stamped. When the NPA-NXX is re-created, that new record will have a different time stamp, and it requires a manual task for the Service Provider to search for new NPA-NXX records which might match the NPA Split. If identified and corrected, it will be added. If not identified, it will affect call routing after PDP. |  | FRS, IIS, GDMO | Func Backwards Compatible: NO This activity would only be allowed by NPAC personnel, via the GUI, to modify the NPA-NXX Effective Date. At the time of modification request, all existing pending subscription versions must have a due date greater than the new effective date in order for the change to occur. If one or more pending subscription versions have a due date less than the new effective date, a change would not be made and an error message would be returned to the NPAC user. It would be the responsibility of the owner of the NPA-NXX to resolve issues of pending versions with due dates prior to the new effective date before a change could be made. For valid requests, the NPAC will notify the SOA/LSMS of a modified effective date (M-SET). Jan ’03 LNPAWG, approved, move to accepted category. | Med-Low | TBD / TBD |
| NANC 357 | Bellsouth 4/12/02 | Unique Identifiers for wireline versus wireless carriers (long term solution) Business Need: In the LSR process, there is a need to identify a Service Provider’s port request as that from or to a Wireline or Wireless Service Provider in order to process the port request correctly within internal systems. This information must match up with NPAC information on each Service Provider’s Type. Without this information, port requests may be handled incorrectly thus effecting customer phone service including related E911 records. This is especially crucial in fully mechanized LSR processing systems. This long-term solution replaces the interim solution provided by the associated NANC Change Order, 356. |  | FRS, IIS, GDMO | Func Backwards Compatible: NO The NPAC SMS shall provide a Service Provider Type indicator for each Service Provider. This new indicator shall initially distinguish each Service Provider as either a Wireline Service Provider or a Wireless Service Provider. The Service Provider Type indicator shall be able to distinguish additional “types” as deemed necessary in the future (e.g., it may be advantageous in the future to identify other Service Provider Types such as Reseller or Service Bureau). This information shall be sent to the SOA/LSMS upon initial creation of the Service Provider, upon modification of a Service Provider’s Type and when the SP is removed (deleted) from the NPAC. The Service Provider Type indicator shall be added to the Bulk Data Download file, available to a Service Provider’s SOA/LSMS. The Service Provider Type indicator shall be Recoverable across the SOA/LSMS with the implementation of NANC 352. Jan ’03 LNPAWG, approved, move to accepted category. | Med-Low | TBD / TBD |
| NANC 358 | NeuStar 4/12/02 | Change for ASN.1: Change SPID definition Business Need: The current ASN.1 definition allows the SPID to be variable 1-4 alphanumeric characters. The current behavior in the NPAC requires SPID to be four alphanumeric characters, as defined in the current data model in the FRS – a “New Service Provider ID, Character (4), Old Service Provider ID, Character (4)”, and the GDMO “Valid values are the Facilities Id (or OCN) of the service provider.” The OCN in the GDMO is the same OCN as defined by OBF (http://www.atis.org/pub/clc/niif/nrri/issue177/MACompany%20Code.doc): “Company Code/Operating Company Number (OCN) - A unique four-character alphanumeric code assigned by NECA that identifies a telecommunications service provider, as outlined in the ANSI T1.251 standard, Identification of Telecommunications Service Provider Codes for the North American Telecommunications System. The code set is used in mechanized systems and documents throughout the industry to facilitate the exchange of information. Company Codes assigned by NECA are referred to as OCNs in Telcordia’s BIRRDs system. NANPA requires a carrier’s Company Code in order to obtain numbering resources. The FCC requires a carrier’s Company Code on FCC Form 502, the North American Numbering Plan Numbering Resource Utilization/Forecast Report.” This change order will correct the ASN.1 definition to match the current implementation. |  | ASN.1 | Func Backwards Compatible: YES Current ASN.1 definition: ServiceProvId ::= GraphicString4 GraphicString4 ::= GraphicStringBase(SIZE(1..4)) New ASN.1 definition (new is bold): ServiceProvId ::= GraphicFixedString4 GraphicFixedString4 ::= GraphicStringBase(SIZE(4)) Jan ’03 LNPAWG, approved, move to accepted category. | Low | TBD / TBD |
| NANC 363 | NeuStar 6/14/02 | Lockheed-to-NeuStar private enterprise number: Change to NeuStar registration number. Business Need: The current ASN.1 uses the Lockheed Martin private enterprise number. This needs to be changed to the NeuStar registration number, as was provided by IANA (Internet Assigned Number Authority). The following three areas in the ASN.1 will be changed: LNP-OIDS {iso(1) org(3) dod(6) internet(1) private(4) enterprises(1) lockheedMartin(103) cis(7) npac(0) iis(0) oids(0)} lnp-npac OBJECT IDENTIFIER ::= {iso(1) org(3) dod(6) internet(1) private(4) enterprises(1) lockheedMartin(103) cis(7) npac(0)} -- LNP General ASN.1 Definitions LNP-ASN1 {iso(1) org(3) dod(6) internet(1) private(4) enterprises(1) lockheed(103) cis(7) npac(0) iis(0) asn1(1)} |  | ASN.1 | Func Backwards Compatible: NO Change the current ASN.1 definition from lockheedMartin (103) to NeuStar (13568). Jan ’03 LNPAWG, approved, move to accepted category. Need to get SOA/LSMS vendor feedback during Feb ’03 LNPAWG meeting. Feb ’03 LNPAWG, SOA/LSMS vendor feedback. Colleen Collard (Tekelec), more than a recompile, but LOE is low. Logistical implementation an issue since non-backwards compatible (for vendors with single platform and different regions with different implementation dates). Need to consider efficiency of roll-out. To alleviate this problem would need all regions upgraded at same time. Burden will be somewhere for someone to support both (either NPAC or vendor side). This change should be incorporated at the next regular release, and not during it’s own release. | TBD (change to TBD, since NPAC may support both old and new number. Would set short sunset | Low / Low |
| NANC 368 | NeuStar 10/18/02 | Outbound Flow Control Business Need: - During the Oct ’02 LNPAWG meeting, a discussion took place surrounding outbound flow control, and the merits of changing the flow control of messages from the receiving end to the sending end. The current implementation of flow control between the NPAC and SOA/LSMS systems is completely determined by the receiving end of the CMIP connection. This approach works, but it allows the large buffers between the sender and the receiver to act as a queue when the receiver can’t keep up with the sender. These buffers allow for, in some cases, hundreds of messages to be backed up between the sender and the receiver before the sender gets a congestion indication. In some cases, the queue that builds up cannot be processed in 5 minutes, thereby causing departure times to expire and the association to be aborted. - Another negative impact of the current flow control approach is the lack of ability to correctly prioritize outbound messages. In the LNP systems, the sender, not the OSI stack, manage the priority that is assigned to a message. Once a large backlog of low priority messages is built up, any subsequent high priority message must wait for all those messages ahead of it in the queue. If the sender carefully manages the outbound queue, then high priority messages won’t have to wait as long to be sent by the receiving system. Refer to the Oct ’02 LNPAWG meeting minutes for a full recap of the discussion items regarding this topic. |  | FRS, IIS | Pure Backwards Compatible: YES By implementing outbound flow control on the sender system, the various buffers in the OSI stack would not fill up as done currently. It would be the sender’s responsibility to detect that (n) number of messages have been sent without receiving a response. In this case, the sender should stop sending until the number of non-responsive messages drops below a threshold (t). If implemented on both ends (NPAC and SP), outbound flow control would prevent congestion because neither side would fill the buffers between the 2 systems. Oct ’02 LNPAWG, outbound flow control could be implemented at the NPAC without impacting Service Provider systems. Service Providers are not required to implement this feature concurrently with NPAC. Nov ‘02 LNPAWG, Outbound Flow Control would be set up for every connection to the NPAC. Message processing speed and message prioritization for each SP is independent of other SPs (just like today, where one slow SP doesn't mean others are directly affected), regardless of each SP's setting. Move to accepted. Start working on detailed requirements. Feb ’03 APT Meeting, need to consider how the implementation of Outbound Flow Control would affect SLRs 2, 3, 4, and 5. | TBD | TBD / TBD |
| NANC 368 (cont) | Continuation of NANC 368, Outbound Flow Control, Proposed Resolution section: May ’03 APT – Major points/processing flow/high-level requirements: - Flow Control will be implemented on the NPAC side of the CMIP interface, no action or modification is required by the SOA/LSMS. - The implementation of Flow Control by the sending system is independent of any implementation by the receiving system. However, there is a clear benefit to having both sides implement this functionality. - Flow Control is applicable on a per association basis. - Flow Control activity and behavior applies to both normal mode and recovery mode. - Flow Control activity is applicable for the following types of data: SP, network, NPB, SV, notification. - No reports are required for Flow Control. - NPAC tunables for Flow Control include: - Flow Control Upper Threshold Tunable, unit = messages, range = 50-500, default = 100, definition = Number of non-responsive messages sent to a SOA/LSMS before Flow Control is invoked, on a per association basis. - Flow Control Lower Threshold Tunable, unit = messages, range = 1-500, default = 10, definition = Number of non-responsive messages sent to a SOA/LSMS that is in a Flow Control state before normal processing is resumed, on a per association basis. - The NPAC sends messages to the associated SOA/LSMS. - Under normal conditions where the SOA/LSMS is able to keep up with the NPAC, Flow Control is not encountered. - Under some load conditions, the SOA/LSMS is not able to keep up with the messages sent from the NPAC. In this situation, Flow Control is encountered. - NPAC implements a real-time flag indicating whether a SOA/LSMS is in a Flow Control state. - When getting ready to send a message to a SOA/LSMS, NPAC checks this flag to determine if it’s OK to send this message. - If the flag is false, the message is sent. - If the flag is true, the message is held/queued. - For a SOA/LSMS that is currently in a normal state (not in Flow Control), the NPAC monitors the number of outstanding non-responsive messages sent to that SOA/LSMS. - If the number of outstanding non-responsive messages is equal to the Flow Control Upper Threshold, the NPAC sends the current message it is handling, and sets the Flow Control flag to true. Since the check is performed on a per message basis, the Upper Threshold number will not be exceeded, just equaled. - If the number of outstanding non-responsive messages is less than the Flow Control Upper Threshold, NPAC sends the current message it is handling, and continues with normal processing. - For a SOA/LSMS that is currently in a Flow Control state, the NPAC monitors the number of outstanding non-responsive messages sent to that SOA/LSMS. - If the number of outstanding non-responsive messages is greater than the Flow Control Lower Threshold, no action is taken. - If the number of outstanding non-responsive messages is less than or equal to the Flow Control Lower Threshold, the NPAC resumes sending messages (whether queued or normal). - A SOA/LSMS that is in a Flow Control state will have outstanding non-responsive messages. - For all outstanding non-responsive messages that were sent, NPAC response timers and abort behavior will apply. For all messages not sent but held because the Flow Control flag is set to true, NPAC response timers and abort behavior will NOT apply. |  |  |  |  |  |  |
| NANC 368 (cont) | Continuation of NANC 368, Outbound Flow Control, Proposed Resolution section: - Audit behavior in an Outbound Flow Control environment. - An audit to an LSMS that is in a Flow Control state will behave as it currently does for a congestion situation (i.e., the audit would be initiated, queries would be generated, and once the message is sent it would timeout after 15 minutes if neither responded or aborted within 15 minutes). The audit will continue to be serial, and will only move to the next step once the current step is completed. - All other audit behavior remains the same. May ’03 – FIFO of messages remains the same. (within priority groups) |  |  |  |  |  |  |
| NANC 382 | NeuStar 4/4/03 | “Port-Protection” System - (The following is the original request. Subsequent modifications were made during several LNPAWG meetings. Refer to the bottom of this change order for the current version.) Overview: The “Port Protection” system is a competitively neutral approach to preventing inadvertent ports that gives end-users the ability to define their portable telephone numbers as “not-portable.” The NPAC SMS enforces the “not-portable” status of a telephone number so long as it remains in effect. No Local Service Provider (LSP) can invoke or revoke “port protection” on a working telephone number; end-users completely control the portability of their portable telephone numbers. Business Need: Inadvertent porting of working numbers is a concern to both Local Service Providers (LSPs) and their customers. In today’s LNP environment, an LSP cannot absolutely assure its customers that their terminating service will not be interrupted, even if it can insure that physical plant is operated without failure. This is because any LSP by mistake may port a telephone number away from that number’s current serving switch. The inadvertent port can occur in a number of ways, but the most common occurrences appear to be caused by two errors: (1.) when the wrong telephone number submitted to NPAC for a conventional inter-SP port, and (2.) when intra-SP ports are not done before a pooled block is created. There is a similar inadvertent port problem for non-working numbers, but erroneous moves of non-working numbers are not directly service-affecting and are not addressed here. NeuStar suggests the following competitively neutral method to prevent inadvertent ports of working TNs. | TBD | FRS, IIS, GDMO, ASN.1 | Interface and Functional Backwards Compatible: NO Description of Change: - (The following is the original request. Subsequent modifications were made during several LNPAWG meetings. Refer to the bottom of this change order for the current version.) See next page. | TBD | TBD / TBD |
| NANC 382 (con’t) | Continuation of NANC 382, Port-Protection System, Proposed Resolution section: -- System Architecture -- Changes to the NPAC SMS are required, to establish a table of “Port-Protected TNs” in which portable numbers that no longer can be ported are listed. A step must be added to the NPAC SMS’s validation process in order to check this new table whenever an inter-SP port or pooled block create is attempted. An interface change could be required as well if industry wishes to know when a request’s rejection is due to the involved number being on the “Port Protection” list. Creation of an IVR system is required, to receive end-user requests for protection of their numbers from porting (or to remove this protection) and to relay the information to the NPAC SMS. The system would automatically modify the NPAC’s “Port-Protection” tables based on the end-user requests it receives. Access to the IVR would be through the end-user’s current LSP customer rep. Any other LSP willing to assist the end-user could be involved. The end-user’s telephone number is entered in the NPAC’s “Port Protection” tables whenever “port-protection” is requested. The end-user cannot reach the “Port-Protection” IVR system directly, but instead must be connected through a local Service Provider’s customer contact system, much like what is done in the PIC selection process, where the Service Provider’s customer rep advances the call to a third-party verification service, then leaves the call to allow the third-party verifier and end-user to converse. The IVR system must recognize the LSP as authorized to participate in the “Port Protect” process. (The LSP need not be a facility-based provider.) Arrangements for security handshakes must be made in advance with each participating LSP. A telephone number may be added to or removed from the “Port Protection” list whenever and as often as the end-user wishes. To maintain the proposal’s competitive neutrality, the process assumes any LSP may assist the end-user. However, the possibility of end-users invoking or revoking “Port Protection” on telephone numbers other than their own would be mitigated if only an LSP with which the end-user had a contractual relationship could particpate, i.e., only the current LSP or a new LSP in a pending port request situation. (con’t) |  |  |  |  |  |  |
| NANC 382 (con’t) | Continuation of NANC 382, Port-Protection System, Proposed Resolution section: -- System Operation -- The end-user’s telephone number is entered in the NPAC’s “Port Protection” tables whenever “port-protection” is requested. The end-user cannot reach the “Port-Protection” IVR system directly, but instead must be connected through a local Service Provider’s customer contact system, much like what is done in the PIC selection process, where the Service Provider’s customer rep advances the call to a third-party verification service, then leaves the call to allow the third-party verifier and end-user to converse. The IVR system must recognize the LSP as authorized to participate in the “Port Protect” process. (The LSP need not be a facility-based provider.) Arrangements for security handshakes must be made in advance with each participating LSP. A telephone number may be added to or removed from the “Port Protection” list whenever and as often as the end-user wishes. To maintain the proposal’s competitive neutrality, the process assumes any LSP may assist the end-user. However, the possibility of end-users invoking or revoking “Port Protection” on telephone numbers other than their own would be mitigated if only an LSP with which the end-user had a contractual relationship could particpate, i.e., only the current LSP or a new LSP in a pending port request situation. When the NPAC attempts to create a pending SV or a pooled block, the NPAC will check the “Port Protection” list in its validation process for inter-SP port (including Port-to-Original) and “-X” create requests. The “Port Protection” validation does not occur for intra-SP ports. These may represent inadvertent ports, but validation necessary to determine whether override would be appropriate is not feasible. The validation occurs for only those deletes that are “Port-to-Original” situations. (con’t) |  |  |  |  |  |  |
| NANC 382 (con’t) | Continuation of NANC 382, Port-Protection System, Proposed Resolution section: -- Process Flow -- The end-user contacts an LSP (or an LSP contacts the end-user). (It is not inherently necessary for there to be Service Provider involvement in this process, but NeuStar is not prepared to operate a system which does not involve LSP participation.) End-user indicates desire to invoke (or revoke) “Port Protection.” LSP customer rep places end-user on hold and calls the “Port-Protection” IVR. LSP provides its pre-assigned ID information to IVR system. (LSP arrange for security codes before attempting to assist end-users with the “Port-protection” process.) LSP brings end-user on to the active line and leaves call; end-user interacts with IVR. Using a standard script, the IVR confirms caller is authorized to make changes to the telephone number account, determines the caller’s name, and lists the telephone number(s) to be added to (or removed from) the “port-protection” table. The customer may actually enter the TN desired. The call is recorded. The IVR system then enters this information into an automated ticket system. Completion of the ticket automatically sends triggers an update of the NPAC’s “port-protection” table. In the case of a number that has been entered in the port-protection table, but is no longer assigned to an end-user, the current Service Provider itself can ask that the number be removed from the “port-protection” table. The provider would have to be recognized by the NPAC as the code/block owner and would have to state that the number is not assigned to an end-user. |  |  |  |  |  |  |
| Continuation of NANC 382, “Port-Protection” System - This change order was reviewed and revised during the May through Sep ’03 LNPAWG meetings. The final version of the open change order at the time of acceptance (for development of more detailed information) is shown below: Overview: The “Port Protection” system is a competitively neutral approach to preventing inadvertent ports. The system makes it possible for end-users to define their portable telephone numbers as “not-portable.” The NPAC SMS prevents the port of a “not-portable” telephone number (TN) through its automated validation processes. A Local Service Provider (LSP) can invoke or revoke “port protection” for a working TN, but only at the end-user’s request. Business Need: Inadvertent porting of working TNs is a concern to both Local Service Providers (LSPs) and their customers. In today’s LNP environment, an LSP cannot absolutely assure its customers that their terminating service will not be interrupted, even if it can insure that the physical plant is operated without failure. This is because another LSP by mistake may port a TN away from that number’s current serving switch. The inadvertent port can occur in a number of ways, but the most common occurrences appear to be caused by two errors: (1.) the wrong TN is submitted to the NPAC SMS for a conventional inter-SP port, and (2.) intra-SP ports are not done before a thousands-block is created. There are similar inadvertent port scenarios for non-working TNs, but erroneous moves of non-working TNs are not immediately service-affecting and are not addressed here. NeuStar suggests the following competitively neutral method to prevent inadvertent ports of working TNs. |  |  |  | Interface and Functional Backwards Compatible: NO - This change order was reviewed and revised during the May through Sep ’03 LNPAWG meetings. The final version of the open change order at the time of acceptance (for development of more detailed information) is shown below: Description of Change: -- System Architecture -- Changes to the NPAC SMS are required to establish a table of “Port Protected” TNs, in which portable numbers that no longer can be ported are listed, and to add a validation step that rejects attempts to port a TN that is on the list. The validation is performed on the new-SP’s Create message for an inter-SP port, when a thousands block is created, and, optionally, for an intra-SP port. (The optional intra-SP port validation is invoked on a SPID-specific basis.) The rejection notification sent when a request fails this NPAC SMS validation will indicate that the TN is on the Port Protection list. No interface change is required for this rejection message, since a new optional attribute will be added to accommodate the new error text. LSP requests to add TNs to the Port Protection table are made to the NPAC Help Desk via e-mail (the TNs involved are shown on an Excel attachment to the e-mail message). LSPs use the same approach to delete TNs from the table. (con’t) |  |  |  |
| NANC 382 (con’t) | Continuation of NANC 382, Port-Protection System, Proposed Resolution section: -- System Operation -- A TN is added to the NPAC’s Port Protection table when an LSP requests this action. The same process applies when an LSP requests the removal of a TN from the table. The NPAC Help Desk accepts requests to change Port Protection table entries only from pre-authorized representatives of an LSP. (The LSP need not be a facility-based provider.) A TN may be added to or removed from the “Port Protection” list as often as required. When the NPAC SMS receives the new SP’s Create request, it will check the Port Protection table during the Pending SV Create validation process for inter-SP ports (including Port-to-Original SV deletes). Optionally, the validation is performed for intra-SP ports. The NPAC SMS also will make this validation check in connection with “-X” create requests. The validation is not applied to Modify requests In the disconnect scenario, the NPAC SMS will check the Port Protection list and, if the TN is found, will remove the involved disconnected ported TN from the list. This automatic removal of a disconnected TN from the Port Protection list can occur only in the case of a disconnected TN that was ported. A non-ported TN that is disconnected must be removed from the list by the LSP having the disconnected non-ported TN in its inventory. (con’t) |  |  |  |  |  |  |
| NANC 382 (con’t) | Continuation of NANC 382, Port-Protection System, Proposed Resolution section: -- Process Flow -- # NPAC Help Desk - The end-user contacts an LSP (or an LSP contacts the end-user). - End-user indicates to LSP his desire to invoke (or revoke) “Port Protection.” - LSP contacts NPAC Help Desk via e-mail to request change. - The NPAC Help Desk updates the Port Protection table. # NPAC SMS - NPAC SMS applies the Port Protection validation (1.) to the new-SP Create request of an inter-SP port, (2.) to a Block Creation request, and (3.) optionally at the individual SPID level, to an intra-SP port request. If the TN is found on the Port Protection list, NPAC SMS rejects the request and indicates that a Port Protection validation failure is the reason for the request’s rejection. - Disconnect of a ported TN results in automatic removal of the TN from the Port Protection list; disconnect of a non-ported TN requires owning LSP to request the disconnected TN’s removal from the list. - An LSP’s regional NPAC SMS Profile indicates whether the Port Protection validation should be applied also to its intra-SP port requests. |  |  |  |  |  |  |
| 382 (cont) | Continuation of NANC 382, “Port-Protection” System, Proposed Resolution section: CMA Write-Up to review at Nov’03 LNPAWG – Major points/processing flow/high-level requirements: - There are open issues that need to be resolved regarding the implementation details. The current write-up makes no assumptions regarding the resolution to these open issues. - A request (delivery method TBD, interface TBD) is received to add a TN to the Port-Protect List (PPL). - NPAC validates the request for required data. - NPAC verifies the number is not already on the PPL. - If yes, send error message indicating TN already on PPL, and exit the process. - If no, continue. - NPAC adds the TN to the PPL. - NPAC receives a request (SOA Interface or LTI) to Create an inter-Service Provider SV. - Create is from New SP. - NPAC checks to see if TN on PPL. - If yes, send error message indicating TN on PPL, so number is protected, and exit the process. - If no, continue with Create process. - Create is from Old SP. - NPAC checks to see if TN on PPL. - If yes, remove from PPL, then continue with Create process. - If no, continue with Create process. - A new SP profile flag is added to define whether or not an SP supports the restriction of intra-Service Provider ports on TNs on the PPL. Once the flag is set to TRUE, the NPAC will invoke the edit shown in #6 below. - NPAC receives a request (SOA Interface or LTI) to Create an intra-Service Provider SV. Create is from Current SP (whether Code Holder for non-ported number or New SP for a currently active SV). NPAC checks SP profile flag for restricting intra-Service Provider ports on TNs on the PPL. - If yes, send error message indicating TN on PPL, so number is protected, and exit the process. - If no, continue with Create process. - NPAC receives a request (NPAC Ops GUI) to Create network data for a thousands block for Number Pooling (this is the NPA-NXX-X). Create is from the Block Holder. NPAC checks to see if any of the non-ported TNs in the thousands block are on the PPL. - If yes, send error message indicating non-ported TN on PPL, so number is protected, and exit the process. - If no, continue with NPA-NXX-X Create process. - At the time of a disconnect of a ported TN, the NPAC will check to see if the TN is on the PPL, and if yes, remove it from the PPL. |  |  |  |  |  |  |
| NANC 388 | Nextel 9/17/03 | Change a “Cancel Pending” SV Business Need: Currently there are no requirements in the NPAC that allow a Subscription Version (SV) to be manually changed from “Cancel Pending” status to “Pending” status. Without any “un-do” functionality, both Service Providers (SPs) must wait for the Cancellation-Initial Concurrence Window and the Cancellation-Final Concurrence Window to expire (nine hours each), let the SV go to Conflict, and then resolve the Conflict or wait for the Conflict Restriction timer (six hours) to expire in order for it to return to “Pending” (when the Cancel Request was initiated by the Old SP). Alternatively, both SPs could send in cancel requests to the NPAC, at which point the SV would immediately go to “Canceled”, then they could initiate the porting process again. The current NPAC functionality for a concurred port (where both SPs have sent in Create Requests and the SV is in “Pending” status), then one of the two SPs has sent in a Cancel Request (SV is now in “Cancel Pending” status) is as follows: - The New SP initiates the Cancel. The Old SP concurs with the Cancellation-Initial or the Cancellation-Final Concurrence Requests. The status will be changed to “Canceled” upon receipt of the cancel concurrence. Both SPs would have to re-initiate the porting process for this TN. - The New SP initiates the Cancel. The Old SP does not concur with the Cancellation-Initial or the Cancellation-Final Concurrence Requests, the status will be changed to “Canceled” at the expiration of the Final Concurrence expiration. Both SPs would have to re-initiate the porting process for this TN. | TBD | FRS, IIS, GDMO, ASN.1 | The recommendation is for a change to the NPAC functionality, such that an SP that sent up a Cancel Request in error, could “un-do” the request by sending a “retract cancel request” message to the NPAC. This new message would allow the SV to change from a “Cancel Pending” status back to a “Pending” status. The NPAC would verify that the SP sending the “retract cancel request” message to the NPAC is the same SP that initiated the Cancel Request (otherwise return an error). There would not be any restriction on when this new message could be sent (i.e., during the 18 hour window that the SV is in Cancel Pending). No backwards-compatibility flags needed. The change in status (from Cancel Pending back to Pending) can be handled with the existing Status Attribute Value Change. However, SPs should verify with their SOA vendors that an SAVC that is updating a Cancel Pending SV to a Pending SV will not be rejected. In order to use this new functionality, an SP would need to implement a change in their SOA. | TBD | TBD / TBD |
| NANC 388 (con’t) | - The Old SP initiates the Cancel. The New SP concurs with the Cancellation-Initial or the Cancellation-Final Concurrence Requests. The status will be changed to “Canceled” upon receipt of the cancel concurrence. Both SPs would have to re-initiate the porting process for this TN. - The Old SP initiates the Cancel. The New SP does not concur with the Cancellation-Initial or the Cancellation-Final Concurrence Requests, the status will be changed to “Conflict” at the expiration of the Final Concurrence expiration. The Old SP and New SP must then resolve the conflict, or wait for the Conflict Restriction Window to expire (six hours) for the SV to be eligible to be changed back to “Pending” by the New SP. In case #4, the porting process could continue after the expiration of the Cancellation Concurrence timers (18 hours), and either the resolution of the conflict (0-6 hours) or waiting for the Conflict timer to expire (6 hours). |  |  |  |  |  |  |
| Continuation of NANC 388, Change a “Cancel-Pending” SV, Proposed Resolution section: CMA Write-Up for Nov’03 LNPAWG – Major points/processing flow/high-level requirements: - An SV is in cancel-pending status. - The Service Provider that issued the cancel message to the NPAC, requests the NPAC to “un-do” the cancel request: - The Service Provider sends a “retract cancel request” message to the NPAC. - The NPAC validates the message is from the Service Provider that issued the cancel request. - If yes, continue. - If no, return an error to the requesting Service Provider, and exit the process. - The NPAC changes the status of the SV to pending. - The NPAC sends a Status Attribute Value Change notification to the involved Service Providers: - New Service Provider receives Status Attribute Value Change notification updating the status to pending. - Old Service Provider receives Status Attribute Value Change notification updating the status to pending. Requirements - NPAC SMS shall inform both Old and New Service Providers when the status of a Subscription Version is set from cancel-pending back to pending for an Inter-Service Provider port. - NPAC SMS shall receive the following data from a requesting Service Provider to identify a Subscription Version to have a cancel request retracted: - Ported TN (or a specified range of numbers) - Subscription Version ID - NPAC SMS shall send an appropriate error message to the originating user if the status is not cancel-pending. - NPAC SMS shall send an appropriate error message to the originating user if the originating user is neither the New nor the Old Service Provider in the existing Subscription Version upon Subscription Version cancellation retraction. - NPAC SMS shall set the Subscription Version cancellation retraction date and time to current upon setting the Subscription Version status back to pending. - NPAC SMS shall return an error if a Service Provider sends a cancellation retraction for a subscription version that has not been created by that Service Provider. - NPAC SMS shall return an error if a Service Provider sends a cancellation retraction for a subscription version that has not been cancelled by that Service Provider. - NPAC SMS shall set the subscription version status to Pending upon receiving a cancellation retraction from either the Old or New Service Provider for a subscription version with a cancel-pending status (both Service Providers have done a create) for an Inter-Service Provider or Port to original port. Message Flows None. |  |  |  |  |  |  |  |

# Next Documentation Release Change Orders

| Next Documentation Release Change Orders |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chg Order # | Orig. / Date | Description | Priority | Category | Proposed Resolution | Level of Effort |  |
|  |  |  |  |  |  | NPAC | SOA LSMS |
| NANC 359 | NeuStar 4/12/02 | Doc Only Change Order for SPID and Billing ID: Change definition for SPID and Billing ID The current documentation does NOT explicitly state that SPID must be 4 alphanumeric characters, and Billing ID can be variable 1-4 alphanumeric characters. The Billing ID is sometimes associated with a SPID value, so different interpretations said that it must be 4 alphacharacters, whereas others said it could be variable 1-4 as currently defined in the ASN.1. |  | ASN.1 | Func Backwards Compatible: YES Change the current documentation to explicitly state SPID must be 4 alphanumeric characters, and Billing ID can be variable 1-4 alphanumeric characters. Jan ’03 LNPAWG, approved, move to next documentation category. | N/A | N/A / N/A |
| NANC 360 | NeuStar 4/12/02 | Doc Only Change Order for Recovery: Maximum TN Recovery Tunable A recent business situation has created an implementation of a new Service Provider-specific tunable. This doc-only change order will add this definition to the appropriate documentation. |  | FRS, IIS, GDMO | Func Backwards Compatible: YES Change the current documentation to explicitly state that the Service Provider-specific tunable (Maximum_TN_Recovery) is a tunable with a range of 1-10000, a default value of 2000, and is applicable for time-based recovery. Jan ’03 LNPAWG, approved, move to next documentation category. | N/A | N/A / N/A |
| NANC 361 | World Com 5/13/02 | Doc Only Change Order for GDMO: Range Version of Object Creation Notification The definition and behavior of the range notification associated with NANC 179 (SOA range notifications) in NPAC Release 3.1 should be modified. According to the current specification, the range version of the object creation notification can support multiple sets of attributes. However, the intent of NANC 179 was to only support one set of attributes for all TN/SVIDs in the range. This change order requests that the definition for this notification be changed to only support one set of attributes per TN/SVIDs instead of potentially multiple sets of attributes. Below is an excerpt of the ASN.1 definition for the RangeObjectCreation is: RangeObjectCreationInfo ::= SEQUENCE { tn-version-id RangeNotifyTN-ID-Info, object-info SET OF ObjectInfo } |  | IIS, GDMO | Func Backwards Compatible: YES Change the current documentation to explicitly state that the current NPAC implementation supports only one (1) element in the object-info. Jan ’03 LNPAWG, approved, move to next documentation category. | N/A | N/A / N/A |
| NANC 364 | NeuStar 7/15/02 | Doc Only Change Order for ASN.1: Create Action comment A comment should be removed. According to the current specification, the TN Range attribute is related to Release 1.4 pooling. However, optional attribute is valid for other downloads to the LSMS. This change order requests that the comment be removed to avoid confusion. Below is an excerpt of the ASN.1 definition for the CreateAction: LocalSMS-CreateAction ::= SEQUENCE { actionId INTEGER, subscriptionVersionObjects SET OF SubscriptionVersionObject, tn-range TN-Range OPTIONAL -- used only on pooled ports for release 1.4 } |  | IIS, ASN.1 | Pure Backwards Compatible: YES Change the current documentation by removing the “used only on pooled ports for release 1.4”. Jan ’03 LNPAWG, approved, move to next documentation category. | N/A | N/A / N/A |
| NANC 365 | TSE 8/30/02 | Doc Only Change Order for IIS/GDMO: PTO and SV Query discrepancies between the two documents 1. PTO Processing Discrepencies The GDMO states for subscriptionVersionNewSP-CreateBehavior that the new service provider must specify valid values for the LRN and GTT data. In addition it states, "If the value of subscriptionPortingToOriginal-SPSwitch is TRUE, the LRN and GTT data should be specified as NULL." However, data flows B.5.1.2 and B.5.1.3 both state that LRN and GTT data must be provided UNLESS subscriptionPortingToOriginal-SP is true. So, in the one case the requirement is to provide NULL values for LRN and GTT data and in the other case the requirement is to not provide LRN and GTT data. The GDMO and the data flows need to be made consistent. 2. SV Query Discrepencies The GDMO states for subscriptionVersionNPAC-Behavior that subscriptionTimerType and subscriptionBusinessType are only returned on SOA queries to service providers that support these attributes. However, data flow B.5.6 shows that subscriptionTimerType and subscriptionBusinessType are returned unconditionally. The GDMO and the data flow need to be made consistent. |  | IIS, GDMO | Pure Backwards Compatible: YES Change the current documentation to be consistent and reflect the current behavior. Jan ’03 LNPAWG, approved, move to accepted category. Need to verify if it should be NULL or not specified. Update the documentation to reflect this. Upon further analysis, it was determined that the correct reference should be the following: - PTO - “not specified” - SV Query – “returned only if the SOA supports these attributes” | N/A | N/A / N/A |
| NANC 371 | AT&T 11/6/02 | Doc Only Change Order for Audits: Update Behavior The current documentation does NOT explicitly state that the NPAC requires audit names to be unique. |  | FRS, IIS, GDMO | Pure Backwards Compatible: YES Update the documentation to reflect the behavior of audit name within the NPAC. Dec ’02 LNPAWG, approved, move to next documentation category. | N/A | N/A / N/A |
| NANC 373 | NeuStar 11/19/02 | Doc Only Change Order: Conflict AVC The current documentation does NOT list the AttributeValueChange notification when the NPAC automatically sets an SV from cancel-pending to conflict, upon exipiration of the appropriate timer. |  | FRS, IIS, GDMO | Pure Backwards Compatible: YES Update the current documentation to reflect the behavior of this notification within the NPAC. Dec ’02 LNPAWG, approved, move to next documentation category. | N/A | N/A / N/A |
| NANC 374 | NeuStar 11/20/02 | Doc Only Change Order: PTO SP The current documentation does NOT indicate that for a PTO subscription version, the new SP must be the code holder (block holder if a NPB exists). |  | FRS, IIS, GDMO | Pure Backwards Compatible: YES Update the current documentation to reflect the behavior of this PTO SV activity within the NPAC. Dec ’02 LNPAWG, approved, move to next documentation category. | N/A | N/A / N/A |
| NANC 376 | NeuStar 12/2/02 | Doc Only Change Order: Modify Active with Failed List The current documentation does NOT indicate that for a Modify Active of a subscription version with an existing Failed List, should be rejected by the NPAC. |  | FRS, IIS, GDMO | Pure Backwards Compatible: YES Update the current documentation to reflect the behavior of this Modify Active SV activity within the NPAC. Dec ’02 LNPAWG, approved, move to next documentation category. | N/A | N/A / N/A |
| NANC 377 | NeuStar 12/4/02 | Doc Only Change Order: Missing IIS Flow for 2nd Create by Old SP with Auth=FALSE The current documentation does NOT have an IIS flow for this scenario. |  | FRS, IIS, GDMO | Pure Backwards Compatible: YES Update the current documentation to reflect the behavior of this Old SP Create activity within the NPAC. Dec ’02 LNPAWG, approved, move to next documentation category. | N/A | N/A / N/A |
| NANC 378 | TSE 12/5/02 | Doc Only Change Order: Missing IIS Flow for cancellation of a disconnect-pending SV The current documentation does NOT have an IIS flow for this scenario. |  | IIS, GDMO | Pure Backwards Compatible: YES Update the current documentation to reflect the behavior of this cancellation activity within the NPAC. Dec ’02 LNPAWG, approved, move to next documentation category. | N/A | N/A / N/A |

# LTI Change Orders

| - LTI Change Orders |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chg Order # | Orig. / Date | Description | Priority | Category | Proposed Resolution | Level of Effort |  |
|  |  |  |  |  |  | NPAC | SOA LSMS |

# Cancel – Pending Change Orders

| - Cancel - Pending Change Orders |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chg Order # | Orig. / Date | Description | Priority | Category | Proposed Resolution | Level of Effort |  |
|  |  |  |  |  |  | NPAC | SOA LSMS |

# Current Release Change Orders

| - Current Release Change Orders |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chg Order # | Orig. / Date | Description | Priority | Category | Proposed Resolution | Level of Effort |  |
|  |  |  |  |  |  | NPAC | SOA LSMS |
|  |  | See Implemented List for details on Release 3.2. |  |  |  |  |  |

# MR Change Orders

|  | - MR Change Orders |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chg Order # |  | Orig. / Date | Description | Priority | Category | Proposed Resolution | Level of Effort |  |
|  |  |  |  |  |  |  | NPAC | SOA LSMS |

# Summary of Change Orders

| Release # / Target Date | Change Orders | Backwards Compatible |
| --- | --- | --- |
| Open | NANC 147 – Version ID Rollover Strategy NANC 340 – Doc Only Change Order for IIS: Update Appendix A NANC 349 – Batch File Processing NANC 353 – Round-Robin Broadcasts Across SOA and LSMS Associations with separate SOA channel for notifications (son of ILL 5) NANC 362 – Vendor Metrics NANC 372 – SOA/LSMS Interface Protocol Alternatives NANC 382 – “Port-Protection” System NANC 383 – Separate SOA channel for notifications (subset of NANC 353) NANC 384 – NPAC Change Order Effectiveness Metrics NANC 385 – Timer Calculation – Maintenance Window Behavior same as Holidays NANC 386 – Single Association for SOA/LSMS NANC 387 – Doc Only Change Order: IIS Updates NANC 388 – Change a “Cancel Pending” SV NANC 389 – Production Equivalent Test-Bed NANC 390 – New Interface Confirmation Messages SOA/LSMS – to - NPAC |  |
| Accepted | ILL 5 – Round-Robin Broadcast Across LSMS Associations ILL 130 – Application Level Errors NANC 138 – Definition of Cause Code Values-REVISITED NANC 151 – TN and Number Pool Block Addition to Notifications NANC 193 – TN Processing During NPAC SMS NPA Split Processing NANC 200 – Notification of NPA Splits NANC 219 – NPAC Monitoring of SOA/LSMS Associations NANC 227 – 10-digit TN Filters (previously know as: “Ability to Modify/Delete of Partial Failure SV”) NANC 232 – Web Site for First Port Notifications NANC 254 – NPAC Requirements – Subsequent Ports of Active SV with a Failed SP List NANC 285 – SOA Requested Subscription Version Query Max Size NANC 299 – NPAC Monitoring of SOA and LSMS Associations via Heartbeat NANC 300 – 7 Digit Block Filters for Number Pooling NANC 321 – NPAC Edit of Service Provider Network Data – NPA-NXX Data NANC 343 – Doc Only Change Order for IIS: Exhibit 12 of IIS section 4.2.2 does not reflect all filtering operations currently supported by the NPAC SMS. NANC 346 – GDMO Change to Number Pool Block Data Managed Object Class (Section 29.0) NANC 347/350 – CMIP Interface Enhancements – abort behavior NANC 348 – Bulk Data Download File for Notifications NANC 351 – Recovery Enhancements – “Send me what I missed” recovery message NANC 352 – Recovery Enhancements – recovery of SPID (customer data) NANC 355 – Modification of NPA-NXX Effective Date (son of ILL 77) NANC 357 – Unique Identifiers for wireline versus wireless carriers (long term solution) NANC 358 – Change for ASN.1: Change SPID definition NANC 363 – Lockheed-to-NeuStar private enterprise number NANC 368 – Outbound Flow Control NANC 382 – “Port-Protection” System NANC 388 – Change a “Cancel Pending” SV |  |
| Next Documentation Release | NANC 359 – Doc Only Change Order for SPID and Billing ID: Change definition for SPID and Billing ID NANC 360 – Doc Only Change Order for Recovery: Maximum TN Recovery Tunable NANC 361 – Doc Only Change Order for GDMO: Range Version of Object Creation Notification NANC 364 – Doc Only Change Order for ASN.1: Create Action comment NANC 365 – Doc Only Change Order for IIS/GDMO: SV Query and PTO discrepancies between the two documents NANC 371 – Documentation Only – Audit Behavior NANC 373 – Doc Only Change Order: Conflict AVC NANC 374 – Doc Only Change Order: PTO LISP NANC 376 – Doc Only Change Order: Modify Active with Failed List NANC 377 – Doc Only Change Order: Missing IIS Flow for 2nd Create by Old SP with Auth=FALSE NANC 378 – Doc Only Change Order: Missing IIS Flow for cancellation of a disconnect-pending SV |  |
| LTI |  |  |
| Cancel-Pending |  |  |
| Current Release | See Implemented List for details on R3.2 |  |
| MR |  |  |

It is appropriate to prevent the creation of a pooled block if any non-ported number in the block is “port-protected” since to allow the block’s creation would result in an inadvertent port of these numbers if the block eventually is assigned to another switch.  But the intra-SP porting activity required before creating a contaminated block must be allowed to occur without requiring end-users to temporarily lift the port restrictions on their numbers.  It therefore appears that an exception to the port protection validation is required, to allow a protected number to be intra-SP ported even if the number is “Port Protected.”  Without network data that is unavailable to NPAC today, the NPAC could not reliably determine whether an intra-SP port maintains the telephone number’s association with the same switch from which the number was served before the intra-SP port occurred.  A reasonable compromise appears to suppress the “Port-Protect” check when validating intra-SP ports rather than develop an elaborate validation process to address this scenario more completely.

A modify of an active SV’s or block’s LRN can result in the move of a telephone number to a different switch and thus could result in an inadvertent port.  NeuStar is not proposing the “Port Protect” validation be applied to Modify actions because of the complexity of such validation.

The validation of intra-SP ports occurs only if the involved SP has indicated in its NPAC SMS profile that this validation is desired.

It is appropriate to prevent the creation of a pooled block if any non-ported number in the block is on the Port Protection list, since to allow the block’s creation would result in an inadvertent port of these numbers when (if) the block eventually is assigned to another switch.  But the intra-SP porting activity, necessary before creating a contaminated block, is allowed to occur without requiring that the port restrictions be lifted from TNs in the block.  This exception to the Port Protection validation is provided in order to allow a TN to be intra-SP ported even if the TN is on the Port Protection list.  The option to include intra-SP ports in the Port Protection validation process is provided at the individual LSP’s request.

A modify of the LRN in an active SV or block record also can result in the move of a telephone number to a different switch and thus could result in an inadvertent port.  However, NeuStar is not proposing the Port Protection validation be applied to Modify actions because of the complexity of such a validation.