NPAC SMS Release 3.2

Turn Up Test Plan

Release 3.2.2b

February 12, 2004

Publication History

| Version | Release Date | Description |
| --- | --- | --- |
| 0.1 |  | Initial draft of NPAC Release 3.2.0 Test Cases |
| 0.2 | 1/17/03 | Second draft of NPAC Release 3.2.0 Test Cases reflecting comments received from 12/17 and 12/18/02 industry review. |
| 3.2.0c | 2.14.03 | Third revision of NPAC Release 3.2.0 Test Cases reflecting comments received after last industry publication on 1/17/03. The revision scheme has changed with this release to reflect the standard format across the FRS, IIS, Test Plan and M&Ps. |
| 3.2.2a | 1.19.04 | Revision incorporating comments received after 3.2.0 turn-up test. This publication comes after a point release of the software. No new functional test cases for the software point release were required. |
| 3.2.2b | 2.12.04 | Deleted procedures for NANC 323-1 test case due to delay in testing. These test case procedures will be updated to more appropriately support the unique testing approach required to efficiently test this functionality and will be published in coordination with the actual testing cycle. |

Table of Contents

1.	Preface	6

1.1	Purpose of this Document	6

1.2	Assumptions	6

1.3	Audience	7

1.4	Conventions Used in this Document	7

1.4.1.	Test Case Template	7

1.4.2.	Test Case Numbering	9

1.4.3.	Test Case Priority	10

1.4.4.	Test Case Prerequisites	10

1.4.5.	Test Case Steps and Expected Results	10

1.4.6.	Pass/Fail Analysis	10

1.5	Related Documents	10

1.6	Document Structure	11

1.7	Requirements for Turn-Up Testing	11

2.	NANC 169 – Delta Download File Creation by Time Range for SVs	13

3.	NANC 187 – Linked Action Replies	31

4.	NANC 191 DPC/SSN Value Edits and NANC 291 SSN Edits in the NPAC SMS	56

5.	NANC 192 – NPA Split NPAC SMS Load File	79

6.	NANC 218 – Conflict Timestamp Broadcast to SOA	82

7.	NANC 230 – Donor SOA Port-To-Original of Intra-Service Provider Port	89

8.	NANC 249 – Modification of Dates for a Disconnect Pending SV	97

9.	NANC 297 – Sending SV Problem During Recovery	106

10.	NANC 319 – NPAC Edit to Ensure NPA-NXX of LRN is in Same LATA as NPA-NXX of Ported TN	107

11.	NANC 322 – Clean Up of Failed SP List Based on Service Provider BDD Response File	122

12.	NANC 323 – Partial Migration of SPID via Mass Update Test Cases	129

13.	NANC 354 – Delta Download File Creation by Time Range for Network Data            	                                                                                                                                                                     130

14.	Group Testing	136

Appendix A:	Test Case Matrix	155

Appendix B:	Test Plan Issues	167

# Preface

## Purpose of this Document

The purpose of this document is to identify the NPAC Release 3.2 Test Cases.  These Test Cases are based on NPAC SMS Release 3.2 (and all respective point release) requirements.

Actual Entrance and Exit criteria for test execution/completion are an agreement between individual Service Providers and the NPAC SMS vendor based upon the functionality supported by the local Service Provider SOA and/or LSMS systems.

This Test Plan contains Test Cases per functional component of the Software Release.  The Test Cases cover basic Success and Error scenarios.  Test Case Priority is indicated by the systems that participate in each respective Test Case.  It is assumed that the NPAC SMS/NPAC Personnel participate in every Test Case of the Turn Up Test Plan.  If the Test Case Priority for a system is marked as Required that system shall participate as the Test Case describes.  A Test Case Priority of Conditional for a system means that the system shall participate in the Test Case as described, if the respective functionality has been implemented for that system.  When the Test Case Priority is marked as Optional for a system, it is at the discretion of the Service Provider if they use the respective system to participate in the Test Case as described.  Finally, the Test Case Priority may be marked as N/A for a Service Provider system which means that the functionality tested in this Test Case does not apply to this respective Service Provider system.

The different NPAC regions will turn-up Release 3.2 software at different times.  As a result Service Providers that operate in multiple regions will need to handle Release 2, Release 3.1 and Release 3.2 interfaces (and respective data) simultaneously.  This test plan does not include any guidelines or test cases for the purpose of testing backward compatibility between NPAC SMS releases.

## Assumptions

All Test Cases should be executed where the Service Provider profile attributes are set such that they emulate the Service Provider’s production environment unless otherwise stated in an individual test case.

A list of Service Provider Profile Flags and the valid values are provided in the following table:

| Service Provider Profile Flag | Valid Values |
| --- | --- |
| LSMS Network Data Management | True/False |
| LSMS Queries | True/False |
| Support EDR Download | True/False |
| LSMS Support NPA-NXX-X | True/False |
| LSMS Support WSMSC Data | True/False |
| Port In Timer Type | Long/Short |
| Port Out Timer Type | Long/Short |
| SP Business Hours | Normal/Extended |
| SOA Management | True/False |
| SOA Network Data Management | True/False |
| SOA Data Download | True/False |
| SOA Support Business Hours | True/False |
| SOA Support NPA-NXX-X | True/False |
| SOA Support Timer Type | True/False |
| SOA Support WSMSC Data | True/False |
| Support Service Bureau | True/False |
| Customer TN Range Notification | True/False |
| No New SP Concurrence Notification | True/False |
| SOA Notification Priority NOTE: For SOA Notifications there is a flag for each notification listed in Table C-7, Appendix C of the NANC FRS Release 3.1.0) | High/Medium/Low/None |
| SOA Linked Replies Indicator | True/False |
| LSMS Linked Replies Indicator | True/False |

## Audience

The intended audience for this document is NPAC SMS, SOA and LSMS system testers and anyone who is involved with NPAC SMS, SOA and LSMS testing.  It is assumed that individuals using this test plan have an understanding of Local Number Portability, Number Pooling and related specification documents.  The Test Cases are written from the Interface Interoperability Specification (IIS) perspective so users should have an understanding of this document specifically.

## Conventions Used in this Document

### Test Case Template

Test Cases are the bulk of the information presented in this document.  Test Cases are comprised of the following information:

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | Unique Test Case Identifier |  | SUT Priority: |  | SOA |  | Required –This Service Provider systems shall participate. Conditional – If the Service Provider system has implemented the functionality represented in this Test Case, then the system shall participate. Optional – Service Provider may include this system as indicated by the Test Case. N/A – This Test Case does not apply to this system. |
|  |  |  |  |  |  |  | LSMS |  | Required, Conditional, Optional or N/A. |
|  | Objective: |  | Test Case Objective. The Title specifies relevant systems to the test (NPAC SMS, SOA or LSMS) and the type of Test Case (success or error). |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  | If a change order revision is relevant – it’s indicated here. |  | Change Order Number(s): |  | If a Change Order(s) is relevant – it is indicated here. |  |  |
|  | NANC FRS Version Number: |  | FRS version is indicated here. |  | Relevant Requirement(s): |  | Requirement(s) related to this Test Case are indicated here. |  |  |
|  | NANC IIS Version Number: |  | IIS version is indicated here. |  | Relevant Flow(s): |  | IIS Flow(s) related to this Test Case are indicated here. |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  | Test Case, if any, to be successfully executed prior to this Test Case |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Steps to be executed by NPAC Personnel prior to Test Case execution |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | Steps to be executed by Service Provider Personnel prior to Test Case execution |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | [system indicated here] | This test step is described here. |  | [system indicated here] |  | The expected results associated with this respective test step are indicated here. |  |  |  |
| E. | Pass/Fail Analysis, TC # |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

### Test Case Numbering

Test Case Numbers are alphanumeric numbers that identify the sections of functional component based on the respective Change Order to ensure a unique Test Case number.  Below is a table identifying the Change Orders that are included in this release and their associated alphanumeric numbering pre-fix.  These test case numbers are assumed to be static:

| Numeric Pre-Fix | Respective Functional Component |
| --- | --- |
| NANC 169 | Change Order NANC 169 Delta Download File Creation by Time Range for SVs Test Cases |
| NANC 187 | Change Order NANC 187 Linked Action Replies Test Cases |
| NANC 191/291 | Change Order NANC 191 DPC/SSN Value Edits and Change Order NANC 291 SSN Edits in the NPAC SMS Combined Test Cases |
| NANC 192 | NPA Split NPAC SMS Load File |
| NANC 218 | Change Order NANC 218 Conflict Timestamp Broadcast to SOA Test Cases |
| NANC 230 | Change Order NANC 230 Donor SOA Port-To-Original of Intra-Service Provider Port Test Cases |
| NANC 249 | Change Order NANC 249 Modification of Dates for a Disconnect Pending SV Test Cases |
| NANC 297 | Change Order NANC 297 Sending SV Problem During Recovery |
| NANC 319 | Change Order NANC 319 NPAC Edit to Ensure NPA-NXX of LRN is in Same LATA as NPA-NXX of Ported TN Test Cases |
| NANC 322 | Change Order NANC 322 Clean Up of Failed SP List based on Service Provider BDD Response File Test Cases |
| NANC 323 | Change Order NANC 323 Partial Migration of a SPID via Mass Update Test Cases |
| NANC 354 | Change Order NANC 354 Delta Download File Creation by Time Range for Network Data Test Cases |

### Test Case Priority

Each Test Case will have an associated Test Case Priority.

Required: This Test Case represents required functionality and shall be executed by the respective Service Provider system and/or NPAC SMS Vendor.

Conditional: This Test Case represents optional functionality.  If a Service Provider has implemented the suggested functionality for this respective Service Provider system in the Test Case, they shall execute the Test Case as written.  If there are not any Service Providers that have implemented the functionality, and therefore cannot verify the NPAC SMS behavior, the NPAC Personnel shall execute the Test Case with the use of simulators.

Optional: Service Provider may execute the Test Case as written if they have implemented the suggested functionality for this respective system.  Typically ‘optional’ Test Cases verify ‘additional’ attributes of a requirement.

N/A: This Test Case does not apply to this Service Provider system.  Thus the Service Provider does not need to test this respective system during this Test Case.

### Test Case Prerequisites

Each Test Case contains a section for Prerequisites including Prerequisite Test Cases and/or Prerequisite NPAC Setup and/or Prerequisite SP Setup.  When Prerequisite Test Cases are identified this is simply referencing a Test Case that, when appropriately executed, will setup the proper scenario for executing that respective Test Case.  Prerequisite Test Cases are not a good source for Test Case ordering to ensure efficient execution.  Ordering of Test Cases for efficient execution should be reviewed on a Service Provider by Service Provider basis, based on their specific suite of Test Cases for exiting Turn Up Test.

### Test Case Steps and Expected Results

Test Case steps and Expected results have fields to indicate the respective systems, test steps and their expected results.

### Pass/Fail Analysis

Each Test Case contains a general analysis of either Pass or Fail.

## Related Documents

North American Number Council (NANC) Functional Requirements Specification Number Portability Administration Center (NPAC) Service Management System (SMS), Release 3.2.0a

NPAC SMS Interoperable Specifications NANC Version 3.2.0a

## Document Structure

This document is organized into sections as defined below:

|  | Preface | This section describes the purpose and structure of this document |
| --- | --- | --- |
|  | Chapters 2 - 14 | Test Cases – one chapter for each change order and a chapter for the Group Test Cases |
|  | Appendix A | Test Case Matrix including a List of Objectives and Results Table |
|  | Appendix B | Issues [indicate open/date and closed/date] |

## Requirements for Turn-Up Testing

Turn-Up Testing, which includes new NPAC SMS software release functionality testing and regression testing, must be performed on a Service Provider’s SOA/LSMS software anytime that a change is made to the interface (GDMO or ASN.1) of the NPAC SMS.  In the event that the interface change is initiated by the NPAC SMS, Service Providers shall perform Turn-Up Testing on each version of SOA/LSMS software that may potentially be used with the new NPAC SMS interface.

If any of the following scenarios apply, Turn-Up Testing is required by Service Providers.  The following outlines the required level of testing for specific scenarios:

- 1.	When a local product (SOA/LSMS) is compiled with the current interface model, and a new local feature (SOA/LSMS feature) is implemented that does NOT involve a change in the use of the interface model, and the NPAC SMS is compiled with the current model, then Turn-Up Testing is optional.  Test cases to be performed at the discretion of the Service Provider. [standard regression test cases].

- 2.	When a local product is compiled with the current interface model, and no new local features are implemented that involve the interface, and the NPAC SMS is compiled with the new interface model, then Turn-Up Testing is required [standard regression test cases].

- 3.	When a local product is compiled with the new interface model, and no new local features are implemented that involve the interface, and the NPAC SMS is compiled with the new interface model, then Turn-Up Testing is required [standard regression test cases].

- 4.	When a local product is compiled with the new interface model, and new local features are implemented that involve the interface, and the NPAC SMS is compiled with the new interface model, then Turn-Up Testing is required [standard regression test cases and new functionality test cases].

- 5.	When a local product is compiled with the current interface model, and new local features are implemented that involve the interface, and the NPAC SMS is compiled with the current model, then Turn-Up Testing is required [standard regression test cases and new functionality test cases].

# NANC 169 – Delta Download File Creation by Time Range for SVs

NOTE:  When executing the NANC 169 Test Case, verify the NPAC Customer Allowable Functions, LSMS Queries/Audits – is set to TRUE.

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 169-1 |  | SUT Priority: |  | SOA |  | N/A |
|  |  |  |  |  |  |  | LSMS |  | Optional |
|  | Objective: |  | NPAC OP GUI – NPAC Personnel initiate a Bulk Data Download of Subscription Data – Specifying Active/Disconnect Pending/Partial Failure Subscription Versions Only and NOT specifying a TN range. Verification steps are performed to ensure the BDD file was processed successfully by the Service Provider system - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 169 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0a |  | Relevant Requirement(s): |  | RR3-320, RR3-312, RR3-313, RR3-314, RR3-319, RR3-323 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0a |  | Relevant Flow(s): |  | N/A |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | While the LSMS is ‘dis-associated’ from the NPAC SMS, NPAC personnel perform the following functions: a) Modify a (unique) range of 500, ‘Active’ Subscription Versions where the Service Provider under test is the Current Service Provider. Use simulators that are not associated with the NPAC and are receiving downloads for this NPA-NXX. Verify these Subscription Versions exist with a status of ‘Active’ and a Failed SP List. (SV group a______________) b) Create a filter for the NPA-NXX for which you created 500, ‘Pending’ Subscription Versions in 1a) above. - Activate these 500, ‘Pending’ Subscription Versions. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify that the status for all 500 is ‘Active’ on the NPAC SMS. (SV group b______________________) - Disconnect 250 of these now, ‘Active’ Subscription Versions specifying Effective Release and Customer Disconnect dates in the future. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify that the status of these 250 Subscription Versions is ‘Disconnect-Pending’. (SV group b1___________) - Remove the filter for this NPA-NXX for the Service Provider under test so that this range of Subscription Versions will be included in the Bulk Data Download File. c) Create and Activate 100 Intra-Service Provider Subscription Versions using an NPA-NXX that is open for porting and for which the Service Provider under test is accepting downloads for this NPA-NXX. This Service Provider is neither the Old or New Service Provider for these Subscription Versions. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify that the Subscription Versions have a status of ‘Partial-Fail’. (SV group c ___________________) d) Activate 50 Subscription Versions with a status of ‘Pending’. The Service Provider under test is the New Service Provider for these Subscription Versions. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify that these Subscription Versions have a status of ‘Partial-Fail’. (SV group d_________________) e) Activate a Number Pool Block for an NPA-NXX for which this Service Provider under test is accepting downloads, but it is another Service Provider’s Number Pool Block. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify that the Pooled Subscription Versions have a status of ‘Partial-Fail’. (SV group e_____________________) (NPB e______________) f) Create and concur to a range of 100, ‘Pending’ Subscription Versions where the Service Provider under test is the New Service Provider. (SV group f__________________) g) Put simulated SPID LSMS in recovery. Use at least one simulator that is associated with the NPAC and is accepting downloads for this NPA-NXX. Verify that the Service Provider under test is accepting downloads for this NPA-NXX. Activate 50 ‘pending’ SVs in group f above. Verify that these subscription versions have a status of ‘sending’. (SV group g__________________) During the test case retry timers will exhaust, and then the status of the SVs should be ‘Partial-Fail’. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel request a Bulk Data Download for Subscription Data, specifying Active/Disconnect Pending/Partial Failure Subscription Versions Only and NOT specifying a TN range , for the Service Provider under test. |  | NPAC |  | The NPAC SMS performs the request, generates the appropriate Bulk Data Download File(s) and automatically “FTP’s” the file(s) to the Service Provider’s directory on the NPAC SMS. |  |  |  |
| 2. | SP | Service Provider Personnel receive the Bulk Data Download File(s) and load the file into their LSMS. |  | SP |  | The LSMS successfully processes the Bulk Data Download file(s) and reflects the updates described in the prerequisites above. The systems are still ‘dis-associated’ from the NPAC SMS. |  |  |  |
| 3. | SP | Service Provider Personnel, using their LSMS, perform a local query for the Subscription Data to verify that the Subscription Version data was loaded. - SV group a - SV group b - SV group b1 - SV group c - SV group d - For non EDR LSMS, SV group e - For EDR LSMS, SV group e was not loaded - SV group g |  | SP |  | Using the LSMS system, verify: - SV group a exists on the LSMS. Verify that all of them reflect the ‘modified’ SV values from the prerequisites above. - SV group b exists on the LSMS. - SV group b1 exists on the LSMS. - SV group c exists on the LSMS. - SV group d exists on the LSMS. - For non EDR LSMS, SV group e exists on the LSMS. - For EDR LSMS, SV group e DOES NOT exist on the LSMS. - SV group g exists on the LSMS. |  |  |  |
| 4. | SP | After all NPAC ‘retry timers’ for the Subscription Versions specified in the prerequisites above have expired, Service Provider personnel perform appropriate steps to ‘associate’ with the NPAC SMS such that they will not recover additional information. |  | SP |  | The LSMS successfully re-associates without recovering additional information. |  |  |  |
| 5. | NPAC | NPAC Personnel bring the simulated SPID LSMS that was in recovery in Prerequisite step g above, out of recovery. |  | NPAC |  | Verify that the simulated SPID that was in recovery during step g of the prerequisites is now out of recovery. Verify that the 50 subscription versions that were activated while this SPID was in recovery now have a status of ‘Partial Fail’. |  |  |  |
| 6. | NPAC | NPAC Personnel perform multiple, Full audits for each NPA-NXX included in the range of TNs specified in the prerequisites above for the Service Provider’s LSMS to verify that all the appropriate updates were processed from the Bulk Data Download File by the LSMS. |  | NPAC |  | Using the Audit Results Log, verify that there were no updates made. If any updates were made as a result of running this audit, this test case fails. Verify that: - SV group a exists on the LSMS. Verify that all of them reflect the ‘modified’ SV values from the prerequisites above. - SV group b exists on the LSMS. - SV group b1 exists on the LSMS. - SV group c exists on the LSMS. - SV group d exists on the LSMS. - For non-EDR LSMS, SV group e exists on the LSMS. - For EDR LSMS, SV group e DOES NOT exists on the LSMS. - SV group g exists on the LSMS. |  |  |  |
| 7. | NPAC | NPAC Personnel ‘re-send’ the following to the Service Provider under test: - SV group a that exists on the NPAC SMS with a status of ‘Active with a Failed SP List. - SV group c that exists on the NPAC SMS with a status of ‘Partial-Fail’. - SV group d that exists on the NPAC SMS with a status of ‘Partial-Fail’. - If the Service Provider’s EDR Indicator is set to FALSE, SV group e that exists on the NPAC SMS with a status of ‘Partial-Fail’. - SV group g that exists on the NPAC SMS with a status of ‘Partial-Fail’. - . NPAC SMS issues the appropriate messages to the LSMS in order to update the LSMS for these SVs. |  | SP |  | LSMS receives the resend requests from the NPAC SMS and issues a ‘duplicate object’ response to the NPAC SMS for: - SV group a - SV group c - SV group d - If the Service Provider’s EDR Indicator is set to FALSE, SV group e - SV group g |  |  |  |
| 8. | NPAC | NPAC Personnel perform multiple Full audits for each NPA-NXX of the following SVs, to verify that all the appropriate updates were processed from the NPAC ‘re-send’ for the ‘Partial-Fail’ objects: - SV group a - SV group c - SV group d - SV group 2e - SV group g |  | NPAC |  | Using the Audit Results Log, verify that there were no updates made. If any updates were made as a result of running this audit, this test case fails. Verify that: - SV group a exists on the LSMS. - SV group c exists on the LSMS. - SV group d exists on the LSMS. - SV group e exists on the non-EDR LSMS. - SV group e DOES NOT exist on the EDR LSMS. - SV group g exists on the LSMS. |  |  |  |
| E. | Pass/Fail Analysis, NANC 169-1 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel were able to successfully process the Bulk Data Download file updates with their local databases in a timely fashion. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider LSMS was able to successfully process the ‘re-send’ request from the NPAC SMS for the ‘Partial-Fail’ objects. |  |  |  |  |  |  |  |

Note:  Since the Bulk Data Download file selection criteria does not include a TN Range in this test case, the file may contain additional Subscription Versions.  Testers can verify appropriate behavior for any additional data that may be in the file as is stated in the Test Steps above.

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 169-2 |  | SUT Priority: |  | SOA |  | N/A |
|  |  |  |  |  |  |  | LSMS |  | Optional |
|  | Objective: |  | NPAC OP GUI – NPAC Personnel initiate a Bulk Data Download of Subscription Data – Specifying Active/Disconnect Pending/Partial Failure Subscription Versions Only and specifying a TN range that is a subset of the prerequisite test data. Verification steps are performed to ensure the BDD file was processed successfully by the Service Provider system - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 169 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0a |  | Relevant Requirement(s): |  | RR3-318, RR3-319, RR3-320, RR3-323 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0a |  | Relevant Flow(s): |  | N/A |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | While the LSMS is ‘dis-associated’ with the NPAC SMS, NPAC personnel perform the following functions: a) Modify a (unique) range of 500, ‘Active’ Subscription Versions where the Service Provider under test is the Current Service Provider. . Use simulators that are not associated with the NPAC and are receiving downloads for this NPA-NXX. Verify these Subscription Versions exist with a status of ‘Active’ and a Failed SP List. (SV group a______________) b) Create and Activate 100 Intra-Service Provider Subscription Versions using an NPA-NXX that is open for porting and for which an NPA-NXX filter exists for the Service Provider under test. This Service Provider is neither the Old or New Service Provider for these Subscription Versions. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify that the Subscription Versions have a status of ‘Active’. (SV group b ___________________) Remove the filter for this NPA-NXX. c) Submit a Deferred Disconnect request for 50 of the Intra-Service Provider Subscription Versions that were created and activated in 2b) above. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify that these Subscription Versions have a status of ‘Disconnect-Pending’. (SV group c ___________________________). d) Activate 50 Subscription Versions with a status of ‘Pending’. The Service Provider under test is the New Service Provider for these Subscription Versions. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify that these Subscription Versions have a status of ‘Partial-Fail’. (SV group d_________________) e) Activate a Number Pool Block for an NPA-NXX for which this Service Provider under test is accepting downloads, but it is another Service Provider’s Number Pool Block. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify that the Pooled Subscription Versions have a status of ‘Partial-Fail’. (SV group e_____________________) (NPB e______________) f) Put simulated SPID LSMS in recovery. Use at least one simulator that is associated with the NPAC and is accepting downloads for this NPA-NXX. Verify that the Service Provider under test is accepting downloads for this NPA-NXX. Activate an uncontaminated Number Pool Block on behalf of another Service Provider. Verify that the Pooled SVs and NPB have a status of ‘Sending’. (SV group f_______________), (NPB f_________________) During the test case retry timers will exhaust, and then the status of the Pooled SVs and NPB should be ‘Partial-Fail’. The TN Range specified in the Bulk Data Download Selection Criteria should be a ‘subset’ of the total of the TN Range used in the prerequisite steps. TN Range _________________________ |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel request a Bulk Data Download for Subscription Data, specifying Active/Disconnect Pending/Partial Failure Subscription Versions Only and specifying the TN Range identified in the prerequisites above, for the Service Provider participating in the test case. By specifying this TN range, the content of the BDD will actually be a subset of the prerequisite data. |  | NPAC |  | The NPAC SMS performs the request, generates the appropriate Bulk Data Download File(s) and automatically “FTP’s” the file(s) to the Service Provider’s directory on the NPAC SMS. |  |  |  |
| 2. | SP | Service Provider Personnel receive the Bulk Data Download File(s) and load the file(s) into their LSMS systems. |  | SP |  | The LSMS successfully processes the Bulk Data Download file(s) and reflect the updates described in the prerequisites above. The systems are still ‘dis-associated’ from the NPAC SMS. |  |  |  |
| 3. | SP | Service Provider Personnel, using their LSMS, perform a local query for the Subscription Data to verify that the data was loaded. NOTE: The BDD request was a subset of the total TNs manipulated in the Prerequisite Setup above. Verify the subset of data. - SV group a - SV group b - SV group c - SV group d - If the Service Provider’s EDR Indicator is set to FALSE, SV group e - If the Service Provider’s EDR Indicator is set to TRUE, SV group e DOES NOT exist on the LSMS. - If the Service Provider’s EDR Indicator is set to FALSE, SV group f - If the Service Provider’s EDR Indicator is set to TRUE, SV group f DOES NOT exists on the LSMS. |  | SP |  | Using the LSMS system, verify: NOTE: The BDD request was a subset of the total TNs manipulated in the Prerequisite Setup above. Verify the subset of data. - SV group a exists on the LSMS. - SV group b exists on the LSMS. - SV group c exists on the LSMS. - SV group d exists on the LSMS. - If the Service Provider’s EDR Indicator is set to FALSE, SV group e exists on the LSMS. - If the Service Provider’s EDR Indicator is set to TRUE, SV group e DOES NOT exist on the LSMS. - If the Service Provider’s EDR Indicator is set to FALSE, SV group f exists on the LSMS. - If the Service Provider’s EDR Indicator is set to TRUE, SV group f DOES NOT exist on the LSMS. |  |  |  |
| 4. | SP | After all NPAC ‘retry timers’ for the Subscription Versions specified in the prerequisites above have expired, Service Provider personnel perform appropriate steps to ‘associate’ with the NPAC SMS such that they will not recover additional information. |  | SP |  | The LSMS successfully re-associates with the NPAC SMS without recovering additional information. |  |  |  |
| 5. | NPAC | NPAC Personnel bring the simulated SPID LSMS that was in recovery in Prerequisite step f above, out of recovery. |  | NPAC |  | Verify that the simulated SPID that was in recovery during step f of the prerequisites is now out of recovery. Verify that the number pool block and respective pooled subscription versions that were activated while this SPID was in recovery now have a status of ‘Partial Fail’. |  |  |  |
| 6. | NPAC | NPAC Personnel perform multiple, Full audits for each NPA-NXX included in the range of TNs specified in the prerequisites above for the Service Provider’s LSMS to verify that all the appropriate updates were processed from the Bulk Data Download File by the LSMS. |  | NPAC |  | Using the Audit Results Log, verify that there were no updates made. If any updates were made as a result of running this audit, this test case fails. Verify that: - SV group a exists on the LSMS. - SV group b exists on the LSMS. - SV group c exists on the LSMS. - SV group d exists on the LSMS. - If the Service Provider’s EDR Indicator is set to FALSE, SV group e exists on the LSMS. - If the Service Provider’s EDR Indicator is set to TRUE, SV group e DOES NOT exist on the LSMS. - If the Service Provider’s EDR Indicator is set to FALSE, SV group f exists on the LSMS. - If the Service Provider’s EDR Indicator is set to TRUE, SV group f DOES NOT exist on the LSMS. |  |  |  |
| 7. | NPAC | NPAC Personnel ‘re-send’ the following to the Service Provider under test: NOTE: The BDD request was a subset of the total TNs manipulated in the Prerequisite Setup above. Resend the respective subset of data. - SV group a that exists on the NPAC SMS with a status of ‘Partial-Fail’. - SV group d that exists on the NPAC SMS with a status of ‘Partial-Fail’. - If the Service Provider’s EDR Indicator is set to FALSE, SV group e that exists on the NPAC SMS with a status of ‘Partial-Fail’. - If the Service Provider’s EDR Indicator is set to FALSE, SV group f that exists on the NPAC SMS with a status of ‘Partial-Fail’. NPAC SMS issues the appropriate messages to the LSMS in order to update the LSMS for these SVs. |  | LSMS |  | LSMS receives the resend requests from the NPAC SMS and issues a ‘duplicate object’ response to the NPAC SMS for: NOTE: The BDD request was a subset of the total TNs manipulated in the Prerequisite Setup above. Verify the subset of data. - SV group a - SV group d - If the Service Provider’s EDR Indicator is set to FALSE, SV group e - If the Service Provider’s EDR Indicator is set to FALSE SV group f |  |  |  |
| 8. | NPAC | NPAC Personnel perform multiple Full audits for each NPA-NXX of the following SVs to verify that all the appropriate updates were processed from the NPAC ‘re-send’ for the ‘Partial-Fail’ objects: - SV group 2a - SV group 2d - If the Service Provider’s EDR Indicator is set to FALSE, SV group 2e - If the Service Provider’s EDR Indicator is set to FALSE SV group 2f |  | NPAC |  | Using the Audit Results Log, verify that there were no updates made. If any updates were made as a result of running this audit, this test case fails. Verify that: - SV group 2a - SV group 2d - If the Service Provider’s EDR Indicator is set to FALSE, SV group 2e - If the Service Provider’s EDR Indicator is set to FALSE, SV group 2f |  |  |  |
| E. | Pass/Fail Analysis, NANC 169-2 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel were able to successfully process the Bulk Data Download file updates with their local databases in a timely fashion. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider LSMS was able to successfully process the ‘re-send’ request from the NPAC SMS for the ‘Partial-Fail’ objects. |  |  |  |  |  |  |  |

Note: Since the TN Range specified in the Bulk Data Download selection criteria is a subset of the test data created in the prerequisites, the Bulk Data Download file will not contain the full set of TNs.  Testers should verify this ‘subset’ of TN in the verification steps.

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 169-3 |  | SUT Priority: |  | SOA |  | N/A |
|  |  |  |  |  |  |  | LSMS |  | Optional |
|  | Objective: |  | NPAC OP GUI – NPAC Personnel initiate a Bulk Data Download of Subscription Data – Specifying Latest View of Subscription Version Activity a valid Time Range, and NOT specifying a TN range. Verification steps are performed to ensure the BDD file was processed successfully by the Service Provider system - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 169 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0a |  | Relevant Requirement(s): |  | RR3-315, RR3-316, RR3-317, RR3-319 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0a |  | Relevant Flow(s): |  | N/A |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | While the LSMS is ‘dis-associated’ with the NPAC SMS, NPAC personnel perform the following functions: a) Modify a range of 250, ‘Active’ Subscription Versions where the Service Provider under test is the Current Service Provider. Use simulators that are not associated with the NPAC and are receiving downloads for this NPA-NXX. Use a subset of SV group 1a above, and verify that these Subscription Versions exist with a status of ‘Active’ and a Failed SP List. (SV group 2a______________) b) Activate 50 Subscription Versions with a status of ‘Pending’, for which a filter for this NPA-NXX exists for the Service Provider under test. The Service Provider under test is the New Service Provider for these Subscription Versions. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify that these Subscription Versions have a status of ‘Active’. Remove the filter for this NPA-NXX so that these Subscription Versions will be included in the BDD for the service provider under test. (SV group 2b_________________) c) Disconnect a subset of the 250, ‘Active’ Subscription Versions (in step 2b) specifying Effective Release and Customer Disconnect dates in the future. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify that the status of these Subscription Versions is ‘Disconnect-Pending’. (SV group 2c___________) d) Activate a Number Pool Block for an NPA-NXX for which this Service Provider under test is accepting downloads, but it is another Service Provider’s Number Pool Block. Use simulators that are associated with the NPAC and accepting downloads for this NPA-NXX. Verify that the Pooled Subscription Versions have a status of ‘Partial-Fail’. (SV group 2d_____________________) (NPB 2d______________) e) Activate a range of 100, ‘Pending’ Subscription Versions on behalf of another Service Provider. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Set a filter for this NPA-NXX on behalf of the Service Provider under test. Verify that the Subscription Versions have a status of ‘Active’ with an empty Failed SP List. Remove filter for this NPA-NXX for the service provider under test before performing following steps. (SV group e____________) f) Put simulated SPID LSMS in recovery. Use at least one simulator that is associated with the NPAC and is accepting downloads for this NPA-NXX. Verify that the Service Provider under test is accepting downloads for this NPA-NXX. Modify 50 of the TNs that were activated in step e above. Verify that the Subscription Versions have a status of ‘Sending’. (SV group f___________________). During the test case retry timers will exhaust, and then the status of the SVs should be ‘Partial-Fail’. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel request a Bulk Data Download for Subscription Data, specifying Latest View of Subscription Version Activity a valid Time Range and not specifying a TN range, for the Service Provider participating in the test case. |  | NPAC |  | The NPAC SMS performs the request, generates the appropriate Bulk Data Download File(s) and automatically “FTP’s” the file(s) to the Service Provider’s directory on the NPAC SMS. |  |  |  |
| 2. | SP | Service Provider Personnel receive the Bulk Data Download File(s) and load the file(s) into their LSMS systems. |  | SP |  | The LSMS successfully processes the Bulk Data Download file(s) and reflects the updates described in the prerequisites above. The system is still ‘dis-associated’ from the NPAC SMS. |  |  |  |
| 3. | SP | Service Provider Personnel, using their LSMS, perform a local query for the Subscription Data to verify that the Subscription Version data was loaded. - SV group a - SV group b - SV group c - If the Service Provider’s EDR Indicator is set to FALSE, SV group d - If the Service Provider’s EDR Indicator is set to TRUE, SV group d is not included - SV group f |  | SP |  | Using the LSMS system, verify: - SV group a exists on the LSMS. - SV group b exists on the LSMS. - SV group c exists on the LSMS. - If the Service Provider’s EDR Indicator is set to FALSE, SV group d exists on the LSMS. - If the Service Provider’s EDR Indicator is set to TRUE, SV group d DOES NOT exist on the LSMS. - SV group f exists on the LSMS. |  |  |  |
| 4. | SP | After all NPAC ‘retry timers’ for the Subscription Versions specified in the prerequisites above have expired, Service Provider personnel perform appropriate steps to ‘associate’ with the NPAC SMS such that they will not recover additional information. |  | SP |  | The LSMS successfully re-associates with the NPAC SMS without recovering additional information. |  |  |  |
| 5. | NPAC | NPAC Personnel bring the simulated SPID LSMS that was in recovery in Prerequisite step f above, out of recovery. |  | NPAC |  | Verify that the simulated SPID that was in recovery during step f of the prerequisites is now out of recovery. Verify that the 50 subscription versions that were modified while this SPID was in recovery now have a status of ‘Active’ with a Failed SP List – including the service provider under test. |  |  |  |
| 6. | NPAC | NPAC Personnel perform multiple Full audits for each NPA-NXX included in the range of TNs specified in the prerequisites above for the Service Provider’s LSMS to verify that all the appropriate updates were processed from the Bulk Data Download File by the LSMS. |  | NPAC |  | Using the Audit Results Log, verify that there were no updates made. If any updates were made as a result of running this audit, this test case fails. Verify that: - SV group a exists on the LSMS. - SV group b exists on the LSMS. - SV group c exists on the LSMS. - If the Service Provider’s EDR Indicator is set to FALSE, SV group d exists on the LSMS. - If the Service Provider’s EDR Indicator is set to TRUE, SV group d DOES NOT exist on the LSMS. - SV group f exists on the LSMS. |  |  |  |
| 7. | NPAC | NPAC Personnel ‘re-send’ the following to the Service Provider under test: - SV group a that exists on the NPAC SMS with a status of ‘Partial-Fail’. - If the Service Provider’s EDR Indicator is set to FALSE, SV group d that exists on the NPAC SMS with a status of ‘Partial-Fail’. - SV group f that exists with a status of ‘Active’ and a Failed SP List including the service provider under test. NPAC SMS issues the appropriate messages to the LSMS in order to update the LSMS for these SVs. |  | SP |  | LSMS receives the resend requests from the NPAC SMS and issues a ‘duplicate object’ response to the NPAC SMS for: - SV group a - If the Service Provider’s EDR Indicator is set to FALSE, SV group d - SV group f |  |  |  |
| 8. | NPAC | NPAC Personnel perform multiple, Full audits for each NPA-NXX of the following SVs to verify that all the appropriate updates were processed from the NPAC ‘re-send’ for the ‘Partial-Fail’ objects: - SV group a - If the Service Provider’s EDR Indicator is set to FALSE, SV group d - SV group f |  | NPAC |  | Using the Audit Results Log, verify that there were no updates made. If any updates were made as a result of running this audit, this test case fails. Verify that: - SV group a - If the Service Provider’s EDR Indicator is set to FALSE, SV group d - SV group f |  |  |  |
| E. | Pass/Fail Analysis, NANC 169-3 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel were able to successfully process the Bulk Data Download file updates with their local databases in a timely fashion. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider LSMS was able to successfully process the ‘re-send’ request from the NPAC SMS for the ‘Partial-Fail’ objects. |  |  |  |  |  |  |  |

Note:  Since the Bulk Data Download file selection criteria does not include a TN Range in this test case, the file may contain additional Subscription Versions.  Testers can verify appropriate behavior for any additional data that may be in the file as is stated in the Test Steps above.

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 169-4 |  | SUT Priority: |  | SOA |  | N/A |
|  |  |  |  |  |  |  | LSMS |  | Optional |
|  | Objective: |  | NPAC OP GUI – NPAC Personnel initiate a Bulk Data Download of Subscription Data – Specifying Latest View of Subscription Version Activity a valid Time Range, and a TN range that is a subset of the prerequisite test data. Verification steps are performed to ensure the BDD file was processed successfully by the Service Provider system - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 169 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0a |  | Relevant Requirement(s): |  | RR3-319, RR3-320, RR3-321, RR3-322 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0a |  | Relevant Flow(s): |  | N/A |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | While the LSMS is ‘dis-associated’ from the NPAC SMS, NPAC personnel perform the following functions: a) Activate range of 500, ‘Pending’ Subscription Versions where the Service Provider under test is the Current Service Provider. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify these Subscription Versions exist with a status of ‘Partial Fail’. (SV group 2a______________) b) Activate 500, unique, ‘Pending’ Subscription Versions for which a filter for this NPA-NXX exists for the Service Provider under test. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify that the status of these 500 Subscription Versions is ‘Active’. (SV group 2b___________) Remove the filter for this NPA-NXX. c) Submit a Deferred Disconnect request for 50, unique, ‘Active’ Subscription Versions. The Service Provider under test is the Current Service Provider for these Subscription Versions. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify that these Subscription Versions have a status of ‘Disconnect-Pending’. (SV group 2c_________________) d) Activate a Number Pool Block for an NPA-NXX for which this Service Provider under test is accepting downloads, but it is another Service Provider’s Number Pool Block. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify that the Pooled Subscription Versions have a status of ‘Partial-Fail’. (SV group 2d_____________________) (NPB 2d______________) e) Activate an uncontaminated Number Pool Block on behalf of another Service Provider. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Set a filter for this NPA-NXX on behalf of the Service Provider under test. Verify that the Number Pool Block has a status of ‘Active’ with an empty Failed SP List. Remove the filter for this NPA-NXX for the Service Provider under test prior before performing following steps. (SV group e ________________), (NPB group e_________________________) f) Put simulated SPID LSMS in recovery. Use at least one simulator that is associated with the NPAC and is accepting downloads for this NPA-NXX. Verify that the Service Provider under test is accepting downloads for this NPA-NXX. Modify the Number Pool Block that was activated in step e above. Verify that the Number Pool Block has a status of ‘Sending’. (SV group f___________________), (NPB group f_________________________) During the test case retry timers will exhaust, and then the status of the SVs and NPB should be ‘Partial-Fail’. The TN Range specified in the Bulk Data Download Selection Criteria should be a ‘subset’ of the total of the TN Range used in the prerequisite steps. TN Range _________________________ |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel request a Bulk Data Download for Subscription Data, specifying Latest View of Subscription Version Activity a valid Time Range and specifying the TN range identified in the prerequisites above, for the Service Provider participating in the test case. By specifying this TN range, the content of the BDD will actually be a subset of the prerequisite data. |  | NPAC |  | The NPAC SMS performs the request, generates the appropriate Bulk Data Download File(s) and automatically “FTP’s” the file(s) to the Service Provider’s directory on the NPAC SMS. |  |  |  |
| 2. | SP | Service Provider Personnel receive the Bulk Data Download File(s) and load the file(s) into their LSMS systems. |  | SP |  | The LSMS successfully processes the Bulk Data Download file(s) and reflects the updates described in the prerequisites above. The system is still ‘dis-associated’ from the NPAC SMS. |  |  |  |
| 3. | SP | Service Provider Personnel, using their LSMS, perform a local query for the Subscription Data to verify that the Subscription Version data that matched the BDD criteria was loaded. NOTE: The BDD request was a subset of the total TNs manipulated in the Prerequisite Setup above. Verify the subset of data. - SV group a - SV group b - SV group c - If the Service Provider’s EDR Indicator is set to FALSE, SV group d - If the Service Provider’s EDR Indicator is set to TRUE, SV group d DOES NOT exist - If the Service Provider’s EDR Indicator is set to FALSE, SV group f - If the Service Provider’s EDR Indicator is set to TRUE, SV group f DOES NOT exist |  | SP |  | Using the LSMS system, verify: NOTE: The BDD request was a subset of the total TNs manipulated in the Prerequisite Setup above. Verify the subset of data. - SV group a exists on the LSMS. - SV group b exists on the LSMS. - SV group c exists on the LSMS. - If the Service Provider’s EDR Indicator is set to FALSE, SV group d exists on the LSMS. - If the Service Provider’s EDR Indicator is set to TRUE, SV group d DOES NOT exist on the LSMS. - If the Service Provider’s EDR Indicator is set to FALSE, SV group f exists on the LSMS. - If the Service Provider’s EDR Indicator is set to TRUE, SV group f DOES NOT exist on the LSMS. |  |  |  |
| 4. | SP | After all NPAC ‘retry timers’ for the Subscription Versions specified in the prerequisites above have expired, Service Provider personnel perform appropriate steps to ‘associate’ with the NPAC SMS such that they will not recover additional information. |  | SP |  | The LSMS successfully re-associates with the NPAC SMS without recovering additional information. . |  |  |  |
| 5. | NPAC | NPAC Personnel bring the simulated SPID LSMS that was in recovery in Prerequisite step f above, out of recovery. |  | NPAC |  | Verify that the simulated SPID that was in recovery during step f of the prerequisites is now out of recovery. Verify that the number pool block and respective pooled subscription versions that were modified while this SPID was in recovery now have a status of ‘Active’ with a Failed SP List that includes the service provider under test. |  |  |  |
| 6. | NPAC | NPAC Personnel perform multiple Full audits for each NPA-NXX in the range of TNs specified in the prerequisites above for the Service Provider’s LSMS to verify that all the appropriate updates were processed from the Bulk Data Download File by the LSMS. |  | NPAC |  | Using the Audit Results Log, verify that there were no updates made. If any updates were made as a result of running this audit, this test case fails. Verify that: - SV group a exists on the LSMS. - SV group b exists on the LSMS. - SV group c exists on the LSMS. - If the Service Provider’s EDR Indicator is set to FALSE, SV group d exists on the LSMS. - If the Service Provider’s EDR Indicator is set to TRUE, SV group d DOES NOT exist on the LSMS. - If the Service Provider’s EDR Indicator is set to FALSE, SV group f exists on the LSMS. - If the Service Provider’s EDR Indicator is set to TRUE, SV group f DOES NOT exist on the LSMS. |  |  |  |
| 7. | NPAC | NPAC Personnel ‘re-send’ the following to the Service Provider under test: NOTE: The BDD request was a subset of the total TNs manipulated in the Prerequisite Setup above. Resend the respective subset of data. - SV group a that exists on the NPAC SMS with a status of ‘Partial-Fail’. - If the Service Provider’s EDR Indicator is set to FALSE, SV group d that exists on the NPAC SMS with a status of ‘Partial-Fail’. - If the Service Provider’s EDR Indicator is set to FALSE, pooled SV group f that exists on the NPAC SMS with a status of ‘Active’ with a Failed SP List that includes the service provider under test. NPAC SMS issues the appropriate messages to the LSMS in order to update the LSMS for these SVs. |  | SP |  | LSMS receives the resend requests from the NPAC SMS and issues a ‘duplicate object’ response to the NPAC SMS for: NOTE: The BDD request was a subset of the total TNs manipulated in the Prerequisite Setup above. Verify the subset of data. - SV group a - If the Service Provider’s EDR Indicator is set to FALSE, SV group d - If the Service Provider’s EDR Indicator is set to FALSE, pooled SV group f |  |  |  |
| 8. | NPAC | NPAC Personnel perform multiple Full audits for each NPA-NXX in the following SVs to verify that all the appropriate updates were processed from the NPAC ‘re-send’ for the ‘Partial-Fail’ objects: - SV group a - If the Service Provider’s EDR Indicator is set to FALSE, SV group d - If the Service Provider’s EDR Indicator is set to FALSE, SV group f |  | NPAC |  | Using the Audit Results Log, verify that there were no updates made. If any updates were made as a result of running this audit, this test case fails. Verify that: - SV group a - If the Service Provider’s EDR Indicator is set to FALSE, SV group d - If the Service Provider’s EDR Indicator is set to FALSE, SV group f |  |  |  |
| E. | Pass/Fail Analysis, NANC 169-4 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel were able to successfully process the Bulk Data Download file updates with their local databases in a timely fashion. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider LSMS was able to successfully process the ‘re-send’ request from the NPAC SMS for the ‘Partial-Fail’ objects. |  |  |  |  |  |  |  |

Note: Since the TN Range specified in the Bulk Data Download selection criteria is a subset of the test data created in the prerequisites, the Bulk Data Download file will not contain the full set of TNs.  Testers should verify this ‘subset’ of TN in the verification steps.

# NANC 187 – Linked Action Replies

NOTE:  Service Provider’s Local SMS Linked Replies Indicator, Service Provider’s SOA Linked Replies Indicator, NPAC Customer LSMS NPA-NXX-X Indicator, and NPAC Customer SOA NPA-NXX-X Indicator should be set to production values for testing NANC 187 functionality.

The Network Data, Block Data, Subscription Version Data and Notification Data Linked Replies Blocking Factors and Maximum Linked Recovered Objects should be set to predetermined values  identified by the test team.  These parameters may not be set to ‘production or default’ values so as to be able to complete testing in an expeditious manner.

The test data identified in the prerequisites for these test cases are recommendations to achieve the test case objectives.  Many permutations of prerequisite data may create the appropriate test scenario and thus meet the test objective.  Test Engineers should consider each Service Provider’s capabilities and tailor the test data as is appropriate to meet the test objective and execute the test case expeditiously.  For example, consider whether the Service Provider supports NPA-NXX-X’s, Ranged Notifications and Linked Action Replies.  Based on this information, you may need to perform more or less activity to meet the test case objective.

This is a recovery test case written to cover both Service Provider systems that DO and DO NOT support Linked Replies, thus, this test case will supersede Test Case 8.1 from the NPAC SMS/Service Provider Certification & Regression Test Plan.

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 187-1 |  | SUT Priority: |  | SOA |  | N/A |
|  |  |  |  |  |  |  | LSMS |  | Required |
|  | Objective: |  | LSMS – Service Provider Personnel for either an EDR or non-EDR LSMS submit a resynchronization request for Network Data, Block Data, Subscription Version Data and Notification Data by time range, over the LSMS to NPAC SMS Interface, with the Service Provider’s Local SMS Linked Replies Indicator set to their production setting. The recovery response includes a number of Network Data objects, Number Pool Block objects, Notifications and Subscription Versions less than or equal to their respective Linked Replies Blocking Factors. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 187 and NANC 297 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0 |  | Relevant Requirement(s): |  | RR6-85, RR6-93, RR6-97, RR6-98, RR6-99, RR6-100, RR6-101, RR6-102, RR6-103, RR6-90, RR6-105 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0 |  | Relevant Flow(s): |  | B.7.1.1, B.7.1.2 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Prerequisite data may be set up different depending on if this test case is being run during Individual testing versus Group Testing. For example, during Individual Testing, if the service provider under test does not support NPA-NXX-X’s, don’t perform any of the related tasks or verify related data. During Group testing, need to create prerequisite data such that you meet the test case objective. If service provider’s under test don’t support Ranged Notifications for example, you will either need to not perform the number pool block activities, or you may filter the NPA-NXX of the number pool block data for the service provider(s) that doesn’t support Range Notifications. 1. While the EDR/non-EDR LSMS is disconnected from the NPAC SMS, NPAC Personnel should perform the following functions: - Create an LRN. - Delete an LRN for a different Service Provider. - Create an NPA-NXX. - Delete an NPA-NXX for a different Service Provider. - Activate a new Number Pool Block. - DePool an existing Number Pool Block. - Create NPA-NXX-X Information for different Service Providers. - Modify NPA-NXX-X Information for different Service Providers. - Delete NPA-NXX-X Information for different Service Providers. - Create an Inter-SP Subscription Version for a Pooled TN. - Disconnect a Pooled Ported TN. - Create an Inter-SP, Port-To-Original Subscription Version for a Pooled Ported TN. - Create a Subscription Version with the NPA-NXX created above. - Issue an activate request for an Inter-Service Provider Subscription Version. - Issue an Activate request for a range of two Inter-Service Provider Subscription Versions, where the broadcast to the LSMSs goes to a ‘partial failure’ state. - Using an NPANXX with filters set for the LSMS under test, and using a simulator Activate 2 ‘pending’ SVs. Verify that these subscription versions have a status of ‘Sending’. - Put simulated SPID LSMS in recovery. Use at least one simulator that is associated with the NPAC and is accepting downloads for this NPA-NXX. Verify that the Service Provider under test is accepting downloads for this NPA-NXX. Activate an uncontaminated Number Pool Block on behalf of another Service Provider. Verify that the Pooled SVs and NPB have a status of ‘Sending’. 2. While the EDR/non-EDR LSMS is in recovery, NPAC personnel should perform the following functions: - Create an NPA-NXX. - Activate a Subscription Version. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | The Service Provider LSMS should be ‘disassociated’ from the NPAC SMS while NPAC Personnel are performing the setup specified above. |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | The Service Provider establishes an association from their LSMS to the NPAC SMS with the resynchronization flag set to TRUE. |  | NPAC |  | The NPAC SMS receives the association bind request from the LSMS. Once the association is established, the NPAC SMS queues all current updates. |  |  |  |
| 2. | SP | The LSMS issues an M-ACTION Request lnpDownload (network data) to the NPAC SMS and specifies the time range for the resync request. |  | NPAC |  | The NPAC SMS receives the M-ACTION. 1) If the Service Provider’s Local SMS Linked Replies Indicator is set to FALSE, NPAC issues a single, normal M-ACTION Response lnpDownload message back to the LSMS with the Network Data. 2) If the Service Provider’s Local SMS Linked Replies Indicator is set to TRUE, NPAC issues a single, normal M-ACTION Response lnpDownload message back to the LSMS with the Network Data updates. The data does not exceed the Network Data Blocking factor, so there shall be only 1 normal message sent in this instance. |  |  |  |
| 3. | NPAC | As soon as the M-ACTION Request is received, NPAC Personnel issue an activate for an Intra or Inter Service Provider Subscription Version AND create a new NPA-NXX, belonging to any Service Provider. |  | NPAC |  | The NPAC SMS receives the M-ACTION Requests for the activate and NPA-NXX create. It then sends an M-ACTION response to itself for the NPA-NXX create. |  |  |  |
| 4. | NPAC | NPAC SMS issues an M-SET Request to itself to set the subscriptionVersionNPAC object (subscription version). |  | NPAC |  | NPAC SMS issues an M-SET Response to itself. The subscription version status is set to ‘sending.’ The subscriptionVersionActivationTimeStamp and subscriptionVersionModifiedTimeStamp are set. |  |  |  |
| 5. | NPAC | The NPAC SMS checks to see if the M-CREATE subscriptionVersion can be sent to the Service Provider LSMS |  | NPAC |  | The NPAC SMS does NOT send the M-CREATE subscriptionVersion to the Service Provider LSMS, since the LSMS is still in recovery mode. |  |  |  |
| 6. | NPAC | The NPAC SMS checks to see if the M-CREATE serviceProvNPA-NXX can be sent to the Service Provider LSMS |  | NPAC |  | The NPAC SMS does NOT send the M-CREATE seviceProvNPA-NXX to the Service Provider LSMS, since the LSMS is still in recovery mode. |  |  |  |
| 7. | SP | The LSMS Service Provider issues an M-ACTION Request lnpDownload (subscription data) to the NPAC SMS and specifies the start time for the resync request. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the LSMS. 1) If the Service Provider’s Local SMS Linked Replies Indicator is set to FALSE, NPAC issues a single, normal M-ACTION Response lnpDownload message back to the LSMS with the Subscription Version Data. 2) If the Service Provider’s Local SMS Linked Replies Indicator is set to TRUE, NPAC issues a single, normal M-ACTION Response lnpDownload message back to the LSMS with the Subscription Version data. The data does not exceed the Subscription Version Data Blocking factor, so there shall be only 1 normal message sent in this instance. |  |  |  |
| 8. conditional | SP | The EDR LSMS Service Provider issues an M-ACTION Request lnpDownload (number pool block data) to the NPAC SMS and specifies the resync start time. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the EDR LSMS. 1) If the Service Provider’s Local SMS Linked Replies Indicator is set to FALSE, NPAC issues single, normal M-ACTION Response lnpDownload message back to the LSMS with the number pool block updates. 2) If the Service Provider’s Local SMS Linked Replies Indicator is set to TRUE, NPAC issues a single, normal M-ACTION Response lnpDownload message back to the LSMS with the number pool block updates. The data does not exceed the Number Pool Block Data Blocking factor, so there shall be only 1 normal message sent in this instance. |  |  |  |
| 9. | SP | The LSMS Service Provider issues an M-ACTION Request lnpNotificationRecovery (notification data) to the NPAC SMS and specifies a time range. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the LSMS: 1) If the Service Provider’s Local SMS Linked Replies Indicator is set to FALSE, NPAC issues single, normal M-ACTION Response lnpDownload message back to the LSMS with the notification updates. 2) If the Service Provider’s Local SMS Linked Replies Indicator is set to TRUE, NPAC issues a single, normal M-ACTION Response lnpDownload message back to the LSMS with the notification updates. |  |  |  |
| 10. | NPAC | The NPAC SMS sends the M-EVENT-REPORT(s) to the Block Holder SOA for a number pool block with the SOA-Origination flag set to TRUE whose subscriptionFailedSP-List was just updated due to the number pool block download. The status attribute value change contains the current status and the subscriptionFailedSP-List of the number pool block object. |  | SP |  | Block Holder SOA, to which the NPAC SMS issued an M-EVENT-REPORT, issue an M-EVENT-REPORT Confirmation back to the NPAC SMS. |  |  |  |
| 11. | SP | The LSMS Service Provider issues an M-ACTION Request lnpRecoveryComplete to the NPAC SMS to set the resynchronization flag to FALSE. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the LSMS and sets the resynchronization flag to ‘off’. |  |  |  |
| 12. | NPAC | NPAC SMS issues the following messages to the LSMS for the requests made while the LSMS was in recovery: - M-CREATE Request serviceProvNPA-NXX for the NPA-NXX that was created during recovery. - M-CREATE Request subscriptionVersion for the subscription version that was activated during recovery. |  | SP |  | The service provider’s LSMS receives the requests from the NPAC SMS for the requests that occurred during recovery and issues the following responses: - M-CREATE Response serviceProvNPA-NXX for the NPA-NXX that was created during recovery, indicating the LSMS successfully received/processed the request. - M-CREATE Response subscriptionVersion for the subscription version that was activated during recovery, indicating the LSMS successfully received/processed the request. |  |  |  |
| 13. | SP | Service Provider Personnel, using the LSMS, perform a local query for the data updated in this test case. |  | SP |  | Verify that the following updates were sent: - 1 LRN create. - 1 LRN delete. - 1 NPA-NXX create. - 1 NPA-NXX delete. - 1 Number Pool Block activate. - 1 Number Pool Block depool. - 1 NPA-NXX-X create – if supported by the Service Provider LSMS. - 1 NPA-NXX-X modify – if supported by the Service Provider LSMS. - 1 NPA-NXX-X delete – if supported by the Service Provider LSMS. - 1 Pooled Ported TN disconnect. - 1 First port of NPA-NXX notification. - 1 Single subscription version activate. - 2 subscription versions that were activated. - 1 Number Pool Block activate. - 1 NPA-NXX create after recovery is complete - 1 Subscription Version activate after recovery is complete |  |  |  |
| 14. | NPAC | NPAC Personnel perform a Full audit for the Subscription Versions that were activated during this test case. |  | NPAC |  | Using the Audit Results Log, verify that there were no updates made. If any updates were made as a result of running this audit, this test case fails. |  |  |  |
| E. | Pass/Fail Analysis, NANC 187-1 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 187-2 |  | SUT Priority: |  | SOA |  | N/A |
|  |  |  |  |  |  |  | LSMS |  | Required |
|  | Objective: |  | LSMS – Service Provider Personnel for either an EDR or non-EDR LSMS submit a resynchronization request for Network Data, and Subscription Version Data by time range, over the LSMS to NPAC SMS Interface, with the Service Provider’s Local SMS Linked Replies Indicator set to their production setting. The recovery response includes a number of Network Data objects greater than the Network Data Linked Replies Blocking Factor and less than the Network Data Maximum Linked Recovered Objects as well as a number of Subscription Version objects greater than the Subscription Data Linked Replies Blocking Factor and less than the Subscription Data Maximum Linked Recovered Objects. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 187 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0 |  | Relevant Requirement(s): |  | RR6-87, RR6-88, RR6-93, RR6-103, RR6-90, RR6-95, RR6-104, RR6-96 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0 |  | Relevant Flow(s): |  | B.7.1.1, B.7.1.2 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. While the EDR/non-EDR LSMS is disconnected from the NPAC SMS, NPAC Personnel should perform the following functions: a) Create 10 LRNs. (LRN group a) b) Delete 10 LRNs for a different Service Provider. (LRN group b) c) Create 20 NPA-NXXs. (NPA-NXX group c) d) Delete 10 NPA-NXXs for a different Service Provider. (NPA-NXX group d) e) Activate 40 new Blocks. (NPB group e) f) DePool 20 existing Blocks. (NPB group f) g) Create 2 NPA-NXX-Xs for different Service Provider. (Dash X group g) h) Modify an NPA-NXX-X for different Service Provider. (Dash X group h) i) Delete an NPA-NXX-X for a different Service Provider. (Dash X group i) j) Activate 20 Inter-SP Subscription Version for a Pooled TN. (SV group j) k) Disconnect 25 Pooled Ported TN. (SV group k) l) Activate 20 Inter-SP, Port-To-Original Subscription Version for a Pooled Ported TN. (SV group l) m) Create 50 Subscription Version with the NPA-NXX created above. (SV group m) n) Issue an activate request for 20 Inter-Service Provider Subscription Version. (SV group n) o) Issue an Activate request for a range of 10 Inter-Service Provider Subscription Versions. (SV group o) 2. While the EDR/non-EDR LSMS is in recovery, NPAC personnel should perform the following functions: - Create an NPA-NXX. - Activate a Subscription Version. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | The Service Provider LSMS should be ‘disassociated’ from the NPAC SMS while NPAC Personnel are performing the setup specified above. |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | The Service Provider establishes an association from their LSMS to the NPAC SMS with the resynchronization flag set to TRUE. |  | NPAC |  | The NPAC SMS receives the association bind request from the LSMS. Once the association is established, the NPAC SMS queues all current updates. |  |  |  |
| 2. | SP | The LSMS issues an M-ACTION Request lnpDownload (network data) to the NPAC SMS and specifies the time range for the resync request. |  | NPAC |  | The NPAC SMS receives the M-ACTION. 1) If the Service Provider’s Local SMS Linked Replies Indicator is set to FALSE, NPAC issues a single M-ACTION Response lnpDownload message back to the LSMS with the Network Data updates for - LRN group a - LRN group b - NPA-NXX group c - NPA-NXX group d - Dash X group g, if supported by the Service Provider under test - Dash X group h, if supported by the Service Provider under test - Dash X group i, if supported by the Service Provider under test 2) If the Service Provider’s Local SMS Linked Replies Indicator is set to TRUE, NPAC issues multiple, linked M-ACTION replies, lnpDownload, followed by a non-linked, empty, normal response (indicating the end of the linked reply data) back to the LSMS with the Network Data updates. These messages shall be linked for groups of (50) objects – there should be 2 linked replies. |  |  |  |
| 3. | NPAC | As soon as the M-ACTION Request is received, NPAC Personnel issue an activate for an Intra or Inter Service Provider Subscription Version (SV 3) AND create a new NPA-NXX, belonging to any Service Provider. |  | NPAC |  | The NPAC SMS receives the M-ACTION Requests for the activate and NPA-NXX create. It then sends an M-ACTION response to itself for the NPA-NXX create. |  |  |  |
| 4. | NPAC | NPAC SMS issues an M-SET Request to itself to set the subscriptionVersionNPAC object (subscription version). |  | NPAC |  | NPAC SMS issues an M-SET Response to itself. The subscription version status is set to ‘sending.’ The subscriptionVersionActivationTimeStamp and subscriptionVersionModifiedTimeStamp are set. |  |  |  |
| 5. | NPAC | The NPAC SMS checks to see if the M-CREATE subscriptionVersion can be sent to the Service Provider LSMS |  | NPAC |  | The NPAC SMS does NOT send the M-CREATE subscriptionVersion to the Service Provider LSMS, since the LSMS is still in recovery mode. |  |  |  |
| 6. | NPAC | The NPAC SMS checks to see if the M-CREATE serviceProvNPA-NXX can be sent to the Service Provider LSMS |  | NPAC |  | The NPAC SMS does NOT send the M-CREATE seviceProvNPA-NXX to the Service Provider LSMS, since the LSMS is still in recovery mode. |  |  |  |
| 7. | SP | The EDR and/or non-EDR LSMS Service Provider issues an M-ACTION Request lnpDownload (subscription data) to the NPAC SMS and specifies the start time for the resync request. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the LSMS Service Provider. 1) If the Service Provider’s Local SMS Linked Replies Indicator is set to FALSE, NPAC issues a single M-ACTION Response lnpDownload messages back to the LSMS with the Subscription Version Data updates for: - If non-EDR LSMS, Pooled Subscription Versions associated with NPB group e - If non-EDR LSMS, Pooled Subscription Versions associated with NPB group f - SV group j - SV group k - SV group l - SV group n - SV group o 2) If the Service Provider’s Local SMS Linked Replies Indicator is set to TRUE, NPAC issues multiple, linked M-ACTION replies, lnpDownload, followed by a non-linked, empty, normal response (indicating the end of the linked reply data) back to the LSMS with the ‘non-pooled’ Subscription Version Data updates. This message shall be linked for groups of (50) objects – there should be at least 3 linked replies, if the Service Provider under test is non-EDR and does not support Ranged Notifications, there will be at least 43 linked replies. If the service provider under test supports Ranged Notifications, there may be fewer than 43 linked replies based on the parameter setting. |  |  |  |
| 8. conditional | SP | The EDR LSMS Service Provider issues an M-ACTION Request lnpDownload (number pool block data) to the NPAC SMS and specifies a time range. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the EDR LSMS: If the Service Provider’s Local SMS Linked Replies Indicator is set to FALSE, NPAC issues single, normal M-ACTION Response lnpDownload message back to the with the number pool block updates for: - NPB group e - NPB group f 2) If the Service Provider’s Local SMS Linked Replies Indicator is set to TRUE, NPAC issues multiple, linked M-ACTION replies, lnpDownload, followed by a non-linked, empty, normal response (indicating the end of the linked reply data) back to the EDR LSMS with the number pool block updates. These messages shall be linked for groups of (50) objects – there should be 2 linked replies. |  |  |  |
| 9. conditional | SP | The LSMS Service Provider issues an M-ACTION Request lnpNotificationRecovery (notification data) to the NPAC SMS and specifies a time range. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the LSMS: 1) If the Service Provider’s Local SMS Linked Replies Indicator is set to FALSE, NPAC issues single, normal M-ACTION Response lnpDownload message back to the LSMS with the notification updates. 2) If the Service Provider’s Local SMS Linked Replies Indicator is set to TRUE, NPAC issues a single, normal M-ACTION Response lnpDownload message back to the LSMS with the notification updates. |  |  |  |
| 10. | SP | The LSMS Service Provider issues an M-ACTION Request lnpRecoveryComplete to the NPAC SMS to set the resynchronization flag to FALSE. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the LSMS and sets the resynchronization flag to ‘off’. |  |  |  |
| 11. | NPAC | NPAC SMS issues the following messages to the LSMS for the request made while the LSMS was in recovery: - M-CREATE Request serviceProvNPA-NXX for the NPA-NXX that was created during recovery. - M-CREATE Request subscriptionVersion for the subscription version that was activated during recovery. |  | SP |  | The service provider’s LSMS receives the requests from the NPAC SMS for the requests that occurred during recovery and issues the following responses: - M-CREATE Response serviceProvNPA-NXX for the NPA-NXX that was created during recovery, indicating the LSMS successfully received/processed the request. - M-CREATE Response subscriptionVersion for the subscription version that was activated during recovery, indicating the LSMS successfully received/processed the request. |  |  |  |
| 12. | SP | Service Provider Personnel, using the LSMS, perform a local query for the data updated in this test case. |  | SP |  | Verify that the following updates were sent: - LRN group a was created. - LRN group b was delete. - NPA-NXX group c was activated. - NPA-NXX group d was depooled. - On non-EDR LSMSs, Pooled Subscription Versions associated with NPB e were created. - On non-EDR LSMSs, Pooled Subscription Versions associated with NPB f were deleted. - NPA-NXX-X (Dash X group g) was created – if supported by the Service Provider LSMS. - NPA-NXX-X (Dash X group h) was modified – if supported by the Service Provider LSMS. - NPA-NXX-X (Dash X group i) was deleted – if supported by the Service Provider LSMS. - SV group j was created/activated. - SV group k was disconnected. - SV group l was created/activated. - First port of NPA-NXX notification associated with SV group m was sent. - SV group n was activated. - SV group o was activated. - 1 NPA-NXX create after recovery is complete - SV3 was activated after recovery is complete. |  |  |  |
| 13. | NPAC | NPAC Personnel perform a Full audit for the Subscription Versions that were activated during this test case. |  | NPAC |  | Using the Audit Results Log, verify that there were no updates made. If any updates were made as a result of running this audit, this test case fails. |  |  |  |
| E. | Pass/Fail Analysis, NANC 187-2 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 187-3 |  | SUT Priority: |  | SOA |  | N/A |
|  |  |  |  |  |  |  | LSMS |  | Required |
|  | Objective: |  | LSMS – Service Provider Personnel for either an EDR or non-EDR LSMS submit a resynchronization request for Network Data, Number Pool Block data and Subscription Version Data by time range, over the LSMS to NPAC SMS Interface, with the Service Provider’s Local SMS Linked Replies Indicator set to their production setting. The recovery response includes a number of Network Data objects, Number Pool Block objects and Subscription Version objects greater than the respective Maximum Linked Recovered Objects and Maximum Number Download Records parameters. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 187 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0 |  | Relevant Requirement(s): |  | RR6-95, RR6-104, RR6-106 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0 |  | Relevant Flow(s): |  | B.7.1.1, B.7.1.2 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Determine the settings for the Maximum Recovered objects and then substitute the ‘X number references following in order to exceed these parameter settings. 2. While the EDR/non-EDR LSMS is disconnected from the NPAC SMS, NPAC Personnel should perform the following functions: a) Create 10 LRNs. (LRN group a) b) Delete 10 LRNs for a different Service Provider. (LRN group b) c) Create 10 NPA-NXXs. (NPA-NXX group c) d) Delete 10 NPA-NXXs for a different Service Provider. (NPA-NXX group d) e) Activate 20 new Blocks. (NPB group e) f) DePool 20 existing Blocks. (NPB group f) g) Create 5 NPA-NXX-Xs for different Service Provider. (Dash X group g) h) Modify 5 NPA-NXX-X for different Service Provider. (Dash X group h) i) Delete 5 NPA-NXX-X for a different Service Provider. (Dash X group i) j) Activate 25 Inter-SP Subscription Version for a Pooled TN. (SV group j) k) Disconnect 25 Pooled Ported TN. (SV group k) l) Activate 25 Inter-SP, Port-To-Original Subscription Version for a Pooled Ported TN. (SV group l) m) Create 1 Subscription Version with the NPA-NXX created above. (SV group m) n) Issue an activate request for 25 Inter-Service Provider Subscription Version. (SV group n) 0) Issue an Activate request for a range of 25 Inter-Service Provider Subscription Versions. (SV group 0) |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | The Service Provider LSMS should be ‘disassociated’ from the NPAC SMS while NPAC Personnel are performing the setup specified above. |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | The Service Provider establishes an association from their LSMS to the NPAC SMS with the resynchronization flag set to TRUE. |  | NPAC |  | The NPAC SMS receives the association bind request from the LSMS. Once the association is established, the NPAC SMS queues all current updates. |  |  |  |
| 2. | SP | The LSMS issues an M-ACTION Request lnpDownload (network data) to the NPAC SMS and specifies the time range for the resync request. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the LSMS and determines that for the time range requested, the Network Data exceeds the Network Data Maximum Recovered Objects parameter (if the service provider supports linked replies) or the Maximum Number of Download Records (if the service provider does not support linked replies). The NPAC SMS issues an M-ACTION Response lnpDownload, Criteria-too-large to the requesting LSMS. |  |  |  |
| 3. | SP | After receiving the M-ACTION Response ‘Criteria-to-large’ the LSMS issues an M-ACTION Request lnpDownload (network data) to the NPAC SMS and specifies a smaller time range (than that indicated in row 2 above) for the resync request. Note: Row 2 may occur over and over until the M-ACITON Request indicates a time range that yields data less than the ‘Network Data Maximum Linked Recovered Objects’ for service provider’s that support Linked Replies – or less than the Maximum Number of Download Records for those service provider’s that don’t support Linked Replies. |  | NPAC |  | Once the NPAC SMS receives an M-ACTION request that specifies time range that yields an amount of data less than the ‘Network Data Maximum Linked Recovered Objects’ for service provider’s that support Linked Replies, or less than the ‘Maximum Number of Download Records’ for service provider’s that don’t support Linked Replies: 1) If the Service Provider’s Local SMS Linked Replies Indicator is set to FALSE, NPAC issues a single M-ACTION Response lnpDownload message back to the LSMS with the Network Data updates for - LRN group a - LRN group b - NPA-NXX group c - NPA-NXX group d - Dash X group g, if supported by the Service Provider under test - Dash X group h, if supported by the Service Provider under test - Dash X group i, if supported by the Service Provider under test 2) If the Service Provider’s Local SMS Linked Replies Indicator is set to TRUE, NPAC issues multiple, linked M-ACTION replies, lnpDownload, followed by a non-linked, empty, normal response (indicating the end of the linked reply data) back to the LSMS with the Network Data updates. These messages shall be linked for groups of (50) objects – there should be 2 linked replies. |  |  |  |
| 4. | SP | The LSMS Service Provider issues an M-ACTION Request lnpDownload (subscription data) to the NPAC SMS and specifies the start time for the resync request. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the LSMS and determines that for the time range requested, the Subscription Version data exceeds the Subscription Version Data Maximum Recovered Objects parameter (if the service provider supports linked replies) or the Maximum Number of Subscription Records (if the service provider does not support linked replies). The NPAC SMS issues an M-ACTION Response lnpDownload, Criteria-too-large to the requesting LSMS. |  |  |  |
| 5. | SP | After receiving the M-ACTION Response ‘Criteria-to-large’ the LSMS issues an M-ACTION Request lnpDownload (subscription data) to the NPAC SMS and specifies a smaller time range (than that indicated in row 4 above) for the resync request. Note: Row 4 may occur over and over until the M-ACITON Request indicates a time range that yields data less than the ‘Subscription Data Maximum Linked Recovered Objects’ for service provider’s that support Linked Replies – or less than the Maximum Number of Subscription Records for those service provider’s that don’t support Linked Replies. |  | NPAC |  | Once the NPAC SMS receives an M-ACTION request that specifies time range that yields an amount of data less than the ‘Subscription Data Maximum Linked Recovered Objects’ for service provider’s that support Linked Replies, or less than the ‘Maximum Number of Subscription Records’ for service provider’s that don’t support Linked Replies: 1) If the Service Provider’s Local SMS Linked Replies Indicator is set to FALSE, NPAC issues a single M-ACTION Response lnpDownload message back to the LSMS with the Network Data updates for - If non-EDR LSMS, Pooled Subscription Versions associated with NPB group e - If non-EDR LSMS, Pooled Subscription Versions associated with NPB group f - SV group j - SV group k - SV group l - SV group n - SV group o 2) If the Service Provider’s Local SMS Linked Replies Indicator is set to TRUE, NPAC issues multiple, linked M-ACTION replies, lnpDownload, followed by a non-linked, empty, normal response (indicating the end of the linked reply data) back to the LSMS with the Subscription Version Data updates. This message shall be linked for groups of (50) objects – For the EDR LSMS there should be at least 3 linked replies of non-pooled subscription version data, if the Service Provider under test is non-EDR and does not support Ranged Notifications, there will be at least 43 linked replies. If the service provider under test supports Ranged Notifications, there may be fewer than 43 linked replies based on the parameter setting. |  |  |  |
| 6. | SP | The LSMS Service Provider issues an M-ACTION Request lnpDownload (number pool block data) to the NPAC SMS and specifies the resync start time. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the LSMS and determines that for the time range requested, the Number Pool Block data exceeds the Number Pool Block Data Maximum Recovered Objects parameter (if the service provider supports linked replies) or the Maximum Number of Download Records (if the service provider does not support linked replies). The NPAC SMS issues an M-ACTION Response lnpDownload, Criteria-too-large to the requesting LSMS. |  |  |  |
| 7. | SP | After receiving the M-ACTION Response ‘Criteria-to-large’ the LSMS issues an M-ACTION Request lnpDownload (number pool block data) to the NPAC SMS and specifies a smaller time range (than that indicated in row 6 above) for the resync request. Note: Row 4 may occur over and over until the M-ACITON Request indicates a time range that yields data less than the ‘Number Pool Block Data Maximum Linked Recovered Objects’ for service provider’s that support Linked Replies – or less than the Maximum Number of Download Records for those service provider’s that don’t support Linked Replies. |  | NPAC |  | Once the NPAC SMS receives an M-ACTION request that specifies a time range that yields an amount of data less than the ‘Number Pool Block Data Maximum Linked Recovered Objects’ for service provider’s that support Linked Replies, or less than the ‘Maximum Number of Download Records’ for service provider’s that don’t support Linked Replies: 1) If the Service Provider’s Local SMS Linked Replies Indicator is set to FALSE, NPAC issues a single M-ACTION Response lnpDownload message back to the LSMS with the Network Data updates for - NPB group e - NPB group f 2) If the Service Provider’s Local SMS Linked Replies Indicator is set to TRUE, NPAC issues multiple, linked M-ACTION replies, lnpDownload, followed by a non-linked, empty, normal response (indicating the end of the linked reply data) back to the EDR LSMS with the number pool block updates. These messages shall be linked for groups of (50) objects – there should be 2 linked replies. |  |  |  |
| 8. conditional | SP | The LSMS Service Provider issues an M-ACTION Request lnpNotificationRecovery (notification data) to the NPAC SMS and specifies a time range. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the LSMS: 1) If the Service Provider’s Local SMS Linked Replies Indicator is set to FALSE, NPAC issues single, normal M-ACTION Response lnpDownload message back to the LSMS with the notification updates. 2) If the Service Provider’s Local SMS Linked Replies Indicator is set to TRUE, NPAC issues a single, normal M-ACTION Response lnpDownload message back to the LSMS with the notification updates. |  |  |  |
| 9. | SP | The LSMS Service Provider issues an M-ACTION Request lnpRecoveryComplete to the NPAC SMS to set the resynchronization flag to FALSE. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the LSMS and replies back to the LSMS with data updates at the next scheduled interval for the NPA-NXX that was created during resynchronization and the Subscription Version that was activated during resynchronization. |  |  |  |
| 10. | SP | The LSMS receives the M-ACTION Response from the NPAC SMS.. |  |  |  |  |  |  |  |
| 11. | NPAC | NPAC Personnel verify that no data was sent in the initial action responses for the network data, subscription data and number pool block data requests from the LSMS.. |  | NPAC |  | Verify that no data was sent in the initial action responses sent for network data, subscription data and number pool block data request from the LSMS. Note to Test Engineers: NPAC Personnel may watch the router of the service provider under test to verify the 1st request for data types (network data, subscription data and number pool block data) result in a ‘criteria-too-large’ response. |  |  |  |
| E. | Pass/Fail Analysis, NANC 187-3 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

This is a recovery test case written to cover both Service Provider systems that DO and DO NOT support Linked Replies, thus, this test case will supersede TC 8.3 from the NPAC SMS/Service Provider Certification & Regression Test Plan

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 187-4 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | SOA – Service Provider Personnel submit a resynchronization request for Network Data and Notification Data by time range, over the SOA to NPAC SMS Interface, with the Service Provider’s SOA Linked Replies Indicator set to their production setting. The recovery response includes a number of Network Data objects less than or equal to the Network Data Linked Replies Blocking Factor and a number of Notifications less than or equal to the Notification Data Linked Replies Blocking Factor. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 187 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0 |  | Relevant Requirement(s): |  | RR6-92, RR6-89 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0 |  | Relevant Flow(s): |  | B.7.2 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Prerequisite data may be set up different depending on if this test case is being run during Individual testing versus Group Testing. For example, during Individual Testing, if the service provider under test does not support NPA-NXX-X’s, don’t perform any of the related tasks or verify related data. 1) While the SOA is disconnected from the NPAC SMS, NPAC Personnel should perform the following functions for data within the time range to be resynced: - Create an LRN. - Delete an LRN for a different Service Provider. - Create an NPA-NXX. - Delete an NPA-NXX for a different Service Provider. - Create NPA-NXX-X Information for different Service Providers. - Modify NPA-NXX-X Information for different Service Providers. - Delete NPA-NXX-X Information for different Service Providers. - Activate a Block on behalf of the Service Provider that is ‘down’ with SOA Origination TRUE. - Create a Subscription Version with the NPA-NXX created above on behalf of the Old Service Provider and where the Service Provider Under Test is the New Service Provider; let the Initial and Final Concurrence timers expire. - Issue a Scheduled Downtime Notification. - Issue an immediate disconnect for a subscription version where the Service Provider Under Test is the Donor Service Provider. - Issue a Cancel request for a pending Inter-Service Provider Subscription Version for which both service providers have concurred to the pending port, on behalf of the Service Provider Under Test, let the Cancellation Initial Concurrence Timer expire. - Issue a Create request for a range of two pending subscription versions that were initially created by the New Service Provider, on behalf of the Old Service Provider, where the Authorization Flag is set to “False” and provide a Cause Code. - Issue an activate request for an Inter-Service Provider Subscription Version on behalf of the Service Provider Under Test. - Issue an Activate request for a range of two Inter-Service Provider Subscription Versions where a broadcast to the LSMSs goes into a Partial Failure status. 2) While the SOA is in recovery, NPAC personnel should perform the following functions: - Create an NPA-NXX. - Activate a Subscription Version as the Service Provider Under Test. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | The service provider SOA should be ‘disassociated’ from the NPAC SMS while NPAC Personnel are performing the setup specified above |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | The Service Provider establishes an association from their SOA to the NPAC SMS with the resynchronization flag set to TRUE. |  | NPAC |  | The NPAC SMS receives the association bind request from the SOA. Once the association is established, the NPAC SMS queues all current updates. |  |  |  |
| 2. | SP | The SOA issues an M-ACTION Request lnpDownload (network data) to the NPAC SMS and specifies the time range for the resync request. |  | NPAC |  | The NPAC SMS receives the M-ACTION. 1) If the Service Provider’s SOA Linked Replies Indicator is set to FALSE, NPAC issues a single, normal M-ACTION Response lnpDownload messages back to the SOA with the Network Data. 2) If the Service Provider’s SOA Linked Replies Indicator is set to TRUE, NPAC issues a single, normal M-ACTION Response lnpDownload message back to the SOA with the Network Data updates. The data does not exceed the Network Data Blocking factor, so there shall be only 1 message sent in this instance. |  |  |  |
| 3. | NPAC | As soon as the M-ACTION Request is received, NPAC Personnel issue a create for an NPA-NXX. |  | NPAC |  | The NPAC SMS receives the M-CREATE Request serviceProvNPA-NXX. |  |  |  |
| 4. | NPAC | The NPAC SMS checks to see if the M-CREATE servProvNPA-NXX can be sent to the SOA in recovery. |  | NPAC |  | The NPAC SMS does NOT issue the M-CREATE servProvNPA-NXX to the SOA since the SOA is still in recovery. |  |  |  |
| 5. | NPAC | NPAC Personnel issue an SV activate request. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request. The NPAC SMS issues an M-SET Request to itself and sets the SV’s status to ‘sending.’ The NPAC SMS issues an M-SET Response to itself. |  |  |  |
| 6. | NPAC | The NPAC SMS checks to see if the M-ACTION subscriptionVersionActivate can be sent to the SOA in recovery. |  | NPAC |  | The NPAC SMS does NOT issue the M-ACTION subscriptionVersionActivate to the SOA since the SOA is still in recovery. |  |  |  |
| 7. | NPAC | The NPAC SMS checks to see if the M-EVENT-REPORT objectCreation can be sent to the SOA in recovery. |  | NPAC |  | The NPAC SMS does NOT issue the M-EVENT-REPORT objectCreation to the SOA since the SOA is still in recovery. |  |  |  |
| 8. | SP | The SOA Service Provider issues an M-ACTION Request lnpNotificationRecovery (notification data) to the NPAC SMS and specifies a time range. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the SOA. 1) If the Service Provider’s SOA Linked Replies Indicator is set to FALSE, NPAC issues a single, normal M-ACTION Response lnpDownload messages back to the SOA with the Notification updates. 2) If the Service Provider’s SOA Linked Replies Indicator is set to TRUE, NPAC issues a single, normal M-ACTION Response lnpDownload message back to the SOA with Notification updates. The data does not exceed the Notification Data Blocking factor, so there shall be only 1 message sent in this instance. |  |  |  |
| 9. | SP | The SOA Service Provider issues an M-ACTION Request lnpRecovery to the NPAC SMS to set the resynchronization flag to FALSE. |  | NPAC |  | - The NPAC SMS receives the M-ACTION Request from the SOA and sets the resynchronization flag to ‘off’. |  |  |  |
| 10. | NPAC | NPAC SMS issues the following messages to the SOA for the request made while the SOA was in recovery: - M-CREATE Request serviceProvNPA-NXX for the NPA-NXX that was created during recovery. - The NPAC SMS will issue, depending upon the new service provider’s TN Range Notification Indicator, a subscriptionVersionStatusAttributeValueChange or subscriptionVersionRangeStatusAttributeValueChange M-EVENT-REPORT notifications to the new service provider SOA of the status change using an M-EVENT-REPORT subscriptionVersionStatusAttributeValueChange |  | SP |  | The service provider’s SOA receives the requests from the NPAC SMS for the requests that occurred during recovery and issues the following responses: - M-CREATE Response serviceProvNPA-NXX for the NPA-NXX that was created during recovery, indicating the SOA successfully received/processed the request. - M-EVENT-REPORT Confirmation for the subscription version that NPAC personnel activated on behalf of the service provider during recovery, indicating the SOA successfully received the M-EVENT-REPORT. |  |  |  |
| 11. | SP | The SOA receives the M-ACTION Response from the NPAC SMS with the data updates since the association was re-established. |  |  |  |  |  |  |  |
| 12. | NPAC | NPAC Personnel verify the data was sent in the action response. |  | NPAC |  | Verify that the appropriate data was sent. |  |  |  |
| 13. | SP | Service Provider Personnel, using the SOA, perform a local query for the actions taken in this test case. |  | SP |  | Verify that the following updates were made: - 1 LRN create. - 1 LRN delete. - 1 NPA-NXX create. - 1 NPA-NXX delete. - 1 NPA-NXX-X create – if supported by the Service Provider SOA. - 1 NPA-NXX-X modify – if supported by the Service Provider SOA. - 1 NPA-NXX-X delete – if supported by the Service Provider SOA. - 1 First port of NPA-NXX notification. - 1 Scheduled Downtime notification. - 1 NPA-NXX create after recovery is complete - 1 Subscription Version activate after recovery is complete |  |  |  |
| E. | Pass/Fail Analysis, NANC 187-4 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 187-5 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | SOA – Service Provider Personnel submit a resynchronization request for Network Data and Notification Data by time range, over the SOA to NPAC SMS Interface, with the Service Provider’s SOA Linked Replies Indicator set to their production setting. The recovery response includes a number of Network Data objects and Notifications greater than the respective Linked Replies Blocking Factor and less than the respective Maximum Linked Recovered Notifications. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 187 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0 |  | Relevant Requirement(s): |  | RR6-85, RR6-86, RR6-84, RR6-92, RR6-89, RR6-94, RR6-91 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0 |  | Relevant Flow(s): |  | B.7.2 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Prerequisite data may be set up different depending on if this test case is being run during Individual testing versus Group Testing in order to meet test case objectives. Evaluate each service provider’s capabilities and tailor the prerequisite data to meet the test case objective. Consider which category the service provider under test fits into: - The service provider under test does not support linked replies or ranged notifications. - The service provider under test supports linked replies but does not support ranged notifications. - The service provider under test supports linked replies and ranged notifications. Set the Network Data Blocking Factor parameter to a low number (for example 5 – to create linked replies based on the network data in the prerequisites that follow). While the SOA is disconnected from the NPAC SMS, NPAC Personnel should perform the following functions for data within the time range to be resynced: a) Activate a Block on behalf of the Service Provider that is ‘down’ with SOA Origination TRUE. (NPB group a) b) Create a range of 10 Subscription Versions on behalf of the Old Service Provider and where the Service Provider Under Test is the New Service Provider; let the Initial Concurrence timer expire. When you create, do this in two ranges, where the last half of the TNs in the range is the first range that you create. In a second request, create the first half of the TNs in the range. (SV group b2 and SV group b1) c) Issue a Scheduled Downtime Notification. d) Issue an immediate disconnect for 20 subscription versions where the Service Provider Under Test is the Donor Service Provider. (SV group d) e) Issue a Cancel request for each subscription version in a range of 10 pending Inter-Service Provider Subscription Versions for which both service providers have concurred to the pending port, on behalf of the Service Provider Under Test, let each Cancellation Initial Concurrence Timer expire for each of the TNs that were cancelled. (SV group e) f) On behalf of the service provider under test, acting as the Old service provider, issue a Create request for a range of 20 pending subscription versions that were initially created by the New Service Provider, where the Authorization Flag is set to “False” and provide a Cause Code. ( SV group f) g) After the Initial Concurrence Timer has expired, but prior to the Final Concurrence Timer expiration, on behalf of the service provider under test, where they are the ‘New’ service provider, concur to the range created in (b) above. (SV group g ______________). h) Create 10 LRNs. (LRN group h) i) Create 15 NPA-NXXs. (NPA-NXX group i) |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | The Service Provider establishes an association from their SOA to the NPAC SMS with the resynchronization flag set to TRUE. |  | NPAC |  | The NPAC SMS receives the association bind request from the SOA. Once the association is established, the NPAC SMS queues all current updates. |  |  |  |
| 2. | SP | The SOA Service Provider issues an M-ACTION Request lnpNotificationRecovery (notification data) to the NPAC SMS and specifies a time range. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the SOA. 1) If the Service Provider’s SOA Linked Replies Indicator is set to FALSE, NPAC issues a single, normal M-ACTION Response lnpDownload message back to the SOA with the Notification updates. - Number Pool Block Status Attribute Value Change Notification or if the SOA supports ranges, Number Pool Block Range Status Attribute Value Change for (NPB group a) - Subscription Version New SP Create Request Notification or if the SOA supports ranges, Subscription Version Range New SP-Create Request for (SV group b) - Downtime Notification - Subscription Version Donor SP – Customer Disconnect Date or if the SOA supports ranges, Subscription Version Range Donor SP – Customer Disconnect Date for (SV group d) - Subscription Version Status Attribute Value Change Notification for (SV group e) - Subscription Version Status Attribute Value Change or if the SOA supports ranges, Subscription Version Range Status Attribute Value Change for (SV group f) - Subscription Version Status Attribute Value Change or if the SOA supports ranges, Subscription Version Range Status Attribute Value Change with a SVID list for (SV group g2 and SV group g1) 2) If the Service Provider’s SOA Linked Replies Indicator is set to TRUE, NPAC issues multiple, linked M-ACTION replies, lnpDownload, followed by a non-linked, empty, normal response (indicating the end of the linked reply data) back to the SOA with Notification updates. The data does exceeds the Notification Data Blocking factor, so there shall be at least (2) messages sent in this instance. |  |  |  |
| 3. conditional | SP | The SOA issues an M-ACTION Request lnpDownload (network data) to the NPAC SMS and specifies a time range. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the SOA: 1) If the Service Provider’s SOA Linked Replies Indicator is set to FALSE, NPAC issues single, normal M-ACTION Response lnpDownload message back to the SOA with the network data updates for - LRN group h - NPA-NXX group i 2) If the Service Provider’s SOA Linked Replies Indicator is set to TRUE, NPAC issues multiple, linked M-ACTION replies, lnpDownload followed by a non-linked, empty, normal response (indicating the end of the linked reply data) back to the SOA with the network data updates. These messages shall be linked for groups of (5) objects (based on the special Network Data Blocking Factor setting for this test case) – there should be 5 linked replies. |  |  |  |
| 4. | SP | The SOA Service Provider issues an M-ACTION Request lnpRecovery to the NPAC SMS to set the resynchronization flag to FALSE. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the SOA and sets the resynchronization flag to ‘off’. |  |  |  |
| 5. |  | There weren’t any actions taken while the Service Provider was in recovery so there aren’t any subsequent actions to send/receive/or verify. |  |  |  |  |  |  |  |
| 6. | SP | Service Provider Personnel, using the SOA, perform a local query for the actions taken in this test case. |  | SP |  | Verify that the notifications were received: - Number Pool Block Status Attribute Value Change Notification or if the SOA supports ranges, Number Pool Block Range Status Attribute Value Change for (NPB group a) - Subscription Version New SP Create Request Notification or if the SOA supports ranges, Subscription Version Range New SP Create Request for (SV group b) - Downtime Notification - Subscription Version Donor SP – Customer Disconnect Date or if the SOA supports ranges, Subscription Version Range Donor SP – Customer Disconnect Date for (SV group d) - Subscription Version Status Attribute Value Change Notification for (SV group e) - Subscription Version Status Attribute Value Change or if the SOA supports ranges, Subscription Version Range Status Attribute Value Change for (SV group f) - Subscription Version Status Attribute Value Change or if the SOA supports ranges, Subscription Version Range Status Attribute Value Change with a SVID list for (SV group g2 and SV group g1) |  |  |  |
| E. | Pass/Fail Analysis, NANC 187-5 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 187-6 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | SOA – Service Provider Personnel submit a resynchronization request for Network Data and Notification Data by time range, over the SOA to NPAC SMS Interface, with the Service Provider’s SOA Linked Replies Indicator set to their production setting. The recovery response includes a number of Network Data objects greater than the Network Data Maximum Linked Recovered Objects and Notifications greater than the Notification Data Maximum Linked Recovered Notifications and Maximum Number of Download Records. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 187 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0 |  | Relevant Requirement(s): |  | RR6-94, RR6-91 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0 |  | Relevant Flow(s): |  | B.7.2 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1) Decide what the Notification Data Maximum Linked Recovered Notifications setting should be for this test case and then substitute the ‘X number references’ following in order to exceed this parameter setting. 2) Decide what the Network Data Maximum Linked Recovered Objects setting should be for this test case and then substitute the ‘X number references’ following in order to exceed this parameter setting. 3) While the SOA is disconnected from the NPAC SMS, NPAC Personnel should perform the following functions for data within the time range to be resynced: a) Create X number Subscription Versions on behalf of the Old Service Provider and where the Service Provider Under Test is the New Service Provider; let the Initial Concurrence timer expire. (SV group a) b) Modify X number of Subscription Versions on behalf of the Old Service Provider, the Service Provider under test, setting authorization to false with a valid cause code (SV group b) c) Issue an immediate disconnect for X number subscription versions where the Service Provider Under Test is the Donor Service Provider. (SV group c). d) Activate a Number Pool Block on behalf of the Service Provider under test. (NPB group d) e) Issue a Cancel request for a range of X number pending Inter-Service Provider Subscription Version for which both service providers have concurred to the pending port, on behalf of the Service Provider Under Test, let the Cancellation Initial Concurrence Timer expire. (SV group e) f) Issue a Create request for a range of 20 pending subscription versions that were initially created by the New Service Provider, on behalf of the Old Service Provider, where the Authorization Flag is set to “False” and provide a Cause Code. ( SV group f) g) Create X number of LRNs on behalf of the service provider under test. (LRN group g) h) Create X number of NPA-NXXs on behalf of the service provider under test. (NPA-NXX group h). |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | The service provider SOA should be ‘disassociated’ from the NPAC SMS while NPAC Personnel are performing the setup specified above |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | The Service Provider establishes an association from their SOA to the NPAC SMS with the resynchronization flag set to TRUE. |  | NPAC |  | The NPAC SMS receives the association bind request from the SOA. Once the association is established, the NPAC SMS queues all current updates. |  |  |  |
| 2. | SP | The SOA issues an M-ACTION Request lnpDownload (notification data) to the NPAC SMS and specifies the time range for the resync request. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the SOA and determines that for the time range requested, the Notification Data exceeds the Notification Data Maximum Recovered Notifications parameter (if the service provider supports linked replies) or the Maximum Number of Download Records (if the service provider does not support linked replies). The NPAC SMS issues an M-ACTION Response lnpDownload, Criteria-too-large to the requesting LSMS. |  |  |  |
| 3. | SP | After receiving the M-ACTION Response ‘Criteria-to-large’ the SOA issues an M-ACTION Request lnpDownload (notification data) to the NPAC SMS and specifies a smaller time range (than that indicated in row 2 above) for the resync request. Note: Row 2 may occur over and over until the M-ACITON Request indicates a time range that yields data less than the ‘Notification Data Maximum Linked Recovered Notifications’ for service provider’s that support Linked Replies – or less than the Maximum Number of Download Records for those service provider’s that don’t support Linked Replies. |  | NPAC |  | Once the NPAC SMS receives an M-ACTION request that specifies time range that yields an amount of data less than the ‘Notification Data Maximum Linked Recovered Notifications for service provider’s that support Linked Replies, or less than the ‘Maximum Number of Download Records’ for service provider’s that don’t support Linked Replies: 1) If the Service Provider’s SOA Linked Replies Indicator is set to FALSE, NPAC issues a single M-ACTION Response lnpDownload message back to the SOA with the Notification Data updates for - Subscription Version New SP Create Request Notification or if the SOA supports ranges, Subscription Version Range New SP Create Request for (SV group a) - Subscription Version Status Attribute Value Change or if the SOA supports ranges, Subscription Version Range Status Attribute Value Change for (SV group b) - Subscription Version Status Attribute Value Change or if the SOA supports ranges, Subscription Version Range Status Attribute Value Change for (SV group c) - If SOA Origination is marked as TRUE, Number Pool Block Status Attribute Value Change or if SOA supports ranges, Number Pool Block Range Status Attribute Value Change for (NPB group d) - Subscription Version Status Attribute Value Change or if the SOA supports ranges, Subscription Version Range Status Attribute Value Change for (SV group e) - Subscription Version Status Attribute Value Change or if the SOA supports ranges, Subscription Version Range Status Attribute Value Change for (SV group f) 2) If the Service Provider’s SOA Linked Replies Indicator is set to TRUE, NPAC issues multiple, linked M-ACTION replies, lnpDownload, followed by a non-linked, empty, normal response (indicating the end of the linked reply data) back to the LSMS with the Notification Data updates. These messages shall be linked for groups of (X) objects – there should be ( X )linked replies. |  |  |  |
| 4. | SP | The SOA issues an M-ACTION Request lnpDownload (network data) to the NPAC SMS and specifies the time range for the resync request. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the SOA and determines that for the time range requested, the Network Data exceeds the Network Data Maximum Recovered Objects parameter (if the service provider supports linked replies) or the Maximum Number of Download Records (if the service provider does not support linked replies). The NPAC SMS issues an M-ACTION Response lnpDownload, Criteria-too-large to the requesting LSMS. |  |  |  |
| 5. | SP | After receiving the M-ACTION Response ‘Criteria-to-large’ the SOA issues an M-ACTION Request lnpDownload (network data) to the NPAC SMS and specifies a smaller time range (than that indicated in row 4 above) for the resync request. Note: Row 4 may occur over and over until the M-ACITON Request indicates a time range that yields data less than the ‘Network Data Maximum Linked Recovered Objects’ for service provider’s that support Linked Replies – or less than the Maximum Number of Download Records for those service provider’s that don’t support Linked Replies. |  | NPAC |  | Once the NPAC SMS receives an M-ACTION request that specifies time range that yields an amount of data less than the ‘Network Data Maximum Linked Recovered Objects’ for service provider’s that support Linked Replies, or less than the ‘Maximum Number of Download Records’ for service provider’s that don’t support Linked Replies: 1) If the Service Provider’s SOA Linked Replies Indicator is set to FALSE, NPAC issues a single M-ACTION Response lnpDownload message back to the SOA with the Network Data updates for - LRN group g - NPA-NXX group h 2) If the Service Provider’s SOA Linked Replies Indicator is set to TRUE, NPAC issues multiple, linked M-ACTION replies, lnpDownload, followed by a non-linked, empty, normal response (indicating the end of the linked reply data) back to the LSMS with the Network Data updates. These messages shall be linked for groups of (X) objects – there should be ( X )linked replies. |  |  |  |
| 6. | SP | The SOA Service Provider issues an M-ACTION Request lnpRecoveryComplete to the NPAC SMS to set the resynchronization flag to FALSE. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the SOA and replies back to the SOA with data updates at the next scheduled interval for the NPA-NXX that was created during resynchronization and the Subscription Version that was activated during resynchronization. |  |  |  |
| 7. | SP | The SOA receives the M-ACTION Response from the NPAC SMS. |  |  |  |  |  |  |  |
| 8.. | NPAC | NPAC Personnel verify that no data was sent in the initial action response for notification data. |  | NPAC |  | Verify that no data was sent in the initial action response for notification data. Note to Test Engineers: NPAC Personnel may watch the router of the service provider under test to verify the 1st request for notification data resulted in a ‘criteria-too-large’ response. |  |  |  |
| E. | Pass/Fail Analysis, NANC 187-6 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

# NANC 191 DPC/SSN Value Edits and NANC 291 SSN Edits in the NPAC SMS

NOTE:  Identify combinations of valid/invalid DPC/SSN values for each Service Provider under test for a minimum of CNAM, CLASS, ISVM and LIDB even if the Service Provider under test does not support all of these enhanced features in production.  If the Service Provider supports WSMSC data – this must also be included in the NANC 191/291 test case test data suite.

Service Provider’s whose systems cannot create the ‘failure’ scenarios that follow pass those test cases be default.  If their system does not ‘stop’ the invalid message before it goes across the interface, then their system must be able to successfully execute the test case and handle the failure response from the NPAC SMS.

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 191/291-1 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | SOA – Service Provider Personnel attempt to create a Subscription Version specifying some valid and some invalid DPC/SSN information. The regional SSN Edit Flags (CLASS, LIDB, CNAM, ISVM and WSMSC) are set to production values. - Failure |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 191/NANC 291 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0a |  | Relevant Requirement(s): |  | RR3-380, RR3-381, RR3-382, RR3-383, RR3-384, RR3-385, RR3-386, RR3-387, RR3-388, RR3-389, RR3-375, RR3-376, RR3-377, RR3-378, RR3-378 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0a |  | Relevant Flow(s): |  | B.5.1.1 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Verify the NPA-NXX exists and is open for porting for the TN that is going to be used during this test case. 2. Verify that the LRN exists for the Service Provider under test. 3. Verify that the ‘SSN Edit Flag’ indicators are set to production settings for the regions in which the Service Provider under test operates. If the Service Provider operates in regions where the ‘SSN Edit Flag’ indicators may be set to different settings, be sure to test all scenarios with the Service Provider. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | For Row 1 of the test steps that follow use some combination of the following ‘invalid’ DPC/SSN data entry scenarios to create a Subscription Version request with invalid DPC/SSN data: If the ‘SSN Edit Flags’ are set to TRUE, invalid data would include - Specifying DPC values of (network 001-255, cluster 000-255, member 000-255) and corresponding SSN values other than (000). - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and corresponding SSN values of (000). - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and corresponding SSN values other than (000). If the ‘SSN Edit Flags’ are set to FALSE, invalid data would include - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) when the SSN value is between 000-255. - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and also not specifying a value for the corresponding SSN value. - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and also not specifying a valid SSN value is between 000-255. |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | 1. Using their SOA system, Service Provider Personnel submit a Subscription Version Create request for a single TN. 2. The SOA system sends an M-ACTION Request subscriptionVersionNewSP-Create to the NPAC SMS to create the subscriptionVersionNPAC (Subscription Version) on the NPAC SMS. The following attributes must be specified: - subscriptionTN or a valid subscriptionVersionTN-Range - subscriptionNewCurrentSP - subscriptionOldSP - subscriptionNewSP-DueDate (seconds set to zero) - subscriptionLNPType - subscriptionLRN Specify a combination of valid and invalid DPC/SSN data for the following attributes. - subscriptionCLASS-DPC - subscriptionCLASS-SSN - subscriptionLIDB-DPC - subscriptionLIDB-SSN - subscriptionCNAM-DPC - subscriptionCNAM-SSN - subscriptionISVM-DPC - subscriptionISVM-SSN - subscriptionWSMSC-DPC - if supported by the Service provider SOA - subscriptionWSMSC-SSN - if supported by the Service Provider SOA The following attributes are optional: - subscriptionEndUser LocationValue - subscriptionEndUser LocationType - subscriptionBillingID |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the Request from the Service Provider’s SOA and determines the following: The request contains invalid DPC/SSN data based on system requirements and the regional ‘SSN Edit Flag’ settings. (This violates system requirements.) |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-ACTION Response failure indicating an error with the request to the SOA. |  | SP |  | The Service Provider SOA receives the M-ACTION Response. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a query for the Subscription Version. |  | NPAC |  | NPAC Personnel verify that the Subscription Version does not exist on the NPAC SMS. |  |  |  |
| 4. | SP | Service Provider Personnel, perform a local query for the Subscription Version. |  | SP |  | Verify that the Subscription Version does not exist on the local database. |  |  |  |
| E. | Pass/Fail Analysis, NANC 191/291 - 1 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written for each ‘SSN Edit Flag’ indicator setting for which the Service Provider under test may operate in production. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written for each ‘SSN Edit Flag’ indicator setting where they may operate in production. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 191/291-2 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | SOA – Service Provider Personnel attempt to modify a ‘Pending’ Subscription Version specifying some valid and some invalid DPC/SSN information. The regional SSN Edit Flags (CLASS, LIDB, CNAM, ISVM and WSMSC) are set to production values. - Failure |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 191/NANC 291 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0a |  | Relevant Requirement(s): |  | RR3-380, RR3-381, RR3-382, RR3-383, RR3-384, RR3-385, RR3-386, RR3-387, RR3-388, RR3-389, RR3-405, RR3-406, RR3-407, RR3-408, RR3-409, RR3-375, RR3-376, RR3-377, RR3-378, RR3-378 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0a |  | Relevant Flow(s): |  | B.5.2.3 or B.5.2.4 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Verify that the ‘SSN Edit Flag’ indicators are set to production settings for the regions in which the Service Provider under test operates. If the Service Provider operates in regions where the ‘SSN Edit Flag’ indicators may be set to different settings, be sure to test all scenarios with the Service Provider. 2. Verify that a ‘pending’ subscription version exists for the TN that is going to be used during this test case. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | For Row 1 of the test steps that follow use some combination of the following ‘invalid’ DPC/SSN data entry scenarios to modify a Subscription Version request with invalid DPC/SSN data: If the ‘SSN Edit Flags’ are set to TRUE, invalid data would include - Specifying DPC values of (network 001-255, cluster 000-255, member 000-255) and corresponding SSN values other than (000). - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and corresponding SSN values of (000). - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and corresponding SSN values other than (000). If the ‘SSN Edit Flags’ are set to FALSE, invalid data would include - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) when the SSN value is between 000-255. - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and also not specifying a value for the corresponding SSN value. - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and also not specifying a valid SSN value is between 000-255. |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | Using their SOA system, Service Provider Personnel submit a request to the NPAC SMS to modify a single TN, ‘Pending’ Subscription Version that already exists on the NPAC SMS. The request must specify the TN and the version status or the version ID of the Subscription Version to be modified and the data to be modified. The following attributes must be specified: Specify a combination of valid and invalid DPC/SSN data for the following attributes. - subscriptionCLASS-DPC - subscriptionCLASS-SSN - subscriptionLIDB-DPC - subscriptionLIDB-SSN - subscriptionCNAM-DPC - subscriptionCNAM-SSN - subscriptionISVM-DPC - subscriptionISVM-SSN - subscriptionWSMSC-DPC - if supported by the Service provider SOA - subscriptionWSMSC-SSN - if supported by the Service Provider SOA The Service Provider SOA submits an M-ACTION Request subscriptionVersionModify or an M-SET Request subscriptionVersionNPAC (depending on the system implementation) to the NPAC SMS lnpSubscription object to update the ‘Pending’ Subscription Version. |  | NPAC |  | The NPAC SMS receives the M-ACTION/M-SET Request from the Service Provider’s SOA and determines the following: The request contains invalid DPC/SSN data based on system requirements and the regional ‘SSN Edit Flag’ settings. (This violates system requirements.) |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-ACTION Response failure or M-SET Response failure (depending on the message received in Row 1) indicating an error with the request to the SOA. |  | SP |  | The Service Provider SOA receives the Response from the NPAC SMS. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a query for the Subscription Version. |  | NPAC |  | NPAC Personnel verify that the Subscription Version exists with a status of ‘Pending’ however, the attributes were not modified. |  |  |  |
| 4. | SP | Service Provider Personnel, perform a local query for the Subscription Version. |  | SP |  | Verify that the Subscription Version exists on the local database with the original attribute values. |  |  |  |
| E. | Pass/Fail Analysis, NANC 191/291 - 2 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written for each ‘SSN Edit Flag’ indicator setting for which the Service Provider under test may operate in production. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written for each ‘SSN Edit Flag’ indicator setting where they may operate in production. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 191/291-3 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | SOA – Service Provider Personnel attempt to activate a ‘Pending’ Subscription Version that contains some valid and some invalid DPC/SSN information. The regional SSN Edit Flags (CLASS, LIDB, CNAM, ISVM and WSMSC) are set to production values. - Failure |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 191/NANC 291 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0a |  | Relevant Requirement(s): |  | RR3-427 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0a |  | Relevant Flow(s): |  | B.5.1.5 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Verify that the ‘SSN Edit Flag’ indicators are set to production settings for the regions in which the Service Provider under test operates. If the Service Provider operates in regions where the ‘SSN Edit Flag’ indicators may be set to different settings, be sure to test all scenarios with the Service Provider. 2. Verify that a ‘Pending’ Subscription Version with some combination of the valid and invalid DPC/SSN data scenarios following exists on the NPAC and local databases: If the ‘SSN Edit Flags’ are set to TRUE, invalid data would include - Specifying DPC values of (network 001-255, cluster 000-255, member 000-255) and corresponding SSN values other than (000). - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and corresponding SSN values of (000). - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and corresponding SSN values other than (000). If the ‘SSN Edit Flags’ are set to FALSE, invalid data would include - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) when the SSN value is between 000-255. - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and also not specifying a value for the corresponding SSN value. - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and also not specifying a valid SSN value is between 000-255. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | Using their SOA system, Service Provider Personnel submit a request to the NPAC SMS to activate a single TN, ‘Pending’ Subscription Version that already exists on the NPAC SMS with valid and invalid DPC/SSN information as described in the prerequisites. The request must specify the subscription version ID, subscription version TN, or a range of subscription version TNs to be activated. The Service Provider SOA submits an M-ACTION Request subscriptionVersionActivate to the NPAC SMS lnpSubscription object to activate the ‘Pending’ Subscription Version. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the Service Provider’s SOA and determines the following: The request to activate is for a ‘Pending’ Subscription Version that contains invalid DPC/SSN data based on system requirements and the regional ‘SSN Edit Flag’ settings. (This violates system requirements.) |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-ACTION Response failure indicating an error with the request to the SOA. |  | SP |  | The Service Provider SOA receives the Response from the NPAC SMS. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a query for the Subscription Version. |  | NPAC |  | NPAC Personnel verify that the Subscription Version exists with a status of ‘Pending’ and invalid DPC/SSN data. |  |  |  |
| 4. | SP | Service Provider Personnel, perform a local query for the Subscription Version. |  | SP |  | Verify that the Subscription Version exists on the local database with the invalid DPC/SSN data. |  |  |  |
| E. | Pass/Fail Analysis, NANC 191/291 - 3 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written for each ‘SSN Edit Flag’ indicator setting for which the Service Provider under test may operate in production. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written for each ‘SSN Edit Flag’ indicator setting where they may operate in production. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 191/291-4 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | SOA – Service Provider Personnel attempt to modify an ‘Active’ Subscription Version that contains some valid and some invalid DPC/SSN information. The regional SSN Edit Flags (CLASS, LIDB, CNAM, ISVM and WSMSC) are set to production values. - Failure |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 191/NANC 291 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0a |  | Relevant Requirement(s): |  | RR3-380, RR3-381, RR3-382, RR3-383, RR3-384, RR3-385, RR3-386, RR3-387, RR3-388, RR3-389, RR3-405, RR3-406, RR3-407, RR3-408, RR3-409, RR3-375, RR3-376, RR3-377, RR3-378, RR3-378 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0a |  | Relevant Flow(s): |  | B.5.2.1 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Verify that the ‘SSN Edit Flag’ indicators are set to production settings for the regions in which the Service Provider under test operates. If the Service Provider operates in regions where the ‘SSN Edit Flag’ indicators may be set to different settings, be sure to test all scenarios with the Service Provider. 2. Verify that an ‘Active’ Subscription Version with some combination of the valid and invalid DPC/SSN data scenarios following exists on the NPAC and local databases: If the ‘SSN Edit Flags’ are set to TRUE, invalid data would include - Specifying DPC values of (network 001-255, cluster 000-255, member 000-255) and corresponding SSN values other than (000). - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and corresponding SSN values of (000). - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and corresponding SSN values other than (000). If the ‘SSN Edit Flags’ are set to FALSE, invalid data would include - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) when the SSN value is between 000-255. - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and also not specifying a value for the corresponding SSN value. - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and also not specifying a valid SSN value is between 000-255. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | Using their SOA system, Service Provider Personnel submit a request to the NPAC SMS to modify a single TN, ‘Active’ Subscription Version that already exists on the NPAC SMS with valid and invalid DPC/SSN information as described in the prerequisites. The request must specify the TN, TN range, and the version status, or the version ID of the subscription version to be modified; and the data to be modified. Modify any of the following attributes: - subscriptionLRN - subscriptionEndUserLocationValue - subscriptionEndUserLocationType - subscriptionBillingId The Service Provider SOA submits an M-ACTION Request subscriptionVersionModify to the NPAC SMS lnpSubscription object to modify the ‘Active’ Subscription Version. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the Service Provider’s SOA and determines the following: The request to modify the ‘Active’ Subscription Version contains invalid DPC/SSN data based on system requirements and the regional ‘SSN Edit Flag’ settings. (This violates system requirements.) |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-ACTION Response failure indicating an error with the request to the SOA. |  | SP |  | The Service Provider SOA receives the Response from the NPAC SMS. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a query for the Subscription Version. |  | NPAC |  | NPAC Personnel verify that the Subscription Version exists with a status of ‘Active’ and invalid DPC/SSN data. |  |  |  |
| 4. | SP | Service Provider Personnel, perform a local query for the Subscription Version. |  | SP |  | Verify that the Subscription Version exists on the local database with the invalid DPC/SSN data. |  |  |  |
| E. | Pass/Fail Analysis, NANC 191/291 - 4 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written for each ‘SSN Edit Flag’ indicator setting for which the Service Provider under test may operate in production. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written for each ‘SSN Edit Flag’ indicator setting where they may operate in production. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 191/291-5 |  | SUT Priority: |  | SOA |  | N/A |
|  |  |  |  |  |  |  | LSMS |  | Required |
|  | Objective: |  | NPAC OP GUI – NPAC Personnel attempt to submit a mass update request for a range of Subscription Versions that currently exist. Some of these Subscription Versions have valid DPC/SSN data and some of these Subscription Versions have invalid DPC/SSN data. The Mass Update request specifies new DPC/SSN values that will correct some but not all of the Subscription Versions that currently exist with invalid DPC/SSN attributes. The NPAC SMS processes the Mass Update request, modifies some but not all of the DPC/SSN attributes for the range specified in the Mass Update Request and logs the objects that could not be updated to the Mass Update Exception Report. The regional SSN Edit Flags (CLASS, LIDB, CNAM, ISVM and WSMSC) are set to production values. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 191/NANC 291 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0a |  | Relevant Requirement(s): |  | RR3-429 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0a |  | Relevant Flow(s): |  | B.8.3 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Verify that the ‘SSN Edit Flag’ indicators are set to production settings for the regions in which the Service Provider under test operates. If the Service Provider operates in regions where the ‘SSN Edit Flag’ indicators may be set to different settings, be sure to test all scenarios with the Service Provider. 2. Verify that a range of ‘Active’ Subscription Versions exists. - One subset range of Subscription Versions should have all valid DPC/SSN. (SV group 2a) - One subset range of Subscription Versions should exist with invalid CNAM DPC/SSN data (all other DPC/SSN data should be valid. (SV group 2b) - One subset range of Subscription Versions should exist with all invalid DPC/SSN data – at a minimum should be CNAM plus at least one other feature set. (SV group 2c) 3. Identify the appropriate TN range to use in this test case ________________________. 4. Identify DPC/SSN attributes that should be specified in the Mass Update request such that some of the Subscription Versions that currently have invalid DPC/SSN attributes will be corrected, and some will remain invalid. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel submit a Mass Update request for a range of ‘Active’ Subscription Versions that exist, some with valid DPC/SSN data and some with invalid DPC/SSN (identified in the prerequisites above). Modify CNAM DPC/SSN data, specifying valid values for the following attributes. - subscriptionCNAM-DPC - subscriptionCNAM-SSN |  | NPAC |  | The NPAC SMS searches the Subscription Version database for Subscription Versions that match the input Mass Update criteria. The NPAC SMS determines that a subset of the TNs in the Subscription Version range exist with invalid DPC/SSN data that is not corrected by the new values specified in the Mass Update request. The NPAC SMS makes an entry to the Mass Update Exception report for these TNs, and continues updating the remaining Subscription Versions that meet the Mass Update criteria. request is valid. |  |  |  |
| 2. | NPAC | NPAC SMS sends multiple M-SET(s) for each contiguous range of Subscription Versions that met the Mass Update criteria to all LSMSs that are accepting downloads for the NPA-NXX of the Subscription Versions to update the valid DPC/SSN values. |  | SP |  | All LSMSs that are accepting downloads for the NPA-NXXs of the Subscription Versions being updated, receive the M-SET request(s) from the NPAC SMS to modify the DPC/SSN values. The LSMSs issue an M-SET Response(s) indicating they successfully processed the NPAC SMS request(s). These M-SET Requests are for the subscription versions that were successfully updated based on the Mass Update criteria – and does not include those subscription versions that could not be updated based on the Mass Update criteria. The subscription versions that could not be updated are included on the Mass Update Exception report. |  |  |  |
| 3. | NPAC | 1. If the current Service Provider’s TN Range Notification Indicator is set to TRUE, NPAC SMS issues an M-EVENT-REPORT subscriptionVersionRangeStatusAttributeValueChange for the range of Subscription Versions that were updated indicating the status is now ‘Active’. 2. If the current Service Provider’s TN Range Notification Indicator is set to ‘FALSE’, NPAC SMS issues a subscriptionVersionStatusAttributeValueChange for each Subscription Version that was updated, indicating the status is now ‘Active’. |  | SP |  | The current Service Provider receives the M-EVENT-REPORT from the NPAC SMS and issues an M-EVENT-REPORT response indicating it successfully received the message. |  |  |  |
| 4. | NPAC | NPAC Personnel generate a Mass Update Exception report. |  | NPAC |  | Verify that the subset of Subscription Versions (SV group 2c) within the Mass Update criteria that’s invalid DPC/SSN data was not corrected by the new specified attributes are included on the report. |  |  |  |
| 5. | SP | Service Provider Personnel perform a local query on their LSMS to verify the Mass Update was completed. |  | SP |  | On the LSMS verify: 1. The subset of Subscription Versions (SV group 2c) within the Mass Update request who’s invalid DPC/SSN data was not corrected by the new specified DPC/SSN attributes were not updated with the new DPC/SSN values. 2. The subsets of Subscription Versions (SV groups 2a and 2b) within the Mass Update request who’s DPC/SSN values were previously valid, or were invalid but the Mass Update specified attributes corrected the previous issues were updated with the new DPC/SSN values. |  |  |  |
| 6. | NPAC | NPAC Personnel perform a full audit for the subscription version range specified in the Mass Update request. |  | NPAC |  | Verify that there are no discrepancies found. The subscription versions (SV group 2c) that previously existed with invalid DPC/SSN data that were not corrected by the Mass Update specified attributes – still exist in their previous state. All other subscription versions (SV groups 2a and 2b) specified in the Mass Update criteria were updated appropriately. |  |  |  |
| E. | Pass/Fail Analysis, NANC 191/291 - 5 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 191/291-6 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | SOA – Service Provider Personnel attempt to create a Number Pool Block specifying some valid and some invalid DPC/SSN information. The regional SSN Edit Flags (CLASS, LIDB, CNAM, ISVM and WSMSC) are set to production values. - Failure |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 191/NANC 291 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0a |  | Relevant Requirement(s): |  | RR3-390, RR3-391, RR3-392, RR3-393, RR3-394, RR3-395, RR3-396, RR3-397, RR3-398, RR3-399, RR3-400, RR3-401, RR3-402, RR3-403, RR3-404, RR3-375, RR3-376, RR3-377, RR3-378, RR3-378 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0a |  | Relevant Flow(s): |  | B.4.4.2 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Verify that the ‘SSN Edit Flag’ indicators are set to production settings for the regions in which the Service Provider under test operates. If the Service Provider operates in regions where the ‘SSN Edit Flag’ indicators may be set to different settings, be sure to test all scenarios with the Service Provider. 2. Verify that the NPA-NXX exists and is open for porting for the Number Pool Block that is going to be used during this test case. 3. Verify that the NPA-NXX-X exists respective to the Number Pool Block that is going to be used during this test case. 4. Verify that there are no contaminated TNs or ‘pending-like’ Subscription Versions for the range of TNs in the NPA-NXX-X. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | 1. Verify that the NPA-NXX-X exists for the Number Pool Block that Service Provider Personnel will create during this Test Case. 2. Verify that the current date is equal to or greater than the NPA-NXX-X Effective Date. 3. For Row 1 of the test steps that follow use some combination of the following ‘invalid’ DPC/SSN data entry scenarios to create a Subscription Version request with invalid DPC/SSN data: If the ‘SSN Edit Flags’ are set to TRUE, invalid data would include - Specifying DPC values of (network 001-255, cluster 000-255, member 000-255) and corresponding SSN values other than (000). - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and corresponding SSN values of (000). - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and corresponding SSN values other than (000). If the ‘SSN Edit Flags’ are set to FALSE, invalid data would include - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) when the SSN value is between 000-255. - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and also not specifying a value for the corresponding SSN value. - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and also not specifying a valid SSN value is between 000-255. |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | Using the SOA, Service Provider Personnel, submit a M-ACTION numberPoolBlock-Create Request to the NPAC SMS to create a Number Pool Block. The request must include the following attributes: - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN Specify a combination of valid and invalid DPC/SSN data for the following attributes: - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | NPAC |  | The NPAC SMS receives the M-ACTION numberPoolBlock-Create Request from the Service Provider’s SOA and determines the following: The request contains invalid DPC/SSN data based on system requirements and the regional ‘SSN Edit Flag’ settings. (This violates system requirements.) |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-ACTION Response failure indicating an error with the request to the SOA. |  | SP |  | The Service Provider SOA receives the M-ACTION Response. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a query for the Number Pool Block and respective ‘Pooled’ Subscription Versions Service Provider personnel attempted to schedule during this test case. |  | NPAC |  | NPAC Personnel verify that the Number Pool Block and respective ‘Pooled’ Subscription Versions do not exist on the NPAC SMS. |  |  |  |
| 4. | SP | Service Provider Personnel, perform a local query for the Number Pool Block and the respective ‘Pooled’ Subscription Versions they attempted to schedule during this test case. |  | SP |  | Verify that the Number Pool Block and the respective ‘Pooled’ Subscription Versions do not exist on the local database. |  |  |  |
| E. | Pass/Fail Analysis, NANC 191/291 - 6 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written for each ‘SSN Edit Flag’ indicator setting for which the Service Provider under test may operate in production. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written for each ‘SSN Edit Flag’ indicator setting where they may operate in production. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 191/291-7 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | SOA – Service Provider Personnel attempt to modify a Number Pool Block specifying some valid and some invalid DPC/SSN information. The regional SSN Edit Flags (CLASS, LIDB, CNAM, ISVM and WSMSC) are set to production values. - Failure |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 191/NANC 291 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0a |  | Relevant Requirement(s): |  | RR3-390, RR3-391, RR3-392, RR3-393, RR3-394, RR3-395, RR3-396, RR3-397, RR3-398, RR3-399, RR3-400, RR3-401, RR3-402, RR3-403, RR3-404, RR3-405, RR3-406, RR3-407, RR3-408, RR3-409, RR3-421, RR3-422, RR3-423, RR3-424, RR3-425, RR3-426, RR3-375, RR3-376, RR3-377, RR3-378, RR3-378 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0a |  | Relevant Flow(s): |  | B.4.4.13 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Verify that the ‘SSN Edit Flag’ indicators are set to production settings for the regions in which the Service Provider under test operates. If the Service Provider operates in regions where the ‘SSN Edit Flag’ indicators may be set to different settings, be sure to test all scenarios with the Service Provider. 2. Verify the Number Pool Block to be modified exists on the NPAC SMS with a status of ‘active’ and an empty Failed SP List. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | For Row 1 of the test steps that follow use some combination of the following ‘invalid’ DPC/SSN data entry scenarios to modify a Number Pool Block with invalid DPC/SSN data: If the ‘SSN Edit Flags’ are set to TRUE, invalid data would include - Specifying DPC values of (network 001-255, cluster 000-255, member 000-255) and corresponding SSN values other than (000). - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and corresponding SSN values of (000). - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and corresponding SSN values other than (000). If the ‘SSN Edit Flags’ are set to FALSE, invalid data would include - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) when the SSN value is between 000-255. - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and also not specifying a value for the corresponding SSN value. - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and also not specifying a valid SSN value is between 000-255. |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | Using the SOA, Service Provider Personnel submit an M-SET Request numberPoolBlock to modify a Number Pool Block. The following attributes may be modified: - numberPoolBlockLRN Specify a combination of valid and invalid DPC/SSN data for the following attributes: - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | NPAC |  | The NPAC SMS receives the M-SET Request numberPoolBlock from the Service Provider’s SOA and determines the following: The request contains invalid DPC/SSN data based on system requirements and the regional ‘SSN Edit Flag’ settings. (This violates system requirements.) |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-SET Response failure indicating an error with the request to the SOA. |  | SP |  | The Service Provider SOA receives the M-SET Response. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a query for the Number Pool Block and respective ‘Pooled’ Subscription Versions Service Provider personnel attempted to modify during this test case. |  | NPAC |  | NPAC Personnel verify that the Number Pool Block and respective ‘Pooled’ Subscription Versions were not modified on the NPAC SMS. |  |  |  |
| 4. | SP | Service Provider Personnel, perform a local query for the Number Pool Block and the respective ‘Pooled’ Subscription Versions they attempted to modify during this test case. |  | SP |  | Verify that the Number Pool Block and the respective ‘Pooled’ Subscription Versions were not modified on the local database. |  |  |  |
| E. | Pass/Fail Analysis, NANC 191/291 - 7 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written for each ‘SSN Edit Flag’ indicator setting for which the Service Provider under test may operate in production. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written for each ‘SSN Edit Flag’ indicator setting where they may operate in production. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 191/291-8 |  | SUT Priority: NPAC One Time Only |  | SOA |  | N/A |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | NPAC – Upon Number Pool Block scheduled activation, NPAC SMS fails the Number Pool Block activation based on some invalid DPC/SSN information. The regional SSN Edit Flags (CLASS, LIDB, CNAM, ISVM and WSMSC) are set to production values. - Failure |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 191/NANC 291 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0a |  | Relevant Requirement(s): |  | RR3-390, RR3-391, RR3-392, RR3-393, RR3-394, RR3-395, RR3-396, RR3-397, RR3-398, RR3-399, RR3-400, RR3-401, RR3-402, RR3-403, RR3-404, RR3-428, RR3-375, RR3-376, RR3-377, RR3-378, RR3-378 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0a |  | Relevant Flow(s): |  | B.4.4.2 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | NPAC Test Engineers, set SSN Edit Flags to FALSE, create an NPA-NXX-X, schedule a respective Number Pool Block for at least 1 day in the future containing invalid DPC/SSN data (as described below). After the Number Pool Block has been scheduled, but prior to activation, set the SSN Edit Flags to TRUE. Perform test steps as specified below. 1. Verify that the ‘SSN Edit Flag’ indicators are set to production settings for the regions in which the Service Provider under test operates. If the Service Provider operates in regions where the ‘SSN Edit Flag’ indicators may be set to different settings, be sure to test all scenarios with the Service Provider. 2. Verify the Number Pool Block create event exists on the NPAC SMS with some combination of the following invalid DPC/SSN data value scenarios: If the ‘SSN Edit Flags’ are set to TRUE, invalid data would include - Specifying DPC values of (network 001-255, cluster 000-255, member 000-255) and corresponding SSN values other than (000). - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and corresponding SSN values of (000). - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and corresponding SSN values other than (000). If the ‘SSN Edit Flags’ are set to FALSE, invalid data would include - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) when the SSN value is between 000-255. - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and also not specifying a value for the corresponding SSN value. - Specifying DPC values other than (network 001-255, cluster 000-255, member 000-255) and also not specifying a valid SSN value is between 000-255. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Upon reaching the Number Pool Block scheduled date, the NPAC SMS attempts to automatically ‘Create’ the Number Pool Block and respective ‘Pooled’ Subscription Versions for a Number Pool Block create event that has been scheduled with some combination of invalid DPC/SSN information outlined in the prerequisites. |  | NPAC |  | The NPAC SMS determines the following: The Number Pool Block and respective ‘Pooled’ Subscription Version Create request contains invalid DPC/SSN data based on system requirements and the regional ‘SSN Edit Flag’ settings. (This violates system requirements.) |  |  |  |
| 2. | NPAC | The NPAC SMS fails the Number Pool Block Create request and generates appropriate errors. |  | NPAC |  | The error log is appropriately updated to reflect the Number Pool Block and respective ‘Pooled’ Subscription Version create request failure. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a query for the Number Pool Block and respective ‘Pooled’ Subscription Versions the NPAC SMS attempted to automatically create during this test case. |  | NPAC |  | NPAC Personnel verify that the Number Pool Block and respective ‘Pooled’ Subscription Versions were not created on the NPAC SMS. |  |  |  |
| 4. | SP | Service Provider Personnel, perform a local query for the Number Pool Block and the respective ‘Pooled’ Subscription Versions the NPAC SMS attempted to automatically create during this test case. |  | SP |  | Verify that the Number Pool Block and the respective ‘Pooled’ Subscription Versions were not created on the local database. |  |  |  |
| E. | Pass/Fail Analysis, NANC 191/291 - 8 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written for each ‘SSN Edit Flag’ indicator setting for which the Service Provider under test may operate in production. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written for each ‘SSN Edit Flag’ indicator setting where they may operate in production. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 191/291-9 |  | SUT Priority: |  | SOA |  | N/A |
|  |  |  |  |  |  |  | LSMS |  | Required |
|  | Objective: |  | NPAC OP GUI – NPAC Personnel attempt to submit a mass update request that includes at least three complete, ‘Active’ Number Pool Blocks. One of these Number Pool Blocks should currently exist with valid DPC/SSN data, two should exist with invalid DPC/SSN data. The Mass Update criteria shall include all three Number Pool Blocks and the request specifies new DPC/SSN values that will correct one, but not both of the Number Pool Blocks that currently exists with invalid DPC/SSN data. The NPAC SMS processes the Mass Update request, modifies some but not all of the DPC/SSN attributes for the range specified in the Mass Update Request and logs the objects that could not be updated to the Mass Update Exception report. The regional SSN Edit Flags (CLASS, LIDB, CNAM, ISVM and WSMSC) are set to production values. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 191/NANC 291 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0a |  | Relevant Requirement(s): |  | RR3-429 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0a |  | Relevant Flow(s): |  | B.8.3 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Verify that the ‘SSN Edit Flag’ indicators are set to production settings for the regions in which the Service Provider under test operates. If the Service Provider operates in regions where the ‘SSN Edit Flag’ indicators may be set to different settings, be sure to test all scenarios with the Service Provider. 2. Verify that at least 3 ‘Active’ Number Pool Block exist. - One of these Number Pool Blocks should exist with valid DPC/SSN values. (NPB 2a) - One Number Pool Block should exist with invalid CNAM DPC/SSN data (all other DPC/SSN data should be valid. (NPB 2b) - One Number Pool Block should exist with all invalid DPC/SSN data – at a minimum should be CNAM plus at least one other feature set. (NPB 2c) 3. Identify the appropriate Number Pool Block range to use in this test case _____________________. 4. Identify DPC/SSN attributes that should be specified in the Mass Update request such that one of the Number Pool Blocks that currently have invalid DPC/SSN attributes will be corrected, and the other Number Pool Block will remain invalid. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel submit a Mass Update request that includes at least 3 complete, ‘Active’ Number Pool Blocks. (Identified in the prerequisites above). Modify CNAM DPC/SSN data, specifying valid values for the following attributes. - subscriptionCNAM-DPC - subscriptionCNAM-SSN |  | NPAC |  | The NPAC SMS receives the Mass Update Request from the NPAC OP GUI and searches the Number Pool Block and Subscription Version databases for Number Pool Blocks and Subscription Versions that match the input Mass Update criteria. The NPAC SMS determines that a subset of the Number Pool Blocks and Subscription Versions requested exist with invalid DPC/SSN data that is not corrected by the new values specified in the Mass Update request. The NPAC SMS makes an entry to the Mass Update Exception report for these Number Pool Block and Subscription Version objects, and continues updating the remaining Number Pool Blocks and Subscription Versions that meet the Mass Update criteria. |  |  |  |
| 2. | NPAC | NPAC SMS sends to all LSMSs that are accepting downloads for the NPA-NXX(s): - to those EDR LSMSs, NPAC SMS issues M-SET Request(s) numberPoolBlock to update the DPC/SSN data. - to those EDR LSMSs, NPAC SMS issues M-SET Request(s) subscriptionVersion for each contiguous range of non-pooled TN’s within the Mass Update TN range to update the DPC/SSN data. - to those non-EDR LSMSs, NPAC SMS issues M-SET Request(s) subscriptionVersion for each contiguous range of ‘Pooled’ and ‘non-Pooled’ TNs to update the DPC/SSN data. |  | SP |  | All LSMSs that are accepting downloads for the NPA-NXXs of the Number Pool Block objects and Subscription Versions being updated, receive the M-SET requests from the NPAC SMS to modify the DPC/SSN values. The LSMSs issue M-SET Responses indicating they successfully processed the NPAC SMS request. These M-SET Requests are for the Number Pool Blocks and Subscription Versions that were successfully updated based on the Mass Update criteria – and does not include those Number Pool Blocks or Subscription Versions that could not be updated based on the Mass Update criteria. The Number Pool Blocks and Subscription Versions that could not be updated are included on the Mass Update Exception report. |  |  |  |
| 3. | NPAC | 1. If the current Service Provider’s TN Range Notification Indicator is set to TRUE, NPAC SMS issues an M-EVENT-REPORT subscriptionVersionRangeStatusAttributeValueChange for the range of Subscription Versions that were updated indicating the status is now ‘Active’. 2. If the current Service Provider’s TN Range Notification Indicator is set to ‘FALSE’, NPAC SMS issues a subscriptionVersionStatusAttributeValueChange for each Subscription Version that was updated, indicating the status is now ‘Active’. |  | SP |  | The current Service Provider receives the M-EVENT-REPORT from the NPAC SMS and issues an M-EVENT-REPORT response indicating it successfully received the message. |  |  |  |
| 4. | NPAC | NPAC Personnel generate a Mass Update Exception report. |  | NPAC |  | Verify Number Pool Block (NPB 2c) and the respective Subscription Versions within the Mass Update criteria who’s invalid DPC/SSN data was not corrected by the new specified attributes are included on the report. |  |  |  |
| 5. | SP | Service Provider Personnel perform a local query on their LSMS to verify the Mass Update was completed. |  | SP |  | On the LSMS verify: 1. Number Pool Block (NPB 2c) and the respective Subscription Versions within the Mass Update request who’s invalid DPC/SSN data was not corrected by the new specified DPC/SSN attributes were not updated with the new DPC/SSN values. 2. Number Pool Blocks (NPB 2a and 2b) and the respective Subscription Versions within the Mass Update request who’s DPC/SSN values were previously valid, or were invalid but the Mass Update specified attributes corrected the previous issues were updated with the new DPC/SSN values. |  |  |  |
| 6. | NPAC | NPAC Personnel perform a full audit for the range specified in the Mass Update request. |  | NPAC |  | Verify that there are no discrepancies found. Number Pool Block (NPB 2c) and respective Subscription Versions that previously existed with invalid DPC/SSN data that were not corrected by the Mass Update specified attributes – still exist in their previous state. Number Pool Blocks (NPB 2a and 2b) and respective Subscription Versions specified in the Mass Update criteria were updated appropriately. |  |  |  |
| E. | Pass/Fail Analysis, NANC 191/291 - 9 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

# NANC 192 – NPA Split NPAC SMS Load File

With release 3.2.0.c of this document NANC 192-1 was in the group section and was vast in scope.  With release 3.2.1.a of this document, this test case has been appropriately scaled down to test the specific anomalies of NPA Split behavior.  This test case is appropriate for test by ALL Service Providers in an Individual Test Environment, and does not need a coordinated group effort.

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 192-1 |  |  |  | Test Case Number: |  | NANC 192-1 |
|  |  |  |  |  |  |  | LSMS |  | Required |
|  | Objective: |  | SOA/LSMS - Service Provider Personnel perform basic LNP functions before, during and after Permissive Dial Period for NPA Splits that are created on the NPAC SMS. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  |  |  | NANC Change Order Revision Number: |  |  |
|  | NANC FRS Version Number |  | 3.2.0 |  |  |  | NANC FRS Version Number |  |  |
|  | NANC IIS Version Number |  | 3.2.0 |  |  |  | NANC IIS Version Number |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. NPAC Personnel create an NPA Split Load Flat file to be used to specify respective NPA Split data to be used during this test case. Work with the Service Provider under test, and existing Network Data in the test bed to determine precise data to be used. For example: SPID; Old NPA-NXX; New NPA-NXX; PDP Start Date/Time; PDP End Date/Time. SUT; 303-771; 304-771; This is current plus 2 days; This is PDP Start plus 2 days. A; 720-771; 721-771. B; 718-771; 719-771. - Use appropriate data such that the NPA Split Load file should be successfully loaded and new NPA-NXXs appropriately created/broadcast. - Load the NPA Split Load File prior to starting test steps. - NOTE: The NPA Split must exist on the NPAC SMS prior to existing on the local Service Provider systems. Also, the New NPA-NXX may not exist on the NPAC SMS prior to loading the respective NPA Split Load File. - NOTE: It is also possible for NPAC Personnel to use the manual process to create the respective NPA Splits on the NPAC SMS necessary to execute this test case. |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | Row # |  | NPAC or SP |  |  |  |
| 1. | NPAC | 1. NPAC SMS automatically creates the respective New NPA-NXXs involved in the NPA Splits specified in the NPA Split load file. 2. NPAC SMS issues an M-CREATE Request serviceProv NPA-NXX for each New NPA-NXX specified in the NPA Split Load file to each SOA and LSMS accepting downloads for these NPA-NXXs. NOTE:These creates will indicate an Effective Date equal to the PDP Start indicated in the NPA Split Load File. Following the example in the prerequisites this would be 3 requests for (304-771, 721-771 and 719-771 respectively). |  | 1. |  | NPAC |  |  |  |
| 2. | SPs | Service Provider personnel, perform the necessary steps to “create” the NPA Splits on your local/respective systems. |  | 2. |  | SPs |  |  |  |
| 3. | SP | 1. Service Provider personnel create a Subscription Version indicating the Old NPA-NXX value for which you are not the ‘holder’ indicated in the NPA-Split Load File above. In other words you are porting a number away from another service provider. 2. SOA issues an M-ACTION Request subscriptionVersionNewSP-Create to the NPAC SMS indicating the Old NPA-NXX value which is scheduled for an NPA Split. NOTE: if the request specifies the ‘New’ NPA-NXX value, the NPAC SMS will fail the request because the New NPA-NXX Effective Date has not been reached, nor has PDP started. |  | 3. |  | SP |  |  |  |
| 4. |  | Permissive Dial Period Starts |  | 4. |  |  |  |  |  |
| 5. | SP | 1. Service Provider personnel activate the Subscription Version created in row 3 above, specifying the Old NPA-NXX value. 2. SOA issues an M-ACTION Reques subscriptionVersionActivate to the NPAC SMS indicating the Old NPA-NXX value that is currently in PDP. NOTE: If your system automatically only sends the New NPA-NXX value during PDP, this is ok, just continue. NOTE: The difference in behavior during PDP is that Service Provider systems may successfully submit requests that specify the Old NPA-NXX value involved in an NPA Split, but the response from the NPAC SMS will specify the New NPA-NXX value involved in an NPA Split. |  | 5. |  | SP |  |  |  |
| 6. | NPAC | NPAC Personnel perform an audit for the TN that was activated to the LSMS of the Service Provider Under Test to verify the activate request was appropriately processed. The audit request will specify the New NPA-NXX value involved in the NPA Split. |  | 6. |  | NPAC |  |  |  |
| 7. |  | Permissive Dial Period Ends. |  | 7. |  |  |  |  |  |
| 8. | NPAC | NPAC SMS automatically performs ‘clean-up’ process after PDP ends. 2. NPAC SMS issues an M-DELETE Request for each of the Old NPA-NXXs involved in an NPA Split to all LSMSs and SOAs in the region, accepting downloads for the NPA-NXX(s). |  | 8. |  | NPAC |  |  |  |
| 9. | NPAC | 1. NPAC SMS receives the M-DELETE Response from each LSMS that received the original request. 2. NPAC SMS receives the M-DELETE Response from each SOA that received the original request. |  | 9. |  | NPAC |  |  |  |

# NANC 218 – Conflict Timestamp Broadcast to SOA

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 218-1 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | SOA – (Old) Service Provider Personnel submit a single TN, subscription version modify request specifying Authorization (FALSE) and a valid status change cause code, setting the subscription version status to conflict after both Service Providers have created/concurred to the port, and prior to the Conflict Restriction Window - SUCCESS |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 218 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0a |  | Relevant Requirement(s): |  | RR5-44.2, RR5-44.3 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0a |  | Relevant Flow(s): |  | B.5.2.3, B.5.2.4 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Verify that a Subscription Version with a status of ‘Pending’ exists for the TN that will be used in the Subscription Version modify request by the Old Service Provider in this test case, exists on the NPAC SMS. 2. Verify that the current time is prior to the Conflict Restriction Window expiration. 3. The Subscription Version that is going to be used during this Test Case should not previously have been put in conflict before, should have never had a status of ‘Conflict’, and should not have a Conflict Time Stamp value. 4. TN Used:______________________________ |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | 1. Using their SOA system, Old Service Provider Personnel submit a Subscription Version Modify request for a ‘Pending’ Subscription Version that has previously been created by the New Service Provider and concurred to by the Old Service Provider. Specify the TN identified in the prerequisite steps above. 2. The SOA sends either an M-ACTION Request subscriptionVersionModify or an M-SET Request subscriptionVersionNPAC specifying the TN identified in the prerequisite above and setting the subscriptionOldSP-Authorization to FALSE and indicating a valid subscriptionStatusChangeCauseCode. |  | NPAC |  | NPAC SMS receives the request from the Old SP SOA and verifies that each attribute specified is valid according to system requirements. |  |  |  |
| 2. | NPAC | The NPAC SMS: 1. If an M-ACTION Request subscriptionVersionModify was sent, issues an M-ACTION Response back to the Old SP SOA followed by an (internal) M-SET Request subscriptionVersionNPAC and M-SET Response subscriptionVersionNPAC to itself. 2. If an M-SET Request subscriptionVersionNPAC was sent, issues an M-SET Response subscriptionVersionNPAC to the Old SP SOA. |  | SP |  | The Old Service Provider SOA receives the Response from the NPAC SMS. |  |  |  |
| 3. | NPAC | NPAC SMS issues an M-EVENT-REPORT to the Old SP SOA based on their Customer TN Range Notification Indicator. 1. If the setting is TRUE, NPAC SMS issues an M-EVENT-REPORT subscriptionVersionRangeAttributeValueChange including the attributes bulleted below: 2…..If the setting is FALSE, NPAC SMS issues an M-EVENT-REPORT attributeValueChange including the attributes bulleted below: - subscriptionNewSP-DueDate - subscriptionOldSP-Authorization (set to FALSE) - subscriptionOldSP-AuthorizationTimeStamp - subscriptionStatusChangeCauseCode - subscriptionVersionStatus (Conflict) - subscriptionConflictTimeStamp |  | SP |  | The Old Service Provider SOA receives the M-EVENT-REPORT from the NPAC SMS. |  |  |  |
| 4. | SP | Old SP SOA issues an M-EVENT-REPORT Confirmation to the NPAC SMS indicating it successfully received the M-EVENT-REPORT from the NPAC SMS. |  | NPAC |  | NPAC SMS receives the M-EVENT-REPORT Confirmation from the Old SP SOA. |  |  |  |
| 5. | SP | At the same time as row 3 above, NPAC SMS issues an M-EVENT-REPORT to the New SP SOA based on their Customer TN Range Notification Indicator. 1. If the setting is TRUE, NPAC SMS issues an M-EVENT-REPORT subscriptionVersionRangeAttributeValueChange including the attributes bulleted below: 2…..If the setting is FALSE, NPAC SMS issues an M-EVENT-REPORT attributeValueChange including the attributes bulleted below: - subscriptionNewSP-DueDate - subscriptionOldSP-Authorization (set to FALSE) - subscriptionOldSP-AuthorizationTimeStamp - subscriptionStatusChangeCauseCode - subscriptionVersionStatus (Conflict) - subscriptionConflictTimeStamp |  | SP |  | The New Service Provider SOA receives the M-EVENT-REPORT from the NPAC SMS. |  |  |  |
| 6. | SP | New SP SOA issues an M-EVENT-REPORT Confirmation to the NPAC SMS indicating it successfully received the M-EVENT-REPORT from the NPAC SMS. |  | NPAC |  | NPAC SMS receives the M-EVENT-REPORT Confirmation from the New SP SOA. |  |  |  |
| 7. | SP | Using their SOA, Old SP Personnel perform a local query for the subscription version modified during this test case. |  | SP |  | The subscription version exists with a status of ‘conflict’ and that the ConflictTimeStamp is set appropriately. |  |  |  |
| 8. | NPAC | NPAC Personnel perform a query for the subscription version modified in this test case. |  | NPAC |  | The subscription version exists with a status of ‘conflict’. |  |  |  |
| E. | Pass/Fail Analysis, NANC 218-1 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel confirm they received all attributes included in the M-EVENT-REPORT request from the NPAC SMS listed in row 3 above. |  |  |  |  |  |  |  |

This test case will supersede NANC 214-1 in the functional and regression test plan.

| A. |  |  |  | TEST IDENTITY |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test Case Number: |  |  | NANC 218-2 | SUT Priority: |  |  | SOA |  |  | Required |  |  |
|  |  |  |  |  |  |  |  |  |  |  | LSMS |  |  | N/A |  |  |
|  |  |  |  | Objective: |  |  | SOA – Old Service Provider personnel successfully put a pending Subscription Version into conflict using an Old Service Provider create after the Conflict Restriction Window Tunable Time has been reached but before the Final Concurrence Timer (T2) has expired. – Success |  |  |  |  |  |  |  |  |  |
| B. |  |  |  | REFERENCES |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  | NANC 218 |  |  |  |  |
|  |  |  |  | NANC FRS Version Number: |  |  | 3.2.0.a | Relevant Requirement(s): |  |  |  | RR5-44.2, RR5-44.3 |  |  |  |  |
|  |  |  |  | NANC IIS Version Number: |  |  | 3.2.0.a | Relevant Flow(s): |  |  |  | Based on B.5.1.4 |  |  |  |  |
| C. |  |  |  | PREREQUISITE |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Prerequisite NPAC Setup: |  |  | Verify that a New Service Provider pending Subscription Version has been created where the Service Provider under test is the Old Service Provider, the due date is today and the Final Concurrence Timer has not expired. |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |  |  |  |  |
| D. |  |  |  | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |  |  |  |  |
| Row # |  | NPAC or SP |  |  | Test Step |  |  |  | NPAC or SP | Expected Result |  |  |  |  |  |  |
| 1. |  | SP |  |  | Using the SOA, Old Service Provider personnel create a subscriptionVersionOldSP-Create M-ACTION Request with the authorization flag set to “FALSE” for a ‘pending’ Subscription Version created by the New Service Provider where the due date is today and the Final Concurrence Timer has not expired. |  |  |  | SP | The SOA issues a subscriptionVersionOldSP-Create M-ACTION to the NPAC SMS. |  |  |  |  |  |  |
| 2. |  | NPAC |  |  | The NPAC SMS accepts the M-ACTION Request from the Service Provider. |  |  |  | NPAC | The NPAC SMS sets the Subscription Version to conflict and sets all of the other values from the subscriptionVersionOldSP-Create M-ACTION Request. |  |  |  |  |  |  |
| 3. |  | NPAC |  |  | The NPAC SMS issues an M-ACTION Response. |  |  |  | SP | The SOA receives the successful subscriptionVersionOldSP-Create M-ACTION Response. |  |  |  |  |  |  |
| 4. |  | NPAC |  |  | NPAC SMS issues an M-EVENT-REPORT to the Old SP SOA based on their Customer TN Range Notification Indicator. If the setting is TRUE, NPAC SMS issues an M-EVENT-REPORT subscriptionVersionRangeAttributeValueChange including the attributes bulleted below: 2…..If the setting is FALSE, NPAC SMS issues an M-EVENT-REPORT attributeValueChange including the attributes bulleted below: - subscriptionOldSP-DueDate - subscriptionOldSP-Authorization (set to FALSE) - subscriptionOldSP-AuthorizationTimeStamp - subscriptionStatusChangeCauseCode - subscriptionVersionStatus (Conflict) - subscriptionConflictTimeStamp |  |  |  | SP | The Old Service Provider SOA receives the M-EVENT-REPORT from the NPAC SMS. |  |  |  |  |  |  |
| 5. |  | SP |  |  | Old SP SOA issues an M-EVENT-REPORT Confirmation to the NPAC SMS indicating it successfully received the M-EVENT-REPORT from the NPAC SMS. |  |  |  | NPAC | NPAC SMS receives the M-EVENT-REPORT Confirmation from the Old SP SOA. |  |  |  |  |  |  |
| 6. |  | NPAC |  |  | At the same time as row 4 above, NPAC SMS issues an M-EVENT-REPORT to the New SP SOA based on their Customer TN Range Notification Indicator. 1. If the setting is TRUE, NPAC SMS issues an M-EVENT-REPORT subscriptionVersionRangeAttributeValueChange including the attributes bulleted below: 2…..If the setting is FALSE, NPAC SMS issues an M-EVENT-REPORT attributeValueChange including the attributes bulleted below: - subscriptionOldSP-DueDate - subscriptionOldSP-Authorization (set to FALSE) - subscriptionOldSP-AuthorizationTimeStamp - subscriptionStatusChangeCauseCode - subscriptionVersionStatus (Conflict) - subscriptionConflictTimeStamp |  |  |  | SP | The New Service Provider SOA receives the M-EVENT-REPORT from the NPAC SMS. |  |  |  |  |  |  |
| 7. |  | SP |  |  | New SP SOA issues an M-EVENT-REPORT Confirmation to the NPAC SMS indicating it successfully received the M-EVENT-REPORT from the NPAC SMS. |  |  |  | NPAC | NPAC SMS receives the M-EVENT-REPORT Confirmation from the New SP SOA. |  |  |  |  |  |  |
| 8. |  | SP |  |  | Using their SOA, Old SP Personnel perform a local query for the subscription version they created during this test case. |  |  |  | SP | The subscription version exists with a status of ‘conflict’ and that the ConflictTimeStamp is set appropriately. |  |  |  |  |  |  |
| 9. |  | NPAC |  |  | NPAC Personnel perform a query for the Subscription Version to verify that it has a status of ‘conflict’. |  |  |  | NPAC | The Subscription Version has a status of ‘conflict’, the cause code, the authorization time stamp, the conflict time stamp and the Old Service Provider due date is set and the authorization flag is set to False. |  |  |  |  |  |  |
|  | E. |  | Pass/Fail Analysis, NANC 218-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pass |  | Fail |  |  | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |  |  |  |
|  | Pass |  | Fail |  |  | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |  |  |  |
|  | Pass |  | Fail |  |  | Service Provider Personnel confirm they received all attributes included in the M-EVENT-REPORT request from the NPAC SMS listed in row 4 above. |  |  |  |  |  |  |  |  |  |  |

# NANC 230 – Donor SOA Port-To-Original of Intra-Service Provider Port

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 230-1 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | SOA – Service Provider Personnel create an Intra-Service Provider, Port-to-Original Subscription Version where a previously ‘Active’ Subscription Version exists, that is not part of a Number Pool Block – Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 230 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0a |  | Relevant Requirement(s): |  | RR5-4, RR5-122, RR5-6.1 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0a |  | Relevant Flow(s): |  | B.5.1.11 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Verify that a Subscription Version with a status of ‘Active’ exists for the TN that will be used in the Intra-Service Provider, Port-to-Original Subscription Version create request in this test case, for the current Service Provider, on the NPAC SMS. 2. The TN that is going to be used during this Test Case should not be part of a Number Pool Block or be associated in any way with an existing NPA-NXX-X on the NPAC SMS. 3. TN Used:______________________________ |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | 1. Using the SOA, SP Personnel submit an M-ACTION subscriptionVersionNewSP-Create request to the NPAC SMS for an Intra-Service Provider, Port-to-Original, single TN, Subscription Version for which there is a currently ‘Active’ Subscription Version for which they are the current Service Provider. 2. The SOA sends an M-ACTION subscriptionVersionNewSP-Create to the NPAC SMS for the single TN and includes only the following attributes: - subscriptionTN - subscriptionNewCurrentSP - subscriptionOldSP - subscriptionNewSP-DueDate (seconds set to zeros) - subscriptionPortingToOriginal-SPSwitch |  | NPAC |  | NPAC SMS receives the M-ACTION subscriptionVersionNewSP-Create request from the New/Current SP SOA. |  |  |  |
| 2. | NPAC | NPAC SMS issues an M-CREATE Request subscriptionVersionNPAC to itself for the TN to create the Subscription Version, sets the subscriptionVersionStatus to ‘Pending’, and sets the subscriptionNewSPCreationTimeStamp, and the subscriptionModifedTimeStamp to the current date and time. |  | NPAC |  | NPAC SMS issues an M-CREATE Response to itself. |  |  |  |
| 3. | NPAC | NPAC SMS issues an M-ACTION Response subscriptionVersionNPAC to the New/Current SP indicating it successfully received the Intra-Service Provider, Port-to-Original, Subscription Version create request. |  | SP |  | New/Current SP SOA receives the M-ACTION Response. |  |  |  |
| 4. | NPAC | NPAC SMS issues an M-EVENT-REPORT to the New/Current SP SOA based on their Customer TN Range Notification Indicator: 1. If the setting is TRUE, NPAC SMS issues an M-EVENT-REPORT subscriptionVersionRangeObjectCreation specifying the following attributes: - start TN - end TN - start SVID - end SVID - those attributes bulleted below: 2. If the setting is FALSE, NPAC SMS issues an M-EVENT-REPORT objectCreation specifying the following attributes:: - subscriptionVersionId - subscriptionTN - The following attributes will also be sent in the ObjectCreation or subscriptionVersionRangeObjectCreation notification: - subscriptionOldSP - subscriptionNewCurrentSP - subscriptionNewSP-DueDate - subscriptionNewSP-CreationTimeStamp - subscriptionVersionStatus - subscriptionTimerType (if supported) - subscriptionBusinessType (if supported) |  | NPAC |  | New/Current SP SOA receives the M-EVENT-REPORT from the NPAC SMS. |  |  |  |
| 5. | SP | New/Current SP SOA issues an M-EVENT-REPORT Confirmation to the NPAC SMS indicating it successfully received the M-EVENT-REPORT from the NPAC SMS. |  | NPAC |  | NPAC SMS receives the M-EVENT-REPORT Confirmation from the New/Current SP SOA. |  |  |  |
| 6. | SP | Using their SOA, New/Current SP Personnel perform a local query for the Subscription Version created in this test case. |  | SP |  | The Subscription Version exists with a status of ‘Pending’, an LNP type of ‘LISP’ and the Port-to-Original indicator set to TRUE. |  |  |  |
| 7. | NPAC | NPAC Personnel perform a query for the Subscription Version created in this test case. |  | NPAC |  | The Subscription Version exists with a status of ‘Pending’, an LNP type of ‘LISP’, and the Port-to-Original indicator set to TRUE. |  |  |  |
| E. | Pass/Fail Analysis, NANC 230-1 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 230-2 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | SOA – Service Provider Personnel create an Intra-Service Provider, Port-to-Original Subscription Version where a previously ‘Active’ Subscription Version exists with a matching NPA-NXX-X, after the NPA-NXX-X Creation and prior to the Number Pool Block Activation - Failure |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 230 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0a |  | Relevant Requirement(s): |  | RR5-121 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0a |  | Relevant Flow(s): |  | B.5.1.11 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Verify that a Subscription Version with a status of ‘Active’ exists for the TN that will be used in the Intra-Service Provider, Port-to-Original Subscription Version create request in this test case, for the current Service Provider, on the NPAC SMS. 2. The TN/Subscription Version that is going to be used during this Test Case should have an NPA-NXX-X that exists on the NPAC SMS, but the respective Number Pool Block has not yet been Activated. 3. TN Used:______________________________ |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | Using the SOA, SP Personnel submit an M-ACTION subscriptionVersionNewSP-Create request to the NPAC SMS for an Intra-Service Provider, Port-to-Original, single TN, Subscription Version for which there is a currently ‘Active’ Subscription Version for which they are the current Service Provider. This TN should have a respective NPA-NXX-X that has been created on the NPAC SMS but respective Number Pool Block has not yet been activated. The SOA sends an M-ACTION subscriptionVersionNewSP-Create to the NPAC SMS for the single TN and includes only the following attributes: - subscriptionTN - subscriptionNewCurrentSP - subscriptionOldSP - subscriptionNewSP-DueDate (seconds set to zeros) - subscriptionPortingToOriginal-SPSwitch |  | NPAC |  | NPAC SMS receives the M-ACTION subscriptionVersionNewSP-Create request from the New/Current SP SOA and determines the request is invalid because system requirements have been violated: NPAC SMS shall reject a request for an Intra-Service Provider, Port-to-Original Subscription Version create after the creation of the NPA-NXX-X and prior to the existence of the respective Block. |  |  |  |
| 2. | NPAC | 1. NPAC SMS issues an M-ACTION Response subscriptionVersionNPAC to the New/Current SP SOA indicating it did not successfully validate the Intra-Service Provider, Port-to-Original, Subscription Version create request. 2. Further processing is ceased. |  | SP |  | New/Current SP SOA receives the M-ACTION Response. |  |  |  |
| 3. | SP | Using their SOA, New/Current SP Personnel perform a local query for the Subscription Version they attempted to create in this test case. |  | SP |  | Verify the Subscription Version does not exist. Verify that an error response was received from the NPAC SMS. |  |  |  |
| 4. | NPAC | NPAC Personnel perform a query for the Subscription Version that the SP personnel attempted to create in this test case. |  | NPAC |  | Verify that the Subscription Version does not exist. |  |  |  |
| E. | Pass/Fail Analysis, NANC 230-2 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 230-3 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | SOA – Service Provider Personnel create an Intra-Service Provider, Porting to Original Subscription Version after NPA-NXX-X Effective Date and Block Activation – Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 230 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0a |  | Relevant Requirement(s): |  | RR5-57 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0a |  | Relevant Flow(s): |  | B.5.1.11 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. On behalf of the service provider under test, create and activate a Subscription Version from a Code Holder. The service provider under test is the ‘New’ service provider. 2. On behalf of the service provider under test, create an NPA-NXX-X whereby the Subscription Version created in step 1 above, is respective to the NPA-NXX-X to be created. 3. On behalf of the service provider under test, activate a respective Number Pool Block for the NPA-NXX-X that was created in step 2 above. Verify that the Number Pool Block and Pooled Subscription Versions exist. Verify that the Subscription Versions that was activated in step 1 above exists as an LSPP port with a status of ‘Active’ where the service provider under test is the current service provider for the port. 4. TN Used:______________________________ |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | 1. Using the SOA, SP Personnel submit an M-ACTION subscriptionVersionNewSP-Create request to the NPAC SMS for an Intra-Service Provider, Port-to-Original, single TN, Subscription Version for which there is a currently ‘Active’ Subscription Version for which they are the current Service Provider. Specify the TN identified in the prerequisites above. 2. The SOA sends an M-ACTION subscriptionVersionNewSP-Create to the NPAC SMS for the single TN and includes only the following attributes: - subscriptionTN - subscriptionNewCurrentSP - subscriptionOldSP - subscriptionNewSP-DueDate (seconds set to zeros) - subscriptionPortingToOriginal-SPSwitch |  | NPAC |  | NPAC SMS receives the M-ACTION subscriptionVersionNewSP-Create request from the New/Current SP SOA. |  |  |  |
| 2. | NPAC | NPAC SMS issues an M-CREATE Request subscriptionVersionNPAC to itself for the TN to create the Subscription Version, sets the subscriptionVersionStatus to ‘Pending’, and sets the subscriptionNewSPCreationTimeStamp, and the subscriptionModifedTimeStamp to the current date and time. |  | NPAC |  | NPAC SMS issues an M-CREATE Response to itself. |  |  |  |
| 3. | NPAC | NPAC SMS issues an M-ACTION Response subscriptionVersionNPAC to the New/Current SP indicating it successfully received the Intra-Service Provider, Port-to-Original, Subscription Version create request. |  | SP |  | New/Current SP SOA receives the M-ACTION Response. |  |  |  |
| 4. | NPAC | NPAC SMS issues an M-EVENT-REPORT to the New/Current SP SOA based on their Customer TN Range Notification Indicator: 1. If the setting is TRUE, NPAC SMS issues an M-EVENT-REPORT subscriptionVersionRangeObjectCreation specifying the following attributes: - start TN - end TN - start SVID - end SVID - those attributes bulleted below: 2. If the setting is FALSE, NPAC SMS issues an M-EVENT-REPORT objectCreation specifying the following attributes: - subscriptionVersionId - subscriptionTN - The following attributes will also be sent in the ObjectCreation or subscriptionVersionRangeObjectCreation notification: - subscriptionOldSP - subscriptionNewCurrentSP - subscriptionNewSP-DueDate - subscriptionNewSP-CreationTimeStamp - subscriptionVersionStatus - subscriptionTimerType (if supported) - subscriptionBusinessType (if supported) |  | NPAC |  | New/Current SP SOA receives the M-EVENT-REPORT from the NPAC SMS. |  |  |  |
| 5. | SP | New/Current SP SOA issues an M-EVENT-REPORT Confirmation to the NPAC SMS indicating it successfully received the M-EVENT-REPORT from the NPAC SMS. |  | NPAC |  | NPAC SMS receives the M-EVENT-REPORT Confirmation from the New/Current SP SOA. |  |  |  |
| 6. | SP | Using their SOA, New/Current SP Personnel perform a local query for the Subscription Version created in this test case. |  | SP |  | The Subscription Version exists with a status of ‘Pending’, an LNP type of ‘LISP’ and the Port-to-Original indicator set to TRUE. |  |  |  |
| 7. | NPAC | NPAC Personnel perform a query for the Subscription Version created in this test case. |  | NPAC |  | The Subscription Version exists with a status of ‘Pending’, an LNP type of ‘LISP’, and the Port-to-Original indicator set to TRUE. |  |  |  |
| E. | Pass/Fail Analysis, NANC 230-3 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

# NANC 249 – Modification of Dates for a Disconnect Pending SV

NOTE:  Service Provider’s whose systems cannot create the ‘failure’ scenarios that follow pass those test cases be default.  If their system does not ‘stop’ the invalid message before it goes across the interface, then their system must be able to successfully execute the test case and handle the failure response from the NPAC SMS.

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 249-1 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | Required |
|  | Objective: |  | SOA – Service Provider Personnel submit a Subscription Version modify request for a ‘Disconnect-Pending’ Subscription Version, modifying the Effective Release Date and Customer Disconnect Date to the current date/time or a date/time in the past. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 249 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0a |  | Relevant Requirement(s): |  | RR5-124, RR5-125, RR5-127, RR5-129, RR5-126, RR5-11 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0a |  | Relevant Flow(s): |  | B.5.2.7, B.5.4.1, B.5.4.1.1 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Verify that a Subscription Version with a status of ‘Disconnect-Pending’ exists on the NPAC SMS for the Service Provider participating in this Test Case. The Subscription Version should have an Effective Release Date and a Customer Disconnect Date currently set to at least one day in the future. 2. TN Used __________________________________ |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | Disconnect an ‘Active’ Subscription Version for which you are the Current Service Provider and specify an Effective Release Date and a Customer Disconnect Date that are at least one day in the future. The Effective Release and Customer Disconnect Dates do not have to be the same date/time. |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | 1. Using the SOA, SP Personnel submit an M-ACTION Request subscriptionVersionModify to the NPAC SMS to modify the Effective Release Date and Customer Disconnect Date to either the current dates/time or some other date in the past, for a single TN Subscription Version that has a current status of ‘Disconnect-Pending’. 2. The SOA sends an M-ACTION subscriptionVersionModify request to the NPAC SMS for the single TN and modifies the subscriptionCustomerDisconnectDate and the subscriptionEffectiveReleaseDate to the current or past date/times. |  | NPAC |  | NPAC SMS receives the M-ACTION Request subscriptionVersionModify from the Current SP SOA. |  |  |  |
| 2. | NPAC | The NPAC SMS validates the SOA request and issues an M-SET Request subscriptionVersionNPAC to itself, updating the modified attributes and setting the subscriptionModifiedTimeStamp to the current date/time. |  | NPAC |  | NPAC SMS receives the M-SET Request subscriptionVersionNPAC. |  |  |  |
| 3. | NPAC | The NPAC SMS issues an M-ACTION Response subscriptionVersionModify to the Current SP SOA indicating the request was successfully processed by the NPAC SMS. |  | SP |  | Current SP SOA receives the M-ACTION Response from the NPAC SMS. |  |  |  |
| 4. | NPAC | NPAC SMS issues an M-SET Request subscriptionVersionNPAC updating the subscriptionVersionStatus to ‘Sending’ and setting the subscriptionCustomerDisconnectDate and subscriptionBroadcastTimeStamp. |  | NPAC |  | NPAC SMS receives the M-SET subscriptionVersionNPAC and issues an M-SET Response to itself. |  |  |  |
| 5. | NPAC | - If the donor Service Provider’s TN Range Notification indicator is set to TRUE, NPAC SMS issues an M-EVENT-REPORT subscriptionVersionRangeDonorSP-CustomerDisconnectDate to the donor Service Provider. - If the donor Service Provider’s TN Range Notification indicator is set to FALSE, NPAC SMS issues an M-EVENT-REPORT subscriptionVersionDonorSP-CustomerDisconnectDate to the donor Service Provider. The M-EVENT-REPORT indicates the TN is being disconnected with the customer disconnect date. |  | SP |  | The donor Service Provider receives the M-EVENT-REPORT from the NPAC SMS and issues an M-EVENT-REPORT Confirmation back. |  |  |  |
| 6. | NPAC | NPAC SMS issues an M-DELETE Request subscriptionVersion to all LSMSs that are accepting downloads for the NPA-NXX for Subscription Version that was modified in Row 1 above. |  | LSMS |  | Each LSMS receives the M-DELETE request from the NPAC SMS. |  |  |  |
| 7. | LSMS | Each LSMS issues an M-DELETE Response back to the NPAC SMS indicating they successfully processed the M-DELETE request. |  | NPAC |  | NPAC SMS receives an M-DELETE Response from each LSMS accepting downloads for this NPA-NXX. |  |  |  |
| 8. | NPAC | NPAC SMS issues an M-SET Request subscriptionVersionNPAC updating the subscriptionVersionStatus to ‘Old’ and setting the subscriptionModifiedTimeStamp and subscriptionDisconnectCompleteTimeStamp. |  | NPAC |  | NPAC SMS receives the M-SET subscriptionVersionNPAC and issues an M-SET Response to itself. |  |  |  |
| 9. | NPAC | NPAC SMS issues an M-EVENT-REPORT subscriptionVersionStatusAttributeValueChange to the Service Provider SOA that issued the disconnect request, indicating the Subscription Version status is ‘Old’. |  | SP |  | The SOA receives the M-EVENT-REPORT subscriptionVersionStatusAttributeValueChange from the NPAC SMS. |  |  |  |
| 10. | SP | Using their SOA, New/Current SP Personnel perform a local query for the ‘Disconnect-Pending’ Subscription Version that was modified and then subsequently ‘disconnected’ from the NPAC network during this test case. |  | SP |  | The Subscription Version exists with a status of ‘Old’, or something equivalent to indicate that this Subscription Version is no longer active. |  |  |  |
| 11. |  | NPAC Personnel perform a query for the ‘Disconnect-Pending’ Subscription Version that was modified and then subsequently ‘disconnected’ from the NPAC network during this test case. |  | NPAC |  | The Subscription Version exists with a status of ‘Old’. |  |  |  |
| E. | Pass/Fail Analysis, NANC 249-1 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 249-2 |  | SUT Priority: |  | SOA |  | Conditional |
|  |  |  |  |  |  |  | LSMS |  | Conditional |
|  | Objective: |  | SOA – Service Provider Personnel submit a Subscription Version modify request for a range of ‘Disconnect-Pending’ Subscription Versions, modifying the Effective Release Date and Customer Disconnect Date to a different date/time in the future. The range of Subscription Versions had Effective Release Dates that were not the same prior to the modification. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 249 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0a |  | Relevant Requirement(s): |  | RR5-127 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0a |  | Relevant Flow(s): |  | B.5.2.7 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Verify that a range of at least 20 Subscription Version with a status of ‘Disconnect-Pending’ exist on the NPAC SMS for the Service Provider participating in this Test Case. These Subscription Versions should have an Effective Release Date and a Customer Disconnect Date currently set to at least one day in the future. Make sure that the set of Subscription Versions have a wide range of different Effective Release Dates and a Customer Disconnect Dates from one another. 2. TNs Used __________________________________ |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | Disconnect a range of at least 20 ‘Active’ Subscription Versions for which you are the Current Service Provider and specify a variety of different Effective Release Dates and a Customer Disconnect Dates that are at least one day in the future. The Effective Release and Customer Disconnect Dates should not have the same date/time – all should be deferred disconnects. |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | 1. Using the SOA, SP Personnel submit an M-ACTION Request subscriptionVersionModify to the NPAC SMS to modify the Effective Release Date and Customer Disconnect Date to a date/time at least one day in the future, for a range of at least 20 TNs that have a current status of ‘Disconnect-Pending’. 2. The SOA sends an M-ACTION subscriptionVersionModify request to the NPAC SMS for the range of at least 20 TNs and modifies the subscriptionCustomerDisconnectDate and the subscriptionEffectiveReleaseDate to a date/time at least one day in the future. |  | NPAC |  | NPAC SMS receives the M-ACTION Request subscriptionVersionModify from the Current SP SOA. |  |  |  |
| 2. | NPAC | The NPAC SMS validates the SOA request and issues an M-SET Request subscriptionVersionNPAC to itself, updating the modified attributes and setting the subscriptionModifiedTimeStamp to the current date/time. |  | NPAC |  | NPAC SMS receives the M-SET Request subscriptionVersionNPAC. |  |  |  |
| 3. | NPAC | The NPAC SMS issues an M-ACTION Response subscriptionVersionModify to the Current SP SOA indicating the request was successfully processed by the NPAC SMS. |  | SP |  | Current SP SOA receives the M-ACTION Response from the NPAC SMS. |  |  |  |
| 4. | SP | Using their SOA, New/Current SP Personnel perform a local query for the ‘Disconnect-Pending’ Subscription Versions that were modified during this test case. |  | SP |  | The Subscription Versions exists with a status of ‘Disconnect-Pending’ with the new Effective Release and Customer Disconnect Dates that were specified in Row 1 above. |  |  |  |
| 5. |  | NPAC Personnel perform a query for the ‘Disconnect-Pending’ Subscription Versions that were modified during this test case. |  | NPAC |  | The Subscription Versions exists with a status of ‘Disconnect-Pending’ with the new Effective Release and Customer Disconnect Dates that were specified in Row 1 above. |  |  |  |
| E. | Pass/Fail Analysis, NANC 249-2 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 249-3 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | SOA – Service Provider Personnel submit a Subscription Version modify request for a ‘Disconnect-Pending’ Subscription Version, without specifying the Customer Disconnect Date - Failure |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 249 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0a |  | Relevant Requirement(s): |  | RR5-128, R5-39.1, R5-29.2 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0a |  | Relevant Flow(s): |  | B.5.2.7 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Verify that a Subscription Version with a status of ‘Disconnect-Pending’ exists on the NPAC SMS for the Service Provider participating in this Test Case. The Subscription Version should have an Effective Release Date and a Customer Disconnect Date currently set to at least one day in the future. 2. TN Used __________________________________. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | Disconnect an ‘Active’ Subscription Version for which you are the Current Service Provider and specify an Effective Release Date and a Customer Disconnect Date that are at least one day in the future. The Effective Release and Customer Disconnect Dates do not have to be the same date/time. |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | 1. Using the SOA, SP Personnel submit an M-ACTION Request subscriptionVersionModify to the NPAC SMS to modify the Effective Release Date to either the current date/time or some other date in the past, for a single TN Subscription Version that has a current status of ‘Disconnect-Pending’. 2. The SOA sends an M-ACTION subscriptionVersionModify request to the NPAC SMS for the single TN and modifies the subscriptionEffectiveReleaseDate to the current or past date/times. The message leaves the subscriptionCustomerDisconnectDate unpopulated. |  | NPAC |  | NPAC SMS receives the M-ACTION Request subscriptionVersionModify from the Current SP SOA and determines that the request violates system requirements. The subscriptionCustomerDisconnectDate must be populated in the modify request message. |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-ACTION Response subscriptionVersionModify to the Current SP SOA indicating the request was not successfully processed by the NPAC SMS and includes an appropriate error message, ‘invalidAurgumentValue’. |  | SP |  | Current SP SOA receives the M-ACTION Response from the NPAC SMS. |  |  |  |
| 3. | SP | Using their SOA, New/Current SP Personnel perform a local query for the ‘Disconnect-Pending’ Subscription Version that they attempted to modify during this test case. |  | SP |  | The Subscription Version exists with a status of ‘Disconnect-Pending’ with the original Effective Release and Customer Disconnect Dates that were specified in the prerequisites above. |  |  |  |
| 4. |  | NPAC Personnel perform a query for the ‘Disconnect-Pending’ Subscription Version that they attempted to modify during this test case. |  | NPAC |  | The Subscription Version exists with a status of ‘Disconnect-Pending’ with the original Effective Release and Customer Disconnect Dates that were specified in the prerequisites above. |  |  |  |
| E. | Pass/Fail Analysis, NANC 249-3 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 249-4 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | SOA – Service Provider Personnel submit a Subscription Version modify request for a ‘Disconnect-Pending’ Subscription Version, specifying an invalid format for the Effective Release Date and/or Customer Disconnect Date - Failure |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 249 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0a |  | Relevant Requirement(s): |  | RR5-125 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0a |  | Relevant Flow(s): |  | B.5.2.7 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Verify that a Subscription Version with a status of ‘Disconnect-Pending’ exists on the NPAC SMS for the Service Provider participating in this Test Case. The Subscription Version should have an Effective Release Date and a Customer Disconnect Date currently set to at least one day in the future. 2. TN Used __________________________________. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | Disconnect an ‘Active’ Subscription Version for which you are the Current Service Provider and specify an Effective Release Date and a Customer Disconnect Date that are at least one day in the future. The Effective Release and Customer Disconnect Dates do not have to be the same date/time. |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | 1. Using the SOA, SP Personnel submit an M-ACTION Request subscriptionVersionModify to the NPAC SMS to modify the Effective Release and Customer Disconnect Dates to either the current date/time or some other date in the past, for a single TN Subscription Version that has a current status of ‘Disconnect-Pending’. 2. The SOA sends an M-ACTION subscriptionVersionModify request to the NPAC SMS for the single TN and modifies the subscriptionCustomerDisconnectDate and the subscriptionEffectiveReleaseDate to the current or past date/times. The message includes both attributes but at least one of these date/time attributes is in an invalid format. The valid format is defined in Table 3-6 Subscription Version Data Model of the FRS. This should be a timestamp, month, day, year, hour, minute, and seconds. |  | NPAC |  | NPAC SMS receives the M-ACTION Request subscriptionVersionModify from the Current SP SOA and determines that the request violates system requirements. The subscriptionCustomerDisconnectDate and/or subscriptionEffectiveReleaseDate must follow the valid format defined in Table 3-6 Subscription Version Data Model, of the FRS.. |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-ACTION Response subscriptionVersionModify to the Current SP SOA indicating the request was not successfully processed by the NPAC SMS and includes an appropriate error message, ‘invalidAurgumentValue’. |  | SP |  | Current SP SOA receives the M-ACTION Response from the NPAC SMS. |  |  |  |
| 3. | SP | Using their SOA, New/Current SP Personnel perform a local query for the ‘Disconnect-Pending’ Subscription Version that they attempted to modify during this test case. |  | SP |  | The Subscription Version exists with a status of ‘Disconnect-Pending’ with the original Effective Release and Customer Disconnect Dates that were specified in the prerequisites above. |  |  |  |
| 4. |  | NPAC Personnel perform a query for the ‘Disconnect-Pending’ Subscription Version that they attempted to modify during this test case. |  | NPAC |  | The Subscription Version exists with a status of ‘Disconnect-Pending’ with the original Effective Release and Customer Disconnect Dates that were specified in the prerequisites above. |  |  |  |
| E. | Pass/Fail Analysis, NANC 249-4 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

# NANC 297 – Sending SV Problem During Recovery

This section of test cases has been incorporated into test case 187-1.

# NANC 319 – NPAC Edit to Ensure NPA-NXX of LRN is in Same LATA as NPA-NXX of Ported TN

NOTE:  Service Provider’s whose systems cannot create the ‘failure’ scenarios that follow pass those test cases be default.  If their system does not ‘stop’ the invalid message before it goes across the interface, then their system must be able to successfully execute the test case and handle the failure response from the NPAC SMS.

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 319–1 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | SOA – Service Provider Personnel attempt to create a Subscription Version specifying a TN and an LRN with different LATA Ids. - Failure |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 319 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0 |  | Relevant Requirement(s): |  | RR5-120 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0 |  | Relevant Flow(s): |  | B.5.1.1, B.5.1.2 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Verify the NPA-NXX exists and is open for porting for the TN that is going to be used during this test case. 2. Verify that the LRN exists for the Service Provider under test. Note the LATA ID for this LRN _______________________. 3. Identify which TN is to be used in this test case TN: _____________________ and respective LATA ID: ________________________. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | Using their SOA system, Service Provider Personnel submit a Subscription Version Create request for a single TN. The SOA system sends an M-ACTION Request subscriptionVersionNewSP-Create to the NPAC SMS to create the subscriptionVersionNPAC (Subscription Version) on the NPAC SMS. The following attributes must be specified: - subscriptionTN or a valid subscriptionVersionTN-Range (specify the TN identified in the prerequisites) - subscriptionNewCurrentSP - subscriptionOldSP - subscriptionNewSP-DueDate (seconds set to zero) - subscriptionLNPType - subscriptionLRN (specify the LRN identified in the prerequisites) Specify a combination of valid and invalid DPC/SSN data for the following attributes. - subscriptionCLASS-DPC - subscriptionCLASS-SSN - subscriptionLIDB-DPC - subscriptionLIDB-SSN - subscriptionCNAM-DPC - subscriptionCNAM-SSN - subscriptionISVM-DPC - subscriptionISVM-SSN - subscriptionWSMSC-DPC - if supported by the Service provider SOA - subscriptionWSMSC-SSN - if supported by the Service Provider SOA The following attributes are optional: - subscriptionEndUser LocationValue - subscriptionEndUser LocationType - subscriptionBillingID |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the Request from the Service Provider’s SOA and determines the following: The LATA ID for the TN does not match the LATA ID for the LRN. (This violates system requirements.) |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-ACTION Response failure indicating an error with the request to the SOA. |  | SP |  | The Service Provider SOA receives the M-ACTION Response. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a query for the Subscription Version. |  | NPAC |  | NPAC Personnel verify that the Subscription Version does not exist on the NPAC SMS. |  |  |  |
| 4. | SP | Service Provider Personnel, perform a local query for the Subscription Version. |  | SP |  | Verify that the Subscription Version does not exist on the local database. |  |  |  |
| E. | Pass/Fail Analysis, NANC 319-1 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 319–2 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | SOA – Service Provider Personnel attempt to modify a ‘Pending’, Subscription Version specifying an LRN with a different LATA Id from the NPA-NXX of the TN in the Subscription Version. - Failure |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 319 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0 |  | Relevant Requirement(s): |  | RR5-123 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0 |  | Relevant Flow(s): |  | B.5.2.3, B.5.2.4 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Verify that the NPA-NXX exists and is open for porting for the TN that is going to be used during this test case. TN__________________ 2. Verify that a ‘Pending’ Subscription Version exists for the TN that is going to be used during this test case. Note the current LRN for the Subscription Version _____________. 3. Identify another LRN that exists for the Service Provider under test which has a different LATA ID from the TN of the Subscription Version used during this test case ______________. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | Using their SOA system, Service Provider Personnel submit a request to the NPAC SMS to modify a single TN, ‘Pending’ Subscription Version that already exists on the NPAC SMS. Specify the TN and the new LRN identified in the prerequisites above. The request must specify the TN and the version status or the version ID of the Subscription Version to be modified and the data to be modified. The Service Provider SOA submits an M-ACTION Request subscriptionVersionModify or an M-SET Request subscriptionVersionNPAC (depending on the system implementation) to the NPAC SMS lnpSubscription object to update the ‘Pending’ Subscription Version. |  | NPAC |  | The NPAC SMS receives the M-ACTION/M-SET Request from the Service Provider’s SOA and determines the following: The LATA ID for the TN of the Subscription Version and the LATA ID for the modified LRN value do not match. (This violates system requirements.) |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-ACTION Response failure or M-SET Response failure (depending on the message received in Row 1) indicating an error with the request to the SOA. |  | SP |  | The Service Provider SOA receives the Response from the NPAC SMS. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a query for the Subscription Version. |  | NPAC |  | NPAC Personnel verify that the Subscription Version exists with a status of ‘Pending’ however, the attributes were not modified. The original LRN identified in the prerequisites above is still associated with the TN/Subscription Version used during this test case. |  |  |  |
| 4. | SP | Service Provider Personnel, perform a local query for the Subscription Version. |  | SP |  | Verify that the Subscription Version exists on the local database with the original attribute values. The original LRN identified in the prerequisites above is still associated with the TN/Subscription Version used during this test case. |  |  |  |
| E. | Pass/Fail Analysis, NANC 319-2 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 319–3 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | NPAC OP GUI – NPAC Personnel submit a mass update request for a range of ‘Active’, Subscription Versions where some of the Subscription Versions exist with valid LATA ID relationships and some of the Subscription Versions exist with invalid LATA ID relationships. Specify new DPC/SSN data. Subscription Versions with valid LATA ID relationships will be updated and Subscription Versions that exist without valid LATA ID relationships will not be updated. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 319 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0 |  | Relevant Requirement(s): |  | RR3-254 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0 |  | Relevant Flow(s): |  | B.8.3 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Activate a contiguous range of at least 100 SVs for the Service Provider under test where the LATA ID of the NPA-NXX for the range of SVs is THE SAME AS the LATA ID of the LRN specified in the subscription versions. SV Range A________________________. - For example, LRN=303-555-0000 has a LATA ID of 656 and is owned by SP under test. - Activate SVs (303-100-1000 through 303-100-1100) with LRN (303-555-0000), NPA-NXX 303-100 has a LATA ID of 656 2. Activate another contiguous range of at least 100 SVs for the Service Provider under test where the LATA ID of the NPA-NXX for the range of SVs is DIFFERENT THAN the LATA ID of the LRN specified in the subscription versions (pre 3.2 valid data). SV Range B __________________________________. - For example, LRN=303-888-0000 has a LATA ID of 658 and is owned by SP under test. - Activate SVs (303-100-2000 through 303-100-2100) with LRN (303-888-0000), NPA-NXX 303-100 has a LATA ID of 656 |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel submit a Mass Update request for a range of ‘Active’ Subscription Versions (specify SV Range A and SV Range B identified in the prerequisites above). Modify the at least one set of DPC/SSN data for these Subscription Versions.. |  | NPAC |  | The NPAC SMS searches the Subscription Version database for Subscription Versions that match the input Mass Update criteria. The NPAC SMS determines that a subset of the TNs in the Subscription Version range have a LATA ID different from the LATA ID of the associated LRN attribute. The NPAC SMS makes an entry to the Mass Update Exception Report for these TNs that contain NPA-NXXs where the LATA ID does not match the LATA ID of the associated LRN, and continues updating the remaining Subscription Versions that meet the Mass Update criteria. |  |  |  |
| 2. | NPAC | NPAC SMS sends multiple M-SET(s) for each contiguous range of Subscription Versions to all LSMSs that are accepting downloads for the NPA-NXXs of the Subscription Versions to update the DPC/SSN value for those Subscription Versions whose LATA IDs of the NPA-NXXs matches the LATA ID of the associated LRN. |  | SP |  | All LSMSs that are accepting downloads for the NPA-NXXs of the Subscription Versions being updated, receive the M-SET request(s) from the NPAC SMS to modify the DPC/SSN data. The LSMSs issue an M-SET Response(s) indicating they successfully processed the NPAC SMS request(s). These M-SET Requests are for the Subscription Versions that were successfully updated based on the Mass Update criteria – and does not include those Subscription Versions that could not be updated based on invalid LATA ID relationships. The Subscription Versions that could not be updated are included on the Mass Update Exception report. |  |  |  |
| 3. | NPAC | 1. If the current Service Provider’s TN Range Notification Indicator is set to TRUE, NPAC SMS issues an M-EVENT-REPORT subscriptionVersionRangeStatusAttributeValueChange for the range of Subscription Versions that were updated indicating the status is now ‘Active’. 2. If the current Service Provider’s TN Range Notification Indicator is set to ‘FALSE’, NPAC SMS issues a subscriptionVersionStatusAttributeValueChange for each Subscription Version that was updated, indicating the status is now ‘Active’. |  | SP |  | The current Service Provider receives the M-EVENT-REPORT from the NPAC SMS and issues an M-EVENT-REPORT response indicating it successfully received the message. |  |  |  |
| 4. | NPAC | NPAC Personnel generate a Mass Update Exception report. |  | NPAC |  | Verify that the subset of Subscription Versions within the Mass Update request who’s LATA ID for the NPA-NXX of the TN that did not match the LATA ID for the associated LRN attribute are included on the report. |  |  |  |
| 5. | SP | Service Provider Personnel perform a local query on their LSMS to verify the Mass Update was completed. |  | SP |  | On the LSMS verify: 1. The subset of Subscription Versions within the Mass Update request who’s LATA ID for the NPA-NXX of the TN that did not match the LATA ID for the associated LRN attribute were not updated with the new DPC/SSN value(s). 2. The subset of Subscription Versions within the Mass Update request who’s LATA ID for the NPA-NXX of the TNs did match the LATA ID for the associated LRN attribute were updated with the new DPC/SSN value. |  |  |  |
| 6. | NPAC | NPAC Personnel perform a full audit for the subscription version range specified in the Mass Update request. |  | NPAC |  | Verify that there are no discrepancies found. 1. The subscription versions who’s LATA ID for the NPA-NXX of the TNs did not match the LATA ID for the associated LRN attribute were not updated – they still exist in their previous state. 2. All other subscription versions specified in the Mass Update criteria were updated appropriately. |  |  |  |
| E. | Pass/Fail Analysis, NANC 319-3 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel were able to view the Mass Update Exception report that included the TNs that were not updated because the LATA ID of their NPA-NXX does not match the LATA ID for the associated LRN attribute. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 319–4 |  | SUT Priority: |  | SOA |  | Conditional |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | SOA – Service Provider Personnel attempt to create a Number Pool Block specifying an LRN with a different LATA Id than the TNs in the Number Pool Block. - Failure |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 319 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0 |  | Relevant Requirement(s): |  | RR3-334 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0 |  | Relevant Flow(s): |  | B.4.4.1 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Identify the Number Pool Block that will be used during this test case (NPA-NXX-X _________________. 2. Verify that the NPA-NXX exists and is open for porting for the Number Pool Block that is going to be used during this test case. 3. Verify that the NPA-NXX-X exists respective to the Number Pool Block that is going to be used during this test case. 4. Verify that there are no contaminated TNs or ‘Pending-Like’ Subscription Versions for the range of TNs in the NPA-NXX-X. 5. Identify an LRN that already exists on the NPAC SMS for the Service Provider under test which has a LATA ID different than the Number Pool Block identified above to be used during this test case ___________________. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | Using the SOA, Service Provider Personnel, submit a M-ACTION numberPoolBlock-Create Request to the NPAC SMS to create a Number Pool Block. The request must include the following attributes: numberPoolBlockNPA-NXX-X (specify the NPA-NXX-X identified in the prerequisites) numberPoolBlockSPID numberPoolBlockLRN (specify the LRN identified in the prerequisites) |  | NPAC |  | The NPAC SMS receives the M-ACTION numberPoolBlock-Create Request from the Service Provider’s SOA and determines the following: The LATA ID for the NPA-NXX-X specified in the request and the LATA ID for the LRN specified in the request do not match. (This violates system requirements.) |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-ACTION Response failure indicating an error with the request to the SOA. |  | SP |  | The Service Provider SOA receives the M-ACTION Response. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a query for the Number Pool Block and respective ‘Pooled’ Subscription Versions Service Provider personnel attempted to schedule during this test case. |  | NPAC |  | NPAC Personnel verify that the Number Pool Block and respective ‘Pooled’ Subscription Versions do not exist on the NPAC SMS. |  |  |  |
| 4. | SP | Service Provider Personnel, perform a local query for the Number Pool Block and the respective ‘Pooled’ Subscription Versions they attempted to schedule during this test case. |  | SP |  | Verify that the Number Pool Block and the respective ‘Pooled’ Subscription Versions do not exist on the local database. |  |  |  |
| E. | Pass/Fail Analysis, NANC 319-4 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 319–5 |  | SUT Priority: |  | SOA |  | Conditional |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | SOA – Service Provider Personnel attempt to modify a Number Pool Block specifying an LRN with a different LATA ID than the TNs in the Number Pool Block. - Failure |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 319 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0 |  | Relevant Requirement(s): |  | RR3-335 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0 |  | Relevant Flow(s): |  | B.4.4.13 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Identify a Number Pool Block that already exists (with a status of ‘Active’ and an empty Failed SP List) on the NPAC SMS for the Service Provider under test to be used during this test case ____________________________. 2. Identify an LRN that already exists on the NPAC SMS for the Service Provider under test that has a LATA ID different from the Number Pool Block that has been identified to be used during this test case _______________________. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | Using the SOA, Service Provider Personnel submit an M-SET Request numberPoolBlock to modify a Number Pool Block. Modify the numberPoolBlockLRN (specify the LRN identified in the prerequisites) |  | NPAC |  | The NPAC SMS receives the M-SET Request numberPoolBlock from the Service Provider’s SOA and determines the following: The LATA ID of the Number Pool Block specified in the modify request and the LATA ID of the LRN to be modified to, do not match. (This violates system requirements.) |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-ACTION Response failure indicating an error with the request to the SOA. |  | SP |  | The Service Provider SOA receives the M-ACTION Response. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a query for the Number Pool Block and respective ‘Pooled’ Subscription Versions Service Provider personnel attempted to modify during this test case. |  | NPAC |  | NPAC Personnel verify that the Number Pool Block and respective ‘Pooled’ Subscription Versions were not modified on the NPAC SMS. The original LRN is still associated with the Number Pool Block. |  |  |  |
| 4. | SP | Service Provider Personnel, perform a local query for the Number Pool Block and the respective ‘Pooled’ Subscription Versions they attempted to modify during this test case. |  | SP |  | Verify that the Number Pool Block and the respective ‘Pooled’ Subscription Versions were not modified on the local database. The original LRN is still associated with the Number Pool Block. |  |  |  |
| E. | Pass/Fail Analysis, NANC 319-5 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 319–6 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | NPAC OP GUI – NPAC Personnel submit a mass update request including at least three complete Number Pool Blocks where two of the Number Pool Blocks exist with valid LATA ID relationships and one Number Pool Block exists with invalid LATA ID relationships. Specify new DPC/SSN data. Number Pool Blocks with valid LATA ID relationships will be updated and the Number Pool Block that exists with invalid LATA ID relationships will not be updated. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 319 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0 |  | Relevant Requirement(s): |  | RR3-254 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0 |  | Relevant Flow(s): |  | B.8.3 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. Activate two NPBs for the Service Provider under test where the LATA ID of the NPA-NXX for the TNs within the block is the SAME AS the LATA ID of the LRN(s) specified in the NPBs. NPB A __________________________, NPB B _______________________. - For example, LRN=303-333-0000 has a LATA ID of 656 and is owned by SP under test. - Activate NPB A (303-100-3) with LRN (303-333-0000), and Activate NPB B (303-100-7) with LRN (303-333-0000). NPA-NXX 303-100 has a LATA ID of 656. 2. Activate another NPB for the Service Provider under test where the LATA ID of the NPA-NXX for the TNs within the block is DIFFERENT THAN the LATA ID of the LRN specified in the NPB (pre 3.2 valid data). NPB C_________________________________. - For example, LRN=303-555-0000 has a LATA ID of 658 and is owned by SP under test. - Activate NPB C (303-100-4) with LRN (303-888-0000), NPA-NXX 303-100 has a LATA ID of 656 |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel submit a Mass Update request for a range TNs that includes three Number Pool Blocks (identified in the prerequisites above). Modify at least one set of DPC/SSN data for these Number Pool Blocks |  | NPAC |  | The NPAC SMS searches the Subscription Version database for Subscription Versions that match the input Mass Update criteria. The NPAC SMS determines that the request completely includes three Number Pool Blocks and the range of TNs for one of the Number Pool Blocks have a LATA ID different from the LATA ID of the associated LRN attribute. The NPAC SMS makes an entry to the Mass Update Exception Report for these TNs/Number Pool Block where the LATA ID for the associated LRN does not match the LATA ID of the respective NPA-NXX, and continues updating the remaining Subscription Versions that meet the Mass Update criteria. |  |  |  |
| 2. | NPAC | NPAC SMS sends to all LSMSs that are accepting downloads for the NPA-NXX(s): - to those EDR LSMSs, NPAC SMS issues M-SET Request(s) numberPoolBlock to update the DPC/SSN attribute(s) for the two Number Pool Blocks who’s LATA ID for the respective NPA-NXX matches the LATA ID for the associated LRN attribute value – to update the DPC/SSN value(s). - to those EDR LSMSs, NPAC SMS issues M-SET Request(s) subscriptionVersion for each contiguous range of non-pooled TN’s within the Mass Update TN range who’s LATA ID for the respective NPA-NXX match the associated LRN attribute value – to update the DPC/SSN value(s). - to those non-EDR LSMSs, NPAC SMS issues M-SET Request(s) subscriptionVersion for each contiguous range of ‘Pooled’ and ‘non-Pooled’ SVs who’s LATA ID or the respective NPA-NXX matches the LATA ID of the associated LRN attribute – to update the DPC/SSN value(s). |  | SP |  | All LSMSs that are accepting downloads for the NPA-NXX of the Number Pool Block objects and Subscription Versions being updated, receive the M-SET Request(s) from the NPAC SMS to modify the DPC/SSN value(s). The LSMSs issue an M-SET Response indicating they successfully processed the NPAC SMS request. These M-SET Requests are for the Number Pool Blocks and Subscription Versions that were successfully updated based on the Mass Update criteria – and does not include those Number Pool Blocks or Subscription Versions that could not be updated based because the LATA ID of the respective NPA-NXX did not match the LATA ID of the associated LRN.. The Number Pool Blocks and Subscription Versions that could not be updated are included on the Mass update Exception report. |  |  |  |
| 3. | NPAC | 1. If the current Service Provider’s TN Range Notification Indicator is set to TRUE, NPAC SMS issues an M-EVENT-REPORT subscriptionVersionRangeStatusAttributeValueChange for the range of Subscription Versions not of LNP Type = ‘POOL’ out of the range of Subscription Versions that were updated indicating the status is now ‘Active’. 2. If the current Service Provider’s TN Range Notification Indicator is set to ‘FALSE’, NPAC SMS issues a subscriptionVersionStatusAttributeValueChange for each Subscription Version not of LNP Type = ‘POOL’ out of the range of Subscription Versions that were updated, indicating the status is now ‘Active’. 3 If the numberPoolBlockSOA-Origination indicator is set to ‘TRUE’ for the Number Pool Block(s) updated, NPAC SMS issues an M-EVENT-REPORT numberPoolBlockStatusAttributeValueChange to the Block Holder SOA for the Number Pool Block objects that were updated indicating the status is ‘Active’. |  | SP |  | The current/Block Holder Service Provider receives the M-EVENT-REPORT(s) from the NPAC SMS and issues an M-EVENT-REPORT response indicating it successfully received the message. |  |  |  |
| 4. | NPAC | NPAC Personnel generate a Mass Update Exception report. |  | NPAC |  | Verify that the subset of Number Pool Blocks/Subscription Versions within the Mass Update request who’s LATA ID for the respective NPA-NXX that did not match the LATA ID for the associated LRN attribute are included on the report. |  |  |  |
| 5. | SP | Service Provider Personnel perform a local query on their LSMS to verify the Mass Update was completed. |  | SP |  | On the LSMS verify: 1. The subset Subscription Versions (both Pooled and non-Pooled) within the Mass Update request who’s LATA ID for the respective NPA-NXX that did not match the LATA ID for the associated LRN attribute were not updated with the new DPC/SSN value(s). 2. The subset of Subscription Versions (both Pooled and non-Pooled) within the Mass Update request who’s LATA ID for the respective NPA-NXX did match the LATA ID for the associated LRN attribute were updated with the new DPC/SSN value(s). 3. EDR LSMSs only, the Number Pool Blocks within the Mass Update request who’s LATA ID for the respective NPA-NXX that did not match the LATA ID for the associated LRN attribute was not updated with the new DPC/SSN value(s). 4. EDR LSMSs only, the Number Pool Blocks within the Mass Update request who’s LATA ID for the respective NPA-NXX that did match the LATA ID for the associated LRN attribute was updated with the new DPC/SSN value(s). |  |  |  |
| E. | Pass/Fail Analysis, NANC 319-6 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel were able to view the Mass Update Exception report that included the TNs/Number Pool Blocks that were not updated because the LATA ID of their respective NPA-NXX does not match the LATA ID for the associated LRN attribute. |  |  |  |  |  |  |  |

# NANC 322 – Clean Up of Failed SP List Based on Service Provider BDD Response File

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 322-1 |  | SUT Priority: |  | SOA |  | N/A |
|  |  |  |  |  |  |  | LSMS |  | Conditional |
|  | Objective: |  | LSMS – Service Provider Personnel create a Bulk Data Download Response File for Subscription Version data. NPAC Personnel process the Bulk Data Download Response File. The Service Provider was previously on the Failed SP List for at least some of the Subscription Versions in the respective file. Verification steps are performed to ensure the Service Provider’s LSMS is now in synch with the NPAC SMS. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 322 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0 |  | Relevant Requirement(s): |  | RR3-330, RR3-329, RR3-332, RR3-333, RR3-325, RR3-326 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0 |  | Relevant Flow(s): |  | N/A |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  | NANC 169-1 |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | While the LSMS is ‘dis-associated’ from the NPAC SMS, NPAC personnel perform the following functions: a) Modify a (unique) range of 500, ‘Active’ Subscription Versions where the Service Provider under test is the Current Service Provider. . Use simulators that are not associated with the NPAC and are receiving downloads for this NPA-NXX. Verify these Subscription Versions exist with a status of ‘Active’ and a Failed SP List. (SV group 2a______________) b) Create a filter for the NPA-NXX for which you created 500, ‘Pending’ Subscription Versions in 1a) above. - Activate these 500, ‘Pending’ Subscription Versions. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify that the status for all 500 is ‘Active’ on the NPAC SMS. (SV group 2b______________________) - Disconnect 250 of these now, ‘Active’ Subscription Versions specifying Effective Release and Customer Disconnect dates in the future. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify that the status of these 250 Subscription Versions is ‘Disconnect-Pending’. (SV group 2b1___________) - Remove the filter for this NPA-NXX for the Service Provider under test so that this range of Subscription Versions will be included in the Bulk Data Download File. c) Create and Activate 100 Intra-Service Provider Subscription Versions using an NPA-NXX that is open for porting and for which the Service Provider under test is accepting downloads for this NPA-NXX. This Service Provider is neither the Old or New Service Provider for these Subscription Versions. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify that the Subscription Versions have a status of ‘Partial-Fail’. (SV group 2c ___________________) d) Activate 50 Subscription Versions with a status of ‘Pending’. The Service Provider under test is the New Service Provider for these Subscription Versions. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify that these Subscription Versions have a status of ‘Partial-Fail’. (SV group 2d_________________) . e) Create and concur to a range of 100, ‘Pending’ Subscription Versions where the Service Provider under test is the New Service Provider. (SV group e__________________) f) Put simulated SPID LSMS in recovery. Use at least one simulator that is associated with the NPAC and is accepting downloads for this NPA-NXX. Verify that the Service Provider under test is accepting downloads for this NPA-NXX. Activate 50 ‘pending’ SVs in group f above. Verify that these subscription versions have a status of ‘sending’. (SV group f__________________) During the test case retry timers will exhaust, and then the status of the SVs should be ‘Partial-Fail’. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel request a Bulk Data Download for Subscription Data, specifying Active/Disconnect Pending/Partial Failure Subscription Versions Only and specifying the TN range identified in the prerequisites above, for the Service Provider under test. |  | NPAC |  | The NPAC SMS performs the request, generates the appropriate Bulk Data Download File and automatically “FTP’s” the file to the Service Provider’s directory on the NPAC SMS. |  |  |  |
| 2. | SP | Service Provider Personnel receive the Bulk Data Download File and load the file into their LSMS. |  | SP |  | The LSMS successfully processes the Bulk Data Download file and reflects the updates described in the prerequisites above. The systems are still ‘dis-associated’ from the NPAC SMS. |  |  |  |
| 3. | SP | Service Provider Personnel, using their LSMS, perform a local query for the Subscription Data to verify that the Subscription Version data was loaded. - SV group a - SV group b - SV group b1 - SV group c - SV group d - SV group f |  | SP |  | Using the LSMS system, verify: - SV group a exists on the LSMS. Verify that all of them reflect the ‘modified’ SV values from the prerequisites above. - SV group b exists on the LSMS. - SV group b1 exists on the LSMS. - SV group c exists on the LSMS. - SV group d exists on the LSMS. - SV group f exists on the LSMS. |  |  |  |
| 4. | SP | Service Provider personnel/system generate a Bulk Data Download Response File for the original Bulk Data Download file and places it in the Service Provider’s directory on the NPAC SMS. |  | NPAC |  | NPAC SMS processes the Bulk Data Download Response File for the original file, and updates the Subscription Versions appropriately. |  |  |  |
| 5. | NPAC | NPAC Personnel query for the Subscription Version data included in the Bulk Data Download File: - SV group a - SV group b - SV group b1 - SV group c - SV group d - SV group f |  | NPAC |  | Verify the following: - SV group a exists with a status of ‘Active’ and an empty Failed SP List. - SV group b exists with a status of ‘Active’ and an empty Failed SP List. - SV group b1 exists with a status of ‘Disconnect-Pending’ and an empty Failed SP List. - SV group c exists with a status of ‘Active’ and an empty Failed SP List. - SV group d exists with a status of ‘Active’ and an empty Failed SP List. - SV group f exists with a status of ‘Sending’. |  |  |  |
| 6. | SP | After all NPAC ‘retry timers’ for the Subscription Versions specified in the prerequisites above have expired, Service Provider personnel perform appropriate steps to ‘associate’ with the NPAC SMS such that they will not recover additional information. |  | SP |  | The LSMS successfully re-associates with the NPAC SMS without recovering additional information |  |  |  |
| 7. | NPAC | NPAC Personnel bring the simulated SPID LSMS that was in recovery in Prerequisite step f above, out of recovery. |  | NPAC |  | Verify that the simulated SPID that was in recovery during step g of the prerequisites is now out of recovery. Verify that the 50 subscription versions that were activated while this SPID was in recovery now have a status of ‘Partial Fail’. |  |  |  |
| 8. | NPAC | NPAC Personnel perform multiple Full audits for each NPA-NXX included in the range of TNs specified in the prerequisites above for the Service Provider’s LSMS to verify that all the appropriate updates were processed from the Bulk Data Download File by the LSMS. |  | NPAC |  | Using the Audit Results Log, verify that there were no updates made. If any updates were made as a result of running this audit, this test case fails. Verify that: - SV group a exists on the LSMS. Verify that all of them reflect the ‘modified’ SV values from the prerequisites above. - SV group b exists on the LSMS. - SV group b1 exists on the LSMS. - SV group c exists on the LSMS. - SV group d exists on the LSMS. - SV group f exists on the LSMS. |  |  |  |
| E. | Pass/Fail Analysis, NANC 322-1 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel were able to successfully process the Bulk Data Download file updates with their local databases in a timely fashion. |  |  |  |  |  |  |  |
| Pass | Fail | Bulk Data Download Response File was processed in a timely fashion. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 322-2 |  | SUT Priority: |  | SOA |  | N/A |
|  |  |  |  |  |  |  | EDR LSMS |  | Conditional |
|  |  |  |  |  |  |  | Non-EDR LSMS |  | N/A |
|  | Objective: |  | LSMS – Service Provider Personnel create a Bulk Data Download Response File for Number Pool Block data. NPAC Personnel process the Bulk Data Download Response File. The Service Provider was previously on the Failed SP List for at least some of the Number Pool Blocks in the respective file. Verification steps are performed to ensure the Service Provider’s LSMS is now in synch with the NPAC SMS. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 322 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0 |  | Relevant Requirement(s): |  | RR3-330, RR3-331, RR3-332, RR3-333, RR3-325, RR3-327 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0 |  | Relevant Flow(s): |  | N/A |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | While the LSMS is ‘dis-associated’ from the NPAC SMS, NPAC personnel perform the following functions: a) Modify an existing Number Pool Block for which the Service Provider under test is accepting downloads. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify that the Number Pool Block and respective Subscription Versions have a status of ‘Partial-Fail’ and the Service Provider under test is on the Failed SP List (SV group a______________), (NPB group a ______________). b) Delete an existing NPA-NXX-X with a respective Number Pool Block for which the Service Provider under test is accepting downloads, and none of the respective ‘Pooled’ Subscription Versions have been ported away. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. (SV group 2b___________), (NPB group 2b__________________). c) Activate a Number Pool Block (associated with another NPA-NXX-X that has reached its Effective date ) for an NPA-NXX for which the Service Provider under test is NOT accepting downloads - it is another Service Provider’s Number Pool Block. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify that the Number Pool Block and respective Subscription Versions have a status of ‘Active’. (SV group c_____________________) (NPB c______________) d) Modify the Number Pool Block that was just activated in step ‘c’ above. Make sure that the Service Provider under test is now accepting downloads for this NPA-NXX. Use simulators that are associated with the NPAC and are receiving downloads for this NPA-NXX. Verify that the Number Pool Block and respective Subscription Versions have been updated and have a status of ‘Active’ with a Failed SP List. (SV group c and NPB c). |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel request a Bulk Data Download for Number Pool Block Data, specifying Latest View of Activity a valid Time Range and specifying the range identified in the prerequisites above, for the Service Provider participating in the test case. |  | NPAC |  | The NPAC SMS performs the request, generates the appropriate Bulk Data Download File(s) and automatically “FTP’s” the file(s) to the Service Provider’s directory on the NPAC SMS. |  |  |  |
| 2. | SP | Service Provider Personnel receive the Bulk Data Download File(s) and load the file(s) into their LSMS systems. |  | SP |  | The LSMS successfully processes the Bulk Data Download file(s) and reflects the updates described in the prerequisites above. The system is still ‘dis-associated’ from the NPAC SMS. |  |  |  |
| 3. | SP | Service Provider Personnel, using their LSMS, perform a local query for the Pooled Subscription and/or Number Pool Block Data to verify that the data was loaded. - Pooled SV group a - Pooled SV group b - Pooled SV group c - NPB group a - NPB group b - NPB group c |  | SP |  | Using the LSMS system, verify: - Pooled SV group a exists on the LSMS with the modified attributes specified in step a of the prerequisites above. - Pooled SV group b does not exist on the LSMS. - Pooled SV group c exists on the LSMS with the modified attributes specified in step d of the prerequisites above. - NPB group a exists on the LSMS with the modified attributes specified in step a of the prerequisites above. - NPB group b does not exist on the LSMS. - NPB group c exists on the LSMS with the modified attributes specified in step d of the prerequisites above. |  |  |  |
| 4. | SP | Service Provider personnel/system generate a Bulk Data Download Response File for the original Bulk Data Download file(s) and places it in the Service Provider’s directory on the NPAC SMS. |  | NPAC |  | NPAC SMS processes the Bulk Data Download Response File(s) for the original file, and updates the Pooled Subscription Versions/Number Pool Blocks appropriately. |  |  |  |
| 5. | NPAC | NPAC Personnel query for the Pooled Subscription Version and Number Pool Block data included in the Bulk Data Download File: - Pooled SV group a - Pooled SV group b - Pooled SV group c - NPB group a - NPB group b - NPB group c |  | NPAC |  | Verify the following: - Pooled SV group a exists with a status of ‘Active’ and an empty Failed SP List. - Pooled SV group b exists with a status of ‘Old’ and an empty Failed SP List. - Pooled SV group c exists with a status of ‘Active’ and an empty Failed SP List. - NPB group a exists with a status of ‘Active’ and an empty Failed SP List. - NPB group b exists with a status of ‘Old’ and an empty Failed SP List. - NPB group c exists with a status of ‘Active’ and an empty Failed SP List. |  |  |  |
| 6. | SP | After all NPAC ‘retry timers’ for the Subscription Versions and Number Pool Blocks specified in the prerequisites above have expired, Service Provider personnel perform appropriate steps to ‘associated’ with the NPAC SMS such that they will not recover additional information. |  | SP |  | The LSMS successfully re-associates with the NPAC SMS without recovering additional information. |  |  |  |
| 7. | NPAC | NPAC Personnel perform multiple Full audits for each NPA-NXX included in the range of TNs specified in the prerequisites above for the Service Provider’s LSMS to verify that all the appropriate updates were processed from the Bulk Data Download File by the LSMS. |  | NPAC |  | Using the Audit Results Log, verify that there were no updates made. If any updates were made as a result of running this audit, this test case fails. Verify that: - Pooled SV group a exists on the LSMS with the modified attributes specified in step a of the prerequisites above. - Pooled SV group b does not exist on the LSMS. - Pooled SV group c exists on the LSMS with the modified attributes specified in step d of the prerequisites above. - NPB group a exists on the LSMS with the modified attributes specified in step a of the prerequisites above. - NPB group b does not exist on the LSMS. - NPB group c exists on the LSMS with the modified attributes specified in step d of the prerequisites above. |  |  |  |
| E. | Pass/Fail Analysis, NANC 322-2 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel were able to successfully process the Bulk Data Download file updates with their local databases in a timely fashion. |  |  |  |  |  |  |  |
| Pass | Fail | Bulk Data Download Response File was processed in a timely fashion. |  |  |  |  |  |  |  |

# NANC 323 – Partial Migration of SPID via Mass Update Test Cases

This section of test cases shall be executed only during the group test phase due to the impact to the entire test environment.

# NANC 354 – Delta Download File Creation by Time Range for Network Data

NOTE:  SOA Network Data Management, LSMS Network Data Management, NPAC Customer SOA NPA-NXX-X Indicator and NPAC Customer LSMS NPA-NXX-X Indicator (s) are all set to production values for the Service Provider under test.

Network Data filters need not be set to execute NANC 354 test cases.

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 354-1 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | Required |
|  | Objective: |  | NPAC OP GUI – NPAC Personnel initiate a Bulk Data Download of Network Data – Specifying the Latest View of Network Data Activity and a valid time range. Verification steps are preformed to ensure the BDD file was processed successfully by the Service Provider system. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 354 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0 |  | Relevant Requirement(s): |  | RR3-220, RR3-301, RR3-302, RR3-304, RR3-311 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0 |  | Relevant Flow(s): |  | N/A |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - While the SOA and LSMS are ‘associated’ with the NPAC SMS, NPAC personnel perform the following functions: - Create an NPA-NXX that is not yet open for porting (Effective date is in the future) on behalf of the Service Provider under test( NPA-NXX 1a_____________). - Create an NPA-NXX-X respective to NPA-NXX 1a on behalf of the Service Provider under test (NPA-NXX-X 1a_________________). - Create an NPA-NXX with an Effective date equal to today, on behalf of the Service Provider under test (so that it is now open for porting) (NPA-NXX 1c_________________). - Immediately after initiating requests 1a), 1b) and 1c) above, dis-associate the Service Provider’s SOA and LSMS systems. - While the SOA and LSMS are ‘dis-associated’ with the NPAC SMS, NPAC personnel perform the following functions: - Create an NPA-NXX that is not yet open for porting (Effective date is in the future) on behalf of the Service Provider under test NPA-NXX 2a _____________. - Create an NPA-NXX-X respective to NPA-NXX 2a on behalf of the Service Provider under test (NPA-NXX-X 2a ___________________). - Modify NPA-NXX-X respective to 2a on behalf of the Service Provider under test, note the modified attributes. - Modify existing NPA-NXX-X 1a above, note the modified attributes. - Create an NPA-NXX that is not yet open for porting (Effective date is in the future) on behalf of another Service Provider under test NPA-NXX 2e __________________. - Delete the NPA-NXX (NPA-NXX 1c above) that is ‘owned’ by the Service Provider under test and is currently open for porting and for which respective Subscription Versions, Number Pool Blocks and NPA-NXX-X DO NOT exist, NPA-NXX 1c ____________________________. - Delete NPA-NXX-X 1a above. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel request a Bulk Data Download for Network Data, specifying Latest View of Network Data Activity and a valid time range for the Service Provider under test. |  | NPAC |  | The NPAC SMS performs the request, generates the appropriate Bulk Data Download File(s) and automatically “FTP’s” the file(s) to the Service Provider’s directory on the NPAC SMS. |  |  |  |
| 2. | SP | Service Provider Personnel receive the Bulk Data Download File(s) and load the file(s) into their SOA and LSMS. |  | SP |  | The SOA and LSMS successfully process the Bulk Data Download file(s) and reflect the updates described in the prerequisites above. The systems are still ‘dis-associated’ from the NPAC SMS. |  |  |  |
| 3. | SP | Service Provider Personnel, using their SOA and LSMS, perform a local query for the Network Data to verify that the NPA-NXX and NPA-NXX-X data was loaded. |  | SP |  | Verify the following on the respective systems. On the SOA verify: - If the Service Provider’s SOA Network Data Management Indicator is set to TRUE, - NPA-NXX 1a exists on the SOA (wasn’t in the BDD file). - NPA-NXX 1c does not exist on the SOA. - NPA-NXX 2a exists on the SOA. - NPA-NXX 2e exists on the SOA. - If the Service Provider’s SOA NPA-NXX-X Indicator is set to TRUE, - NPA-NXX-X 1a does not exist on the SOA (see step 2g above). - NPA-NXX-X 2a with the modified attributes (see step 2c above) exists on the SOA On the LSMS verify - If the Service Provider’s LSMS Network Data Management Indicator is set to TRUE, - NPA-NXX 1a exists on the LSMS (wasn’t in the BDD file). - NPA-NXX 1c DOES NOT exist on the LSMS. - NPA-NXX 2a exists on the LSMS. - NPA-NXX 2e exists on the LSMS. - If the Service Provider’s NPA-NXX-X Indicator is set to TRUE, - NPA-NXX-X 1a does not exist on the LSMS. (see step 2g above). - NPA-NXX-X 2a with the modified attributes (see step 2c above) exists on the LSMS. |  |  |  |
| 4. | SP | Service Provider personnel perform appropriate steps to ‘associate’ with the NPAC SMS. |  | SP |  | The SOA and LSMS successfully re-associate with the NPAC SMS. |  |  |  |
| 5. | SP | Service Provider Personnel, using their SOA and LSMS, perform an NPAC query for the Network Data in the prerequisites: On the SOA and LSMS query: - If the Service Provider’s SOA/LSMS Network Data Management Indicator is set to TRUE, - NPA-NXX 1a . - NPA-NXX 1c - NPA-NXX 2a - NPA-NXX 2e - If the Service Provider’s SOA/LSMS NPA-NXX-X Indicator is set to TRUE, - NPA-NXX-X 1a - NPA-NXX-X 2a |  | SP |  | Verify the following on the respective systems. On the SOA/LSMS verify: - If the Service Provider’s SOA/LSMS Network Data Management Indicator is set to TRUE, - NPA-NXX 1a exists. - NPA-NXX 1c does not exist. - NPA-NXX 2a exists. - NPA-NXX 2e exists. - If the Service Provider’s SOA NPA-NXX-X Indicator is set to TRUE, - NPA-NXX-X 1a does not exist. - NPA-NXX-X 2a exists with the new, modified attributes (see step 2c above). |  |  |  |
| E. | Pass/Fail Analysis, NANC 354-1 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel were able to successfully process the Bulk Data Download file updates with their local databases in a timely fashion. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 354-2 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | Required |
|  | Objective: |  | NPAC OP GUI – NPAC Personnel initiate a Bulk Data Download of Network Data – Specifying the All Network Data. Verification steps are preformed to ensure the BDD file was processed successfully by the Service Provider system. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 354 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0 |  | Relevant Requirement(s): |  | RR3-303, RR3-307, RR3-308, RR3-309, RR3-310, RR3-311 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0 |  | Relevant Flow(s): |  | N/A |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - While the SOA and LSMS are ‘associated’ with the NPAC SMS, NPAC personnel perform the following functions: - Create an LRN that does not yet exist on the NPAC SMS on behalf of the Service Provider under test( LRN 1a_____________). - Create an NPA-NXX with an Effective date equal to today, on behalf of the Service Provider under test (so that it is now open for porting) (NPA-NXX 1b_______________). - Immediately after initiating requests 1a) and 1b) above, dis-associate the Service Provider’s SOA and LSMS systems. - While the SOA and LSMS are ‘dis-associated’ with the NPAC SMS, NPAC personnel perform the following functions: - Modify LRN 1a (above), note the modified attributes. - Create an NPA-NXX-X respective to NPA-NXX 1b on behalf of the Service Provider under test (NPA-NXX-X 1b ___________________). - Modify NPA-NXX-X respective to 1b on behalf of the Service Provider under test, note the modified attributes. - Modify LRN 1a (above) again, note the modified attributes. - Create an NPA-NXX that is not yet open for porting (Effective date is in the future) on behalf of another Service Provider under test NPA-NXX 2e __________________. - Delete NPA-NXX 2e that was just created above. - Create a unique NPA-NXX that is not yet open for porting (Effective date is in the future) on behalf of another Service Provider, NPA-NXX 2g __________________. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel request a Bulk Data Download for Network Data, specifying All Network Data for the Service Provider under test. |  | NPAC |  | The NPAC SMS performs the request, generates the appropriate Bulk Data Download File(s) and automatically “FTP’s” the file(s) to the Service Provider’s directory on the NPAC SMS. |  |  |  |
| 2. | SP | Service Provider Personnel receive the Bulk Data Download File(s) and load the file(s) into their SOA and LSMS. |  | SP |  | The SOA and LSMS successfully process the Bulk Data Download file(s) and reflect the updates described in the prerequisites above. The systems are still ‘dis-associated’ from the NPAC SMS. |  |  |  |
| 3. | SP | Service Provider Personnel, using their SOA and LSMS, perform a local query for the Network Data to verify that the NPA-NXX and NPA-NXX-X data was loaded. |  | SP |  | Verify the following on the respective systems. On the SOA verify: - If the Service Provider’s SOA Network Data Management Indicator is set to TRUE, - LRN 1a exists on the SOA with the modified attributes from perquisite step 2d above. - NPA-NXX 1b exists on the SOA, this was not part of the BDD. - NPA-NXX 2e does not exist on the SOA. - NPA-NXX 2g exists on the SOA. - If the Service Provider’s SOA NPA-NXX-X Indicator is set to TRUE, - NPA-NXX-X 1b exists on the SOA with the new, modified attributes (see step 2c above). On the LSMS verify - If the Service Provider’s LSMS Network Data Management Indicator is set to TRUE, - LRN 1a exists on the LSMS with the modified attributes from prerequisite step 2d above. - NPA-NXX 1bexists on the LSMS, this was not part of the BDD. - NPA-NXX 2e does not exist on the LSMS. - NPA-NXX 2g exists on the LSMS. - If the Service Provider’s LSMS NPA-NXX-X Indicator is set to TRUE, - NPA-NXX-X 1b exists on the LSMS with the new, modified attributes (see step 2c above). |  |  |  |
| 4. | SP | Service Provider personnel perform appropriate steps to ‘associated’ with the NPAC SMS. |  | SP |  | The SOA and LSMS successfully re-associate with the NPAC SMS. |  |  |  |
| 5. | SP | Service Provider Personnel, using their SOA and LSMS, perform an NPAC query for the Network Data in the prerequisites: On the SOA and LSMS query: - If the Service Provider’s SOA/LSMS Network Data Management Indicator is set to TRUE, - LRN 1a . - NPA-NXX 1b. - NPA-NXX 2e. - NPA-NXX 2g. - If the Service Provider’s SOA/LSMS NPA-NXX-X Indicator is set to TRUE, - NPA-NXX-X 1b. |  | SP |  | Verify the following on the respective systems. On the SOA/LSMS verify: - If the Service Provider’s SOA/LSMS Network Data Management Indicator is set to TRUE, - LRN 1a exists with the modified attributes from prerequisite step 2d above. - NPA-NXX 1b exists. - NPA-NXX 2e does not exist. - NPA-NXX 2g exists. - If the Service Provider’s SOA/LSMS NPA-NXX-X Indicator is set to TRUE, - NPA-NXX-X 1b exists with the new, modified attributes, see step 2c above. |  |  |  |
| E. | Pass/Fail Analysis, NANC 354-2 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel were able to successfully process the Bulk Data Download file updates with their local databases in a timely fashion. |  |  |  |  |  |  |  |

Note:  The Bulk Data Download file may contain additional network data.  Testers can verify appropriate behavior for any additional data that may be in the file as is stated in the Test Steps above.

# Group Testing

Service Provider ID (SPID) Migration)

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 323-1 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | Required |
|  | Objective: |  | NPAC OP GUI – NPAC Personnel submit a request for a Partial SPID migration via Mass Update, where NPA-NXX, LRN, Subscription Version, NPA-NXX-X and Block Information exist for the migrating away from SPID. Verification steps are performed to ensure the Service Provider system is now in synch with the NPAC SMS. – Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 323 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0 |  | Relevant Requirement(s): |  | RR3-255, RR3-256, RR3-257, RR3-258, RR3-259, RR3-260, RR3-261, RR3-262, RR3-263, RR3-264, RR3-265, RR3-266, RR3-267, RR3-268, RR3-269, RR3-270, RR3-271, RR3-272, RR3-273, RR3-274, RR3-276, RR3-277 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0 |  | Relevant Flow(s): |  |  |  |  |

NANC 323 requires unique industry coordination unlike any other aspect of NPAC functionality.  Due to a delay in industry readiness, functional testing of NANC 323 functionality has been postponed.  These test case procedures will be updated to more appropriately support the unique testing approach required to efficiently test this functionality and will be published in coordination with the actual testing cycle.

Linked Action Replies

NOTE:  Service Provider’s Local SMS Linked Replies Indicator, Service Provider’s SOA Linked Replies Indicator, NPAC Customer LSMS NPA-NXX-X Indicator, and NPAC Customer SOA NPA-NXX-X Indicator should be set to production values for testing NANC 187 functionality.

The Network Data, Block Data, Subscription Version Data and Notification Data Linked Replies Blocking Factors and Maximum Linked Recovered Objects should be set to predetermined values identified by the test team.  These parameters may not be set to ‘production or default’ values so as to be able to complete testing in an expeditious manner.

The test data identified in the prerequisites for these test cases are recommendations to achieve the test case objectives.  Many permutations of prerequisite data may create the appropriate test scenario and thus meet the test objective.  Test Engineers should consider each Service Provider’s capabilities and tailor the test data as is appropriate to meet the test objective and execute the test case expeditiously.  For example, consider whether the Service Provider supports NPA-NXX-X’s, Ranged Notifications and Linked Action Replies.  Based on this information, you may need to perform more or less activity to meet the test case objective.

This is a recovery test case written to cover both Service Provider systems that DO and DO NOT support Linked Replies, thus, this test case will supersede Test Case 8.1 from the NPAC SMS/Service Provider Certification & Regression Test Plan.

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 187-1 |  | SUT Priority: |  | SOA |  | N/A |
|  |  |  |  |  |  |  | LSMS |  | Required |
|  | Objective: |  | LSMS – Service Provider Personnel for either an EDR or non-EDR LSMS submit a resynchronization request for Network Data, Block Data, Subscription Version Data and Notification Data by time range, over the LSMS to NPAC SMS Interface, with the Service Provider’s Local SMS Linked Replies Indicator set to their production setting. The recovery response includes a number of Network Data objects, Number Pool Block objects, Notifications and Subscription Versions less than or equal to their respective Linked Replies Blocking Factors. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 187 and NANC 297 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0 |  | Relevant Requirement(s): |  | RR6-85, RR6-93, RR6-97, RR6-98, RR6-99, RR6-100, RR6-101, RR6-102, RR6-103, RR6-90, RR6-105 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0 |  | Relevant Flow(s): |  | B.7.1.1, B.7.1.2 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Prerequisite data may be set up different depending on if this test case is being run during Individual testing versus Group Testing. For example, during Individual Testing, if the service provider under test does not support NPA-NXX-X’s, don’t perform any of the related tasks or verify related data. During Group testing, need to create prerequisite data such that you meet the test case objective. If service provider’s under test don’t support Ranged Notifications for example, you will either need to not perform the number pool block activities, or you may filter the NPA-NXX of the number pool block data for the service provider(s) that doesn’t support Range Notifications. 1. While the EDR/non-EDR LSMS is disconnected from the NPAC SMS, NPAC Personnel should perform the following functions: - Create an LRN. - Delete an LRN for a different Service Provider. - Create an NPA-NXX. - Delete an NPA-NXX for a different Service Provider. - Activate a new Number Pool Block. - DePool an existing Number Pool Block. - Create NPA-NXX-X Information for different Service Providers. - Modify NPA-NXX-X Information for different Service Providers. - Delete NPA-NXX-X Information for different Service Providers. - Create an Inter-SP Subscription Version for a Pooled TN. - Disconnect a Pooled Ported TN. - Create an Inter-SP, Port-To-Original Subscription Version for a Pooled Ported TN. - Create a Subscription Version with the NPA-NXX created above. - Issue a Scheduled Downtime Notification. - Issue an activate request for an Inter-Service Provider Subscription Version. - Issue an Activate request for a range of two Inter-Service Provider Subscription Versions, where the broadcast to the LSMSs goes to a ‘partial failure’ state. - Create and concur to a range of 100 ‘Pending’ Subscription Versions where the Service Provider under test is the New Service Provider. Put simulated SPID LSMS in recovery. Use at least one simulator that is associated with the NPAC and is accepting downloads for this NPA-NXX. Verify that the Service Provider under test is accepting downloads for this NPA-NXX. Activate 50 ‘pending’ SVs from this group. Verify that these subscription versions have a status of ‘Sending’. - Put simulated SPID LSMS in recovery. Use at least one simulator that is associated with the NPAC and is accepting downloads for this NPA-NXX. Verify that the Service Provider under test is accepting downloads for this NPA-NXX. Activate an uncontaminated Number Pool Block on behalf of another Service Provider. Verify that the Pooled SVs and NPB have a status of ‘Sending’. 2. While the EDR/non-EDR LSMS is in recovery, NPAC personnel should perform the following functions: - Create an NPA-NXX. - Activate a Subscription Version. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | The Service Provider LSMS should be ‘disassociated’ from the NPAC SMS while NPAC Personnel are performing the setup specified above. |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | The Service Provider establishes an association from their LSMS to the NPAC SMS with the resynchronization flag set to TRUE. |  | NPAC |  | The NPAC SMS receives the association bind request from the LSMS. Once the association is established, the NPAC SMS queues all current updates. |  |  |  |
| 2. | SP | The LSMS issues an M-ACTION Request lnpDownload (network data) to the NPAC SMS and specifies the time range for the resync request. |  | NPAC |  | The NPAC SMS receives the M-ACTION. 1) If the Service Provider’s Local SMS Linked Replies Indicator is set to FALSE, NPAC issues a single, normal M-ACTION Response lnpDownload message back to the LSMS with the Network Data. 2) If the Service Provider’s Local SMS Linked Replies Indicator is set to TRUE, NPAC issues a single, normal M-ACTION Response lnpDownload message back to the LSMS with the Network Data updates. The data does not exceed the Network Data Blocking factor, so there shall be only 1 normal message sent in this instance. |  |  |  |
| 3. | NPAC | As soon as the M-ACTION Request is received, NPAC Personnel issue an activate for an Intra or Inter Service Provider Subscription Version AND create a new NPA-NXX, belonging to any Service Provider. |  | NPAC |  | The NPAC SMS receives the M-ACTION Requests for the activate and NPA-NXX create. It then sends an M-ACTION response to itself for the NPA-NXX create. |  |  |  |
| 4. | NPAC | NPAC SMS issues an M-SET Request to itself to set the subscriptionVersionNPAC object (subscription version). |  | NPAC |  | NPAC SMS issues an M-SET Response to itself. The subscription version status is set to ‘sending.’ The subscriptionVersionActivationTimeStamp and subscriptionVersionModifiedTimeStamp are set. |  |  |  |
| 5. | NPAC | The NPAC SMS checks to see if the M-CREATE subscriptionVersion can be sent to the Service Provider LSMS |  | NPAC |  | The NPAC SMS does NOT send the M-CREATE subscriptionVersion to the Service Provider LSMS, since the LSMS is still in recovery mode. |  |  |  |
| 6. | NPAC | The NPAC SMS checks to see if the M-CREATE serviceProvNPA-NXX can be sent to the Service Provider LSMS |  | NPAC |  | The NPAC SMS does NOT send the M-CREATE seviceProvNPA-NXX to the Service Provider LSMS, since the LSMS is still in recovery mode. |  |  |  |
| 7. | SP | The LSMS Service Provider issues an M-ACTION Request lnpDownload (subscription data) to the NPAC SMS and specifies the start time for the resync request. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the LSMS. 1) If the Service Provider’s Local SMS Linked Replies Indicator is set to FALSE, NPAC issues a single, normal M-ACTION Response lnpDownload message back to the LSMS with the Subscription Version Data. 2) If the Service Provider’s Local SMS Linked Replies Indicator is set to TRUE, NPAC issues a single, normal M-ACTION Response lnpDownload message back to the LSMS with the Subscription Version data. The data does not exceed the Subscription Version Data Blocking factor, so there shall be only 1 normal message sent in this instance. |  |  |  |
| 8. conditional | SP | The EDR LSMS Service Provider issues an M-ACTION Request lnpDownload (number pool block data) to the NPAC SMS and specifies the resync start time. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the EDR LSMS. 1) If the Service Provider’s Local SMS Linked Replies Indicator is set to FALSE, NPAC issues single, normal M-ACTION Response lnpDownload message back to the LSMS with the number pool block updates. 2) If the Service Provider’s Local SMS Linked Replies Indicator is set to TRUE, NPAC issues a single, normal M-ACTION Response lnpDownload message back to the LSMS with the number pool block updates. The data does not exceed the Number Pool Block Data Blocking factor, so there shall be only 1 normal message sent in this instance. |  |  |  |
| 9. | SP | The LSMS Service Provider issues an M-ACTION Request lnpNotificationRecovery (notification data) to the NPAC SMS and specifies a time range. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the LSMS: 1) If the Service Provider’s Local SMS Linked Replies Indicator is set to FALSE, NPAC issues single, normal M-ACTION Response lnpDownload message back to the LSMS with the notification updates. 2) If the Service Provider’s Local SMS Linked Replies Indicator is set to TRUE, NPAC issues a single, normal M-ACTION Response lnpDownload message back to the LSMS with the notification updates. |  |  |  |
| 10. | NPAC | The NPAC SMS sends the M-EVENT-REPORT(s) to the Block Holder SOA for a number pool block with the SOA-Origination flag set to TRUE whose subscriptionFailedSP-List was just updated due to the number pool block download. The status attribute value change contains the current status and the subscriptionFailedSP-List of the number pool block object. |  | SP |  | Block Holder SOA, to which the NPAC SMS issued an M-EVENT-REPORT, issues an M-EVENT-REPORT Confirmation back to the NPAC SMS. |  |  |  |
| 11. | SP | The LSMS Service Provider issues an M-ACTION Request lnpRecoveryComplete to the NPAC SMS to set the resynchronization flag to FALSE. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the LSMS and sets the resynchronization flag to ‘off’. |  |  |  |
| 12. | NPAC | NPAC SMS issues the following messages to the LSMS for the requests made while the LSMS was in recovery: - M-CREATE Request serviceProvNPA-NXX for the NPA-NXX that was created during recovery. - M-CREATE Request subscriptionVersion for the subscription version that was activated during recovery. |  | SP |  | The service provider’s LSMS receives the requests from the NPAC SMS for the requests that occurred during recovery and issues the following responses: - M-CREATE Response serviceProvNPA-NXX for the NPA-NXX that was created during recovery, indicating the LSMS successfully received/processed the request. - M-CREATE Response subscriptionVersion for the subscription version that was activated during recovery, indicating the LSMS successfully received/processed the request. |  |  |  |
| 13. | SP | Service Provider Personnel, using the LSMS, perform a local query for the data updated in this test case. |  | SP |  | Verify that the following updates were sent: - 1 LRN create. - 1 LRN delete. - 1 NPA-NXX create. - 1 NPA-NXX delete. - 1 Number Pool Block activate. - 1 Number Pool Block depool. - 1 NPA-NXX-X create – if supported by the Service Provider LSMS. - 1 NPA-NXX-X modify – if supported by the Service Provider LSMS. - 1 NPA-NXX-X delete – if supported by the Service Provider LSMS. - 1 Pooled Ported TN disconnect. - 1 First port of NPA-NXX notification. - 1 Scheduled Downtime notification. - 1 Single subscription version activate. - 50 subscription versions that were activated. - 1 Number Pool Block activate. - 1 NPA-NXX create after recovery is complete - 1 Subscription Version activate after recovery is complete |  |  |  |
| 14. | NPAC | NPAC Personnel perform a Full audit for the Subscription Versions that were activated during this test case. |  | NPAC |  | Using the Audit Results Log, verify that there were no updates made. If any updates were made as a result of running this audit, this test case fails. |  |  |  |
| E. | Pass/Fail Analysis, NANC 187-1 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 187-2 |  | SUT Priority: |  | SOA |  | N/A |
|  |  |  |  |  |  |  | LSMS |  | Required |
|  | Objective: |  | LSMS – Service Provider Personnel for either an EDR or non-EDR LSMS submit a resynchronization request for Network Data, and Subscription Version Data by time range, over the LSMS to NPAC SMS Interface, with the Service Provider’s Local SMS Linked Replies Indicator set to their production setting. The recovery response includes a number of Network Data objects greater than the Network Data Linked Replies Blocking Factor and less than the Network Data Maximum Linked Recovered Objects as well as a number of Subscription Version objects greater than the Subscription Data Linked Replies Blocking Factor and less than the Subscription Data Maximum Linked Recovered Objects. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 187 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0 |  | Relevant Requirement(s): |  | RR6-87, RR6-88, RR6-93, RR6-103, RR6-90, RR6-95, RR6-104, RR6-96 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0 |  | Relevant Flow(s): |  | B.7.1.1, B.7.1.2 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | 1. While the EDR/non-EDR LSMS is disconnected from the NPAC SMS, NPAC Personnel should perform the following functions: a) Create 10 LRNs. (LRN group a) b) Delete 10 LRNs for a different Service Provider. (LRN group b) c) Create 20 NPA-NXXs. (NPA-NXX group c) d) Delete 10 NPA-NXXs for a different Service Provider. (NPA-NXX group d) e) Activate 40 new Blocks. (NPB group e) f) DePool 20 existing Blocks. (NPB group f) g) Create 2 NPA-NXX-Xs for different Service Provider. (Dash X group g) h) Modify an NPA-NXX-X for different Service Provider. (Dash X group h) i) Delete an NPA-NXX-X for a different Service Provider. (Dash X group i) j) Create/Activate 20 Inter-SP Subscription Version for a Pooled TN. (SV group j) k) Disconnect 25 Pooled Ported TN. (SV group k) l) Create/Activate 20 Inter-SP, Port-To-Original Subscription Version for a Pooled Ported TN. (SV group l) m) Create 50 Subscription Version with the NPA-NXX created above. (SV group m) n) Issue a Scheduled Downtime Notification. o) Issue an activate request for 20 Inter-Service Provider Subscription Version. (SV group o) p) Issue an Activate request for a range of 10 Inter-Service Provider Subscription Versions. (SV group p) 2. While the EDR/non-EDR LSMS is in recovery, NPAC personnel should perform the following functions: - Create an NPA-NXX. - Activate a Subscription Version. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | The Service Provider LSMS should be ‘disassociated’ from the NPAC SMS while NPAC Personnel are performing the setup specified above. |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | The Service Provider establishes an association from their LSMS to the NPAC SMS with the resynchronization flag set to TRUE. |  | NPAC |  | The NPAC SMS receives the association bind request from the LSMS. Once the association is established, the NPAC SMS queues all current updates. |  |  |  |
| 2. | SP | The LSMS issues an M-ACTION Request lnpDownload (network data) to the NPAC SMS and specifies the time range for the resync request. |  | NPAC |  | The NPAC SMS receives the M-ACTION. 1) If the Service Provider’s Local SMS Linked Replies Indicator is set to FALSE, NPAC issues a single M-ACTION Response lnpDownload message back to the LSMS with the Network Data updates for - LRN group a - LRN group b - NPA-NXX group c - NPA-NXX group d - Dash X group g, if supported by the Service Provider under test - Dash X group h, if supported by the Service Provider under test - Dash X group i, if supported by the Service Provider under test 2) If the Service Provider’s Local SMS Linked Replies Indicator is set to TRUE, NPAC issues multiple, linked M-ACTION replies, lnpDownload, followed by a non-linked, empty, normal response (indicating the end of the linked reply data) back to the LSMS with the Network Data updates. These messages shall be linked for groups of (50) objects – there should be 2 linked replies. |  |  |  |
| 3. | NPAC | As soon as the M-ACTION Request is received, NPAC Personnel issue an activate for an Intra or Inter Service Provider Subscription Version (SV 3) AND create a new NPA-NXX, belonging to any Service Provider. |  | NPAC |  | The NPAC SMS receives the M-ACTION Requests for the activate and NPA-NXX create. It then sends an M-ACTION response to itself for the NPA-NXX create. |  |  |  |
| 4. | NPAC | NPAC SMS issues an M-SET Request to itself to set the subscriptionVersionNPAC object (subscription version). |  | NPAC |  | NPAC SMS issues an M-SET Response to itself. The subscription version status is set to ‘sending.’ The subscriptionVersionActivationTimeStamp and subscriptionVersionModifiedTimeStamp are set. |  |  |  |
| 5. | NPAC | The NPAC SMS checks to see if the M-CREATE subscriptionVersion can be sent to the Service Provider LSMS |  | NPAC |  | The NPAC SMS does NOT send the M-CREATE subscriptionVersion to the Service Provider LSMS, since the LSMS is still in recovery mode. |  |  |  |
| 6. | NPAC | The NPAC SMS checks to see if the M-CREATE serviceProvNPA-NXX can be sent to the Service Provider LSMS |  | NPAC |  | The NPAC SMS does NOT send the M-CREATE seviceProvNPA-NXX to the Service Provider LSMS, since the LSMS is still in recovery mode. |  |  |  |
| 7. | SP | The EDR and/or non-EDR LSMS Service Provider issues an M-ACTION Request lnpDownload (subscription data) to the NPAC SMS and specifies the start time for the resync request. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the LSMS Service Provider. 1) If the Service Provider’s Local SMS Linked Replies Indicator is set to FALSE, NPAC issues a single M-ACTION Response lnpDownload messages back to the LSMS with the Subscription Version Data updates for: - If non-EDR LSMS, Pooled Subscription Versions associated with NPB group e - If non-EDR LSMS, Pooled Subscription Versions associated with NPB group f - SV group j - SV group k - SV group l - SV group o - SV group p 2) If the Service Provider’s Local SMS Linked Replies Indicator is set to TRUE, NPAC issues multiple, linked M-ACTION replies, lnpDownload, followed by a non-linked, empty, normal response (indicating the end of the linked reply data) back to the LSMS with the ‘non-pooled’ Subscription Version Data updates. This message shall be linked for groups of (50) objects – there should be at least 3 linked replies, if the Service Provider under test is non-EDR and does not support Ranged Notifications, there will be at least 43 linked replies. If the service provider under test supports Ranged Notifications, there may be fewer than 43 linked replies based on the parameter setting. |  |  |  |
| 8. conditional | SP | The EDR LSMS Service Provider issues an M-ACTION Request lnpDownload (number pool block data) to the NPAC SMS and specifies a time range. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the EDR LSMS: If the Service Provider’s Local SMS Linked Replies Indicator is set to FALSE, NPAC issues single, normal M-ACTION Response lnpDownload message back to the with the number pool block updates for: - NPB group e - NPB group f 2) If the Service Provider’s Local SMS Linked Replies Indicator is set to TRUE, NPAC issues multiple, linked M-ACTION replies, lnpDownload, followed by a non-linked, empty, normal response (indicating the end of the linked reply data) back to the EDR LSMS with the number pool block updates. These messages shall be linked for groups of (50) objects – there should be 2 linked replies. |  |  |  |
| 9. conditional | SP | The LSMS Service Provider issues an M-ACTION Request lnpNotificationRecovery (notification data) to the NPAC SMS and specifies a time range. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the LSMS: 1) If the Service Provider’s Local SMS Linked Replies Indicator is set to FALSE, NPAC issues single, normal M-ACTION Response lnpDownload message back to the LSMS with the notification updates for - Scheduled Downtime - lnpNPAC-SMS-Operational-Information 2) If the Service Provider’s Local SMS Linked Replies Indicator is set to TRUE, NPAC issues a single, normal M-ACTION Response lnpDownload message back to the LSMS with the notification updates. |  |  |  |
| 10. | SP | The LSMS Service Provider issues an M-ACTION Request lnpRecoveryComplete to the NPAC SMS to set the resynchronization flag to FALSE. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the LSMS and sets the resynchronization flag to ‘off’. |  |  |  |
| 11. | NPAC | NPAC SMS issues the following messages to the LSMS for the request made while the LSMS was in recovery: - M-CREATE Request serviceProvNPA-NXX for the NPA-NXX that was created during recovery. - M-CREATE Request subscriptionVersion for the subscription version that was activated during recovery. |  | SP |  | The service provider’s LSMS receives the requests from the NPAC SMS for the requests that occurred during recovery and issues the following responses: - M-CREATE Response serviceProvNPA-NXX for the NPA-NXX that was created during recovery, indicating the LSMS successfully received/processed the request. - M-CREATE Response subscriptionVersion for the subscription version that was activated during recovery, indicating the LSMS successfully received/processed the request. |  |  |  |
| 12. | SP | Service Provider Personnel, using the LSMS, perform a local query for the data updated in this test case. |  | SP |  | Verify that the following updates were sent: - LRN group a was created. - LRN group b was delete. - NPA-NXX group c was activated. - NPA-NXX group d was depooled. - On non-EDR LSMSs, Pooled Subscription Versions associated with NPB e were created. - On non-EDR LSMSs, Pooled Subscription Versions associated with NPB f were deleted. - NPA-NXX-X (Dash X group g) was created – if supported by the Service Provider LSMS. - NPA-NXX-X (Dash X group h) was modified – if supported by the Service Provider LSMS. - NPA-NXX-X (Dash X group i) was deleted – if supported by the Service Provider LSMS. - SV group j was created/activated. - SV group k was disconnected. - SV group l was created/activated. - First port of NPA-NXX notification associated with SV group m was sent. - Scheduled Downtime notification was sent. - SV group o was activated. - SV group p was activated. - 1 NPA-NXX create after recovery is complete - SV3 was activated after recovery is complete. |  |  |  |
| 13. | NPAC | NPAC Personnel perform a Full audit for the Subscription Versions that were activated during this test case. |  | NPAC |  | Using the Audit Results Log, verify that there were no updates made. If any updates were made as a result of running this audit, this test case fails. |  |  |  |
| E. | Pass/Fail Analysis, NANC 187-2 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

This is a recovery test case written to cover both Service Provider systems that DO and DO NOT support Linked Replies, thus, this test case will supersede TC 8.3 from the NPAC SMS/Service Provider Certification & Regression Test Plan

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | NANC 187-4 |  | SUT Priority: |  | SOA |  | Required |
|  |  |  |  |  |  |  | LSMS |  | N/A |
|  | Objective: |  | SOA – Service Provider Personnel submit a resynchronization request for Network Data and Notification Data by time range, over the SOA to NPAC SMS Interface, with the Service Provider’s SOA Linked Replies Indicator set to their production setting. The recovery response includes a number of Network Data objects less than or equal to the Network Data Linked Replies Blocking Factor and a number of Notifications less than or equal to the Notification Data Linked Replies Blocking Factor. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  | NANC 187 |  |  |
|  | NANC FRS Version Number: |  | 3.2.0 |  | Relevant Requirement(s): |  | RR6-92, RR6-89 |  |  |
|  | NANC IIS Version Number: |  | 3.2.0 |  | Relevant Flow(s): |  | B.7.2 |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Prerequisite data may be set up different depending on if this test case is being run during Individual testing versus Group Testing. For example, during Individual Testing, if the service provider under test does not support NPA-NXX-X’s, don’t perform any of the related tasks or verify related data. 1) While the SOA is disconnected from the NPAC SMS, NPAC Personnel should perform the following functions for data within the time range to be resynced: - Create an LRN. - Delete an LRN for a different Service Provider. - Create an NPA-NXX. - Delete an NPA-NXX for a different Service Provider. - Create NPA-NXX-X Information for different Service Providers. - Modify NPA-NXX-X Information for different Service Providers. - Delete NPA-NXX-X Information for different Service Providers. - Activate a Block on behalf of the Service Provider that is ‘down’ with SOA Origination TRUE. - Create a Subscription Version with the NPA-NXX created above on behalf of the Old Service Provider and where the Service Provider Under Test is the New Service Provider; let the Initial and Final Concurrence timers expire. - Issue a Scheduled Downtime Notification. - Issue an immediate disconnect for a subscription version where the Service Provider Under Test is the Donor Service Provider. - Issue a Cancel request for a pending Inter-Service Provider Subscription Version for which both service providers have concurred to the pending port, on behalf of the Service Provider Under Test, let the Cancellation Initial Concurrence Timer expire. - Issue a Create request for a range of two pending subscription versions that were initially created by the New Service Provider, on behalf of the Old Service Provider, where the Authorization Flag is set to “False” and provide a Cause Code. - Issue an activate request for an Inter-Service Provider Subscription Version on behalf of the Service Provider Under Test. - Issue an Activate request for a range of two Inter-Service Provider Subscription Versions where a broadcast to the LSMSs goes into a Partial Failure status. 2) While the SOA is in recovery, NPAC personnel should perform the following functions: - Create an NPA-NXX. - Activate a Subscription Version as the Service Provider Under Test. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | The service provider SOA should be ‘disassociated’ from the NPAC SMS while NPAC Personnel are performing the setup specified above |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | The Service Provider establishes an association from their SOA to the NPAC SMS with the resynchronization flag set to TRUE. |  | NPAC |  | The NPAC SMS receives the association bind request from the SOA. Once the association is established, the NPAC SMS queues all current updates. |  |  |  |
| 2. | SP | The SOA issues an M-ACTION Request lnpDownload (network data) to the NPAC SMS and specifies the time range for the resync request. |  | NPAC |  | The NPAC SMS receives the M-ACTION. 1) If the Service Provider’s SOA Linked Replies Indicator is set to FALSE, NPAC issues a single, normal M-ACTION Response lnpDownload messages back to the SOA with the Network Data. 2) If the Service Provider’s SOA Linked Replies Indicator is set to TRUE, NPAC issues a single, normal M-ACTION Response lnpDownload message back to the SOA with the Network Data updates. The data does not exceed the Network Data Blocking factor, so there shall be only 1 message sent in this instance. |  |  |  |
| 3. | NPAC | As soon as the M-ACTION Request is received, NPAC Personnel issue a create for an NPA-NXX. |  | NPAC |  | The NPAC SMS receives the M-CREATE Request serviceProvNPA-NXX. |  |  |  |
| 4. | NPAC | The NPAC SMS checks to see if the M-CREATE servProvNPA-NXX can be sent to the SOA in recovery. |  | NPAC |  | The NPAC SMS does NOT issue the M-CREATE servProvNPA-NXX to the SOA since the SOA is still in recovery. |  |  |  |
| 5. | NPAC | NPAC Personnel issue an SV activate request. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request. The NPAC SMS issues an M-SET Request to itself and sets the SVs status to ‘sending.’ The NPAC SMS issues an M-SET Response to itself. |  |  |  |
| 6. | NPAC | The NPAC SMS checks to see if the M-ACTION subscriptionVersionActivate can be sent to the SOA in recovery. |  | NPAC |  | The NPAC SMS does NOT issue the M-ACTION subscriptionVersionActivate to the SOA since the SOA is still in recovery. |  |  |  |
| 7. | NPAC | The NPAC SMS checks to see if the M-EVENT-REPORT objectCreation can be sent to the SOA in recovery. |  | NPAC |  | The NPAC SMS does NOT issue the M-EVENT-REPORT objectCreation to the SOA since the SOA is still in recovery. |  |  |  |
| 8. | SP | The SOA Service Provider issues an M-ACTION Request lnpNotificationRecovery (notification data) to the NPAC SMS and specifies a time range. |  | NPAC |  | The NPAC SMS receives the M-ACTION Request from the SOA. 1) If the Service Provider’s SOA Linked Replies Indicator is set to FALSE, NPAC issues a single, normal M-ACTION Response lnpDownload messages back to the SOA with the Notification updates. 2) If the Service Provider’s SOA Linked Replies Indicator is set to TRUE, NPAC issues a single, normal M-ACTION Response lnpDownload message back to the SOA with Notification updates. The data does not exceed the Notification Data Blocking factor, so there shall be only 1 message sent in this instance. |  |  |  |
| 9. | SP | The SOA Service Provider issues an M-ACTION Request lnpRecovery to the NPAC SMS to set the resynchronization flag to FALSE. |  | NPAC |  | - The NPAC SMS receives the M-ACTION Request from the SOA and sets the resynchronization flag to ‘off’. |  |  |  |
| 10. | NPAC | NPAC SMS issues the following messages to the SOA for the request made while the SOA was in recovery: - M-CREATE Request serviceProvNPA-NXX for the NPA-NXX that was created during recovery. - The NPAC SMS will issue, depending upon the new service provider’s TN Range Notification Indicator, a subscriptionVersionStatusAttributeValueChange or subscriptionVersionRangeStatusAttributeValueChange M-EVENT-REPORT notifications to the new service provider SOA of the status change using an M-EVENT-REPORT subscriptionVersionStatusAttributeValueChange |  | SP |  | The service provider’s SOA receives the requests from the NPAC SMS for the requests that occurred during recovery and issues the following responses: - M-CREATE Response serviceProvNPA-NXX for the NPA-NXX that was created during recovery, indicating the SOA successfully received/processed the request. - M-EVENT-REPORT Confirmation for the subscription version that NPAC personnel activated on behalf of the service provider during recovery, indicating the SOA successfully received the M-EVENT-REPORT. |  |  |  |
| 11. | SP | The SOA receives the M-ACTION Response from the NPAC SMS with the data updates since the association was re-established. |  |  |  |  |  |  |  |
| 12. | NPAC | NPAC Personnel verify the data was sent in the action response. |  | NPAC |  | Verify that the appropriate data was sent. |  |  |  |
| 13. | SP | Service Provider Personnel, using the SOA, perform a local query for the actions taken in this test case. |  | SP |  | Verify that the following updates were made: - 1 LRN create. - 1 LRN delete. - 1 NPA-NXX create. - 1 NPA-NXX delete. - 1 NPA-NXX-X create – if supported by the Service Provider SOA. - 1 NPA-NXX-X modify – if supported by the Service Provider SOA. - 1 NPA-NXX-X delete – if supported by the Service Provider SOA. - 1 First port of NPA-NXX notification. - 1 Scheduled Downtime notification. - 1 NPA-NXX create after recovery is complete - 1 Subscription Version activate after recovery is complete |  |  |  |
| E. | Pass/Fail Analysis, NANC 187-4 |  |  |  |  |  |  |  |  |
| Pass | Fail | NPAC Personnel performed the test case as written. |  |  |  |  |  |  |  |
| Pass | Fail | Service Provider Personnel performed the test case as written. |  |  |  |  |  |  |  |

NANC 187-5 has been removed from Group (phase) Testing.  It will be tested by individual Service Provider testors in the regular (phase) of Turn-Up test.

This test case was modified and moved to the Individual test phase section 5 above.

# Appendix A:	Test Case Matrix

| NANC 169 – Delta Download File Creation by Time Range for SVs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Case Priority | Test Case # | Test Case Description | Req. | IIS Flow | Test Results/Issues/Comments |  |  |  |
| SOA – N/A | 169-1 | NPAC OP GUI – NPAC Personnel initiate a Bulk Data Download of Subscription Data – Specifying Active/Disconnect Pending/Partial Failure Subscription Versions Only and NOT specifying a TN range. Verification steps are performed to ensure the BDD file was processed successfully by the Service Provider system - Success | RR3-320, RR3-312, RR3-313, RR3-314, RR3-319, RR3-323 | N/A |  |  |  |  |
| LSMS - Optional |  |  |  |  |  |  |  |  |
| SOA – N/A | 169-2 | NPAC OP GUI – NPAC Personnel initiate a Bulk Data Download of Subscription Data – Specifying Active/Disconnect Pending/Partial Failure Subscription Versions Only and specifying a TN range that is a subset of the prerequisite test data. Verification steps are performed to ensure the BDD file was processed successfully by the Service Provider system - Success | RR3-318, RR3-319, RR3-320, RR3-323 | N/A |  |  |  |  |
| LSMS – Optional |  |  |  |  |  |  |  |  |
| SOA – N/A | 169-3 | NPAC OP GUI – NPAC Personnel initiate a Bulk Data Download of Subscription Data – Specifying Latest View of Subscription Version Activity a valid Time Range, and NOT specifying a TN range. Verification steps are performed to ensure the BDD file was processed successfully by the Service Provider system - Success | RR3-315, RR3-316, RR3-317, RR3-319 | N/A |  |  |  |  |
| LSMS – Optional |  |  |  |  |  |  |  |  |
| SOA – N/A | 169-4 | NPAC OP GUI – NPAC Personnel initiate a Bulk Data Download of Subscription Data – Specifying Latest View of Subscription Version Activity a valid Time Range, and a TN range that is a subset of the prerequisite test data. Verification steps are performed to ensure the BDD file was processed successfully by the Service Provider system - Success | RR3-319, RR3-320, RR3-321, RR3-322 | N/A |  |  |  |  |
| LSMS – Optional |  |  |  |  |  |  |  |  |
| NANC 187 – Linked Action Replies |  |  |  |  |  |  |  |  |
| Test Case Priority | Test Case # | Test Case Description | Requirements | IIS Flow | Test Results/Issues/Comments |  |  |  |
| SOA – N/A | 187-1 | LSMS – Service Provider Personnel for either an EDR or non-EDR LSMS submit a resynchronization request for Network Data, Block Data, Subscription Version Data and Notification Data by time range, over the LSMS to NPAC SMS Interface, with the Service Provider’s Local SMS Linked Replies Indicator set to their production setting. The recovery response includes a number of Network Data objects, Number Pool Block objects, Notifications and Subscription Versions less than or equal to their respective Linked Replies Blocking Factors. - Success | RR6-85, RR6-93, RR6-97, RR6-98, RR6-99, RR6-100, RR6-101, RR6-102, RR6-103, RR6-90, RR6-105 | B.7.1.1, B.7.1.2 |  |  |  |  |
| LSMS – Required |  |  |  |  |  |  |  |  |
| SOA – N/A | 187-2 | LSMS – Service Provider Personnel for either an EDR or non-EDR LSMS submit a resynchronization request for Network Data, and Subscription Version Data by time range, over the LSMS to NPAC SMS Interface, with the Service Provider’s Local SMS Linked Replies Indicator set to their production setting. The recovery response includes a number of Network Data objects greater than the Network Data Linked Replies Blocking Factor and less than the Network Data Maximum Linked Recovered Objects as well as a number of Subscription Version objects greater than the Subscription Data Linked Replies Blocking Factor and less than the Subscription Data Maximum Linked Recovered Objects. – Success | RR6-87, RR6-88, RR6-93, RR6-103, RR6-90, RR6-95, RR6-104, RR6-96 | B.7.1.1, B.7.1.2 |  |  |  |  |
| LSMS – Required |  |  |  |  |  |  |  |  |
| SOA – N/A | 187-3 | LSMS – Service Provider Personnel for either an EDR or non-EDR LSMS submit a resynchronization request for Network Data, Number Pool Block data and Subscription Version Data by time range, over the LSMS to NPAC SMS Interface, with the Service Provider’s Local SMS Linked Replies Indicator set to their production setting. The recovery response includes a number of Network Data objects, Number Pool Block objects and Subscription Version objects greater than the respective Maximum Linked Recovered Objects and Maximum Number Download Records parameters. – Failure | RR6-95, RR6-104, RR6-106 | B.7.1.1, B.7.1.2 |  |  |  |  |
| LSMS – Required |  |  |  |  |  |  |  |  |
| SOA – Required | 187-4 | SOA – Service Provider Personnel submit a resynchronization request for Network Data and Notification Data by time range, over the SOA to NPAC SMS Interface, with the Service Provider’s SOA Linked Replies Indicator set to their production setting. The recovery response includes a number of Network Data objects less than or equal to the Network Data Linked Replies Blocking Factor and a number Notifications less than or equal to the Notification Data Linked Replies Blocking Factor. – Success | RR6-92, RR6-89 | B.7.2 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| SOA – Required | 187-5 | SOA – Service Provider Personnel submit a resynchronization request for Network Data and Notification Data by time range, over the SOA to NPAC SMS Interface, with the Service Provider’s SOA Linked Replies Indicator set to their production setting. The recovery response includes a number of Network Data objects and Notifications greater than the respective Linked Replies Blocking Factor and less than the respective Maximum Linked Recovered Notifications. – Success | RR6-85, RR6-86, RR6-84, RR6-92, RR6-89, RR6-94, RR6-91 | B.7.2 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| SOA – Required | 187-6 | SOA – Service Provider Personnel submit a resynchronization request for Network Data and Notification Data by time range, over the SOA to NPAC SMS Interface, with the Service Provider’s SOA Linked Replies Indicator set to their production setting. The recovery response includes a number of Network Data objects greater than the Network Data Maximum Linked Recovered Objects and Notifications greater than the Notification Data Maximum Linked Recovered Notifications and Maximum Number of Download Records. - Failure | RR6-94, RR6-91 | B.7.2 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| NANC 191 DPC/SSN Value Edits and NANC 291 SSN Edits in the NPAC SMS |  |  |  |  |  |  |  |  |
| Test Case Priority | Test Case # | Test Case Description | Requirements | IIS Flow | Test Results/Issues/Comments |  |  |  |
| SOA – Required | 191/291-1 | SOA – Service Provider Personnel attempt to create a Subscription Version specifying some valid and some invalid DPC/SSN information. The regional SSN Edit Flags (CLASS, LIDB, CNAM, ISVM and WSMSC) are set to production values. - Failure | RR3-380, RR3-381, RR3-382, RR3-383, RR3-384, RR3-385, RR3-386, RR3-387, RR3-388, RR3-389, RR3-375, RR3-376, RR3-377, RR3-378, RR3-378 | B.5.1.1 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| SOA – Required | 191/291-2 | SOA – Service Provider Personnel attempt to modify a ‘Pending’ Subscription Version specifying some valid and some invalid DPC/SSN information. The regional SSN Edit Flags (CLASS, LIDB, CNAM, ISVM and WSMSC) are set to production values. - Failure | RR3-380, RR3-381, RR3-382, RR3-383, RR3-384, RR3-385, RR3-386, RR3-387, RR3-388, RR3-389, RR3-405, RR3-406, RR3-407, RR3-408, RR3-409, RR3-375, RR3-376, RR3-377, RR3-378, RR3-378 | B.5.2.3 or B.5.2.4 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| SOA – Required | 191/291-3 | SOA – Service Provider Personnel attempt to activate a ‘Pending’ Subscription Version that contains some valid and some invalid DPC/SSN information. The regional SSN Edit Flags (CLASS, LIDB, CNAM, ISVM and WSMSC) are set to production values. - Failure | RR3-427 | B.5.1.5 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| SOA – Required | 191/291-4 | SOA – Service Provider Personnel attempt to modify an ‘Active’ Subscription Version that contains some valid and some invalid DPC/SSN information. The regional SSN Edit Flags (CLASS, LIDB, CNAM, ISVM and WSMSC) are set to production values. - Failure | RR3-380, RR3-381, RR3-382, RR3-383, RR3-384, RR3-385, RR3-386, RR3-387, RR3-388, RR3-389, RR3-405, RR3-406, RR3-407, RR3-408, RR3-409, RR3-375, RR3-376, RR3-377, RR3-378, RR3-378 | B.5.2.1 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| SOA – N/A | 191/291-5 | NPAC OP GUI – NPAC Personnel attempt to submit a mass update request for a range of Subscription Versions that currently exist. Some of these Subscription Versions have valid DPC/SSN data and some of these Subscription Versions have invalid DPC/SSN data. The Mass Update request specifies new DPC/SSN values that will correct some but not all of the Subscription Versions that currently exist with invalid DPC/SSN attributes. The NPAC SMS processes the Mass Update request, modifies some but not all of the DPC/SSN attributes for the range specified in the Mass Update Request and logs the objects that could not be updated to the Mass Update Exception Report. The regional SSN Edit Flags (CLASS, LIDB, CNAM, ISVM and WSMSC) are set to production values. - Success | RR3-429 | B.8.3 |  |  |  |  |
| LSMS – Required |  |  |  |  |  |  |  |  |
| SOA – Required | 191/291-6 | SOA – Service Provider Personnel attempt to create a Number Pool Block specifying some valid and some invalid DPC/SSN information. The regional SSN Edit Flags (CLASS, LIDB, CNAM, ISVM and WSMSC) are set to production values. - Failure | RR3-390, RR3-391, RR3-392, RR3-393, RR3-394, RR3-395, RR3-396, RR3-397, RR3-398, RR3-399, RR3-400, RR3-401, RR3-402, RR3-403, RR3-404, RR3-375, RR3-376, RR3-377, RR3-378, RR3-378 | B.4.4.2 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| SOA – Required | 191/291-7 | SOA – Service Provider Personnel attempt to modify a Number Pool Block specifying some valid and some invalid DPC/SSN information. The regional SSN Edit Flags (CLASS, LIDB, CNAM, ISVM and WSMSC) are set to production values. - Failure | RR3-390, RR3-391, RR3-392, RR3-393, RR3-394, RR3-395, RR3-396, RR3-397, RR3-398, RR3-399, RR3-400, RR3-401, RR3-402, RR3-403, RR3-404, RR3-405, RR3-406, RR3-407, RR3-408, RR3-409, RR3-421, RR3-422, RR3-423, RR3-424, RR3-425, RR3-426, RR3-375, RR3-376, RR3-377, RR3-378, RR3-378 | B.4.4.13 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| SOA – N/A | 191/291-8 | NPAC – Upon Number Pool Block scheduled activation, NPAC SMS fails the Number Pool Block activation based on some invalid DPC/SSN information. The regional SSN Edit Flags (CLASS, LIDB, CNAM, ISVM and WSMSC) are set to production values. – Failure | RR3-390, RR3-391, RR3-392, RR3-393, RR3-394, RR3-395, RR3-396, RR3-397, RR3-398, RR3-399, RR3-400, RR3-401, RR3-402, RR3-403, RR3-404, RR3-428, RR3-375, RR3-376, RR3-377, RR3-378, RR3-378 | B.4.4.2 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| SOA – N/A | 191/291-9 | NPAC OP GUI – NPAC Personnel attempt to submit a mass update request that includes at least three complete, ‘Active’ Number Pool Blocks. One of these Number Pool Blocks should currently exist with valid DPC/SSN data, two should exist with invalid DPC/SSN data. The Mass Update criteria shall include all three Number Pool Blocks and the request specifies new DPC/SSN values that will correct one, but not both of the Number Pool Blocks that currently exists with invalid DPC/SSN data. The NPAC SMS processes the Mass Update request, modifies some but not all of the DPC/SSN attributes for the range specified in the Mass Update Request and logs the objects that could not be updated to the Mass Update Exception report. The regional SSN Edit Flags (CLASS, LIDB, CNAM, ISVM and WSMSC) are set to production values. – Success | RR3-429 | B.8.3 |  |  |  |  |
| LSMS – Required |  |  |  |  |  |  |  |  |
| NANC 192 – NPA Split, NPAC SMS Load File |  |  |  |  |  |  |  |  |
| Test Case Priority | Test Case # | Test Case Description | Requirements | IIS Flows | Test Results/Issues/Comments |  |  |  |
| SOA – Required | 192-1 | SOA/LSMS -Service Provider Personnel perform basic LNP functions before, during and after Permissive Dial Period for NPA Splits that are created on the NPAC SMS. - Success | RN3-1, RN 3-2, RR3-281, RN3-282, RN3-283, RR3-285, RR3-288, RN3-4.1, RR3-437, RN3-4.14, RN3-4.27, RN3-4.28, RN3-4.30, RR3-219 | B.8.2, B.4.1.1, B.5.1.2, B.5.1.5, B.5.1.6 |  |  |  |  |
| LSMS – Required |  |  |  |  |  |  |  |  |
| NANC 218 – Conflict Timestamp Broadcast to SOA |  |  |  |  |  |  |  |  |
| Test Case Priority | Test Case # | Test Case Description | Requirements | IIS Flows | Test Results/Issues/Comments |  |  |  |
| SOA – Required | 218-1 | SOA – (Old) Service Provider Personnel submit a single TN, subscription version modify request specifying Authorization (FALSE) and a valid status change cause code, setting the subscription version status to conflict - SUCCESS | RR5-44.2, RR5-44.3 | B.5.2.3, B.5.2.4 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| SOA – Required | 218-2 | SOA – Old Service Provider personnel successfully put a pending Subscription Version into conflict using an Old Service Provider create after the Conflict Restriction Window Tunable Time has been reached but before the Final Concurrence Timer (T2) has expired. – Success | RR5-44.2, RR5-44.3 | Based on B.5.1.4 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| NANC 230 – Donor SOA Port-To-Original of Intra-Service Provider Port |  |  |  |  |  |  |  |  |
| Test Case Priority | Test Case # | Test Case Description | Requirements | IIS Flow | Test Results/Issues/Comments |  |  |  |
| SOA – Required | 230-1 | SOA – Service Provider Personnel create an Intra-Service Provider, Port-to-Original Subscription Version where a previously ‘Active’ Subscription Version exists, that is not part of a Number Pool Block – Success | RR5-4, RR5-122, RR5-6.1 | B.5.1.11 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| SOA – Required | 230-2 | SOA – Service Provider Personnel create an Intra-Service Provider, Port-to-Original Subscription Version where a previously ‘Active’ Subscription Version exists with a matching NPA-NXX-X, after the NPA-NXX-X Creation and prior to the Number Pool Block Activation - Failure | RR5-121 | B.5.1.11 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| SOA – Required | 230-3 | SOA – Service Provider Personnel create an Intra-Service Provider, Porting to Original Subscription Version after NPA-NXX-X Effective Date and Block Activation – Success | RR5-57 | B.5.1.11 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| NANC 249 – Modification of Dates for a Disconnect Pending SV |  |  |  |  |  |  |  |  |
| Test Case Priority | Test Case # | Test Case Description | Requirements | IIS Flow | Test Results/Issues/Comments |  |  |  |
| SOA – Required | 249-1 | SOA – Service Provider Personnel submit a Subscription Version modify request for a ‘Disconnect-Pending’ Subscription Version, modifying the Effective Release Date and Customer Disconnect Date to the current date/time or a date/time in the past. – Success | RR5-124, RR5-125, RR5-127, RR5-129, RR5-126, RR5-11 | B.5.2.7, B.5.4.1, B.5.4.1.1 |  |  |  |  |
| LSMS – Required |  |  |  |  |  |  |  |  |
| Conditional | 249-2 | SOA – Service Provider Personnel submit a Subscription Version modify request for a range of ‘Disconnect-Pending’ Subscription Versions, modifying the Effective Release Date and Customer Disconnect Date to a different date/time in the future. The range of Subscription Versions had Effective Release Dates that were not the same prior to the modification. – Success | RR5-127 | B.5.2.7 |  |  |  |  |
| LSMS – Conditional |  |  |  |  |  |  |  |  |
| SOA – Required | 249-3 | SOA – Service Provider Personnel submit a Subscription Version modify request for a ‘Disconnect-Pending’ Subscription Version, without specifying the Customer Disconnect Date – Failure | RR5-128, R5-39.1, R5-29.2 | B.5.2.7 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| SOA – Required | 249-4 | SOA – Service Provider Personnel submit a Subscription Version modify request for a ‘Disconnect-Pending’ Subscription Version, specifying an invalid format for the Effective Release Date and/or Customer Disconnect Date - Failure | RR5-125 | B.5.2.7 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| NANC 297 – Sending SV Problem During Recovery |  |  |  |  |  |  |  |  |
| Test Case Priority | Test Case # | Test Case Description | Requirements | IIS Flow | Test Results/Issues/Comments |  |  |  |
| SOA – | 297-1 | DELETED, Incorporated into 187-1 |  |  |  |  |  |  |
| LSMS - |  |  |  |  |  |  |  |  |
| SOA | 297-2 | –DELETED, Incorporated into 187-1 |  |  |  |  |  |  |
| LSMS – |  |  |  |  |  |  |  |  |
| NANC 319 – NPAC Edit to Ensure NPA-NXX of LRN is in Same LATA as NPA-NXX of Ported TN |  |  |  |  |  |  |  |  |
| Test Case Priority | Test Case # | Test Case Description | Requirements | IIS Flow | Test Results/Issues/Comments |  |  |  |
| SOA – Required | 319-1 | SOA – Service Provider Personnel attempt to create a Subscription Version specifying a TN and an LRN with different LATA Ids. - Failure | RR5-120 | B.5.1.1, B.5.1.2 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| SOA – Required | 319-2 | SOA – Service Provider Personnel attempt to modify a ‘Pending’, Subscription Version specifying an LRN with a different LATA Id from the NPA-NXX of the TN in the Subscription Versions. – Failure | RR5-123 | B.5.2.3, B.5.2.4 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| SOA – Required | 319-3 | NPAC OP GUI – NPAC Personnel submit a mass update request for a range of ‘Active’, Subscription Versions where some of the Subscription Versions exist with valid LATA ID relationships and some of the Subscription Versions exist with invalid LATA ID relationships. Specify new DPC/SSN data. Subscription Versions with valid LATA ID relationships will be updated and Subscription Versions that exist without valid LATA ID relationships will not be updated. – Success | RR3-254 | B.8.3 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| SOA – Conditional | 319-4 | SOA – Service Provider Personnel attempt to create a Number Pool Block specifying an LRN with a different LATA Id than the TNs in the Number Pool Block. – Failure | RR3-334 | B.4.4.1 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| SOA – Conditional | 319-5 | SOA – Service Provider Personnel attempt to modify a Number Pool Block specifying an LRN with a different LATA ID than the TNs in the Number Pool Block. – Failure | RR3-335 | B.4.4.13 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| SOA – Required | 319-6 | NPAC OP GUI – NPAC Personnel submit a mass update request including at least three complete Number Pool Blocks where two of the Number Pool Blocks exits with valid LATA ID relationships and one Number Pool Block exists with invalid LATA ID relationships. Specify new DPC/SSN data. Number pool Blocks with valid LATA ID relationships will be updated and the Number Pool Block that exists with invalid LATA ID relationships will not be updated. – Success | RR3-254 | B.8.3 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| NANC 322 – Clean Up of Failed SP List Based on Service Provider BDD Response File |  |  |  |  |  |  |  |  |
| Test Case Priority | Test Case # | Test Case Description | Requirements | IIS Flow | Test Results/Issues/Comments |  |  |  |
| SOA – N/A | 322-1 | LSMS – Service Provider Personnel create a Bulk Data Download Response File for Subscription Version data. NPAC Personnel process the Bulk Data Download Response File. The Service Provider was previously on the Failed SP List for at least some of the Subscription Versions in the respective file. Verification steps are performed to ensure the Service Provider’s LSMS is now in synch with the NPAC SMS. – Success | RR3-330, RR3-329, RR3-332, RR3-333, RR3-325, RR3-326 | N/A |  |  |  |  |
| LSMS – Conditional |  |  |  |  |  |  |  |  |
| SOA – N/A | 322-2 | LSMS – Service Provider Personnel create a Bulk Data Download Response File for Number Pool Block data. NPAC Personnel process the Bulk Data Download Response File. The Service Provider was previously on the Failed SP List for at least some of the Number Pool Blocks in the respective file. Verification steps are performed to ensure the Service Provider’s LSMS is now in synch with the NPAC SMS. – Success | RR3-330, RR3-331, RR3-332, RR3-333, RR3-325, RR3-327 | N/A |  |  |  |  |
| EDR LSMS – Conditional |  |  |  |  |  |  |  |  |
| Non-EDR LSMS – N/A |  |  |  |  |  |  |  |  |
| NANC 323 – Partial Migration of SPID via Mass Update Test Cases |  |  |  |  |  |  |  |  |
| Test Case Priority | Test Case # | Test Case Description | Requirements | IIS Flow | Test Results/Issues/Comments |  |  |  |
| SOA – Conditional | 323-1 | DELETED. Will be executed during Group Test Phase only due to impact to test environment. | RR3-255, RR3-256, RR3-257, RR3-258, RR3-259, RR3-260, RR3-261, RR3-262, RR3-263, RR3-264, RR3-265, RR3-266, RR3-267, RR3-268, RR3-269, RR3-270, RR3-271, RR3-272, RR3-273, RR3-274, RR3-276, RR3-277 | N/A |  |  |  |  |
| LSMS – Conditional |  |  |  |  |  |  |  |  |
| SOA – N/A | 323-2 | DELETED. | RR3-275 | N/A |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |
| NANC 354 – Delta Download File Creation by Time Range for Network Data |  |  |  |  |  |  |  |  |
| Test Case Priority | Test Case # | Test Case Description | Requirements | IIS Flow | Test Results/Issues/Comments |  |  |  |
| SOA – Required | 354-1 | NPAC OP GUI – NPAC Personnel initiate a Bulk Data Download of Network Data – Specifying the Latest View of Network Data Activity and a valid time range. Verification steps are preformed to ensure the BDD file was processed successfully by the Service Provider system. – Success | RR3-220, RR3-301, RR3-302, RR3-304, RR3-311 | N/A |  |  |  |  |
| LSMS – Required |  |  |  |  |  |  |  |  |
| SOA – Required | 354-2 | NPAC OP GUI – NPAC Personnel initiate a Bulk Data Download of Network Data – Specifying the All Network Data . Verification steps are preformed to ensure the BDD file was processed successfully by the Service Provider system. – Success | RR3-303, RR3-307, RR3-308, RR3-309, RR3-310, RR3-311 | N/A |  |  |  |  |
| LSMS – Required |  |  |  |  |  |  |  |  |
| Group Testing |  |  |  |  |  |  |  |  |
| Test Case Priority | Test Case # | Test Case Description | Requirements | IIS Flow | Test Results/Issues/Comments |  |  |  |
| SOA – Required | 323-1 | NPAC OP GUI – NPAC Personnel submit a request for a Partial SPID migration via Mass Update, where NPA-NXX, LRN, Subscription Version, NPA-NXX-X and Block Information exist for the migrating away from SPID. Verification steps are performed to ensure the Service Provider system is now in synch with the NPAC SMS. – Success NANC 323 requires unique industry coordination unlike any other aspect of NPAC functionality. Due to a delay in industry readiness, functional testing of NANC 323 functionality has been postponed. These test case procedures will be updated to more appropriately support the unique testing approach required to efficiently test this functionality and will be published in coordination with the actual testing cycle. | RR3-255, RR3-256, RR3-257, RR3-258, RR3-259, RR3-260, RR3-261, RR3-262, RR3-263, RR3-264, RR3-265, RR3-266, RR3-267, RR3-268, RR3-269, RR3-270, RR3-271, RR3-272, RR3-273, RR3-274, RR3-276, RR3-277 | N/A |  |  |  |  |
| LSMS – Required |  |  |  |  |  |  |  |  |
| SOA – N/A | 187-1 | LSMS – Service Provider Personnel for either an EDR or non-EDR LSMS submit a resynchronization request for Network Data, Block Data, Subscription Version Data and Notification Data by time range, over the LSMS to NPAC SMS Interface, with the Service Provider’s Local SMS Linked Replies Indicator set to their production setting. The recovery response includes a number of Network Data objects, Number Pool Block objects, Notifications and Subscription Versions less than or equal to their respective Linked Replies Blocking Factors. - Success | RR6-85, RR6-93, RR6-97, RR6-98, RR6-99, RR6-100, RR6-101, RR6-102, RR6-103, RR6-90, RR6-105 | B.7.1.1, B.7.1.2 |  |  |  |  |
| LSMS – Required |  |  |  |  |  |  |  |  |
| SOA – N/A | 187-2 | LSMS – Service Provider Personnel for either an EDR or non-EDR LSMS submit a resynchronization request for Network Data, and Subscription Version Data by time range, over the LSMS to NPAC SMS Interface, with the Service Provider’s Local SMS Linked Replies Indicator set to their production setting. The recovery response includes a number of Network Data objects greater than the Network Data Linked Replies Blocking Factor and less than the Network Data Maximum Linked Recovered Objects as well as a number of Subscription Version objects greater than the Subscription Data Linked Replies Blocking Factor and less than the Subscription Data Maximum Linked Recovered Objects. - Success | RR6-87, RR6-88, RR6-93, RR6-103, RR6-90, RR6-95, RR6-104, RR6-96 | B.7.1.1, B.7.1.2 |  |  |  |  |
| LSMS – Required |  |  |  |  |  |  |  |  |
| SOA – Required | 187-4 | SOA – Service Provider Personnel submit a resynchronization request for Network Data and Notification Data by time range, over the SOA to NPAC SMS Interface, with the Service Provider’s SOA Linked Replies Indicator set to their production setting. The recovery response includes a number of Network Data objects less than or equal to the Network Data Linked Replies Blocking Factor and a number of Notifications less than or equal to the Notification Data Linked Replies Blocking Factor. - Success | RR6-92, RR6-89 | B.7.2 |  |  |  |  |
| LSMS – N/A |  |  |  |  |  |  |  |  |

# Appendix B:	Test Plan Issues

Following are issues related to the NPAC Release 3.2 Test Plan:

| # | Date | Issue | Status |
| --- | --- | --- | --- |
| 1. | 11/22/02 | Need to review NANC 187 test cases and consider if we need to break out test cases in possibly EDR/Non-EDR test cases to simplify. | Updated in version 1/17/03. CLOSED |
| 2. | 12/17/02 | Sunset issue related to NANC 169 – question of why test cases are marked as required. To be addressed in January LNPA WG meeting. | Updated to ‘optional’ 1/19/04. |
| 3. |  |  |  |
| 4. |  |  |  |