| ## 11. Bulk Data Downloads: |  |  |
| --- | --- | --- |
| 11.1 | NPAC OP GUI - NPAC Personnel generate a Bulk Data Download File for Block Information specifying ‘Active and Partial Failure Blocks Only’ with a Block Range for an EDR System. – Success | RR3-198, RR3-199, RR3-200.1, RR3-200.2, RR3-202, RR3-203, RR3-204, RR3-205, RR3-206, RR3-207 *Appendix E – Bulk Data Download file Formats |
| 11.2 | NPAC OP GUI - NPAC Personnel generate a Bulk Data Download File for Subscription Version information for an EDR system. - Success | RR5-112 |
| 11.3 | NPAC OP GUI - NPAC Personnel generate a Bulk Data Download File for Network Data for a Service Provider whose NPA-NXX-X indicator is set to ‘FALSE’. - Success | RR3-116, RR3-117, RR3-118 *Appendix E – Bulk Data Download file Formats |
| 11.4 | NPAC OP GUI – NPAC Personnel generate a Bulk Data Download File Network Data for a Service Provider whose NPA-NXX-X indicator is set to ‘TRUE’. – Success | RR3-116, RR3-117, RR3-118 |
| 11.5 | NPAC OP GUI – NPAC Personnel generate a Bulk Data Download File for Block Information specifying ‘Latest View of Block Activity’ and a Time Range for a non-EDR system. – Success | RR3-198, RR3-199, RR3-200.1, RR3-200.3, RR3-201.1, RR3-201.2, RR3-203, RR3-204, RR3-205, RR3-207 *Appendix E – Bulk Data Download file Formats |
| 11.6 | NPAC OP GUI – NPAC Personnel generate a Bulk Data Download File for Block Information specifying ‘Latest View of Block Activity’, a Time Range, and a Block Range for an EDR = ‘TRUE’ system. – Success | RR3-203, RR3-204, RR3-205, RR3-207 *Appendix E – Bulk Data Download file Formats |
| 11.7 | NPAC OP GUI – NPAC Personnel generate a Bulk Data Download File for Block Information specifying ‘Active and Partial Failure Blocks Only’ and a Time Range. – Error | RR3-203 *Appendix E – Bulk Data Download file Formats |
| 11.8 | NPAC OP GUI – NPAC Personnel generate a Migration File. – Success |  |

## 2.0 integration has been completed in this section.  The version was rolled to 0.4 dated 02/05/99.

## Added comments from “redline” review.  Rolled to 0.5 dated 03/10/99.

## This section has been updated per the meeting on 3/23/99, and is version 0.6 dated 06/17/99.

## This section has been updated to version 0.7, 08/01/99.

## This section has been updated after review by Lockheed/ESI, and is version 0.8 dated 11/24/99.

This section has been updated after industry review and is version 0.9 dated 01/24/00.

This section has been updated after industry review and is version 0.10 dated 02/11/00.

## Bulk Data Download Test Cases:

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 11.1 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | ## O |
|  |  |  |  |  |  |  | non-EDR LSMS |  | O |
|  |  |  |  |  |  |  | # EDR LSMS |  | O |
|  | Objective: |  | NPAC OP GUI - NPAC Personnel generate a Bulk Data Download File for Block Information specifying ‘Active and Partial Failure Blocks Only’ with a Block Range for an EDR System. – Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-198, RR3-199, RR3-200.1, RR3-200.2, RR3-202, RR3-203, RR3-204, RR3-205, RR3-206, RR3-207 *Appendix E – Bulk Data Download file Formats |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  |  |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that at least two blocks exists on the NPAC for the Block Range being specified in the Bulk Data Download File. - Verify that at least one block is contained within an NPA-NXX that is NOT filtered for the requesting Service Provider and a status of ‘partial failure’. - Verify that at least one block is contained within an NPA-NXX that is filtered for the requesting Service Provider. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel submit a request to generate a Bulk Data Download File for Block Information by specifying the ‘Active and Partial Failure Blocks Only’ and a Block Range. |  | NPAC |  | The NPAC SMS accepts the request and generates the Bulk Data Download File for the Block Information specified by the input criteria, to the appropriate Service Provider directory on the NPAC SMS Server. |  |  |  |
| 2. | NPAC | NPAC Personnel examine the Bulk Data Download File that was created. |  | NPAC |  | - Verify that the Block Bulk Data Download File was placed in the correct directory and contains ‘active’ and one ‘partial failure’ Blocks with the following information: - Block ID - NPA-NXX-X - LRN - New Current Service Provider ID - Activation Timestamp - CLASS DPC - CLASS SSN - LIDB DPC - LIDB SSN - ISVM DPC - ISVM SSN - CNAM DPC - CNAM SSN - WSMSC DPC – if supported by the Service Provider LSMS - WSMSC SSN – if supported by the Service Provider LSMS - Download Reason - Verify that the Block Bulk Data Download File DOES contain the Block that was NOT filtered, and DOES NOT contain the block that is filtered. - Verify that the Block Bulk Data Download File was named appropriately. |  |  |  |
| 3. | SP – optional | Service Provider Personnel verify the Bulk Data Download file on their local system. |  | SP |  | Verify that the Bulk Data Download file contains appropriate data and follows the file format. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 11.2 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | ## O |
|  |  |  |  |  |  |  | non-EDR LSMS |  | O |
|  |  |  |  |  |  |  | EDR LSMS |  | O |
|  | Objective: |  | NPAC OP GUI - NPAC Personnel generate a Bulk Data Download File for Subscription Version information for an EDR system. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR5-112 |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  |  |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that Pooled ,Non-Pooled, and Ported Pooled Subscription Versions exist on the NPAC for the range being specified in the Bulk Data Download File. - Verify that the requesting SP has EDR = ‘TRUE’. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel submit a request to generate a Bulk Data Download File for Subscription Version Information by specifying an NPA-NXX Range. |  | NPAC |  | The NPAC SMS accepts the request and generates the Bulk Data Download File based on this Service Provider’s EDR indicator (set to ‘TRUE’) and contains information for Subscription Versions that fall within the NPA-NXX range specified in the input criteria – that DO NOT have an LNP Type of ‘POOL’. The NPAC SMS places this file in the appropriate Service Provider directory on the NPAC SMS. |  |  |  |
| 2. | NPAC | NPAC Personnel examine the Bulk Data Download File that was created. |  | NPAC |  | - Verify that the Subscription Version Bulk Data Download File was placed in the correct directory and contains the following information by Subscription Version: - Version ID - TN - LRN - New Current Service Provider ID - Activation TimeStamp - CLASS DPC - CLASS SSN - LIDB DPC - LIDB SSN - ISVM DPC - ISVM SSN - CNAM DPC - CNAM SSN - WSMSC DPC – if supported by the Service Provider LSMS - WSMSC SSN – if supported by the Service Provider LSMS - End User Location Value - End User Location Type - Billing ID - LNP Type - Download Reason - Verify that there are not any Subscription Version with LNP Type set to ‘POOL’ included in the download information. |  |  |  |
| 3. | SP – optional | Service Provider Personnel verify the Bulk Data Download file on their local system. |  | SP |  | Verify that the Bulk Data Download file contains appropriate data and follows the file format. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 11.3 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | ## O |
|  |  |  |  |  |  |  | non-EDR LSMS |  | O |
|  |  |  |  |  |  |  | EDR LSMS |  | O |
|  | Objective: |  | NPAC OP GUI - NPAC Personnel generate a Bulk Data Download File for Network Data for a Service Provider whose NPA-NXX-X indicator is set to ‘FALSE’. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-116, RR3-117, RR3-118 *Appendix E – Bulk Data Download file Formats |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  |  |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that at least two NPA-NXX-X objects exist on the NPAC for any Service Provider within each of the NPA-NXXs criteria for the Bulk Data Download File. - Verify that the NPA-NXX-X indicator is set to ‘FALSE’ for the requesting Service Provider. - Verify that at least one of the requested NPA-NXXs is not filtered and at least one of the requested NPA-NXXs is filtered for the requesting SP. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel submit a request to generate a Bulk Data Download File for Network Data by specifying an NPA-NXX range that includes at least two NPA-NXX-X objects. |  | NPAC |  | The NPAC SMS accepts the request and generates the Bulk Data Download File for the NPA-NXX Information as well as a separate file for the respective NPA-NXX-X Information specified by the input criteria, to the appropriate Service Provider directory on the NPAC SMS Server. |  |  |  |
| 2. | NPAC | NPAC Personnel examine the Bulk Data Download File that was created. |  | NPAC |  | - Verify that the Bulk Data Download Files were placed in the correct directory. - Verify that the NPA-NXX that was filtered does not exist in either Bulk Data Download File. - Verify that the NPA-NXX Bulk Data Download File contains the following information by NPA-NXX: - Service Provider ID - NPA-NXX ID - NPA-NXX Value - Creation TimeStamp - Verify that the NPA-NXX-X Bulk Data Download File contains the following information by NPA-NXX-X: - Service Provider ID - NPA-NXX-X ID - NPA-NXX-X Value - Creation TimeStamp - Effective TimeStamp - Download Reason - Verify that the Block Bulk Data Download File was named appropriately. |  |  |  |
| 3. | SP – optional | Service Provider Personnel verify the Bulk Data Download file on their local system. |  | SP |  | Verify that the Bulk Data Download file contains appropriate data and follows the file format. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 11.4 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | ## O |
|  |  |  |  |  |  |  | non-EDR LSMS |  | O |
|  |  |  |  |  |  |  | EDR LSMS |  | O |
|  | Objective: |  | NPAC OP GUI – NPAC Personnel generate a Bulk Data Download File Network Data for a Service Provider whose NPA-NXX-X indicator is set to ‘TRUE’. – Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-116, RR3-117, RR3-118 |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  |  |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that at least one NPA-NXX-X object exists on the NPAC for NPA-NXX range and the Service Provider specified in the Bulk Data Download File criteria. - Verify that the NPA-NXX-X indicator is set to ‘TRUE’ for this Service Provider. - Verify that at least one of the requested NPA-NXXs is not filtered and at least one of the requested NPA-NXXs is filtered for the requesting SP. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel submit a request to generate a Bulk Data Download File for Network Data by specifying an NPA-NXX range that includes at least one NPA-NXX-X object. |  | NPAC |  | The NPAC SMS accepts the request and generates the Bulk Data Download File for the NPA-NXX Information as well as a separate file for the respective NPA-NXX-X Information specified by the input criteria, to the appropriate Service Provider directory on the NPAC SMS Server. |  |  |  |
| 2. | NPAC | NPAC Personnel examine the Bulk Data Download File that was created. |  | NPAC |  | - Verify that the Bulk Data Download Files were placed in the correct directory. - Verify that the NPA-NXX Bulk Data Download File contains the following information by NPA-NXX: - Service Provider ID - NPA-NXX ID - NPA-NXX Value - Creation TimeStamp - Verify that the NPA-NXX-X Bulk Data Download File contains the following information by NPA-NXX-X: - Service Provider ID - NPA-NXX-X ID - NPA-NXX-X Value - Creation TimeStamp - Effective TimeStamp - Download Reason |  |  |  |
| 3. | SP – optional | Service Provider Personnel verify the Bulk Data Download file on their local system. |  | SP |  | Verify that the Bulk Data Download file contains appropriate data and follows the file format. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 11.5 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | N/A |
|  |  |  |  |  |  |  | non-EDR LSMS |  | O |
|  |  |  |  |  |  |  | EDR LSMS |  | O |
|  | Objective: |  | NPAC OP GUI – NPAC Personnel generate a Bulk Data Download File for Block Information specifying ‘Latest View of Block Activity’ and a Time Range for a non-EDR system. – Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-198, RR3-199, RR3-200.1, RR3-200.3, RR3-201.1, RR3-201.2, RR3-203, RR3-204, RR3-205, RR3-207 *Appendix E – Bulk Data Download file Formats |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  |  |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that multiple blocks exists on the NPAC with a status of ‘Old’, ‘Active’, ‘Failed’ and ‘Partial-Failure’ within the time range specified as well as outside of the specified time range for the Time Range being specified in the Bulk Data Download File. - Verify that the Service Providers EDR Indicator is set to ‘FALSE’. - Verify that at least one of the requested NPA-NXX(s) is NOT filtered for the requesting SP. - Verify that at least one of the requested NPA-NXX(s) is filtered for the requesting SP. - Verify that the ‘Old’ and ‘Failed’ Blocks are NOT filtered. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel submit a request to generate a Bulk Data Download File for Block Information by specifying the ‘Latest View of Block Activity’ and a Time Range. |  | NPAC |  | The NPAC SMS accepts the request and generates the Bulk Data Download File for the Block Information specified by the input criteria, to the appropriate Service Provider directory on the NPAC SMS Server. |  |  |  |
| 2. | NPAC | NPAC Personnel examine the Bulk Data Download File that was created. |  | NPAC |  | - Verify that only those blocks within the specified time range (based on the selection criteria) exist in the download file. The ‘Old’ and ‘Failed’ Blocks should NOT be present in the file. - Verify that the Block Bulk Data Download File was placed in the correct directory and contains the following information by Block: - Block ID - NPA-NXX-X - LRN - New Current Service Provider ID - Activation Timestamp - CLASS DPC - CLASS SSN - LIDB DPC - LIDB SSN - ISVM DPC - ISVM SSN - CNAM DPC - CNAM SSN - WSMSC DPC – if supported by the Service Provider LSMS - WSMSC SSN – if supported by the Service Provider LSMS - Download Reason |  |  |  |
| 3. | SP – optional | Service Provider Personnel verify the Bulk Data Download file on their local system. |  | SP |  | Verify that the Bulk Data Download file contains appropriate data and follows the file format. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 11.6 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | N/A |
|  |  |  |  |  |  |  | non-EDR LSMS |  | O |
|  |  |  |  |  |  |  | EDR LSMS |  | O |
|  | Objective: |  | NPAC OP GUI – NPAC Personnel generate a Bulk Data Download File for Block Information specifying ‘Latest View of Block Activity’, a Time Range, and a Block Range for an EDR = ‘TRUE’ system. – Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-203, RR3-204, RR3-205, RR3-207 *Appendix E – Bulk Data Download file Formats |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  |  |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that multiple blocks exists on the NPAC with a status of ‘Old’, ‘Active’, ‘Failed’ and ‘Partial-Failure’ within the time range specified as well as outside of the specified time range for the Time Range being specified in the Bulk Data Download File. - Verify that the Service Provider’s EDR Indicator is set to ‘TRUE’. - Verify that at least one of the requested NPA-NXX(s) is not filtered for the requesting SP. - Verify that at least one of the requested NPA-NXX(s) is filtered for the requesting SP. - Verify that the ‘Old’ and ‘Failed’ Blocks are NOT filtered. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel submit a request to generate a Bulk Data Download File for Block Information by specifying the ‘Latest View of Block Activity’, a Time Range and a Block Range. |  | NPAC |  | The NPAC SMS accepts the request and generates the Bulk Data Download File for the Block Information specified by the input criteria, to the appropriate Service Provider directory on the NPAC SMS Server. |  |  |  |
| 2. | NPAC | NPAC Personnel examine the Bulk Data Download File that was created. |  | NPAC |  | - Verify that only those blocks within the specified time range (based on the selection criteria) exist in the download file. The ‘Old’ and ‘Failed’ Blocks should NOT be present in the file. - Verify that the Bulk Data Download File was placed in the correct directory and contains the following information by Block: - Block ID - NPA-NXX-X - LRN - New Current Service Provider ID - Activation Timestamp - CLASS DPC - CLASS SSN - LIDB DPC - LIDB SSN - ISVM DPC - ISVM SSN - CNAM DPC - CNAM SSN - WSMSC DPC – if supported by the Service Provider LSMS - WSMSC SSN – if supported by the Service Provider LSMS - Download Reason |  |  |  |
| 3. | SP – optional | Service Provider Personnel verify the Bulk Data Download file on their local system. |  | SP |  | Verify that the Bulk Data Download file contains appropriate data and follows the file format. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 11.7 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | N/A |
|  |  |  |  |  |  |  | non-EDR LSMS |  | N/A |
|  |  |  |  |  |  |  | EDR LSMS |  | N/A |
|  | Objective: |  | NPAC OP GUI – NPAC Personnel generate a Bulk Data Download File for Block Information specifying ‘Active and Partial Failure Blocks Only’ and a Time Range. – Error |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-203 *Appendix E – Bulk Data Download file Formats |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  |  |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Verify that at least one block exists on the NPAC for the Block Range being specified in the Bulk Data Download File. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel submit a request to generate a Bulk Data Download File for Block Information by specifying the ‘Active and Partial Failure Blocks Only’ and a Time Range. |  | NPAC |  | - The NPAC SMS receives the request and determines that the selection criteria (Active and Partial Failure Blocks Only and a Time Range)is not valid (this violates system requirements). - The NPAC SMS rejects the request to generate the Bulk Data Download File, and returns an error to the NPAC Personnel. |  |  |  |
| 2. | NPAC | NPAC Personnel examine the directory where the Bulk Data Download File should be created. |  | NPAC |  | Verify that the Bulk Data Download File was not created. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 11.8 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | N/A |
|  |  |  |  |  |  |  | non-EDR LSMS |  | O |
|  |  |  |  |  |  |  | EDR LSMS |  | O |
|  | Objective: |  | NPAC OP GUI – NPAC Personnel generate a Migration File. – Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  |  |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  |  |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that multiple NPAC SMS version 1.4 Number Pool Blocks exists on the NPAC with a status of ‘Old’, ‘Active’, ‘Failed’ and ‘Partial-Failure’. - Verify that the Service Provider’s EDR Indicator is set to ‘TRUE’. - Verify that none of the Number Pool Blocks are filtered. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the appropriate mechanism within the NPAC SMS, NPAC Personnel submit a request to generate a Migration File. |  | NPAC |  | The NPAC SMS accepts the request and generates the Migration File for the Number Pool Block Information, to the appropriate Service Provider directory on the NPAC SMS Server. |  |  |  |
| 2. | NPAC | NPAC Personnel examine the Migration File that was created. |  | NPAC |  | - Verify that only those blocks within the specified time range (based on the selection criteria) exist in the download file. The ‘Old’ and ‘Failed’ Blocks should NOT be present in the file. - Verify that the Bulk Data Download File was placed in the correct directory and contains the following information by Block: - Block ID - NPA-NXX-X - LRN - New Current Service Provider ID - Activation Timestamp - CLASS DPC - CLASS SSN - LIDB DPC - LIDB SSN - ISVM DPC - ISVM SSN - CNAM DPC - CNAM SSN - WSMSC DPC – if supported by the Service Provider LSMS - WSMSC SSN – if supported by the Service Provider LSMS - Download Reason |  |  |  |
| 3. | SP – optional | Service Provider Personnel verify the Migration file on their local system. |  | SP |  | Verify that the Migration file contains appropriate data and follows the file format. |  |  |  |