| 4. Block Information: |  |  |
| --- | --- | --- |
| 4.1 | Create Block Information |  |
| 4.1.1 | SOA - Service Provider Personnel create a non-contaminated Number Pool Block – Success. | - RR3-124, RR3-125, RR3-126, RR3-130, RR3-132, RR3-144, RR3-146, RR3-150, RR3-151, RR3-152, RR3-143, RR3-180, RR5-85, RR5-86, RR5-87, RR5-89 |
| 4.1.2 | NPAC OP GUI - NPAC Personnel schedule a Number Pool Block Create for a contaminated Block to be run at a future date, and the NPAC SMS activates upon scheduled date and time – Success | RR3-75.2, RR5-92 |
| 4.1.3 | SOA - Service Provider Personnel create a Number Pool Block that already exists. - Error | RR3-129, RR3-131 |
| 4.1.4 | SOA – Service Provider Personnel create a Number Pool Block prior to the NPA-NXX-X Effective Date – Error | RR3-127 |
| 4.1.5 | SOA - Service Provider Personnel attempt to create a Number Pool Block when ‘pending-like, no-active’ Subscription Versions exist – Error | RR3-148 |
| 4.1.6 | NPAC OP GUI - NPAC Personnel re-schedule a Number Pool Block Create Event to run immediately. The initial Number Pool Block Create Request that was initiated by the NPA-NXX-X Holder SOA has failed due to ‘pending-like, no active’ Subscription Versions. – Success | RR3-75.2, RR3-76.2, RR3-77, RR3-81.1, RR3-81.2, RR3-82.2, RR5-90, RR5-91, RR5-92, RR5-93, RR5-94, RR5-96, RR5-97 |
| 4.1.7 | NPAC OP GUI – NPAC Personnel attempt to schedule a Number Pool Block create to execute immediately, when ‘pending-like, no-active’ Subscription Versions exist. Generate the ‘pending-like, no-active’ Subscription Version Report to a file. – Error | RR3-147, RR3-82.3, RR3-83.1, RR3-83.2, RR3-83.3, RR3-83.4 |
| 4.1.8 | SOA - Service Provider Personnel create a Number Pool Block - (to at least 3 LSMSs – at least 1 EDR and 2 non-EDR) that results in a Full Failure – Success | RR3-132, RR3-141.1, Table RR3-137.2RR3-137.2 (Row 15), Table RR3-138.2 (Row 15), RR3-142.1, RR3-153, RR5-95 |
| 4.1.9 | NPAC OP GUI - NPAC Personnel re-send a full failure Number Pool Block create to 3 LSMSs (1 EDR and 2 non-EDR systems) resulting in Partial Failure (2 non-EDR systems fail) – Success | Table RR3-137.2RR3-137.2 (Row 14), RR3-138.1, RR3-138.2, Table RR3-138.2 (Row 14), RR3-139, RR3-153, RR3-185, RR3-186.1, RR3-186.2, RR3-187, RR3-188, RR3-189, RR3-190, RR3-195, RR3-196, RR3-197, RR5-85, RR5-72, RR5-73, RR5-77, RR5-78, RR5-79 |
| 4.1.10 | NPAC - NPAC Personnel perform a resend of a previously ‘partial failure’ Number Pool Block to all Service Providers in the Failed SP List (2 non-EDR) – Success | - RR3-120, RR3-121, RR3-138.1, RR3-140, RR3-153, RR3-186.1, RR3-186.2, RR3-187, RR3-188, RR3-189, RR3-191, RR3-194, RR3-195, RR3-196, RR5-100, RR5-101, RR5-72, RR5-74, RR5-78 |
| 4.1.11 | SOA – Service Provider Personnel create a Number Pool Block (to at least 4 LSMSs - 2 non-EDR and 2 EDR) that results in a Partial Failure - Success | RR3-132, RR3-138.1, RR3-153, RR5-100, RR5-101, RR5-95 |
| 4.1.12 | NPAC OP GUI – NPAC Personnel create a Number Pool Block when an orphaned TN exists and the NPAC performs a Subscription Version completion check - Success | RR5-96, RR5-97, RR5-98, RR5-99, RR5-102 |
| 4.1.13 | SOA – Service Provider Personnel create a Number Pool Block (to at least 4 LSMSs - 2 non-EDR and 2 EDR) that results in a Partial Failure (1 non-EDR system fails one TN and 1 EDR system fails) – Success | RR3-132, RR3-134, RR3-135, RR3-136, RR3-137.1, RR3-137.2, Table RR3-137.2 (Row 9), RR3-138.1, Table RR3-138.2 (Row 9), RR3-153, RR5-88, RR5-100, RR5-101, RR5-95 |
| 4.1.14 | SOA – Service Provider Personnel create a Number Pool Block (to at least 4 LSMSs - 2 non-EDR and 2 EDR) that results in a Partial Failure (1 non-EDR system fails one TN and 2 EDR systems fails) – Success | RR3-132, Table RR3-137.2 (Row 12), RR3-138.1, Table RR3-138.2 (Row 12), RR3-153, RR5-100, RR5-101, RR5-95 |
| 4.2 Modify Number Pool Block |  |  |
| 4.2.1 | SOA- Service Provider Personnel modify an active Number Pool Block with the SOA Origination Indicator set to FALSE (and contains Subscription Versions with LNP Types of ‘POOL’, ‘LISP’ and ‘LSPP’) for at least 4 LSMSs (2 non-EDR and 2 EDR). – Success | RR3-119, RR3-120, RR3-121, RR3-122, RR3-128, RR3-133, RR3-157, RR3-159, RR3-160, RR3-162, RR3-163, RR3-164, RR3-165, RR3-167, RR3-168, RR5-85, RR5-86, RR5-87, RR5-103, RR5-104, RR5-105 |
| 4.2.2 | SOA – Service Provider Personnel modify the LRN for an active Number Pool Block and broadcast to at least 4 LSMSs (2 EDR and 2 non-EDR systems) resulting in Full Failure – Success | RR3-132, RR3-137.3, Table RR3-137.3 (Row 15), RR3-138.2, Table RR3-138.2 (Row 15), RR3-128, RR3-141.3, RR3-157, RR3-159, RR3-160, RR3-162, RR3-163, RR3-164, RR3-165, RR3-166, RR5-85, RR5-87, RR5-103, RR5-104, RR5-105, RR5-106 |
| 4.2.3 | SOA - Service Provider Personnel modify the routing data for an active Number Pool Block and broadcast to at least 4 LSMSs (2 EDR and 2 non-EDR systems) resulting in Partial Failure - Success | RR3-132, RR3-137.3, Table RR3-137.3 (Row 9), RR3-138.2, Table RR3-138.2 (Row 9), RR3-128, RR3-157, RR3-159, RR3-160, RR3-162, RR3-163, RR3-164, RR3-165, RR3-166, RR5-85, RR5-87, RR5-103, RR5-104, RR5-105, RR5-106 |
| 4.2.4 | NPAC OP GUI - NPAC Personnel re-send a failed Number Pool Block Modify Request for both EDR and non-EDR LSMSs – Success | RR3-140, RR3-142.2, RR3-185, RR3-192, RR3-193, RR3-194, RR3-195, RR3-196, RR3-197, RR5-85, RR5-86, RR5-75, RR5-77, RR5-78, RR5-79 |
| 4.2.5 | SOA – Service Provider Personnel modify an active Number Pool Block with the SOA Origination Indicator set to TRUE, using an LRN that does not exist on the NPAC SMS for that Service Provider. – Error | RR3-131 |
| 4.2.6 | SOA – Service Provider Personnel attempt to modify a Number Pool Block for a Number Pool Block that has a status of ‘active’ with a Failed SP List. – Error | RR3-161 |
| 4.2.7 | NPAC OP GUI – NPAC Personnel modify the SOA Origination Indicator for a Number Pool Block – Success | RR3-154, RR3-155 |
| 4.2.8 | NPAC OP GUI - NPAC Personnel modify the scheduled date/time and routing data for a scheduled Number Pool Block Create Event prior to the NPA-NXX-X Effective Date – Success | RR3-80.1, RR3-80.2, RR3-80.3, RR3-82.2 |
| 4.2.9 | SOA - Service Provider Personnel modify the routing data for an active Number Pool Block and broadcast to at least 4 LSMSs (1 EDR and 3 non-EDR systems) resulting in Partial Failure (2 non-EDR systems fail two different TNs, the EDR system and one non-EDR system is successful) – Success | RR3-132, RR3-137.3, Table RR3-137.3 (Row 2), RR3-138.2, Table RR3-138.2 (Row 2), RR3-128, RR3-157, RR3-159, RR3-160, RR3-162, RR3-163, RR3-164, RR3-165, RR3-166, RR5-85, RR5-87, RR5-103, RR5-104, RR5-105, RR5-106 |
| 4.2.10 | SOA - Service Provider Personnel modify the routing data for an active Number Pool Block and broadcast to at least 4 LSMSs (2 EDR and 2 non-EDR systems) resulting in a Partial Failure (1 non-EDR systems fails one TN, and 1 EDR system fails) – Success | RR3-132, RR3-137.3, Table RR3-137.3 (Row 9), RR3-138.2, Table RR3-138.2 (Row 9), RR3-128, RR3-157, RR3-159, RR3-160, RR3-162, RR3-163, RR3-164, RR3-165, RR3-166, RR5-85, RR5-87, RR5-103, RR5-104, RR5-105, RR5-106 |
| 4.2.11 | SOA - Service Provider Personnel modify the routing data for an active Number Pool Block and broadcast to at least 4 LSMSs (2 EDR and 2 non-EDR systems) resulting in a Partial Failure (1 non-EDR systems fails one TN, and 2 EDR systems fail) – Success | RR3-132, RR3-137.3, Table RR3-137.3 (Row 12), RR3-138.2, Table RR3-138.2 (Row 12), RR3-128, RR3-157, RR3-159, RR3-160, RR3-162, RR3-163, RR3-164, RR3-165, RR3-166, RR5-85, RR5-87, RR5-103, RR5-104, RR5-105, RR5-106 |
| 4.3 Delete Number Pool Block |  |  |
| 4.3.1 | NPAC OP GUI - NPAC Personnel attempt to delete a Number Pool Block - Error | RR3-169, RR5-107, RR5-108, RR5-109 |
| 4.3.2 | SOA - Service Provider Personnel attempt to delete a Number Pool Block over the SOA to NPAC SMS interface – Error | RR3-170 |
| 4.4 Query a Number Pool Block |  |  |
| 4.4.1 | SOA - Service Provider Personnel submit a Query Number Pool Block Request to the NPAC SMS using an NPA-NXX-X value as filter criteria. – Success | RR3-181, RR3-182 |
| 4.4.2 | LSMS - Service Provider Personnel submit a Number Pool Block query request over the LSMS to NPAC SMS Interface using a Number Pool Block ID as filter criteria – Success | RR3-181, RR3-182 |

This section has been updated per the 12/21 requirements and 12/16 flows.

This section has been updated to reflect the comments from the New Jersey reviews on 1/27/99 and 1/28/99 and for R2.0 Integration. Version was rolled to 0.5 dated 03/08/99.

This section has been updated to version 0.7, 08/01/99.

This section has been updated after review by Lockheed/ESI and is version 0.8 dated 11/24/99.

This section has been updated after industry review and is version 0.9 dated 01/24/00.

Note: Reference to NPA-NXX-X Holder SOA is the same thing as the Block Holder SOA.

This section has been updated after industry review and is version 0.10 dated 02/11/00.

## 4.1 Create Number Pool Block Test Cases:

| - A. | - TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: |  | - 4.1.1 |  | - SUT Priority: |  | - SOA LTI |  | - N/A |
|  |  |  |  |  |  |  | - SOA |  | - C |
|  |  |  |  |  |  |  | - non-EDR LSMS |  | - R |
|  |  |  |  |  |  |  | - EDR LSMS |  | - R |
|  | - Objective: |  | - SOA - Service Provider Personnel create a non-contaminated Number Pool Block – Success |  |  |  |  |  |  |
| - B. | - REFERENCES |  |  |  |  |  |  |  |  |
|  | - NANC Change Order Revision Number: |  |  |  | - Change Order Number(s): |  |  |  |  |
|  | - NANC FRS Version Number: |  |  |  | - Relevant Requirement(s): |  | - RR3-124, RR3-125, RR3-126, RR3-130, RR3-132, RR3-144, RR3-146, RR3-150, RR3-151, RR3-152, RR3-143, RR3-180, RR5-85, RR5-86, RR5-87, RR5-89 |  |  |
|  | - NANC IIS Version Number: |  |  |  | - Relevant Flow(s): |  | - 2.1 Number Pool Block Create by SOA - 2.3.1 Number Pool Block Create Broadcast to Local SMS - 2.3.2 Number Pool Block Create: Successful Broadcast |  |  |
| - C. | - PREREQUISITE |  |  |  |  |  |  |  |  |
|  | - Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | - Prerequisite NPAC Setup: |  | Verify that there are no contaminated TNs or ‘pending-like’ Subscription Versions for the range of TNs in the NPA-NXX-X. |  |  |  |  |  |  |
|  | - Prerequisite SP Setup: |  | - Verify that the NPA-NXX-X exists for the Number Pool Block that Service Provider Personnel will create during this Test Case. - Verify that the current date is equal to or greater than the NPA-NXX-X Effective Date. |  |  |  |  |  |  |
| - D. | - TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| - Row # | - NPAC or SP | - Test Step |  | - NPAC or SP |  | - Expected Result |  |  |  |
| - 1. | - SP | - Using the SOA, Service Provider Personnel, submit a M-ACTION numberPoolBlock-Create request to the NPAC SMS to create a Number Pool Block. - The request must include the following attributes: - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | - NPAC |  | - The NPAC SMS receives the M-ACTION numberPoolBlock-Create Request. - The NPAC SMS verifies the following information: - The requesting SOA is the NPA-NXX-X Holder SOA. - The serviceProvNPA-NXX-X object exists for the NPA-NXX-X (respective NPA-NXX-X information). - All attributes specified are valid. - A numberPoolBlockNPAC object does not already exist for the NPA-NXX-X (a duplicate Number Pool Block does not already exist). - The current date is greater than or equal to the NPA-NXX-X-EffectiveTimeStamp. - There are not any ‘pending-like, no-active’ Subscription Version objects within the given TN range. |  |  |  |
| - 2. | - NPAC | - The NPAC SMS issues an M-CREATE Request numberPoolBlockNPAC to itself. - The NPAC SMS sets the numberPoolBlockSOA-Origination Indicator to TRUE. - The NPAC SMS sets the numberPoolBlockStatus to 'sending'. - The NPAC SMS sets the following timestamps to the current date and time: - numberPoolBlockCreationTimeStamp - numberPoolBlockActivationTimeStamp - numberPoolBlockBroadcastTimeStamp - numberPoolBlockModifiedTimeStamp |  | - NPAC |  | - The NPAC SMS issues an M-CREATE Response numberPoolBlockNPAC to itself. |  |  |  |
| - 3. | - NPAC | - The NPAC SMS issues an M-CREATE Request subscriptionVersionNPAC to itself. - The NPAC SMS sets the LNP Type to ‘POOL’ for the Subscription Versions it creates within the 1K Block. - The NPAC SMS sets the Subscription Versions to ‘sending’. - The NPAC SMS sets the following timestamps to the current date and time for the Subscription Versions: - subscriptionModifiedTimeStamp - subscriptionActivationTimeStamp - subscriptionBroadcastTimeStamp - subscriptionCreationTimeStamp |  | - NPAC |  | - The NPAC SMS issues an M-CREATE Response subscriptionVersionNPAC to itself. |  |  |  |
| - 4. | - NPAC | - The NPAC SMS issues an M-ACTION Response numberPoolBlock-Create to the respective NPA-NXX-X Holder SOA that initiated the Number Pool Block Create request. |  | - SP |  | - The NPA-NXX-X Holder SOA receives the M-ACTION Response from the NPAC SMS. |  |  |  |
| - 5. | - NPAC | - The NPAC SMS issues an M-EVENT-REPORT objectCreation for the numberPoolBlockNPAC to the NPA-NXX-X Holder SOA. - The following attributes are sent in the objectCreation notification: - numberPoolBlockId - numberPoolBlockSOA-Origination - numberPoolBlockCreationTimeStamp - numberPoolBlockStatus - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | - SP |  | - The NPA-NXX-X Holder SOA issues an M-EVENT-REPORT Confirmation to the NPAC SMS. |  |  |  |
| - 6. | - NPAC | - The NPAC SMS issues an M-ACTION Request subscriptionVersionLocalSMS-Create to the non-EDR LSMSs in the region that are accepting downloads for this NPA-NXX. - At the same time as step 6.1, the NPAC SMS issues an M-CREATE Request numberPoolBlock to the EDR LSMSs in the region that are accepting downloads for this NPA-NXX. |  | - SP |  | - The non-EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-ACTION Request, verify that the action is valid and return an M-ACTION Response subscriptionVersionLocalSMS-Create. - The EDR LSMSs that are accepting downloads for this NPA-NXX receives the M-CREATE Request, and return an M-CREATE Response numberPoolBlock. - The non-EDR LSMSs proceed to execute all the creates specified by the M-ACTION Request and issues an M-EVENT-REPORT subscriptionVersionLocalSMS-CreateResults specifying the Subscription Versions were successfully created. - The NPAC SMS waits for all subscriptionVersionLocalSMS-CreateResults notifications for a tunable amount of time. |  |  |  |
| - 7. | - NPAC | - Upon the first successful response from an LSMS, the NPAC SMS sets the following timestamps to the current date and time: - numberPoolBlockActivationCompleteTimeStamp - subscriptionActivationCompleteTimeStamp - numberPoolBlockModifiedTimeStampsubscriptionModifiedTimeStamp |  | - NPAC |  | The NPAC SMS responds to each of the M-EVENT-REPORT subscriptionVersionLocalSMS-CreateResults as it receives these notifications with M-EVENT-REPORT Confirmations. |  |  |  |
| - 8. | - NPAC | - The NPAC SMS issues M-SET Request subscriptionVersionNPAC to itself. - The NPAC SMS updates the following attributes for each Subscription Version within the 1K Block with LNP Type set to ‘POOL’: - sets the subscriptionVersionStatus to 'active'. - sets the Subscription Version Failed SP List to empty. - sets the subscriptionModifiedTimeStamp to the current date and time. - The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself to update the following attributes: - sets the numberPoolBlockStatus to 'active'. - sets the Number Pool Block Failed SP List to empty. - sets the numberPoolBlockModifiedTimeStamp to the current date and time. |  | - NPAC |  | - The NPAC SMS issues an M-SET subscriptionVersionNPAC Response to itself. - The NPAC SMS issues an M-SET numberPoolBlockNPAC Response to itself. |  |  |  |
| - 9. | - NPAC | - The NPAC SMS determines the SOA Origination Indicator is set to TRUE and issues an M-EVENT-REPORT numberPoolBlockStatusAttributeValueChange to the NPA-NXX-X Holder SOA to set the Number Pool Block status to 'active' and the Failed SP List to empty. |  | - SP |  | - The NPA-NXX-X Holder SOA receives the M-EVENT-REPORT from the NPAC SMS and issues an M-EVENT-REPORT Confirmation back to the NPAC SMS. |  |  |  |
| - 10. | - NPAC | - NPAC Personnel perform a query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that Service Provider Personnel created during this Test Case. |  | - NPAC |  | - Verify the Number Pool Block exists with status of ‘active’ and an empty Failed SP List. - Verify the 1K Block of Subscription Versions exist with LNP Type set to ‘POOL’, a status of ‘active’ and an empty Failed SP List. |  |  |  |
| - 11. | - SP – Optional | - Service Provider Personnel perform a local query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that Service Provider Personnel created during this Test Case. |  | - SP |  | - Verify the Number Pool Block exists with status of ‘active’ and an empty Failed SP List on the SOA. - Verify the Number Pool Block exists on the EDR LSMS. - Verify the 1K Block of Subscription Versions exist with LNP Type set to ‘POOL’, a status of ‘active’ on the non-EDR LSMS. |  |  |  |
| - 12. | - SP – Conditional | - Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that Service Provider Personnel created during this Test Case. |  | - SP |  | - Verify the Number Pool Block exists on the NPAC SMS with status of ‘active’ and an empty Failed SP List. - Verify the 1K Block of Subscription Versions exist on the NPAC SMS with LNP Type set to ‘POOL’, a status of ‘active’ and an empty Failed SP List. |  |  |  |

| - A. | - TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: |  | - 4.1.2 |  | - SUT Priority: |  | - SOA LTI |  | - N/A |
|  |  |  |  |  |  |  | - SOA |  | - O |
|  |  |  |  |  |  |  | - non-EDR LSMS |  | - R |
|  |  |  |  |  |  |  | - EDR LSMS |  | - R |
|  | - Objective: |  | - NPAC OP GUI - NPAC Personnel schedule a Number Pool Block Create for a contaminated Block to be run at a future date, and the NPAC SMS activates upon scheduled date and time – Success |  |  |  |  |  |  |
| - B. | - REFERENCES |  |  |  |  |  |  |  |  |
|  | - NANC Change Order Revision Number: |  |  |  | - Change Order Number(s): |  |  |  |  |
|  | - NANC FRS Version Number: |  |  |  | - Relevant Requirement(s): |  | - RR3-75.2, RR5-92 |  |  |
|  | - NANC IIS Version Number: |  |  |  | - Relevant Flow(s): |  | - 2.2 Number Pool Block Create by NPAC SMS - 2.3 Number Pool Block Create Broadcast: Successful - 2.3.1 Number Pool Block Create: Broadcast Successful to Local SMS - 2.3.2 Number Pool Block Create: Successful Broadcast |  |  |
| - C. | - PREREQUISITE |  |  |  |  |  |  |  |  |
|  | - Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | - Prerequisite NPAC Setup: |  | - Verify that the NPA-NXX-X for the Number Pool Block Create Event to be scheduled exists and the Effective Date has passed. - Verify that a respective Number Pool Block Create Event does not yet exist on the NPAC SMS. (In the original NPA-NXX-X create the SOA Origination Flag was set to TRUE but the Service Provider did not submit the Number Pool Block Create and has requested the NPAC to do it on his behalf.) - Verify that all possible cases of ‘active-like’ Subscription Versions exist for the Number Pool Block to be scheduled. - Verify that there are not any ‘pending-like, no-active’ Subscription Versions for the Number Pool Block to be scheduled. |  |  |  |  |  |  |
|  | - Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| - D. | - TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| - Row # | - NPAC or SP | - Test Step |  | - NPAC or SP |  | - Expected Result |  |  |  |
| - 1. | - NPAC | - Using the NPAC OP GUI, NPAC Personnel submit a request to schedule the Number Pool Block Create for a future date. |  | - NPAC |  | The NPAC SMS schedules the Number Pool Block Create Event. |  |  |  |
| - 2. | - NPAC | - NPAC Personnel perform a query for the Number Pool Block Create Event that was scheduled during this Test Case. |  | - NPAC |  | - Verify the Number Pool Block Create Event has been scheduled to run on the date and time entered in Row 1 above. |  |  |  |
| - 3. | - NPAC | - The Scheduled Date/Time of the Number Pool Block Create Event is reached. |  | - NPAC |  | - On the scheduled date specified in the Number Pool Block Create Event, the NPAC SMS issues an M-ACTION Request numberPoolBlock-Create to itself. - The NPAC SMS verifies the following information: - The serviceProvNPA-NXX-X object exists for the NPA-NXX-X (respective NPA-NXX-X information). - All attributes specified are valid. - A numberPoolBlockNPAC object does not already exist for the NPA-NXX-X (a duplicate Number Pool Block does not already exist). - The current date is greater than or equal to the NPA-NXX-X-EffectiveTimeStamp. - There are not any ‘pending-like, no-active’ Subscription Version objects within the given TN range. |  |  |  |
| - 4. | - NPAC | - The NPAC SMS issues an M-CREATE Request numberPoolBlockNPAC to itself. - The NPAC SMS sets the numberPoolBlockSOA-Origination Indicator to FALSE. - The NPAC SMS sets the numberPoolBlockStatus to ‘sending’. - The NPAC SMS sets the following timestamps to the current date and time: - numberPoolBlockCreationTimeStamp - numberPoolBlockActivationTimeStamp - numberPoolBlockBroadcastTimeStamp - numberPoolBlockModifiedTimeStamp are set to the current date and time. |  | - NPAC |  | - The NPAC SMS issues an M-CREATE Response numberPoolBlockNPAC to itself. |  |  |  |
| - 5. | - NPAC | - For each non-ported TN within the 1K Block, the NPAC SMS issues an M-CREATE Request subscriptionVersionNPAC to itself. - The NPAC SMS sets the LNP Type to ‘POOL’ for the Subscription Versions it creates within the 1K Block. - The NPAC SMS sets the Subscription Version to ‘sending’. - The NPAC SMS sets the following timestamps to the current date and time for the Subscription Versions: - subscriptionModifiedTimeStamp - subscriptionActivationTimeStamp - subscriptionBroadcastTimeStamp - subscriptionCreationTimeStamp |  | - NPAC |  | - The NPAC SMS issues an M-CREATE Response subscriptionVersionNPAC to itself. |  |  |  |
| - 6. | - NPAC | The NPAC SMS issues an M-ACTION Response numberPoolBlock-Create to itself. |  |  |  |  |  |  |  |
| - 7. | - NPAC | - The NPAC SMS issues an M-ACTION Request subscriptionVersionLocalSMS-Create to the non-EDR LSMSs in the region that are accepting downloads for this NPA-NXX. - At the same time as step 7.1, the NPAC SMS issues an M-CREATE Request numberPoolBlock to the EDR LSMSs in the region that are accepting downloads for this NPA-NXX. |  | - SP |  | - The non-EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-ACTION Request, verify that the action is valid and return an M-ACTION Response subscriptionVersionLocalSMS-Create. - The EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-CREATE Request, and return an M-CREATE Response numberPoolBlock. - The non-EDR LSMSs proceed to execute all the creates specified by the M-ACTION Request and issues an M-EVENT-REPORT subscriptionVersionLocalSMS-CreateResults specifying the Subscription Versions were successfully created. - The NPAC SMS waits for all subscriptionVersionLocalSMS-CreateResults notification for a tunable amount of time. |  |  |  |
| - 8. | - NPAC | Upon the first successful response from an LSMS, the NPAC SMS sets the following timestamps to the current date and time: - numberPoolBlockActivationCompleteTimeStamp - subscriptionActivationCompleteTimeStamp - numberPoolBlockModifiedTimeStamp - subscriptionModifiedTimeStamp |  | - NPAC |  | The NPAC SMS responds to each of the M-EVENT-REPORT subscriptionVersionLocalSMS-CreateResults as it receives these notifications with M-EVENT-REPORT Confirmations. |  |  |  |
| - 9. | - NPAC | - The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself and updates the following attributes for each Pooled Subscription Version within the 1K Block: - sets the subscriptionVersionStatus to ‘active’. - sets the Subscription Version Failed SP List to empty. - sets the subscriptionModifiedTimeStamp to the current date and time. - The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself and updates the following attributes: - sets the numberPoolBlockStatus to 'active' - sets the Number Pool Block Failed SP List to empty. - sets the numberPoolBlockModifiedTimeStamp to the current date and time. |  | - NPAC |  | - The NPAC SMS issues an M-SET subscriptionVersionNPAC Response to itself. - The NPAC SMS issues an M-SET numberPoolBlockNPAC Response to itself. |  |  |  |
| - 10. | - NPAC | - The NPAC SMS determines the SOA Origination Indicator is set to FALSE and terminates processing here. |  |  |  |  |  |  |  |
| - 11. | - NPAC | - NPAC Personnel perform a query for the Number Pool Block, the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that were created during this Test Case, and the ‘active-like’ Subscription Versions that do not have LNP Type set to ‘POOL’ but are within the 1K Block. |  | - NPAC |  | - Verify the Number Pool Block exists with a status of ‘active’ and an empty Failed SP List. - Verify the 1K Block of Subscription Versions exists with LNP Type set to ‘POOL’, an ‘active’ status and an empty Failed SP List. - Verify that the ‘active-like’ Subscription Versions do not have LNP Type set to ‘POOL’ and were not modified when the Number Pool Block was created during this Test Case. |  |  |  |
| - 12. | - SP – Optional | - Service Provider Personnel perform a local query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that were created during this Test Case. |  | - SP |  | - Verify the Number Pool Block exists on the EDR LSMS. - Verify the 1K Block of Subscription Versions exists on the non-EDR LSMS. |  |  |  |
| - 13. | - SP – Conditional | - Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that were created during this Test Case. |  | - SP |  | - Verify the Number Pool Block exists with a status of ‘active’ and an empty Failed SP List on the NPAC SMS. - Verify the 1K Block of Subscription Versions exists with LNP Type set to ‘POOL’, an ‘active’ status and an empty Failed SP List on the NPAC SMS. |  |  |  |

| - A. | - TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: |  | - 4.1.3 |  | - SUT Priority: |  | - SOA LTI |  | - N/A |
|  |  |  |  |  |  |  | - SOA |  | - C |
|  |  |  |  |  |  |  | - non-EDR LSMS |  | - O |
|  |  |  |  |  |  |  | - EDR LSMS |  | - O |
|  | - Objective: |  | - SOA - Service Provider Personnel create a Number Pool Block that already exists. - Error |  |  |  |  |  |  |
| - B. | - REFERENCES |  |  |  |  |  |  |  |  |
|  | - NANC Change Order Revision Number: |  |  |  | - Change Order Number(s): |  |  |  |  |
|  | - NANC FRS Version Number: |  |  |  | - Relevant Requirement(s): |  | - RR3-129, RR3-131 |  |  |
|  | - NANC IIS Version Number: |  |  |  | - Relevant Flow(s): |  | - 2.1 Number Pool Block Create by SOA |  |  |
| - C. | - PREREQUISITE |  |  |  |  |  |  |  |  |
|  | - Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | - Prerequisite NPAC Setup: |  |  |  |  |  |  |  |  |
|  | - Prerequisite SP Setup: |  | - Verify that the NPA-NXX-X exists for the Number Pool Block that Service Provider Personnel will create during this Test Case. - Verify that the current date is equal to or greater than the respective NPA-NXX-X Effective Date. - Verify that a Number Pool Block with a status other than ‘old’ with an empty Failed SP List already exists for the NPA-NXX-X that Service Provider Personnel will specify in their Number Pool Block Create Request and make a note of the Block ID. |  |  |  |  |  |  |
| - D. | - TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| - Row # | - NPAC or SP | - Test Step |  | - NPAC or SP |  | - Expected Result |  |  |  |
| - 1. | - SP | - Using the SOA, Service Provider Personnel, submit a M-ACTION numberPoolBlock-Create request to the NPAC SMS to create a Number Pool Block. - The request must include the following attributes: - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | - NPAC |  | - The NPAC SMS receives the M-ACTION numberPoolBlock-Create request. - The NPAC SMS verifies the following information: - The requesting SOA is the NPA-NXX-X Holder SOA. - The serviceProvNPA-NXX-X object exists for the NPA-NXX-X (respective NPA-NXX-X information). - All attributes specified are valid. - A numberPoolBlockNPAC object already exists for the NPA-NXX-X (a duplicate Number Pool Block with a status of other than ‘old’ with an empty Failed SP List already exist). (This violates system requirements.) |  |  |  |
| - 3. | - NPAC | - The NPAC SMS rejects the request and issues an M-ACTION Error Response to the NPA-NXX-X Holder SOA indicating the error and further processing is terminated. |  | - SP |  | - The NPA-NXX-X Holder SOA receives the M-ACTION Error Response. |  |  |  |
| - 4. | - NPAC | - NPAC Personnel perform a query for the Number Pool Block and 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that Service Provider Personnel attempted to create during this Test Case. |  | - NPAC |  | - Verify the original Number Pool Block with the original Block ID is the only one that exists on the NPAC SMS and that it has not been modified. - Verify the original Subscription Versions with LNP Type set to ‘POOL’ are the only ones that exist on the NPAC SMS. |  |  |  |
| - 5. | - SP – Optional | - Service Provider Personnel perform a local query for the Number Pool Block and 1K Block of Subscription Versions of Subscription Versions with LNP Type set to ‘POOL’ that Service Provider Personnel attempted to create during this Test Case. |  | - SP |  | - Verify the original Number Pool Block with the original Block ID is the only one that exists on the SOA and/or EDR LSMS and that it has not been modified. - Verify the original Subscription Versions with LNP Type set to ‘POOL’ are the only ones that exist on the non-EDR LSMS. |  |  |  |
| - 6. | - SP – Conditional | - Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that Service Provider Personnel attempted to create during this Test Case. |  | - SP |  | - Verify the original Number Pool Block with the original Block ID is the only one that exists on the NPAC SMS and that it has not been modified. - Verify the original Subscription Versions with LNP Type set to ‘POOL’ are the only ones that exist on the NPAC SMS |  |  |  |

| - A. | - TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: |  | - 4.1.4 |  | - SUT Priority: |  | - SOA LTI |  | - N/A |
|  |  |  |  |  |  |  | - SOA |  | - C |
|  |  |  |  |  |  |  | - non-EDR LSMS |  | - O |
|  |  |  |  |  |  |  | - EDR LSMS |  | - O |
|  | - Objective: |  | - SOA – Service Provider Personnel create a Number Pool Block prior to the NPA-NXX-X Effective Date – Error |  |  |  |  |  |  |
| - B. | - REFERENCES |  |  |  |  |  |  |  |  |
|  | - NANC Change Order Revision Number: |  |  |  | - Change Order Number(s): |  |  |  |  |
|  | - NANC FRS Version Number: |  |  |  | - Relevant Requirement(s): |  | - RR3-127 |  |  |
|  | - NANC IIS Version Number: |  |  |  | - Relevant Flow(s): |  | - 2.2 Number Pool Block Create by NPAC SMS |  |  |
| - C. | - PREREQUISITE |  |  |  |  |  |  |  |  |
|  | - Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | - Prerequisite NPAC Setup: |  | - Verify the NPA-NXX-X exists with the SOA Origination Indicator set to TRUE for the Number Pool Block that is to be created during this Test Case. - Verify the current date is less than the NPA-NXX-X Effective Date. |  |  |  |  |  |  |
|  | - Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| - D. | - TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| - Row # | - NPAC or SP | - Test Step |  | - NPAC or SP |  | - Expected Result |  |  |  |
| - 1. | - SP | - Prior to the NPA-NXX-X Effective Date, using the SOA, Service Provider Personnel, submit a M-ACTION numberPoolBlock-Create request to the NPAC SMS to create a Number Pool Block. - The request must include the following attributes: - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | - NPAC |  | - The NPAC SMS receives the M-ACTION numberPoolBlock-Create request. - The NPAC SMS verifies the following information: - The requesting SOA is the NPA-NXX-X Holder SOA. - The serviceProvNPA-NXX-X object exists for the NPA-NXX-X (respective NPA-NXX-X information). - All attributes specified are valid. - A numberPoolBlockNPAC object does not already exists for the NPA-NXX-X (a duplicate Number Pool Block does not already exist). - The scheduled date is prior to the NPA-NXX-X Effective Timestamp. (This violates system requirements.) |  |  |  |
| - 2. | - NPAC | - The NPAC SMS rejects the request and issues an M-ACTION Error Response indicating the error. Further processing is terminated. |  | - SP |  | - The NPA-NXX-X Holder SOA receives the M-ACTION Error Response. |  |  |  |
| - 3. | - NPAC | - NPAC Personnel perform a query for the Number Pool Block that Service Provider Personnel attempted to create during this Test Case. |  | - NPAC |  | - Verify the Number Pool Block was not created on the NPAC SMS. |  |  |  |
| - 4. | - SP – Optional | - Service Provider Personnel perform a local query for the Number Pool Block that Service Provider Personnel attempted to create during this Test Case. |  | - SP |  | - Verify the Number Pool Block does not exist on the SOA and/or EDR LSMS. - Verify that the 1K Block of Subscription Versions do not exist on the non-EDR LSMS. |  |  |  |
| - 5. | - SP - Conditional | - Service Provider Personnel perform an NPAC SMS query for the Number Pool Block that Service Provider Personnel attempted to create during this Test Case. |  | - SP |  | - Verify the Number Pool Block was not created on the NPAC SMS. - Verify that the 1K Block of Subscription Versions do not exist on the NPAC SMS. |  |  |  |

| - A. | - TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: |  | - 4.1.5 |  | - SUT Priority: |  | - SOA LTI |  | - N/A |
|  |  |  |  |  |  |  | - SOA |  | - C |
|  |  |  |  |  |  |  | - non-EDR LSMS |  | - O |
|  |  |  |  |  |  |  | - EDR LSMS |  | - O |
|  | - Objective: |  | - SOA - Service Provider Personnel attempt to create a Number Pool Block when ‘pending-like, no-active’ Subscription Versions exist – Error |  |  |  |  |  |  |
| - B. | - REFERENCES |  |  |  |  |  |  |  |  |
|  | - NANC Change Order Revision Number: |  |  |  | - Change Order Number(s): |  |  |  |  |
|  | - NANC FRS Version Number: |  |  |  | - Relevant Requirement(s): |  | - RR3-148 |  |  |
|  | - NANC IIS Version Number: |  |  |  | - Relevant Flow(s): |  | - 2.1 Number Pool Block Create by SOA |  |  |
| - C. | - PREREQUISITE |  |  |  |  |  |  |  |  |
|  | - Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | - Prerequisite NPAC Setup: |  | - Verify that the NPA-NXX-X for the Number Pool Block that Service Provider Personnel will attempt to create during this Test Case exists and the Effective Date has passed. - Verify that a respective Number Pool Block does not exist on the NPAC SMS. - Verify that all-possible cases of ‘pending-like, no-active’ Subscription Versions exist for the Number Pool Block to be created. |  |  |  |  |  |  |
|  | - Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| - D. | - TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| - Row # | - NPAC or SP | - Test Step |  | - NPAC or SP |  | - Expected Result |  |  |  |
| - 1. | - SP | - Using the SOA, Service Provider Personnel, submit a M-ACTION numberPoolBlock-Create request to the NPAC SMS to create a Number Pool Block. - The request must include the following attributes: - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | - NPAC |  | - The NPAC SMS receives the M-ACTION numberPoolBlock-Create request. - The NPAC SMS verifies the following information: - The requesting SOA is the NPA-NXX-X Holder SOA. - The serviceProvNPA-NXX-X object exists for the NPA-NXX-X (respective NPA-NXX-X information). - All attributes specified are valid. - A numberPoolBlockNPAC object does not already exist for the NPA-NXX-X (a duplicate Number Pool Block does not already exist). - The current date is greater than or equal to the NPA-NXX-X-EffectiveTimeStamp. - Determines there are ‘pending-like, no-active’ Subscription Version objects within the given TN range. (This violates system requirements.) |  |  |  |
| - 2. | - NPAC | - The NPAC SMS issues an M-ACTION Error Response to the NPA-NXX-X Holder SOA indicating the error. Further processing is terminated. - (The Number Pool Block is not created on the NPAC SMS.) |  | - SP |  | - The NPA-NXX-X Holder SOA receives the M-ACTION Error Response. |  |  |  |
| - 3. | - NPAC | - NPAC Personnel perform a query for the Number Pool Block that Service Provider Personnel attempted to create during this Test Case. |  | - NPAC |  | - Verify the Number Pool Block was not created on the NPAC SMS |  |  |  |
| - 4. | - SP – Optional | - Service Provider Personnel perform a local query for the Number Pool Block that Service Provider Personnel attempted to create during this Test Case. |  | - SP |  | - Verify the Number Pool Block does not exist on the SOA and/or EDR LSMS. - Verify that the 1K Block of Subscription Versions do not exist on the non-EDR LSMS. |  |  |  |
| - 5. | - SP – Conditional | - Service Provider Personnel perform an NPAC SMS query for the Number Pool Block that Service Provider Personnel attempted to create during this Test Case. |  | - SP |  | - Verify the Number Pool Block was not created on the NPAC SMS. - Verify that the 1K Block of Subscription Versions do not exist on the NPAC SMS. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.1.6 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | R |
|  |  |  |  |  |  |  | EDR LSMS |  | R |
|  | Objective: |  | NPAC OP GUI - NPAC Personnel re-schedule a Number Pool Block Create Event to run immediately. The initial Number Pool Block Create Request that was initiated by the NPA-NXX-X Holder SOA has failed due to ‘pending-like, no active’ Subscription Versions. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-75.2, RR3-76.2, RR3-77, RR3-81.1, RR3-81.2, RR3-82.2, RR5-90, RR5-91, RR5-92, RR5-93, RR5-94, RR5-96, RR5-97 |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  | 2.2 Number Pool Block Create by NPAC SMS 2.3 Number Pool Block Create Broadcast: Successful 2.3.1 Number Pool Block Create Broadcast Successful to Local SMS 2.3.2 Number Pool Block Create: Successful Broadcast |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  | 4.1.5 SOA - Service Provider Personnel attempt to create a Number Pool Block when ‘pending-like, no-active’ Subscription Versions exist – Error |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that the NPA-NXX-X for the Number Pool Block Create Event to be re-scheduled during this Test Case exists and the Effective Date has passed. - Cancel the ‘pending-like’ Subscription Versions within the Number Pool Block to be re-scheduled during this Test Case. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel submit a request to re-schedule a Number Pool Block Create Event to run immediately. The NPAC SMS issues an M-ACTION numberPoolBlock-Create request to create the Number Pool Block. The following attributes are required: - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | NPAC |  | - The NPAC SMS receives the M-ACTION numberPoolBlock-Create request. - The NPAC SMS verifies the following information: - The serviceProvNPA-NXX-X object exists for the NPA-NXX-X (respective NPA-NXX-X information). - All attributes specified are valid. - A numberPoolBlockNPAC object does not already exist for the NPA-NXX-X (a duplicate Number Pool Block does not already exist) or if one exists it has a status of ‘old’ with an empty Failed SP List. - The current date is greater than or equal to the NPA-NXX-X-EffectiveTimeStamp. - There are not any ‘pending-like, no-active’ Subscription Version objects within the 1K Block. |  |  |  |
| - 2. | - NPAC | - For each non-ported TN within the 1K Block, the NPAC SMS issues an M-CREATE Request numberPoolBlockNPAC to itself. - The NPAC SMS sets the numberPoolBlockSOA-Origination Indicator to FALSE. - The NPAC SMS sets the numberPoolBlockStatus to ‘sending’. - The NPAC SMS sets the following timestamps to the current date and time: - numberPoolBlockCreationTimeStamp - numberPoolBlockActivationTimeStamp - numberPoolBlockBroadcastTimeStamp - numberPoolBlockModifiedTimeStamp are set to the current date and time. |  | - NPAC |  | - The NPAC SMS issues an M-CREATE Response numberPoolBlockNPAC to itself. |  |  |  |
| - 3. | - NPAC | - The NPAC SMS issues an M-CREATE Request subscriptionVersionNPAC to itself. - The NPAC SMS sets the LNP Type to ‘POOL’ for the Subscription Versions it creates within the 1K Block. - The NPAC SMS sets the Subscription Versions to ‘sending’. - The NPAC SMS sets the following timestamps to the current date and time for the Subscription Versions: - subscriptionModifiedTimeStamp - subscriptionActivationTimeStamp - subscriptionBroadcastTimeStamp - subscriptionCreationTimeStamp |  | - NPAC |  | - The NPAC SMS issues an M-CREATE Response subscriptionVersionNPAC to itself. |  |  |  |
| - 4. | - NPAC | - The NPAC SMS issues an M-ACTION Response numberPoolBlock-Create to itself. |  |  |  |  |  |  |  |
| - 5. | - NPAC | - The NPAC SMS issues an M-ACTION Request subscriptionVersionLocalSMS-Create to the non-EDR LSMSs in the region that are accepting downloads for this NPA-NXX. - At the same time as step 5.1, the NPAC SMS issues an M-CREATE Request numberPoolBlock to the EDR LSMSs in the region that are accepting downloads for this NPA-NXX. |  | - SP |  | - The non-EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-ACTION Request, verify that the action is valid and return an M-ACTION Response subscriptionVersionLocalSMS-Create. - The EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-CREATE Request, and return an M-CREATE Response numberPoolBlock. - The non-EDR LSMSs proceed to execute all the creates specified by the M-ACTION Request and issues an M-EVENT-REPORT specifying the Subscription Versions were successfully created. - The NPAC SMS waits for all subscriptionVersionLocalSMS-CreateResults notification for a tunable amount of time. |  |  |  |
| - 6. | - NPAC | Upon the first successful response from an LSMS, the NPAC SMS sets the following timestamps to the current date and time: - numberPoolBlockActivationCompleteTimeStamp - subscriptionActivationCompleteTimeStamp - numberPoolBlockModifiedTimeStamp - subscriptionModifiedTimeStamp |  | - NPAC |  | The NPAC SMS responds to each of the M-EVENT-REPORT subscriptionVersionLocalSMS-CreateResults as it receives these notifications with M-EVENT-REPORT Confirmations. |  |  |  |
| - 7. | - NPAC | - The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself and updates the following attributes for each Pooled Subscription Version within the 1K Block: - sets the subscriptionVersionStatus to ‘active’. - sets the Subscription Version Failed SP List to empty. - Sets the subscriptionModifiedTimeStamp to the current date and time. - The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself and updates the following attributes: - sets the numberPoolBlockStatus to 'active'. - sets the Number Pool Block Failed SP List to empty. - sets the numberPoolBlockModifiedTimeStamp to the current date and time. |  | - NPAC |  | - The NPAC SMS issues an M-SET subscriptionVersionNPAC Response to itself. - The NPAC SMS issues an M-SET numberPoolBlockNPAC Response to itself. |  |  |  |
| - 8. | - NPAC | - The NPAC SMS determines the SOA Origination Indicator is set to FALSE and terminates processing here. |  |  |  |  |  |  |  |
| - 9. | - NPAC | - NPAC Personnel perform a query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that NPAC Personnel re-scheduled during this Test Case. |  | - NPAC |  | - Verify the Number Pool Block exists with a status of ‘active’ and an empty Failed SP List. - Verify the 1K Block of Subscription Versions exists with LNP Type set to ‘POOL’, a status of ‘active’ and an empty Failed SP List. |  |  |  |
| - 10. | - SP –Optional | - Service Provider Personnel perform a local query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that NPAC Personnel re-scheduled during this Test Case. |  | - SP |  | - Verify that the Number Pool Block exists on the EDR LSMS. - Verify that the 1K Block of Subscription Versions exist on the non-EDR LSMS. |  |  |  |
| - 11. | - SP – Conditional | - Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that NPAC Personnel re-scheduled during this Test Case. |  | - SP |  | - Verify the Number Pool Block exists on the NPAC SMS with status of ‘active’ and an empty Failed SP List. - Verify the 1K Block of Subscription Versions exist on the NPAC SMS with LNP Type set to ‘POOL’, a status of ‘active’ and an empty Failed SP List. |  |  |  |

| - A. | - TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: |  | - 4.1.7 |  | - SUT Priority: |  | - SOA LTI |  | - N/A |
|  |  |  |  |  |  |  | - SOA |  | - O |
|  |  |  |  |  |  |  | - non-EDR LSMS |  | - O |
|  |  |  |  |  |  |  | - EDR LSMS |  | - O |
|  | - Objective: |  | - NPAC OP GUI – NPAC Personnel attempt to schedule a Number Pool Block create to execute immediately, when ‘pending-like, no-active’ Subscription Versions exist. Generate the ‘pending-like, no-active’ Subscription Version Report to a file. – Error |  |  |  |  |  |  |
| - B. | - REFERENCES |  |  |  |  |  |  |  |  |
|  | - NANC Change Order Revision Number: |  |  |  | - Change Order Number(s): |  |  |  |  |
|  | - NANC FRS Version Number: |  |  |  | - Relevant Requirement(s): |  | - RR3-147, RR3-82.3, RR3-83.1, RR3-83.2, RR3-83.3, RR3-83.4 |  |  |
|  | - NANC IIS Version Number: |  |  |  | - Relevant Flow(s): |  | - 2.2 Number Pool Block Create by NPAC SMS |  |  |
| - C. | - PREREQUISITE |  |  |  |  |  |  |  |  |
|  | - Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | - Prerequisite NPAC Setup: |  | - Verify that the NPA-NXX-X for Number Pool Block that NPAC Personnel will attempt to schedule during this Test Case exists and the Effective Date has passed. - Verify that a respective Number Pool Block Create Event does not exist on the NPAC SMS. - Verify that all-possible cases of ‘pending-like, no-active’ Subscription Versions exist for the Number Pool Block to be created. |  |  |  |  |  |  |
|  | - Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| - D. | - TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| - Row # | - NPAC or SP | - Test Step |  | - NPAC or SP |  | - Expected Result |  |  |  |
| - 1. | - NPAC | - Using the NPAC OP GUI, NPAC Personnel submit a request to schedule the Number Pool Block Create Event to run immediately. The NPAC OP GUI issues an M-ACTION numberPoolBlock-Create request to create the Number Pool Block. The following attributes are required: - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | - NPAC |  | - The NPAC SMS receives the M-ACTION numberPoolBlock-Create request. - The NPAC SMS verifies the following information: - The serviceProvNPA-NXX-X object exists for the NPA-NXX-X (respective NPA-NXX-X information). - All attributes specified are valid. - A numberPoolBlockNPAC object does not already exist for the NPA-NXX-X (a duplicate Number Pool Block does not already exist). - The current date is greater than or equal to the serviceProvNPA-NXX-X-EffectiveTimeStamp. - Determines there are ‘pending-like, no-active’ Subscription Version objects within the given TN range. (This violates system requirements.) |  |  |  |
| - 2. | - NPAC | - The NPAC rejects the request and returns an error message containing the contents of the Pending-Like, No-Active Report to the user and indicates the Number Pool Block was not created. |  | - NPAC |  | - The NPAC OP GUI receives the error message indicating the request was rejected. |  |  |  |
| - 3. | - NPAC | - Using the NPAC OP GUI, NPAC Personnel choose to view the report from the error message. |  | - NPAC |  | - Verify the NPAC OP GUI displays the report with all the Subscription Versions that caused the ‘Pending-Like, No-Active Subscription Version’ discrepancy. |  |  |  |
| - 4. | - NPAC | - NPAC Personnel choose to save the contents of the report to a file. |  | - NPAC |  | - The NPAC SMS saves the contents of the report to the specified file. |  |  |  |
| - 5. | - NPAC | - NPAC Personnel locate the specified report file. |  | - NPAC |  | - Verify that the report contains the Subscription Versions that caused the ‘Pending-Like, No-Active Subscription Version’ discrepancy. |  |  |  |
| - 6. | - NPAC | - NPAC Personnel choose to fax the contents of the report. |  | - NPAC |  | - The NPAC SMS faxes the contents of the report to the specified fax destination. |  |  |  |
| - 7. | - NPAC | - NPAC Personnel check the fax the specified destination. |  | - NPAC |  | - Verify that the report contains the Subscription Versions that caused the ‘Pending-Like, No-Active Subscription Version’ discrepancy. |  |  |  |
| - 8. | - NPAC | - NPAC Personnel choose to email the contents of the report. |  | - NPAC |  | - The NPAC SMS emails the contents of the report to the specified email destination. |  |  |  |
| - 9. | - NPAC | - NPAC Personnel check the email destination. |  | - NPAC |  | - Verify that the report contains the Subscription Versions that caused the ‘Pending-Like, No-Active Subscription Version’ discrepancy. |  |  |  |
| - 10. | - NPAC | - NPAC Personnel perform a query for the Number Pool Block Create Event that NPAC Personnel attempted to schedule during this Test Case. |  | - NPAC |  | - Verify that the Number Pool Block Create Event does not exist on the NPAC SMS. |  |  |  |
| - 11. | - SP – Optional | - Service Provider Personnel perform a local query for the Number Pool Block and 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that NPAC Personnel attempted to schedule during this Test Case. |  | - SP |  | - Verify that the Number Pool Block does not exist on the SOA and/or EDR LSMS. - Verify that the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ do not exist on the non-EDR LSMS. |  |  |  |
| - 12. | - SP – Conditional | - Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that NPAC Personnel attempted to schedule during this Test Case. |  | - SP |  | - Verify the Number Pool Block does not exist on the NPAC SMS. - Verify that the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ do not exist on the NPAC SMS. |  |  |  |

| - A. | - TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: |  | - 4.1.8 |  | - SUT Priority: |  | - SOA LTI |  | - N/A |
|  |  |  |  |  |  |  | - SOA |  | - C |
|  |  |  |  |  |  |  | - non-EDR LSMS |  | - R |
|  |  |  |  |  |  |  | - EDR LSMS |  | - R |
|  | - Objective: |  | - SOA - Service Provider Personnel create a Number Pool Block - (to at least 3 LSMSs – at least 1 EDR and 2 non-EDR) that results in a Full Failure – Success |  |  |  |  |  |  |
| - B. | - REFERENCES |  |  |  |  |  |  |  |  |
|  | - NANC Change Order Revision Number: |  |  |  | - Change Order Number(s): |  |  |  |  |
|  | - NANC FRS Version Number: |  |  |  | - Relevant Requirement(s): |  | - RR3-132, RR3-141.1, Table RR3-137.2RR3-137.2 (Row 15), Table RR3-138.2 (Row 15), RR3-142.1, RR3-153, RR5-95 |  |  |
|  | - NANC IIS Version Number: |  |  |  | - Relevant Flow(s): |  | - 2.1 Number Pool Block Create by SOA - 2.4. Number Pool Block Create Broadcast to Local SMS: Failure |  |  |
| - C. | - PREREQUISITE |  |  |  |  |  |  |  |  |
|  | - Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | - Prerequisite NPAC Setup: |  | - Verify that at least three LSMSs (at least one EDR and two non-EDR) are configured to receive downloads for this NPA-NXX, but keep them disconnected from the NPAC SMS to create a failure scenario. - Verify that the respective NPA-NXX-X exists for which Service Provider Personnel will attempt to create the respective Number Pool Block during this Test Case. - Verify that the current date is equal to or greater than the NPA-NXX-X Effective Date. - Verify that no ‘pending-like, no active’ nor ‘active-like’ Subscription Versions exist for the 1K Block so that a non-contaminated Number Pool Block may be created. |  |  |  |  |  |  |
|  | - Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| - D. | - TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| - Row # | - NPAC or SP | - Test Step |  | - NPAC or SP |  | - Expected Result |  |  |  |
| - 1. | - SP | - Using the SOA, Service Provider Personnel submit a M-ACTION numberPoolBlock-Create request to the NPAC SMS to create a Number Pool Block. - The request must include the following attributes: - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | - SP |  | - The NPAC SMS receives the M-ACTION Request numberPoolBlock-Create request. - The NPAC SMS verifies the following information: - The requesting SOA is the NPA-NXX-X Holder SOA. - The serviceProvNPA-NXX-X object exists for the NPA-NXX-X (respective NPA-NXX-X information). - All attributes specified are valid. - A numberPoolBlockNPAC object does not already exist for the NPA-NXX-X (a duplicate Number Pool Block does not already exist). - The current date is greater than or equal to the NPA-NXX-X-EffectiveTimeStamp. - There are not any ‘pending-like, no-active’ Subscription Version objects within the given TN range. |  |  |  |
| - 2. | - NPAC | - The NPAC SMS issues an M-CREATE Request numberPoolBlockNPAC to itself. - The NPAC SMS sets the numberPoolBlockSOA-Origination Indicator to TRUE. - The NPAC SMS sets the numberPoolBlockStatus to 'sending'. - The NPAC SMS sets the following timestamps to the current date and time: - numberPoolBlockCreationTimeStamp - numberPoolBlockActivationTimeStamp - numberPoolBlockBroadcastTimeStamp - numberPoolBlockModifiedTimeStamp |  | - NPAC |  | - The NPAC SMS issues an M-CREATE Response numberPoolBlockNPAC to itself. |  |  |  |
| - 3. | - NPAC | - The NPAC SMS issues an M-CREATE Request subscriptionVersionNPAC to itself. - The NPAC SMS sets the LNP Type to ‘POOL’ for the Subscription Versions it creates within the 1K Block. - The NPAC SMS sets the Subscription Versions to ‘sending’. - The NPAC SMS sets the following timestamps to the current date and time for the Subscription Versions: - subscriptionModifiedTimeStamp - subscriptionActivationTimeStamp - subscriptionBroadcastTimeStamp - subscriptionCreationTimeStamp |  | - NPAC |  | - The NPAC SMS issues an M-CREATE Response subscriptionVersionNPAC to itself. |  |  |  |
| - 4. | - NPAC | - The NPAC SMS issues an M-ACTION Response numberPoolBlock-Create to the respective NPA-NXX-X Holder SOA that initiated the Number Pool Block Create request. |  | - SP |  | - The NPA-NXX-X Holder SOA receives the M-ACTION Response from the NPAC SMS. |  |  |  |
| - 5. | - NPAC | - The NPAC SMS issues an M-EVENT-REPORT objectCreation for the numberPoolBlockNPAC to the NPA-NXX-X Holder SOA. - The following attributes are sent in the objectCreation notification: - numberPoolBlockId - numberPoolBlockSOA-Origination - numberPoolBlockCreationTimeStamp - numberPoolBlockStatus - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | - SP |  | - The NPA-NXX-X Holder SOA issues an M-EVENT-REPORT Confirmation to the NPAC SMS. |  |  |  |
| - 6. | - NPAC | - The NPAC SMS issues an M-ACTION Request subscriptionVersionLocalSMS-Create to the non-EDR LSMSs in the region that are accepting downloads for this NPA-NXX. - At the same time as step 6.1, the NPAC SMS issues an M-CREATE Request numberPoolBlock to the EDR LSMSs in the region that are accepting downloads for this NPA-NXX. |  | - NPAC |  | - The NPAC SMS waits for all M-ACTION Responses from non-EDR LSMSs and all M-CREATE Responses from all EDR LSMSs. - The NPAC SMS automatically retries any LSMS who does not respond within a tunable amount of time. - The NPAC SMS does not receive a response to the create requests from all LSMSs (EDR and non-EDR). |  |  |  |
| - 7. | - NPAC | - After all retries have been exhausted, the NPAC SMS issues an M-SET subscriptionVersionNPAC to itself and updates the following attributes for each Subscription Version within the 1K Block with LNP Type set to ‘POOL’: - sets the Subscription Version status to ‘failed’. - sets the Subscription Version Failed SP List to reflect the Service Providers that did not respond. - sets the subscriptionModifiedTimeStamp is set to the current date and time. - The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself to update the following attributes: - sets the numberPoolBlockStatus to 'failed'. - sets the Number Pool Block Failed SP List to reflect the Service Providers that did not respond. - sets the numberPoolBlockModifiedTimeStamp to the current date and time. |  | - NPAC |  | - The NPAC SMS issues an M-SET subscriptionVersionNPAC Response to itself. - The NPAC SMS issues an M-SET numberPoolBlockNPAC Response to itself. |  |  |  |
| - 8. | - NPAC | - The NPAC SMS determines the SOA Origination Indicator is set to TRUE and issues an M-EVENT-REPORT numberPoolBlockStatusAttributeValueChange to the NPA-NXX-X Holder SOA with the numberPoolBlockStatus set to ‘failed’ and the list of Service Providers that failed the create request. |  | - SP |  | - The NPA-NXX-X Holder SOA receives the M-EVENT-REPORT and issues an M-EVENT-REPORT Confirmation. |  |  |  |
| - 9. | - NPAC | - NPAC Personnel perform a query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that Service Provider Personnel created during this Test Case. |  | - NPAC |  | - Verify the Number Pool Block exists with status of ‘failed’ and Failed SP List that reflects all Service Providers that failed the request. - Verify the 1K Block of Subscription Versions exist with LNP Type set to ‘POOL’, a status of ‘failed’ and a Failed SP List that reflects all Service Providers that failed the request. - Verify data integrity (LRN and GTT data) has been maintained between the Number Pool Block and 1K Block of Subscription Versions with LNP Type set to ‘POOL’ on the NPAC SMS. |  |  |  |
| - 10. | - SP – Optional | - Service Provider Personnel perform a local query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that Service Provider Personnel created during this Test Case. |  | - SP |  | - Verify the Number Pool Block exists with a status of ‘failed’ and a Failed SP List that reflects all SPs that did not successfully process the NPAC SMS request on the SOA. - Verify the Number Pool Block does not exist on the EDR LSMS. - Verify the 1K Block of Subscription Versions do not exist on the non-EDR LSMS. |  |  |  |
| - 11. | - SP – Conditional | - Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that Service Provider Personnel created during this Test Case. |  | - SP |  | - Verify the Number Pool Block exists on the NPAC SMS with status of ‘failed’ and a Failed SP List that reflects all Service Providers that failed the request. - Verify the 1K Block of Subscription Versions exist on the NPAC SMS with LNP Type set to ‘POOL’, a status of ‘failed’ and a Failed SP List that reflects all Service Providers that failed the request. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.1.9 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | O |
|  |  |  |  |  |  |  | non-EDR LSMS |  | R |
|  |  |  |  |  |  |  | EDR LSMS |  | R |
|  | Objective: |  | NPAC OP GUI - NPAC Personnel re-send a full failure Number Pool Block create to 1 LSMS (1 EDR ) resulting in success (2 non-EDR systems are still on the Failed SP List) – Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | Table RR3-137.2RR3-137.2 (Row 14), RR3-138.1, RR3-138.2, Table RR3-138.2 (Row 14), RR3-139, RR3-153, RR3-185, RR3-186.1, RR3-186.2, RR3-187, RR3-188, RR3-189, RR3-190, RR3-195, RR3-196, RR3-197, RR5-85, RR5-72, RR5-73, RR5-77, RR5-78, RR5-79 |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  | 2.6 Number Pool Block Create Resend Broadcast 2.9 Number Pool Block Create Partial Failure Resend NPAC SMS Updates |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  | 4.1.8 SOA - Service Provider Personnel create a Number Pool Block - (to at least 3 LSMSs – at least 1 EDR and 2 non-EDR) that results in a Full Failure – Success |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that a Number Pool Block exists with a status of ‘failed’ and a Failed SP List that contains 3 Service Providers (1 non-EDR and 2 EDR systems). - Verify that the Service Providers on the Failed SP List are configured and connected such the EDR LSMS could now successfully process the Number Pool Block resend request. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | - Using the NPAC OP GUI, NPAC Personnel take action to resend a ‘failed’, Number Pool Block to the EDR Service Provider in the Number Pool Block Failed SP List. - The NPAC SMS issues an M-SET numberPoolBlockNPAC to itself to set the following attributes: - set the numberPoolBlockStatus to ‘sending’. - set the numberPoolBlockModifiedTimeStamp and numberPoolBlockBroadcastTimeStamp to the current date and time. - The NPAC SMS issues an M-SET subscriptionVersionNPAC to itself for all the Pooled Subscription Versions within the 1K Block to set the following attributes: - set the subscriptionVersionStatus to ‘sending’. - set the subscriptionModifiedTimeStamp and subscriptionBroadcastTimeStamp to the current date and time. |  | NPAC |  | - The NPAC SMS issues an M-SET Response numberPoolBlockStatus to itself. - The NPAC SMS issues an M-SET Response subscriptionVersionNPAC to itself. |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-CREATE Request numberPoolBlock to the EDR LSMS that NPAC Personnel indicated in the Number Pool Block resend request. |  | NPAC |  | - The EDR LSMS receives the M-CREATE Request and returns an M-CREATE Response numberPoolBlock. - The NPAC SMS waits for the M-CREATE Response from the EDR LSMS. |  |  |  |
| 3. | NPAC | The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself to set the following attributes: - set the numberPoolBlock status to ‘partial failure’. - update the numberPoolBlockFailedSP-List is to reflect the two non-EDR systems that the Number Pool Block create resend request was not sent to. - set the numberPoolBlockModifiedTimeStamp to the current date and time. |  | NPAC |  | The NPAC SMS issues an M-SET Response to itself. |  |  |  |
| 4. | NPAC | The NPAC SMS issues an M-SET subscriptionVersionNPAC to itself to set the following attributes for the Pooled Subscription Versions within the 1K Block: - set the Subscription Version status to ‘partial failure’. - update the subscriptionFailedSP-List to reflect the name of the two non-EDR systems that the Number Pool Block create resend request was not sent to. - set the subscriptionModifiedTimeStamp to the current date and time. |  | NPAC |  | The NPAC SMS issues an M-SET Response back to itself. |  |  |  |
| 5. | NPAC | The NPAC SMS determines that the SOA Origination Indicator is set to FALSE and processing terminates here. |  |  |  |  |  |  |  |
| 6. | NPAC | NPAC Personnel perform a local query for the Number Pool Block and the 1K Block of Pooled Subscription Versions that NPAC Personnel resent during this Test Case. |  | NPAC |  | - Verify the Number Pool Block exists with a status of ‘partial failure’ with a Failed SP List that contains the name of the two Service Providers that the Number Pool Block create was not resent to during this Test Case. - Verify the Pooled Subscription Versions within the 1K Block exist with a status of ‘partial failure’ with a Failed SP List that contains the name of the two Service Providers that the Number Pool Block create was not resent to during this Test Case. |  |  |  |
| 7. | SP – Optional | Block Holder Service Provider Personnel perform a local query for the Number Pool Block and the 1K Block of Pooled Subscription Versions that NPAC Personnel resent during this Test Case. |  | SP |  | - Verify that the Number Pool Block exists on the EDR LSMS that successfully processed the resend request in this Test Case. - Verify the 1K Block of Subscription Versions do not exist on the non-EDR LSMSs that did not receive the resend request in this Test Case. |  |  |  |
| 8. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and the 1K Block of Pooled Subscription Versions that NPAC Personnel resent during this Test Case. |  | SP |  | - Verify the Number Pool Block exists with a status of ‘partial failure’ with a Failed SP List on the NPAC SMS. The Failed SP List contains the name of the two Service Providers that the Number Pool Block create was not resent to during this Test Case. - Verify the Pooled Subscription Versions within the 1K Block exist with a status of ‘partial failure’ with a Failed SP List on the NPAC SMS. The Failed SP List contains the name of the two Service Providers that the Number Pool Block create was not resent to during this Test Case. |  |  |  |

| - A. | - TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: |  | - 4.1.10 |  | - SUT Priority: |  | - SOA LTI |  | - N/A |
|  |  |  |  |  |  |  | - SOA |  | - O |
|  |  |  |  |  |  |  | - non-EDR LSMS |  | - R |
|  |  |  |  |  |  |  | - EDR LSMS |  | - O |
|  | - Objective: |  | - NPAC - NPAC Personnel perform a resend of a previously ‘partial failure’ Number Pool Block to all Service Providers in the Failed SP List (2 non-EDR) – Success |  |  |  |  |  |  |
| - B. | - REFERENCES |  |  |  |  |  |  |  |  |
|  | - NANC Change Order Revision Number: |  |  |  | - Change Order Number(s): |  |  |  |  |
|  | - NANC FRS Version Number: |  |  |  | - Relevant Requirement(s): |  | - RR3-120, RR3-121, RR3-138.1, RR3-140, RR3-153, RR3-186.1, RR3-186.2, RR3-187, RR3-188, RR3-189, RR3-191, RR3-194, RR3-195, RR3-196, RR5-100, RR5-101, RR5-72, RR5-74, RR5-78 |  |  |
|  | - NANC IIS Version Number: |  |  |  | - Relevant Flow(s): |  | 2.6 Number Pool Block Create Resend Broadcast - 2.7 Number Pool Block Create Successful Resend NPAC SMS Updates |  |  |
| - C. | - PREREQUISITE |  |  |  |  |  |  |  |  |
|  | - Prerequisite Test Cases: |  | - 4.1.9 NPAC OP GUI - NPAC Personnel re-send a full failure Number Pool Block create to 3 LSMSs (1 EDR and 2 non-EDR systems) resulting in Partial Failure - Success |  |  |  |  |  |  |
|  | - Prerequisite NPAC Setup: |  | - Verify that all LSMSs that are listed in the Failed Service Provider List for the Number Pool Block that NPAC Personnel will resend during this Test Case (2 non-EDR) are configured to be associated with the NPAC SMS and receive downloads for the NPA-NXX. - Verify that Number Pool Block exists in a ‘partial failure’ state. - Verify that the SOA Origination Indicator is set to ‘FALSE’ for this Number Pool Block. |  |  |  |  |  |  |
|  | - Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| - D. | - TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| - Row # | - NPAC or SP | - Test Step |  | - NPAC or SP |  | - Expected Result |  |  |  |
| - 1. | - NPAC | - Using the NPAC OP GUI, NPAC Personnel submit a request to resend a ‘partial failure’ Number Pool Block to each Service Provider in the Failed SP List. - The NPAC SMS issues an M-SET Request numberPoolBlock to itself to set the following attributes: - set the numberPoolBlockStatus to ‘sending’ - set the numberPoolBlockModifiedTimeStamp to the current date and time. - The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself for all the Pooled Subscription Versions within the 1K Block to set the following attributes: - set the subscriptionVersionStatus to ‘sending’ - set the subscriptionModifiedTimeStamp and subscriptionBroadcastTimeStamp to the current date and time. |  | - NPAC |  | - The NPAC SMS issues an M-SET Response numberPoolBlockNPAC to itself. - The NPAC SMS issues an M-SET Response subscriptionVersionNPAC to itself. |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-ACTION Request subscriptionVersionLocalSMS-Create to the non-EDR LSMSs that are on the Subscription Version Failed SP List. |  | NPAC |  | The non-EDR LSMSs respond to the NPAC SMS request with a successful M-ACTION response. |  |  |  |
| 3. | NPAC | - Upon the first successful response from an LSMS, the NPAC SMS sets the following timestamps to the current date and time: - numberPoolBlockActivationCompleteTimeStamp - subscriptionActivationCompleteTimeStamp - numberPoolBlockModifiedTimeStamp - subscriptionModifiedTimeStamp - After a successful response from all LSMSs the resend request was sent to, the NPAC SMS issues an M-SET subscriptionVersionNPAC to itself to set the following attributes: - update the Subscription Version status for Subscription Versions of LNP Type set to ‘POOL’ to ‘active’ and set the Failed SP List to empty. - set the subscriptionModifiedTimeStamp is set to the current date and time. |  | NPAC |  | The NPAC SMS issues an M-SET Response to itself. |  |  |  |
| 4. | NPAC | The NPAC SMS issues an M-SET numberPoolBlockNPAC to itself and performs the following steps: - The numberPoolBlock status is set to ‘active’ and the Failed SP List updated to empty. - The numberPoolBlockModifiedTimeStamp is set to the current date and time. |  | NPAC |  | The NPAC SMS issues an M-SET Response to itself. |  |  |  |
| 5. | NPAC | The NPAC SMS determines that the SOA Origination Indicator is set to FALSE and processing terminates here. |  |  |  |  |  |  |  |
| 6. | NPAC | NPAC Personnel perform a query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that NPAC Personnel resent during this Test Case. |  | NPAC |  | - Verify the Number Pool Block exists with a status of ‘active’ and has an empty Failed SP List. - Verify the Subscription Versions in the 1K Block with LNP Type set to ‘POOL’ exist with a status of ‘active’ and an empty Failed SP List. |  |  |  |
| 7. | SP – Optional | Service Provider Personnel perform a local query for the Number Pool Block and the 1K Block of Pooled Subscription Versions that NPAC Personnel resent during this Test Case. |  | SP |  | Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ exist on the non-EDR LSMSs. |  |  |  |
| 8. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that NPAC Personnel resent during this Test Case. |  | SP |  | - Verify the Number Pool Block exists with a status of ‘active’ and has an empty Failed SP List on the NPAC SMS. - Verify the Subscription Versions in the 1K Block with LNP Type set to ‘POOL’ exist with a status of ‘active’ and an empty Failed SP List on the NPAC SMS. |  |  |  |

| - A. | - TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: |  | - 4.1.11 |  | - SUT Priority: |  | - SOA LTI |  | - N/A |
|  |  |  |  |  |  |  | - SOA |  | - C |
|  |  |  |  |  |  |  | - non-EDR LSMS |  | - R |
|  |  |  |  |  |  |  | - EDR LSMS |  | - R |
|  | - Objective: |  | - SOA – Service Provider Personnel create a Number Pool Block (to at least 4 LSMSs - 2 non-EDR and 2 EDR) that results in a Partial Failure - Success |  |  |  |  |  |  |
| - B. | - REFERENCES |  |  |  |  |  |  |  |  |
|  | - NANC Change Order Revision Number: |  |  |  | - Change Order Number(s): |  |  |  |  |
|  | - NANC FRS Version Number: |  |  |  | - Relevant Requirement(s): |  | - RR3-132, RR3-138.1, RR3-153, RR5-100, RR5-101, RR5-95 |  |  |
|  | - NANC IIS Version Number: |  |  |  | - Relevant Flow(s): |  | - 2.1 Number Pool Block Create by SOA - 2.5.1 Number Pool Block Create Partial-Failure Broadcast to Local SMSs - 2.5.2 Number Pool Block Create Broadcast Partially Failed NPAC SMS Updates |  |  |
| - C. | - PREREQUISITE |  |  |  |  |  |  |  |  |
|  | - Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | - Prerequisite NPAC Setup: |  | - Verify that at least four LSMSs (2 non-EDR and 2 EDR) are configured to be associated with the NPAC SMS and receive downloads for this NPA-NXX. One LSMS should be disconnected from the NPAC SMS to achieve a ‘partial-failure’ download. - Verify that the respective NPA-NXX-X exists for which Service Provider Personnel will attempt to create the respective Number Pool Block during this Test Case. - Verify that the current date is equal to or greater than the NPA-NXX-X Effective Date. - Verify that no ‘pending-like, nor active-like’ Subscription Versions exist for the 1K Block so that a non-contaminated Number Pool Block may be created. |  |  |  |  |  |  |
|  | - Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| - D. | - TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| - Row # | - NPAC or SP | - Test Step |  | - NPAC or SP |  | - Expected Result |  |  |  |
| - 1. | - SP | - Using the SOA, Service Provider Personnel submit a M-ACTION numberPoolBlock-Create request to the NPAC SMS to create a Number Pool Block. - The request must include the following attributes: - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | - NPAC |  | - The NPAC SMS receives the M-ACTION Request numberPoolBlock-Create request. - The NPAC SMS verifies the following information: - The requesting SOA is the NPA-NXX-X Holder SOA. - The serviceProvNPA-NXX-X object exists for the NPA-NXX-X (respective NPA-NXX-X information). - All attributes specified are valid. - A numberPoolBlockNPAC object does not already exist for the NPA-NXX-X (a duplicate Number Pool Block does not already exist). - The current date is greater than or equal to the NPA-NXX-X-EffectiveTimeStamp. - There are not any ‘pending-like, no-active’ Subscription Version objects within the given TN range. |  |  |  |
| - 2. | - NPAC | - The NPAC SMS issues an M-CREATE Request numberPoolBlockNPAC to itself. - The NPAC SMS sets the numberPoolBlockSOA-Origination Indicator to TRUE. - The NPAC SMS sets the numberPoolBlockStatus to 'sending'. - The NPAC SMS sets the following timestamps to the current date and time: - numberPoolBlockCreationTimeStamp - numberPoolBlockActivationTimeStamp - numberPoolBlockBroadcastTimeStamp - numberPoolBlockModifiedTimeStamp |  | - NPAC |  | - The NPAC SMS issues an M-CREATE Response numberPoolBlockNPAC to itself. |  |  |  |
| - 3. | - NPAC | - The NPAC SMS issues an M-CREATE Request subscriptionVersionNPAC to itself. - The NPAC SMS sets the LNP Type to ‘POOL’ for the Subscription Versions it creates within the 1K Block. - The NPAC SMS sets the Subscription Versions to ‘sending’. - The NPAC SMS sets the following timestamps to the current date and time for the Subscription Versions: - subscriptionModifiedTimeStamp - subscriptionActivationTimeStamp - subscriptionBroadcastTimeStamp - subscriptionCreationTimeStamp |  | - NPAC |  | - The NPAC SMS issues an M-CREATE Response subscriptionVersionNPAC to itself. |  |  |  |
| - 4. | - NPAC | - The NPAC SMS issues an M-ACTION Response numberPoolBlock-Create to the respective NPA-NXX-X Holder SOA that initiated the Number Pool Block Create request. |  | - SP |  | - The NPA-NXX-X Holder SOA receives the M-ACTION Response from the NPAC SMS. |  |  |  |
| - 5. | - NPAC | - The NPAC SMS issues an M-EVENT-REPORT objectCreation for the numberPoolBlockNPAC to the NPA-NXX-X Holder SOA. - The following attributes are sent in the objectCreation notification: - numberPoolBlockId - numberPoolBlockSOA-Origination - numberPoolBlockCreationTimeStamp - numberPoolBlockStatus - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | - SP |  | - The NPA-NXX-X Holder SOA issues an M-EVENT-REPORT Confirmation to the NPAC SMS. |  |  |  |
| 6. | NPAC | - The NPAC SMS issues an M-ACTION Request subscriptionVersionLocalSMS-Create to the non-EDR LSMSs in the region that are accepting downloads for this NPA-NXX. - At the same time as step 6.1, the NPAC SMS issues an M-CREATE Request numberPoolBlock to the EDR LSMSs in the region that are accepting downloads for this NPA-NXX. |  | NPAC |  | - The non-EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-ACTION Request, verify that the action is valid and return an M-ACTION Response subscriptionVersionLocalSMS-Create. - The EDR LSMSs that are accepting downloads for this NPA-NXX receives the M-CREATE Request, and return an M-CREATE Response numberPoolBlock. - The non-EDR LSMSs proceed to execute all the creates specified by the M-ACTION Request and issues an M-EVENT-REPORT subscriptionVersionLocalSMS-CreateResults specifying the Subscription Versions were successfully created. - The NPAC SMS waits for all M-ACTION Responses from non-EDR LSMSs and all M-CREATE Responses from all EDR LSMSs. - The NPAC SMS automatically retries any LSMS who does not respond within a tunable amount of time. If a non-EDR does not respond, the NPAC will retry with an M-ACTION Request. If an EDR LSMS does not respond, the NPAC will retry with an M-CREATE Request. |  |  |  |
| 7. | NPAC | Upon the first successful response from an LSMS, the NPAC SMS sets the following timestamps to the current date and time: - numberPoolBlockActivationCompleteTimeStamp - subscriptionActivationCompleteTimeStamp - numberPoolBlockModifiedTimeStamp - subscriptionModifiedTimeStamp |  | NPAC |  | The NPAC SMS does not receive a response from one of the LSMSs. |  |  |  |
| 8. | NPAC | - The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself and updates the following attributes for each Pooled Subscription Version within the 1K Block: - sets the subscriptionVersionStatus to ‘partial failure’. - sets the Subscription Version Failed SP List to reflect the Service Provider that did not respond to the NPAC request. - sets the subscriptionModifiedTimeStamp to the current date and time. - The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself and updates the following attributes: - sets the numberPoolBlockStatus to 'partial failure’ - sets the Number Pool Block Failed SP List to reflect the Service Provider that did not respond to the NPAC request. - sets the numberPoolBlockModifiedTimeStamp to the current date and time. |  | NPAC |  | - The NPAC SMS issues an M-SET subscriptionVersionNPAC Response to itself. - The NPAC SMS issues an M-SET numberPoolBlockNPAC Response to itself |  |  |  |
| - 9. | - NPAC | - The NPAC SMS determines the SOA Origination Indicator is set to TRUE and issues an M-EVENT-REPORT numberPoolBlockStatusAttributeValueChange to the NPA-NXX-X Holder SOA to set the Number Pool Block status to 'partial failure' and set the Failed SP List to reflect those Service Providers that did not successfully process the request. |  | - SP |  | - The NPA-NXX-X Holder SOA receives the M-EVENT-REPORT from the NPAC SMS and issues an M-EVENT-REPORT Confirmation back to the NPAC SMS. |  |  |  |
| 10. | NPAC | NPAC Personnel perform a query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that Service Provider Personnel created during this Test Case. |  | NPAC |  | - Verify the Number Pool Block exists with a status of ‘partial failure’ and has a Failed SP List that reflects the Service Provider that failed the NPAC request. - Verify the Subscription Versions in the 1K Block with LNP Type set to ‘POOL’ exist with a status of ‘partial failure’ and a Failed SP List that reflects the Service Provider that failed the NPAC request. |  |  |  |
| 11. | SP – Optional | Block Holder Service Provider Personnel perform a local query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that Service Provider Personnel created during this Test Case. |  | SP |  | - Verify the Number Pool Block exists with a status of ‘partial failure’ and has a Failed SP List that reflects the Service Provider that failed the NPAC request on the SOA. - Verify that the Number Pool Block exists for those EDR LSMSs that received the NPAC request. - Verify that the Subscription Versions in the 1K Block with LNP Type set to ‘POOL’ exist for those non-EDR LSMSs that received the NPAC request. |  |  |  |
| 12. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that Service Provider Personnel created during this Test Case. |  | SP |  | - Verify the Number Pool Block exists with a status of ‘partial failure’ and has a Failed SP List on the NPAC SMS. The Failed SP List reflects the Service Provider that failed the NPAC request. - The Subscription Versions in the 1K Block with LNP Type set to ‘POOL’ exist with a status of ‘partial failure’ and a Failed SP List on the NPAC SMS. The Failed SP List reflects the Service Provider that failed the NPAC request. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.1.12 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | O |
|  |  |  |  |  |  |  | non-EDR LSMS |  | R |
|  |  |  |  |  |  |  | EDR LSMS |  | R |
|  | Objective: |  | NPAC OP GUI – NPAC Personnel create a Number Pool Block when an orphaned TN exists and the NPAC performs a Subscription Version completion check - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR5-96, RR5-97, RR5-98, RR5-99, RR5-102 |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  | 2.2 Number Pool Block Create by NPAC SMS 2.5 Number Pool Block Create Broadcast to Local SMS: Partial Failure 2.5.1 Number Pool Block Create Partial-Failure Broadcast to Local SMSs 2.5.2 Number Pool Block Create Broadcast Partially Failed NPAC SMS Updates 2.6 Number Pool Block Create Resend Broadcast 2.7 Number Pool Block Create Successful Resend Updates B.5.4.5 Subscription Version Disconnect Resend Successful to Local SMS |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that a LISP Subscription Version exists within the 1K Block for which NPAC Personnel are going to create a Number Pool Block. - Verify that there are at least 2 LSMSs (1EDR and 1 non-EDR) that are configured to receive downloads for this NPA-NXX. - Verify that the EDR LSMS is associated to the NPAC SMS and configured to receive downloads for the NPA-NXX respective to the Number Pool Block that NPAC Personnel will create during this test case. - Verify that the non-EDR LSMS is not associated to the NPAC SMS, but is configured to receive downloads for the NPA-NXX respective to the Number Pool Block that NPAC Personnel will create during this test case. - Disconnect the LISP Subscription Version so that the status goes to ‘old’ with a Failed SP List that reflects the one non-EDR LSMS. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel submit a request to schedule a Number Pool Block Create Event to run immediately. The NPAC SMS issues an M-ACTION Request numberPoolBlock-Create request to create a Number Pool Block, specifying a 1K Block for which there is already an NPA-NXX-X that has reached its NPA-NXX-X Effective Date and does not have an ‘Active’ Number Pool Block. The request must include the following attributes: - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | NPAC |  | - The NPAC SMS receives the M-ACTION Request numberPoolBlock-Create . - The NPAC SMS verifies the following information: - The serviceProvNPA-NXX-X object exists for the NPA-NXX-X. - Verifies all attributes specified are valid (the SPID is equal to the SPID on the NPA-NXX-X). - Verifies a numberPoolBlockNPAC object does not already exist for the NPA-NXX-X (a duplicate Number Pool Block does not already exist – or it exists with a status of ‘Old’ and an empty Failed SP List). - Verifies the current date is greater than or equal to the NPA-NXX-X-EffectiveTimeStamp. - Verifies there are not any ‘pending-like, no-active’ Subscription Versions within the given 1K Block. |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-CREATE Request numberPoolBlockNPAC to itself. - The numberPoolBlockSOA-Origination Flag is set to FALSE. - The numberPoolBlockCreationTimeStamp, numberPoolBlockActivationTimeStamp, numberPoolBlockBroadcastTimeStamp and numberPoolBlockModifiedTimeStamp are set to the current date and time. - The numberPoolBlockStatus is set to ‘sending’. |  | NPAC |  | The NPAC SMS responds to the M-CREATE. |  |  |  |
| 3. | NPAC | The NPAC SMS creates the corresponding Subscription Versions for the non-ported TNs within the 1K Block with the following attributes: - sets the LNP Type to ‘POOL’. - sets the status to ‘sending’. - sets the following timestamps to the current date and time: - subscriptionModifiedTimeStamp - subscriptionActivationTimeStamp - subscriptionBroadcastTimeStamp - subscriptionCreationTimeStamp |  | NPAC |  | The NPAC SMS issues and M-CREATE Response subscriptionVersionNPAC to itself. |  |  |  |
| 4. | NPAC | The NPAC issues an M-ACTION Response numberPoolBlock-Create back to itself. |  |  |  |  |  |  |  |
| 5. | NPAC | - The NPAC SMS issues an M-ACTION Request subscriptionVersionLocalSMS-Create to all non-EDR LSMSs in the region that are accepting downloads for this NPA-NXX. - At the same time as step 5.1, the NPAC SMS issues an M-CREATE Request numberPoolBlock to all EDR LSMSs in the region that are accepting downloads for this NPA-NXX. |  | SP |  | - The non-EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-ACTION Request, verify that the action is valid and return an M-ACTION Response subscriptionVersionLocalSMS-Create. - The EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-CREATE Request, and return an M-CREATE Response numberPoolBlock. - Upon the first successful response from an LSMS, the NPAC SMS sets the following timestamps to the current date and time: - numberPoolBlockActivationCompleteTimeStamp - subscriptionActivationCompleteTimeStamp - numberPoolBlockModifiedTimeStamp - subscriptionModifiedTimeStamp - The non-EDR LSMSs proceed to execute all the creates specified by the M-ACTION Request and issues an M-EVENT-REPORT subscriptionVersionLocalSMS-CreateResults specifying the Subscription Versions were successfully created. |  |  |  |
| 6. | NPAC | The NPAC SMS receives the M-EVENT-REPORT subscriptionVersionLocalSMS-CreateResults from each non-EDR LSMS in the region that initiated this message. |  | NPAC |  | The NPAC SMS responds to each of the M-EVENT-REPORT subscriptionVersionLocalSMS-CreateResults as it receives these notifications with M-EVENT-REPORT Confirmations. |  |  |  |
| 7. | NPAC | - The NPAC SMS issues M-SET Request subscriptionVersionNPAC to itself and updates the following attributes for each Subscription Version within the 1K Block with LNP Type set to ‘POOL’: - sets the subscriptionVersionStatus to 'active'. - sets the Subscription Version Failed SP List to empty. - sets the subscriptionModifiedTimeStamp to the current date and time. - The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself to update the following attributes: - sets the numberPoolBlockStatus to 'active'. - sets the Number Pool Block Failed SP List to empty. - sets the numberPoolBlockModifiedTimeStamp to the current date and time. |  | NPAC |  | - The NPAC SMS issues an M-SET subscriptionVersionNPAC Response to itself. - The NPAC SMS issues an M-SET numberPoolBlockNPAC Response to itself. |  |  |  |
| 8. | NPAC | - Using the NPAC OP GUI, NPAC Personnel take action to re-send the disconnect request for the Subscription Version referenced in the Prerequisite NPAC setup above to the non-EDR LSMS that previously failed to process the disconnect request. - The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself to set the Subscription Version Status to ‘sending’. |  | NPAC |  | The NPAC SMS issues an M-SET Response to itself. |  |  |  |
| 9. | NPAC | The NPAC SMS issues an M-DELETE Request to the non-EDR LSMS specified in the re-send request. |  | SP |  | The non-EDR LSMS receives the M-DELETE Request from the NPAC SMS and issues an M-DELETE Response indicating it successfully processed the request. |  |  |  |
| 10. | NPAC | The NPAC SMS issues an M-SET Request subscriptionVersion to itself and performs the following actions: - sets the Subscription Version Status to ‘old’. - updates the Failed SP list to empty. - sets the subscriptionModifiedTimeStampand the subscriptionDisconnectTimeStamp to the current date and time. |  | NPAC |  | The NPAC SMS issues an M-SET Response to itself. |  |  |  |
| 11. | NPAC | The NPAC SMS issues an M-EVENT-REPORT subscriptionVersionStatusAttributeValueChange to the Service Provider SOA, updating the Subscription Version Status to ‘old’ and update the Failed SP List to empty. |  | SP |  | The Service Provider SOA receives the M-EVENT-REPORT Request from the NPAC SMS and issues an M-EVENT REPORT Confirmation. |  |  |  |
| 12. | NPAC | The NPAC SMS runs the Number Pool Block Create Validation Process within 24 hours of Block Creation. |  | NPAC |  | The NPAC SMS performs the following steps: - The NPAC SMS determines that for the 1K Block, there is one missing TN with a status of ‘old’ and an empty Failed SP List. - Logs an information message regarding the ‘orphaned’ TN. - Creates a subscription version of LNP Type set to ‘POOL’ for this TN within the 1K Block, using the Block default routing information. - Sets the Pooled Subscription Version status to ‘sending’. |  |  |  |
| 13. | NPAC | The NPAC SMS issues an M-CREATE Request subscriptionVersionLocalSMS-Create to all non-EDR LSMSs in the region that are accepting downloads for this NPA-NXX for the one orphaned TN within the 1K Block. |  | SP |  | - The non-EDR LSMS(s) that are accepting downloads for this NPA-NXX receive the M-CREATE Request and issue an M-CREATE Response back to the NPAC SMS. - Upon the first successful response from an LSMS the NPAC SMS sets the following timestamps: - subscriptionModifiedTimeStamp - subscriptionActivationCompleteTimeStamp - numberPoolBlockModifiedTimeStamp - numberPoolBlockActivationCompleteTimeStamp |  |  |  |
| 14. | NPAC | The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself and performs the following on the 1K Block of Subscription Versions with LNP Type equal to ‘POOL’: - set the subscriptionVersionStatus to ‘active’ for the one Subscription Version that was created in step 13. - updates the Failed SP List to empty. - set the subscriptionModifiedTimeStamp to the current date and time. |  | NPAC |  | The NPAC SMS issues an M-SET Response subscriptionVersionNPAC to itself. |  |  |  |
| 15. | NPAC | NPAC Personnel perform a query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that NPAC Personnel created during this Test Case. |  | NPAC |  | - Verify the Number Pool Block exists with status of ‘active’ and an empty Failed SP List. - Verify the 1K Block of Subscription Versions exist with LNP Type set to ‘POOL’, a status of ‘active’ and an empty Failed SP List. |  |  |  |
| 16. | SP – Optional | Service Provider Personnel perform a local query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that NPAC Personnel created during this Test Case. |  | SP |  | - Verify the Number Pool Block exists with status of ‘active’ and an empty Failed SP List on the SOA. - Verify the Number Pool Block exists on the EDR LSMS. - Verify the 1K Block of Subscription Versions exist with LNP Type set to ‘POOL’, a status of ‘active’ on the non-EDR LSMS. |  |  |  |
| 17. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ that Service Provider Personnel created during this Test Case. |  | SP |  | - Verify the Number Pool Block exists on the NPAC SMS with status of ‘active’ and an empty Failed SP List. - Verify the 1K Block of Subscription Versions exist on the NPAC SMS with LNP Type set to ‘POOL’, a status of ‘active’ and an empty Failed SP List. |  |  |  |

| - A. | - TEST IDENTITY |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: |  | - 4.1.13 |  | - Priority: |  | - SOA LTI; - N/A. - SOA; - C. - non-EDR LSMS; - R. - EDR LSMS; - R. |
|  | - Objective: |  | - SOA – Service Provider Personnel create a Number Pool Block (to at least 4 LSMSs - 2 non-EDR and 2 EDR) that results in a Partial Failure (1 non-EDR system fails one TN and 1 EDR system fails) – Success |  |  |  |  |  |
| - B. | - REFERENCES |  |  |  |  |  |  |  |
|  | - NANC Change Order Revision Number: |  |  |  | - Change Order Number(s): |  |  |  |
|  | - NANC FRS Version Number: |  |  |  | - Relevant Requirement(s): |  | - RR3-132, RR3-134, RR3-135, RR3-136, RR3-137.1, RR3-137.2, Table RR3-137.2 (Row 9), RR3-138.1, Table RR3-138.2 (Row 9), RR3-153, RR5-88, RR5-100, RR5-101, RR5-95 |  |
|  | - NANC IIS Version Number: |  |  |  | - Relevant Flow(s): |  | - 2.1 Number Pool Block Create by SOA - 2.5.1 Number Pool Block Create Partial-Failure Broadcast to all Local SMSs - 2.5.2 Number Pool Block Create Broadcast Partially Failed NPAC SMS Updates |  |
| - C. | - PREREQUISITE |  |  |  |  |  |  |  |
|  | - Prerequisite Test Cases: |  |  |  |  |  |  |  |
|  | - Prerequisite NPAC Setup: |  | - Verify that at least four LSMSs (2 non-EDR and 2 EDR) are configured to be associated with the NPAC SMS and receive downloads for this NPA-NXX. One of the non-EDR systems shall be made to fail one of the Subscription Versions in the 1K Block. One EDR system shall be made to fail the Number Pool Block Create request. This will achieve a ‘partial-failure’ download. - Verify that the respective NPA-NXX-X exists for which Service Provider Personnel will attempt to create the respective Number Pool Block during this Test Case. - Verify that the current date is equal to or greater than the NPA-NXX-X Effective Date. - Verify that no ‘pending-like, nor active-like’ Subscription Versions exist for the 1K Block so that a non-contaminated Number Pool Block may be created. |  |  |  |  |  |
|  | - Prerequisite SP Setup: |  |  |  |  |  |  |  |
| - D. | - TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |
| - Row # | - NPAC or SP | - Test Step |  | - NPAC or SP |  | - Expected Result |  |  |
| - 1. | - SP | - Using the SOA, Service Provider Personnel submit an M-ACTION numberPoolBlock – Create request to the NPAC SMS to create a Number Pool Block. The request must include the following attributes: - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | - NPAC |  | - The NPAC SMS receives the M-ACTION numberPoolBlock – Create request. - The NPAC SMS verifies the following information: - The requesting SOA is the NPA-NXX-X Holder SOA. - The serviceProvNPA-NXX-X object exists for the NPA-NXX-X (respective NPA-NXX-X information). - All attributes specified are valid - A numberPoolBlockNPAC object does not already exist for the NPA-NXX-X (a duplicate Number Pool Block does not already exist) or if one exists it has a status of ‘old’ with an empty Failed SP List. - The current date is greater than or equal to the NPA-NXX-X-EffectiveTimeStamp. - There are not any ‘pending-lik, no-active’ Subscription Version objects within the given TN range. |  |  |
| - 2. | - NPAC | - The NPAC SMS issues an M-CREATE Request numberPoolBlockNPAC to itself. - The NPAC SMS sets the numberPoolBlockSOA-Origination Indicator to TRUE. - The NPAC SMS sets the numberPoolBlockStatus to’sending’. - The NPAC SMS sets the following timestamps to the current date and time: - numberPoolBlockCreationTimeStamp - numberPoolBlockActivationTimeStamp - numberPoolBlockBroadcastTimeStamp - numberPoolBlockModifiedTimeStamp |  | - NPAC |  | - The NPAC SMS issues an M-CREATE Response numberPoolBlockNPAC to itself. |  |  |
| - 3. | - NPAC | - The NPAC SMS issues an M-CREATE Request subscriptionVersionNPAC to itself. - The NPAC SMS sets the LNP Type to ‘POOL’ for the Subscription Versions it creates within the 1K Block. - The NPAC SMS sets the Subscription Versions status to ‘sending’. - The NPAC SMS sets the following timestamps to the current date and time for the Subscription Versions: - subscriptionModifiedTimeStamp - subscriptionActivationTimeStamp - subscriptionBroadcastTimeStamp - subscriptionCreationTimeStamp |  | - NPAC |  | - The NPAC SMS issues an M-CREATE Response subscriptionVersionNPAC to itself. |  |  |
| - 4. | - NPAC | - The NPAC SMS issues an M-ACTION Response numberPoolBlock – Create to the respective NPA-NXX-X Holder SOA that initiated the Number Pool Block Create request. |  | - SP |  | - The NPA-NXX-X Holder SOA receives the M-ACTION Response from the NPAC SMS. |  |  |
| - 5. | - NPAC | - The NPAC SMS issues an M-EVENT-REPORT objectCreation for the numberPoolBlockNPAC to the NPA-NXX-X Holder SOA. - The following attributes are sent in the objectCreation notification: - numberPoolBlockId - numberPoolBlockSOA-Origination - numberPoolBlockCreationTimeStamp - numberPoolBlockStatus - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | - SP |  | - The NPA-NXX-X Holder SOA receives the M-EVENT-REPORT objectCreation and issues an M-EVENT-REPORT Confirmation back to the NPAC SMS. |  |  |
| - 6. | - NPAC | - The NPAC SMS issues an M-ACTION Request subscriptionVersionLocalSMS-Create to the non-EDR LSMSs in the region that are accepting downloads for this NPA-NXX. - At the same time as step 6.1, the NPAC SMS issues an M-CREATE Request numberPoolBlock to the EDR LSMSs in the region that are accepting downloads for this NPA-NXX. |  | - NPAC |  | - The non-EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-ACTION Request, verify that the action is valid and return an M-ACTION Response subscriptionVersionLocalSMS-Create. - The EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-CREATE Request, and return an M-CREATE Response numberPoolBlock. - The non-EDR LSMSs proceed to execute all the creates specified by the M-ACTION Request and issue an M-EVENT-REPORT subscriptionVersionLocalSMS-CreateResults specifying the Subscription Versions were successfully created. - The NPAC SMS waits for all M-ACTION Responses from non-EDR LSMSs and all M-CREATE Responses from all EDR LSMSs. - The NPAC SMS automatically retries any LSMS who does not respond within a tunable amount of time. If a non-EDR LSMS does not respond, the NPAC will retry with an M-ACTION Request. If an EDR LSMS does not respond, the NPAC will retry with an M-CREATE Request. |  |  |
| - 7. | - NPAC | Upon the first successful response from an LSMS, the NPAC SMS sets the following timestamps to the current date and time: - numberPoolBlockActivationCompleteTimeStamp - subscriptionActivationCompleteTimeStamp - numberPoolBlockModifiedTimeStamp - subscriptionModifiedTimeStamp |  | - NPAC |  | The NPAC SMS does not receive a response from one non-EDR LSMS and one EDR LSMS.. |  |  |
| - 8. | - NPAC | - The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself and updates the following attributes for each Pooled Subscription Version within the 1K Block: - Sets the subscriptionVersionStatus to ‘partial failure’. - Sets the subscriptionFailedSP-List to reflect the EDR LSMS that failed the Number Pool Block Create Request. - On Subscription Version ‘A’, sets the subscriptionFailedSP-List to reflect the non-EDR LSMS that failed this TN. - Sets the subscriptionModifiedTimeStamp is set to the current date and time. - The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself and updates the following attributes: - Sets the numberPoolBlockStatus to ‘partial failure’. - Sets the numberPoolBlockFailedSP-List to reflect the EDR LSMS that failed the Number Pool Block Create Request, and the non-EDR LSMS that failed Subscription Version ‘A’. - Sets the numberPoolBlockModifiedTimeStamp to the current date and time. |  | - NPAC |  | - The NPAC SMS issues an M-SET subscriptionVersionNPAC Response to itself. - The NPAC SMS issues an M-SET numberPoolBlockNPAC Response to itself. |  |  |
| - 9. | - NPAC | - The NPAC SMS determines the SOA Origination Indicator is set to TRUE and issues an M-EVENT-REPORT numberPoolBlockStatusAttributeValueChange to the NPA-NXX-X Holder SOA to set the numberPoolBlockStatus to ‘partial failure’, and set the numberPoolBlockFailedSP-List to reflect the EDR LSMS and the non-EDR LSMS that did not successfully process the request. |  | - SP |  | - The NPA-NXX-X Holder SOA receives the M-EVENT-REPORT from the NPAC SMS and issues an M-EVENT-REPORT Confirmation back to the NPAC SMS. |  |  |
| - 10. | - NPAC | - NPAC Personnel perform a query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | - NPAC |  | - Verify the Number Pool Block exists with a status of ‘partial failure’ and a Failed SP List. The Failed SP List contains the names of the non-EDR system that failed Subscription Version ‘A’ and the EDR system that failed the Number Pool Block create. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ exist with a status of ‘partial failure’ and a Failed SP List. The Failed SP List for all Subscription Versions contain the name of the failed EDR system and for Subscription Version ‘A’ the name of the non-EDR system. |  |  |
| - 11. | - SP - Optional | - Block Holder Service Provider Personnel perform a local query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | #### SP |  | Verify the Number Pool Block exists with a status of ‘partial failure’ and a Failed SP List on the SOA. The Failed SP List contains the names of the non-EDR system that failed Subscription Version ‘A’ and the EDR system that failed the Number Pool Block create. |  |  |
| - 15. | - SP – Conditional | - Block Holder Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | #### SP |  | - Verify the Number Pool Block exists with a status of ‘partial failure’ and a Failed SP List on the NPAC SMS. The Failed SP List contains the names of the non-EDR system that failed Subscription Version ‘A’ and the EDR system that failed the Number Pool Block create. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ exist with a status of ‘partial failure’ and a Failed SP List on the NPAC SMS. The Failed SP List for all Subscription Versions contain the name of the failed EDR system and for Subscription Version ‘A’ the name of the non-EDR system. |  |  |

| - A. | - TEST IDENTITY |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Test Case Number: |  | - 4.1.14 |  | - Priority: |  | - SOA LTI; - N/A. - SOA; - C. - non-EDR LSMS; - R. - EDR LSMS; - R. |
|  | - Objective: |  | - SOA – Service Provider Personnel create a Number Pool Block (to at least 4 LSMSs - 2 non-EDR and 2 EDR) that results in a Partial Failure (1 non-EDR system fails one TN and 2 EDR systems fails) – Success |  |  |  |  |  |
| - B. | - REFERENCES |  |  |  |  |  |  |  |
|  | - NANC Change Order Revision Number: |  |  |  | - Change Order Number(s): |  |  |  |
|  | - NANC FRS Version Number: |  |  |  | - Relevant Requirement(s): |  | - RR3-132, Table RR3-137.2 (Row 12), RR3-138.1, Table RR3-138.2 (Row 12), RR3-153, RR5-100, RR5-101, RR5-95 |  |
|  | - NANC IIS Version Number: |  |  |  | - Relevant Flow(s): |  | - 2.1 Number Pool Block Create by SOA - 2.5.1 Number Pool Block Create Partial-Failure Broadcast to all Local SMSs - 2.5.2 Number Pool Block Create Broadcast Partially Failed NPAC SMS Updates |  |
| - C. | - PREREQUISITE |  |  |  |  |  |  |  |
|  | - Prerequisite Test Cases: |  |  |  |  |  |  |  |
|  | - Prerequisite NPAC Setup: |  | - Verify that at least four LSMSs (2 non-EDR and 2 EDR) are configured to be associated with the NPAC SMS and receive downloads for this NPA-NXX. One of the non-EDR system shall be made to fail one TN within the 1K Block. Two EDR systems shall be made to fail the Number Pool Block Create request. This will achieve a ‘partial failure’ download. - Verify that the respective NPA-NXX-X exists for which Service Provider Personnel will attempt to create the respective Number Pool Block during this Test Case. - Verify that the current date is equal to or greater than the NPA-NXX-X Effective Date. - Verify that no ‘pending-like, nor active-like’ Subscription Versions exist for the 1K Block so that a non-contaminated Number Pool Block may be created |  |  |  |  |  |
|  | - Prerequisite SP Setup: |  |  |  |  |  |  |  |
| - D. | - TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |
| - Row # | - NPAC or SP | - Test Step |  | - NPAC or SP |  | - Expected Result |  |  |
| - 1. | - SP | - Using the SOA, Service Provider Personnel submit an M-ACTION numberPoolBlock – Create request to the NPAC SMS to create a Number Pool Block. The request must include the following attributes: - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | #### NPAC |  | - The NPAC SMS receives the M-ACTION numberPoolBlock – Create request. - The NPAC SMS verifies the following information: - The requesting SOA is the NPA-NXX-X Holder SOA. - The serviceProvNPA-NXX-X object exists for the NPA-NXX-X (respective NPA-NXX-X information). - All attributes specified are valid - A numberPoolBlockNPAC object does not already exist for the NPA-NXX-X (a duplicate Number Pool Block does not already exist) or if one exists it has a status of ‘old’ with an empty Failed SP List. - The current date is greater than or equal to the NPA-NXX-X-EffectiveTimeStamp. - There are not any ‘pending-lik, no-active’ Subscription Version objects within the given TN range. |  |  |
| - 2. | - NPAC | - The NPAC SMS issues an M-CREATE Request numberPoolBlockNPAC to itself. - The NPAC SMS sets the numberPoolBlockSOA-Origination Indicator to TRUE. - The NPAC SMS sets the numberPoolBlockStatus to’sending’. - The NPAC SMS sets the following timestamps to the current date and time: - numberPoolBlockCreationTimeStamp - numberPoolBlockActivationTimeStamp - numberPoolBlockBroadcastTimeStamp - numberPoolBlockModifiedTimeStamp |  | - NPAC |  | - The NPAC SMS issues an M-CREATE Response numberPoolBlockNPAC to itself. |  |  |
| - 3. | - NPAC | - The NPAC SMS issues an M-CREATE Request subscriptionVersionNPAC to itself. - The NPAC SMS sets the LNP Type to ‘POOL’ for the Subscription Versions it creates within the 1K Block. - The NPAC SMS sets the Subscription Versions status to ‘sending’. - The NPAC SMS sets the following timestamps to the current date and time for the Subscription Versions: - subscriptionModifiedTimeStamp - subscriptionActivationTimeStamp - subscriptionBroadcastTimeStamp - subscriptionCreationTimeStamp |  | - NPAC |  | - The NPAC SMS issues an M-CREATE Response subscriptionVersionNPAC to itself. |  |  |
| - 4. | - NPAC | - The NPAC SMS issues an M-ACTION Response numberPoolBlock – Create to the respective NPA-NXX-X Holder SOA that initiated the Number Pool Block Create request. |  | - SP |  | - The NPA-NXX-X Holder SOA receives the M-ACTION Response from the NPAC SMS. |  |  |
| - 5. | - NPAC | - The NPAC SMS issues an M-EVENT-REPORT objectCreation for the numberPoolBlockNPAC to the NPA-NXX-X Holder SOA. - The following attributes are sent in the objectCreation notification: - numberPoolBlockId - numberPoolBlockSOA-Origination - numberPoolBlockCreationTimeStamp - numberPoolBlockStatus - numberPoolBlockNPA-NXX-X - numberPoolBlockSPID - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | - SP |  | - The NPA-NXX-X Holder SOA receives the M-EVENT-REPORT objectCreation and issues an M-EVENT-REPORT Confirmation back to the NPAC SMS. |  |  |
| - 6. | - NPAC | - The NPAC SMS issues an M-ACTION Request subscriptionVersionLocalSMS-Create to the non-EDR LSMSs in the region that are accepting downloads for this NPA-NXX. - At the same time as step 6.1, the NPAC SMS issues an M-CREATE Request numberPoolBlock to the EDR LSMSs in the region that are accepting downloads for this NPA-NXX. |  | #### NPAC |  | - The non-EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-ACTION Request, verify that the action is valid and return an M-ACTION Response subscriptionVersionLocalSMS-Create. - The EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-CREATE Request, and return an M-CREATE Response numberPoolBlock. - The non-EDR LSMSs proceed to execute all the creates specified by the M-ACTION Request and issue an M-EVENT-REPORT subscriptionVersionLocalSMS-CreateResults specifying the Subscription Versions were successfully created. - The NPAC SMS waits for all M-ACTION Responses from non-EDR LSMSs and all M-CREATE Responses from all EDR LSMSs. - The NPAC SMS automatically retries any LSMS who does not respond within a tunable amount of time. If a non-EDR LSMS does not respond, the NPAC will retry with an M-ACTION Request. If an EDR LSMS does not respond, the NPAC will retry with an M-CREATE Request. |  |  |
| - 7. | - NPAC | Upon the first successful response from an LSMS, the NPAC SMS sets the following timestamps to the current date and time: - numberPoolBlockActivationCompleteTimeStamp - subscriptionActivationCompleteTimeStamp - numberPoolBlockModifiedTimeStamp - subscriptionModifiedTimeStamp |  | - NPAC |  | The NPAC SMS does not receive a response from one non-EDR LSMS and two EDR LSMSs. |  |  |
| - 8. | - NPAC | - The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself and updates the following attributes for each Pooled Subscription Version within the 1K Block: - Sets the subscriptionVersionStatus to ‘partial failure’. - Sets the subscriptionFailedSP-List to reflect the two EDR LSMSs that failed the Number Pool Block Create Request. - On Subscription Version ‘A’, sets the subscriptionFailedSP-List to reflect the non-EDR LSMS that failed this TN. - Sets the subscriptionModifiedTimeStamp is set to the current date and time. - The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself and updates the following attributes: - Sets the numberPoolBlockStatus to ‘partial failure’. - Sets the numberPoolBlockFailedSP-List to reflect the two EDR LSMSs that failed the Number Pool Block Create Request, and the non-EDR LSMS that failed Subscription Version ‘A’. - Sets the numberPoolBlockModifiedTimeStamp to the current date and time. |  | - NPAC |  | - The NPAC SMS issues an M-SET subscriptionVersionNPAC Response to itself. - The NPAC SMS issues an M-SET numberPoolBlockNPAC Response to itself. |  |  |
| - 9. | - NPAC | - The NPAC SMS determines the SOA Origination Indicator is set to TRUE and issues an M-EVENT-REPORT numberPoolBlockStatusAttributeValueChange to the NPA-NXX-X Holder SOA to set the numberPoolBlockStatus to ‘partial failure’, and set the numberPoolBlockFailedSP-List to reflect the two EDR LSMSs and the non-EDR LSMS that did not successfully process the request. |  | - SP |  | - The NPA-NXX-X Holder SOA receives the M-EVENT-REPORT from the NPAC SMS and issues an M-EVENT-REPORT Confirmation back to the NPAC SMS. |  |  |
| - 10. | - NPAC | - NPAC Personnel perform a query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | - NPAC |  | - Verify the Number Pool Block exists with a status of ‘partial failure’ and a Failed SP List. The Failed SP List contains the names of the non-EDR system that failed Subscription Version ‘A’ and the two EDR systems that failed the Number Pool Block create. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ exist with a status of ‘partial failure’ and a Failed SP List. The Failed SP List for all Subscription Versions contain the name of the two failed EDR systems and for Subscription Version ‘A’ the name of the non-EDR system. |  |  |
| - 11. | - SP - Optional | - Block Holder Service Provider Personnel perform a local query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | #### SP |  | Verify the Number Pool Block exists with a status of ‘partial failure’ and a Failed SP List on the SOA. The Failed SP List contains the names of the non-EDR system that failed Subscription Version ‘A’ and the two EDR systems that failed the Number Pool Block create. |  |  |
| - 15. | - SP – Conditional | - Block Holder Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | #### SP |  | - Verify the Number Pool Block exists with a status of ‘partial failure’ and a Failed SP List on the NPAC SMS. The Failed SP List contains the names of the non-EDR system that failed Subscription Version ‘A’ and the two EDR systems that failed the Number Pool Block create. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ exist with a status of ‘partial failure’ and a Failed SP List on the NPAC SMS. The Failed SP List for all Subscription Versions contain the name of the two failed EDR systems and for Subscription Version ‘A’ the name of the non-EDR system. |  |  |

## 4.2 Modify Number Pool Block Test Cases:

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.2.1 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | R |
|  |  |  |  |  |  |  | EDR LSMS |  | R |
|  | Objective: |  | SOA- Service Provider Personnel modify an active Number Pool Block with the SOA Origination Indicator set to FALSE (and contains Subscription Versions with LNP Types of ‘POOL’, ‘LISP’ and ‘LSPP’) for at least 4 LSMSs (2 non-EDR and 2 EDR). - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-119, RR3-120, RR3-121, RR3-122, RR3-128, RR3-133, RR3-157, RR3-159, RR3-160, RR3-162, RR3-163, RR3-164, RR3-165, RR3-167, RR3-168, RR5-85, RR5-86, RR5-87, RR5-103, RR5-104, RR5-105 |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  | 2.11 Number Pool Block Modify by Block Holder SOA 2.12.1 Number Pool Block modify Successful Broadcast to Local SMS 2.12.2 Number Pool Block Modify Successful Broadcast NPAC SMS Updates |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify the Number Pool Block to be modified exists on the NPAC SMS with a status of ‘active’ and an empty Failed SP List. - Verify that at least two Service Providers are connected to the NPAC SMS and configured to receive data downloads for this NPA-NXX (one EDR and one non-EDR LSMS). - Verify that the Number Pool Block SOA-Origination Indicator is set to FALSE. - Verify that LISP and LSPP Subscription Versions exist for some TNs in the 1K Block. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | All Service Providers verify either the Number Pool Block or 1K Block of Subscription Versions with LNP Type set to ‘POOL’ to be modified exists locally. |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the SOA, Service Provider Personnel submit an M-SET Request numberPoolBlock to modify the LRN for a Number Pool Block. The following attributes may be modified: - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | NPAC |  | - The NPAC SMS receives the M-SET Request numberPoolBlock. - The NPAC SMS performs the following actions: - Updates the modified attributes in the Number Pool Block object. - Sets the numberPoolBlockStatus to 'sending'. - Updates the numberPoolBlockBroadcastTimeStamp and numberPoolBlockModifiedTimeStamp to the current date and time. |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-SET Response numberPoolBlock to the Service Provider SOA. |  | SP |  | The Service Provider SOA receives the M-SET Response numberPoolBlock. |  |  |  |
| 3. | NPAC | The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself to modify the attribute data on the corresponding subscriptionVersionNPAC object(s). |  | SP |  | - The NPAC SMS issues an M-SET Response subscriptionVersionNPAC to itself. - The NPAC SMS performs the following actions: - Updates the modified attributes in the Subscription Versions within the 1K Block with LNP Type set to ‘POOL’. - Sets the subscriptionVersionStatus to ‘sending’. - Updates the subscriptionVersionBroadcastTimeStamp and the subscriptionVersionModifiedTimeStamp to the current date and time. |  |  |  |
| 4. | NPAC | - The NPAC SMS issues an M-SET Request subscriptionVersion to update the attributes on all Subscription Versions in the 1K Block with LNP Type set to ‘POOL’ to the non-EDR LSMSs that are accepting downloads for this NPA-NXX. - At the same time as step 4.1, the NPAC SMS issues an M-SET Request numberPoolBlock to update the attributes on the Number Pool Block object to the EDR LSMSs that are accepting downloads for this NPA-NXX. |  | SP |  | - All non-EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-SET Request, verify that the action is valid and return an M-SET Response subscriptionVersion back to the NPAC SMS. - All EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-SET Request, verify that the action is valid and return an M-SET Response numberPoolBlock back to the NPAC SMS. |  |  |  |
| 5. | NPAC | Once the first successful response is received from any LSMS, the following occurs: - The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself to set the Subscription Version Status to 'active', update the Failed SP List to empty, and update the subscriptionModifiedTimeStamp to the current date and time. - The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself to set the Number Pool Block status to 'active', update the Failed SP List to empty and update the numberPoolBlockModifiedTimeStamp to the current date and time. |  | NPAC |  | - The NPAC SMS issues an M-SET Response subscriptionVersionNPAC. - The NPAC SMS issues an M-SET Response numberPoolBlockNPAC. |  |  |  |
| 6. | NPAC | The NPAC SMS determines the numberPoolBlockSOA-Origination indicator is set to FALSE, and further processing is terminated here. |  |  |  |  |  |  |  |
| 7. | NPAC | NPAC Personnel perform a query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ as well as ‘LISP’ and ‘LSPP’. |  | NPAC |  | - Verify the Number Pool Block was successfully modified and the status is set to ‘active’ with an empty Failed SP List. - Verify the Subscription Versions with LNP Type set to ‘POOL’ in the 1K Block were successfully modified and their status is set to ‘active’ with an empty Failed SP List. - Verify the Subscription Versions within the 1K Block with LNP Type set to ‘LISP’ and ‘LSPP’ have not been modified on any LSMS. - Verify the NPAC SMS generated a Number Pool Block with a unique ID, all attributes prior to modification, and the status is set to ‘old’ with an empty Failed SP List. |  |  |  |
| 8. | NPAC | NPAC Personnel verify that the ‘old’ Number Pool Block that was created as a result of the modification, did not get broadcast. |  | NPAC |  | Verify the NPAC SMS did not broadcast the ‘old’ Number Pool Block. |  |  |  |
| 9. | SP – Optional | Service Provider Personnel perform a local query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ as well as ‘LISP’ and ‘LSPP’. |  | SP |  | - Verify the Service Providers that were receiving downloads for the NPA-NXX and have an EDR LSMS received the modification for Number Pool Block and that it was modified appropriately. - Verify the Service Providers that were receiving downloads for the NPA-NXX and have a non-EDR LSMS received the modifications for the Subscription Versions with LNP Type set to ‘POOL’ in the 1K Block, and that the Subscription Versions were modified appropriately. - Verify the Service Providers that were not receiving downloads for the NPA-NXX (whether EDR or non-EDR LSMSs) did not receive the modifications and the Number Pool Block as well as the 1K Block of Subscription Version with LNP Type set to ‘POOL’, remain unchanged. - Verify the Subscription Versions within the 1K Block with LNP Type set to ‘LISP’ and ‘LSPP’ have not been modified on any LSMS. |  |  |  |
| 10. | SP - Conditional | Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and 1K Block of Subscription Versions with LNP Type set to ‘POOL’ as well as ‘LISP’ and ‘LSPP’. |  | SP |  | - Verify the Number Pool Block was successfully modified and the status is set to ‘active’ with an empty Failed SP List on the NPAC SMS. - Verify the Subscription Versions of LNP Type set to ‘POOL’ in the 1K Block were successfully modified and their status is set to ‘active’ with an empty Failed SP List on the NPAC SMS. - Verify the Subscription Versions within the 1K Block with LNP Type set to ‘LISP’ and ‘LSPP’ have not been modified on the NPAC SMS - Verify the Number Pool Block exists on the NPAC SMS with a unique ID, all attributes prior to modification, and the status is set to ‘old’ with an empty Failed SP List. |  |  |  |
| 11. | SP – Conditional | Service Provider Personnel verify that the ‘old’ Number Pool Block that was created as a result of the modification, did not get broadcast. |  | SP |  | Verify the ‘old’ Number Pool Block did not get broadcast. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.2.2 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | R |
|  |  |  |  |  |  |  | EDR LSMS |  | R |
|  | Objective: |  | SOA – Service Provider Personnel modify the LRN for an active Number Pool Block and broadcast to at least 4 LSMSs (2 EDR and 2 non-EDR systems) resulting in Full Failure – Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-137.3, Table RR3-137.3 (Row 15), RR3-138.2, Table RR3-138.2 (Row 15), RR3-128, RR3-141.3, RR3-157, RR3-159, RR3-160, RR3-162, RR3-163, RR3-164, RR3-165, RR3-166, RR5-85, RR5-87, RR5-103, RR5-104, RR5-105, RR5-106 |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  | 2.11 Number Pool Block Modify by Block Holder SOA 2.13 Number Pool Block Modify Broadcast to Local SMS Failure |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify the Number Pool Block to be modified exists on the NPAC SMS with a status of ‘active’, an empty Failed SP List and the SOA Origination Indicator set to TRUE. - Verify that at least 4 LSMSs (2 EDR and 2 non-EDR) are configured such that they will be sent downloads for this NPA-NXX. Keep all 4 LSMSs disconnected from the NPAC SMS to create a full failure scenario. - Verify the LRN that is to be used exists on the NPAC SMS and is owned by the Number Pool Block Holder. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | All Service Providers verify that the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ to be modified exist locally. |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the SOA, Service Provider Personnel submit an M-SET Request numberPoolBlock to the NPAC SMS to modify a Number Pool Block. The following attributes may be modified: - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | NPAC |  | - The NPAC SMS receives the M-SET Request numberPoolBlock - The NPAC SMS performs the following actions: - Updates the LRN in the Number Pool Block object. - Sets the numberPoolBlockStatus to 'sending'. - Updates the numberPoolBlockBroadcastTimeStamp and numberPoolBlockModifiedTimeStamp to the current date and time. |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-SET Response numberPoolBlock to the Service Provider SOA |  | NPAC |  | The Service Provider SOA receives the M-SET Response numberPoolBlock. |  |  |  |
| 3. | NPAC | The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself. |  | NPAC |  | - The NPAC SMS issues an M-SET Response subscriptionVersionNPAC to itself. - The NPAC SMS performs the following actions: - Updates the LRN in the Subscription Versions within the 1K Block with LNP Type set to ‘POOL’. - Sets the subscriptionVersionStatus to ‘sending’. - Updates the subscriptionVersionBroadcastTimeStamp and the subscriptionVersionModifiedTimeStamp to the current date and time. |  |  |  |
| 4. | NPAC | - The NPAC SMS issues an M-SET Request subscriptionVersion to update the attributes on the Subscription Version objects on all Subscription Versions of LNP Type set to ‘POOL’ to the non-EDR LSMSs that are accepting downloads for this NPA-NXX. - At the same time as step 4.1, the NPAC SMS issues an M-SET Request numberPoolBlock to update the attributes on the Number Pool Block object to the EDR LSMSs that are accepting downloads for this NPA-NXX. |  | SP |  | - The NPAC SMS waits for a response from all LSMSs that are accepting downloads for this NPA-NXX. - The NPAC SMS retries any LSMS that does not respond within a tunable amount of time. - None of the LSMSs that are accepting downloads for this NPA-NXX respond to the M-SET Request. |  |  |  |
| 5. | NPAC | After all retries have been exhausted, the NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself and performs the following actions: - updates the subscriptionVersionStatus to 'active' and the Failed SP List to empty for Subscription Versions within the 1K Block with LNP Type set to ‘POOL’. - updates the Failed SP List to include all 4 of the Service Providers in the region that are accepting downloads for that NPA-NXX (2 EDR and 2 non-EDR systems) and did not respond to the NPAC SMS request. - updates the subscriptionModifiedTimeStamp to the current date and time |  | NPAC |  | The NPAC SMS issues an M-SET Response to itself. |  |  |  |
| 6. | NPAC | The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself and performs the following actions: - updates the numberPoolBlockStatus to 'active'. - updates the numberPoolBlockFailedSP-List to include all 4 of the Service Providers in the region that are accepting downloads for that NPA-NXX (2 EDR and 2 non-EDR systems) and did not respond to the NPAC SMS request. - updates the numberPoolBlockModifiedTimeStamp to the current date and time |  | NPAC |  | The NPAC SMS issues an M-SET Response to itself. |  |  |  |
| 7. | NPAC | The NPAC SMS determines the numberPoolBlockSOA-Origination Indicator is set to TRUE and issues an M-EVENT-REPORT numberPoolBlockStatusAttributeValueChange with the numberPoolBlockStatus set to ‘active’ and the numberPoolBlockFailedSP List reflecting the 4 Service Providers that failed to process the NPAC SMS request to the NPA-NXX-X Holder SOA. |  | SP |  | The NPA-NXX-X Holder SOA issues an M-EVENT-REPORT Confirmation back to the NPAC SMS. |  |  |  |
| 8. | NPAC | NPAC Personnel perform a query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | NPAC |  | - Verify the Number Pool Block was successfully modified. - Verify the Number Pool Block has a status of ‘active’ with a Failed SP List. The Failed SP List contains the names of the 4 Service Providers that failed to receive the downloads. - Verify the Subscription Versions of LNP Type set to ‘POOL’ in the 1K Block were successfully modified. - Verify the Subscription Versions of LNP Type set to ‘POOL’ in the 1K Block have a status of ‘active’ with a Failed SP List. The Failed SP List contains the names of the 4 Service Providers that failed to receive the downloads. |  |  |  |
| 9. | SP – Optional | Service Provider Personnel perform a local query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ |  | SP |  | - Verify the Number Pool Block was not modified. - Verify the Subscription Versions of LNP Type set to ‘POOL’ in the 1K Block were not modified. |  |  |  |
| 10. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ |  | SP |  | - Verify the Number Pool Block was successfully modified on the NPAC SMS. - Verify the Number Pool Block has a status of ‘active’ with a Failed SP List on the NPAC SMS. The Failed SP List contains the names of the 4 Service Providers that failed to receive the downloads. - Verify the Subscription Versions of LNP Type set to ‘POOL’ in the 1K Block were successfully modified on the NPAC SMS. - Verify the Subscription Versions of LNP Type set to ‘POOL’ in the 1K Block have a status of ‘active’ with a Failed SP List on the NPAC SMS. The Failed SP List contains the names of the 4 Service Providers that failed to receive the downloads. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.2.3 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | R |
|  |  |  |  |  |  |  | EDR LSMS |  | R |
|  | Objective: |  | SOA - Service Provider Personnel modify the routing data for an active Number Pool Block and broadcast to at least 4 LSMSs (2 EDR and 2 non-EDR systems). resulting in Partial Failure - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-137.3, Table RR3-137.3 (Row 9), RR3-138.2, Table RR3-138.2 (Row 9), RR3-128, RR3-157, RR3-159, RR3-160, RR3-162, RR3-163, RR3-164, RR3-165, RR3-166, RR5-85, RR5-87, RR5-103, RR5-104, RR5-105, RR5-106 |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  | 2.11 Number Pool Block Modify by Block Holder SOA 2.14.1 Number Pool Block Modify Partial Failure Broadcast to Local SMSs 2.14.2 Number Pool Block Modify Partial Failure Broadcast NPAC SMS Updates |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify the Number Pool Block to be modified exists on the NPAC SMS with a status of ‘active’, an empty Failed SP List and the SOA Origination Indicator is set to TRUE. - Verify that at least 4 LSMSs (2 EDR and 2 non-EDR) are configured such that they will be sent downloads for this NPA-NXX. - Verify that only one LSMS (non-EDR) system that is accepting downloads for the NPA-NXX is associated with the NPAC SMS. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  | All Service Providers verify the Number Pool Block and 1K Block of Pooled Subscription Versions with LNP Type set to ‘POOL’ to be modified exist locally. |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the SOA, Service Provider Personnel submit an M-SET Request numberPoolBlock to the NPAC SMS to modify a Number Pool Block. The following attributes may be modified: - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | NPAC |  | - The NPAC SMS receives the M-SET Request numberPoolBlock - The NPAC SMS performs the following actions: - Updates the LRN in the Number Pool Block object. - Sets the numberPoolBlockStatus to 'sending'. - Updates the numberPoolBlockBroadcastTimeStamp and numberPoolBlockModifiedTimeStamp to the current date and time. |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-SET Response numberPoolBlock to the Service Provider SOA |  | NPAC |  | The Service Provider SOA receives the M-SET Response numberPoolBlock. |  |  |  |
| 3. | NPAC | The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself. |  | SP |  | - The NPAC SMS performs the following actions: - Updates the LRN in the Subscription Versions within the 1K Block with LNP Type set to ‘POOL’. - Sets the subscriptionVersionStatus to ‘sending’. - Updates the subscriptionVersionBroadcastTimeStamp and the subscriptionVersionModifiedTimeStamp to the current date and time. - The NPAC SMS issues an M-SET Response subscriptionVersionNPAC to itself. |  |  |  |
| 4. | NPAC | - The NPAC SMS issues an M-SET Request subscriptionVersion to update the attributes on the Subscription Version objects on all Subscription Versions of LNP Type set to ‘POOL’ to the non-EDR LSMSs that are accepting downloads for this NPA-NXX. - At the same time as step 4.1, the NPAC SMS issues an M-SET Request numberPoolBlock to update the attributes on the Number Pool Block object to the EDR LSMSs that are accepting downloads for this NPA-NXX. |  | SP |  | - The NPAC SMS waits for a response from all LSMSs that are accepting downloads for this NPA-NXX. - One non-EDR LSMS that is accepting downloads for this NPA-NXX issues an M-SET Response indicating it successfully received the modify request. - The NPAC SMS retries any LSMS that does not respond within a tunable amount of time. |  |  |  |
| 5. | NPAC | After all retries have been exhausted, the NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself and performs the following actions: - updates the subscriptionVersionStatus to 'active' for Subscription Versions within the 1K Block with LNP Type set to ‘POOL’. - updates the Failed SP List to include the Service Providers in the region that are accepting downloads for that NPA-NXX (2 EDR and 1 non-EDR systems) and did not successfully respond to the NPAC SMS request. - updates the subscriptionModifiedTimeStamp to the current date and time. |  | NPAC |  | The NPAC SMS issues an M-SET Response to itself. |  |  |  |
| 6. | NPAC | The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself and performs the following actions: - updates the numberPoolBlockStatus to 'active'. - updates the numberPoolBlockFailedSP-List to include the Service Providers in the region that are accepting downloads for that NPA-NXX (2 EDR and 1 non-EDR systems) and did not successfully respond to the NPAC SMS request. - updates the numberPoolBlockModifiedTimeStamp to the current date and time. |  | NPAC |  | The NPAC SMS issues an M-SET Response to itself. |  |  |  |
| 7. | NPAC | The NPAC SMS determines the numberPoolBlockSOA-Origination Indicator is set to TRUE and issues an M-EVENT-REPORT numberPoolBlockStatusAttributeValueChange with the numberPoolBlockStatus set to ‘active’ and the numberPoolBlockFailedSP List reflecting the 3 Service Providers that failed to process the NPAC SMS request – to the NPA-NXX-X Holder SOA. |  | SP |  | The NPA-NXX-X Holder SOA issues an M-EVENT-REPORT Confirmation back to the NPAC SMS. |  |  |  |
| 8. | NPAC | NPAC Personnel perform a query for the Number Pool Block and the 1K Block of Subscription Versions with a LNP Type set to ‘POOL’. |  | NPAC |  | - Verify the Number Pool Block was successfully modified. - Verify the Number Pool Block has a status of ‘active’ with a Failed SP List. The Failed SP List contains the name of the EDR and non-EDR systems that failed. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ were successfully modified. - Verify all Subscription Versions in the 1K Block have a status of ‘active’ and the Failed SP List contains the name of the EDR and non-EDR systems that failed. |  |  |  |
| 9. | SP – Optional | Service Provider Personnel perform a local query for the Number Pool Block or the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | SP |  | - Verify the EDR LSMS does not reflect the modifications to the Number Pool Block. - Verify that one non-EDR LSMS does reflect the modifications to the 1K Block of Subscription Versions with LNP Type set to ‘POOL’, while the other non-EDR LSMS does not reflect these modifications. |  |  |  |
| 10. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Number Pool Block or the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | SP |  | - Verify the Number Pool Block was successfully modified on the NPAC SMS. - Verify the Number Pool Block has a status of ‘active’ with a Failed SP List on the NPAC SMS. The Failed SP List contains the name of the EDR and non-EDR systems that failed. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ were successfully modified on the NPAC SMS. - Verify all Subscription Versions in the 1K Block have a status of ‘active’ and a Failed SP List on the NPAC SMS. The Failed SP List contains the name of the EDR and non-EDR systems that failed. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.2.4 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | O |
|  |  |  |  |  |  |  | non-EDR LSMS |  | R |
|  |  |  |  |  |  |  | EDR LSMS |  | R |
|  | Objective: |  | NPAC OP GUI - NPAC Personnel re-send a failed Number Pool Block Modify Request for both EDR and non-EDR LSMSs – Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-140, RR3-142.2, RR3-185, RR3-192, RR3-193, RR3-194, RR3-195, RR3-196, RR3-197, RR5-85, RR5-86, RR5-75, RR5-77, RR5-78, RR5-79 |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  | 2.15 Number Pool Block Modify Resend Broadcast 2.16 Number Pool Block Modify Successful Resend updates |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  | 4.2.2 SOA – Service Provider Personnel modify the LRN for an active Number Pool Block and broadcast to at least 4 LSMSs (2 EDR and 2 non-EDR systems) resulting in Full Failure – Success |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that all LSMSs that are listed in the Failed SP List (at least 1 EDR and 1 non-EDR system) for the Number Pool Block that NPAC Personnel will resend during this Test Case are connected to the NPAC SMS and configured to receive downloads for the NPA-NXX. - Verify the Number Pool Block and 1K Block of Subscription Versions with LNP Type set to ‘POOL’ exist with a status of ‘active’ and an empty Failed SP List. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | - Using the NPAC OP GUI, NPAC Personnel submit a request to resend a failed Number Pool Block Modify Request to each Service Provider in the Failed SP List. - The NPAC SMS issues an M-SET Request numberPoolBlock to itself to set the numberPoolBlockStatus to ‘sending’ and update the numberPoolBlockModifiedTimeStamp and numberPoolBlockBroadcastTimeStamp to the current date and time. - The NPAC SMS issues an M-SET subscriptionVersionNPAC to itself to set the subscriptionVersionStatus to ‘sending’ and update the subscriptionModifiedTimeStamp and subscriptionBroadcastTimeStamp to the current date and time for each Subscription Version within the 1K Block with LNP Type set to ‘POOL’. |  | NPAC |  | - The NPAC SMS issues an M-SET Response numberPoolBlockNPAC to itself. - The NPAC SMS issues an M-SET Response subscriptionVersionNPAC to itself. |  |  |  |
| 2. | NPAC | - The NPAC SMS issues an M-SET Request subscriptionVersion to the non-EDR LSMS(s) that is on the Subscription Version Failed SP List. - At the same time as step 2.1, the NPAC SMS issues an M-SET Request numberPoolBlock to the EDR LSMS(s) that is on the Number Pool Block Failed SP List. |  | SP |  | - All non-EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-SET Request and issue an M-SET Response back to the NPAC SMS. - All EDR LSMSs that are accepting downloads for this NPA-NXX receive the M-SET Request and issue an M-SET Response back to the NPAC SMS. |  |  |  |
| 3. | NPAC | - Upon the first successful rseponse from an LSMS, the NPAC SMS sets the following timestamps to the current date and time: - numberPoolBlockModifiedTimeStamp - subscriptionModifiedTimeStamp - After a successful response from all LSMSs the resend request was sent to, the NPAC SMS issues an M-SET numberPoolBlockNPAC to itself and performs the following steps: - updates the numberPoolBlock status to ‘active’ and the Failed SP List to empty. - updates the numberPoolBlockModifiedTimeStamp to the current date and time. - At the same time as step 3.2, the NPAC SMS issues an M-SET subscriptionVersionNPAC to itself and performs the following steps for each Subscription Version within the 1K Block of LNP Type, ‘POOL’: - updates the subscriptionVersionStatus to ‘active’ and the Failed SP List to empty. - updates the subscriptionModifiedTimeStamp to the current date and time. |  | NPAC |  | The NPAC SMS issues an M-SET Response to itself. |  |  |  |
| 4. | NPAC | The NPAC SMS determines the numberPoolBlockSOA-Origination Indicator is set to TRUE and issues an M-EVENT-REPORT numberPoolBlockStatusAttributeValueChange with the numberPoolBlockStatus set to ‘active’ and the numberPoolBlockFailedSP List is set to empty. |  | SP |  | The NPA-NXX-X Holder SOA issues an M-EVENT-REPORT Confirmation back to the NPAC SMS. |  |  |  |
| 5. | NPAC | NPAC Personnel perform a query for the Number Pool Block and the 1K Block of Subscription Versions with a LNP Type set to ‘POOL’. |  | NPAC |  | - Verify the Number Pool Block was successfully modified. - Verify the Number Pool Block has a status of ‘active’ with an empty Failed SP List. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ were successfully modified. - Verify all Subscription Versions in the 1K Block have a status of ‘active’ and an empty Failed SP List. |  |  |  |
| 6. | SP – Optional | Service Provider Personnel perform a local query for the Number Pool Block and/or the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | SP |  | - Verify the Number Pool Block was successfully modified on the SOA and the EDR LSMS. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ were successfully modified on the non-EDR LSMS. |  |  |  |
| 7. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and/or the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | SP |  | - Verify the Number Pool Block was successfully modified on the NPAC SMS. - Verify the Number Pool Block has a status of ‘active’ with an empty Failed SP List on the NPAC SMS. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ were successfully modified on the NPAC SMS. - Verify all Subscription Versions in the 1K Block have a status of ‘active’ and an empty Failed SP List on the NPAC SMS. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.2.5 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | O |
|  |  |  |  |  |  |  | EDR LSMS |  | O |
|  | Objective: |  | SOA – Service Provider Personnel modify an active Number Pool Block with the SOA Origination Indicator set to TRUE, using an LRN that does not exist on the NPAC SMS for that Service Provider. – Error |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-131 |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  | 2.11 Number Pool Block Modify by Block Holder SOA |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify the Number Pool Block to be modified exists on the NPAC SMS with a status of ‘active’ and an empty Failed SP List. - Verify the LRN to be used does not exist on the NPAC SMS. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the SOA, Service Provider Personnel submit an M-SET Request numberPoolBlock to modify a Number Pool Block, specifying an LRN that does not exist on the NPAC SMS. |  | NPAC |  | - The NPAC SMS receives the M-SET Request numberPoolBlock - The NPAC SMS determines that the LRN value does not exist on the NPAC SMS. (This violates system requirements.) - The NPAC SMS does not modify the attribute on the numberPoolBlockNPAC object. |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-SET Error Response numberPoolBlockNPAC to the NPA-NXX-X Holder SOA indicating the error. |  | SP |  | The NPA-NXX-X Holder SOA receives the M-SET Error Response from the NPAC SMS. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a local query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | NPAC |  | - Verify the Number Pool Block has not been modified. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ has NOT been modified. |  |  |  |
| 4. | SP –Optional | Service Provider Personnel perform a local query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | SP |  | - Verify the Number Pool Block has not been modified. - Verify the 1K Block of Subscription Versions has NOT been modified. |  |  |  |
| 5. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | SP |  | - Verify the Number Pool Block has not been modified on the NPAC SMS. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ has NOT been modified on the NPAC SMS. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.2.6 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | R |
|  |  |  |  |  |  |  | EDR LSMS |  | R |
|  | Objective: |  | SOA – Service Provider Personnel attempt to modify a Number Pool Block for a Number Pool Block that has a status of ‘active’ with a Failed SP List. – Error |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-161 |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  | 2.11 Number Pool Block Modify by Block Holder SOA |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Verify that the Number Pool Block to be modified exists on the NPAC SMS with a status of ‘active’ and a Failed SP List. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the SOA, Service Provider Personnel submit an M-SET Request numberPoolBlock to modify an ‘active’ Number Pool Block with a Failed SP List. |  | NPAC |  | - The NPAC SMS receives the M-SET Request numberPoolBlock. - The NPAC SMS determines that the Number Pool Block specified in the modify request exists on the NPAC SMS with a status of ‘active’ and a Failed SP List. (This violates system requirements.) - The NPAC SMS does not modify the attribute on the numberPoolBlockNPAC object. - |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-SET Error Response numberPoolBlockNPAC to the NPA-NXX-X Holder SOA indicating there was an error. |  | SP |  | The NPA-NXX-X Holder SOA receives the M-SET Error Response from the NPAC SMS. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a local query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | NPAC |  | - Verify the Number Pool Block has not been modified. The status is ‘active’ with the same Failed SP List. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ has not been modified. The status is ‘active’ with the same Failed SP List. |  |  |  |
| 4. | SP – Optional | Service Provider Personnel perform a local query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | SP |  | - Verify the Number Pool Block has not been modified. - Verify the 1K of Subscription Versions with LNP Type set to ‘POOL’ has not been modified. |  |  |  |
| 5. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | SP |  | - Verify the Number Pool Block has not been modified on the NPAC SMS. The status is ‘active’ with the same Failed SP List. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ has not been modified on the NPAC SMS. The status is ‘active’ with the same Failed SP List. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.2.7 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | N/A |
|  |  |  |  |  |  |  | EDR LSMS |  | N/A |
|  | Objective: |  | NPAC OP GUI – NPAC Personnel modify the SOA Origination Indicator for a Number Pool Block - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-154, RR3-155 |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  | 2.18 Number Pool Block Modification of SOA-Origination Indicator |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Verify the Number Pool Block to be modified exists on the NPAC SMS with a status of ‘active’, an empty Failed SP List and the SOA Origination Indicator is set to FALSE. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | - Using the NPAC OP GUI, NPAC Personnel submit a request to modify the SOA Origination Indicator for a Number Pool Block that exists on the NPAC SMS. - The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself to change the value of the numberPoolBlockSOA-Origination to TRUE. |  | NPAC |  | The NPAC SMS receives the M-SET Request and issues an M-SET Response to itself. |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-EVENT-REPORT numberPoolBlockAttributeValueChange to the NPA-NXX-X Holder SOA for the Number Pool Block that contains the numberPoolBlockSOA-Origination Indicator set to TRUE. |  | SP |  | The NPA-NXX-X Holder SOA receives the M-EVENT-REPORT from the NPAC SMS and issues an M-EVENT-REPORT Confirmation back to the NPAC SMS. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a query for the Number Pool Block. |  | NPAC |  | Verify the Number Pool Block has the SOA Origination Indicator set to TRUE. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.2.8 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | N/A |
|  |  |  |  |  |  |  | non-EDR LSMS |  | N/A |
|  |  |  |  |  |  |  | EDR LSMS |  | N/A |
|  | Objective: |  | NPAC OP GUI - NPAC Personnel modify the scheduled date/time and routing data for a scheduled Number Pool Block Create Event prior to the NPA-NXX-X Effective Date – Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-80.1, RR3-80.2, RR3-80.3, RR3-82.2 |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  | N/A |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify the NPA-NXX-X exists on the NPAC SMS with a scheduled Block Create Event with a Scheduled Date later than the NPA-NXX-X Effective Date. - Verify the current date is PRIOR to the current NPA-NXX-X Effective Date. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel submit a request to modify the scheduled date/time and routing information for a scheduled Block Create Event. Specify a date that is equal to or greater than the NPA-NXX-X Effective Date. |  | NPAC |  | - The NPAC SMS performs the following validations for the modify request on the scheduled Number Pool Block Create Event: - Verifies the NPA-NXX-X object exists for the respective Number Pool Block. - Verifies all attributes specified are valid (performs field level validations). - Verifies that the new scheduled date/time is equal to or greater than the NPA-NXX-X Effective Date. - Verifies a numberPoolBlock object does not already exist for the NPA-NXX-X specified. - Verifies there are not any Subscription Versions within the 1K Block with a status of ‘pending’, ‘conflict’, ‘cancel-pending’ or ‘failed’, without a respective ‘active’ Subscription Version. - The NPAC SMS stores the modified information for the scheduled the Number Pool Block Create Event. |  |  |  |
| 2. | NPAC | Using the NPAC OP GUI, NPAC Personnel query for the Number Pool Block Create Event. |  | NPAC |  | - Verify the scheduled date/time and routing information stored in the Block Create Event reflects the modified values. - Verify only one Number Pool Block Create Event exists for the NPA-NXX-X value. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.2.9 |  | Priority: |  | - SOA LTI; - N/A. - SOA; - C. - non-EDR LSMS; - R. - EDR LSMS; - R. |
|  | Objective: |  | SOA - Service Provider Personnel modify the routing data for an active Number Pool Block and broadcast to at least 4 LSMSs (1 EDR and 3 non-EDR systems) resulting in Partial Failure (2 non-EDR systems fail two different TNs, the EDR system and one non-EDR system is successful) – Success |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-137.3, Table RR3-137.3 (Row 2), RR3-138.2, Table RR3-138.2 (Row 2), RR3-128, RR3-157, RR3-159, RR3-160, RR3-162, RR3-163, RR3-164, RR3-165, RR3-166, RR5-85, RR5-87, RR5-103, RR5-104, RR5-105, RR5-106 |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  | 2.10 Number Pool Block Modify by NPAC SMS 2.14.1 Number Pool Block Modify Partial Failure Broadcast to Local SMSs 2.14.2 Number Pool Block Modify Partial Failure Broadcast NPAC SMS Updates |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that the active Number Pool Block to be modified exists on the NPAC SMS with a status of ‘active’, an empty Failed SP List and the SOA Origination Indicator is set to TRUE. - Verify that at least 4 LSMSs (1 EDR and 3 non-EDR) are configured such that they will be sent downloads for this NPA-NXX. - Verify that the EDR LSMS and that only one of the non-EDR LSMSs that are accepting downloads for the NPA-NXX are associated with the NPAC SMS. |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |
| 1. | NPAC | Using the SOA, Service Provider Personnel submit an M-SET Request numberPoolBlock to the NPAC SMS to modify a Number Pool Block. The following attributes may be modified: - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | NPAC |  | - The NPAC SMS receives the M-SET Request numberPoolBlock - The NPAC SMS performs the following actions: - Updates the LRN in the Number Pool Block object. - Sets the numberPoolBlockStatus to 'sending'. - Updates the numberPoolBlockBroadcastTimeStamp and numberPoolBlockModifiedTimeStamp to the current date and time. |  |  |
| 2. | NPAC | The NPAC SMS issues an M-SET Response numberPoolBlock to the Service Provider SOA |  | NPAC |  | The Service Provider SOA receives the M-SET Response numberPoolBlock. |  |  |
| 3. | NPAC | The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself. |  | SP |  | - The NPAC SMS performs the following actions: - Updates the LRN in the Subscription Versions within the 1K Block with LNP Type set to ‘POOL’. - Sets the subscriptionVersionStatus to ‘sending’. - Updates the subscriptionVersionBroadcastTimeStamp and the subscriptionVersionModifiedTimeStamp to the current date and time. - The NPAC SMS issues an M-SET Response subscriptionVersionNPAC to itself. |  |  |
| 4. | NPAC | - The NPAC SMS issues an M-SET Request subscriptionVersion to update the attributes on the Subscription Version objects on all Subscription Versions of LNP Type set to ‘POOL’ to the non-EDR LSMSs that are accepting downloads for this NPA-NXX. - At the same time as step 4.1, the NPAC SMS issues an M-SET Request numberPoolBlock to update the attributes on the Number Pool Block object to the EDR LSMSs that are accepting downloads for this NPA-NXX. |  | #### SP |  | - The NPAC SMS waits for a response from all LSMSs that are accepting downloads for this NPA-NXX. - One non-EDR LSMS that is accepting downloads for this NPA-NXX issues an M-SET Response indicating it successfully received the modify request. - The NPAC SMS retries any LSMS that does not respond within a tunable amount of time. |  |  |
| 5. | NPAC | After all retries have been exhausted, the NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself and performs the following actions: - updates the subscriptionVersionStatus to 'active' for Subscription Versions within the 1K Block with LNP Type set to ‘POOL’. - updates the Failed SP List to include the Service Providers in the region that are accepting downloads for that NPA-NXX and did not successfully respond to the NPAC SMS request (2 non-EDR systems). - updates the subscriptionModifiedTimeStamp to the current date and time. |  | NPAC |  | The NPAC SMS issues an M-SET Response to itself. |  |  |
| 6. | NPAC | The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself and performs the following actions: - updates the numberPoolBlockStatus to 'active'. - updates the numberPoolBlockFailedSP-List to include the Service Providers in the region that are accepting downloads for that NPA-NXX and did not successfully respond to the NPAC SMS request (2 non-EDR systems). - updates the numberPoolBlockModifiedTimeStamp to the current date and time. |  | NPAC |  | The NPAC SMS issues an M-SET Response to itself. |  |  |
| 7. | NPAC | The NPAC SMS determines the numberPoolBlockSOA-Origination Indicator is set to TRUE and issues an M-EVENT-REPORT numberPoolBlockStatusAttributeValueChange with the numberPoolBlockStatus set to ‘active’ and the numberPoolBlockFailedSP List reflecting the 2 Service Providers that failed to process the NPAC SMS request – to the NPA-NXX-X Holder SOA. |  | SP |  | The NPA-NXX-X Holder SOA issues an M-EVENT-REPORT Confirmation back to the NPAC SMS. |  |  |
| 8. | NPAC | NPAC Personnel perform a query for the Number Pool Block and the 1K Block of Subscription Versions with a LNP Type set to ‘POOL’. |  | NPAC |  | - Verify the Number Pool Block was successfully modified. - Verify the Number Pool Block has a status of ‘active’ with a Failed SP List. The Failed SP List contains the name of the two non-EDR systems that failed. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ were successfully modified. - Verify all Subscription Versions in the 1K Block have a status of ‘active’ and the Failed SP List contains the name of the two non-EDR systems that failed. |  |  |
| 9. | SP – Optional | Service Provider Personnel perform a local query for the Number Pool Block or the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | #### SP |  | - Verify the EDR LSMS does not reflect the modifications to the Number Pool Block. - Verify that one non-EDR LSMS does reflect the modifications to the 1K Block of Subscription Versions with LNP Type set to ‘POOL’, while the other two non-EDR LSMS do not reflect these modifications. |  |  |
| 10. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Number Pool Block or the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | #### SP |  | - Verify the Number Pool Block was successfully modified on the NPAC SMS. - Verify the Number Pool Block has a status of ‘active’ with a Failed SP List on the NPAC SMS. The Failed SP List contains the name of the two non-EDR systems that failed. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ were successfully modified on the NPAC SMS. - Verify all Subscription Versions in the 1K Block have a status of ‘active’ and a Failed SP List on the NPAC SMS. The Failed SP List contains the name of the two non-EDR systems that failed. |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.2.10 |  | Priority: |  | - SOA LTI; - N/A. - SOA; - C. - non-EDR LSMS; - R. - EDR LSMS; - R. |
|  | Objective: |  | SOA - Service Provider Personnel modify the routing data for an active Number Pool Block and broadcast to at least 4 LSMSs (2 EDR and 2 non-EDR systems) resulting in a Partial Failure (1 non-EDR systems fails one TN, and 1 EDR system fails) – Success |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-137.3, Table RR3-137.3 (Row 9), RR3-138.2, Table RR3-138.2 (Row 9), RR3-128, RR3-157, RR3-159, RR3-160, RR3-162, RR3-163, RR3-164, RR3-165, RR3-166, RR5-85, RR5-87, RR5-103, RR5-104, RR5-105, RR5-106 |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  | 2.10 Number Pool Block Modify by NPAC SMS 2.14.1 Number Pool Block Modify Partial Failure Broadcast to Local SMSs 2.14.2 Number Pool Block Modify Partial Failure Broadcast NPAC SMS Updates |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that the active Number Pool Block to be modified exists on the NPAC SMS with a status of ‘active’, an empty Failed SP List and the SOA Origination Indicator is set to TRUE. - Verify that at least 4 LSMSs (2 EDR and 2 non-EDR) are configured such that they will be sent downloads for this NPA-NXX. - Verify that one of the EDR LSMS and one of the non-EDR LSMSs that are accepting downloads for the NPA-NXX are associated with the NPAC SMS. |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |
| 1. | NPAC | Using the SOA, Service Provider Personnel submit an M-SET Request numberPoolBlock to the NPAC SMS to modify a Number Pool Block. The following attributes may be modified: - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | #### NPAC |  | - The NPAC SMS receives the M-SET Request numberPoolBlock - The NPAC SMS performs the following actions: - Updates the LRN in the Number Pool Block object. - Sets the numberPoolBlockStatus to 'sending'. - Updates the numberPoolBlockBroadcastTimeStamp and numberPoolBlockModifiedTimeStamp to the current date and time. |  |  |
| 2. | NPAC | The NPAC SMS issues an M-SET Response numberPoolBlock to the Service Provider SOA |  | NPAC |  | The Service Provider SOA receives the M-SET Response numberPoolBlock. |  |  |
| 3. | NPAC | The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself. |  | SP |  | - The NPAC SMS performs the following actions: - Updates the LRN in the Subscription Versions within the 1K Block with LNP Type set to ‘POOL’. - Sets the subscriptionVersionStatus to ‘sending’. - Updates the subscriptionVersionBroadcastTimeStamp and the subscriptionVersionModifiedTimeStamp to the current date and time. - The NPAC SMS issues an M-SET Response subscriptionVersionNPAC to itself. |  |  |
| 4. | NPAC | - The NPAC SMS issues an M-SET Request subscriptionVersion to update the attributes on the Subscription Version objects on all Subscription Versions of LNP Type set to ‘POOL’ to the non-EDR LSMSs that are accepting downloads for this NPA-NXX. - At the same time as step 4.1, the NPAC SMS issues an M-SET Request numberPoolBlock to update the attributes on the Number Pool Block object to the EDR LSMSs that are accepting downloads for this NPA-NXX. |  | #### SP |  | - The NPAC SMS waits for a response from all LSMSs that are accepting downloads for this NPA-NXX. - One non-EDR LSMS that is accepting downloads for this NPA-NXX issues an M-SET Response indicating it successfully received the modify request. - The NPAC SMS retries any LSMS that does not respond within a tunable amount of time. |  |  |
| 5. | NPAC | After all retries have been exhausted, the NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself and performs the following actions: - updates the subscriptionVersionStatus to 'active' for Subscription Versions within the 1K Block with LNP Type set to ‘POOL’. - updates the Failed SP List to include the Service Providers in the region that are accepting downloads for that NPA-NXX and did not successfully respond to the NPAC SMS request (1 EDR system and 1 non-EDR system). - updates the subscriptionModifiedTimeStamp to the current date and time. |  | NPAC |  | The NPAC SMS issues an M-SET Response to itself. |  |  |
| 6. | NPAC | The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself and performs the following actions: - updates the numberPoolBlockStatus to 'active'. - updates the numberPoolBlockFailedSP-List to include the Service Providers in the region that are accepting downloads for that NPA-NXX and did not successfully respond to the NPAC SMS request (1 EDR system and 1 non-EDR system). - updates the numberPoolBlockModifiedTimeStamp to the current date and time. |  | NPAC |  | The NPAC SMS issues an M-SET Response to itself. |  |  |
| 7. | NPAC | The NPAC SMS determines the numberPoolBlockSOA-Origination Indicator is set to TRUE and issues an M-EVENT-REPORT numberPoolBlockStatusAttributeValueChange with the numberPoolBlockStatus set to ‘active’ and the numberPoolBlockFailedSP List reflecting the 2 Service Providers that failed to process the NPAC SMS request – to the NPA-NXX-X Holder SOA. |  | SP |  | The NPA-NXX-X Holder SOA issues an M-EVENT-REPORT Confirmation back to the NPAC SMS. |  |  |
| 8. | NPAC | NPAC Personnel perform a query for the Number Pool Block and the 1K Block of Subscription Versions with a LNP Type set to ‘POOL’. |  | NPAC |  | - Verify the Number Pool Block was successfully modified. - Verify the Number Pool Block has a status of ‘active’ with a Failed SP List. The Failed SP List contains the name of the two LSMS systems that failed. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ were successfully modified. - Verify all Subscription Versions in the 1K Block have a status of ‘active’ and the Failed SP List contains the name of the two LSMS systems that failed. |  |  |
| 9. | SP – Optional | Service Provider Personnel perform a local query for the Number Pool Block or the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | #### SP |  | - Verify the EDR LSMS does not reflect the modifications to the Number Pool Block. - Verify that one non-EDR LSMS does reflect the modifications to the 1K Block of Subscription Versions with LNP Type set to ‘POOL’, while the other non-EDR LSMS does not reflect these modifications. |  |  |
| 10. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Number Pool Block or the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | #### SP |  | - Verify the Number Pool Block was successfully modified on the NPAC SMS. - Verify the Number Pool Block has a status of ‘active’ with a Failed SP List on the NPAC SMS. The Failed SP List contains the name of the two LSMS systems that failed. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ were successfully modified on the NPAC SMS. - Verify all Subscription Versions in the 1K Block have a status of ‘active’ and a Failed SP List on the NPAC SMS. The Failed SP List contains the name of the two LSMS systems that failed. |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.2.11 |  | Priority: |  | - SOA LTI; - N/A. - SOA; - C. - non-EDR LSMS; - R. - EDR LSMS; - R. |
|  | Objective: |  | SOA - Service Provider Personnel modify the routing data for an active Number Pool Block and broadcast to at least 4 LSMSs (2 EDR and 2 non-EDR systems) resulting in a Partial Failure (1 non-EDR systems fails one TN, and 2 EDR systems fail) – Success |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-137.3, Table RR3-137.3 (Row 12), RR3-138.2, Table RR3-138.2 (Row 12), RR3-128, RR3-157, RR3-159, RR3-160, RR3-162, RR3-163, RR3-164, RR3-165, RR3-166, RR5-85, RR5-87, RR5-103, RR5-104, RR5-105, RR5-106 |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  | 2.10 Number Pool Block Modify by NPAC SMS 2.14.1 Number Pool Block Modify Partial Failure Broadcast to Local SMSs 2.14.2 Number Pool Block Modify Partial Failure Broadcast NPAC SMS Updates |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | - Verify that the active Number Pool Block to be modified exists on the NPAC SMS with a status of ‘active’, an empty Failed SP List and the SOA Origination Indicator is set to TRUE. - Verify that at least 4 LSMSs (2 EDR and 2 non-EDR) are configured such that they will be sent downloads for this NPA-NXX. - Verify that only one of the non-EDR LSMSs that are accepting downloads for the NPA-NXX are associated with the NPAC SMS. - Verify that the other two non-EDR LSMSs that are accepting downloads for the NPA-NXX will fail one TN in the 1K Block. |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |
| 1. | NPAC | Using the SOA, Service Provider Personnel submit an M-SET Request numberPoolBlock to the NPAC SMS to modify a Number Pool Block. The following attributes may be modified: - numberPoolBlockLRN - numberPoolBlockCLASS-DPC - numberPoolBlockCLASS-SSN - numberPoolBlockCNAM-DPC - numberPoolBlockCNAM-SSN - numberPoolBlockLIDB-DPC - numberPoolBlockLIDB-SSN - numberPoolBlockISVM-DPC - numberPoolBlockISVM-SSN - numberPoolBlockWSMSC-DPC – if supported by the Service Provider SOA - numberPoolBlockWSMSC-SSN – if supported by the Service Provider SOA |  | #### NPAC |  | - The NPAC SMS receives the M-SET Request numberPoolBlock - The NPAC SMS performs the following actions: - Updates the LRN in the Number Pool Block object. - Sets the numberPoolBlockStatus to 'sending'. - Updates the numberPoolBlockBroadcastTimeStamp and numberPoolBlockModifiedTimeStamp to the current date and time. |  |  |
| 2. | NPAC | The NPAC SMS issues an M-SET Response numberPoolBlock to the Service Provider SOA |  | NPAC |  | The Service Provider SOA receives the M-SET Response numberPoolBlock. |  |  |
| 3. | NPAC | The NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself. |  | SP |  | - The NPAC SMS performs the following actions: - Updates the LRN in the Subscription Versions within the 1K Block with LNP Type set to ‘POOL’. - Sets the subscriptionVersionStatus to ‘sending’. - Updates the subscriptionVersionBroadcastTimeStamp and the subscriptionVersionModifiedTimeStamp to the current date and time. - The NPAC SMS issues an M-SET Response subscriptionVersionNPAC to itself. |  |  |
| 4. | NPAC | - The NPAC SMS issues an M-SET Request subscriptionVersion to update the attributes on the Subscription Version objects on all Subscription Versions of LNP Type set to ‘POOL’ to the non-EDR LSMSs that are accepting downloads for this NPA-NXX. - At the same time as step 4.1, the NPAC SMS issues an M-SET Request numberPoolBlock to update the attributes on the Number Pool Block object to the EDR LSMSs that are accepting downloads for this NPA-NXX. |  | #### SP |  | - The NPAC SMS waits for a response from all LSMSs that are accepting downloads for this NPA-NXX. - One non-EDR LSMS that is accepting downloads for this NPA-NXX issues an M-SET Response indicating it successfully received the modify request. - The NPAC SMS retries any LSMS that does not respond within a tunable amount of time. |  |  |
| 5. | NPAC | After all retries have been exhausted, the NPAC SMS issues an M-SET Request subscriptionVersionNPAC to itself and performs the following actions: - updates the subscriptionVersionStatus to 'active' for Subscription Versions within the 1K Block with LNP Type set to ‘POOL’. - updates the Failed SP List for the specific Subscription Versions to include the Service Providers in the region that are accepting downloads for that NPA-NXX and did not successfully respond to the NPAC SMS request (2 non-EDR systems that failed different Subscription Versions). - updates the subscriptionModifiedTimeStamp to the current date and time. |  | NPAC |  | The NPAC SMS issues an M-SET Response to itself. |  |  |
| 6. | NPAC | The NPAC SMS issues an M-SET Request numberPoolBlockNPAC to itself and performs the following actions: - updates the numberPoolBlockStatus to 'active'. - updates the numberPoolBlockFailedSP-List to include the Service Providers in the region that are accepting downloads for that NPA-NXX and did not successfully respond to the NPAC SMS request (2 non-EDR systems). - updates the numberPoolBlockModifiedTimeStamp to the current date and time. |  | NPAC |  | The NPAC SMS issues an M-SET Response to itself. |  |  |
| 7. | NPAC | The NPAC SMS determines the numberPoolBlockSOA-Origination Indicator is set to TRUE and issues an M-EVENT-REPORT numberPoolBlockStatusAttributeValueChange with the numberPoolBlockStatus set to ‘active’ and the numberPoolBlockFailedSP List reflecting the 2 Service Providers that failed to process the NPAC SMS request – to the NPA-NXX-X Holder SOA. |  | SP |  | The NPA-NXX-X Holder SOA issues an M-EVENT-REPORT Confirmation back to the NPAC SMS. |  |  |
| 8. | NPAC | NPAC Personnel perform a query for the Number Pool Block and the 1K Block of Subscription Versions with a LNP Type set to ‘POOL’. |  | NPAC |  | - Verify the Number Pool Block was successfully modified. - Verify the Number Pool Block has a status of ‘active’ with a Failed SP List. The Failed SP List contains the name of the two LSMS systems that failed. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ were successfully modified. - Verify all Subscription Versions in the 1K Block have a status of ‘active’ and the Failed SP List is empty on all but the two Subscription Versions that the two non-EDR systems failed. On one of those Subscription Versions it should have the name of one of the non-EDR systems and on the other Subscription Version it should have the name of the other non-EDR system. |  |  |
| 9. | SP – Optional | Service Provider Personnel perform a local query for the Number Pool Block or the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | #### SP |  | - Verify the EDR LSMS does not reflect the modifications to the Number Pool Block. - Verify that one non-EDR LSMS does reflect the modifications to the 1K Block of Subscription Versions with LNP Type set to ‘POOL’, while the other two non-EDR LSMS do not reflect these modifications. |  |  |
| 10. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Number Pool Block or the 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | #### SP |  | - Verify the Number Pool Block was successfully modified on the NPAC SMS. - Verify the Number Pool Block has a status of ‘active’ with a Failed SP List on the NPAC SMS. The Failed SP List contains the name of the two LSMS systems that failed. - Verify the 1K Block of Subscription Versions with LNP Type set to ‘POOL’ were successfully modified on the NPAC SMS. - Verify all Subscription Versions in the 1K Block have a status of ‘active’ and the Failed SP List is empty on all but the two Subscription Versions that the two non-EDR systems failed. On one of those Subscription Versions it should have the name of one of the non-EDR systems and on the other Subscription Version it should have the name of the other non-EDR system. |  |  |

## 4.3 Delete Number Pool Block Test Cases:

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.3.1 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | N/A |
|  |  |  |  |  |  |  | non-EDR LSMS |  | N/A |
|  |  |  |  |  |  |  | EDR LSMS |  | N/A |
|  | Objective: |  | NPAC OP GUI - NPAC Personnel attempt to delete a Number Pool Block - Error |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-169, RR5-107, RR5-108, RR5-109 |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  | N/A |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Verify that an active Number Pool Block with an empty Failed SP List exists on the NPAC SMS. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | NPAC | Using the NPAC OP GUI, NPAC Personnel attempt to delete a Number Pool Block. |  | NPAC |  | There is not a mechanism via the NPAC OP GUI to “delete” a Number Pool Block. The GUI will not allow this scenario. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.3.2 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | N/A |
|  |  |  |  |  |  |  | non-EDR LSMS |  | N/A |
|  |  |  |  |  |  |  | EDR LSMS |  | N/A |
|  | Objective: |  | SOA - Service Provider Personnel attempt to delete a Number Pool Block over the SOA to NPAC SMS interface - Error |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-170 |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  |  |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Verify that an active Number Pool Block with an empty Failed SP List exists on the NPAC SMS. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | Using the SOA, Service Provider Personnel attempt to submit an M-DELETE Request numberPoolBlock for a Number Pool Block to the NPAC SMS. |  | NPAC |  | - The NPAC SMS receives the M-DELETE Request numberPoolBlock from the Service Provider SOA. - The NPAC SMS determines the request to delete the Number Pool Block is invalid. (This violates system requirements). |  |  |  |
| 2. | NPAC | The NPAC SMS issues an M-DELETE Error Response to the Service Provider SOA. |  | SP |  | The Service Provider SOA receives the M-DELETE Error Response. |  |  |  |
| 3. | NPAC | NPAC Personnel perform a query for the Number Pool Block and 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | NPAC |  | - Verify the Number Pool Block exists on the NPAC SMS with a status of ‘active’ and an empty Failed SP List. - Verify the 1K Block of Subscription Version with LNP Type set to ‘POOL’ exist on the NPAC SMS with a status of ‘active’ and an empty Failed SP List. |  |  |  |
| 4. | SP - Optional | Service Provider Personnel perform a local query for the Number Pool Block and 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | SP |  | Verify the Number Pool Block exists with a status of ‘active’ and an empty Failed SP List. (Assuming that the Block existed on your SOA prior to attempting to delete it in this Test Case. If the Block did not exist on your SOA, then you do not need to perform Row 4.) |  |  |  |
| 5. | SP – Conditional | Service Provider Personnel perform an NPAC SMS query for the Number Pool Block and 1K Block of Subscription Versions with LNP Type set to ‘POOL’. |  | SP |  | - Verify the Number Pool Block exists on the NPAC SMS with a status of ‘active’ and an empty Failed SP List. - Verify the 1K Block of Subscription Version with LNP Type set to ‘POOL’ exist on the NPAC SMS with a status of ‘active’ and an empty Failed SP List. |  |  |  |

* There is not a flow for this functionality – so this test case is based on the assumption that the Service Provider SOA would issue an M-DELETE numberPoolBlock in an attempt to delete a Number Pool Block.  Functional Requirements prohibit a Number Pool Block Delete Request (of any type) over the SOA to NPAC SMS Interface.

## 4.4 Query Number Pool Block Test Cases:

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.4.1 |  | SUT Priority: |  | SOA LTI |  | N/A |
|  |  |  |  |  |  |  | SOA |  | C |
|  |  |  |  |  |  |  | non-EDR LSMS |  | N/A |
|  |  |  |  |  |  |  | EDR LSMS |  | N/A |
|  | Objective: |  | SOA - Service Provider Personnel submit a Query Number Pool Block Request to the NPAC SMS using an NPA-NXX-X value as filter criteria. - Success |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-181, RR3-182 |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  | 2.27 Number Pool Number Pool Block Query by SOA/LSMS |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Verify that more than one active Number Pool Block with an empty Failed SP List exist for a given Service Provider on the NPAC SMS. |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |
| 1. | SP | - Using the SOA, Service Provider Personnel submit a numberPoolBlock object query to the NPAC SMS for a Number Pool Block. Filter criteria used for the query is the NPA-NXX-X value. - The SOA issues an M-GET Request numberPoolBlock requesting a single numberPoolBlock object by NPA-NXX-X value to the NPAC SMS. |  | NPAC |  | The NPAC SMS receives the M-GET Request over the SOA to NPAC SMS interface. |  |  |  |
| 2. | NPAC | - The NPAC SMS locates the numberPoolBlock object that matched the query criteria submitted by the SOA. - The NPAC SMS issues an M-GET Response numberPoolBlock with a single M-GET reply with all the attributes associated with the numberPoolBlock. |  | SP |  | The SOA receives the M-GET response for the numberPoolBlock query results: |  |  |  |
| 3. | SP | - Service Provider personnel view the Number Pool Blocks that the NPAC SMS returned and verify the following Number Pool Block attributes are provided for each Number Pool Block: - Block Id - Block Holder SPID - NPA-NXX-X - LRN - CLASS DPC - CLASS SSN - LIDB DPC - LIDB SSN - CNAM DPC - CNAM SSN - ISVM DPC ISVM SSN - WSMSC DPC - WSMSC SSN - Creation Date - Activation Start TimeStamp - Activation Broadcast TimeStamp - Last Modified TimeStamp - Disconnect Request TimeStamp - Disconnect Broadcast TimeStamp - Disconnect Broadcast Complete TimeStamp - Old Time Stamp - Modify Request TimeStamp - Modify Broadcast TimeStamp - Modify Broadcast Complete TimeStamp - SOA Origination Indicator - Status - Download Reason |  | SP |  | All attributes are returned to the SOA. |  |  |  |

| A. | TEST IDENTITY |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Case Number: |  | 4.4.2 |  | SUT Priority: |  | SOA LTI |  | N/A |  |
|  |  |  |  |  |  |  | SOA |  | N/A |  |
|  |  |  |  |  |  |  | non-EDR LSMS |  | C |  |
|  |  |  |  |  |  |  | EDR LSMS |  | C |  |
|  | Objective: |  | LSMS - Service Provider Personnel submit a Number Pool Block query request over the LSMS to NPAC SMS Interface using SPID as filter criteria Success |  |  |  |  |  |  |  |
| B. | REFERENCES |  |  |  |  |  |  |  |  |  |
|  | NANC Change Order Revision Number: |  |  |  | Change Order Number(s): |  |  |  |  |  |
|  | NANC FRS Version Number: |  |  |  | Relevant Requirement(s): |  | RR3-181, RR3-182 |  |  |  |
|  | NANC IIS Version Number: |  |  |  | Relevant Flow(s): |  | 2.27 Number Pool Block Query by SOA/LSMS |  |  |  |
| C. | PREREQUISITE |  |  |  |  |  |  |  |  |  |
|  | Prerequisite Test Cases: |  |  |  |  |  |  |  |  |  |
|  | Prerequisite NPAC Setup: |  | Verify that more than one active Number Pool Block with an empty Failed SP List exist for the query criteria on the NPAC SMS. |  |  |  |  |  |  |  |
|  | Prerequisite SP Setup: |  |  |  |  |  |  |  |  |  |
| D. | TEST STEPS and EXPECTED RESULTS |  |  |  |  |  |  |  |  |  |
| Row # | NPAC or SP | Test Step |  | NPAC or SP |  | Expected Result |  |  |  |  |
| 1. | SP | - Using the LSMS, Service Provider Personnel submit a numberPoolBlock object query to the NPAC SMS for a Number Pool Block. Filter criteria used for the query is the Number Pool Block ID. - The LSMS issues an M-GET Request numberPoolBlock requesting a single numberPoolBlock object by numberPoolBlockId to the NPAC SMS. |  | NPAC |  | - The NPAC SMS receives the M-GET Request over the LSMS to NPAC SMS interface. |  |  |  |  |
| 2. | NPAC | - The NPAC SMS locates the numberPoolBlock object that matched the query criteria submitted by the LSMS. - The NPAC SMS issues an M-GET Response numberPoolBlock with a single M-GET reply with all the attributes associated with the numberPoolBlock. |  | SP |  | The Service Provider Personnel, receives the M-GET response for the numberPoolBlock query results. |  |  |  |  |
| 3. | SP | Service Provider personnel view the Number Pool Blocks that the NPAC SMS returned and verify the following Number Pool Block attributes for each Number Pool Block: - Block Id - Block Holder SPID - NPA-NXX-X - LRN - CLASS DPC - CLASS SSN - LIDB DPC - LIDB SSN - CNAM DPC - CNAM SSN - ISVM DPC ISVM SSN - WSMSC DPC - WSMSC SSN - Creation Date - Activation Start TimeStamp - Activation Broadcast TimeStamp - Last Modified TimeStamp - Disconnect Request TimeStamp - Disconnect Broadcast TimeStamp - Disconnect Broadcast Complete TimeStamp - Old Time Stamp - Modify Request TimeStamp - Modify Broadcast TimeStamp - Modify Broadcast Complete TimeStamp - SOA Origination Indicator - Status - Download Reason |  | SP |  | All attributes are returned to the LSMS. |  |  |  |  |