### 1.2.3.7  - Cause Code Value 26

1.2.3.7.1	Ported Number to Ported Number (Direct Trunk)

Test Description:

This will test that Cause Code Value 26 will be sent back appropriately on a call from a ported number to a ported number.

Test Procedure:

- From a ported number

- Call a ported number

- Terminating LSP identifies own LRN, but does not find ported number on switch

- Document Test Results, including time required for all transitional steps

Test Results:

- Originating LSP routes call to destinations LSP

- Originating LSP performs LRN database dip

- Originating LSP sends JIP

- Originating LSP ISUP IAM contains LRN in CdPN parameter, ported destination number in GAP, bit M of FCI set, JIP

- Terminating LSP recognizes LRN as its own, but does not find ported destination number on switch

- Terminating LSP releases call with Cause Code 26 and sends Release message back to originating LSP

1.2.3.7.2	Ported Number to Ported Number (Via Access Tandem)

Test Description:

This will test that Cause Code Value 26 will be sent back appropriately on a call from a ported number to a ported number.

Test Procedure:

- From a ported number

- Call a ported number

- Terminating LSP identifies own LRN, but does not find ported number on switch

- Document Test Results, including time required for all transitional steps

Test Results:

- Originating LSP routes call to access tandem

- Originating LSP performs LRN database dip

- Originating LSP sends JIP

- Originating LSP ISUP IAM contains LRN in CdPN parameter, ported destination number in GAP, bit M of FCI set, JIP

- ISUP message is not the LRN of AT, AT forwards call to destination

- Terminating LSP recognizes LRN as its own, but does not find ported destination number on switch

- Terminating LSP releases call with Cause Code 26 and sends Release message back to originating LSP

1.2.3.7.3	Ported to Ported Number (Via Toll Carrier)

Test Description:

This will test that Cause Code Value 26 will be sent back appropriately on a call from a ported number to a ported number.

Test Procedure:

- From a ported number

- Call a ported number

- Terminating LSP identifies own LRN, but does not find ported number on switch

- Document Test Results, including time required for all transitional steps

Test Results:

- Originating LSP routes call to toll carrier

- Originating LSP does not performs LRN database dip

- Originating LSP sends JIP

- Originating LSP ISUP IAM contains ported number in CdPN parameter, no GAP, bit M of FCI not set, JIP

- Toll carrier performs LRN dip

- Toll carrier ISUP IAM contains LRN in CdPN parameter, ported number in GAP, bit M of FCI set, JIP

- Terminating LSP releases call with Cause Code 26 and sends Release message back to originating LSP