### 1.2.3.2  - Default Routing

1.2.3.2.1	Intra-LATA Default routing (SS7) Ported Number to Ported Number

Test Description:

This will test that an Intra-LATA call can be completed from a ported number to a ported number using default routing.

Test Procedure:

- From a ported number

- Call a ported number

- Originating LSP not successful in performing LRN database dip

- Document Test Results, including time required for all transitional steps

Test Results:

- Originating LSP is not successful in performing LRN database dip

- Originating LSP routes call to default LSP

- Originating LSP ISUP IAM contains ported destination number in CdPN parameter, no GAP, bit M of FCI not set

- Default LSP performs LRN database dip

- Default LSP routes call to LSP serving ported number

- Default LSP to serving LSP ISUP IAM contains LRN in CdPN parameter, ported destination number in GAP, bit M of FCI set

- Serving LSP completes call to ported number.

1.2.3.2.3	Intra-LATA Toll Default routing (SS7) Ported Number to Ported Number

Test Description:

This will test that an Intra-LATA call can be completed from a ported number to a ported number through an access tandem using default routing.

Test Procedure:

- From a ported number

- Call a ported number

- Originating LSP not successful in performing LRN database dip

- Document Test Results, including time required for all transitional steps

Test Results:

- Originating LSP is not successful in performing LRN database dip

- Originating LSP routes call to access tandem

- Originating LSP ISUP IAM contains ported destination number in CdPN parameter, no GAP, bit M of FCI not set

- Access Tandem performs LRN database dip

- Access Tandem routes call to LSP serving ported number

- Default LSP to serving LSP ISUP IAM contains LRN in CdPN parameter, ported destination number in GAP, bit M of FCI set

- Serving LSP completes call to ported number

1.3.11.3.1	Inter-LATA Default Routing: Ported Number to Ported Number

Test Description:

This will test that an inter-LATA call can be completed from a ported number to a ported number via default routing.

Test Procedure:

- From a ported number

- Call a ported number

- IXC not successful in performing LRN database dip

Test Results:

- Originating LSP routes call to appropriate IXC

- Originating LSP does not perform LRN database dip

- Originating LSP sends JIP

- IXC is not successful in performing LRN database dip

- IXC routes call to default LSP

- IXC to LSP ISUP IAM contains ported destination number in CdPN parameter, no GAP, bit M of FCI not set, JIP

- Default LSP performs LRN database dip

- Default LSP routes call to LSP serving ported number

- Default LSP to serving LSP ISUP IAM contains LRN in CdPN parameter, ported destination number in GAP, bit M of FCI set, JIP

- Serving LSP completes call to ported number

- LRN database dip recorded in default LSP AMA record